ISO/IEC CD 23090-38:2024(E)

ISO/IEC JTC 1/SC 29/WG 7

Secretariat: JISC

Information technology — Coded representation of immersive media — Part 38: Enhanced geometry-based point cloud compression

CD stage

Warning for WDs and CDs

This document is not an ISO International Standard. It is distributed for review and comment. It is subject to change without notice and may not be referred to as an International Standard.

Recipients of this draft are invited to submit, with their comments, notification of any relevant patent rights of which they are aware and to provide supporting documentation.

© ISO/IEC 2024

All rights reserved. Unless otherwise specified, or required in the context of its implementation, no part of this publication may be reproduced or utilized otherwise in any form or by any means, electronic or mechanical, including photocopying, or posting on the internet or an intranet, without prior written permission. Permission can be requested from either ISO at the address below or ISO’s member body in the country of the requester.

ISO copyright office

CP 401 • Ch. de Blandonnet 8

CH-1214 Vernier, Geneva

Phone: +41 22 749 01 11

Fax: +41 22 749 09 47

Email: copyright@iso.org

Website: www.iso.org

Published in Switzerland

Contents

Foreword	ix

Introduction	x

1 Scope	1

2 Normative references	1

3 Terms and definitions	1

3.1 General terms	1

3.2 High-level syntax and entropy coding terms	5

3.3 Tree structure terms	7

3.4 Geometry coding terms	8

3.5 Attribute coding terms	9

3.6 Fine granularity slices terms	10

4 Abbreviated terms	10

4.1 Acronyms	10

4.2 Mnemonics	11

5 Conventions	11

5.1 General	11

5.2 Symbolic names	11

5.3 Numerical representation	12

5.4 Arithmetic operators	12

5.5 Logical operators	12

5.6 Relational operators	12

5.7 Bit-wise operators	13

5.8 Assignment operators	13

5.9 Range notation	13

5.10 Mathematical functions	14

5.10.1 General	14

5.10.2 IntAtan2	15

5.10.3 IntCos and IntSin	15

5.10.4 IntSqrt	16

5.10.5 IntRecipSqrt	16

5.10.6 IntRecip	17

5.10.7 Div	18

5.10.8 Morton	18

5.10.9 FromMorton	18

5.11 Order of operation precedence	19

5.12 Named expressions	19

5.12.1 General	19

5.12.2 Scope	20

5.12.3 Arguments of named expressions	20

5.12.4 Sub-expressions	21

5.12.5 Definitions with multiple statements	21

5.12.6 Textual definitions	21

5.13 Variables, syntax elements and tables	21

6 Point cloud format and relationship to coded and output representations	22

6.1 General format	22

6.2 Attributes	22

6.2.1 General	22

6.2.2 Colour	22

6.2.3 Opacity	23

6.2.4 Reflectance	23

6.2.5 Normal vector	23

6.2.6 Material identifier	23

6.2.7 Frame number/index	23

6.2.8 User defined attributes	24

6.3 Codec-derived attributes	24

6.3.1 General	24

6.3.2 Slice identifier	24

6.3.3 Slice tag	24

6.3.4 Canonical point order	24

6.3.5 Point Morton order	24

6.4 Coded point cloud format	25

6.4.1 Sequence coordinate system	25

6.4.2 Coding coordinate system	26

6.4.3 Coded point cloud sequence	27

6.4.4 Coded point cloud frame	27

6.4.5 Slice of a coded point cloud frame	27

6.4.6 Repetition of slices	28

6.4.7 Relationship between tiles and slices	28

6.4.8 Parameter sets	29

6.5 Output point cloud format	30

6.5.1 General	30

6.5.2 Coordinate system	30

6.5.3 Fixed-point conformance output	30

6.5.4 Attributes	30

6.5.5 Output point cloud sequence	30

6.5.6 Output point cloud frame	30

7 Syntax and semantics	31

7.1 Method of specifying syntax in tabular form	31

7.2 Specification of syntax functions and descriptors	32

7.3 Syntax in tabular form	32

7.3.1 General	32

7.3.2 Parameter sets, ancillary data and byte alignment	32

7.3.3 Geometry data unit	42

7.3.4 Attribute data unit	50

7.3.5 Defaulted attribute data unit syntax	53

7.4 Semantics	54

7.4.1 General	54

7.4.2 Parameter sets, ancillary data and byte alignment	54

7.4.3 Geometry data unit	70

7.4.4 Attribute data unit	72

7.4.5 Defaulted attribute data unit semantics	73

8 Decoding process	73

8.1 General decoding process	73

8.2 Frame decoding processes	74

8.2.1 General	74

8.2.2 Frame counter	74

8.3 Slice decoding processes	74

8.3.1 General	74

8.3.2 State variables	74

8.3.3 Geometry decoding process	75

8.3.4 Default attribute values	75

8.3.5 Attribute decoding process	75

8.3.6 At the end of a slice	75

8.3.7 Update of inter prediction buffer	75

9 Slice geometry	76

9.1 General	76

9.2 Occupancy tree	76

9.2.1 General	76

9.2.2 Coded occupancy tree	76

9.2.3 Occupancy tree syntax element semantics	79

9.2.4 Node dimensions per tree level	79

9.2.5 State representation	80

9.2.6 Occupancy tree node coding	80

9.2.7 Occupied neighbourhood patterns	86

9.2.8 Neighbourhood-permuted node occupancy bitmap	101

9.2.9 Dictionary coding of occupancy_byte	102

9.2.10 Bitwise occupancy coding	107

9.2.11 Planar occupancy coding	116

9.2.12 Direct nodes	125

9.2.13 Angular coding	131

9.2.14 Subtree scaling	142

9.2.15 Reference frame generation	147

9.3 Predictive tree	150

9.3.1 General	150

9.3.2 Syntax element semantics	151

9.3.3 Tree traversal for reconstruction of point positions	153

9.3.4 Reconstruction of point coordinates	159

9.4 TriSoup	162

9.4.1 General	162

9.4.2 Syntax element semantics	165

9.4.3 TriSoup decoding process	169

10 Slice attributes	188

10.1 General	188

10.2 Point coordinates	188

10.2.1 General	188

10.2.2 Conversion to scaled angular coordinates	189

10.3 Syntax element semantics	189

10.4 Raw attribute decoding	190

10.5 Attribute decoding using the region-adaptive hierarchical transform	190

10.5.1 General	190

10.5.2 Transform tree	191

10.5.3 Coefficient order	194

10.5.4 Coefficient scaling	196

10.5.5 Transform domain prediction	198

10.5.6 Sample domain prediction and last component prediction	206

10.5.7 Inverse transform	208

10.5.8 Reconstructed attribute values	211

10.6 Attribute decoding using levels of detail	211

10.6.1 General	211

10.6.2 Syntax element semantics	212

10.6.3 Reconstruction process	212

10.6.4 State variables	212

10.6.5 Levels of detail	213

10.6.6 Predictor search	220

10.6.7 Reconstruction of attribute values	230

10.6.8 Prediction mode coding	232

10.6.9 Scaling	233

10.6.10 Coefficient prediction	234

10.6.11 Transform coefficient weights	235

10.6.12 Transform	235

10.6.13 Point quantization weights	236

10.7 Attribute quantization parameters	237

10.7.1 Syntax element semantics	237

10.7.2 Per-point regional QP offset	238

10.7.3 Attribute coefficient QP	238

10.7.4 Definition of AttrQstep	239

10.8 Reference slice generation for attribute	239

10.8.1 General	239

10.8.2 Reconstructed frame generation	239

10.8.3 Reference frame generation	239

10.8.4 Reference slice generation	241

11 Parsing process	241

11.1 General	241

11.2 Data unit buffer	244

11.2.1 General	244

11.2.2 State	244

11.2.3 Initialization at the start of parsing a data unit	244

11.2.4 Initialization at the start of parsing a geometry data unit footer	244

11.2.5 Definition of DuNextBit	245

11.3 Chunked bytestream parsing	245

11.3.1 General	245

11.3.2 Chunk syntax	245

11.3.3 Chunk semantics	246

11.3.4 State	246

11.3.5 Span of chunked bytestream data within a data unit	246

11.3.6 The chunk buffer	246

11.3.7 State update at the start of every CBS	247

11.3.8 Unpacking a single chunk	247

11.3.9 Definition of ChunkNextAeBit	247

11.3.10 Definition of ChunkNextBpBit	247

11.3.11 Boundary between spliced chunked bytestreams	247

11.3.12 Location of chunked bytestream boundaries	248

11.4 General inverse binarization processes	249

11.4.1 Parsing unsigned fixed-length codes (FL)	249

11.4.2 Parsing signed fixed-length codes (FL+S)	249

11.4.3 Parsing 𝑘-th order exp-Golomb codes (EGk)	249

11.4.4 Parsing concatenated truncated unary and 𝑘-th order exp-Golomb codes (TU+EGk)	249

11.4.5 Parsing truncated unary codes (TU)	250

11.4.6 Mapping process for signed codes	250

11.4.7 Parsing ASN.1 object identifiers	250

11.5 CABAC parsing processes	251

11.5.1 Initialization	251

11.5.2 Definition of AeReadBin	251

11.5.3 Contextual probability models	251

11.5.4 Arithmetic decoding engine	257

11.6 Parsing state memorization and restoration	259

11.6.1 General	259

11.6.2 Geometry data units	259

11.6.3 Attribute data units	260

11.6.4 Defaulted attribute data units	260

12 OBUF parsing process	260

12.1 General	260

12.2 Creation of an OBUF instance	261

12.2.1 Creation and initialization of OBUF trees	261

12.2.2 Creation and initialization of an array of OBUF ACPMs	262

12.3 Creation of a buffer of OBUF tree leaves	262

12.4 Call and update of an OBUF instance	263

12.4.1 Decode and update according to OBUF trees	263

12.4.2 Decode and update according to an element of the buffer of OBUF tree leaves	265

12.5 Decode of a bin based on an OBUF ACPM	265

12.5.1 enProbability correction based on probability bounds	266

12.5.2 Decoding of a bin using CABAC	266

Annex A (normative) Profiles and levels	267

A.1	Permitted ranges for syntax elements	267

Annex B (normative) Type-length-value encapsulated bytestream format	272

B.1	General	272

B.2	Syntax and semantics	272

B.2.1	Syntax	272

B.2.2	Semantics	272

B.3	Parsing process	273

Annex C (informative) Arithmetic encoding engine	274

C.1	General	274

C.2	State variables	274

C.3	Initial state	274

C.4	Encoding process for a single binary symbol	274

C.5	Arithmetic encoder state renormalization process	275

C.6	Arithmetic encoding engine termination process	275

Annex D (normative) Partial decoding and spatial scalability	276

D.1	General	276

D.2	Partial slice geometry	276

D.2.1	General	276

D.2.2	Selection of partial point positions	276

D.2.3	Partial point positions	277

D.2.4	Output points	277

D.3	Partial attribute decoding	277

D.3.1	General	277

D.3.2	The finest detail level	277

Annex E (normative) Fine granularity slices	278

E.1	General	278

E.2	Coded point cloud format	278

E.2.1	Fine granularity slices	278

E.2.2	Layer-group structure	278

E.3	Syntax and semantics for fine granularity slices	279

E.3.1	Syntax in tabular form	279

E.3.2	Semantics	284

E.4	Decoding process	288

E.4.1	General	288

E.4.2	Fine granularity slice decoding process	288

E.5	Fine granularity slice geometry	290

E.5.1	General	290

E.5.2	Partial tree of occupancy tree	290

E.6	Fine granularity slice attributes	294

E.6.1	General	294

E.6.2	Point coordinates	294

E.6.3	Attribute decoding using levels of detail	294

E.6.4	Parent FGS generation for attributes	299

E.7	Parsing process	300

E.7.1	General	300

E.7.2	Data unit buffer	300

E.7.3	CABAC parsing processes	300

E.7.4	Parsing state memorization and restoration	300

E.8	Partial decoding	302

E.8.1	General	302

E.8.2	Partial density decoding	303

E.8.3	Partial region decoding	304

Bibliography	305

Index of named expressions and variables	306

Foreword

ISO (the International Organization for Standardization) and IEC (the International Electrotechnical Commission) form the specialized system for worldwide standardization. National bodies that are members of ISO or IEC participate in the development of International Standards through technical committees established by the respective organization to deal with particular fields of technical activity. ISO and IEC technical committees collaborate in fields of mutual interest. Other international organizations, governmental and non-governmental, in liaison with ISO and IEC, also take part in the work.

The procedures used to develop this document and those intended for its further maintenance are described in the ISO/IEC Directives, Part 1. In particular, the different approval criteria needed for the different types of document should be noted. This document was drafted in accordance with the editorial rules of the ISO/IEC Directives, Part 2 (see www.iso.org/directives).

Attention is drawn to the possibility that some of the elements of this document may be the subject of patent rights. ISO and IEC shall not be held responsible for identifying any or all such patent rights. Details of any patent rights identified during the development of the document will be in the Introduction and/or on the ISO list of patent declarations received (see www.iso.org/patents) or the IEC list of patent declarations received (see patents.iec.ch).

Any trade name used in this document is information given for the convenience of users and does not constitute an endorsement.

For an explanation of the voluntary nature of standards, the meaning of ISO specific terms and expressions related to conformity assessment, as well as information about ISO's adherence to the World Trade Organization (WTO) principles in the Technical Barriers to Trade (TBT), see www.iso.org/iso/foreword.html.

This document was prepared by Joint Technical Committee ISO/IEC JTC 1, Subcommittee SC 29, Coding of audio, picture, multimedia and hypermedia information.

A list of all parts in the ISO/IEC 23090 series can be found on the ISO website.

Any feedback or questions on this document should be directed to the user’s national standards body. A complete listing of these bodies can be found at www.iso.org/members.html.

Introduction

Advancements in 3D capturing and rendering technologies are enabling new applications and services in the fields of assisted and autonomous driving, cartography, cultural heritage, industrial processes, immersive real-time communication, and Virtual/Augmented/Mixed reality (VR/AR/MR) content creation, transmission and communication.  Point clouds have arisen as one of the main representations for such applications.  A point cloud frame consists of a set of 3D points.  Every point, in addition to having a 3D position, may also be associated with numerous other attributes such as colour, transparency, reflectance, timestamp, surface normal and classification.  Such representations require a large amount of data, which can be costly in terms of storage and transmission.  This document provides the method for efficiently compressing point cloud representations.

Information technology — Coded representation of immersive media — Part 38: Enhanced geometry-based point cloud compression

- Scope

This document specifies enhanced geometry-based point cloud compression.

- Normative references

The following documents are referred to in the text in such a way that some or all of their content constitutes requirements of this document.  For dated references, only the edition cited applies.  For undated references, the latest edition of the referenced document (including any amendments) applies.

Recommendation ITU-T T.35, Procedure for the allocation of ITU-T defined codes for non-standard facilities

ISO/IEC 8825-1 (Rec. ITU-T X.690), Information technology — ASN.1 encoding rules — Part 1: Specification of Basic Encoding Rules (BER), Canonical Encoding Rules (CER) and Distinguished Encoding Rules (DER)

ISO/IEC 9834-1 (Rec. ITU-T X.660), Information technology — Procedures for the operation of object identifier registration authorities — Part 1: General procedures and top arcs of the international object identifier tree

ISO/IEC 9834-8 (Rec. ITU-T X.667), Information technology — Procedures for the operation of object identifier registration authorities — Part 8: Generation of universally unique identifiers (UUIDs) and their use in object identifiers

ISO/IEC 23091-2, Information technology — Coding-independent code points — Part 2: Video

- Terms and definitions

For the purposes of this document, the following terms and definitions apply.

ISO and IEC maintain terminological databases for use in standardization at the following addresses:

- ISO Online browsing platform: available at https://www.iso.org/obp

- IEC Electropedia: available at http://www.electropedia.org/

  - General terms

point

fundamental element of a point cloud (3.1.2) comprising a position specified as Cartesian coordinates (3.1.9) and zero or more attributes (3.1.20)

point cloud

unordered list of points (3.1.1)

point cloud sequence

sequence of one or more point clouds (3.1.2)

point cloud frame

point cloud (3.1.2) in a point cloud sequence (3.1.3)

coded point cloud frame

coded representation of a point cloud frame (3.1.4)

reference point cloud frame

a coded point cloud frame (3.1.5) which contains points (3.1.1) that may be used for inter prediction (3.1.28) in the decoding process of subsequent point cloud frames (3.1.4) in decoding order

canonical point order
canonical decoding order

order of points (3.1.1) decoded from a slice (3.1.22) according to the decoding and parsing processes specified in this document

bounding box

axis-aligned cuboid defining a spatial region that bounds a set of points (3.1.1)

coordinates

<Cartesian> three scalar multiples of respective orthogonal XYZ (3.1.12) unit vectors with finite precision and bounds that specify a position relative to a fixed reference

coordinates

<angular> a position specified as the radial distance 𝜌 from the V axis, an azimuth angle 𝜑 in the S-T plane and an indexed elevation

coordinates

<attribute> either STV (3.1.13) or scaled RPI (3.1.14) point coordinates used to code an attribute

XYZ (axes)

X, Y and Z axes, in that order, used to represent Cartesian coordinates (3.1.9)

STV (axes)

S, T and V axes, in that order, that are a sequence-dependent permutation of the XYZ axes (3.1.12); used to represent the coded geometry (3.1.19)

RPI (axes)

R, P and I axes, in that order, used to represent angular coordinates (3.1.10)

sequence coordinate system

scaled and translated application-specific coordinate system that applies to an entire coded point cloud sequence (3.1.3), and in which all points (3.1.1) have non-negative, fixed-point coordinates

coding coordinate system

scaled sequence coordinate system (3.1.15) that applies for an entire coded point cloud sequence (3.1.3), and in which all points (3.1.1) have non-negative integer coordinates

slice coordinate system

translated coding coordinate system (3.1.16) that applies for a single slice (3.1.22), and in which all points (3.1.1) in the slice have non-negative integer coordinates

beam

sampler of point positions using angular coordinates by rays cast with a fixed elevation and from a point on and rotating around the V axis at the angular origin

geometry

point positions (3.4.1) associated with a set of points (3.1.1)

attribute

scalar or vector property associated with each point (3.1.1) in a point cloud (3.1.2)

EXAMPLE	Colour, reflectance, frame index, etc.

position

<bit> bit in a binary string or value, representing the factor 2position

EXAMPLE	The LSB has bit position 0.

slice

geometry and attributes for part of, or an entire, coded point cloud frame (3.1.5)

Note 1 to entry: the bounding boxes of any two slices can intersect.

tile

set of slices (3.1.22) identified by a common slice_tag syntax element value (3.2.15) whose geometry (3.1.19) should be contained within a bounding box (3.1.8) specified in a tile inventory data unit

prediction

an embodiment of the prediction process (3.1.25)

prediction process

the use of a predictor (3.1.26) to provide an estimate of the data element currently being decoded

predictor

a combination of specified values or previously decoded data elements used in the decoding process of subsequent data elements

intra prediction

a prediction (3.1.24) derived from only data elements (e.g., point positions (3.4.1) or attributes (3.1.20)) of the same decoded slice (3.1.22)

inter prediction

a prediction (3.1.24) derived in a manner that is dependent on data elements (e.g., point positions (3.4.1) or attributes(3.1.20)) of one or more reference point cloud frames (3.1.6)

uni-prediction

an inter prediction (3.1.28) derived in a manner that is dependent on data elements (e.g., point positions (3.4.1) or attributes(3.1.20)) of one reference point cloud frame (3.1.6)

bi-prediction

an inter prediction (3.1.28) derived in a manner that is dependent on data elements (e.g., point positions (3.4.1) or attributes(3.1.20)) of two reference point cloud frames (3.1.6)

I-slice
intra slice

a slice (3.1.22) that is decoded using intra prediction (3.1.27) without referring to a reference point cloud frame (3.1.6)

reference slice

a coded slice (3.1.22) which contains points (3.1.1) that may be used for inter prediction (3.1.28) in the decoding process of subsequent slices (3.1.22) in decoding order

I-frame
intra frame

a point cloud frame (3.1.4) that is decoded using intra prediction (3.1.27) without referring to a reference point cloud frame (3.1.6)

P-frame
predictive frame

a point cloud frame (3.1.4) that is decoded using intra prediction (3.1.27) or inter prediction (3.1.28) from at most one reference point cloud frame (3.1.6)

B-frame
bi-predictive frame

a point cloud frame (3.1.4) that is decoded using intra prediction (3.1.27) or inter prediction (3.1.28) from at most two reference point cloud frames (3.1.6)

Morton code

non-negative integer obtained by interleaving the bits of three integers

Morton order

elements ordered according to their Morton code (3.1.36)

sparse array

array with fewer set elements than total addressable elements; unset elements can have an inferred value when accessed

temporal ID

An identifier associated with a point cloud frame which may be used to support temporal scalability.

  - High-level syntax and entropy coding terms

ASN.1
abstract syntax notation one

notation specified by Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1 that is used for the definition of data types, values and constraints on data types

[SOURCE: Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1]

bin

binary symbol (bit) of the binarized (3.2.3) representation of a syntax element value (3.2.15)

binarization

specification of a syntax element's value (3.2.15) as a sequence of bins (3.2.2)

bypass

<symbol> a static, equiprobable probability model

bypass

<stream> bypass symbols (3.2.4) that are not encoded in an arithmetic-coded bitstream (3.2.6)

bitstream

<data> sequence of bits

bitstream

<coded sequence> sequence of bits, in the form of encapsulated data units (3.2.12), that represents a coded point cloud sequence (3.1.3)

set bit

bit with the value 1

unset bit

bit with the value 0

byte

sequence of 8 bits, typeset with the most significant bit on the left and the least significant bit on the right.

Note 1 to entry: When represented in a bitstream, the most significant bit of a byte is first.

byte aligned

bitstream (3.2.6) position that is an integer multiple of eight bits from the position of the first bit in the bitstream

data unit
DU

sequence of bytes (3.2.10) conveying a single syntax structure (3.2.16) of known length

data unit header

parameters, located from the start of a data unit (3.2.12)

data unit footer

parameters, located from the end of a data unit (3.2.12)

syntax element

element of data represented in the bitstream (3.2.6)

syntax structure

zero or more syntax elements (3.2.15) present together in the bitstream (3.2.6) in a specified order

parameter set

collection of parameters that apply when activated

sequence parameter set
SPS

parameters for an entire coded point cloud sequence (3.1.3), conveyed by an SPS data unit (3.2.12) and activated when referenced by a geometry data unit

geometry parameter set
GPS

parameters for the coding of slice (3.1.22) geometry, conveyed by a GPS data unit (3.2.12) and activated when referenced by a geometry data unit

attribute parameter set
APS

parameters for the coding of a slice (3.1.22) attribute, conveyed by an APS data unit (3.2.12) and activated when referenced by an attribute data unit

object identifier
OID

<ASN.1> ordered list of primary integer values from the root of the international object identifier tree (3.2.22) to a node, which unambiguously identifies that node

[SOURCE: Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1]

international object identifier tree

tree whose root corresponds to Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1 and whose nodes correspond to registration authorities (3.2.24) responsible for allocating arcs from a parent node

[SOURCE: Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1]

registration

<object identifier> assignment of an unambiguous name to an object in a way which makes the assignment available to interested parties

[SOURCE: Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1]

registration authority

<international object identifier tree> an entity such as an organization, a standard or an automated facility that performs registration (3.2.23) of one or more types of objects

[SOURCE: Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1]

application specific

defined by an application or an application standard

unspecified

when used in subclauses specifying values of a particular syntax element, indicates that the values have no specified meaning in this document and will not have a specified meaning in future versions of this document

  - Tree structure terms

tree

recursive structure of nodes (3.3.7) without loops, and containing a single root node (3.3.5)

top

<tree> tree level (3.3.4) with depth of 0 (3.3.8), consisting of the root node (3.3.5)

bottom

<tree> tree level (3.3.4) with the greatest depth (3.3.8)

tree level

set of nodes (3.3.7) at the same depth (3.3.8) in a tree (3.3.1)

root node

<tree> node (3.3.7) without a parent node (3.3.10)

leaf node

terminal node (3.3.7) without any child nodes (3.3.9)

node

<tree> element of a tree (3.3.1)

depth

<node> number of descendent hops from the root node (3.3.5) to a node (3.3.7)

child node

direct descendent of a node (3.3.7)

parent node

direct ancestor of a node (3.3.7)

grandparent node

direct ancestor of a node's (3.3.7) parent node (3.3.10)

great-grandparent node

direct ancestor of a node's (3.3.7) grandparent node (3.3.11)

sibling nodes

nodes (3.3.7) that are child nodes (3.3.9) of the same parent node (3.3.10)

subtree

part of a tree (3.3.1) comprising a subtree root node (3.3.15) and all its descendents over all subsequent tree levels (3.3.4)

root node

<subtree> single node (3.3.7) of a subtree (3.3.14) from which all other nodes in the same subtree are descendents

TriSoup node

occupied leaf node (3.3.6) of an occupancy tree (3.4.2) representing a sub-volume of the 3D space (or volume) containing at least one point of the point cloud (3.1.2)

  - Geometry coding terms

position

<point> three-dimensional coordinates of a point (3.1.1)

occupancy tree

eight-ary tree (3.3.1) of occupancy tree nodes (3.4.4) representing the geometry (3.1.19) of a slice (3.1.22)

predictive tree

tree (3.3.1) of predictive tree nodes (3.4.5) representing the geometry (3.1.19) of a slice (3.1.22)

node

<occupancy tree> node (3.3.7) of an occupancy tree (3.4.2) representing a sub-volume of the 3D space (or volume) containing the point cloud (3.1.2)

node

<predictive tree> node (3.3.7) of a predictive tree (3.4.3) representing a single position (3.4.1) for one or more points (3.1.1)

direct node

<occupancy tree> terminal node (3.4.4) that codes one or more point positions (3.4.1)

occupancy bitmap

8-bit bitmap for an occupancy tree node (3.4.4) whose bits indicate the existence of child nodes (3.3.9) at particular locations in the next tree level (3.3.4)

occupied neighbourhood pattern

indicates the existence and arrangement of the six possible occupancy tree nodes (3.4.4) that share faces with a central node

TriSoup

representation of the geometry of the point cloud (3.1.2) in a TriSoup node (3.3.16) by a set of triangles

TriSoup edge

any edge belonging to a cuboid volume associated with a TriSoup node (3.3.16)

TriSoup edge vertex

a TriSoup vertex (3.4.13) located on a TriSoup edge (3.4.10)

TriSoup face vertex

a TriSoup vertex (3.4.13) located on a face shared by two adjacent TriSoup nodes (3.3.16)

TriSoup vertex

a vertex of TriSoup triangle (3.4.14) including TriSoup edge vertex (3.4.11) and TriSoup face vertex (3.4.12)

TriSoup triangle

a triangle belonging to a TriSoup node (3.3.16) and whose vertex are defined from the TriSoup vertices (3.4.13) belonging to the cuboid volume associated with said TriSoup node

voxelization (of a TriSoup triangle)

process of transforming a TriSoup triangle (3.4.14) into a set of points (3.1.1)

  - Attribute coding terms

primary attribute component

first, or only, attribute component, identified by the index 0

secondary attribute component

attribute component other than the first component, identified by an index greater than 0

detail level

set of points (3.1.1) that represent a subsampled version of the slice geometry (3.1.19)

refinement list

set of points (3.1.1) present in one detail level (3.5.3) that are not present in the next coarsest detail level

refinement point

point (3.1.1) in a refinement list (3.5.4)

predictor set

set of neighbouring points (3.1.1) from which an attribute (3.1.20) value is predicted

  - Fine granularity slices terms

fine granularity slices

a subset of a slice which carries a geometry or an attribute of a subgroup (3.6.3) in a layer-group (3.6.2)

layer-group

a group of consecutive tree levels (3.3.4) of an occupancy tree (3.4.2)

subgroup

a spatial subset of a layer-group (3.6.2) where the bounding box of a subgroup shall not overlap with other subgroups in the same layer-group

root layer-group

a layer-group (3.6.2) which contains a root node (3.3.5) of the occupancy tree (3.4.2)

parent subgroup

a layer-group (3.6.2) which contains a root node (3.3.5) of the occupancy tree (3.4.2)

child subgroup

a subgroup (3.6.3) in a layer-group (3.6.2) adjacent to the maximum depth of the current subgroup, where the bounding box of the child subgroup is a subset of the bounding box of the current subgroup

- Abbreviated terms

  - Acronyms

APS	Attribute parameter set

ADU	Attribute data unit

CBS	Chunked bytestream

CPM	Contextual probability model

DADU	Dependent attribute data unit

DGDU	Dependent geometry data unit

DU	Data unit

FBDU	Frame boundary marker data unit

FGS	Fine granularity slice

FSAP	Frame-specific attribute properties

GDU	Geometry data unit

GOF	Group of frames

GPS	Geometry parameter set

G-PCC	Geometry-based point cloud compression

LoD	Level(s) of detail

LSB	Least significant bit

MSB	Most significant bit

NA	Not applicable

QP	Quantization parameter

RAHT	Region adaptive hierarchical transform

SPS	Sequence parameter set

  - Mnemonics

EGk	Exponential Golomb code of order 𝑘

FL	Fixed-length code

FL+S	Fixed-length code plus conditional sign bit

TU	Truncated unary code

attr	Attribute

cnt	Count

geom	Geometry

idx	Index

occtree	Occupancy tree

occ	Occupancy tree node

ptree	Predictive tree

ptn	Predictive tree node

ti	Tile inventory

tlv	Type-length-value

seq	Sequence

- Conventions

  - General

- The mathematical operators used in this document are similar to those used in the C programming language.  However, the results of integer division and arithmetic shift operations are defined more precisely, and additional operations are defined, such as exponentiation and real-valued division.  Numbering and counting conventions generally begin from 0.

  - Symbolic names

Variables and expressions use the following case-insensitive naming conventions to indicate their use:

- 𝑖, 𝑗: general loop or index variable

- 𝑘: component of an XYZ/STV/RPI position, coordinate or location

- 𝑐: component of an attribute

- qc: quantization-parameterized component: 0 – primary; 1 – secondary

- dpth: the depth of a node or level in a tree

- lvl: tree level or detail level, counted from the bottom of a tree or hierarchy

- 𝑚: Morton-coded location

- ns, nt, nv: node coordinates

- nsc, ntc, nvc: child node coordinates

- nsp, ntp, nvp: parent node coordinates

- ptIdx: index of a point in canonical decoding order

- rfmtIdx: index of a point in an array of LoD refinement points

- ni: index for an element in a point's predictor set

  - Numerical representation

| binary | typeset as 'X…XX' where each base 2-digit X is 0 or 1 |
| --- | --- |
| octal | typeset as X…XX8 where each base 8-digit X is 0 to 7 |
| decimal | typeset as X…XX where each base 10-digit X is 0 to 9 |
| hexadecimal | typeset as 0xX…XX where each base 16-digit X is 0 to 9 or A to F |

  - Arithmetic operators

| + | Addition |
| --- | --- |
| − | Subtraction (as a two-argument operator) or negation (as a unary prefix operator) |
| × | Multiplication |
| xy | Exponentiation. Specifies 𝑥 to the power of 𝑦. In other contexts, such notation is used for superscripting not intended for interpretation as exponentiation. |
| / | Integer division with truncation of the result toward zero. For example, 7 / 4 and −7 / −4 are truncated to 1 and −7 / 4 and 7 / −4 are truncated to −1. |
| ÷ | Division where no truncation or rounding is intended |
| xy | Division in mathematical equations where no truncation or rounding is intended |
| i=xyf(i) | Summation of 𝑓( 𝑖 ) with 𝑖 taking all integer values from 𝑥 up to and including 𝑦 |
| 𝑥 % 𝑦 | Modulus, remainder of 𝑥 divided by 𝑦, defined only for integers 𝑥 and 𝑦 with 𝑥 ≥ 0 and 𝑦 > 0 |

  - Logical operators

| 𝑥 && 𝑦 | Conditional boolean logical "and" of 𝑥 and 𝑦; the operand 𝑦 is only evaluated if 𝑥 is true. |
| --- | --- |
| 𝑥 ¦¦ 𝑦 | Conditional boolean logical "or" of 𝑥 and 𝑦; the operand 𝑦 is only evaluated if 𝑥 is false. |
| ¬ | Boolean logical "not" |
| 𝑥 ? 𝑦 : 𝑧 | If 𝑥 is true or not equal to 0, evaluates to 𝑦; otherwise, evaluates to 𝑧 |

  - Relational operators

| > | Greater than |
| --- | --- |
| ≥ | Greater than or equal to |
| < | Less than |
| ≤ | Less than or equal to |
| == | Equal to |
| ≠ | Not equal to |

  - Bit-wise operators

| & | Bit-wise "and". When operating on integer arguments, operates upon a two's complement representation of the integer value. When operating upon a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding MSBs equal to 0. |
| --- | --- |
| ¦ | Bit-wise "or". When operating on integer arguments, operates upon a two's complement representation of the integer value. When operating upon a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding MSBs equal to 0. |
| ^ | Bit-wise "exclusive or". When operating on integer arguments, operates upon a two's complement representation of the integer value. When operating upon a binary argument that contains fewer bits than another argument, the shorter argument is extended by adding MSBs equal to 0. |
| 𝑥 >> 𝑦 | Arithmetic right shift as specified by DivExp2Floor( 𝑥, 𝑦 ). It is equivalent to shifting a two's complement integer representation of 𝑥 by 𝑦 binary digits. This operator is defined only for non-negative integer values of 𝑦. |
| 𝑥 << 𝑦 | Arithmetic left shift of a two's complement integer representation of 𝑥 by 𝑦 binary digits. This operator is defined only for non-negative integer values of 𝑦. Bits shifted into the LSBs as a result of the left shift have a value equal to 0. |

- According to the rules of precedence (5.11), the expressions 𝑎 + 𝑏 << 𝑐 + 𝑑 and 𝑎 | 𝑏 << 𝑐 | 𝑑 are identical to ( 𝑎 + 𝑏 ) << ( 𝑐 + 𝑑 ) and 𝑎 | ( 𝑏 << 𝑐 ) | 𝑑, respectively, and not 𝑎 + ( 𝑏 << 𝑐 ) + 𝑑 or ( 𝑎 | 𝑏 ) << ( 𝑐 | 𝑑 ).

  - Assignment operators

| = | Assignment operator |
| --- | --- |
| := | Expression definition (5.12) |
| ++ | Increment, i.e. 𝑥++ is equivalent to 𝑥 = 𝑥 + 1; when used in an array index, evaluates to the value of the variable prior to the increment operation |
| −− | Decrement, i.e. 𝑥−− is equivalent to 𝑥 = 𝑥 − 1; when used in an array index, evaluates to the value of the variable prior to the decrement operation |
| ×= | Multiply by amount specified and update, i.e. 𝑥 ×= 3 is equivalent to 𝑥 = 𝑥 × 3 |
| += | Increment by amount specified, i.e. 𝑥 += 3 is equivalent to 𝑥 = 𝑥 + 3, and 𝑥 += (−3) is equivalent to 𝑥 = 𝑥 + (−3) |
| −= | Decrement by amount specified, i.e. 𝑥 −= 3 is equivalent to 𝑥 = 𝑥 − 3, and 𝑥 −= (−3) is equivalent to 𝑥 = 𝑥 − (−3) |
| >>= | Arithmetic right shift by amount specified, i.e. 𝑥 >>= 1 is equivalent to 𝑥 = 𝑥 >> 1 |
| <<= | Arithmetic left shift by amount specified, i.e. 𝑥 <<= 1 is equivalent to 𝑥 = 𝑥 << 1 |

  - Range notation

| 𝑥 = 𝑎 .. 𝑏 | 𝑥 takes on monotonically increasing integer values starting from 𝑎 and proceeding to 𝑏, inclusive, with 𝑥, 𝑎 and 𝑏 being integer numbers |
| --- | --- |

  - Mathematical functions

    - General

| ArcSin( 𝑥 ) | Trigonometric arc sine function |
| --- | --- |
| Abs( 𝑥 ) | x;x≥0-x;x<0 |
| Bit( 𝑥, 𝑖 ) | ( 𝑥 >> 𝑖 ) & 1 |
| Ceil( 𝑥 ) | Lowest integer greater than or equal to 𝑥 |
| Clip3( min, max, 𝑥 ) | min;x<minmax;x>maxx;otherwise |
| CrossProduct( v1 , v2 ) | v1[1]×v2[2]-v1[2]×v2[1]v1[2]×v2[0]-v1[0]×v2[2]v1[0]×v2[1]-v1[1]×v2[0] |
| DivExp2Floor( 𝑥, 𝑝 ) | Floorx÷2p |
| DivExp2Fz( 𝑥, 𝑝 ) | RoundFzx÷2p |
| DivExp2Tz( 𝑥, 𝑝 ) | 𝑥 / Exp2( 𝑝 ) |
| DivExp2Up( 𝑥, 𝑝 ) | RoundUpx÷2p |
| DivExp2Inf( 𝑥, 𝑝 ) | RoundUpx÷2p;x≥0-RoundUp-x÷2p;x<0 |
| Exp2( 𝑝 ) | 2p |
| Floor( 𝑥 ) | Greatest integer less than or equal to 𝑥 |
| Gcd( 𝑎, 𝑏 ) | Greatest integer that is a factor of both 𝑎 and 𝑏 |
| InnerProduct( v1 , v2 ) | v1[0]×v2[0]+v1[1]×v2[1]+v1[2]×v2[2] |
| IntLog2( 𝑥 ) | Floor( Log( 𝑥 ) ÷ Log( 2 ) ) |
| Log( 𝑥 ) | Natural logarithm of the argument 𝑥 |
| Min( 𝑥, 𝑦 ) | x;x≤yy;x>y |
| MinVec( v ) | Min( v[0], Min( v[1], v[2] ) ) where v is a three-dimensional vector |
| Max( 𝑥, 𝑦 ) | x;x≥yy;x<y |
| MaxVec( v ) | Max( v[0], Max( v[1], v[2] ) ) where v is a three-dimensional vector |
| PopCnt( 𝑥 ) | Number of set bits present in the binary representation of 𝑥 |
| RoundFz( 𝑥 ) | ( 2 × 𝑥 + Sign( 𝑥 ) ) / 2 |
| RoundUp( 𝑥 ) | ( 2 × 𝑥 + 1 ) / 2 |
| Sign( 𝑥 ) | 1;x>00;x==0-1;x<0 |
| Sin( 𝑥 ) | Trigonometric sine function |
| Sqrt( 𝑥 ) | x |

    - IntAtan2

The function theta = IntAtan2( 𝑦, 𝑥 ) is a 20-bit fixed-point approximation of the arc tangent of y÷x that accounts for the Cartesian quadrant of the parameters.  Its:

- parameters 𝑥 and 𝑦 are integer, Cartesian coordinates;

- result shall be equal to the value of the expression intAtan2[ 𝑦 ][ 𝑥 ].

The expression sineThetaI is for a right-angled triangle with catheti adj and opp in the first octant.

sineThetaI := opp × IntRecipSqrt(adj × adj + opp × opp) >> 20
  where
    opp := Min(Abs(y), Abs(x))
    adj := Max(Abs(y), Abs(x))

The angle thetaI is derived by interpolating between values of ArcSinFp for a 9-bit approximation of sine theta.

thetaI := ArcSinFp[idx0] + (alpha × (ArcSinFp[idx1] − ArcSinFp[idx0]) >> 11)
  where
    idx0 := sineThetaI >> 11
    idx1 := Min(362, idx0 + 1)
    alpha = sineThetaI % Exp2(11)

The expression ArcSinFp[ 𝑥 ] specifies the 20-bit fixed-point approximation for arc sine.

ArcSinFpx≔220 ArcSinx29+12

The result is obtained by mapping thetaI to the correct octant according to the signs of the parameters.

intAtan2[y][x] := Sign(y) × thetaIh
  where thetaIh :=
    x > 0 && Abs(x) ≥ Abs(y) ? thetaI :
    x > 0 && Abs(x) < Abs(y) ? 1647099 − thetaI :
    x < 0 && Abs(x) < Abs(y) ? 1647100 + thetaI :
    x < 0 && Abs(x) ≥ Abs(y) ? 3294177 − thetaI : 0

    - IntCos and IntSin

The functions 𝑥 = IntCos( 𝜃, piBits ) and 𝑦 = IntSin( 𝜃, piBits ) are 24-bit fixed-point approximations of the cosine and sine of 𝜃.  Their:

- parameters 𝜃 and piBits specify an angle measured in units of 2-piBits half turns;

- result shall be equal to the value of the expression cathetus.

The fixed-point cathetus for the unit circle is calculated by interpolating between values of SinFp.  The values of sgn and theta are determined from 𝜃 for the corresponding function as specified by Table 1.  The variable pi, equal to Exp2( piBits ), represents one half turn.

cathetus := sgn × DivExp2Up(iFrac0 × SinFp[idx0] + iFrac1 × SinFp[idx1], fracBits)
  where
    fracBits := piBits − 11
    iFrac1 := theta − ((theta >> fracBits) << fracBits)
    iFrac0 := Exp2(fracBits) − iFrac1
    idx0 := Min(1024, theta >> fracBits)
    idx1 := Min(1024, idx0 + 1)

Table 1 — Values of sgn and theta for functions IntCos and IntSin

| Domain | IntCos |  | IntSin |  |
| --- | --- | --- | --- | --- |
|  | sgn | theta | sgn | theta |
| 𝜃 ≤ −pi | −1 | pi/2 | 0 | 0 |
| −pi < 𝜃 < −pi/2 | −1 | −𝜃 − pi/2 | −1 | pi + 𝜃 |
| −pi/2 ≤ 𝜃 < 0 | 1 | 𝜃 + pi/2 | −1 | −𝜃 |
| 0 ≤ 𝜃 < pi/2 | 1 | −𝜃 + pi/2 | 1 | 𝜃 |
| pi/2 ≤ 𝜃 < pi | −1 | 𝜃 − pi/2 | 1 | pi − 𝜃 |
| 𝜃 ≥ pi | −1 | pi/2 | 0 | 0 |

The expression SinFp[ 𝑥 ] specifies the 24-bit fixed-point approximation of sine.

SinFpx≔224 Sinπ x211+12

    - IntSqrt

The function 𝑟 = IntSqrt( 𝑥 ) is an integer approximation of the principal square root of 𝑥.  Its:

- parameter 𝑥 is a non-negative integer;

- result shall be equal to the value of the expression intSqrt[ 𝑥 ].

It is specified in terms of the fixed-point reciprocal square root.  If the parameter 𝑥 is greater than or equal to 246, the calculation uses a quantized value of 𝑥 to ensure computability using 64-bit arithmetic.

- IntSqrt( 0 ) is 1.

intSqrt[x] := x ≤ Exp2(46)
    ? 1 + (x  × IntRecipSqrt(x)  >> 40)
    : 1 + (x8 × IntRecipSqrt(x8) >> 32)
  where
    x8 := DivExp2(x, 16) + 1

    - IntRecipSqrt

The function rRecip = IntRecipSqrt( 𝑥 ) is a 40-bit fixed-point approximation of the reciprocal square root of 𝑥.  Its:

- parameter 𝑥 is a non-negative integer;

- result shall be equal to the value of the expression intRecipSqrt[ 𝑥 ].

- IntRecipSqrt( 0 ) is 0.

The parameter 𝑥 is scaled to be in the range 230,232-1 in the expression xScaled by multiplying or dividing by a power of four.  The expression xScaleLog4 is the log4 scale factor.

xScaled := Floor(x × Exp2(2 × xScaleLog4))
xScaleLog4 := 15 − IntLog2(x) / 2

The reciprocal square root shall be determined by two rounds of the Newton–Raphson method.  The initial approximation for the scaled parameter 𝑥 is specified by the expression approxR0. Table 2 specifies the initial approximants over the domain of approxR with 18 fractional bit precision.

approxR0 := approxR[xScaled >> 25]

The second approximation from the first round of the Newton–Raphson method is specified by the expression approxR1.

approxR1 := threeR0[approxR0] − (rCubed0[approxR0] × xScaled >> 32)
threeR0[r] := 3 × DivExp2Fz(r, 18) << 22
rCubed0[r] := DivExp2Fz(r × r × r, 54) << 8

The third approximation from the second round of the Newton–Raphson method is specified by the expression approxR2.

approxR2 := threeR1[approxR1] − rCubed1[approxR1] >> 32
threeR1[r] := r × 3 << 28
rCubed1[r] := r × (r × (r × pInScaled >> 32) >> 32)

The result is obtained by scaling the third approximation by the square root of the initial scale factor.

intRecipSqrt[x] := x > 0 ? Floor(approxR2 × Exp2(xScaleLog4 − 3)) : 0

Table 2 — Initial approximations approxR[ 𝑖 + 𝑗 ] for IntRecipSqrt( 𝑖 + 𝑗 << 25 ).  Values are typeset in hexadecimal form without the 0x prefix.

| 𝑗 | 𝑖 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 32 | 3F7FFDA | 3E7FFB7 | 3DBFFBD | 3CC0013 | 3BFFFEE | 3AFFFE1 | 3A3FFDE | 397FF96 |
| 40 | 38FFFDE | 383FFC0 | 3780063 | 36FFFCD | 3640014 | 35BFFFA | 34FFF8B | 3480010 |
| 48 | 3400039 | 3380042 | 3300008 | 328002B | 3200046 | 317FFC2 | 3100012 | 307FFDB |
| 56 | 303FFC5 | 2FC004F | 2F3FFE0 | 2EFFF93 | 2E7FF91 | 2E3FF83 | 2DC0037 | 2D7FFB6 |
| 64 | 2D0000B | 2CBFF96 | 2C7FF66 | 2C00017 | 2BC0053 | 2B7FFA4 | 2AFFF43 | 2AC004B |
| 72 | 2A80061 | 2A3FF6A | 2A00032 | 29BFF3B | 297FF74 | 293FFC9 | 28C001E | 287FF6D |
| 80 | 283FF9E | 27FFF93 | 27BFFB1 | 27BFFE8 | 277FF3C | 2700056 | 2700000 | 26C0069 |
| 88 | 26800CE | 26400CD | 25FFF5A | 25BFFA8 | 25BFFD6 | 258008C | 25400B9 | 2500020 |
| 96 | 24BFF93 | 24C00B6 | 24800E6 | 24400B3 | 2400011 | 2400054 | 23C0049 | 237FF98 |
| 104 | 2380104 | 233FFFC | 2300047 | 2300024 | 22C0029 | 2280000 | 2280012 | 223FF79 |
| 112 | 223FF54 | 21FFF56 | 21BFFDE | 21C0078 | 2180111 | 217FF3D | 2140023 | 20FFF0F |
| 120 | 21000F1 | 20C0019 | 20C0137 | 2080015 | 2080091 | 204004F | 20400F7 | 200006B |

    - IntRecip

The function ( recip, fracBits ) = IntRecip(divisor ) returns a fixed-point approximation of 1 ÷ divisor and the number of fractional bits in the fixed-point approximation.  Its:

- parameter divisor is a non-null integer;

- result is the tuple specified by the value of the expressions recip and fracBits .

The number of fractional bits in the fixed-point approximation is provided by fracBits expression.

fracBits := (31 << 1) – fracBitsOffset

Where the fracBitsOffset expression is determined by

fracBitsOffset := 30 – IntLog2(divisor)

- fracBitsOffset is determined to maximise the number of bits during the computation of recip while letting the possibly of computing it into 64 bits signed integer registers without any risk of overflow.

The parameter divisor is scaled to be in the range 230,231-1 in the expression scaledDivisor by multiplying or dividing by a power of two.

scaledDivisor := Floor(divisor × Exp2(fracBitsOffset))

The approximation of 1 ÷ divisor shall be determined by one iteration of the Newton–Raphson division approximation method.  The initial approximation for the method is specified by the expression recip0 and corresponds to a first order approximation of 1 ÷ divisor.

recip0 := (Round(48 ÷ 17 × Exp2(28)) << 31) - Round(32 ÷ 17 × Exp2(28)) × scaledDivisor

The recip expression corresponds to the final approximation and is specified from the first round of the Newton–Raphson method.

recip := recip0 × ((1 << 31) - (scaleDivisor × recip0 >> 31)) >> 31

    - Div

The function quotient = Div( dividend, divisor, fracBits ) is a fixed-point approximation of dividend ÷ divisor.  Its:

- parameters dividend and divisor are integers;

- parameter fracBits is the number of fractional bits in the fixed-point result;

- result is specified by the value of the expression quotient.

quotient := dividend × recipDivisor[idx] >> 16 + excess − fracBits
  where
    idx := DivExp2Fz(divisor, excess)
    excess := Max(0, IntLog2(divisor) − 7)
    recipDivisor[idx] := RoundFz(Exp2(16) ÷ idx) − 1

    - Morton

The function 𝑚 = Morton( 𝑠, 𝑡, 𝑣 ) converts its parameters to a 3D Morton code.  Its:

- parameters 𝑠, 𝑡 and 𝑣 are non-negative integers;

- result is specified by the value of the expression morton.

The conversion interleaves the bits of each parameter 𝑣, 𝑡 and 𝑠; in that order, starting from the LSBs.  The LSB of 𝑣 is the LSB of 𝑚. Table 3 illustrates the construction of 3D Morton codes from the bit string representation of the parameters 𝑠, 𝑡 and 𝑣.

morton≔i=023i+2Bits,i+23i+1Bitt,i+23iBitv,i

The expression Morton[ expr ] performs the same conversion for an expression expr that takes an argument 𝑘, 𝑘 ∈ { 0, 1, 2 }.

Morton[expr] := Morton(expr[0], expr[1], expr[2])

Table 3 — Construction of 3D Morton codes 𝑚 from the tuple ( 𝑠, 𝑡, 𝑣 )

| Bit string form |  |  |  | Decimal form |
| --- | --- | --- | --- | --- |
| 𝑠 | 𝑡 | 𝑣 | 𝑚 | 𝑚 |
| '0 0' | '0 0' | '0 0' | '0 0 0 0 0 0' | 0 |
| '0 0' | '0 0' | '0 1' | '0 0 0 0 0 1' | 1 |
| '0 1' | '1 1' | '1 0' | '0 1 1 1 1 0' | 30 |
| '0 1' | '1 1' | '1 1' | '0 1 1 1 1 1' | 31 |
| '1 0' | '0 1' | '1 0' | '1 0 1 0 1 0' | 42 |
| '1 0' | '0 1' | '1 1' | '1 0 1 0 1 1' | 43 |
| '1 1' | '1 0' | '0 0' | '1 1 0 1 0 0' | 52 |
| '1 1' | '1 0' | '0 1' | '1 1 0 1 0 1' | 53 |
| sn … s1 s0 | tn … t1 t0 | vn … v1 v0 | sn tn vn … s1 t1 v1 s0 t0 v0 | … |

    - FromMorton

The function ( 𝑠, 𝑡, 𝑣 ) = FromMorton( 𝑚 ) is the inverse of Morton( 𝑠, 𝑡, 𝑣 ).  Its:

- parameter 𝑚 is a non-negative, integer, 3D Morton code;

- result is the tuple specified by the value of the expressions 𝑠, 𝑡 and 𝑣.

The conversion deinterleaves the bits of 𝑣, 𝑡 and 𝑠; in that order, starting from the LSB.  The LSB of 𝑚 is the LSB of 𝑣.

s≔i=02iBitm,3i+2
t≔i=02iBitm,3i+1
v≔i=02iBitm,3i

  - Order of operation precedence

When order of precedence in an expression is not indicated explicitly by round brackets, the following rules apply:

- Operations of a higher precedence are evaluated before any operation of a lower precedence.

- Operations of the same precedence are evaluated sequentially from left to right.

Table 4 specifies the precedence of operations from highest to lowest; a higher position in the table indicates a higher precedence.

- For those operators that are also used in the C programming language, the order of precedence used in this document is the same as used in the C programming language.

Table 4 — Operation precedence from highest (at top of table) to lowest (at bottom of table)

Operations (with operands 𝑥, 𝑦 and 𝑧)

𝑥++, 𝑥−−

¬𝑥, −𝑥 (as a unary prefix operator)

xy

𝑥 × 𝑦, 𝑥 / 𝑦, 𝑥 ÷ 𝑦, xy, 𝑥 % 𝑦

𝑥 + 𝑦, 𝑥 − 𝑦 (as a two-argument operator), i=xyf(i)

𝑥 << 𝑦, 𝑥 >> 𝑦

𝑥 < 𝑦, 𝑥 ≤ 𝑦, 𝑥 > 𝑦, 𝑥 ≥ 𝑦

𝑥 == 𝑦, 𝑥 ≠ 𝑦

𝑥 & 𝑦

𝑥 ^ 𝑦

𝑥 ¦ 𝑦

𝑥 && 𝑦

𝑥 ¦¦ 𝑦

𝑥 ? 𝑦 : 𝑧

𝑥 .. 𝑦

𝑥 = 𝑦, 𝑥 += 𝑦, 𝑥 −= 𝑦, 𝑥 ×= 𝑦, 𝑥 <<= 𝑦, 𝑥 >>= 𝑦

  - Named expressions

    - General

Operations and values in this document are sometimes specified in the form of named expressions.  Exemplar named expressions are described in Table 5.

A named expression is a named macro-like statement.  Every occurrence of a named expression is substituted by its definition when evaluated.  The definition is provided by the ≔ operator.

Substitution is atomic.  For example, the substitution for 3 × ExAPlusB is equivalent to 3 × ( 𝑎 + 𝑏 ), not ( 3 × 𝑎 ) + 𝑏.

The definition for a named expression is immutable.  For example, ExTwo is equivalent to the value 2; unlike a variable it cannot be modified.  All instances of ExTwo could be substituted by the numeric value 2.

The substitution for a named expression can be a variable.  The substituted variable in such cases can be modified.  For example, ExVar++ increments the variable Var.

Table 5 — Examples of named expressions

| Example | Remarks |
| --- | --- |
| ExTwo := 2 | ExTwo is equivalent to the value 2 |
| ExAPlusB := a + b | ExAPlusB is equivalent to ( 𝑎 + 𝑏 ) |
| ExTwoIndirect := ExTwo | ExTwoIndirect is equivalent to ExTwo |
| Var = 2 ExVar := Var | ExVar is equivalent to (an alias of) the variable Var |
| ExTimesTwo[i] := 2 × i | ExTimesTwo[ 𝑗 + 1 ] is equivalent to 2 × ( 𝑗 + 1 ) |
| ExSquared[i] := i × i | ExSquared[ ExVar++ ] is equivalent to Var × Var, with Var incremented after the evaluation of ExSquared |
| for (Var = 0; Var ≤ 10; Var++) sum += ExTimesTwo[Var] | sum is incremented, in total, by 110 |
| ExWhere[i] := ExTimesTwo[inner] where inner := i + 1 | ExWhere[ 𝑗 ] is equivalent to 2 × ( 𝑗 + 1 ) |
| ExSumA[i] := i > 0 ? i + ExSumA[i − 1] : 0 | Recursive definition. ExSumA[ 10 ] evaluates to 55 |
| ExSumB[i] := ExSumB = 0 for (; i > 0; i−−) ExSumB += i | Imperative definition using multiple statements. ExSumB[ 10 ] evaluates to 55 |
| ExSum10[expr] := ExSum10 = 0 for (i = 0; i ≤ 10; i++) ExSum10 += expr[i] | ExSum10 applies expr to each 𝑖, 𝑖 ∈ 0 .. 10, summing the result. ExSum10[ ExprTimesTwo ] evaluates to 110 ExSum10[ ExprSquared ] evaluates to 385 |

    - Scope

The scope of a named expression is not affected by the relative order of its definition and use; a named expression can be referenced earlier in the document than its definition.

Named expressions identified by a capital initial are "global" definitions that apply to the whole document.  They may be directly referenced in other subclauses.

Named expressions identified by a lower-case initial are "local" definitions that apply to the subclause in which they are defined.

If a global definition references a local definition in the same subclause, that local definition is used when the global definition is referenced in another subclause.

    - Arguments of named expressions

The definition of an expression can be in terms of one or more parameters.  Each parameter is enclosed in square brackets.  For example, the definition ExTimesTwo[ 𝑖 ] has a single parameter 𝑖.

A named expression can be applied to one or more arguments.  When the definition is substituted for a named expression, every instance of each parameter is replaced by the text of the corresponding argument.

Replacements are atomic.  For example, ExTimesTwo[ 𝑗 + 1 ] is equivalent to 2 × ( 𝑗 + 1 ), not ( 2 × 𝑗 ) + 1.

    - Sub-expressions

A definition can contain a where-clause that defines further named expressions.  They apply only to the definition containing the where-clause.  For example, the definition of ExWhere[ 𝑖 ] defines the sub-expression inner.

    - Definitions with multiple statements

Some definitions cannot be succinctly expressed by a single statement.  In such cases, a definition can consist of multiple statements.  The evaluated value for the whole definition is specified by assignments or modifications to a variable with the same name as the named expression.  For example, ExSumB.

    - Textual definitions

Some definitions are provided by a descriptive equivalence in textual or tabular form.  For example:

- "The expression Ex[ 𝑖 ] is specified by Table X (Value for Ex[ 𝑖 ])."

- "The value for the expression Ex is specified by Table X for each axis 𝑘."

- "The expression Ex is equivalent to the following [procedural code]."

  - Variables, syntax elements and tables

Syntax elements in the bitstream are represented in bold type.  Each syntax element is described by its name (all lower-case letters with underscore characters) and one descriptor for its method of coded representation.  The decoding process behaves according to the value of the syntax element and to the values of previously decoded syntax elements.  When a value of a syntax element is used in the syntax tables or the text, it appears in regular (i.e. not bold) type.

In some cases the syntax tables use the values of variables derived from other syntax elements' values.  Such variables appear in the syntax tables, or text, named by a mixture of lower- and upper-case letters and without any underscore characters.  Variables with a capital initial are valid for the decoding of the current syntax structure and all dependent syntax structures.  They may be used in the decoding process for later syntax structures without mentioning their origin.  Variables with a lower-case initial are only used within the clause in which they are derived.

- The syntax is described in a manner that closely follows the C language syntactic constructs.

Functions that specify properties of the current position in the bitstream are referred to as syntax functions (7.2).  These functions assume the existence of a bitstream pointer with an indication of the position of the next bit to be read by the decoding process from the bitstream.  Syntax functions are described by their names, which are constructed as syntax element names and end with left and right round brackets including zero or more parameter names (for definition) or arguments (for usage), separated by commas (if more than one).

Functions that are not syntax functions (including mathematical functions specified in 5.10) are described by their names, which start with a capital initial, contain a mixture of lower- and upper-case letters without any underscore characters and end with left and right round brackets surrounding zero or more parameter names (for definition) or arguments (for usage), separated by commas (if more than one).

Arrays are sequences of values identified by a common name.  Both syntax elements and variables can be arrays.  Subscripts or square brackets are used to index an array.

Boolean true and false values are interchangeable with the integers 1 and 0, respectively; non-zero integers are equivalent to true.

- Point cloud format and relationship to coded and output representations

  - General format

A point cloud is an unordered list of points representing geometry, optional attribute information and associated metadata.  Geometry information describes the location of points in a three-dimensional Cartesian coordinate system.  Attributes are typed properties of each point, such as its colour or reflectance.  Metadata is information used to interpret the point cloud, the point geometry and the attribute data.

Each point in a point cloud is a tuple of a three-dimensional position and attribute values for every attribute present in the point cloud.  All points shall have the same number of attributes in the same order.

Point cloud metadata may describe, for example, a geometric transformation used to map points to another coordinate system, spatial regions (tiles) within a point cloud, the identification of attribute types and how attribute values are interpreted.

An N-point point cloud P is symbolically illustrated in Figure 1.  Rows of P are points.  Each point Pn comprises a position with components pn,k and values for the components an,cm of each attribute Am of A1 to AM.

Figure 1 — A symbolic representation of a point cloud.

Two point clouds are identical if there exists a one-to-one mapping between points in the two point clouds.  For example, a permutation of points (rows in P) preserves identity.

  - Attributes

    - General

An attribute comprises one or more components.

A point cloud, unless otherwise specified, may contain more than one instance of a particular attribute.  The significance or interpretation of multiple instances of the same attribute is unspecified.

Metadata can be associated with each attribute instance.  Attribute metadata conveys sequence level characteristics such as an attribute label identifier or a frame level interpretation such as an attribute scale and offset information.

    - Colour

The colour attribute specifies the colour of a point.  The attribute shall comprise one of the following configurations:

- A luma (Y') component (monochrome).

- A luma component and two chroma components (Y'CBCR or Y'CGCO).

- Green, blue and red components (G'B'R', also known as RGB).

- Other unspecified monochrome or tri-stimulus colour systems (e.g., YZX, also known as XYZ).

The actual colour representation method in use is described using ISO/IEC 23091-2 coding-independent video code points and is indicated using syntax specified in 7.3.2.7.

The ordering of attribute components is specified by Table 6.

Table 6 — Relationship between colour components and attribute components

| Colour representation | Attribute component index |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| Monochrome | Y' | – | – |
| Y'CBCR | Y' | CB | CR |
| Y'CGCO | Y' | CG | CO |
| G'B'R' or RGB | G' | B' | R' |
| YZX or XYZ | Y | Z | X |

    - Opacity

Opacity is a single component attribute.  When normalized to the interval [ 0, 1 ], the value 0 indicates that a point is completely transparent and the value 1 indicates complete opacity.  The opacity attribute may be used to control colour blending when rendering a colour attribute.

- Opacity is often called an alpha channel or transparency.

    - Reflectance

Reflectance is a single component attribute that represents the ratio of incident light reflected by a point; it is a dimensionless quantity.  Values are bounded by a minimum that indicates complete absorption and a maximum that indicates complete reflection or saturation.

    - Normal vector

A normal vector is a three-component attribute representing a vector perpendicular to the surface tangent plane at an associated point.  The axes identification of the normal vectors is identical to that of the STV axes for the coded point cloud geometry.  The length of a normal vector is not required to be one.

Normal vectors may be used when rendering a point cloud.  A point's appearance may be modified according to the difference between the incident light direction and its normal vector.

    - Material identifier

A material identifier is a single component attribute that associates a point with a material from a range of materials.  Points with a common material identifier share a characteristic that may be used to identify an object or type of object.  Materials are not specified by this document.

    - Frame number/index

The frame number and frame index attributes are single component attributes that indicate how a point cloud frame may be partitioned into one or more ordered sub-frames.  Each sub-frame is a partial representation of a point cloud frame, comprising points with the same frame number/index attribute value.

- Sub-frame partitioning does not form part of the decoding or output processes specified by this document.

A point cloud sequence shall contain no more than one instance of a frame number/index attribute.  A point cloud sequence shall not contain both frame number and frame index attributes.

The frame number attribute may be used to order all sub-frames over the entire point cloud sequence.  Points from different point cloud frames shall not have the same value for the frame number attribute.

The frame index attribute may be used to order the sub-frames of a single point cloud frame.

An example of the relationship between frames, sub-frames and their ordering is shown in Table 7.  The point cloud frames a, b and c are partitioned into sub-frames.  Sub-frame orders are shown for the cases where the attribute is a frame number or a frame index.

Table 7 — Example partitioning of three consecutive frames a, b and c into sub-frames

|  | Frame |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | a |  |  | b |  | c |  |  |
| Frame number attribute |  |  |  |  |  |  |  |  |
| Sub-frame attribute value | 0 | 2 | 1 | 3 | 5 | 4 | 6 | 7 |
| Sub-frame presentation order | a0 | a1 | a2 | b3 | c4 | b5 | c6 | c7 |
| Frame index attribute |  |  |  |  |  |  |  |  |
| Sub-frame attribute value | 0 | 2 | 1 | 0 | 1 | 0 | 1 | 3 |
| Sub-frame presentation order | a0 | a1 | a2 | b0 | b1 | c0 | c1 | c3 |

    - User defined attributes

The point cloud format supports attributes other than those specified in this document.  A user defined attribute shall be identified by an international object identifier.  The international object identifier shall either be assigned by a registration authority in accordance with Rec. ITU-T X.660﻿ |​ ISO/IEC 9834-1, or generated without registration using a universally unique identifier (UUID) as specified by Rec. ITU-T X.667﻿ |​ ISO/IEC 9834-8.

  - Codec-derived attributes

    - General

Codec-derived attributes represent values that are determined as side-effects of a processes specified in this document.

A decoder may, but is not required to, output one or more codec-derived attributes.  Any codec-derived attributes output by a decoder shall conform to the definitions in 6.3.

    - Slice identifier

The slice identifier attribute shall be a single component attribute that identifies the slice from which a point is decoded.  Identification shall use the slice_id syntax element value.

    - Slice tag

The slice tag attribute shall be a single component attribute that identifies the group of slices from which a point is decoded.  Identification shall use the slice_tag syntax element value.

    - Canonical point order

The canonical point order attribute shall be a single component attribute that specifies the order within a slice in which points are decoded by the geometry decoder as specified in this document.

Values of the point decoding order attribute shall be equal to ptIdx of the corresponding point PointPos[ ptIdx ] in a slice.

    - Point Morton order

The point Morton order attribute shall be a single component attribute that specifies the order of points within a slice according to ascending values of Morton-coded STV slice position (i.e. prior to 8.3.6).

The Morton order shall be equivalent to the order of points in the finest detail level specified in 10.6.5.2 as if both attr_canonical_order_enabled and attr_coord_conv_enabled are both 0.

For example, if three points 𝑎, 𝑏 and 𝑐 in canonical point order are ordered { 𝑎, 𝑐, 𝑏 } in the finest detail level, then the respective values for the Morton order attribute are 0, 2 and 1.

  - Coded point cloud format

    - Sequence coordinate system

The sequence coordinate system is specified by the position of its origin in an externally defined application-specific coordinate system and by the length of its unit vectors.

All points in a coded point cloud sequence shall have non-negative coordinates in the sequence coordinate system.

A position PSCS in the sequence coordinate system is related to the position PACS in the application coordinate system by the sequence origin SeqOrigin and the unit vector length SeqUnit specified by the active SPS:

PACS=PSCS+SeqOrigin×SeqUnit

The maximum bound on the sequence coordinate system depends upon the level to which the coded point cloud sequence conforms, as specified in Annex A.

An example sequence coordinate system (marked SCS) is illustrated in Figure 2.  A point PSCS with an 𝑥-coordinate of 75 in the sequence coordinate system has a position in the application-specific coordinate system of 105 SeqUnit.  If SeqUnit is 0,8 AppUnit, the 𝑥-coordinate of the point in the application-specific coordinate system (marked ACS) is 4,2.

Key

| ACS | Application-specific coordinate system |
| --- | --- |
| CCS | Coding coordinate system |
| SCS | Sequence coordinate system |
| 𝑎:𝑏 | Ratio of SCS to CCS |

Figure 2 — Relationship between application, sequence, and coding coordinate systems.

    - Coding coordinate system

The coding coordinate system is a non-negative integer coordinate system used to code point positions.  It is either identical to, or a geometric contraction of the sequence coordinate system.  Its origin is coincident with the sequence coordinate system origin.

A position PCCS in the coding coordinate system is related to the position PSCS in the sequence coordinate system by the binary fixed-point scale factor SeqCodedScale:

PSCS=PCCS×SeqCodedScale

An example coding coordinate system (marked CCS) is illustrated in Figure 2. A point PCCS with an 𝑥-coordinate of 48 in the coding coordinate system has an 𝑥-coordinate, PSCS, of 75 when scaled by the scale factor SeqCodedScale=a÷b=25÷16.

Point position components in the coding coordinate system shall satisfy the following level dependent (Annex A) constraint:

PSCS×SeqCodedScaleN<Min2MaxSeqBboxDimLog2,232-12 SeqCodedScaleD

Where SeqCodedScaleN and SeqCodedScaleD are the numerator and denominator, respectively, of SeqCodedScale when represented as an irreducible fraction:

SeqCodedScaleNSeqCodedScaleD=SeqCodedScale,  GcdSeqCodedScaleN, SeqCodedScaleD=1

- This constraint guarantees that conversion from the coding coordinate system to the sequence coordinate system can be performed using 32-bit arithmetic.

    - Coded point cloud sequence

The coded representation of a point cloud sequence comprises one or more point cloud frames encoded as a sequence of encapsulated DUs that convey syntax structures as specified in 7.3. An encapsulation format is specified by Annex B. Alternative encapsulation formats may be specified by the application.

The coded point cloud sequence shall include:

- An SPS that enumerates the attributes present in the coded point cloud format and conveys both metadata and decoding parameters that pertain to the whole coded point cloud sequence.

- Any GPSs that convey parameters required for the decoding of geometry data.

- Any APSs that convey parameters required for the decoding of attribute data.

- The slices comprising each coded point cloud frame.

    - Coded point cloud frame

A coded point cloud frame comprises a sequence of zero or more slices with the same value of a notional frame counter FrameCtr (8.2.2).  An optional frame boundary marker data unit explicitly signals the end of a frame.

It is a requirement of bitstream conformance that:

- Every coded point cloud frame shall have a unique value of FrameCtr within the sequence.

- Coded point cloud frames shall be ordered such that the notional frame counter increases for each successive coded point cloud frame when biprediction_enabled is equal to 0.

- The value of of gdu_temporal_id of each GDU in the frame and adu_temporal_id of each ADU in the frame is equal to the same value.

- The decoding of current frame shall not depend on any point cloud frame F in the bitstream where the temporal ID of frame F is greater than the temporal ID of the current frame.

An empty frame shall be signalled by a frame boundary marker data unit without any preceding slices with the same value of FrameCtr.

A coded point cloud frame independently codes a single point cloud frame without dependencies upon any previous or subsequent point cloud frame.

A decoded point cloud frame is the concatenation of all points in all constituent slices of the frame.

Unless prohibited by an SPS constraint, coincident points in a point cloud frame may arise from:

- points coded in a single slice with a non-zero duplicate point count;

- distinct points with the same position in a single slice; or

- the concatenation of multiple slices.

    - Slice of a coded point cloud frame

Every slice shall include a GDU that codes the slice geometry and ADUs or defaulted attribute DUs that code the slice attributes.  A slice is identified by the GDU slice_id.

The slice geometry is coded in the slice's coordinate system.  The bounding boxes of slices may intersect, including within a single frame.

A slice shall start with a GDU.  This GDU may be followed by optional redundant GDUs that duplicate the slice geometry.  ADUs and defaulted attribute DUs shall occur after all GDUs in the slice.  DUs belonging to different slices shall not be interleaved.

Within a slice, other DUs may be present.  For example, an APS can occur within a slice to convey parameters for attribute decoding.

It is a requirement of bitstream conformance that:

- All GDUs present in a slice shall reconstruct the same geometry in the same canonical point order.

- Every slice shall have a corresponding ADU or defaulted attribute DU for every attribute enumerated in the SPS.

- All ADUs present in a slice with the same value of adu_sps_attr_idx shall reconstruct the same attribute values.

Only one GDU in a slice shall be decoded; all others shall be ignored when decoding (removed from the bitstream and discarded).  A decoder shall choose which GDU is decoded.

ADU parsing depends upon certain GDU header parameters.  ADU decoding depends upon the reconstructed slice geometry.

Slices are either independent or dependent.  An independent slice does not require any other slice to be decoded first.  A dependent slice requires that the immediately preceding slice in bitstream order is decoded first.  A slice shall be directly depended upon by no more than a single dependent slice.

A dependent slice shall not depend upon a slice in a different point cloud frame.

    - Repetition of slices

Slices may be repeated within a coded point cloud frame.  Repetition shall not change the value of slice_id.

A slice set is the set of slices with the same value of slice_id within a coded point cloud frame.

It is a requirement of bitstream conformance that all slices in each slice set shall reconstruct the same points in the same canonical order.

From each slice set, only one slice shall be decoded; all others shall be ignored for decoding (removed from the bitstream and discarded).  A decoder shall choose which slice is decoded.

    - Relationship between tiles and slices

A group of slices can be identified by a common slice tag identifier (slice_tag).

The tile inventory DU provides a means to associate a bounding box with a group of slices.  Each tile comprises a single bounding box and an identifier (tileId).  Tile bounding boxes may overlap.  Implementations can use a tile inventory to aid spatial random access.

When a tile inventory is present in the bitstream, slice_tag shall identify a tile by its tileId.  Otherwise, the use of slice tags is application specific.

When a slice tag identifies a tile, a dependent slice should not depend upon a slice in a different tile.  To do otherwise can prevent decoding of individual tiles (for example, in spatial random access decoding).

Tile information is not used by the decoding processes specified in this document.

Key

| St,n | Slice 𝑛, associated with tile 𝑡 |
| --- | --- |
| Tt | Bounding box of tile 𝑡 |

Figure 3 — Example arrangement of tiles and slices.

An example arrangement of tiles and slices within a coded point cloud frame is shown in Figure 3.  Slices Sa,1 and Sa,2 are associated with tile Ta and slices Sb,3, Sb,4 and Sb,5 are associated with tile Tb; the bounding box of Tb does not include Sb,3.  A decoder that performs spatial random access to decode a region R (not shown) can use the tile inventory to determine tile IDs for the set of tiles that intersect R.  Only slices with matching slice tags would need to be decoded.  Since the slice Sb,3 is not included in the bounding box of tile Tb, if R intersects Sb,3 but not Tb, the slice is not discoverable using the tile inventory.  However, in the case that R and Tb intersect, Sb,3 would have a matching slice tag.

    - Parameter sets

      - Activation of parameter sets

The parameters contained in an SPS, GPS or APS shall not have any effect until the activation of the respective parameter set.

At most one SPS, GPS and APS are active at any given moment during the decoding process.  The activation of a parameter-set shall deactivate any previously active parameter set of the same type.

At the start of a coded point cloud sequence, no parameter sets are active.

An SPS shall be activated by the parsing of a GDU.  Once activated, it shall remain active for the whole of the coded point cloud sequence.

A GPS shall be activated by the parsing of a GDU.

An APS shall be activated by the parsing of an ADU.

- Other DUs that contain references to SPS, GPS or APS DUs do not cause the referenced parameter-set to be activated.

      - Order of parameter sets

DUs shall be conveyed to a decoder in an order such that any parameter-set to be activated is available prior to the point of activation.

      - Duplication of parameter sets

Parameter-set DUs may be repeated at any point in the coded point cloud sequence.

All parameter-set DUs with the same parameter-set identifier shall be identical for the duration of the coded point cloud sequence.

- Parameter-set identifiers are distinct for each type of parameter set.

  - Output point cloud format

    - General

Point cloud frames decoded from a G-PCC bitstream shall be output in the output point cloud format (6.5).

    - Coordinate system

A decoder shall output points in the sequence coordinate system.

The output point cloud format shall indicate the sequence origin SeqOrigin and the sequence unit SeqUnit as point cloud metadata.

    - Fixed-point conformance output

A decoder that is configured to output 𝑛-fractional-bit fixed-point positions shall round half-values of PSCS away from zero prior to output as POUT:

POUT=RoundFzPCCS×SeqCodedScale×2n÷2n

    - Attributes

Attribute values shall be interpreted according to the semantics of the attribute type and any per-sequence or frame-specific attribute properties.  For example, if a frame-specific scale and offset property is present for an attribute, the output attribute values for that frame would be interpreted according to 7.4.2.2.5.

    - Output point cloud sequence

Decoding a conforming G-PCC bitstream generates a sequence of output point cloud frames.  Output point clouds frames are output in the order of FrameCtr of each point cloud frame within the sequence.

    - Output point cloud frame

Each output point cloud frame is specified in terms of the following state variables:

- The variable RecCloudPointCnt, the cumulative number of points in the output point cloud frame.

- The array RecCloudPos of decoded point positions; RecCloudPos[ ptIdx ][ 𝑘 ] is the 𝑘-th coordinate of the ptIdx-th output point in the coding coordinate system.

- The array RecCloudAttr of decoded point attributes; RecCloudAttr[ ptIdx ][ attrIdx ][ 𝑐 ] is the 𝑐-th component of the identified attribute for the ptIdx-th point.  Attributes are identified by the index attrIdx into the active SPS attribute list.

Decoder implementations may output points in a different order to the canonical order specified by this document.

Immediately prior to outputting the decoded point cloud frame, point positions shall be converted to the sequence coordinate system.

Each decoded point cloud frame shall be stored in buffer. One decoded point cloud frame stored in buffer shall be output when it is the first decoded point cloud frame or the FrameCtr of it is equal to the FrameCtr of the previously one output point cloud frame plus 1.  The buffer for one decoded point cloud frame shall be released when the point cloud frame does not serve as the reference frame for any other point cloud frames to be decoded.

Syntax and semantics

  - Method of specifying syntax in tabular form

The syntax tables specify a superset of the syntax of all allowed bitstreams.  Additional constraints on the syntax may be specified, either directly or indirectly, in other clauses.

The following table lists examples of pseudo code used to describe the syntax.  When syntax_element appears, it specifies that a syntax element is parsed from the bitstream and the bitstream pointer is advanced to the next position beyond the syntax element in the bitstream parsing process.

|  | Descriptor |
| --- | --- |
| /* A statement can be a syntax element with an associated descriptor or can be a statement used to specify conditions for the existence, type and quantity of syntax elements, as in the following two examples */ |  |
| syntax_element | ue(v) |
| statement |  |
| /* A group of statements enclosed in curly brackets is a compound statement and is treated functionally as a single statement. */ |  |
| { |  |
| statement |  |
| statement |  |
| … |  |
| } |  |
| /* A "while" structure specifies a test of whether a condition is true, and if true, specifies evaluation of a statement (or compound statement) repeatedly until the condition is no longer true */ |  |
| while( condition ) |  |
| statement |  |
| /* A "do … while" structure specifies evaluation of a statement once, followed by a test of whether a condition is true, and if true, specifies repeated evaluation of the statement until the condition is no longer true */ |  |
| do { |  |
| statement |  |
| } while( condition ) |  |
| /* An "if … else" structure specifies a test of whether a condition is true, and if the condition is true, specifies evaluation of a primary statement, otherwise, specifies evaluation of an alternative statement. The "else" part of the structure and the associated alternative statement is omitted if no alternative statement evaluation is needed */ |  |
| if( condition ) |  |
| primary statement |  |
| else |  |
| alternative statement |  |
| /* A "for" structure specifies evaluation of an initial statement, followed by a test of a condition, and if the condition is true, specifies repeated evaluation of a primary statement followed by a subsequent statement until the condition is no longer true. */ |  |
| for( initial statement; condition; subsequent statement ) |  |
| primary statement |  |

  - Specification of syntax functions and descriptors

The functions presented here are used in the syntactical description.  These functions are expressed in terms of the value of the bitstream pointer DataUnitReadIdx that indicates the position of the next bit to be read from the bitstream by the decoding process.

byte_aligned( ) is specified as:

- If the next bit in the bitstream is the first bit in a byte (DataUnitReadIdx % 8 == 0), the value of byte_aligned( ) is true.

- Otherwise, the value of byte_aligned( ) is false.

more_data_in_data_unit( ) is specified as:

- If parsing of the DU is incomplete (DataUnitReadIdx / 8 < DataUnitLength), the value of more_data_in_data_unit( ) is true.

- Otherwise, the value of more_data_in_data_unit( ) is false.

Length( 𝑥 ) is the length in bits of the coded syntax element 𝑥 as measured by the change in DataUnitReadIdx between the start and end of the syntax element.

The following descriptors specify the parsing process of every syntax element.  The parsing processes are specified in Clause 11.

- ae(v): adaptive arithmetic entropy-coded syntax element.

- de(v): dictionary coded syntax element.

- oid(v): an ASN.1 object identifier.

- s(𝑛): signed integer using an 𝑛-bit magnitude and a sign bit.

- se(v): signed integer 0-th order Exp-Golomb-coded syntax element.

- u(𝑛): unsigned integer using 𝑛 bits.  When 𝑛 is "v" in the syntax table, the number of bits varies in a manner dependent upon the value of other syntax elements.

- ue(v): unsigned integer 0-th order Exp-Golomb-coded syntax element.

  - Syntax in tabular form

    - General

The syntax structures and the syntax elements within these structures are specified in 7.3.2.  Any values that are not specified in the tables shall not be present in the bitstream unless otherwise specified in this document.

    - Parameter sets, ancillary data and byte alignment

      - Sequence parameter set data unit syntax

| seq_parameter_set( ) { | Descriptor |
| --- | --- |
| simple_profile_compliant | u(1) |
| dense_profile_compliant | u(1) |
| predictive_profile_compliant | u(1) |
| main_profile_compliant | u(1) |
| reserved_profile_18bits | u(18) |
| slice_reordering_constraint | u(1) |
| unique_point_positions_constraint | u(1) |
| level_idc | u(8) |
| sps_seq_parameter_set_id | u(4) |
| frame_ctr_lsb_bits | u(5) |
| slice_tag_bits | u(5) |
| seq_origin_bits | ue(v) |
| if( seq_origin_bits) { |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| seq_origin_xyz[ 𝑘 ] | s(v) |
| seq_origin_log2_scale | ue(v) |
| } |  |
| seq_bbox_size_bits | ue(v) |
| if( seq_bbox_size_bits ) |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| seq_bbox_size_minus1_xyz[ 𝑘 ] | u(v) |
| seq_unit_numerator_minus1 | ue(v) |
| seq_unit_denominator_minus1 | ue(v) |
| seq_unit_is_metres | u(1) |
| seq_coded_scale_exponent | ue(v) |
| seq_coded_scale_mantissa_bits | ue(v) |
| seq_coded_scale_mantissa | u(v) |
| num_attributes | ue(v) |
| for( attrIdx = 0; attrIdx < num_attributes; attrIdx++ ) { |  |
| attr_components_minus1[ attrIdx ] | ue(v) |
| attr_instance_id[ attrIdx ] | ue(v) |
| attr_bitdepth_minus1[ attrIdx ] | ue(v) |
| attr_label_known[ attrIdx ] | u(1) |
| if( attr_label_known[ attrIdx ] ) |  |
| attr_label[ attrIdx ] | ue(v) |
| else |  |
| attr_label_oid[ attrIdx ] | oid(v) |
| attr_property_cnt | ue(v) |
| byte_alignment( ) |  |
| for( 𝑗 = 0; 𝑗 < attr_property_cnt; 𝑗++ ) |  |
| attribute_property( attrIdx ) |  |
| } |  |
| geom_axis_order | u(3) |
| bypass_stream_enabled | u(1) |
| entropy_continuation_enabled | u(1) |
| sps_extension_present | u(1) |
| if( sps_extension_present ) { |  |
| if(num_attributes > 1) |  |
| cross_attr_prediction_enabled | u(1) |
| bypass_bin_coding_prob_update_disabled | u(1) |
| inter_frame_prediction_enabled | u(1) |
| if(inter_frame_prediction_enabled) |  |
| inter_entropy_continuation_enabled | u(1) |
| fgs_layer_group_enabled | u(1) |
| while( more_data_in_data_unit( ) ) |  |
| sps_extension_data | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

      - Attribute property syntax

| attribute_property( attrIdx ) { | Descriptor |
| --- | --- |
| attr_prop_type | u(8) |
| attr_prop_len | u(8) |
| AttrPropDataLen = attr_prop_len |  |
| if( attr_prop_type == 0 ) { |  |
| attr_prop_itu_t_t35_country_code | u(8) |
| AttrPropDataLen−− |  |
| if( attr_prop_itu_t_t35_country_code == 255 ) { |  |
| attr_prop_itu_t_t35_country_code_extension_byte | u(8) |
| AttrPropDataLen−− |  |
| } |  |
| attribute_property_data( attrIdx, AttrPropDataLen ) |  |
| } else if( attr_prop_type == 1 ) { |  |
| attr_prop_oid | oid(v) |
| AttrPropDataLen −= Length( attr_prop_oid ) / 8 |  |
| attribute_property_data( attrIdx, AttrPropDataLen ) |  |
| } else if( attr_prop_type == 2 ) { |  |
| attr_cicp_colour_primaries[ attrIdx ] | ue(v) |
| attr_cicp_transfer_characteristics[ attrIdx ] | ue(v) |
| attr_cicp_matrix_coeffs[ attrIdx ] | ue(v) |
| attr_cicp_video_full_range[ attrIdx ] | u(1) |
| } else if( attr_prop_type == 3 ) { |  |
| attr_offset_bits | ue(v) |
| attr_offset[ attrIdx ] | s(v) |
| attr_scale_bits | ue(v) |
| attr_scale_minus1[ attrIdx ] | u(v) |
| attr_frac_bits[ attrIdx ] | ue(v) |
| } else if( attr_prop_type == 4 ) { |  |
| for( 𝑐 = 0; 𝑐 ≤ attr_components_minus1[ attrIdx ]; 𝑐++ ) |  |
| attr_default_value[ attrIdx ][ 𝑐 ] | u(v) |
| } else |  |
| attribute_property_data( attrIdx, attr_prop_len ) |  |
| byte_alignment( ) |  |
| } |  |

      - Attribute property data syntax

| attribute_property_data( attrIdx, numBytes ) { | Descriptor |
| --- | --- |
| for( 𝑖 = 0; 𝑖 < numBytes; 𝑖++) |  |
| attr_prop_byte[ 𝑖 ] | u(8) |
| } |  |

      - Tile inventory data unit syntax

| tile_inventory( ) { | Descriptor |
| --- | --- |
| ti_seq_parameter_set_id | u(4) |
| ti_frame_ctr_lsb_bits | u(5) |
| ti_frame_ctr_lsb | u(v) |
| tile_cnt | u(16) |
| if( tile_cnt > 0 ) { |  |
| tile_id_bits | u(5) |
| tile_origin_bits_minus1 | u(8) |
| tile_size_bits_minus1 | u(8) |
| for( tileIdx = 0; tileIdx < tile_cnt; tileIdx++ ) { |  |
| tile_id[ tileIdx ] | u(v) |
| tileId = tile_id_bits ? tile_id[ tileIdx ] : tileIdx |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| tile_origin_xyz[ tileId ][ 𝑘 ] | s(v) |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| tile_size_minus1_xyz[ tileId ][ 𝑘 ] | u(v) |
| } |  |
| ti_origin_bits_minus1 | ue(v) |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| ti_origin_xyz[ 𝑘 ] | s(v) |
| ti_origin_log2_scale | ue(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

      - Geometry parameter set data unit syntax

| geometry_parameter_set( ) { | Descriptor |
| --- | --- |
| gps_geom_parameter_set_id | u(4) |
| gps_seq_parameter_set_id | u(4) |
| slice_geom_origin_scale_present | u(1) |
| if( ¬slice_geom_origin_scale_present ) |  |
| gps_geom_origin_log2_scale | ue(v) |
| geom_dup_point_counts_enabled | u(1) |
| geom_tree_type | u(1) |
| if( geom_tree_type == 0 ) { |  |
| occtree_point_cnt_list_present | u(1) |
| occtree_direct_coding_mode | u(2) |
| if( occtree_direct_coding_mode ) |  |
| occtree_direct_joint_coding_enabled | u(1) |
| occtree_coded_axis_list_present | u(1) |
| occtree_neigh_window_log2_minus1 | u(3) |
| if( occtree_neigh_window_log2_minus1 > 0 ) { |  |
| occtree_adjacent_child_enabled | u(1) |
| occtree_intra_pred_max_nodesize_log2 | ue(v) |
| } |  |
| occtree_bitwise_coding | u(1) |
| occtree_planar_enabled | u(1) |
| if( occtree_planar_enabled ) { |  |
| for( 𝑖 = 0; 𝑖 < 3; 𝑖++) |  |
| occtree_planar_threshold[ 𝑖 ] | ue(v) |
| if( occtree_direct_coding_mode == 1 ) |  |
| occtree_direct_node_rate_minus1 | u(5) |
| } |  |
| } |  |
| geom_angular_enabled | u(1) |
| if( geom_angular_enabled ) { |  |
| slice_angular_origin_present | u(1) |
| if( ¬slice_angular_origin_present ) { |  |
| gps_angular_origin_bits_minus1 | ue(v) |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| gps_angular_origin_xyz[ 𝑘 ] | s(v) |
| } |  |
| if( geom_tree_type == 1 ) { |  |
| ptree_ang_azimuth_pi_bits_minus11 | ue(v) |
| ptree_ang_azimuth_step_minus1 | ue(v) |
| ptree_ang_radius_scale_log2 | ue(v) |
| } |  |
| num_beams_minus1 | ue(v) |
| beam_elevation_init | se(v) |
| beam_voffset_init | se(v) |
| if( geom_tree_type == 0 ) |  |
| beam_steps_per_rotation_init_minus1 | ue(v) |
| for( 𝑖 = 1; 𝑖 ≤ num_beams_minus1; 𝑖++ ) { |  |
| beam_elevation_diff[ 𝑖 ] | se(v) |
| beam_voffset_diff[ 𝑖 ] | se(v) |
| if( geom_tree_type == 0 ) |  |
| beam_steps_per_rotation_diff[ 𝑖 ] | se(v) |
| } |  |
| if( occtree_planar_enabled ) |  |
| occtree_planar_buffer_disabled | u(1) |
| } |  |
| geom_scaling_enabled | u(1) |
| if( geom_scaling_enabled ) { |  |
| geom_qp | ue(v) |
| geom_qp_mul_log2 | u(2) |
| if( geom_tree_type == 1 ) |  |
| ptree_qp_period_log2 | ue(v) |
| else if( occtree_direct_coding_mode ) |  |
| occtree_direct_node_qp_offset | se(v) |
| } |  |
| gps_extension_present | u(1) |
| if( gps_extension_present ) { |  |
| if(geom_tree_type == 0) { |  |
| trisoup_enabled | u(1) |
| if(trisoup_enabled) { |  |
| trisoup_non_cubic_node_start_edge_present | u(1) |
| trisoup_non_cubic_node_end_edge_present | u(1) |
| } |  |
| } |  |
| if(geom_tree_type == 0 && geom_angular_enabled) |  |
| occtree_angular_extension_enabled | u(1) |
| if(geom_tree_type == 0 ¦¦ geom_angular_enabled) |  |
| inter_prediction_enabled | u(1) |
| if(inter_prediction_enabled) { |  |
| global_motion_enabled | u(1) |
| if(geom_tree_type == 1) { |  |
| inter_azim_scale_log2 | ue(v) |
| resampling_enabled | u(1) |
| max_points_per_entry_minus1 | ue(v) |
| if(max_points_per_entry_minus1 > 0) |  |
| down_sampling_range | ue(v) |
| } |  |
| biprediction_enabled | ue(v) |
| if(biprediction_enabled > 0) |  |
| frame_merge_enabled | u(1) |
| } |  |
| if( geom_tree_type == 0 && geom_angular_enabled && occtree_direct_coding_mode && inter_prediction_enabled) |  |
| occtree_inter_angular_direct_coding_enabled | u(1) |
| if(occtree_planar_enabled && geom_angular_enabled && occtree_direct_coding_mode) |  |
| geo_planar_idcm_angular_disabled | u(1) |
| if( geom_tree_type == 1 && geom_angular_enabled) { |  |
| ptree_sec_resid_disabled | u(1) |
| ptree_ang_azimuth_scaling_enabled | u(1) |
| if(ptree_ang_azimuth_scaling_enabled) { |  |
| ptree_ang_max_pred_index | ue(v) |
| ptree_ang_pred_list_radius_resid_threshold | ue(v) |
| ptree_ang_radius_resid_context_qphi_threshold_present | u(1) |
| if(ptree_ang_radius_resid_context_qphi_threshold_present) { |  |
| ptree_ang_radius_resid_context_qphi_threshold | ue(v) |
| } |  |
| } |  |
| } |  |
| if(occtree_planar_enabled && ¬geom_angular_enabled ) |  |
| octree_planar_neigh_prediction_enabled | u(1) |
| while( more_data_in_data_unit( ) ) |  |
| gps_extension_data | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

      - Attribute parameter set data unit syntax

| attribute_parameter_set( ) { | Descriptor |
| --- | --- |
| aps_attr_parameter_set_id | u(4) |
| aps_seq_parameter_set_id | u(4) |
| attr_coding_type | ue(v) |
| attr_primary_qp_minus4 | ue(v) |
| attr_secondary_qp_offset | se(v) |
| attr_qp_offsets_present | u(1) |
| if( attr_coding_type == 0 ) { |  |
| raht_prediction_enabled | u(1) |
| if( raht_prediction_enabled ) { |  |
| raht_prediction_subtree_min | ue(v) |
| raht_prediction_samples_min | ue(v) |
| } |  |
| } else if( attr_coding_type ≤ 2 ) { |  |
| pred_set_size_minus1 | ue(v) |
| pred_inter_lod_search_range | ue(v) |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |
| pred_dist_bias_minus1_xyz[ 𝑘 ] | ue(v) |
| if( attr_coding_type == 2 ) |  |
| last_comp_pred_enabled | u(1) |
| lod_scalability_enabled | u(1) |
| if( lod_scalability_enabled ) |  |
| pred_max_range_minus1 | ue(v) |
| else { |  |
| lod_max_levels_minus1 | ue(v) |
| if( ¬lod_max_levels_minus1 ) |  |
| attr_canonical_order_enabled | u(1) |
| else { |  |
| lod_decimation_mode | ue(v) |
| if( lod_decimation_mode > 0 ) |  |
| for( lvl = 0; lvl < lod_max_levels_minus1; lvl++ ) |  |
| lod_sampling_period_minus2[ lvl ] | ue(v) |
| lod_initial_dist_log2 | ue(v) |
| lod_dist_log2_offset_present | u(1) |
| } |  |
| } |  |
| if( attr_coding_type == 1 ) { |  |
| pred_direct_max_idx_plus1 | ue(v) |
| if( pred_direct_max_idx_plus1 ) { |  |
| pred_direct_threshold | u(8) |
| pred_direct_avg_disabled | u(1) |
| } |  |
| pred_intra_lod_search_range | ue(v) |
| if( pred_intra_lod_search_range ) |  |
| pred_intra_min_lod | ue(v) |
| inter_comp_pred_enabled | u(1) |
| pred_blending_enabled | u(1) |
| } |  |
| } else if( attr_coding_type == 3 ) |  |
| raw_attr_width_present | u(1) |
| if( ¬lod_scalability_enabled ) |  |
| attr_coord_conv_enabled | u(1) |
| if( attr_coord_conv_enabled ) |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) { |  |
| attr_coord_conv_scale_bits_minus1[ 𝑘 ] | u(5) |
| attr_coord_conv_scale[ 𝑘 ] | u(v) |
| } |  |
| aps_extension_present | u(1) |
| if( aps_extension_present ) { |  |
| if(cross_attr_prediction_enabled) { |  |
| cross_attr_prediction_enabled_this_type | u(1) |
| if( cross_attr_prediction_enabled_this_type) |  |
| refAttrIdx | ue(v) |
| } |  |
| if( attr_coding_type == 0 ) { |  |
| lossless_coding_enabled | u(1) |
| raht_last_comp_pred_enabled | u(1) |
| } |  |
| if( attr_coding_type == 1) |  |
| for( i = 0; i <= pred_set_size_minus1; i++ ) |  |
| quant_neigh_weight[ i ] | ue(v) |
| attr_inter_prediction_enabled | u(1) |
| if( attr_inter_prediction_enabled) |  |
| if( attr_coding_type == 0 ) { |  |
| raht_inter_layer_depth_minus1 | ue(v) |
| if(lossless_coding_enabled == 0 ) { |  |
| raht_send_inter_filters | u(1) |
| raht_inter_skip_layers | ue(v) |
| } |  |
| raht_inter_layer_code_enabled | u(1) |
| } |  |
| else |  |
| attr_inter_prediction_search_range | ue(v) |
| if( (attr_coding_type == 1 ¦¦ attr_coding_type == 2) && ¬lod_scalability_enabled && ¬lod_max_levels_minus1) |  |
| max_points_per_sort_log2_plus1 | ue(v) |
| if( (attr_coding_type == 1 ¦¦ attr_coding_type == 2) && pred_set_size_minus1 >= 2) |  |
| prediction_with_distribution_enabled | u(1) |
| if( attr_coding_type == 0 ) |  |
| raht_buffer_extension_flag | u(1) |
| if( attr_coding_type == 0 && raht_prediction_enabled) { |  |
| raht_subnode_prediction_enabled | u(1) |
| raht_intra_layer_code_enabled | u(1) |
| if( raht_subnode_prediction_enabled) |  |
| for( i = 0; i < 5; i ++ ) |  |
| raht_prediction_weights[ i ] | ue(v) |
| raht_prediction_search_range | ue(v) |
| } |  |
| if( (attr_coding_type == 1 ¦¦ attr_coding_type == 2) && fgs_layer_group_enabled) |  |
| fgs_attr_parameter( ) |  |
| while( more_data_in_data_unit( ) ) |  |
| aps_extension_data | u(1) |
| } |  |
| byte_alignment( ) |  |
| } |  |

      - Frame-specific attribute properties data unit syntax

| frame_specific_attribute_properties( ) { | Descriptor |
| --- | --- |
| fsap_seq_parameter_set_id | u(4) |
| fsap_frame_ctr_lsb_bits | u(5) |
| fsap_frame_ctr_lsb | u(v) |
| fsap_sps_attr_idx | ue(v) |
| fsap_num_props | ue(v) |
| byte_alignment( ) |  |
| for( 𝑖 = 0; 𝑖 < fsap_num_props; 𝑖++ ) |  |
| attribute_property( fsap_sps_attr_idx ) |  |
| } |  |

      - Frame boundary marker data unit syntax

| frame_boundary_marker( ) { | Descriptor |
| --- | --- |
| fbdu_frame_ctr_lsb_bits | u(5) |
| fbdu_frame_ctr_lsb | u(v) |
| byte_alignment( ) |  |
| } |  |

      - User data data unit syntax

| userdata_data_unit( ) { | Descriptor |
| --- | --- |
| user_data_oid | oid(v) |
| while( more_data_in_data_unit( ) ) |  |
| user_data_byte | u(8) |
| } |  |

      - Byte alignment syntax

| byte_alignment( ) { | Descriptor |
| --- | --- |
| while( ¬byte_aligned( ) ) |  |
| alignment_bit_equal_to_zero /* equal to 0 */ | u(1) |
| } |  |

    - Geometry data unit

      - Geometry data unit syntax

| geometry_data_unit( ) { | Descriptor |
| --- | --- |
| geometry_data_unit_header( ) |  |
| if( geom_tree_type == 0 ) { |  |
| if(fgs_layergroup_enabled) |  |
| fgs_occupancy_tree(startDepth, endDepth ) |  |
| else |  |
| occupancy_tree(occtreeMaxDepthMinus1 ) |  |
| if(trisoup_enabled) |  |
| trisoup( ) |  |
| } else if( geom_tree_type == 1 ) |  |
| predictive_tree( ) |  |
| geometry_data_unit_footer(occtreeMaxDepthMinus1 ) |  |
| } |  |

      - Geometry data unit header syntax

| geometry_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| gdu_geometry_parameter_set_id | u(4) | 7.4.3.2 |
| gdu_temporal_id | u(3) | 7.4.3.2 |
| slice_id | ue(v) | 7.4.3.2 |
| slice_tag | u(v) | 7.4.3.2 |
| frame_ctr_lsb | u(v) | 7.4.3.2 |
| if( entropy_continuation_enabled ) { |  |  |
| slice_entropy_continuation | u(1) | 7.4.3.2 |
| if( slice_entropy_continuation ) |  |  |
| prev_slice_id | ue(v) | 7.4.3.2 |
| } |  |  |
| if( slice_geom_origin_scale_present ) |  |  |
| slice_geom_origin_log2_scale | ue(v) | 7.4.3.2 |
| slice_geom_origin_bits_minus1 | ue(v) | 7.4.3.2 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| slice_geom_origin_xyz[ 𝑘 ] | u(v) | 7.4.3.2 |
| if( slice_angular_origin_present ) { |  |  |
| slice_angular_origin_bits_minus1 | ue(v) | 7.4.3.2 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| slice_angular_origin_xyz[ 𝑘 ] | s(v) | 7.4.3.2 |
| } |  |  |
| if( geom_tree_type == 0 ) { |  |  |
| occtree_depth_minus1 | ue(v) | 9.2.3 |
| if( occtree_coded_axis_list_present ) |  |  |
| for( dpth = 0; dpth ≤ occtreeMaxDepthMinus1; dpth++ ) |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| occtree_coded_axis[ dpth ][ 𝑘 ] | u(1) | 9.2.3 |
| occtree_stream_cnt_minus1 | ue(v) | 9.2.3 |
| } |  |  |
| if( geom_scaling_enabled ) { |  |  |
| slice_geom_qp_offset | se(v) | 7.4.3.2 |
| if( geom_tree_type == 1 ) |  |  |
| slice_ptree_qp_period_log2_offset | se(v) | 9.3.2.1 |
| } |  |  |
| if(trisoup_enabled ) |  |  |
| trisoup_params( ) |  |  |
| if( geom_tree_type == 1 ) { |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| ptn_resid_abs_log2_bits[ 𝑘 ] | u(3) | 9.3.2.1 |
| if( geom_angular_enabled ) |  |  |
| ptn_radius_min | ue(v) | 9.3.2.1 |
| } |  |  |
| if( inter_prediction_enabled) { |  |  |
| slice_inter_prediction | u(1) | 7.4.3.2 |
| if( slice_inter_prediction) { |  |  |
| if( global_motion_enabled) |  |  |
| global_motion_params(0) |  |  |
| if(biprediction_enabled) |  |  |
| slice_biprediction | u(1) | 7.4.3.2 |
| if( slice_biprediction && global_motion_enabled) |  |  |
| global_motion_params(1) |  |  |
| } |  |  |
| } |  |  |
| if( slice_inter_prediction && inter_entropy_continuation_enabled) { |  |  |
| slice_inter_entropy_continuation | u(1) | 7.4.3.2 |
| if( slice_inter_entropy_continuation ) { |  |  |
| prev_inter_entropy_frame_ctr_lsb | u(v) | 7.4.3.2 |
| prev_inter_entropy_slice_id | ue(v) | 7.4.3.2 |
| } |  |  |
| } |  |  |
| if(fgs_layer_group_enabled) |  |  |
| if( geom_tree_type == 0 ) |  |  |
| fgs_geometry_data_unit_parameter() |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

      - Geometry data unit footer syntax

| geometry_data_unit_footer(occtreeMaxDepthMinus1 ) { | Descriptor | Semantics |
| --- | --- | --- |
| byte_alignment( ) |  |  |
| if( occtree_point_cnt_list_present ) |  |  |
| for( dpth = 1; dpth < occtreeMaxDepthMinus1; dpth++ ) |  |  |
| occtree_lvl_point_cnt_minus1[ dpth ] | u(24) | 9.2.3 |
| slice_num_points_minus1 | u(24) | 7.4.3.3 |
| } |  |  |

      - Occupancy tree syntax

| occupancy_tree(occtreeMaxDepthMinus1) { | Descriptor | Semantics |
| --- | --- | --- |
| if( slice_inter_prediction && global_motion_enabled && MotionPartitionType == 1) |  |  |
| for(idx = 0; idx < NumMotionBlocks; idx++) |  |  |
| gm_comp_partition_block[idx] | ae(v) | 9.2.15.2.2 |
| OccQpSubtreeDepth = occtreeMaxDepthMinus1 + 1 |  | 9.2.14.4 |
| for( Dpth = 0; Dpth ≤ occtreeMaxDepthMinus1; Dpth++ ) { |  |  |
| occupancy_tree_level( Dpth ) |  |  |
| if( Dpth + 1 > OcctreeEntropyStreamDepth ) |  | 9.2.3 |
| occtree_end_of_entropy_stream | ae(v) | 9.2.3 |
| } |  |  |
| } |  |  |

      - Occupancy tree level syntax

| occupancy_tree_level( dpth ) { | Descriptor | Semantics |
| --- | --- | --- |
| if( geom_scaling_enabled && dpth < OccQpSubtreeDepth ) |  |  |
| occ_subtree_qp_offset_present | ae(v) | 9.2.14.3 |
| if( occ_subtree_qp_offset_present ) |  |  |
| OccQpSubtreeDepth = dpth |  |  |
| for( NodeIdx = 0; NodeIdx < OccNodeCnt[ dpth ]; NodeIdx++ ) |  |  |
| occupancy_tree_node( dpth, NodeIdx ) |  |  |
| } |  |  |

      - Occupancy tree node syntax

| occupancy_tree_node( dpth, nodeIdx ) { | Descriptor | Semantics |
| --- | --- | --- |
| if( occ_subtree_qp_offset_present ) { |  |  |
| occ_subtree_qp_offset_abs[ Ns ][ Nt ][ Nv ] | ae(v) | 9.2.14.3 |
| if( occ_subtree_qp_offset_abs[ Ns ][ Nt ][ Nv ] ) |  |  |
| occ_subtree_qp_offset_sign[ Ns ][ Nt ][ Nv ] | ae(v) | 9.2.14.3 |
| } |  |  |
| if(occtree_direct_coding_mode && DirectNodePresent && geo_planar_idcm_angular_disabled) |  |  |
| occ_direct_node | ae(v) | 9.2.12.2 |
| if( occtree_planar_enabled ) { |  |  |
| if(gps_extension_present ) { |  |  |
| if(AllowPlanarCopyMode ) |  | 9.2.11.10 |
| planar_copy_mode | ae(v) | 9.2.11.2 |
| if((¬planar_copy_mode && MultiPlanarEligible ) |  | 9.2.11.8 |
| multi_planar_flag | ae(v) | 9.2.11.2 |
| } |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| if( PlanarEligible[ 𝑘 ] ) { |  | 9.2.11.5 |
| if( ¬gps_extension_present ¦¦ ¬PlanarInferred[ 𝑘 ]) |  | 9.2.11.9 |
| occ_single_plane[ 𝑘 ] | ae(v) | 9.2.11.2 |
| if( occ_single_plane[ 𝑘 ] ¦¦ ¬PlanarPosInferred[ 𝑘 ]) |  | 9.2.11.11 |
| occ_plane_pos[ 𝑘 ] | ae(v) | 9.2.11.2 |
| } |  |  |
| } |  |  |
| if( occtree_direct_coding_mode && DirectNodePresent && ¬geo_planar_idcm_angular_disabled ) |  | 9.2.12.3.2 |
| occ_direct_node | ae(v) | 9.2.12.2 |
| if( occ_direct_node ) |  |  |
| occupancy_tree_direct_node( ) |  |  |
| else { |  |  |
| if( OccMaybeSingleChild ) |  | 9.2.6.8 |
| occ_single_child | ae(v) | 9.2.6.2 |
| if( occ_single_child ) |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| if( OccFreeAxis[ 𝑘 ] ) |  | 9.2.6.6 |
| occupancy_idx[ 𝑘 ] | ae(v) | 9.2.6.2 |
| if( OccMapPresent ) |  | 9.2.6.9 |
| if( occtree_bitwise_coding ) { |  |  |
| for( 𝑖 = 0; 𝑖 < 8; 𝑖++ ) |  |  |
| if( OccBitPresent[ 𝑖 ] ) |  | 9.2.10.3 |
| occupancy_bit[ 𝑖 ] | ae(v) | 9.2.6.2 |
| } else |  |  |
| occupancy_byte | de(v) | 9.2.6.2 |
| if( TerminalNode && geom_dup_point_counts_enabled ) |  | 9.2.6.5 |
| for( child = 0; child < OccChildCnt; child++ ) |  |  |
| occ_dup_point_cnt[ child ] | ae(v) | 9.2.6.2 |
| } |  |  |
| } |  |  |

      - Direct node syntax

| occupancy_tree_direct_node( ) { | Descriptor | Semantics |
| --- | --- | --- |
| direct_point_cnt_eq2 | ae(v) | 9.2.12.2 |
| if( geom_dup_point_counts_enabled && ¬direct_point_cnt_eq2 ) |  |  |
| direct_dup_point_cnt | ae(v) | 9.2.12.2 |
| if( occtree_direct_joint_coding_enabled && direct_point_cnt_eq2 ) |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| if( ¬geom_angular_enabled ¦¦ 𝑘 == ( 1 ^ AzimuthAxis ) ) { |  | 9.2.13.3 |
| direct_joint_prefix[ 𝑘 ] | ae(v) | 9.2.12.2 |
| if( DnJointDiffBitPresent[ 𝑘 ] ) |  | 9.2.12.5.4 |
| direct_joint_diff_bit[ 𝑘 ] | ae(v) | 9.2.12.2 |
| } |  |  |
| for( dnPt = 0; dnPt ≤ direct_point_cnt_eq2; dnPt++ ) |  |  |
| if( geom_angular_enabled ) { |  |  |
| direct_rem[ dnPt ][ 1 ^ AzimuthAxis ] | ae(v) | 9.2.12.2 |
| beam_idx_resid_abs[ dnPt ] | ae(v) | 9.2.12.2 |
| if( beam_idx_resid_abs[ dnPt ] ) |  |  |
| beam_idx_resid_sign[ dnPt ] | ae(v) | 9.2.12.2 |
| direct_rem_st_ang[ dnPt ] | ae(v) | 9.2.12.2 |
| if( occtree_angular_extension_enabled) { |  |  |
| if(DnBitsAfterPlanar[2] > 0) |  |  |
| direct_v_ang_resid_abs[ dnPt ] | ae(v) | 9.2.12.2 |
| if( direct_v_ang_resid_abs[ dnPt ] > 0) |  |  |
| direct_v_ang_resid_sign[ dnPt ] | ae(v) | 9.2.12.2 |
| } else |  |  |
| direct_rem_v_ang[ dnPt ] | ae(v) | 9.2.12.2 |
| } else |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| direct_rem[ dnPt ][ 𝑘 ] | ae(v) | 9.2.12.2 |
| } |  |  |

      - Predictive tree syntax

| predictive_tree( ) { | Descriptor | Semantics |
| --- | --- | --- |
| PtnCnt = 0 |  |  |
| do { |  |  |
| predictive_tree_node( 0, PtnCnt ) |  |  |
| ptree_end_of_slice | ae(v) | 9.3.2.1 |
| } while( ¬ptree_end_of_slice ) |  |  |
| } |  |  |

      - Predictive tree node syntax

| predictive_tree_node( dpth, nodeIdx ) { | Descriptor | Semantics |
| --- | --- | --- |
| PtnCnt++ |  |  |
| if( geom_scaling_enabled && ¬( nodeIdx % PtnQpInterval ) ) { |  |  |
| ptn_qp_offset_abs[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if( ptn_qp_offset_abs[ nodeIdx ] ) |  |  |
| ptn_qp_offset_sign[ nodeIdx ] | ae(v) | 9.3.2.2 |
| } |  |  |
| if( geom_dup_point_counts_enabled ) |  |  |
| ptn_dup_point_cnt[ nodeIdx ] | ae(v) | 9.3.2.2 |
| ptn_child_cnt_xor1[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if(slice_inter_prediction && dpth) |  |  |
| ptn_inter_flag[nodeIdx] | ae(v) | 9.3.2.2 |
| if(slice_inter_prediction && slice_biprediction && dpth) |  |  |
| ptn_pred_direction[nodeIdx] | ae(v) | 9.3.2.2 |
| if(ptn_inter_flag[nodeIdx]) |  |  |
| ptn_inter_pred_mode[nodeIdx] | ae(v) | 9.3.2.2 |
| else |  |  |
| if( ¬ptree_ang_azimuth_scaling_enabled) |  |  |
| ptn_pred_mode[ nodeIdx ] | ae(v) | 9.3.2.2 |
| else |  |  |
| ptn_pred_idx[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if( geom_angular_enabled ) { |  |  |
| ptn_phi_mul_abs_prefix[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if( ptn_phi_mul_abs_prefix[ nodeIdx ] == 2 ) |  |  |
| ptn_phi_mul_abs_minus2[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if( ptn_phi_mul_abs_minus2[ nodeIdx ] == 7 ) |  |  |
| ptn_phi_mul_abs_minus9[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if( ptn_phi_mul_abs_prefix[ nodeIdx ] ) |  |  |
| ptn_phi_mul_sign[ nodeIdx ] | ae(v) | 9.3.2.2 |
| } |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) { |  |  |
| if( 𝑘 == 0 && ptree_ang_azimuth_scaling_enabled) { |  |  |
| ptn_radius_resid_abs[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if(ptn_radius_resid_abs[ nodeIdx ] > 0) |  |  |
| ptn_radius_resid_sign[ nodeIdx ] | ae(v) | 9.3.2.2 |
| } |  |  |
| else if( 𝑘 == 1 && ptree_ang_azimuth_scaling_enabled) { |  |  |
| ptn_phi_resid_abs_gt0[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if(ptn_phi_resid_abs_gt0[ nodeIdx ]) { |  |  |
| ptn_phi_resid_sign[ nodeIdx ] | ae(v) | 9.3.2.2 |
| ptn_phi_resid_abs_gt1[ nodeIdx ] | ae(v) | 9.3.2.2 |
| if(ptn_phi_resid_abs_gt1[ nodeIdx ]) |  |  |
| ptn_phi_resid_abs_rem[ nodeIdx ] | ae(v) | 9.3.2.2 |
| } |  |  |
| } |  |  |
| else if( 𝑘 < 2 ¦¦ ¬geom_angular_enabled ¦¦ num_beams_minus1 ) { |  |  |
| ptn_resid_abs_gt0[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| if( ptn_resid_abs_gt0[ nodeIdx ][ 𝑘 ] ) { |  |  |
| ptn_resid_abs_log2[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| ptn_resid_abs_rem[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| if( 𝑘 ¦¦ ptn_pred_mode[ nodeIdx ] ) |  |  |
| ptn_resid_sign[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| } |  |  |
| } |  |  |
| } |  |  |
| if( geom_angular_enabled && ¬ptree_sec_resid_disabled) |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) { |  |  |
| ptn_sec_resid_abs[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| if( ptn_sec_resid_abs[ nodeIdx ][ 𝑘 ] ) |  |  |
| ptn_sec_resid_sign[ nodeIdx ][ 𝑘 ] | ae(v) | 9.3.2.2 |
| } |  |  |
| for( 𝑖 = 0; 𝑖 < ( ptn_child_cnt_xor1[ nodeIdx ] ^ 1 ); 𝑖++ ) |  |  |
| predictive_tree_node( dpth + 1, PtnCnt ) |  |  |
| } |  |  |

      - TriSoup syntax

| trisoup( ) { | Descriptor | Semantics |
| --- | --- | --- |
| for( edgeIdx = 0; edgeIdx < numberTriSoupEdges; edgeIdx++ ) { |  |  |
| vertex_present[edgeIdx] | ae(v) | 9.4.2.2 |
| if( vertex_present[edgeIdx]) |  |  |
| for( bit = 0; bit < trisoup_vertex_number_bits; bit++ ) |  |  |
| vertex_position[edgeIdx][bit] | ae(v) | 9.4.2.2 |
| } |  |  |
| for( nodeIdx = 0; nodeIdx < numberTriSoupNodes; nodeIdx++ ) { |  |  |
| if( trisoup_centroid_vertex_residual_present && trisoup_sampling_value_minus1 ≤ 3 && numVertex > 3) { |  |  |
| centroid_residual_is_zero[nodeIdx] | ae(v) | 9.4.2.3 |
| if(¬centroid_residual_is_zero[nodeIdx]) { |  |  |
| if( highBound[nodeIdx] && lowBound[nodeIdx] ) |  |  |
| centroid_residual_sign[nodeIdx] | ae(v) | 9.4.2.3 |
| magBound = (centroid_residual_sign ? highBound[nodeIdx] : lowBound[nodeIdx]) - 1 |  |  |
| bit = 0 |  |  |
| while( magBound > 0) { |  |  |
| centroid_residual_magnitude[nodeIdx] [bit] | ae(v) | 9.4.2.3 |
| if( centroid_residual_magnitude[nodeIdx][bit] ) |  |  |
| Break |  |  |
| magBound-- |  |  |
| bit++ |  |  |
| } |  |  |
| magBits[nodeIdx] = bit |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if(trisoup_face_vertex_enabled) |  |  |
| for( nodeIdx = 0; nodeIdx < numberTriSoupNodes; nodeIdx++ ) |  |  |
| for(fvIdx = 0; fvIdx < 3; fvIdx++) |  |  |
| if(FaceEligible[nodeIdx][fvIdx]) |  | 9.4.2.4 |
| has_face_vertex[nodeIdx][fvIdx] | ae(v) | 9.4.2.3 |
| } |  |  |

      - TriSoup parameters syntax

| trisoup_params( ) { | Descriptor | Semantics |
| --- | --- | --- |
| trisoup_node_size_log2_minus2 | u(3) | 9.4.2.1 |
| trisoup_sampling_value_minus1 | u(8) | 9.4.2.1 |
| trisoup_num_unique_segments_bits_minus1 | ue(v) | 9.4.2.1 |
| trisoup_num_unique_segments_minus1 | u(v) | 9.4.2.1 |
| trisoup_vertex_number_bits | u(3) | 9.4.2.1 |
| trisoup_centroid_vertex_residual_present | u(1) | 9.4.2.1 |
| if( trisoup_centroid_vertex_residual_present) |  |  |
| trisoup_face_vertex_enabled | u(1) | 9.4.2.1 |
| trisoup_halo_enabled | u(1) | 9.4.2.1 |
| if( trisoup_halo_enabled) |  |  |
| trisoup_adaptive_halo_enabled | u(1) | 9.4.2.1 |
| trisoup_vertex_merge_enabled | u(1) | 9.4.2.1 |
| if( trisoup_non_cubic_node_start_edge_present) { |  |  |
| trisoup_slice_bb_pos_bits | ue(v) | 9.4.2.1 |
| if( trisoup_slice_bb_pos_bits) { |  |  |
| trisoup_slice_bb_pos_log2_scale | ue(v) | 9.4.2.1 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| trisoup_slice_bb_pos_xyz[k] | u(v) | 9.4.2.1 |
| } |  |  |
| } |  |  |
| if( trisoup_non_cubic_node_end_edge_present) { |  |  |
| trisoup_slice_bb_width_bits | ue(v) | 9.4.2.1 |
| if( trisoup_slice_bb_width_bits) { |  |  |
| trisoup_slice_bb_width_log2_scale | ue(v) | 9.4.2.1 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| trisoup_slice_bb_width_xyz[k] | u(v) | 9.4.2.1 |
| } |  |  |
| } |  |  |
| } |  |  |

      - Global motion parameters syntax

| global_motion_params(idx ) { | Descriptor | Semantics |
| --- | --- | --- |
| if( geom_tree_type == 1 ) |  |  |
| slice_inter_frame_ref_gmc[ idx] | u(1) | 7.4.3.2 |
| if( geom_tree_type == 0 ¦¦ slice_inter_frame_ref_gmc[ idx] ) { |  |  |
| for( i = 0; i < 3; i++ ) |  |  |
| for( j = 0; j < 3; j++ ) |  |  |
| gm_matrix[ idx][ i ][ j ] | se(v) | 7.4.3.2 |
| for( j = 0; j < 3; j++ ) |  |  |
| gm_trans[ idx][ j ] | se(v) | 7.4.3.2 |
| } |  |  |
| if( geom_tree_type == 0 && idx == 0) { |  |  |
| motion_partition_type | u(1) | 7.4.3.2 |
| motion_zero_origin | u(1) | 7.4.3.2 |
| if(MotionPartitionType == 1) |  |  |
| for( k = 0; k < 3; k++ ) |  |  |
| motion_block_size[ k] | ue(v) | 7.4.3.2 |
| } |  |  |
| if( MotionPartitionType == 0 ¦¦ slice_inter_frame_ref_gmc[ idx]) { |  |  |
| gm_thres_top[ idx] | se(v) | 7.4.3.2 |
| gm_thres_bot[ idx] | se(v) | 7.4.3.2 |
| } |  |  |
| } |  |  |

    - Attribute data unit

      - Attribute data unit syntax

| attribute_data_unit( ) { | Descriptor |
| --- | --- |
| attribute_data_unit_header( ) |  |
| if( attr_coding_type ≠ 3 ) { |  |
| if( raht_inter_layer_code_enabled ¦¦ raht_intra_layer_code_enabled) |  |
| raht_layer_pred_modes( ) |  |
| attribute_coeffs( ) |  |
| } |  |
| else |  |
| attribute_raw( ) |  |
| byte_alignment( ) |  |
| } |  |

      - Attribute data unit header syntax

| attribute_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| adu_attr_parameter_set_id | u(4) | 7.4.4.2 |
| adu_temporal_id | u(3) | 7.4.4.2 |
| adu_sps_attr_idx | ue(v) | 7.4.4.2 |
| adu_slice_id | ue(v) | 7.4.4.2 |
| if( lod_dist_log2_offset_present ¦¦ attr_inter_prediction_enabled) |  |  |
| lod_dist_log2_offset | se(v) | 10.6.2 |
| if( last_comp_pred_enabled && AttrDim == 3) |  |  |
| for( dpth = 0; dpth ≤ lod_max_levels_minus1; dpth++ ) |  |  |
| last_comp_pred_coeff_diff[ dpth ] | se(v) | 10.6.10.1 |
| if( inter_comp_pred_enabled ) |  |  |
| for( dpth = 0; dpth ≤ lod_max_levels_minus1; dpth++ ) |  |  |
| for( 𝑐 = 1; 𝑐 < AttrDim; 𝑐++) |  |  |
| inter_comp_pred_coeff_diff[ dpth ][ 𝑐 ] | se(v) | 10.6.10.1 |
| if( attr_qp_offsets_present ) |  |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++) |  |  |
| attr_qp_offset[ qc ] | se(v) | 10.7.1 |
| attr_qp_layers_present | u(1) | 10.7.1 |
| if( attr_qp_layers_present ) { |  |  |
| attr_qp_layer_cnt_minus1 | ue(v) | 10.7.1 |
| for( dpth = 0; dpth ≤ attr_qp_layer_cnt_minus1; dpth++ ) |  |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++ ) |  |  |
| attr_qp_layer_offset[ dpth ][ qc ] | se(v) | 10.7.1 |
| } |  |  |
| attr_qp_region_cnt | ue(v) | 10.7.1 |
| if( attr_qp_region_cnt ) |  |  |
| attr_qp_region_bits_minus1 | ue(v) | 10.7.1 |
| for( 𝑖 = 0; 𝑖 < attr_qp_region_cnt; 𝑖++ ) { |  |  |
| if( ¬attr_coord_conv_enabled ) { |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| attr_qp_region_origin_xyz[ 𝑖 ][ 𝑘 ] | u(v) | 10.7.1 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| attr_qp_region_size_minus1_xyz[ 𝑖 ][ 𝑘 ] | u(v) | 10.7.1 |
| } else { |  |  |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| attr_qp_region_origin_rpi[ 𝑖 ][ 𝑘 ] | u(v) | 10.7.1 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| attr_qp_region_size_minus1_rpi[ 𝑖 ][ 𝑘 ] | u(v) | 10.7.1 |
| } |  |  |
| for( ps = 0; ps < Min( 2, AttrDim ); ps++) |  |  |
| attr_qp_region_offset[ 𝑖 ][ ps ] | se(v) | 10.7.1 |
| } |  |  |
| if(attr_coding_type == 0 && lossless_coding_enabled == 0) { |  |  |
| attr_AC_qp_offset_present | u(1) | 10.7.1 |
| if( attr_AC_qp_offset_present){ |  |  |
| attr_AC_qp_layer_cnt_minus1 | ue(v) | 10.7.1 |
| for( dpth = 0; dpth < attr_AC_qp_layer_cnt_minus1; dpth++) |  |  |
| for( ACcompidx = 0; ACcompidx < 7; ACcompidx++) |  |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++) |  |  |
| attr_AC_qp_offset[dpth][qc ][ACcompidx] | se(v) | 10.7.1 |
| } |  |  |
| } |  |  |
| if( attr_inter_prediction_enabled) { |  |  |
| slice_attr_inter_prediction | u(1) | 7.4.4.2 |
| if( slice_biprediction) |  |  |
| slice_attr_inter_prediction2 | u(1) | 7.4.4.2 |
| if( slice_attr_inter_prediction && raht_send_inter_filters && ¬raht_inter_layer_code_enabled) { |  |  |
| num_inter_filters | ue(v) | 7.4.4.2 |
| for( filteridx = 0; filteridx < num_inter_filters; filteridx++) |  |  |
| raht_inter_filter_qidx[filteridx] | se(v) | 7.4.4.2 |
| } |  |  |
| if( raht_inter_layer_code_enabled && slice_attr_inter_prediction) { |  |  |
| layer_code_depth | ue(v) | 10.7.1 |
| if( raht_send_inter_filters) |  |  |
| for( filteridx = 0; filteridx < layer_code_depth + 1 – raht_inter_skip_layer; filteridx++) |  |  |
| if( filteridx == 0 && raht_inter_skip_layer == 0) |  |  |
| raht_inter_filter_qidx[filteridx] | se(v) | 7.4.4.2 |
| else if( slice_raht_inter_layer_code_mode[filteridx + raht_inter_skip_layer - 1]) |  |  |
| raht_inter_filter_qidx[filteridx] | se(v) | 7.4.4.2 |
| } |  |  |
| } |  |  |
| if(fgs_layer_group_enabled) |  |  |
| if(attr_coding_type == 1 ¦¦ attr_coding_type == 2) |  |  |
| fgs_attribute_data_unit_parameter() |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

      - Attribute data unit coefficients syntax

| attribute_coeffs( ) { | Descriptor | Semantics |
| --- | --- | --- |
| for( 𝑖 = 0; 𝑖 < PointCnt; 𝑖++ ) { |  |  |
| zero_run_length_prefix | ae(v) | 10.3.1.1 |
| if( zero_run_length_prefix == 3 ) { |  |  |
| zero_run_length_minus3_div2 | ae(v) | 10.3.1.1 |
| if( zero_run_length_minus3_div2 < 4) |  |  |
| zero_run_length_minus3_mod2 | ae(v) | 10.3.1.1 |
| else |  |  |
| zero_run_length_minus11 | ae(v) | 10.3.1.1 |
| } |  |  |
| 𝑖 += ZeroRunLength |  |  |
| if( 𝑖 < PointCnt ) |  |  |
| attribute_coeff_tuple( 𝑖 ) |  |  |
| } |  |  |
| } |  |  |

      - Attribute coefficient tuple syntax

| attribute_coeff_tuple( coeffIdx ) { | Descriptor | Semantics |
| --- | --- | --- |
| for( 𝑐 = 0, inferLastComp = 1; 𝑐 < AttrDim; 𝑐++ ) { |  |  |
| coeff_abs[ 𝑐 ] | ae(v) | 10.3.1.2 |
| if( coeff_abs[ 𝑐 ] ¦¦ ( 𝑐 == AttrDim – 1 && inferLastComp ) ) |  |  |
| coeff_sign[ 𝑐 ] | ae(v) | 10.3.1.2 |
| inferLastComp &= coeff_abs[ 𝑐 ] == 0 |  |  |
| } |  |  |
| } |  |  |

      - Raw attribute value syntax

| attribute_raw( ) { | Descriptor | Semantics |
| --- | --- | --- |
| for( ptIdx = 0; ptIdx < PointCnt; ptIdx++ ) |  |  |
| for( 𝑐 = 0; 𝑐 < AttrDim; 𝑐++ ) { |  |  |
| if( raw_attr_width_present ) |  |  |
| raw_attr_component_length | u(8) | 10.3.1.3 |
| raw_attr_value[ ptIdx ][ 𝑐 ] | u(v) | 10.3.1.3 |
| } |  |  |
| } |  |  |

      - Region-adaptive hierarchical layer prediction mode syntax

| raht_layer_pred_modes( ) { | Descriptor | Semantics |
| --- | --- | --- |
| start = RahtRootLvl |  |  |
| for( lvl = start; lvl ≥ 0; lvl--) { |  |  |
| if( raht_inter_layer_code_enabled) |  |  |
| slice_raht_inter_layer_code_mode[ lvl ] | ae(v) | 7.4.4.2 |
| if( raht_intra_layer_code_enabled ) |  |  |
| slice_raht_intra_layer_code_mode[ lvl ] | ae(v) | 7.4.4.2 |
| } |  |  |
| } |  |  |

    - Defaulted attribute data unit syntax

| defaulted_attribute_data_unit( ) { | Descriptor | Semantics |
| --- | --- | --- |
| defattr_seq_parameter_set_id | u(4) | 7.4.5 |
| defattr_reserved_zero_3bits | u(3) | 7.4.5 |
| defattr_sps_attr_idx | ue(v) | 7.4.5 |
| defattr_slice_id | ue(v) | 7.4.5 |
| for( 𝑐 = 0; 𝑐 < AttrDim; 𝑐++ ) |  |  |
| defattr_value[ 𝑐 ] | u(v) | 7.4.5 |
| byte_alignment( ) |  |  |
| } |  |  |

  - Semantics

    - General

The semantics associated with the syntax structures and with the syntax elements within these structures are specified either in 7.4 or in the subclause identified by the semantics column of the syntax table.

When the semantics of a syntax element are specified in tabular form, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this document.

General constraints on syntax element values are specified in Annex A.

    - Parameter sets, ancillary data and byte alignment

      - Sequence parameter set data unit semantics

        - General

The parameters specified by an SPS shall apply to any DU where that SPS is activated.

simple_profile_compliant specifies whether (when 1) or not (when 0) the bitstream conforms to the Simple profile.

dense_profile_compliant specifies whether (when 1) or not (when 0) the bitstream conforms to the Dense profile.

predictive_profile_compliant specifies whether (when 1) or not (when 0) the bitstream conforms to the Predictive profile.

main_profile_compliant specifies whether (when 1) or not (when 0) the bitstream conforms to the Main profile.

reserved_profile_18bits shall be equal to 0 in bitstreams conforming to this version of this document.  Other values for reserved_profile_18bits are reserved for future use by ISO/IEC.  Decoders shall ignore the value of reserved_profile_18bits.

slice_reordering_constraint specifies whether (when 1) or not (when 0) the bitstream is sensitive to the reordering or removal of slices within a coded point cloud frame.  If slices are reordered or removed when slice_reordering_constraint is 1, the resulting bitstream might not be fully decodable.

unique_point_positions_constraint equal to 1 specifies that in each coded point cloud frame, all points shall have unique positions.  unique_point_positions_constraint equal to 0 specifies that in any coded point cloud frame, two or more points may have the same position.

  - Even if the points in each slice have unique positions, points from different slices in the same frame can be coincident.  In this case, unique_point_positions_constraint would be set to 0.

  - Points with identical positions in the same frame are prohibited when unique_point_positions_constraint is 1 even if they have different values of the frame index/number attribute.

level_idc specifies the level to which the bitstream conforms as specified in Annex A.  Bitstreams shall not contain values of level_idc other than those specified in Annex A.  Other values of level_idc are reserved for future use by ISO/IEC.

sps_seq_parameter_set_id identifies the SPS for reference by other DUs.  sps_seq_parameter_set_id shall be 0 in bitstreams conforming to this version of this document.  Other values of sps_seq_parameter_set_id are reserved for future use by ISO/IEC.

frame_ctr_lsb_bits specifies the length in bits of the syntax element frame_ctr_lsb.

slice_tag_bits specifies the length in bits of the syntax element slice_tag.

bypass_stream_enabled specifies whether bypass bins for arithmetic-coded syntax elements are conveyed in a separate data stream.  When equal to 1, the two data streams are multiplexed using a sequence of fixed-length chunks (11.3).  When equal to 0, bypass bins form part of the arithmetic-coded bitstream.

cross_attr_prediction_enabled specifies whether (when 1) or not (when 0) attribute values shall be coded using correlations across different types of attributes when num_attributes is greater than 1. When cross_attr_prediction_enabled is not present, it shall be inferred to be 0.

bypass_bin_coding_prob_update_disabled specifies whether (when 1) or not (when 0) probability update for coding bypass bins shall be disabled.  When bypass_stream_enabled is 0, bypass_bin_coding_prob_update_disabled shall be applied.  When bypass_bin_coding_prob_update_disabled is not present, it shall be inferred to be 0.

entropy_continuation_enabled specifies whether (when 1) or not (when 0) the entropy parsing of a DU may depend upon the final entropy parsing state of a DU in the preceding slice.  It is a requirement of bitstream conformance that entropy_continuation_enabled shall be 0 when slice_reordering_constraint is 0.

inter_frame_prediction_enabled equal to 1 specifies that inter prediction may be used to derive the positions and attributes in a DU. inter_frame_prediction_enabled equal to 0 specifies that inter prediction is not used to derive the positions and attributes in a DU.

inter_entropy_continuation_enabled specifies whether (when 1) or not (when 0) the entropy parsing of a DU may depend upon the final entropy parsing state of a DU in the preceding frame in bitstream order. When inter_entropy_continuation_enabled is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that inter_frame_prediction_enabled shall be 0 and inter_entropy_continuation_enabled shall be 0 when slice_reordering_constraint is 0.

sps_extension_present specifies whether (when 1) or not (when 0) sps_extension_data syntax elements are present in the SPS syntax structure.  sps_extension_present shall be 0 in bitstreams conforming to this version of this document.  The value of 1 for sps_extension_present is reserved for future use by ISO/IEC.

sps_extension_data may have any value.  Its presence and value do not affect decoder conformance to profiles specified in this version of this document.  Decoders shall ignore all sps_extension_data syntax elements.

fgs_layer_group_enabled equals to 1 specifies that a slice comprises multiple fine granularity slices of partial slice geometry or partial slice attribute.  fgs_layer_group_enabled equals to 0 specifies that a slice is not comprised by fine granularity slices.  When fgs_layer_group_enabled equals to 1, partially decoded occupancy tree can be reconstructed as specified in Annex E.  When fgs_layer_group_enabled is not present, fgs_layer_group_enabled is inferred to 0.

It is a requirement for bitstream conformance that fgs_layer_group_enabled shall be 0 under any of the following conditions:

- geom_tree_type is 1, or

- occtree_coded_axis_list_present is 1, or

- geom_scaling_enabled is 1 and geom_qp_mul_log2 is not 3, or

- geom_angular_enabled is 1, or

- inter_prediction_enabled is 1.

        - Coordinate systems

seq_origin_bits specifies the length in bits of each seq_origin_xyz syntax element exclusive of any sign bit.

seq_origin_xyz[ 𝑘 ] and seq_origin_log2_scale together specify the XYZ origin of the sequence and coding coordinate systems in units of the sequence coordinate system from the application-specific coordinate system origin.  When seq_origin_bits is 0, seq_origin_xyz[ 𝑘 ] and seq_origin_log2_scale shall be inferred to be 0.  The 𝑘-th XYZ component of the origin is specified by the expression SeqOrigin[ 𝑘 ].

SeqOrigin[k] := seq_origin_xyz[k] << seq_origin_log2_scale

seq_bbox_size_bits specifies the length in bits of each seq_bbox_size_minus1_xyz syntax element.

seq_bbox_size_minus1_xyz[ 𝑘 ] plus 1 specifies the 𝑘-th XYZ component of the coded volume dimensions in the sequence coordinate system.  When seq_bbox_size_bits is 0, the coded volume dimensions are unspecified.

seq_unit_numerator_minus1, seq_unit_denominator_minus1 and seq_unit_is_metres together specify the length represented by the unit vectors of the sequence coordinate system.

seq_unit_is_metres equal to 1 specifies that the sequence unit vectors have a length in metres equal to:

SeqUnit∶=seq_unit_numerator_minus1+1seq_unit_denominator_minus1+1 metre

seq_unit_is_metres equal to 0 specifies that the sequence unit vectors have a length relative to the application-specific coordinate system unit vector length, AppUnit, equal to:

SeqUnit∶=seq_unit_numerator_minus1+1seq_unit_denominator_minus1+1 AppUnit

seq_coded_scale_exponent, seq_coded_scale_mantissa_bits and seq_coded_scale_mantissa together specify the scale factor that converts the coding coordinate system to the sequence coordinate system.  The scale factor is represented by the syntax elements as a normalized binary floating-point value that is greater than or equal to 1.  seq_coded_scale_mantisssa_bits specifies the length in bits of the syntax element seq_coded_scale_mantissa.  The scale factor is specified by the expression SeqCodedScale.

SeqCodedScale≔1+seq_coded_scale_mantissa2seq_coded_scale_mantissa_bits×2seq_coded_scale_exponent

geom_axis_order specifies the correspondence between the XYZ axes and the STV axes of the coded point cloud in accordance with Table 8.

Syntax elements ending in "_xyz" are specified using the XYZ axes.  The expression StvToXyz[ 𝑘 ] is the component index of the XYZ axis that corresponds to 𝑘-th STV component.  Values for StvToXyz[ 𝑘 ] are specified for every geom_axis_order in Table 8.

Table 8 — Definition of StvToXyz[ 𝑘 ] according to the value of geom_axis_order

| geom_axis_order | Axis (𝑘) label |  |  | StvToXyz[ 𝑘 ] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 (S) | 1 (T) | 2 (V) | 0 (S) | 1 (T) | 2 (V) |
| 0 or 4 | Z | Y | X | 2 | 1 | 0 |
| 1 or 7 | X | Y | Z | 0 | 1 | 2 |
| 2 | X | Z | Y | 0 | 2 | 1 |
| 3 | Y | Z | X | 1 | 2 | 0 |
| 5 | Z | X | Y | 2 | 0 | 1 |
| 6 | Y | X | Z | 1 | 0 | 2 |

        - Attributes

Attributes are identified by their index into the SPS.

num_attributes specifies the number of attributes enumerated by the SPS attribute list.

The expressions AttrDim, AttrBitDepth and AttrMaxVal specify the number of components, the bit depth and the maximum value respectively of the attribute identified by the variable AttrIdx.  The decoding of an attribute data unit sets AttrIdx.

AttrDim := attr_components_minus1[AttrIdx] + 1

AttrBitDepth := attr_bitdepth_minus1[AttrIdx] + 1

AttrMaxVal := Exp2(AttrBitDepth) − 1

attr_components_minus1[ attrIdx ] plus 1 specifies the number of components of the identified attribute.

  - Attributes with more than three components can only be coded as raw attribute data (attr_coding_type = 3).

attr_instance_id[ attrIdx ] specifies the instance identifier for the identified attribute.

  - The value of attr_instance_id can be used to differentiate between attributes with identical attribute labels.  For example, a point cloud might have multiple colour attributes sampled from different view points.  In this case, attr_instance_id can be used by an application to discriminate between the view points.

attr_bitdepth_minus1[ attrIdx ] plus 1 specifies the bit depth of every component of the identified attribute.

attr_label_known[ attrIdx ], attr_label[ attrIdx ] and attr_label_oid[ attrIdx ] together identify the type of data conveyed by the identified attribute.  attr_label_known[ attrIdx ] specifies whether (when 1) the attribute is an attribute specified in this document by the value of attr_label[ attrIdx ], or (when 0) an externally specified attribute identified by the object identifier attr_label_oid[ attrIdx ].

Attribute types identified by attr_label are specified in Table 9.  It is a requirement of bitstream conformance that an attribute identified by attr_label shall have only as many components as specified as valid.  Values of attr_label not specified are reserved for future use by ISO/IEC.  A decoder should decode attributes with reserved values of attr_label.

Attribute types identified by attr_label_oid are not specified in this document.  attr_label_oid specifies an ASN.1 object identifier value in the international object identifier tree.  The international object identifier shall either be assigned by a registration authority in accordance with Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1 or generated without registration using a universally unique identifier (UUID) as specified by Rec. ITU-T X.667﻿ |‌ ISO/IEC 9834-8.

Table 9 — Identification of attribute type by attr_label

| attr_label | Attribute type | Valid component counts |
| --- | --- | --- |
| 0 | Colour | 1 or 3 |
| 1 | Reflectance | 1 |
| 2 | Opacity | 1 |
| 3 | Frame index | 1 |
| 4 | Frame number | 1 |
| 5 | Material identifier | 1 |
| 6 | Normal vector | 3 |

attr_property_cnt specifies the number of attribute_property syntax structures present in the SPS for the attribute.

      - Attribute property semantics

        - Identification of an attribute property

An attribute_property( attrIdx ) syntax structure specifies a property of the attribute identified by attrIdx.

attr_prop_type specifies the attribute property type according to Table 10.  The interpretation of attribute properties identified as attribute specific are specified in accordance with the registration of attr_label_oid.

Table 10 — Identification of attribute parameter type by attr_prop_type

| attr_prop_type | Description |
| --- | --- |
| 0 | ITU-T T.35 user defined |
| 1 | G-PCC user defined |
| 2 | ISO/IEC 23091-2 video code points |
| 3 | Attribute scale and offset |
| 4 | Default attribute value |
| 5 .. 127 | Reserved for future use by ISO/IEC |
| 128 .. 255 | Attribute specific |

attr_prop_len shall be the length in bytes of the attribute_property syntax structure excluding the syntax elements attr_prop_type and attr_prop_len.

        - ITU-T T.35 user defined attribute properties

ITU-T T.35 user defined properties contain user data registered in accordance with Rec. ITU-T T.35.  The user data are not specified by this document.

attr_prop_itu_t_t35_country_code is a byte having a value specified as a country code by Annex A of Rec. ITU-T T.35.

attr_prop_itu_t_t35_country_code_extension_byte is a byte having a value specified as a country code by Annex B of Rec. ITU-T T.35.

The ITU-T T.35 terminal provider code and terminal provider oriented code shall be contained in the initial bytes of attr_prop_byte[ ], in the format specified by the administration that issued the terminal provider code.  Any remaining attr_prop_byte data shall be data having syntax and semantics as specified by the entity identified by the ITU-T T.35 country code and terminal provider code.

        - G-PCC user defined attribute properties

G-PCC user defined properties contain user data identified by an ASN.1 object identifier.  The user data are not specified by this document.

attr_prop_oid specifies an ASN.1 object identifier value in the international object identifier tree in accordance with Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1.

Any attr_prop_byte data present shall be data having syntax and semantics as specified in accordance with the registration of the object identifier.

        - ISO/IEC 23091-2 video code points

ISO/IEC 23091-2 video code points establish properties of a video representation.

attr_cicp_colour_primaries[ attrIdx ] specifies the chromaticity coordinates of the attribute's colour primaries in accordance with the ColourPrimaries code point in ISO/IEC 23091-2.

attr_cicp_transfer_characteristics[ attrIdx ] specifies, in accordance with the TransferCharacteristics code point in ISO/IEC 23091-2, either the:

- reference opto-electronic transfer characteristic function of the attribute as a function of a source input, linear, optical intensity LC with a nominal real-valued range of 0 to 1; or

- inverse of the reference electro-optical transfer characteristic function as a function of an output, linear, optical intensity LO with a nominal real-valued range of 0 to 1.

attr_cicp_matrix_coeffs[ attrIdx ] describes the matrix coefficients used to derive the attribute's luma and chroma signals from the green, blue and red, or Y, Z and X primaries in accordance with the MatrixCoefficients code point in ISO/IEC 23091-2.

attr_cicp_video_full_range[ attrIdx ] specifies the black level and range of the attribute's luma and chroma signals as derived from EY', EPB' and EPR', or ER', EG' and EB' real-valued component signals in accordance with the VideoFullRangeFlag code point in ISO/IEC 23091-2.

        - Scale and offset properties

Attribute scale and offset parameters specify how to interpret the range of output attribute values.

- The decoding process in this document does not scale attribute values prior to output.

attr_offset_bits is the length in bits of the subsequent attr_offset[ attrIdx ] syntax element exclusive of any sign bit.

attr_scale_bits is the length in bits of the subsequent attr_scale_minus1[ attrIdx ] syntax element.

attr_offset[ attrIdx ], attr_scale_minus1[ attrIdx ] and attr_frac_bits[ attrIdx ] together specify how coded attribute values shall be interpreted.  When present, the external interpretation AEXT of each coded attribute value A shall be:

AEXT=Aattr_scale_minus1+12attr_frac_bits+attr_offset

        - Default attribute value

A default attribute value property specifies the value for an attribute that is not otherwise determined by an ADU.

attr_default_value[ attrIdx ][ 𝑐 ] specifies the default value of the 𝑐-th component of the identified attribute.  The length in bits of each syntax element shall be attr_bitdepth_minus1[ attrIdx ] + 1.

      - Attribute property data semantics

attr_prop_byte[ 𝑖 ] is a byte containing data having syntax and semantics not specified in this document.

      - Tile inventory data unit semantics

A tile inventory, when present, contains metadata that defines the spatial region of each enumerated tile.  Each tile is identified by either an implicit or explicit tile id.

A tile inventory shall apply from the next coded point cloud frame that follows the tile inventory data unit.  It shall remain valid until it is replaced by another tile inventory.

A tile inventory DU shall occur before the first GDU of the coded point cloud frame from which it applies.  It shall not occur before the last DU of any coded point cloud frame that precedes that from which it applies in data unit order.

ti_seq_parameter_set_id identifies the active SPS by its sps_seq_parameter_set_id.

ti_frame_ctr_lsb_bits specifies the length in bits of the syntax element ti_frame_ctr_lsb.  It is a requirement of bitstream conformance that ti_frame_ctr_lsb_bits shall be equal to frame_ctr_lsb_bits of the active SPS.

ti_frame_ctr_lsb should be the ti_frame_ctr_lsb_bits LSBs of FrameCtr for the next coded point cloud frame.

tile_cnt specifies the number of tiles enumerated by the tile inventory.

tile_id_bits specifies the length in bits of each tile_id syntax element.  tile_id_bits equal to 0 specifies that tiles shall be identified by the index tileIdx.

tile_origin_bits_minus1 plus 1 specifies the length in bits of each tile_origin_xyz syntax element exclusive of any sign bit.

tile_size_bits_minus1 plus 1 specifies the length in bits of each tile_size_minus1_xyz syntax element.

tile_id[ tileIdx ] specifies the identifier of the tileIdx-th tile in the tile inventory.  When tile_id_bits is 0, the value of tile_id[ tileIdx ] shall be inferred to be tileIdx.  It is a requirement of bitstream conformance that all values of tile_id shall be unique within a tile inventory.

tile_origin_xyz[ tileId ][ 𝑘 ] and tile_size_minus1_xyz[ tileId ][ 𝑘 ] indicate a bounding box in the sequence coordinate system encompassing slices identified by slice_tag equal to tileId.

tile_origin_xyz[ tileId ][ 𝑘 ] specifies the 𝑘-th XYZ coordinate of the tile bounding box's lower corner relative to the tile inventory origin.

tile_size_minus1_xyz[ tileId ][ 𝑘 ] plus 1 specifies the 𝑘-th XYZ dimension of the tile bounding box.

ti_origin_bits_minus1 plus 1 specified the length in bits of each ti_origin_xyz syntax element exclusive of any sign bit.

ti_origin_xyz[ 𝑘 ] and ti_origin_log2_scale together indicate the XYZ origin of the sequence coordinate system specified by seq_origin_xyz[ 𝑘 ] and seq_origin_log2_scale.  The values of ti_origin_xyz[ 𝑘 ] and ti_origin_log2_scale should be equal to seq_origin_xyz[ 𝑘 ] and seq_origin_log2_scale, respectively.

The tile inventory's 𝑘-th XYZ origin coordinate is specified by the expression TileInventoryOrigin[ 𝑘 ].

TileInventoryOrigin[k] := ti_origin_xyz[k] << ti_origin_log2_scale

      - Geometry parameter set data unit semantics

        - General parameters

The parameters specified by a GPS shall apply to any DU where that GPS is activated.

gps_geom_parameter_set_id identifies the GPS for reference by other DUs.

gps_seq_parameter_set_id identifies the active SPS by its sps_seq_parameter_set_id.

slice_geom_origin_scale_present specifies whether (when 1) or not (when 0) slice_geom_origin_log2_scale is present in the GDU header.  slice_geom_origin_scale_present equal to 0 specifies that the slice origin scale is specified by gps_geom_origin_log2_scale.

gps_geom_origin_log2_scale specifies the scale factor used to derive the slice origin from slice_geom_origin_xyz when slice_geom_origin_scale_present is 0.

geom_dup_point_counts_enabled specifies whether (when 1) or not (when 0) duplicate points can be signalled in a GDU by a per-point duplication count.

- geom_dup_point_counts_enabled equal to 0 does not prohibit the coding of the same point position multiple times within a single slice by means other than the direct_dup_point_cnt, occ_dup_point_cnt or ptn_dup_point_cnt syntax elements.

geom_tree_type equal to 0 specifies that slice geometry is coded using an occupancy tree (7.3.3.4).  geom_tree_type equal to 1 specifies that slice geometry is coded using a predictive tree (7.3.3.8).

gps_extension_present specifies whether (when 1) or not (when 0) gps_extension_data syntax elements are present in the GPS syntax structure.  gps_extension_present shall be 0 in bitstreams conforming to this version of this document.  The value of 1 for gps_extension_present is reserved for future use by ISO/IEC.

gps_extension_data may have any value.  Its presence and value do not affect decoder conformance to profiles specified in this version of this document.  Decoders shall ignore all gps_extension_data syntax elements.

        - Angular coding parameters

geom_angular_enabled specifies whether (when 1) or not (when 0) slice geometry is coded using information about a set of beams located along and rotating around the V axis of the angular origin.  When enabled, point positions are assumed to have been sampled along a ray cast by a beam.

The angular origin AngularOrigin, the apparent V-axis offset BeamOffsetV, the elevation angle 𝜃 of emitted rays and the rotation step angle 𝜑 advanced between ray emissions are illustrated for a single beam in Figure 4.

Figure 4 — Origin, elevation angle and azimuth step for a beam.

slice_angular_origin_present specifies whether (when 1) or not (when 0) a slice-relative angular origin is signalled in the GDU header.  slice_angular_origin_present equal to 0 specifies that the angular origin is gps_angular_origin_xyz.  When slice_angular_origin_present is not present, it shall be inferred to be 0.

gps_angular_origin_bits_minus1 plus 1 specifies the length in bits of each gps_angular_origin_xyz syntax element.

gps_angular_origin_xyz[ 𝑘 ] specifies the 𝑘-th XYZ coordinate of the angular origin in the coding coordinate system.

num_beams_minus1 plus 1 specifies the number of beams enumerated by the GPS.

beam_elevation_init and beam_elevation_diff[ 𝑖 ] together specify beam elevations as gradients above the S-T plane.  The elevation gradient for the 𝑖-th beam is specified by the expression BeamElev[ 𝑖 ].  It is a binary fixed-point value with 18 fractional bits.

BeamElev[i] :=
  i == 0 ? beam_elevation_init :
  i == 1 ? beam_elevation_init + beam_elevation_diff[1]
        : 2 × BeamElev[i – 1] – BeamElev[i − 2] + beam_elevation_diff[i]

It is a requirement of bitstream conformance that values of BeamElev[ 𝑖 ], 𝑖 ∈ 1 .. num_beams_minus1, shall be greater than BeamElev[ 𝑖 – 1 ].

beam_voffset_init and beam_voffset_diff[ 𝑖 ] together specify the V-axis offsets of the enumerated beams from the angular origin.  The offset is specified in units of the coding coordinate system.  The offset for the 𝑖-th beam is specified by the expression BeamOffsetV[ 𝑖 ].

BeamOffsetV[i] :=
  i == 0 ? beam_voffset_init
        : BeamOffsetV[i − 1] + beam_voffset_diff[i]

beam_steps_per_rotation_init_minus1 and beam_steps_per_rotation_diff[ 𝑖 ] specify the number of steps made per revolution by the rotating beams.  The value for the 𝑖-th beam is specified by the expression BeamStepsPerRev[ 𝑖 ].

BeamStepsPerRev[i] :=
  i == 0 ? beam_steps_per_rotation_init_minus1 + 1
        : BeamStepsPerRev[i – 1] + beam_steps_per_rotation_diff[i]

It is a requirement of bitstream conformance that values of BeamStepsPerRev[ 𝑖 ], 𝑖 ∈ 0 .. num_beams_minus1, shall not be 0.

ptree_ang_azimuth_pi_bits_minus11 plus 11 specifies the number of bits that represent half a turn of a beam around the V axis.  One half-turn is 𝜋 radians.

ptree_ang_radius_scale_log2 specifies a factor used to scale a point's radial angular coordinate during conversion to Cartesian coordinates.

ptree_ang_azimuth_step_minus1 plus 1 specifies the expected change in azimuth angle of the rotating beams between coded points.  Azimuth prediction residuals used in angular predictive tree coding can be coded as a multiple of ptree_ang_azimuth_step_minus1 + 1 and a remainder.

occtree_angular_extension_enabled specifies whether (when 1) or not (when 0) angular coding extension is enabled with occupancy tree coding. When occtree_angular_extension_enabled is not present, it shall be inferred to be 0.

octree_planar_neigh_prediction_enabled specifies whether (when 1) or not (when 0) neighbour prediction coding is enabled with planar occupancy coding. When octree_planar_neigh_prediction_enabled is not present, it shall be inferred to 0.

geo_planar_idcm_angular_disabled equal to 1 specifies that if the geom_angular_enabled is equal to 1, the planar mode is disabled for the IDCM coded nodes. If geom_angular_enabled is equal to 0, geo_planar_idcm_angular_disabled is set equal to 0. When geo_planar_idcm_angular_disabled is equal to 1, occ_direct_node should be derived before deciding per axis planar eligibility.

ptree_sec_resid_disabled specifies whether (when 1) or not (when 0) second coordinate-prediction residual coding is disabled. When ptree_sec_resid_disabled is not present, it shall be inferred to be 0.

ptree_ang_azimuth_scaling_enabled specifies whether (when 1) or not (when 0) predictive geometry azimuth angle residuals shall be scaled according to the radius. The effect is to provide an adaptive quantization step size of the predictive geometry azimuth angle residuals resulting in a uniform quantization of circular arcs for any radius. When ptree_ang_azimuth_scaling_enabled is not present, it shall be inferred to be 0.

ptree_ang_max_pred_index specifies the maximum predictor index being usable in the prediction list for angular coordinates.

ptree_ang_pred_list_radius_resid_threshold specifies a threshold value on the absolute value of predictive geometry radius residual. This threshold is used during the dynamic update process of the prediction list for angular coordinates (9.3.3.7).

ptree_ang_radius_resid_context_qphi_threshold_present specifies whether (when 1) or not (when 0) the threshold of the number of azimuthal angle steps used in selecting the context of the decoding radius residual is present in ptree_ang_redius_resid_context_qphi_threshold. When ptree_ang_radius_resid_context_qphi_threshold_present is not present, it shall be inferred to be 0.

ptree_ang_redius_resid_context_qphi_threshold specifies the threshold of the number of azimuthal angle steps used in selecting the context of the decoding radius residual. When ptree_ang_redius_resid_context_qphi_threshold is not present, it shall be inferred to be 0.

The value thQphi specify the threshold used in the table of values of CtxTbl and CtxIdx for binarized ae(v) coded GDU syntax elements (11.5.3.4).

thQphi := ptree_ang_redius_resid_context_qphi_threshold

        - Occupancy tree parameters

occtree_point_cnt_list_present specifies whether (when 1) or not (when 0) the GDU footer enumerates the number of points in each occupancy tree level.  When occtree_point_cnt_list_present is not present, it shall be inferred to be 0.

occtree_direct_coding_mode greater than 0 specifies that point positions may be coded by eligible direct nodes of the occupancy tree.  occtree_direct_coding_mode equal to 0 specifies that direct nodes shall not be present in the occupancy tree.

- Larger values for occtree_direct_coding_mode generally increase the rate of direct node eligibility.

occtree_direct_joint_coding_enabled specifies whether (when 1) or not (when 0) direct nodes that code two points shall jointly code their positions according to a specific ordering of the points.

occtree_coded_axis_list_present equal to 1 specifies that the GDU header contains occtree_coded_axis syntax elements that are used to derive the node size for each occupancy tree level.  occtree_coded_axis_list_present equal to 0 specifies that occtree_coded_axis syntax elements are not present in the GDU syntax and that the occupancy tree represents a cubic volume specified by the tree depth.

occtree_neigh_window_log2_minus1 plus 1 specifies the number of occupancy tree node locations that form each availability window within a tree level.  Nodes outside a window are unavailable to any process related to nodes within the window.  occtree_neigh_window_log2_minus1 equal to 0 specifies that only sibling nodes shall be considered available to the current node.

occtree_adjacent_child_enabled specifies whether (when 1) or not (when 0) the adjacent children of neighbouring occupancy tree nodes are used in bitwise occupancy contextualization.  When occtree_adjacent_child_enabled is not present, it shall be inferred to be 0.

occtree_intra_pred_max_nodesize_log2 minus 1 specifies the maximum size of an occupancy tree node that is eligible for intra-slice occupancy prediction.  When occtree_intra_pred_max_nodesize_log2 is not present, it shall be inferred to be 0.

occtree_bitwise_coding specifies whether the node occupancy bitmap is coded using (when 1) occupancy_bit syntax elements or (when 0) the dictionary coded syntax element occupancy_byte.

occtree_planar_enabled specifies whether (when 1) or not (when 0) the coding of node occupancy bitmaps is performed, in part, by the signalling of occupied and unoccupied planes.  When occtree_planar_enabled is not present, it shall be inferred to be 0.

occtree_planar_threshold[ 𝑖 ] specify thresholds used in part to determine the per-axis eligibility for planar occupancy coding.  The thresholds are specified from the most (𝑖 = 0) to the least (𝑖 = 2) probable planar axis.  Each threshold specifies the minimum likelihood for an eligible axis that occ_single_plane is expected to be 1.  The range [ 8, 120 ] for occtree_planar_threshold corresponds to the likelihood interval [ 0, 1 ).

occtree_direct_node_rate_minus1 specifies, when present, that of every 32 eligible nodes, only occtree_direct_node_rate_minus1 + 1 are permitted to be coded as direct nodes.

occtree_planar_buffer_disabled specifies whether (when 1) or not (when 0) the contextualization of per-node occupied plane locations using the plane locations of previously coded nodes shall be disabled.  When occtree_planar_buffer_disabled is not present, it shall be inferred to be 0.

        - Scaling parameters

geom_scaling_enabled specifies whether (when 1) or not (when 0) the coded geometry shall be scaled during the geometry decoding process.

geom_qp specifies the geometry QP prior to the addition of per slice and per-node offsets.

geom_qp_mul_log2 specifies the scale factor to be applied to the geometry QP.  There are Exp2( 3 − geom_qp_mul_log2 ) QP values for every doubling of the scaling step size.

ptree_qp_period_log2 specifies the period in nodes at which the predictive tree node QP offset is signalled.  The period is one in every Exp2( ptree_qp_period_log2 ) nodes.

occtree_direct_node_qp_offset specifies an offset relative to the slice geometry QP for scaling direct node coded point positions.

        - Inter prediction parameters

inter_prediction_enabled specifies whether (when 1) or not (when 0) inter prediction may be used to code the points of the point cloud. When inter_prediction_enabled is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that when inter_frame_prediction_enabled is 0, inter_prediction_enabled shall be 0.

biprediction_enabled specifies whether (when 1 or 2) or not (when 0) bi-prediction may be used to code the points of the point cloud. When biprediction_enabled is not present, it shall be inferred to be 0.

frame_merge_enabled specifies whether (when 1) or not (when 0) the two reference frames of bi-prediction may be merged into a merged reference frame. When frame_merge_enabled is not present, it shall be inferred to be 0.

global_motion_enabled specifies whether (when 1) or not (when 0) global motion compensation is applied to the reference frame used for inter prediction. When global_motion_enabled is not present, it shall be inferred to be 0.

inter_azim_scale_log2 specifies a scale factor to be applied to azimuth coordinates used to obtain azimuth look-up values during inter prediction. The values MaxQAzim and MinQAzim specify the maximum and minimum azimuth look up values.

MaxQAzim [i] := 1 << ptree_ang_azimuth_pi_bits_minus11 + 11 – inter_azim_scale_log2 - 1

MinQAzim [i] := -(1 << ptree_ang_azimuth_pi_bits_minus11 + 11 – inter_azim_scale_log2)

resampling_enabled specifies a whether (when 1) or not (when 0) the radii of one or more points in a reference frame is updated using motion parameters.

max_points_per_entry_minus1 plus one specifies the maximum number of points of a reference frame that may be stored for a given laser ID and azimuth look up value derived using inter_azim_scale_log2.

down_sampling_range specifies the range of downsampling a reference frame. The syntax down_sampling_range may be present only when max_points_per_entry_minus1 is greater than to 0. When down_sampling_range is not present, it shall be inferred to be -1.

occtree_inter_angular_direct_coding_enabled specifies whether (when 1) or not (when 0) use angular information to determine the eligibility for direct coding of the occupancy tree nodes. When occtree_inter_angular_direct_coding_enabled is not present, it shall be inferred to be 0.

        - TriSoup parameters

trisoup_enabled specifies whether (when 1) or not (when 0) an occupancy tree (7.3.3.4) used for coding slice geometry is followed by TriSoup (7.3.3.10). When not present, it shall be inferred to 0.

trisoup_non_cubic_node_start_edge_present and trisoup_non_cubic_node_end_edge_present  together specifies whether (when one or both are 1) or not (when both are 0) the location TriSoupNodeLoc[ nodeIdx ][ k ] and the edge length TriSoupNodeSize[ nodeIdx ][ k ] of the nodeIdx-th TriSoup node are modified as specified in 9.4.1.1.

      - Attribute parameter set data unit semantics

        - General parameters

The parameters specified by an APS shall apply to any DU where that APS is activated.

- A single APS can be used by multiple coded attributes.  The attributes are not required to be of the same type or to have the same number of components.

aps_attr_parameter_set_id identifies the APS for reference by other DUs.

aps_seq_parameter_set_id identifies the active SPS by its sps_seq_parameter_set_id.

attr_coding_type specifies the attribute coding method.  Valid values are specified by Table 11.  Other values are reserved for future use by ISO/IEC.  Decoders conforming to this version of this document shall ignore (remove from the bitstream and discard) attribute data units coded with reserved values of attr_coding_type.

Table 11 — Interpretation of attr_coding_type

| attr_coding_type | Description | Decoding process |
| --- | --- | --- |
| 0 | Region Adaptive Hierarchical Transform (RAHT) | 10.5 |
| 1 | LoD with Predicting Transform | 10.6 |
| 2 | LoD with Lifting Transform | 10.6 |
| 3 | Raw attribute data | 10.3 |

attr_primary_qp_minus4 plus 4 specifies the QP for the primary attribute component before the addition of per slice, per region and per-transform-level offsets.

attr_secondary_qp_offset specifies an offset to be applied to the primary attribute QP to derive the QP for any secondary attribute components.

cross_attr_prediction_enabled_this_type specifies whether (when 1) or not (when 0) the cross-attribute prediction is enabled for coding the current attribute if cross_attr_prediction_enabled is 1. When cross_attr_prediction_enabled_this_type is not present, it shall be inferred to be 0.

refAttrIdx specifies the index of attribute identified by its attrIdx that is used for decoding the current attribute. It shall range from 0 to num_attributes - 1 when cross_attr__prediction_enabled_this_type is 1. When refAttrIdx is not present, it shall be inferred to be - 1.

attr_qp_offsets_present specifies whether (when 1) or not (when 0) per-slice attribute QP offsets, attr_qp_offset[ 𝑐 ], are present in the ADU header.

attr_coord_conv_enabled specifies whether (when 1) attribute coding shall use scaled angular coordinates or (when 0) slice-relative STV point positions.  It is a requirement of bitstream conformance that attr_coord_conv_enabled shall be 0 when geom_angular_enabled is 0.  When attr_coord_conv_enabled is not present, it shall be inferred to be 0.

attr_coord_conv_scale_bits_minus1[ 𝑘 ] plus 1 specifies the length in bits of the syntax element attr_coord_conv_scale[ 𝑘 ].

attr_coord_conv_scale[ 𝑘] specifies the scale factor used to scale points' 𝑘-th angular coordinate for attribute coding.  The scale factor shall be in units of 2-8.

aps_extension_present specifies whether aps_extension_data syntax elements are present in the APS syntax structure.  aps_extension_present shall be 0 in bitstreams conforming to this version of this document.  The value of 1 for aps_extension_present is reserved for future use by ISO/IEC.

aps_extension_data may have any value.  Its presence and value do not affect decoder conformance to profiles specified in this version of this document.  Decoders shall ignore all aps_extension_data syntax elements.

        - Region adaptive hierarchical transform parameters

raht_prediction_enabled specifies whether (when 1) or not (when 0) RAHT coefficients are predicted by upsampling and transforming the preceding coarser transform level.

raht_prediction_subtree_min and raht_prediction_samples_min specify thresholds that control the use of RAHT coefficient prediction.

raht_prediction_samples_min specifies the minimum number of spatially adjacent samples from which RAHT coefficient prediction can be performed.

raht_prediction_subtree_min specifies the minimum number of spatially adjacent samples that need to be present to prevent the disabling of RAHT coefficient prediction for every descendant of a RAHT node.

lossless_coding_enabled specifies whether (when 1) or not (when 0) lossless coding is applied by using Haar Transform.  When lossless_coding_enabled is not present, it shall be inferred to be 0.

raht_subnode_prediction_enabled specifies whether (when 1) or not (when 0) the adjoining blocks are replaced by their children blocks when generate the upsampled prediction block.  When raht_subnode_prediction_enabled is not present, it shall be inferred to be 0.

raht_intra_layer_code_enabled specifies whether (when 1) or not (when 0) per-RAHT-layer AC coding mode is present in the ADU.  It is a requirement of bitstream conformance that raht_intra_layer_code_enabled shall be 0 when raht_prediction_enabled is 0.  When raht_intra_layer_code_enabled is not present, it shall be inferred to be 0.

raht_prediction_weights[ i ] specifies the prediction weights applied to the normalized DC value of the blocks in transform domain prediction. raht_prediction_weights[ 0…4] specify the prediction weights for the co-located block, blocks that adjoin by a face, blocks that adjoin by only an edge, child blocks that adjoin by a face and child blocks that adjoin by only an edge, respectively.

raht_buffer_extension_flag specifies whether (when 1) or not (when 0) to use the extended reconstruction buffer for upsampling the attributes.  When equal to 1, the attributes of previous layers are stored without rounding.  When equal to 0, the attributes of previous layers are rounded prior to be stored to the buffer.  When raht_buffer_extension_flag is not present, it shall be inferred to be 0.

raht_prediction_search_range specifies the range of Morton code in the preceding coarser transform level for which searched for spatially adjacent samples.  When raht_prediction_search_range is not present, it shall be inferred to be MaxSlicePoints – 1.

raht_last_comp_pred_enabled specifies whether (when 1) or not (when 0) the second coefficient component of a three-component attribute shall be used to predict the value of the third coefficient component when RAHT is used for attribute coding.  When raht_last_comp_pred_enabled is not present, it shall be inferred to be 0.

        - Level of detail generation and transform parameters

pred_set_size_minus1 plus 1 specifies the maximum size of the per-point predictor set.

pred_inter_lod_search_range specifies the range of indexes around a search centre which can be searched in an extended inter-detail-level search for nearest neighbours to include in a point's predictor set.

pred_dist_bias_minus1_xyz[ 𝑘 ] plus 1 specifies the factor used to weight the 𝑘-th XYZ component of the distance vector between two point positions used to calculate inter-point distances in the predictor search for a single refinement point.  The expression PredBias[ 𝑘 ] specifies the factor for the 𝑘-th STV component.

PredBias[k] := pred_dist_bias_minus1_xyz[StvToXyz[k]] + 1

last_comp_pred_enabled specifies whether (when 1) or not (when 0) the second coefficient component of a three-component attribute shall be used to predict the value of the third coefficient component.  When last_comp_pred_enabled is not present, it shall be inferred to be 0.

lod_scalability_enabled specifies whether (when 1) or not (when 0) attribute values shall be coded using constrained LoD generation and predictor searches.  When equal to 1, attribute values can be reconstructed for a partially decoded occupancy tree as specified in Annex D.  When lod_scalability_enabled is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that lod_scalability_enabled shall be 0 when any of the following conditions are true:

- geom_tree_type is 1, or

- occtree_coded_axis_list_present is 1, or

- geom_scaling_enabled is 1 and geom_qp_mul_log2 is not 3, or

- pred_blending_enabled is 1.

pred_max_range_minus1 plus 1 specifies, when present, the distance beyond which point predictor candidates shall be discarded during predictor set pruning for scalable attribute coding.  The distance is specified in units of the per-detail-level block size.

lod_max_levels_minus1 plus 1 specifies the maximum number of detail levels that can be generated by the LoD generation process.  When lod_max_levels_minus1 is not present, it shall be inferred to be MaxSliceDimLog2 − 1.

attr_canonical_order_enabled specifies whether (when 1) or not (when 0) the order in which point attributes are coded is the canonical order that points are output by the geometry decoding processes specified in this document.  When attr_canonical_order_enabled is not present, it shall be inferred to be 0.

lod_decimation_mode specifies the decimation method used to generate detail levels.  Valid values are specified by Table 12.  Other values are reserved for future use by ISO/IEC.  Decoders conforming to this version of this document shall ignore (remove from the bitstream and discard) attribute data units coded with reserved values of lod_decimation_mode.

Table 12 — Interpretation of lod_decimation_mode

| lod_decimation_mode | Description | Decoding process |
| --- | --- | --- |
| 0 | No decimation | 10.6.5.6 |
| 1 | Periodic subsampling | 10.6.5.5 |
| 2 | Block-based subsampling | 10.6.5.8 |

lod_sampling_period_minus2[ lvl ] plus 2 specifies the sampling period used by LoD generation to sample points in detail level lvl to generate the next coarser detail level lvl + 1.

lod_initial_dist_log2 specifies the block size at the finest detail level for use by LoD generation and predictor searches.  When lod_initial_dist_log2 is not present, it shall be inferred to be 0.

lod_dist_log2_offset_present specifies whether (when 1) or not (when 0) the per-slice block-size offset specified by lod_dist_log2_offset shall be present in the ADU header.  When lod_dist_log2_offset_present is not present, it shall be inferred to be 0.

pred_direct_max_idx_plus1 specifies the maximum number of single point predictors that can be used for direct prediction.

pred_direct_threshold specifies when a point shall be eligible for direct prediction.  The threshold is for the maximum difference between predictor values in a point's predictor set.  When the maximum difference is greater than or equal to the threshold, direct prediction is eligible.  When the attribute bit depth is greater than eight bits, the threshold shall be scaled by Exp2( AttrBitDepth − 8 ).

pred_direct_avg_disabled specifies whether (when 0) or not (when 1) the point predictor set average is a direct prediction mode.

pred_intra_lod_search_range specifies the range of indexes in a detail level's refinement list for which searched for nearest neighbours to include in a point's predictor set.

pred_intra_min_lod specifies the finest detail level in which intra-detail-level prediction is enabled.  When pred_intra_min_lod is not present,  it shall be inferred to be lod_max_levels_minus1 + 1.  It is a requirement of bitstream conformance that pred_intra_min_lod shall be 0 when lod_max_levels_minus1 is 0.

inter_comp_pred_enabled specifies whether (when 1) or not (when 0) the first component of a multi-component attribute coefficient shall be used to predict the coefficients of any subsequent components.  When inter_comp_pred_enabled is not present, it shall be inferred to be 0.

pred_blending_enabled specifies whether (when 1) or not (when 0) the neighbour weights used for neighbourhood average prediction shall be blended according to the relative spatial positions of the associated points.  When pred_blending_enabled is not present, it shall be inferred to be 0.

quant_neigh_weight[k] specifies the weights of k-nearest neighbor points used by point quantization weights derivation.  When quant_neigh_weight[k] is not present, it shall be inferred to be 0.

max_points_per_sort_log2_plus1 minus 1 specifies the max group size that points are ordered by group before point attributes are coded.  When max_points_per_sort_log2_plus1 is equal to 0, the max group size is equal to the points number in slice.  When max_points_per_sort_log2_plus1 is not present, it shall be inferred to be 0.

prediction_with_distribution_enabled specifies whether (when 1) or not (when 0) prediction coefficients are derived based on the spatial distribution of the predictors.  When prediction_with_distribution_enabled is not present, it shall be inferred to be 0.

        - Raw attribute parameters

raw_attr_width_present specifies whether (when 0) raw attribute values shall use the same fixed length encoding for every syntax element or (when 1) a per-syntax-element length.

        - Attribute inter prediction parameters

attr_inter_prediction_enabled specifies whether (when 1) or not (when 0) inter prediction may be used to code the attributes of the point cloud.  When attr_inter_prediction_enabled is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that when inter_frame_enabled_flag is 0 or inter_prediction_enabled is 0, attr_inter_prediction_enabled shall be 0.

raht_inter_layer_depth_minus1 +1 specifies the number of layers of the transform tree where the inter prediction block (10.5.5.9) is used to modify the prediction block.  When raht_inter_layer_depth_minus1 is not present, it shall be inferred to be 0.

raht_send_inter_filters specifies whether (when 1) or not (when 0) RAHT inter-prediction (temporal) filters will be sent to the decoder. When raht_send_inter_filters is not present, it shall be inferred to be 0.

raht_inter_skip_layers specifies the number of layers of the transform tree for which the inter prediction block is skipped from the temporal filtering process (10.5.6.4).  When raht_inter_skip_layers is not present, it shall be inferred to be 0.

raht_inter_layer_code_enabled specifies whether (when 1) or not (when 0) per-RAHT-layer AC coding mode is present in the ADU.  It is a requirement of bitstream conformance that raht_inter_layer_code_enabled shall be 0 when attr_inter_prediction_enabled is 0.  When raht_inter_layer_code_enabled is not present, it shall be inferred to be 0.

attr_inter_prediction_search_range specifies the range of indexes in a detail level's refinement list in the reference slice for which searched for nearest neighbours to include in a point's predictor set.  When attr_inter_prediction_search_range is not present, it shall be inferred to be 0.

      - Frame-specific attribute properties data unit semantics

Frame-specific attribute properties apply to an attribute of a specific frame.  The properties shall:

- override any corresponding properties signalled in the active SPS for the specified frame only;

- apply to all ADUs in the frame with AttrIdx equal to fsap_sps_attr_idx.

All attribute properties with the same value of attr_prop_type shall be identical within a frame for any single attribute.

Each FSAP DU shall occur, at least, before the first ADU within the frame to which it applies.

- The requirements of fsap_frame_ctr_lsb prevent an FSAP DU from preceding the first GDU in the frame to which it applies.

fsap_seq_parameter_set_id identifies the active SPS by its sps_seq_parameter_set_id.

fsap_frame_ctr_lsb_bits specifies the length in bits of the syntax element fsap_frame_ctr_lsb.  It is a requirement of bitstream conformance that fsap_frame_ctr_lsb_bits shall be equal to frame_ctr_lsb_bits of the active SPS.

fsap_frame_ctr_lsb identifies the frame to which the frame-specific attribute properties apply.  Identification shall use fsap_frame_ctr_lsb_bits LSBs of the notional frame counter, FrameCtr.  fsap_frame_ctr_lsb shall be equal to frame_ctr_lsb of the preceding GDU.

fsap_sps_attr_idx identifies the coded attribute to which the frame-specific attribute properties shall apply.  Identification shall be by the index into the active SPS attribute list.

fsap_num_props specifies the number of attribute properties present in the syntax structure.

      - Frame boundary marker data unit semantics

The frame boundary marker DU explicitly marks the end of a frame.

fbdu_frame_ctr_lsb_bits specifies the length in bits of the syntax element fbdu_frame_ctr_lsb.  It is a requirement of bitstream conformance that fbdu_frame_ctr_lsb_bits shall be equal to frame_ctr_lsb_bits of the active SPS.

fbdu_frame_ctr_lsb identifies the frame to which the frame boundary marker applies.  Identification shall use fbdu_frame_ctr_lsb_bits LSBs of the notional frame counter FrameCtr.

      - User data data unit semantics

The user data DU contains user data identified by an ASN.1 object identifier.  The user data are not specified by this document.

user_data_oid specifies an ASN.1 object identifier value in the international object identifier tree, as specified in Rec. ITU-T X.660﻿ |‌ ISO/IEC 9834-1.

user_data_byte is a byte containing data having syntax and semantics as specified by the registration of the object identifier.

      - Byte alignment semantics

The byte_alignment syntax structure causes the bitstream to become byte-aligned.

alignment_bit_equal_to_zero shall be 0.

    - Geometry data unit

      - Geometry data unit semantics

A GDU conveys the geometry of a slice and associated slice information such as a frame counter or a slice origin.  A GDU comprises a GDU header, geometry coded using either an occupancy tree (when geom_tree_type is 0) or a predictive tree (when geom_tree_type is 1), and a GDU footer.

      - Geometry data unit header semantics

gdu_geometry_parameter_set_id specifies the active GPS by its gps_geom_parameter_set_id.

gdu_temporal_id specifies the temporal ID of the frame associated with the geometry data unit.

slice_id identifies the slice for reference by other DUs.

slice_tag identifies the slice as a member of a slice group with the same values for slice_tag.  When a tile inventory DU is present, the slice group shall be a tile identified by a tile id.  Otherwise, when tile inventory DUs are not present, the interpretation of slice_tag is application specific.

frame_ctr_lsb specifies the frame_ctr_lsb_bits LSBs of the notional frame counter FrameCtr.  Consecutive slices with different values of frame_ctr_lsb form parts of separate output point cloud frames.  Consecutive slices with identical values of frame_ctr_lsb without an intervening frame boundary marker data unit form parts of the same coded point cloud frame.

slice_entropy_continuation equal to 1 specifies that the entropy parsing state restoration process (11.6.2.2 and 11.6.3.2) shall be applied at the start of the GDU and any ADUs in the slice.  slice_entropy_continuation equal to 0 specifies that the parsing of the GDU and any ADUs in the slice is independent of any other slice in the frame when slice_inter_entropy_continuation is 0.  When slice_entropy_continuation is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that slice_entropy_continuation shall be 0 when the GDU is the first GDU in a coded point cloud frame.  A decoder shall ignore (remove from the bitstream and discard) all slices in a coded point cloud frame with slice_entropy_continuation equal to 1 that are not preceded by a slice in the same frame with slice_entropy_continuation equal to 0.

prev_slice_id shall be equal to the GDU slice_id of the preceding slice in bitstream order.  A decoder shall ignore (remove from the bitstream and discard) slices where prev_slice_id is both present and not equal to slice_id of the preceding slice in the same frame.

- It is recommended that slice_entropy_continuation is 0 if slice_tag is not equal to the slice_tag of the GDU identified by prev_slice_id.  For example, if slice_tag is used to select a subset of slices, then decoding might be prevented if there are dependencies upon slices that were not selected.

slice_geom_origin_bits_minus1 plus 1 specifies the length in bits of each slice_geom_origin_xyz syntax element.

slice_geom_origin_xyz[ 𝑘 ] and slice_geom_origin_log2_scale specify the 𝑘-th XYZ coordinate of the slice origin in the coding coordinate system.  The slice origin in STV coordinates is specified by the expression SliceOrigin[ 𝑘 ].  When slice_geom_origin_log2_scale is not present, it shall be inferred to be gps_geom_origin_log2_scale.

SliceOrigin[k] := slice_geom_origin_xyz[StvToXyz[k]] << Max(slice_geom_origin_log2_scale, triSoupNodeSizeLog2)

slice_angular_origin_bits_minus1 plus 1 specifies the length in bits of each slice_angular_origin_xyz syntax element.

slice_angular_origin_xyz[ 𝑘 ] specifies the 𝑘-th XYZ coordinate of the angular origin relative in the slice's coordinate system.  When slice_angular_origin_xyz[ 𝑘 ] is not present, it shall be inferred to be 0.

The slice-relative angular origin in STV coordinates is specified by the expression AngularOrigin[ 𝑘 ].

AngularOrigin[k] := slice_angular_origin_present
  ? slice_angular_origin_xyz[StvToXyz[k]]
  : gps_angular_origin_xyz[StvToXyz[k]] − SliceOrigin[k]

slice_geom_qp_offset specifies the slice geometry QP as an offset to the GPS geom_qp.  When slice_geom_qp_offset is not present, it shall be inferred to be 0.

slice_inter_prediction equal to 1 specifies that inter prediction may be used to derive the positions in the GDU. slice_inter_prediction equal to 0 specifies that inter prediction is not used to derive the positions in the GDU.  When slice_inter_prediction is not present, it shall be inferred to be 0.

slice_biprediction equal to 1 specifies that bi-prediction may be used to derive the positions in the GDU. slice_biprediction equal to 0 specifies that bi-prediction is not used to derive the positions in the GDU.  When slice_biprediction is not present, it shall be inferred to be 0.

gm_matrix[ i ][ j ] and gm_trans[ i ] specify the motion compensation parameters in the form of a matrix and an offset that is to be applied when global motion is enabled.  When gm_matrix[ i ][ j ] is not present, it is inferred to be 0. When gm_trans[ i ] is not present, it is inferred to be 0.  The global motion matrix is specified by the expression GMMatrix[ i ][ j ].

GMMatrix[i][j] := (i == j ? 65536 : 0) + gm_matrix[i][j]

gm_thres_top and gm_thres_bot specify the two thresholds used to determine the points to which motion compensation is applied.

slice_inter_frame_ref_gmc specifies whether (when 1) or not (when 0) the inter prediction GMC reference frame to be applied to the GDU unit associated with the GDU header as specified by 9.3.3.11.  When slice_inter_frame_ref_gmc is not present, it shall be inferred to be 0.

gm_matrix2[ i ][ j ] and gm_trans2[ i ] specify the motion compensation parameters in the form of a matrix and an offset that is to be applied to the second reference frame when global motion is enabled.  When gm_matrix2[ i ][ j ] is not present, it is inferred to be 0.  When gm_trans2[ i ] is not present, it is inferred to be 0. The global motion matrix is specified by the expression GMMatrix2[ i ][ j ].

GMMatrix2[i][j] := (i == j ? 65536 : 0) + gm_matrix2[i][j]

gm_thres_top2 and gm_thres_bot2 specify the two thresholds used to determine the points of the second reference frame to which motion compensation is applied.

slice_inter_frame_ref_gmc2 specifies the second inter prediction GMC reference frame to be applied to the GDU unit associated with the GDU header as specified by 9.3.3.11.  When slice_inter_frame_ref_gmc2 is not present, it shall be inferred to be 0.

motion_partition_type equal to 0 specifies that road and object partitioning shall be used to determine the points for which global motion compensation is to be applied.  motion_partition_type equal to 1 specifies that cuboid partitioning shall be used to determine the points for which global motion compensation is to be applied.

The variable MotionPartitionType is set equal to -1 when motion_partition_type is not present and set equal to motion_partition_type when present.

motion_zero_origin equal to 1 specifies that the origin position {0, 0, 0} shall be used in global motion compensation using cuboid partitioning.  motion_zero_origin equal to 0 specifies that the origin position seq_origin_xyz shall be used in global motion compensation using cuboid partitioning.

motion_block_size[k] specifies the partition block size that is to be used in global motion compensation using cuboid partitioning.  When motion_block_size[k] is equal to 0, the block size in the k-th dimension is equal to the size the k-th dimension of the slice bounding box.

The number of motion partition blocks in the k-th axis is specified by the expression NumMotionBlocksPerAxis[k].

NumMotionBlocksPerAxis[k] := motion_block_size[k] 
        ? (PredPointCloudBboxSize[k] + motion_block_size[k] – 1)/ motion_block_size[k] : 1

The number of motion partition blocks is specified by the expression NumMotionBlocks.

NumMotionBlocks := NumMotionBlocksPerAxis[0] × NumMotionBlocksPerAxis[1] × NumMotionBlocksPerAxis[2]

slice_inter_entropy_continuation equal to 1 specifies that the entropy parsing state restoration process (11.6.2.2 and 11.6.3.2) shall be applied at the start of the GDU and any ADUs in the slice.  slice_inter_entropy_continuation equal to 0 specifies that the parsing of the GDU and any ADUs in the slice is independent of any other slice when slice_entropy_continuation is 0.  When slice_inter_entropy_continuation is not present, it shall be inferred to be 0.

It is a requirement of bitstream conformance that slice_inter_entropy_continuation shall be 0 when the GDU is the not the first GDU in a coded point cloud frame.

prev_inter_entropy_frame_ctr_lsb shall be equal to the frame_ctr_lsb of the preceding frame in bitstream order.  A decoder shall ignore (remove from the bitstream and discard) slices where prev_inter_entropy_frame_ctr_lsb is both present and not equal to frame_ctr_lsb of the preceding frame in bitstream order.  The number of bits used to code prev_inter_entropy_frame_ctr_lsb is frame_ctr_lsb_bits

prev_inter_entropy_slice_id shall be equal to the GDU slice_id of the preceding slice of the preceding frame in bitstream order.  A decoder shall ignore (remove from the bitstream and discard) slices where prev_inter_entropy_slice_id is both present and not equal to slice_id of the preceding slice in the preceding frame in bitstream order.

- It is recommended that slice_inter_entropy_continuation is 0 if slice_tag is not equal to the slice_tag of the GDU identified by prev_inter_entropy_slice_id of the preceding frame.  For example, if slice_tag is used to select a subset of slices, then decoding might be prevented if there are dependencies upon slices that were not selected.

num_subsequent_subgroups specifies the number of the subsequent dependent data units which reference the context state of the current data unit.

      - Geometry data unit footer semantics

The start of the GDU footer shall be determined from the end of the GDU as specified by 11.2.4.

slice_num_points_minus1 plus 1 specifies the number of points coded in the DU.  It is a requirement of bitstream conformance that slice_num_points_minus1 plus 1 shall be equal to the number of decodable points in the DU.  Decoders shall not rely upon bitstream conformance to prevent overflow of implementation buffers.

    - Attribute data unit

      - Attribute data unit semantics

An ADU codes attribute values for a single attribute in a slice.  It comprises an ADU header and either attribute coefficients (attribute_coeffs) when transform coding is enabled or directly coded attribute values (attribute_raw).

      - Attribute data unit header semantics

adu_attr_parameter_set_id specifies the active APS by its aps_attr_parameter_set_id.

adu_temporal_id specifies the temporal ID of the frame associated with the attribute data unit.

adu_sps_attr_idx identifies the coded attribute by its index into the active SPS attribute list.

At the start of every ADU, the variable AttrIdx is set to adu_sps_attr_idx:

AttrIdx = adu_sps_attr_idx

The attribute coded by the ADU shall have at most three components when attr_coding_type is not 3.

adu_slice_id specifies the value of the preceding GDU slice_id.

slice_attr_inter_prediction equal to 1 specifies that inter prediction may be used to derive the attributes in the ADU. slice_attr_inter_prediction equal to 0 specifies that inter prediction is not used to derive the attributes in the ADU.  When slice_attr_inter_prediction is not present, it shall be inferred to be 0.

slice_attr_inter_prediction2 equal to 1 specifies that the inter prediction from the second reference frame may be used to derive the attributes in the ADU.  slice_attr_inter_prediction equal to 0 specifies that the inter prediction from the second reference frame is not used to derive the attributes in the ADU.  When slice_attr_inter_prediction2 is not present, it shall be inferred to be 0.

num_inter_filters specifies the number of RAHT temporal filters. When num_inter_filters is present, it is a requirement of bitstream conformance that shall be equal to Min(raht_inter_layer_depth_minus1+1, RahtRootLvl) – raht_send_inter_filters. When num_inter_filters is not presented, it shall be infered to be 0.

raht_inter_filter_qidx[] specifies the quantized residual of the RAHT temporal filters.

layer_code_depth specifies the number of layers in the transform tree for which AC coefficients coding mode are signalled.  When layer_code_depth is present, it is a requirement of bitstream conformance that layer_code_depth shall be equal to Min(raht_inter_layer_depth_minus1, RahtRootLvl - 1).  When layer_code_depth is not present, it shall be inferred to be 0.

slice_raht_inter_layer_code_mode[d] equal to 1 specifies that inter prediction is enabled for AC coefficients at the prediction depth d of the transform tree. slice_raht_inter_layer_code_mode[d] equal to 0 specifies that inter prediction is disabled for AC coefficients at the prediction depth d of the transform tree.

slice_raht_intra_layer_code_mode[d] equal to 1 specifies that intra prediction is enabled for AC coefficients at the prediction depth d of the transform tree.  slice_raht_intra_layer_code_mode[d] equal to 0 specifies that intra prediction is disabled for AC coefficients at the prediction depth d of the transform tree.

    - Defaulted attribute data unit semantics

A defaulted attribute data unit specifies a single attribute value for all points in the slice.

defattr_seq_parameter_set_id specifies the active SPS by its sps_seq_parameter_set_id.

defattr_reserved_zero_3bits shall be 0 in bitstreams conforming to this version of this document.  Other values of defattr_reserved_zero_3bits are reserved for future use by ISO/IEC.  Decoders shall ignore the value of defattr_reserved_zero_3bits.

defattr_sps_attr_idx identifies the coded attribute by its index into the active SPS attribute list.

At the start of every defaulted attribute data unit, the variable AttrIdx is set to defattr_sps_attr_idx:

AttrIdx = defattr_sps_attr_idx

defattr_slice_id specifies the value of the preceding GDU slice_id.

defattr_value[ 𝑐 ] specifies the value of the 𝑐-th attribute component for every point in the slice.  The length in bits of defattr_value[ 𝑐 ] is AttrBitDepth.

Decoding process

  - General decoding process

The reconstruction of a point cloud is specified such that all decoders that conform to a specified profile and level will produce numerically identical output point cloud frames for a bitstream conforming to that profile and level.  Any decoding process that produces an identical output point cloud sequence to that produced by the process described herein conforms to the decoding process requirements of this document.

The frame decoding process (8.2) shall be repeatedly performed for each coded point cloud frame in the coded point cloud sequence.

  - Frame decoding processes

    - General

The result of this process is a reconstructed point cloud frame.

At the start of every coded point cloud frame, the output point cloud frame shall be initialized to the empty point cloud.

RecCloudPointCnt = 0

When inter prediction is enabled, PtnCurrFramePos is initialized to -1 for all entries. When the current frame is not an I-frame and the slices of the frame are coded using octree coding, the reference frame generation process in subclause 9.2.15 is invoked

The slice decoding process (8.3) shall be repeatedly performed for each slice in the coded point cloud frame.

    - Frame counter

The variable FrameCtr represents the notional frame counter.  For the first decoded frame, FrameCtr shall be set equal to frame_ctr_lsb.  Otherwise, the variable FrameCtr shall be updated for each frame:

window = Exp2(frame_ctr_lsb_bits) >> 1
curLsb = FrameCtr % Exp2(frame_ctr_lsb_bits)
curMsb = FrameCtr >> frame_ctr_lsb_bits
if ((frame_ctr_lsb < curLsb) && (curLsb − frame_ctr_lsb) ≥ window)
  curMsb++
else if ((frame_ctr_lsb > curLsb) && (frame_ctr_lsb – curLsb) > window)
  curMsb−−
FrameCtr = (curMsb << frame_ctr_lsb_bits) + frame_ctr_lsb

  - Slice decoding processes

    - General

A slice in a coded point cloud frame shall be decoded as follows:

- Point positions are decoded from one GDU in the slice as specified by 8.3.3.

- Default attribute values are set for each attribute as specified by 8.3.4.

- Point attributes are decoded from each ADU in the slice as specified by 8.3.5.

- The decoded point positions are offset and the output point count incremented as specified by 8.3.6.

Only one slice shall be decoded for every set of slices in a coded point cloud frame with the same value of slice_id as specified in subclause 6.4.6.

    - State variables

Slice decoding is specified in terms of the following state variables:

- The variable PointCnt, a cumulative count of decoded points.

- The array PointAng of angular coordinates for decoded points; PointAng[ ptIdx ][ 𝑘 ] is the 𝑘-th angular coordinate of the point position PointPos[ ptIdx ].

    - Geometry decoding process

The GDU shall be decoded and the reconstructed positions stored in the output point cloud.

The expression PointPos[ ptIdx ][ 𝑘 ] is an alias into the output point cloud for points in the slice.

PointPos[ptIdx][k] := RecCloudPos[RecCloudPointCnt + ptIdx][k]

- The definition of PointPos implicitly concatenates the points of consecutive slices.

When geom_angular_enabled is 1, the geometry decoding process populates the array PointAng with points' angular coordinates.

At the start of every slice, PointCnt is initialized to 0.  It is incremented for each point decoded by the geometry decoding process.

Point positions shall be decoded and reconstructed as specified by Clause 9.

    - Default attribute values

Attribute values for every point in the slice shall be set to their respective default values.  This process shall be equivalent to the following steps for each attribute, attrIdx = 0 .. num_attributes − 1:

- All components of the attribute values shall be set to Exp2( attr_bitdepth_minus1[ attrIdx ] ).

- If the attribute property attr_default_value[ attrIdx ] is present, the attribute values shall be set to attr_default_value[ attrIdx ][ 𝑐 ], for each component 𝑐.

- If the slice contains a defaulted attribute data unit with defattr_sps_attr_idx equal to attrIdx, the attribute values shall be set to defattr_value[ 𝑐 ] of that DU, for each component 𝑐.

    - Attribute decoding process

The ADU shall be decoded and the reconstructed attribute values stored in the corresponding output point cloud attribute.

The expression PointAttr[ ptIdx ][ 𝑐 ] is an alias into the output point cloud attribute array for the points in the slice.

PointAttr[ptIdx][c] := RecCloudAttr[RecCloudPointCnt + ptIdx][AttrIdx][c]

Point attributes shall be decoded and reconstructed as specified by Clause 10.

    - At the end of a slice

The variable RecCloudPointCnt is incremented by the number of points decoded.

RecCloudPointCnt += PointCnt

The slice geometry shall be translated from the slice's coordinate system to the coding coordinate system by the addition of the slice origin, SliceOrigin.

- The attribute decoding processes specified in Clause 10 are performed prior to the coordinate system conversion.

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  for (k = 0; k < 3; k++)
    PointPos[ptIdx][k] += SliceOrigin[k]

    - Update of inter prediction buffer

This subclause is invoked when inter prediction is enabled under predictive geometry coding and biprediction_enabled is 0. The points in the reference frame are downsampled using the azimuth scale and added such that for each beamId and azimuth look value, at most max_points_per_entry_minus1 points are stored.

if (inter_prediction_enabled)
  for(ptIdx = 0; ptIdx < PointCnt; ptIdx++)
    for(j = 0; j <= max_points_per_entry_minus1; j++) {
      if (PtnCurrFramePos[beamId][qAzim][j][0] == -1) {
        if (max_points_per_entry_minus1 > 0 && j > 0) {
          lAttr = PtnCurrFrameAttr[beamId][qAzim][j-1]
          lPtn = PtnCurrFramePos[beamId][qAzim][j-1]
          if (lAttr[0] == PtnCurrFrameAttr[beamId][qAzim][j][0])
            if ((Abs(lPtn[0] - PtnCurrFramePnt[beamId][qAzim][j][0]) > dnRadiusRange) ||
                (Abs(lPtn[1] - PtnCurrFramePnt[beamId][qAzim][j][1]) > dnAzimuthRange)) {
              for(k = 0; k < 3; k ++)
                PtnCurrFramePos[beamId][qAzim][j][k] = PointAng[ptIdx][k]
              PtnCurrFrameAttr[beamId][qAzim][j][0] = PointAttr[ptIdx][0]
              break
            }
        }
        for(k = 0; k < 3; k ++)
          PtnCurrFramePos[beamId][qAzim][j][k] = PointAng[ptIdx][k]
        PtnCurrFrameAttr[beamId][qAzim][j][0] = PointAttr[ptIdx][0]
        break
      }
    }
where
  beamId := PointAng[ptIdx][2]
  qAzim := DivExp2Fz(PointAng[ptIdx][1], inter_azim_scale_log2)
  dnRadiusRange := down_sampling_range × (attr_coord_conv_scale[0] >> 8)
  dnAzimuthRange := down_sampling_range

Slice geometry

  - General

Clause 9 specifies the coding of slice geometry and the reconstruction of point positions, storing the reconstructed geometry in the arrays PointPos and PointAng.

  - Occupancy tree

    - General

Subclause 9.2 specifies the parsing and reconstruction of point positions from a coded occupancy tree.  It applies when geom_tree_type is 0.

An occupancy tree represents the slice geometry as a tree of occupancy tree nodes.  Parsing or traversing a coded occupancy tree implicitly generates a representation of the slice geometry.

    - Coded occupancy tree

      - General tree structure

Individual point positions are represented in the occupancy tree either by the position of leaf nodes, or by direct nodes that encode node-relative positions.

An occupancy tree node shall identify the presence of at least one point contained within the volume of an axis-aligned cuboid.  The volume is defined in the slice's coordinate system by an inclusive lower corner pmin and an exclusive upper corner pmax.  The volume edge lengths are non-negative integer powers of two.  A node's size, nodeSize, is synonymous with the volume dimensions pmax-pmin.

The occupancy tree shall be formed of one or more tree levels.  Every tree level consists of tree nodes with non-overlapping volumes.  All tree nodes within a tree level shall have identical volume dimensions.

The occupancy tree shall contain a single root node.  The root node shall be the only node in the top level of the tree.  The volume identified by the root node shall have a lower corner at position ( 0, 0, 0 ), coincident with the slice origin.  The upper corner shall be at an integer position 2s,2t,2v equal to the root node size.

With each subsequent tree level, starting from the top tree level, the node volume dimensions are halved along one or more coded axes.  The coded axes in each tree level are enumerated in the GDU header, as specified by occtree_coded_axis.

The location of a node, nodeLoc, within a tree level is related to the spatial position of the node volume's lower corner in the slice coordinate system by:

pmin,k=nodeLock×nodeSizek

Two tree nodes are spatially adjacent if their volumes share a face.

Unless an early termination condition applies (9.2.6.5), tree nodes with a volume greater than the unit cube shall have one or more child nodes.  Depending upon the number of coded axes, these nodes shall have at most eight, four, two, or one child nodes, as illustrated for cubic nodes in Figure 5.

Figure 5 — Arrangement of child nodes depending upon coded axes.

Leaf nodes, in the absence of geometry scaling (9.2.14), represent indivisible volumes with dimensions equal to the unit cube.  When duplicate point coding is enabled, a leaf node may represent more than one point.  In such cases, all points represented by the leaf node shall have identical positions.

      - Tree traversal order

The coded occupancy tree shall be traversed in breadth-first order.  Traversal shall start from the top tree level.  All nodes in a tree level shall be sequentially traversed before proceeding to the next level.  Within a tree level, nodes shall be traversed in ascending Morton order of node location.

The traversal order for an example tree is illustrated in Figure 6.  Each tree level progressively refines the slice geometry.  Starting from the root node, a, the node traversal order is from a to y.  Occupied nodes are shaded.

Figure 6 — Occupancy tree traversal order.  Illustrated for three tree levels.

      - Node occupancy bitmap

The structure of the occupancy tree is coded as a sequence of node occupancy bitmaps.

Each node occupancy bitmap shall enumerate the child nodes for a single node.  Each set bit position identifies the relative location of a child node in the next level of the occupancy tree as specified by the expression OccLocC[ bitIdx ][ 𝑘 ] and Table 13.  The count of set bits in the bitmap shall be the number of child nodes.

When a node has fewer than three coded axes, bits in the occupancy bitmap that do not enumerate a valid child node location shall be unset.

OccLocC[bitIdx][k] := Bit(bitIdx, 2 − k)

The tree level location of a child node is related to its parent's by:

nodeLocchild,k=nodeLocparent,k×nodeSizeparent,knodeSizechild,k+relLock

Table 13 — Identification of valid relative child node location relLoc from set bits in an occupancy bitmap

| Coded Axes |  |  | Bit position (bitIdx) in occupancy bitmap |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S | T | V | 7 (MSB) | 6 | 5 | 4 | 3 | 2 | 1 | 0 (LSB) |
| 1 | 1 | 1 | ( 1, 1, 1 ) | ( 1, 1, 0 ) | ( 1, 0, 1 ) | ( 1, 0, 0 ) | ( 0, 1, 1 ) | ( 0, 1, 0 ) | ( 0, 0, 1 ) | ( 0, 0, 0 ) |
| 1 | 1 | 0 |  | ( 1, 1, 0 ) |  | ( 1, 0, 0 ) |  | ( 0, 1, 0 ) |  | ( 0, 0, 0 ) |
| 1 | 0 | 1 |  |  | ( 1, 0, 1 ) | ( 1, 0, 0 ) |  |  | ( 0, 0, 1 ) | ( 0, 0, 0 ) |
| 0 | 1 | 1 |  |  |  |  | ( 0, 1, 1 ) | ( 0, 1, 0 ) | ( 0, 0, 1 ) | ( 0, 0, 0 ) |
| 1 | 0 | 0 |  |  |  | ( 1, 0, 0 ) |  |  |  | ( 0, 0, 0 ) |
| 0 | 1 | 0 |  |  |  |  |  | ( 0, 1, 0 ) |  | ( 0, 0, 0 ) |
| 0 | 0 | 1 |  |  |  |  |  |  | ( 0, 0, 1 ) | ( 0, 0, 0 ) |
| 0 | 0 | 0 |  |  |  |  |  |  |  | ( 0, 0, 0 ) |

      - Terminal nodes

In the coded occupancy tree, leaf nodes are immediately encoded by their parent (terminal) node.

When geom_dup_point_counts_enabled is 1, terminal nodes shall encode the duplicate point counts for the leaf nodes they contain.

    - Occupancy tree syntax element semantics

occtree_depth_minus1 plus 1 specifies the maximum number of tree levels present in the coded occupancy tree.  When occtree_coded_axis_list_present is 0, the root node size is a cubic volume with edge lengths equal to Exp2( occtreeMaxDepthMinus1 + triSoupNodeSizeLog2 + 1 ).

The number of coded tree levels coded is equal to occtreeMaxDepthMinus1 plus 1.  The value of occtreeMaxDepthMinus1 is set equal to occtree_depth_minus1 when trisoup_enabled is equal to 0, otherwise when trisoup_enabled is equal to 1 the value of occtreeMaxDepthMinus1 is set as

occtreeMaxDepthMinus1 = occtree_depth_minus1 - 1

In the latter case, occupancy tree coding is performed until reaching not the unit cube size but node size triSoupNodeSizeLog2 instead.  The TriSoup process (9.4) is performed for each node with node size triSoupNodeSizeLog2.

When trisoup_enabled is equal to 1, the value triSoupNodeSizeLog2 is set to  trisoup_node_size_log2_minus2 + 2 .  Otherwise, when trisoup_enabled is equal to 0, the value triSoupNodeSizeLog2 is set to 0.

occtree_coded_axis[ dpth ][ 𝑘 ] specifies whether (when 1) or not (when 0) a subdivision along the 𝑘-th STV axis is coded by tree nodes at depth dpth.  occtree_coded_axis shall be used to determine the node volume dimensions in each level of the occupancy tree.  When occtree_coded_axis[ dpth ][ 𝑘 ] is not present, it shall be inferred to be 1.

It is a requirement of bitstream conformance that:

- There shall be at least one coded axis in every tree level specified by occtree_coded_axis; i.e. MaxVec( occtree_coded_axis[ dpth ] ) == 1.

- The log2 dimensions of the root node shall be less than or equal to MaxSliceDimLog2.

- The largest log2 dimension of the root node shall be greater than  occtreeMaxDepthMinus1 + triSoupNodeSizeLog2 – 4.

occtree_stream_cnt_minus1 plus 1 specifies the maximum number of entropy streams used to code the occupancy tree.  When occtree_stream_cnt_minus1 is greater than zero, each of the bottom occtree_stream_cnt_minus1 tree levels shall be conveyed in a separate entropy stream; the parsing state shall be memorized and restored according to subclause 1.

The expression OcctreeEntropyStreamDepth is the depth of the last tree level that is coded in the first entropy stream.

OcctreeEntropyStreamDepth := occtreeMaxDepthMinus1 − occtree_stream_cnt_minus1

occtree_end_of_entropy_stream is a non-coded syntax element used to specify the termination point for the arithmetic decoder at the end of an entropy stream.  The syntax element has no value.

occtree_lvl_point_cnt_minus1[ dpth ] plus 1 indicates, when present, the number of points that can be partially decoded (See Annex D) from the root node to the end of the tree level at depth dpth.  occtree_lvl_point_cnt_minus1[ 0 ] shall be inferred to be 0.  occtree_lvl_point_cnt_minus1[ occtreeMaxDepthMinus1 ] shall be inferred to be slice_num_points_minus1.

    - Node dimensions per tree level

The log2 node dimensions at depth dpth are specified by the expression OccLvlNodeSizeLog2[ dpth ][ 𝑘 ].  They are derived from the list of coded axes:

- In the bottom tree level, at depth occtreeMaxDepthMinus1 + 1, the node dimensions shall be equal to the cube with the size triSoupNodeSizeLog2.

- The log2 node dimensions in any tree level shallower than the bottom tree level shall be the count of the respective coded axes, proceeding from the bottom tree level.

OccLvlNodeSizeLog2[dpth][k] := lvl ≤ occtreeMaxDepthMinus1
  ? OccLvlNodeSizeLog2[dpth + 1][k] + occtree_coded_axis[dpth][k]
  : triSoupNodeSizeLog2

    - State representation

      - State variables

The occupancy tree is specified in terms of the following state variable:

- The sparse array OccNodePresent that identifies nodes present in the occupancy tree.  Each element OccNodePresent[ dpth ][ ns ][ nt ][ nv ] equal to either 1 or −1 indicates the presence of a node at location ( ns, nt, nv ) and depth dpth.  Elements equal to −1 identify leaf nodes that are not coded at that depth (9.2.2.4).  Unset elements of OccNodePresent are inferred to be 0.

Traversal of the occupancy tree is specified in terms of the following state variables:

- The array OccNodeCnt; OccNodeCnt[ dpth ] is the cumulative count of nodes present at depth dpth.

- The array OccNodeLoc; OccNodeLoc[ dpth ][ nodeIdx ][ 𝑘 ] identifies the 𝑘-th location component of the nodeIdx-th coded node in the traversal order of the tree level at depth dpth.

      - The root node

At the start of the occupancy tree syntax structure, the arrays OccNodePresent, OccNodeLoc and OccNodeCnt are initialized to represent the root node at location ( 0, 0, 0 ); all other elements are cleared.

OccNodePresent[0][0][0][0] = 1
OccNodeLoc[0][0][0] = OccNodeLoc[0][0][1] = OccNodeLoc[0][0][2] = 0
OccNodeCnt[0] = 1

    - Occupancy tree node coding

      - General

Subclause 9.2.6 specifies the semantics of the NodeIdx-th coded node at tree depth Dpth.

      - Syntax element semantics

occ_single_child equal to 1 specifies that the coded node has a single child.  occ_single_child equal to 0 specifies that the coded node may generate multiple child nodes.  When occ_single_child is not present, it shall be inferred to be 0.

occupancy_idx[ 𝑘 ] specifies the 𝑘-th component of the relative child node location for the only child of the coded node.

occupancy_bit and occupancy_byte specify the child nodes of the coded node as neighbourhood-permuted node occupancy bitmaps.  The syntax elements shall be coded as specified by 9.2.10 and 9.2.9, respectively.

occ_dup_point_cnt[ 𝑖 ] plus 1 specifies the number of points represented by the 𝑖-th coded child leaf node.  All points represented by a child have identical positions.  When occ_dup_point_cnt[ 𝑖 ] is not present in a terminal node, it is inferred to be 0.

When unique_point_positions_constraint is 1, it is a requirement of bitstream conformance that occ_dup_point_cnt[ 𝑖 ] shall be 0.

      - Node, parent, grandparent and child tree-level locations

The tree-level location ( Ns, Nt, Nv ) of the coded node is specified by the expression Nloc[ 𝑘 ].

Nloc[k] := OccNodeLoc[Dpth][NodeIdx][k]

Ns := Nloc[0]
Nt := Nloc[1]
Nv := Nloc[2]

The parent node has a location ( NsP, NtP, NvP ) in the tree level at depth Dpth − 1.  It is specified by the expression NlocP[ 𝑘 ].

NlocP[k] := Dpth ? Nloc[k] >> occtree_coded_axis[Dpth − 1][k] : 0

NsP := NlocP[0]
NtP := NlocP[1]
NvP := NlocP[2]

The grandparent node has a location ( NsG, NtG, NvG ) in the tree level at depth Dpth − 2.  It is specified by the expression NlocG[ 𝑘 ].

NlocG[k] := Dpth > 1 ? NlocP[k] >> occtree_coded_axis[Dpth − 2][k] : 0

NsG := NlocG[0]
NtG := NlocG[1]
NvG := NlocG[2]

The corresponding location ( NsC, NtC, NvC ) in the tree level at depth Dpth + 1 for the coded node is specified by the expression NlocC[ 𝑘 ].

NlocC[k] := Nloc[k] << AxisCoded[k]

NsC := NlocC[0]
NtC := NlocC[1]
NvC := NlocC[2]

      - Node size

The expressions NodeSizeLog2[ 𝑘 ] and ChildNodeSizeLog2[ 𝑘 ] specify the log2 dimensions of the coded node and its children, respectively.

NodeSizeLog2[k] := OccLvlNodeSizeLog2[Dpth][k]

ChildNodeSizeLog2[k] := OccLvlNodeSizeLog2[Dpth + 1][k]

- When geom_scaling_enabled is 0, QuantizedNodeSizeLog2[ 𝑘 ] is equal to NodeSizeLog2[ 𝑘 ].

      - Whether the node is a terminal node

A node is a terminal node, as specified by the expression TerminalNode, if its children are leaf nodes, or it is a direct node.

TerminalNode := MaxVec(ChildNodeSizeLog2) == 0
  || geom_scaling_enabled && MaxVec(QuantizedChildNodeSizeLog2) == 0
  || occtree_direct_coding_mode && occ_direct_node

      - Coded axes

A node shall only code an axis for child locations when specified by the expression AxisCoded[ 𝑘 ].  An axis is coded when:

- it is specified to be coded in the tree level by the coded axis list, and

- if geometry subtree scaling is enabled, the corresponding dimension of the quantized node size is greater than 1.

AxisCoded[k] := occtree_coded_axis[Dpth][k]
  && (¬geom_scaling_enabled || QuantizedNodeSizeLog2[k] > 0)

The expression CodedAxisCnt is the number of axes coded by the node.

CodedAxisCnt := AxisCoded[0] + AxisCoded[1] + AxisCoded[2]

The location of child nodes along each coded axis may be constrained by planar occupancy coding (9.2.11.4).  A free axis is a coded axis that is not constrained so, as specified by the expression OccFreeAxis[ 𝑘 ].  The number of free axes is specified by the expression OccFreeAxisCnt.

- When planar occupancy coding is disabled, OccFreeAxisCnt is equal to CodedAxisCnt.

OccFreeAxis[k] := AxisCoded[k] && (¬occtree_planar_enabled || PlanarFreeAxis[k])

OccFreeAxisCnt := OccFreeAxis[0] + OccFreeAxis[1] + OccFreeAxis[2]

      - Limits to the number of child nodes

The number of child nodes a node can contain is constrained by the tree and node syntax.

The maximum number of child nodes is specified by the expression OccMaxChildren.  Unless occ_single_child is 1, the limit shall be the number of child node locations that can be identified by the free axes.  Otherwise, the limit shall be 1 when occ_single_child is 1.

OccMaxChildren := occ_single_child ? 1 : Exp2(OccFreeAxisCnt)

The minimum number of child nodes is specified by the expression OccMinChildren.  A node shall contain at least one child node unless:

- planar occupancy coding specifies that there shall be at least two child nodes (9.2.11.3), or

- occ_single_child is both present and equal to 0, in which case there shall be at least two child nodes.

OccMinChildren :=
  OccMaybeSingleChild && ¬occ_single_child ? 2 :
  occtree_planar_enabled                   ? PlanarMinChildren : 1

      - Presence of occ_single_child

The presence of occ_single_child is specified by the expression OccMaybeSingleChild.  It shall be present in the occupancy node syntax when all the following conditions are true:

- No nodes are present in the occupied neighbourhood pattern (9.2.7.4).

- OccupancyIsPredictable is 0 or there is at least one free axis to code child node locations.

- Planar occupancy coding does not specify that there shall be at least two child nodes (9.2.11.3).

OccMaybeSingleChild :=
  ¬OccNeighPat && OccFreeAxisCnt > 0 && PlanarMinChildren == 1

      - Presence of occupancy_bit and occupancy_byte

The node occupancy bitmap shall be coded using either occupancy_bit or occupancy_byte when specified by the expression OccMapPresent.  One of the two syntax elements shall be present when both of the following conditions are true:

- The maximum number of child nodes is greater than 1.

- The locations of child nodes are not completely prescribed by constraints on occupancy.  i.e. the maximum number of child nodes is greater than the minimum number of child nodes.

OccMapPresent := OccMaxChildren > 1 && OccMinChildren != OccMaxChildren

      - Node occupancy bitmap

This subclause specifies the node occupancy bitmap by the expression OccupancyMap.

When occupancy_bit syntax elements are present (OccMapPresent is 1), the node occupancy bitmap is specified by bitwise occupancy coding (9.2.10.2).

if (OccMapPresent && occtree_bitwise_coding)
  OccupancyMap = OccBitMap

When occupancy_byte is present (OccMapPresent is 1), the node occupancy bitmap shall be rearranged from the neighbourhood-permuted bitmap (9.2.8) coded by occupancy_byte.

if (OccMapPresent && ¬occtree_bitwise_coding)
  OccupancyMap = OccFromNpOcc(occupancy_byte)

When constraints on occupancy require there to be a single child node, each component 𝑘 of the child location shall be specified by whichever of occ_plane_pos[ 𝑘 ] or occupancy_idx[ 𝑘 ] are present, or shall be 0 if neither is present.

if (OccMaxChildren == 1 && OccMinChildren == 1) {
  occupancyIdx[k] :=
    OccFreeAxis[k] && occupancy_idx[k] || ¬PlanarFreeAxis[k] && occ_plane_pos[k]

  OccupancyMap = 1 << Morton[occupancyIdx]
}

- In this case, an axis cannot be both a free axis and eligible for planar occupancy coding.

When constraints on occupancy require there to be two child nodes, there shall be one child node at both permitted locations along the free axis.

if (OccMaxChildren == 2 && OccMinChildren == 2) {
  baseIdx[k] := ¬PlanarFreeAxis[k] && occ_plane_pos[k]

  if (OccFreeAxis[0]) OccupancyMap = 0x11 << Morton[baseIdx]
  if (OccFreeAxis[1]) OccupancyMap = 0x05 << Morton[baseIdx]
  if (OccFreeAxis[2]) OccupancyMap = 0x03 << Morton[baseIdx]
}

      - Child node count

The number of child nodes is equal to the number of set bits in the node occupancy bitmap, as specified by the expression OccChildCnt.

OccChildCnt := PopCnt(OccupancyMap)

      - Insertion of non-terminal child nodes

Unless the coded node is a terminal node, its child nodes shall be inserted into the state representation of the occupancy tree and included in the traversal list of the next tree level.  The node occupancy bitmap shall be scanned to enumerate the child nodes.

if (¬TerminalNode)
  for (occBitIdx = 0; occBitIdx < 8; occBitIdx++) {
    if (¬Bit(OccupancyMap, occBitIdx))
      continue

    cs = NsC + OccLocC[occBitIdx][0]
    ct = NtC + OccLocC[occBitIdx][1]
    cv = NvC + OccLocC[occBitIdx][2]
    OccNodePresent[Dpth + 1][cs][ct][cv] = 1

    childNodeIdx = OccNodeCnt[Dpth + 1]
    OccNodeCnt[Dpth + 1]++

    OccNode[Dpth + 1][childNodeIdx][0] = cs
    OccNode[Dpth + 1][childNodeIdx][1] = ct
    OccNode[Dpth + 1][childNodeIdx][2] = cv
  }

      - Points represented by child leaf nodes

When the node is a non-direct terminal node, points represented by child leaf nodes shall be scaled (9.2.14.6) and appended to the output point list.  The node occupancy bitmap shall be scanned to enumerate the child nodes.

- When geometry scaling is disabled, this condition is equivalent to the child node size being equal to the unit cube.

When adjacent child occupancy contextualization is enabled, the child leaf nodes shall be inserted into the state representation of the occupancy tree for use by other nodes in the same tree level; but they shall be excluded from traversal in the next tree level.  The child leaf nodes shall not be included in the occupied neighbourhood pattern for any node in the next tree level.

if (TerminalNode && ¬occ_direct_node)
  for (child = 0, occBitIdx = 0; occBitIdx < 8; occBitIdx++) {
    if (¬Bit(OccupancyMap, occBitIdx))
      continue

    cs = NsC + OccLocC[occBitIdx][0]
    ct = NtC + OccLocC[occBitIdx][1]
    cv = NvC + OccLocC[occBitIdx][2]
    if (occtree_adjacent_child_enabled)
      OccNodePresent[Dpth + 1][cs][ct][cv] = −1

    for (i = 0; i < occ_dup_point_cnt[child] + 1; i++, PointCnt++) {
      PointPos[PointCnt][0] = OccPosScaleK(0, cs)
      PointPos[PointCnt][1] = OccPosScaleK(1, ct)
      PointPos[PointCnt][2] = OccPosScaleK(2, cv)
    }

    child++
  }

      - Definition of ChildIdx

This subclause specifies the expression ChildIdx[ pt ] that is the child index of point pt for a given node.

ChildIdx[pt] = (pt[2] & mask[2] ? 1 : 0) + 
                ((pt[1] & mask[1] ? 1 : 0) << 1) +
                ((pt[0] & mask[0] ? 1 : 0) << 2)
where
  mask[k] = (NodeSizeLog2[k] != ChildNodeSizeLog2[k]) ? 1 << ChildNodeSizeLog2[k] : 0

      - Derivation of OccupancyIsPredictable

OccupancyIsPredictable specifies whether the current node satisfies conditions to enable inter prediction of occupancy of child nodes.

occupancyIsPredictable = numSiblingsMispredcted <= 5 && PredOccBitMap > 0
where
  numSimblingsMispredicted = PopCnt(PredOccBitMap ^ OccupancyMap)

      - Derivation of PredOccBitMap

The occupancy of the regions collocated to the child nodes of the current node is stored in the variable PredOccBitMap.

PredOccBitMap is derived as follows

PredOccBitMap1 = 0
if(enableInterPredFromRef)
  for(i = 0;i < 8; i++)
    if(PredCounts1[i])
      PredOccBitMap1 |= 1 << i
PredOccBitMap2 = 0
if(enableInterPredFromRef2)
  for(i = 0;i < 8; i++)
    if(PredCounts2[i])
      PredOccBitMap2 |= 1 << i
PredOccBitMap = predDir ? PredOccBitMap2 : PredOccBitMap1

      - Derivation of nStart, nEnd, nStart2 and nEnd2

When enableInterPredFromRef is 1, the variables nStart and nEnd for a node that is the i-th child node of its parent node specify the start and end indices of a sorted reference frame that contains points belonging to the reference frame that are collocated with the node. Let nStart_parent denote the nStart variable of the parent node.

When the node is the root node, nStart is 0 and nEnd is the total number of points in the slice.

Otherwise, (node is not the root node), nStart and nEnd are derived as follows:

nStart = nStart_parent
for(j = 0; j < i; j++)
  nStart += PredCounts[j]
nEnd = nStart + PredCounts[i]

When enableInterPredFromRef2 is 1, the variables nStart2 and nEnd2 for a node that is the i-th child node of its parent node specify the start and end indices of the second sorted reference frame that contains points belonging to the second reference frame that are collocated with the node. nStart2 and nEnd2 are derived from the total number of points in the slice of second reference frame or PredCounts2, in the same way as nStart and nEnd.

      - Derivation of PredCounts and PredCounts2

When enableInterPredFromRef is 1, the array PredCounts[i] denotes the start indices of the positions of the octree node in the reference frame collocated with the i-th child node.

The frame buffers RefFramePos and RefFramePos2 are set equal to PredPointCloud and SecondPredPointCloud, respectively, at the beginning of the octree slice.

First, the number of points in reference frame that are collocated with each child node, colRefPoints[i] is calculated. colRefPoints[0..7] is initialized to 0.

for(i = nStart; i < nEnd; i++) {
  pt = RefFramePos[i]
  colRefPoints[ChildIdx[pt]]++
}

The array PredCounts is derived as follows:

PredCounts[0] = 0
for(i = 1; i < 8; i++)
  PredCounts[i] = PredCounts[i-1] + colRefPoints[i-1]

When enableInterPredFromRef2 is 1, the array PredCounts2[i] denotes the start indices of the positions of the octree node in the second reference frame collocated with the i-th child node. PredCounts2 is derived from the number of points in second reference frame that are collocated with each child node, in the same way as PredCounts.

      - Update of RefFramePos and RefFramePos2 at the beginning of coding a node

When enableInterPredFromRef1 is 1, the points in the reference frame buffer RefFramePos are sorted based on the child node regions.

nIdx[0] = nStart
for(i = 1; i < 8; i++)
  nIdx[i] = nStart + PredCounts[i]
lastIdx = 0
for (int i = 0; i < Radix; i++) {
  lastIdx += PredCounts[i]
  while (nIdx[i] != lastIdx) {
    radix = ChildIdx[RefFramePos[nIdx[i]]]
    Swap(RefFramePos[nIdx[i]], RefFramePos[nIdx[radix]])
    nIdx[radix]++
  }
}

When enableInterPredFromRef2 is 1, the points in the second reference frame buffer RefFramePos2 are sorted based on the child node regions.

nIdx[0] = nStart2
for(i = 1; i < 8; i++)
  nIdx[i] = nStart2 + PredCounts2[i]
lastIdx = 0
for (int i = 0; i < Radix; i++) {
  lastIdx += PredCounts2[i]
  while (nIdx[i] != lastIdx) {
    radix = ChildIdx[RefFramePos2[nIdx[i]]]
    Swap(RefFramePos2[nIdx[i]], RefFramePos2[nIdx[radix]])
    nIdx[radix]++
  }
}

      - Derivation of enableInterPredFromRef and enableInterPredFromRef2

When slice_biprediction is 1 and frame_merge_enabled is 0, enableBipredDerive is derived as follows:

enableBipredDerive = slice_biprediction && !frame_merge_enabled && (nodeSizeLog2[0] >= 1) && (nodeSizeLog2[1] >= 1) && (nodeSizeLog2[2] >= 1)

The variables enableInterPredFromRef and enableInterPredFromRef2 specify whether (1) or not (0) the first and second reference frames are used in inter prediction. enableInterPredFromRef and enableInterPredFromRef2 are derived as:

enableInterPredFromRef = enableBipredDerive || !predDir
enableInterPredFromRef2 = enableBipredDerive || predDir

      - Derivation of predDir

The variables predDir for a node that is the i-th child node of its parent node specify the inter prediction is from the first reference frame (0) or the second reference frame (1). predDir is initilized to 0 for the root node.

Let predDir_parent denote the predDir variable of the parent node. predDir is derived as follows:

predDir = predDir_parent
if (enableBiPredDerive)
  if (!PredCounts2[i])
    predDir = 0
  else if (!PredCounts[i])
    predDir = 1
  else
    predDir = predFailureCount != predFailureCount2 ? (predFailureCount >= 
      predFailureCount2) : predDir_parent
  Where
    predFailureCount = enableInterPredFromRef ? PopCnt(PredOccBitMap ^ OccupancyMap) : 0
    predFailureCount2 = enableInterPredFromRef2 ? PopCnt(PredOccBitMap2 ^ OccupancyMap) : 
      0

    - Occupied neighbourhood patterns

      - General

Coding of the node occupancy bitmap depends upon the existence and arrangement of up to six spatially adjacent tree nodes within an availability window.  The occupied neighbourhood pattern for an occupancy tree node shall identify the spatial arrangement of these adjacent nodes from the 64 possible combinations.  Examples of occupied neighbourhood patterns are illustrated in Figure 7.

Figure 7 — Characteristic occupied neighbourhood patterns.

      - Neighbour availability

Nodes are grouped into availability windows by their spatial location within their tree level.  Nodes can form part of the occupied neighbourhood pattern of adjacent nodes within the same window.  Nodes shall not form part of the occupied neighbourhood pattern of any node in a different window.

The size of the availability window is specified by the expression OccAvailWinLog2[ 𝑘 ]:

- Unless occtree_neigh_window_log2_minus1 is 0, each availability window shall span 𝑛×𝑛×𝑛 node locations, 𝑛 = Exp2( occtree_neigh_window_log2_minus1 + 1 ), within the tree level.  The availability windows form a contiguous grid starting from the location ( 0, 0, 0 ).

- Otherwise, the availability window for any node shall be restricted to its sibling nodes.

OccAvailWinLog2[k] := occtree_neigh_window_log2_minus1
  + (occtree_neigh_window_log2_minus1 > 0 || Dpth > 0 && occtree_coded_axis[Dpth − 1][k])

- Only the Main profile permits an element of occtree_coded_axis to be 0 when occtree_neigh_window_log2_minus1 is 0.

The expression OccNeighAvail[ ns ][ nt ][ nv ] specifies whether the node at location ( ns, nt, nv ) is within the same availability window as the coded node ( Ns, Nt, Nv ).

OccNeighAvail[ns][nt][nv] :=
     (ns ^ Ns) >> OccAvailWinLog2[0] == 0
  && (nt ^ Nt) >> OccAvailWinLog2[1] == 0
  && (nv ^ Nv) >> OccAvailWinLog2[2] == 0

      - Presence of another coded node within the availability window

The expression OccNeigh[ ns ][ nt ][ nv ] identifies whether there exists a node with tree location ( ns, nt, nv ) and depth Dpth that is not a leaf node and is within the availability window of the coded node ( Ns, Nt, Nv ).

- OccNodePresent[ Dpth ][ ns ][ nt ][ nv ] equal to −1 identifies a leaf node.

OccNeigh[ns][nt][nv] := OccNeighAvail[ns][nt][nv] && OccNodePresent[Dpth][ns][nt][nv] == 1

      - Occupied neighbourhood pattern

The occupied neighbourhood pattern for the coded node located at ( Ns, Nt, Nv ) in the tree level at depth Dpth is specified by the expression OccNeighPat.  It is a linear combination of spatially adjacent nodes coded in the same tree level that are available and adjoin the coded node by a face.  Leaf nodes shall not be included in the occupied neighbourhood pattern.

An occupancy tree node with no spatially adjacent nodes has an occupied neighbourhood pattern equal to 0.

OccNeighPat := (uN << 5) | (dN << 4) | (bN << 3) | (fN << 2) | (rN << 1) | lN
  where
    rN := OccNeigh[Ns + 1][Nt][Nv]
    lN := OccNeigh[Ns − 1][Nt][Nv] && (¬occtree_adjacent_child_enabled || lNadj)
    bN := OccNeigh[Ns][Nt + 1][Nv]
    fN := OccNeigh[Ns][Nt − 1][Nv] && (¬occtree_adjacent_child_enabled || fNadj)
    uN := OccNeigh[Ns][Nt][Nv + 1]
    dN := OccNeigh[Ns][Nt][Nv − 1] && (¬occtree_adjacent_child_enabled || dNadj)

When adjacent child contextualization is enabled (occtree_adjacent_child_enabled is 1), a tree node that adjoins the left (Ns − 1), front (Nt − 1) or bottom (Nv − 1) face shall not be included in the occupied neighbourhood pattern unless it contains at least one child node that also adjoins the same face.  Their inclusion is specified by the expressions lNadj, fNadj and dNadj, equivalent to the following:

lNadj = fNadj = dNadj = 0
for (s = 0; s ≤ occtree_coded_axis[Dpth][0]; s++)
  for (t = 0; t ≤ occtree_coded_axis[Dpth][1]; t++)
    for (v = 0; v ≤ occtree_coded_axis[Dpth][2]; v++) {
      lNadj |= OccNodePresent[Dpth + 1][NsC − 1][NtC + t][NvC + v] ≠ 0
      fNadj |= OccNodePresent[Dpth + 1][NsC + s][NtC − 1][NvC + v] ≠ 0
      dNadj |= OccNodePresent[Dpth + 1][NsC + s][NtC + t][NvC − 1] ≠ 0
    }

The exclusion of a node from the occupied neighbourhood pattern due to adjacent child contextualization is illustrated in Figure 8.  The child node C0 of the left neighbour does not adjoin the left face of the coded node N resulting in its parent node being excluded from the occupied neighbourhood pattern; OccNeighPat is 2.  The child node C7 does adjoin N and its parent node is not excluded; OccNeighPat is 3.

Key

| N | Coded node |
| --- | --- |
| Cm | Child node at location 𝑚 in left neighbour |

Figure 8 — Effect of adjacent child contextualization on an occupied neighbourhood pattern.  (Top) Exclusion.  (Bottom) Inclusion.

      - Reduced occupied neighbourhood pattern

The occupied neighbourhood pattern shall be reduced to one of a smaller set of patterns as specified by the expression OccNeighPatR.

When occtree_neigh_window_log2_minus1 is greater than 0, the smaller set of patterns is specified by Table 14 as a mapping of spatial rotations and reflections in the arrangement of the six spatially adjacent neighbours to produce nine unique arrangements.

Otherwise, when occtree_neigh_window_log2_minus1 is 0, the smaller set of patterns is specified by Table 15 as a mapping of adjacent siblings that produces six arrangements.

OccNeighPatR := occtree_neigh_window_log2_minus1 > 0
  ? NeighPat64to9[OccNeighPat]
  : NeighPat64to6[OccNeighPat]

Table 14 — Reduction of occupied neighbourhood pattern 𝑗 + 𝑖 to nine patterns, NeighPat64to9[ 𝑗 + 𝑖 ]

| 𝑗 | 𝑖 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 3 | 1 | 2 | 2 | 3 | 1 | 3 | 3 | 4 |
| 16 | 1 | 2 | 2 | 3 | 2 | 5 | 5 | 6 | 2 | 5 | 5 | 6 | 3 | 6 | 6 | 7 |
| 32 | 1 | 2 | 2 | 3 | 2 | 5 | 5 | 6 | 2 | 5 | 5 | 6 | 3 | 6 | 6 | 7 |
| 48 | 1 | 3 | 3 | 4 | 3 | 6 | 6 | 7 | 3 | 6 | 6 | 7 | 4 | 7 | 7 | 8 |

Table 15 — Reduction of occupied neighbourhood pattern 𝑗 + 𝑖 to six patterns, NeighPat64to6[ 𝑗 + 𝑖 ]

| 𝑗 | 𝑖 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| 0 | 0 | 5 | 5 | na | 5 | 1 | 1 | na | 5 | 1 | 1 | na | na | na | na | na |
| 16 | 2 | 3 | 3 | na | 3 | 7 | 7 | na | 3 | 7 | 7 | na | na | na | na | na |
| 32 | 2 | 3 | 3 | na | 3 | 7 | 7 | na | 3 | 7 | 7 | na | na | na | na | na |
| 48 | na | na | na | na | na | na | na | na | na | na | na | na | na | na | na | na |
| - The specification of values 5 and 7 aligns with further reductions performed in bitwise occupancy coding. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

      - Advanced occupied neighbourhood patterns

The advanced occupied neighbourhood patterns for one node located at ( Ns, Nt, Nv ) in the tree level at depth Dpth are the occupancy combinations of the decoded spatially adjacent nodes in the tree level at depth Dpth and neighbouring child nodes in the tree level at depth Dpth+1. Leaf nodes shall not be included in the advanced occupied neighbourhood patterns. For coding occupancy bit of each child node, one advanced occupied neighbourhood pattern OccAdvNeiPati is derived for the child node. The derived (OccAdvNeiPat0, OccAdvNeiPat1, OccAdvNeiPat2, OccAdvNeiPat3, OccAdvNeiPat4, OccAdvNeiPat5, OccAdvNeiPat6, OccAdvNeiPat7) are used to construct contextual information CI to be used in Dynamic OBUF according to 9.2.10.6.10.

For one child node, the expression occL specifies the occupancy of the 4 neighbouring child nodes neighbouring current node on the left, occF specifies the occupancy of the 4 neighbouring child nodes neighbouring current node on the front, and occB specifies the occupancy of the 4 neighbouring child nodes neighbouring current coded node on the bottom. An example of occL, occF and occB for the first child node is illustrated in Figure 9.

Figure 9 — occL, occF, occB for the first child node

occL, occF and occB are calculated from the occupancy of child nodes of three spatially adjacent nodes (occLeft, occFront and occBottom) on the left, front and bottom of current node.

occL := occLeft >> 4
occF := ((occFront >> 2) & 3) | ((occFront >> 4) & 12)
occB := ((occBottom >> 1) & 1) | ((occBottom >> 2) & 2) | ((occBottom >> 3) & 4) | ((occBottom >> 4) & 8

The expression occOfLFB specifies the occupancy of child nodes of three spatially adjacent nodes on the left, front and bottom.

occOfLFB := occLeft | occFront | occBottom

The expression edgeSets specifies the occupancy of 6 neighbouring child nodes, fLF0, fLF1, fLB0, fLB1, fFB0 and fFB1. fLF0 and fLF1 share the same faces with both the neighbouring child nodes sets specified by occL and occF. fLB0 and fLB1 share the same faces with both the neighbouring child nodes sets specified by occL and occB. fFB0 and fFB1 share the same faces with both the neighbouring child nodes sets specified by occL and occF. An example of edgeSets the first child node is illustrated in Figure 10.

Figure 10 — example of edgeSets for the first child node

The expression neighPatternRBT specifies the occupancy of 3 spatially adjacent nodes on the right, back and top of the current node. An example of neighPatternRBT for the node and their order to construct neighPatternRBT is illustrated in Figure 11. neighPatternRBT is calculated by accessing occupancy OccNeighPat (9.2.7.4) of six spatially adjacent nodes of the node.

neighPatternRBT := ((neighPattern >> 3) & 4) | ((neighPattern >> 2) & 2) | (neighPattern & 1)

Figure 11 — example of three adjacent nodes of neighPatternRBT of node N

The expression neighPatternLFB specifies the occupancy of 3 spatially adjacent nodes on the bottom, front and left of the current node. neighPatternLFB is calculated by accessing occupancy OccNeighPat (9.2.7.4) of six spatially adjacent nodes of the node.

neighPatternLFB =((neighPattern & 0b110) >> 1)| ((neighPattern & 16) >> 2)

The expression neighb20 specifies of the occupancy of 20 spatially adjacent nodes of the current node which adjoin the current node by an edge or a vertex and not a face. An example of neighb20 for the node is illustrated in Figure 12. neighb20 is calculated by accessing OccNeigh (9.2.7.3) and constructed by an order starting from node 0 to node 19.

neighb20 = 0
for(i = 0; i < 20; i++)
  neighb20 |= OccNeigh[Ns + LUTds[i]][Nt + LUTdt[i]][Nv + LUTdv[i]]<<i
  where
    LUTds[20] := {-1, -1, -1, -1, -1, -1, -1, -1, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 1}
    LUTdt[20] := {-1, -1, -1, 0, 0, 1, 1, 1, -1, -1, 1, 1, -1, -1, -1, 0, 0, 1, 1, 1}
    LUTdv[20] := {-1, 0, 1, -1, 1, -1, 0, 1, -1, 1, -1, 1, -1, 0, 1, -1, 1, -1, 0, 1}

Figure 12 — Example of neighbouring nodes for neighb20 of node N

        - Derivation of OccAdvNeiPat0

The expression OccAdvNeiPat0 specifies advanced occupied neighbourhood pattern constructed to code occupancy bit of the first child node b0 of the current node.

The expression NN0 specifies the number of occupied child nodes in the three sets of neighbouring child nodes specified by occL, occF and occB.

- If NN0 is larger than 1, then the occupied neighbour nodes of child node b0 are not sparse and OccAdvNeiPat0 comprises 19 bits. OccAdvNeiPat0 is derived by evoking 9.2.7.6.2.

- Otherwise, the occupied neighbour nodes of child node b0 are sparse and OccAdvNeiPat0 comprises 16 bits. OccAdvNeiPat0 is derived by evoking 9.2.7.6.3.

        - Derivation of OccAdvNeiPat0 with non-sparse neighbour nodes

When the occupied neighbour nodes of child node b0 are not sparse, OccAdvNeiPat0 is derived as follows.

- If more than one sets within the three sets specified by occL, occF and occB are occupied, OccAdvNeiPat0[0] is set as 1 and OccAdvNeiPat0[1]OccAdvNeiPat0[2] indicates which sets are occupied.

OccAdvNeiPat0[1] = occL && occF && occB ? 0 :
  (!occF ? 0 : 1)

OccAdvNeiPat0[2] = occL && occF && occB ? 0 :
  (!occF ? 1 : (!occL ? 0 : 1))

  - if OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 100,

    - OccAdvNeiPat0[3]…OccAdvNeiPat0[5] indicates the occupancy of the 3 neighboring child nodes sharing the same face with b0 in the three sets specified by occB, occF and occL in order;

    - OccAdvNeiPat0[6]OccAdvNeiPat0[7] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the set specified by occB;

OccAdvNeiPat0[6] = (occB >> 2) & 1

OccAdvNeiPat0[7] = (occB >> 1) & 1

    - OccAdvNeiPat0[8]OccAdvNeiPat0[9] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the set specified by occF;

OccAdvNeiPat0[8] = (occF >> 2) & 1

OccAdvNeiPat0[9] = (occF >> 1) & 1

    - OccAdvNeiPat0[10]OccAdvNeiPat0[11] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the set specified by occL;

OccAdvNeiPat0[10] = (occL >> 2) & 1

OccAdvNeiPat0[11] = (occL >> 1) & 1

    - OccAdvNeiPat0[12]…OccAdvNeiPat0[14] is set as neighPatternRBT; OccAdvNeiPat0[15]…OccAdvNeiPat0[18] is calculated as follows.

OccAdvNeiPat0[15] = neighb20[8]

OccAdvNeiPat0[16] = neighb20[3]

OccAdvNeiPat0[17] = neighb20[1]

OccAdvNeiPat0[18] = neighb20[0]

  - if OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 101,

    - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 neighboring child nodes sharing the same face with b0 in the the sets specified by occB and occL;

    - OccAdvNeiPat0[5]…OccAdvNeiPat0[8] indicates the occupancy of the 4 neighboring child nodes sharing the same edge with b0 in the the set specified by occB and occL;

    - OccAdvNeiPat0[9]OccAdvNeiPat0[10] indicates the occupancy of the 2 neighboring child nodes sharing the same vertex with b0 in the the sets specified by occB and occL;

    - OccAdvNeiPat0[11]…OccAdvNeiPat0[18] is calculated as follows.

OccAdvNeiPat0[11] = neighPatternRBT[1]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

  - if OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 110,

    - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 neighboring child nodes sharing the same face with b0 in the the sets specified by occB and occF;

    - OccAdvNeiPat0[5]…OccAdvNeiPat0[8] indicates the occupancy of the 4 neighboring child nodes sharing the same edge with b0 in the the set specified by occB and occF;

    - OccAdvNeiPat0[9]OccAdvNeiPat0[10] indicates the occupancy of the 2 neighboring child nodes sharing the same vertex with b0 in the the sets specified by occB and occF;

    - OccAdvNeiPat0[11]…OccAdvNeiPat0[18] is calculated as follows.

OccAdvNeiPat0[11] = neighPatternRBT[0]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

  - Otherwise (OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 111),

    - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 neighboring child nodes sharing the same face with b0 in the the sets specified by occL and occF;

    - OccAdvNeiPat0[5]…OccAdvNeiPat0[8] indicates the occupancy of the 4 neighboring child nodes sharing the same edge with b0 in the the set specified by occL and occF;

    - OccAdvNeiPat0[9]OccAdvNeiPat0[10] indicates the occupancy of the 2 neighboring child nodes sharing the same vertex with b0 in the the sets specified by occL and occF;

    - OccAdvNeiPat0[11]…OccAdvNeiPat0[18] is calculated as follows.

OccAdvNeiPat0[11] = neighPatternRBT[2]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

- Otherwise, OccAdvNeiPat0[0] is set as 0 and OccAdvNeiPat0[1]OccAdvNeiPat0[2] indicates which set is occupied.

OccAdvNeiPat0[1] = occL ? 0 : (occF ? 0 : 1)

OccAdvNeiPat0[2] = occL ? 0 : (occF ? 1 : 0)

  - If OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 000,

    - OccAdvNeiPat0[3] indicates the occupancy of the neighboring child node sharing the same face with b0 in the three sets specified by occL;

    - OccAdvNeiPat0[4]OccAdvNeiPat0[5] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the the set specified by occL;

    - OccAdvNeiPat0[6] indicates the occupancy of the neighboring child node sharing the same vertex with b0 in the the set specified by occL;

    - OccAdvNeiPat0[7]…OccAdvNeiPat0[18] is calculated as follows. An example of neighbouring child nodes used to calculate OccAdvNeiPat0[7]OccAdvNeiPat0[8] is illustrated in Figure 13.

OccAdvNeiPat0[7] = 0

OccAdvNeiPat0[8] = 0

OccAdvNeiPat0[9] = edgeSets[2]

OccAdvNeiPat0[10] = edgeSets[3]

OccAdvNeiPat0[11] = neighPatternRBT[0] || neighPatternRBT[1] || neighPatternRBT[2]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

Figure 13 — Example of edge sub-set of current child node when only the set specified by occL is occupied

  - If OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 001,

    - OccAdvNeiPat0[3] indicates the occupancy of the neighboring child node sharing the same face with b0 in the three sets specified by occF;

    - OccAdvNeiPat0[4]OccAdvNeiPat0[5] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the the set specified by occF;

    - OccAdvNeiPat0[6] indicates the occupancy of the neighboring child node sharing the same vertex with b0 in the the set specified by occF;

    - OccAdvNeiPat0[7]…OccAdvNeiPat0[18] is calculated as follows. An example of neighbouring child nodes used to calculate OccAdvNeiPat0[7]OccAdvNeiPat0[8] is illustrated in Figure 14.

OccAdvNeiPat0[7] = edgeSets[4]

OccAdvNeiPat0[8] = edgeSets[5]

OccAdvNeiPat0[9] = neighPatternRBT[0]

OccAdvNeiPat0[10]  neighPatternRBT[1]

OccAdvNeiPat0[11] = neighPatternRBT[2]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

Figure 14 — Example of edge sub-set of current child node when only the set specified by occF is occupied

  - Otherwise (OccAdvNeiPat0[0]OccAdvNeiPat0[1]OccAdvNeiPat0[2] is equal to 010),

    - OccAdvNeiPat0[3] indicates the occupancy of the neighboring child node sharing the same face with b0 in the three sets specified by occB;

    - OccAdvNeiPat0[4]OccAdvNeiPat0[5] indicates the occupancy of the 2 neighboring child nodes sharing the same edge with b0 in the the set specified by occB;

    - OccAdvNeiPat0[6] indicates the occupancy of the neighboring child node sharing the same vertex with b0 in the the set specified by occB;

    - OccAdvNeiPat0[7]…OccAdvNeiPat0[18] is calculated as follows. An example of neighbouring child nodes used to calculate OccAdvNeiPat0[7]OccAdvNeiPat0[8] is illustrated in Figure 15.

OccAdvNeiPat0[7] = edgeSets[0]

OccAdvNeiPat0[8] = edgeSets[1]

OccAdvNeiPat0[9] = neighPatternRBT[0]

OccAdvNeiPat0[10]  neighPatternRBT[1]

OccAdvNeiPat0[11] = neighPatternRBT[2]

OccAdvNeiPat0[12] = neighb20[8]

OccAdvNeiPat0[13] = neighb20[3]

OccAdvNeiPat0[14] = neighb20[1]

OccAdvNeiPat0[15] = neighb20[0]

OccAdvNeiPat0[16] = neighb20[18]

OccAdvNeiPat0[17] = neighb20[19]

OccAdvNeiPat0[18] = neighb20[11]

Figure 15 — Example of edge sub-set of current child node when only the set specified by occB is occupied

        - Derivation of OccAdvNeiPat0 with sparse neighbour nodes

When the occupied neighbour nodes of child node b0 are sparse, OccAdvNeiPat0 is derived as follows.

- If NN0 is equal to 1 and the neighbouring child nodes set specified by occL is occupied,

  - OccAdvNeiPat0[0]OccAdvNeiPat0[1] is set as 01;

  - OccAdvNeiPat0[2] indicates the occupancy of the neighboring child node sharing same face with b0 in the occL;

  - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 spatially adjacent nodes on the bottom and front of the current node and adjoin the current node by a face.

OccAdvNeiPat0[3] = neighPatternLFB[1]

OccAdvNeiPat0[4] = neighPatternLFB[0]

- If NN0 is equal to 1 and the neighbouring child nodes set specified by occF is occupied,

  - OccAdvNeiPat0[0]OccAdvNeiPat0[1] is set as 10;

  - OccAdvNeiPat0[2] indicates the occupancy of the neighboring child node sharing same face with b0 in the occF;

  - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 spatially adjacent nodes on the bottom and left of the current node and adjoin the current node by a face.

OccAdvNeiPat0[3] = neighPatternLFB[4]

OccAdvNeiPat0[4] = neighPatternLFB[1]

- If NN0 is equal to 1 and the neighbouring child nodes set specified by occB is occupied,

  - OccAdvNeiPat0[0]OccAdvNeiPat0[1] is set as 11;

  - OccAdvNeiPat0[2] indicates the occupancy of the neighboring child node sharing same face with b0 in the occB;

  - OccAdvNeiPat0[3]OccAdvNeiPat0[4] indicates the occupancy of the 2 spatially adjacent nodes on the front and left of the current node and adjoin the current node by a face.

OccAdvNeiPat0[3] = neighPatternLFB[1]

OccAdvNeiPat0[4] = neighPatternLFB[0]

- Otherwise (NN0 is equal to 0),

  - OccAdvNeiPat0[0]OccAdvNeiPat0[1] is set as 00;

  - OccAdvNeiPat0[2]…OccAdvNeiPat0[4] indicates the occupancy of the 3 spatially adjacent nodes on the bottom, front and left of the coded node and adjoin the coded node by a face.

OccAdvNeiPat0[2] = neighPatternLFB[2]

OccAdvNeiPat0[3] = neighPatternLFB[1]

OccAdvNeiPat0[4] = neighPatternLFB[0]

- OccAdvNeiPat0[5]…OccAdvNeiPat0[8] is calculated as follows.

OccAdvNeiPat0[5] = neighb20[1]

OccAdvNeiPat0[6] = neighb20[3]

OccAdvNeiPat0[7] = neighb20[8]

OccAdvNeiPat0[8] = neighb20[0]

- If at least one of the 3 adjacent parent nodes on the bottom, front and left of the current node is occupied,

  - If the first child node within the set of neighbouring child nodes specified by occOfLFB is occupied, OccAdvNeiPat0[9]…OccAdvNeiPat0[12] is calculated as follows. OccAdvNeiPat0[10]…OccAdvNeiPat0[12] indicates the occupancy of the next 3 farther neighboring child nodes as illustrated in Figure 16.

OccAdvNeiPat0[9] = 1

OccAdvNeiPat0[10] = occBottom & 1

OccAdvNeiPat0[11] = occFront & 1

OccAdvNeiPat0[12] = occLeft & 1

Figure 16 — Example of farther neighbouring child nodes

  - Otherwise,

    - OccAdvNeiPat0[9] is set as 0;

    - OccAdvNeiPat0[10] indicates whether the 3 edge sub-sets of two farther neighboring nodes of current child node b0 is occupied or not;

OccAdvNeiPat0[10] = !edgeSets[5] && !edgeSets[4] && !edgeSets[3] && !edgeSets[2] &&  
  !edgeSets[1] && !edgeSets[0]

    - OccAdvNeiPat0[11]OccAdvNeiPat0[12] is calculated as follows.

OccAdvNeiPat0[11] = (occLeft & 4) || (occFront & 2) || (occBottom & 4)

OccAdvNeiPat0[12] = (occLeft & 2) || (occFront & 16) || (occBottom & 16)

- OccAdvNeiPat0[13]…OccAdvNeiPat0[15] is calculated as follows.

OccAdvNeiPat0[13] = neighb20[18]

OccAdvNeiPat0[14] = neighb20[19]

OccAdvNeiPat0[15] = neighb20[11]

- Otherwise, OccAdvNeiPat0[9]…OccAdvNeiPat0[15] is calculated as follows.

OccAdvNeiPat0[9] = !(edgeSets[5]edgeSets[4])

OccAdvNeiPat0[10] = !(edgeSets[3]edgeSets[2])

OccAdvNeiPat0[11] = !(edgeSets[1]edgeSets[0])

OccAdvNeiPat0[12] = 0

OccAdvNeiPat0[13]OccAdvNeiPat0[14]OccAdvNeiPat0[15] =
  neighb20[18]neighb20[19]neighb20[11]

        - Derivation of OccAdvNeiPat1

The expression OccAdvNeiPat1 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of second child node b1 of the current node.

The two neighbouring child nodes sets specified by occL and occF for coding the second child node b1 is illustrated in Figure 17.

Figure 17 — Two sets of neighboring child nodes for b1

OccAdvNeiPat1 is derived as follows.

- OccAdvNeiPat1[0] is set as the occupancy of the first child node b0.

- If the nerighouring child nodes set specified by occF is occupied, the occupied neighbour nodes of child node b1 are not sparse.

OccAdvNeiPat1 = (b0 << 18) | (!(occF & 2) << 17) | (!occL << 16) | (occL ? AdvNeiFL :
  AdvNeiFNL) | neighb20[11] << 2 | neighb20[16] << 1 | neighb20[19]
  where
    AdvNeiFL := ((occL & 2) << 15) | ((neighPatternRBT & 4) << 14) | (!(occF & 1)
      << 13) | (!(occF & 8) << 12) | (!(occL & 1) << 11) | (!(occL & 8) << 10) |
      (!(occF & 4) << 9) | (!(occL & 4) << 8) | ((neighPatternRBT & 1) << 7) |
      neighb20[9] << 6 | neighb20[4] << 5 | neighb20[1] << 4 | neighb20[2] << 3
    AdvNeiFNL := (!(neighPatternRBT & 4) << 15) | (!(occF & 1) << 14) | (!(occF &
      8) << 13) | (!(occF & 4) << 12) | neighb20[9] << 11 | neighb20[4] << 10 |
      neighb20[1] << 9 | neighb20[2] << 8 | (!(occBottom & 2) << 7) | (!(occFront &
      2) << 6) | (!(occLeft & 2) << 5) | ((neighPatternRBT & 3) << 3)

- Otherwise, the occupied neighbour nodes of child node b1 are sparse.

OccAdvNeiPat1 = (b0 << 18) | (!(occL & 2) << 17) | ((neighPatternRBT & 4) << 16) | (!(occL
  & 1) << 15) | (!(occL & 8) << 14) | (!(occL & 4) << 13) | ((neighPatternRBT
  & 1) << 12) | neighb20[1] << 11 | neighb20[4] << 10 | neighb20[9] << 9 | neighb20[2]
  << 8 | (occOfLFB & 2 ? AdvBFL : AdvELF) | (!(occB) << 3) | neighb20[11] << 2 |
  neighb20[16] << 1 | neighb20[19]
  where
    AdvBFL := (1 << 7) | (!(occBottom & 2) << 6) | (!(occFront & 2) << 5) | 
      ((occLeft & 2) << 4)
    AdvELF := (!(edgeSets & 53) << 6) | (((occLeft & 8) || (occFront & 32)) << 5) |
      (((occLeft & 1) || (occFront & 1)) << 4)

        - Derivation of OccAdvNeiPat2

The expression OccAdvNeiPat2 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the third child node b2 of the current node. OccAdvNeiPat2 is derived as follows.

- OccAdvNeiPat2[0] is set as the occupancy of the first child node b0.

- If the nerighouring child nodes set specified by occB is occupied, the occupied neighbour nodes of child node b2 are not sparse.

OccAdvNeiPat2 = (b0 << 18) | (!(occB & 2) << 17) | (!occL << 16) | (occL ? AdvNeiBL :
  AdvNeiBNL) | neighb20[0] << 3 | neighb20[18] << 2 | neighb20[19] << 1 | neighb20[11]
  where
    AdvNeiBL := (!(occL & 4) << 15) | (!(neighPatternRBT & 2) << 14) | (!b1 << 13)
      | (!(occB & 8) << 12) | (!(occL & 8) << 11) | (!(occL & 1) << 10) | (!(occB &
      1) << 9) | neighb20[10] << 8 | neighb20[6] << 7 | neighb20[3] << 6 | (!(occB &
      4) << 5) | (!(occL & 2) << 4)
    AdvNeiBNL := (!(neighPatternRBT & 4) << 15) | (!(occF & 1) << 14) | (!(occF &
      8) << 13) | (!(occF & 4) << 12) | neighb20[9] << 11 | neighb20[4] << 10 |
      neighb20[1] << 9 | neighb20[2] << 8 | (!(occBottom & 2) << 7) | (!(occFront &
      2) << 6) | (!(occLeft & 2) << 5) | ((neighPatternRBT & 3) << 3)

- Otherwise, the occupied neighbour nodes of child node b2 are sparse.

OccAdvNeiPat2 = (b0 << 18) | (!(occL & 4) << 17) | (!(neighPatternRBT & 2) << 16) |
  (!(b0) << 15) | (!(occL & 8) << 14) | (!(occL & 1) << 13) | (!(occL & 2) << 12) |
  neighb20[3] << 11 | neighb20[6] << 10 | neighb20[10] << 9 | neighb20[5] << 8 |
  (occOfLFB & 4 ? AdvNeiLBF : AdvNeiLBE)
  where
    AdvNeiLBF := (1 << 7) | (!(occLeft & 4) << 6) | (!(occBottom & 4) << 5) |
      (!(occFront & 4) << 4)
    AdvNeiLBE := (((occLeft & 1) || (occBottom & 1)) << 6) | (((occLeft & 8) ||
      (occBottom & 64)) << 5) | (!(edgeSets & 3) << 4)

        - Derivation of OccAdvNeiPat3

The expression OccAdvNeiPat3 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the fourth child node b3 of the current node. The expression NN3 specifies the number of occupied child nodes in the set of neighbouring child nodes specified by occL and the set of coded child nodes (b0, b1, b2). OccAdvNeiPat3 is derived as follows.

- If NN3 is larger than 1, then the occupied neighbour nodes of child node b3 are not sparse.

OccAdvNeiPat3 = (!b2 << 16) | (!(b1) << 15) | (!(occL & 8) << 14) | (neighPatternRBT
  << 11) | (!b0 << 10) | (!(occL & 4) << 9) | (!(occL & 2) << 8) | (occL & 1) <<  |
  neighb20[11] << 6 | neighb20[6] << 5 | neighb20[4] << 4 | neighb20[0] << 3 |
  neighb20[16] << 2 | neighb20[19] << 1 | neighb20[18]

- Otherwise, the occupied neighbour nodes of child node b3 are sparse.

OccAdvNeiPat3 = (!((b2 << 2) | (b1 << 1) | b0) << 17) | (((b2 << 2) | (b1 << 1) |
  b0) ? AdvNei3Occp : AdvNei3L) | ((neighPatternRBT & 6) << 13) | neighb20[4] << 12 |
  neighb20[6] << 11 | neighb20[11] << 10 | neighb20[7] << 9 | (occOfLFB & 8 ?
  AvdNei3BFL : AvdNei3LE) | (!occB << 4) | (!occF << 3) | neighb20[18] << 2 |
  neighb20[19] << 1 | neighb20[16]
  where
    AdvNei3Occp := (!!((b2 << 2) | (b1 << 1) | b0) + !!((b2 << 1) | b1) + !!b2)
      << 15
    AdvNei3L := (!!(occL >> 1) + !!(occL >> 2) + !!(occL >> 3)) << 15
    AvdNei3BFL := (1 << 8) | (!(occBottom & 8) << 7) | (!(occFront & 8) << 6) |
      (!(occLeft & 8) << 5)
    AvdNei3LE := ((occLeft & 6) << 5) | (!(edgeSets & 50) << 5)

        - Derivation of OccAdvNeiPat4

The expression OccAdvNeiPat4 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the fifth child node b4 of the current node.

The expression occLeftChilds specifies the set of the 4 child nodes b0, b1, b2 and b3. occRightAvailChilds specifies the set of another 3 child nodes b4, b5 and b6.

occLeftChilds = b3 << 3 | b2 << 2 | b1 << 1 | b0

occRightAvailChilds = b6 << 2 | b5 << 1 | b4

The expression NN4 specifies the number of occupied child nodes in the sets of neighbouring child nodes specified by occL, occB and occLeftChilds. OccAdvNeiPat4 is derived as follows.

- If NN4 is larger than 1, then the occupied neighbour nodes of child node b4 are not sparse.

  - The expression NLFB specifies the number NLFB of occupied sets among the sets specified by occF and occB, occLeftChilds.

NLFB = !!occLeftChilds + !!occF + !!occB

  - OccAdvNeiPat4 is derived as follows.

OccAdvNeiPat4 = 0
OccAdvNeiPat4 |= (NLFB == 3 ? AdvNei4NLFB3 : NLFB == 2 ? AdvNei4NLFB2 :
  AdvNei4NLFB1)
  where
    AdvNei4NLFB3 := (8 << 15) | (!(occB & 4) << 17) | (!(occF & 4) << 16) |
      ((occLeftChilds & 1) << 15) | (!(neighPatternRBT & 1) << 14) | (!(occB &
      1) << 13) | (!(occB & 8) << 12) | (!(occF & 1) << 11) | (!(occF & 8) <<
      10) | (!(occLeftChilds & 2) << 9) | (!(occLeftChilds & 4) << 8) | (!(occB
      & 2) << 7) | (!(occF & 2) << 6) | (!(neighPatternRBT >> 1) << 4) |
      neighb20[15] << 3 | neighb20[13] << 2 | neighb20[8] << 2 | neighb20[12]
    AdvNei4NLFB2 := (occLeftChilds && occB ? AdvNei4BChild : occF && occB ?
      AdvNei4FB : AdvNei4Lchild) | neighb20[15] << 5 | neighb20[13] << 4 |
      neighb20[8] << 3 | neighb20[12] << 2 | neighb20[16] << 1 | neighb20[18]
    AdvNei4NLFB1 := (occLeftChilds ? AdvNei4Child : occF ? AdvNei4F : AdvNei4B)
      | ((neighPatternRBT >> 1) << 6) | neighb20[15] << 5 | neighb20[13] << 4 |
      neighb20[8] << 3 | neighb20[12] << 2 | neighb20[16] << 1 | neighb20[18]
    AdvNei4BChild := (4 << 15) | (!(occB & 4) << 14) | (!(occLeftChilds & 1) <<
      13) | (!(neighPatternRBT & 1) << 12) | (!(occB & 1) << 11) | (!(occB & 8)
      << 10) | (!(occLeftChilds & 2) << 9) | (!(occLeftChilds & 4) << 8)) |
      (!(occB & 2) << 7) | (!(occLeftChilds & 8) << 6)
    AdvNei4FB := (5 << 15) | (!(occB & 4) << 14) | (!(occF & 4) << 13) |
      (!(neighPatternRBT & 1) << 12) | (!(occB & 1) << 11) | (!(occB & 8) << 10)
      | (!(occF & 1) << 9) | (!(occF & 8) << 8) | (!(occB & 2) << 7) | (!(occF &
      2) << 6)
    AdvNei4Lchild := (6 << 15) | (!(occF & 4) << 14) | (!(occLeftChilds & 1) <<
      13) | (!(neighPatternRBT & 1) << 12) | (!(occF & 1) << 11) | (!(occF & 8)
      << 10) | (!(occLeftChilds & 2) << 9) | (!(occLeftChilds & 4) << 8) |
      (!(occF & 2) << 7) | (!(occLeftChilds & 8) << 6)
    AdvNei4Child := (0 << 15) | ((occLeftChilds & 1) << 14) |
      (!(neighPatternRBT & 1) << 13) | ((occLeftChilds & 6) << 12) |
      (!(occLeftChilds & 8) << 10) | ((edgeSets & 12) << 8)

- Otherwise, the occupied neighbour nodes of child node b4 are sparse. OccAdvNeiPat4 is derived as follows.

OccAdvNeiPat4=0

if (NN4 <= 1){
  if (NN4 == 1){
    OccAdvNeiPat4 |= occLeftChilds ? ((1 << 14) | (!(occLeftChilds & 1) << 13) |
      (!(neighPatternLFB & 4) << 12) | (!(neighPatternLFB & 2) << 11)) : occF ? ((2
      << 14) | (!(occF & 1) << 13) | (!(neighPatternLFB & 4) << 12) |
      (!(neighPatternLFB & 1) << 11)) : ((3 << 14) | (!(occB & 1) << 13) |
      (!(neighPatternLFB & 2) << 12) | (!(neighPatternLFB & 1) << 11)
  }
  else {
    OccAdvNeiPat4 |= (0 << 14) | (neighPatternLFB << 11)
  }
  OccAdvNeiPat4 |= neighb20[8] << 10 | neighb20[13] << 9 | neighb20[15] << 8
    | neighb20[12] << 7
  OccAdvNeiPat4 |= (neighPatternLFB ? AdvNei4LFB : AdvNei4ELFB) | neighb20[16] << 2
    | neighb20[18] << 1 | neighb20[19] << 0

}
  where
    AdvNei4ELFB := (!(edgeSets & 48) << 6) | (!(edgeSets & 12) << 5) | (!(edgeSets &
      3) << 4)
    AdvNei4LFB := (occOfLFB & 16) ? AdvNei4LFBfar : AdvNei4LFBfarer
    AdvNei4LFBfar := (1 << 6) | (!(occBottom & 16) << 5) | (!(occFront & 16) << 4) |
      (!(occLeft & 16) << 3)
    AdvNei4LFBfarer := (!edgeSets << 5) | (((occLeft & 64) || (occFront & 8) ||
      (occBottom & 8)) << 4) | (((occLeft & 32) || (occFront & 64) || (occBottom & 32))
      << 3)

        - Derivation of OccAdvNeiPat5

The expression OccAdvNeiPat5 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the sixth child node b5 of the current node. OccAdvNeiPat5 is derived as follows.

- If the nerighouring child nodes set specified by occF is occupied, the occupied neighbour nodes of child node b5 are not sparse.

OccAdvNeiPat5 = (b4 << 18) | (!(occF & 8) << 17) | (!occLeftChilds << 16) |
  (occLeftChilds ? AdvNei5occFChilds : AdvNei5fartherBFL) | neighb20[18] << 2 |
  neighb20[19] << 1 | neighb20[11]
  where
    AdvNei5occFChilds := (!(occLeftChilds & 2) << 15) | (!(neighPatternRBT & 4) <<
      14) | (!(neighPatternRBT & 1) << 13) | (!(occF & 2) << 12) | (!(occF & 4) <<
      11) | (!(occLeftChilds & 1) << 10) | (!(occLeftChilds & 8) << 9) | (!(occF &
      1) << 8) | (!(occLeftChilds & 4) << 7) | neighb20[16] << 6 | neighb20[13] << 5
      | neighb20[9] << 4 | neighb20[14] 
    AdvNei5fartherBFL := (!(neighPatternRBT & 4) << 15) | (!(neighPatternRBT & 1) 
      << 14) | (!(occF & 2) << 13) | (!(occF & 4) << 12) | (!(occF & 1) << 11) |
      neighb20[16] << 10 | neighb20[13] << 9 | neighb20[9] << 8 | neighb20[14] << 7
      | (!(occBottom & 32) << 6) | (!(occFront & 32) << 5) | (!(occLeft & 32) << 4)
      | (!(neighPatternRBT & 2) << 3)

- Otherwise, the occupied neighbour nodes of child node b5 are sparse.

OccAdvNeiPat5 = (!b4 << 18) | (!(occLeftChilds & 2) << 17) | (!(neighPatternRBT & 4)
  << 16) | (!(neighPatternRBT & 1) << 15) | (!(occLeftChilds & 1) << 14) |
  (!(occLeftChilds & 8) << 13) | (!(occL & 4) << 12) | neighb20[9] << 11 |neighb20[13]
  << 10 |neighb20[16] << 9 | neighb20[14] << 8 | (occOfLFB & 32 ? AdvNei5BFL :
  AdvNei5EFL) | (!occB << 3) | neighb20[18] << 2 | neighb20[19] << 1 | neighb20[11]

where

AdvNei5BFL := (1 << 7) | (!(occBottom & 32 ) << 6) | (!(occFront & 32) << 5) |
      (!(occLeft & 32) << 4)
    AdvNei5EFL := (!(edgeSets & 60) << 6) | (((occLeft & 128) || (occFront & 2)) <<
      5) | (((occLeft & 16) || (occFront & 16)) << 4)

        - Derivation of OccAdvNeiPat6

The expression OccAdvNeiPat6 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the seventh child node b6 of the current node. OccAdvNeiPat6 is derived as follows.

- If the nerighouring child nodes set specified by occB is occupied, the occupied neighbour nodes of child node b6 are not sparse.

OccAdvNeiPat6 = (b4 << 18) | (!(occB & 8) << 17) | (!occLeftChilds << 16) |
  (occLeftChilds ? AdvNei6occBChilds : AdvNei6occBLFB)
  where
    AdvNei6occBChilds := (!(occLeftChilds & 4) << 15) | (!(neighPatternRBT & 1) << 14) |
      (!(neighPatternRBT & 2) << 13) | (!b5 << 12) | (!(occB & 2) << 11) |
      (!(occLeftChilds & 1) << 10) | (!(occLeftChilds & 8) << 9) | (!(occB & 4) << 8) |
      neighb20[18] << 7 | neighb20[15] << 6 | neighb20[10] << 5 | (!(occB & 1) << 4) |
      (!(occLeftChilds & 2) << 3) | neighb20[17] << 2 | neighb20[0] << 1 | neighb20[11]
    AdvNei6occBLFB := (!(neighPatternRBT & 4) << 15) | (!(neighPatternRBT & 1) << 14) |
      (!(occF & 2) << 13) | (!(occF & 4) << 12) | (!(occF & 1) << 11) |
      neighb20[16] << 10 | neighb20[13] << 9 | neighb20[9] << 8 | neighb20[14] << 7
      | (!(occBottom & 32) << 6) | (!(occFront & 32) << 5) | (!(occLeft & 32) << 4)
      | (!(neighPatternRBT & 2) << 3)

- Otherwise, the occupied neighbour nodes of child node b6 are sparse.

OccAdvNeiPat6 = (!b4 << 18) | (!(occLeftChilds & 4) << 17) | (!(neighPatternRBT & 1) <<
  16) | (!b5 << 15) | (!(occLeftChilds & 8) << 14) | (!(occLeftChilds & 1) << 13) |
  (!(occLeftChilds & 2) << 12) | neighb20[17] << 11 | neighb20[18] << 10 | neighb20[15] <<
  9 | neighb20[10] << 8 | (occOfLFB & 64 ? AdvNei6LBF : AdvNei6LBFE) | (!occF << 3) |
  neighb20[19] << 2 | neighb20[16] << 1 | neighb20[11]

where
    AdvNei6LBF := (1 << 7) | (!(occLeft & 64) << 6) | (!(occBottom & 64) << 5) |
      (!(occFront & 64) << 4)
    AdvNei6LBFE := (((occLeft & 1) || (occBottom & 1)) << 6) | (((occLeft & 8) ||
      (occBottom & 64)) << 5) | (!(edgeSets & 3) << 4)

        - Derivation of OccAdvNeiPat7

The expression OccAdvNeiPat7 specifies advanced occupied neighbourhood pattern constructed to code the occupancy bit of the eighth child node b7 of the current node. The expression NN7 specifies the number of occupied child nodes in the set of neighbouring child nodes specified by occLeftChilds and occRightAvailChilds. OccAdvNeiPat7 is derived as follows.

- If NN7 is larger than 1, then the occupied neighbour nodes of child node b7 are not sparse.

OccAdvNeiPat7 = (!b6 << 16) | (!b5 << 15) | (!(occLeftChilds & 8) << 14) |
  (neighPatternRBT << 11) | (!b4 << 10) | neighb20[11] << 9 | (!(occLeftChilds & 4) << 8)
  | neighb20[16] << 7 | (!(occLeftChilds & 2) << 6) | neighb20[18] << 5 | ((occLeftChilds
  & 1) << 4) | neighb20[19] << 3 | neighb20[0] << 2 | neighb20[17] << 1 | neighb20[10]

- Otherwise, the occupied neighbour nodes of child node b7 are sparse.

OccAdvNeiPat7 = (!occup << 17) | (occup ? AdvNei7occRight : AdvNei7occLeft) |
  (!(neighPatternRBT & 4) << 13) | neighb20[11] << 12 | neighb20[16] << 11 | neighb20[18]
  << 10 | neighb20[19] << 9 | (occOfLFB & 128 ? AdvNei7LFB : AdvNei7LoccFoccB) | (!occB <<
  4) | (!occF << 3) | neighb20[7] << 2 | neighb20[17] << 1 | neighb20[10]
  where
    occup = occRightAvailChilds & 7
    AdvNei7occRight := ((!!occup + !!(occup >> 1) + !!(occup >> 2)) << 15) |
      (!(neighPatternRBT & 2) << 14)
    AdvNei7occLeft := (!!(occLeftChilds >> 1) + !!(occLeftChilds >> 2)
      + !!(occLeftChilds >> 3) << 15) | (!(neighPatternRBT & 1) << 14)
    AdvNei7LFB := (1 << 8) | (!(occLeft & 128) << 7) | (!(occFront & 128) << 6)
      | (!(occBottom & 128) << 5)
    AdvNei7LoccFoccB := ((occLeft & 96) << 1) | (((occF & 3) || (occB & 6)) << 5)

    - Neighbourhood-permuted node occupancy bitmap

The neighbourhood-permuted node occupancy bitmap is a rearrangement of the bits forming the node occupancy bitmap.  It is used in the coding of occupancy_byte and occupancy_bit.  The permutation shall be selected according to the occupied neighbourhood pattern.

The permutations for every occupied neighbourhood pattern are specified by Table 16.  Each entry is a base eight value with digits numbered from right to left.  The 𝑖-th digit is the bit position in the node occupancy bitmap of the 𝑖-th bit in the neighbourhood-permuted bitmap as specified by the expression OccBitIdxFromNpBit[ 𝑖 ].

OccBitIdxFromNpBit[i] := (OccArrangement[OccNeighPat] >> i × 3) & 7

The expression OccFromNpOcc[ npocc ] is the node occupancy bitmap derived from the neighbourhood-permuted node occupancy bitmap npocc.

OccFromNpOcc[npocc] :=
  OccFromNpOcc = 0
  for (i = 0; i < 8; i++)
    OccFromNpOcc |= Bit(npocc, i) << OccBitIdxFromNpBit[i]

Two example derivations of OccupancyMap from a single occupancy_byte by OccFromNpOcc are illustrated in Figure 18.  Each derivation uses a different occupied neighbourhood pattern, OccNeighPat.  Bit b4 of occupancy_byte is permuted to bit b1 of OccupancyMap when OccNeighPat is 17; in this case, OccBitIdxFromNpBit[ 4 ] would be 1.

- occupancy_bit codes the bits of the neighbourhood-permuted node occupancy bitmap in a different order.  i.e. occupancy_bit[ 𝑖 ] does not correspond to bit bi of occupancy_byte.

Figure 18 — Example relationships between the node occupancy bitmap OccupancyMap and neighbourhood-permuted node occupancy bitmap as coded by occupancy_byte.

Table 16 — Arrangements for neighbourhood-permuted node occupancy bitmaps by occupied neighbourhood pattern as OccArrangement[ 𝑖 + 𝑗 ]

| 𝑖 | 𝑗 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| 0 | 765432108 | 103254768 | 765432108 | 765432108 | 541076328 | 103254768 |
| 6 | 541076328 | 327610548 | 327610548 | 327610548 | 765432108 | 541076328 |
| 12 | 327610548 | 765432108 | 103254768 | 765432108 | 260437158 | 203164758 |
| 18 | 465702138 | 571346028 | 041526378 | 012345678 | 450167238 | 236701458 |
| 24 | 627340518 | 236701458 | 674523018 | 450167238 | 735162408 | 674523018 |
| 30 | 012345678 | 674523018 | 371526048 | 312075648 | 574613028 | 460257138 |
| 36 | 150437268 | 103254768 | 541076328 | 327610548 | 736251408 | 327610548 |
| 42 | 765432108 | 541076328 | 624073518 | 765432108 | 103254768 | 765432108 |
| 48 | 371526048 | 647520318 | 021346578 | 574613028 | 263704158 | 574613028 |
| 54 | 736251408 | 135702468 | 405162738 | 150437268 | 312075648 | 753164208 |
| 60 | 736251408 | 371526048 | 517340628 | 765432108 |  |  |

    - Dictionary coding of occupancy_byte

      - General

The occupancy_byte syntax element shall be coded as symbols by one of nine instances of this dictionary codec.  Coding shall proceed according to the syntax and semantics of the occupancy_byte_symbol syntax structure.

Each dictionary instance comprises a list of thirty-two most probable symbols (occupancy_byte values), a list of sixteen recently coded symbols, a histogram of symbol counts and state variables used to control updates to the dictionary state.

      - Syntax of a dictionary coded symbol

| occupancy_byte_symbol( ) { | Descriptor |
| --- | --- |
| occ_histogram_hit | ae(v) |
| if( occ_histogram_hit ) |  |
| occ_histogram_index | ae(v) |
| else { |  |
| occ_recent_hit | ae(v) |
| if( occ_recent_hit ) |  |
| occ_recent_index | ae(v) |
| else |  |
| occ_symbol_escape | ae(v) |
| } |  |
| } |  |

      - Syntax element semantics of a dictionary coded symbol

occ_histogram_hit specifies whether (when 1) or not (when 0) the coded symbol is present in the list of most probable symbols.

occ_histogram_index specifies the index of the coded symbol in the list of most probable symbols.

occ_recent_hit specifies whether (when 1) or not (when 0) the coded symbol is present in the most recently coded symbol list.  When occ_recent_hit is not present, it shall be inferred to be 0.

occ_recent_index specifies the index of the coded symbol in the most recently coded symbol list.

occ_symbol_escape specifies the value of the decoded symbol when occ_histogram_hit and occ_recent_hit are both 0.

      - State variables

The dictionary codec is specified in terms of the following state variables; the index dictIdx identifies an instance of the dictionary codec:

- A 9×256 element array DictsHistogram of symbol occurrence histograms per dictionary instance; DictsHistogram[ dictIdx ][ sym ] is the cumulative count for the symbol sym.

- A 9×32 element array DictsMostProb of thirty-two most probable symbols per dictionary instance; DictsMostProb[ dictIdx ][ 𝑖 ] is the 𝑖-th most probable symbol.

- A 9×16 element array DictsRecent of sixteen recently coded symbols per dictionary instance; DictsRecent[ dictIdx ][ 𝑖 ] is a recently coded symbol that was not coded using the most probable symbols list.

- A 9 element array DictsMostProbAge; DictsMostProbAge[ dictIdx ] is the count of symbols since the last generation of the dictionary's most probable symbol list.

- A 9 element array DictsMostProbMaxAge; DictsMostProbMaxAge[ dictIdx ] is the maximum allowed age in symbols of the dictionary's most probable symbol list.

- A 9 element array DictsNextEvictIdx; DictsNextEvictIdx[ dictIdx ] is the index of the next element to be evicted from the dictionary's DictsRecent array.

      - Initial state

The dictionary state shall be initialized at the start of every GDU.

When slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1, initialization shall be performed by the parsing state restoration process (11.6.2.2).

Otherwise (slice_entropy_continuation is 0 and slice_inter_entropy_continuation is 0), the dictionary state variables shall be initialized:

- Elements of DictsMostProb shall be initialized according to Table 17.

- Elements of DictsHistogram shall be set to 1 if they identify a symbol present in the corresponding most probable symbol sub-array.  i.e. DictsHistogram[ dictIdx ][ 𝑖 ] = 1 if 𝑖 ∈ DictsMostProb[ dictIdx ].  All other elements shall be set to 0.

- Elements of DictsRecent, DictsRecent[ dictIdx ][ 𝑖 ], shall be set to 𝑖.

- Elements of DictsNextEvictIdx and DictsMostProbAge shall be set to 0.

- Elements of DictsMostProbMaxAge shall be set to 16.

if (¬slice_entropy_continuation && ¬slice_inter _entropy_continuation) {
  … /* Initialize DictsMostProb using Table 17 */

  for (dictIdx = 0; dictIdx < 9; dictIdx++) {
    for (i = 0; i < 16; i++)
      DictsRecent[dictIdx][i] = i

    for (i = 0; i < 32; i++) {
      symbol = DictsMostProb[dictIdx][i]
      DictsHistogram[dictIdx][symbol] = 1
    }

    DictsNextEvictIdx[dictIdx] = 0
    DictsMostProbAge[dictIdx] = 0
    DictsMostProbMaxAge[dictIdx] = 16
  }
}

Table 17 — Initial values of DictsMostProb[ dictIdx ][ 𝑖 ]

| 𝑖 | dictIdx |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 | 5 | 85 | 128 | 64 | 85 | 16 | 170 | 170 | 255 |
| 1 | 17 | 255 | 32 | 128 | 170 | 64 | 10 | 255 | 223 |
| 2 | 34 | 170 | 64 | 192 | 255 | 17 | 42 | 128 | 239 |
| 3 | 68 | 64 | 16 | 204 | 119 | 80 | 8 | 160 | 251 |
| 4 | 160 | 80 | 192 | 136 | 127 | 128 | 138 | 136 | 127 |
| 5 | 136 | 252 | 80 | 68 | 254 | 68 | 15 | 168 | 247 |
| 6 | 12 | 223 | 160 | 170 | 87 | 32 | 255 | 204 | 119 |
| 7 | 80 | 84 | 48 | 200 | 223 | 85 | 2 | 240 | 253 |
| 8 | 192 | 117 | 68 | 85 | 95 | 81 | 14 | 250 | 63 |
| 9 | 21 | 68 | 8 | 196 | 117 | 84 | 136 | 192 | 191 |
| 10 | 10 | 247 | 136 | 255 | 245 | 192 | 11 | 238 | 221 |
| 11 | 48 | 221 | 176 | 4 | 213 | 48 | 175 | 162 | 254 |
| 12 | 3 | 4 | 2 | 8 | 247 | 51 | 32 | 234 | 238 |
| 13 | 170 | 192 | 240 | 80 | 93 | 4 | 238 | 223 | 95 |
| 14 | 168 | 128 | 144 | 160 | 234 | 34 | 47 | 138 | 175 |
| 15 | 162 | 174 | 17 | 240 | 69 | 240 | 191 | 254 | 240 |
| 16 | 204 | 253 | 208 | 208 | 238 | 1 | 34 | 10 | 85 |
| 17 | 85 | 204 | 224 | 76 | 21 | 136 | 239 | 186 | 187 |
| 18 | 14 | 240 | 112 | 221 | 221 | 170 | 245 | 8 | 244 |
| 19 | 81 | 69 | 19 | 140 | 191 | 255 | 174 | 251 | 250 |
| 20 | 35 | 127 | 255 | 244 | 253 | 204 | 3 | 2 | 170 |
| 21 | 69 | 213 | 85 | 72 | 187 | 196 | 63 | 127 | 245 |
| 22 | 84 | 5 | 51 | 93 | 16 | 160 | 95 | 125 | 117 |
| 23 | 176 | 119 | 170 | 168 | 251 | 12 | 223 | 247 | 34 |
| 24 | 51 | 238 | 84 | 250 | 171 | 208 | 253 | 85 | 126 |
| 25 | 65 | 175 | 162 | 32 | 17 | 69 | 168 | 171 | 51 |
| 26 | 138 | 160 | 238 | 252 | 5 | 191 | 142 | 32 | 93 |
| 27 | 200 | 87 | 204 | 187 | 174 | 119 | 246 | 197 | 243 |
| 28 | 212 | 136 | 1 | 223 | 125 | 21 | 206 | 221 | 207 |
| 29 | 11 | 16 | 76 | 238 | 239 | 95 | 13 | 87 | 234 |
| 30 | 50 | 244 | 138 | 243 | 12 | 2 | 162 | 42 | 59 |
| 31 | 15 | 23 | 187 | 84 | 241 | 206 | 250 | 239 | 236 |

      - Selection of a dictionary instance

A dictionary instance shall be selected for each coded occupancy_byte syntax element according to the reduced occupied neighbourhood pattern, OccNeighPatR.

The following expressions are aliases used in the specification of operations on the selected dictionary instance:

OccDictHistogram[i]   := DictsHistogram[OccNeighPatR][i]

OccDictMostProb[i]    := DictsMostProb[OccNeighPatR][i]

OccDictRecent[i]      := DictsRecent[OccNeighPatR][i]

OccDictMostProbAge    := DictsMostProbAge[OccNeighPatR]

OccDictMostProbMaxAge := DictsMostProbMaxAge[OccNeighPatR]

OccDictNextEvictIdx   := DictsNextEvictIdx[OccNeighPatR]

      - The value for occupancy_byte

The decoded value of the syntax element shall be:

- when occ_histogram_hit is 1:  OccDictMostProb[ occ_histogram_index ];

- when occ_recent_hit is 1: OccDictRecent[ occ_recent_index ];

- otherwise (neither occ_histogram_hit nor occ_recent_hit is 1): occ_symbol_escape.

      - Update of dictionary state after each coded symbol

        - List of most recently coded symbols

After each coded occupancy_byte syntax element when occ_histogram_hit is 0, the syntax element value shall be used to update the list of most recently coded symbols.

If the syntax element value is already present in the list, its position in the list shall be exchanged with the symbol scheduled to be evicted (at index OccDictNextEvictIdx).

for (i = 0; i < 16; i++)
  if (OccDictRecent[i] == occupancy_byte) {
    OccDictRecent[i] = OccDictRecent[OccDictNextEvictIdx]
    break
  }

The syntax element value shall be inserted into the list, replacing the symbol at index OccDictNextEvictIdx.

OccDictRecent[OccDictNextEvictIdx] = occupancy_byte

After updating the list of most recently coded symbols, the eviction index shall be incremented modulo 16.

OccDictNextEvictIdx = (OccDictNextEvictIdx + 1) % 16

        - Histogram of symbol counts

After each coded occupancy_byte syntax element, the histogram of symbol occurrences shall be updated and the age of the most probable symbol list shall be incremented.

OccDictHistogram[occupancy_byte]++
OccDictMostProbAge++

When the histogram count of symbols equal to occupancy_byte reaches 1 024, all counts in the histogram shall be halved and any fractional parts discarded.

if (OccDictHistogram[occupancy_byte] == 1024)
  for (i = 0; i < 256; i++)
    OccDictHistogram[i] >>= 1

      - Generation of the most probable symbol list

When OccDictMostProbAge is equal to OccDictMostProbMaxAge, the most probable symbol list shall be recalculated from the histogram of symbol counts.

The most probable symbol list shall be a stable descending ordering of the OccDictHistogram array.  Each element OccDictMostProb[ 𝑖 ] shall be the index of the 𝑖-th largest element in the OccDictHistogram array.  The ordering shall be such that the following conditions are true:

- OccDictHistogram[ OccDictMostProb[ 𝑖 ] ] ≥ OccDictHistogram[ OccDictMostProb[ 𝑖 + 1 ] ], and

- OccDictMostProb[ 𝑖 ] < OccDictMostProb[ 𝑖 + 1 ] when OccDictHistogram[ OccDictMostProb[ 𝑖 ] ] is equal to OccDictHistogram[ OccDictMostProb[ 𝑖 + 1 ] ].

The age of the generated most probable symbol list shall be 0.

OccDictMostProbAge = 0

The maximum age of the generated most probable symbol list shall be the next value in the bounded geometric progression specified by the expression OccDictMostProbMaxAgeNext:

OccDictMostProbMaxAgeNext := Min(5 × OccDictMostProbMaxAge >> 2, 1024)
OccDictMostProbMaxAge = OccDictMostProbMaxAge

      - Resetting the histogram of symbol counts

The histogram of symbol counts shall be reset immediately after the first recalculation of the most probable symbol list in each level of the occupancy tree.  Counts for symbols that are present in the most probable symbol list shall be set to 1; all other counts shall be set to 0.

if (OccDictMostProbAge == 0)
  if (… /* First occurrence in tree level */) {
    for (i = 0; i < 256; i++)
      OccDictHistogram[i] = 0

    for (i = 0; i < 32; i++) {
      symbol = OccDictMostProb[i]
      OccDictHistogram[symbol] = 1
    }
  }

      - Determination of CtxIdxDictHg for a bin of occ_histogram_index

Contextualization depends upon the reduced neighbourhood pattern, the bin index and the MSBs of the occ_histogram_index syntax element value.

Table 18 specifies the value ctxIdxInc for the bin.  If the ctxIdxInc is 'bypass', the value of CtxIdxDictHg shall be 'bypass'.  Otherwise, the value of CtxIdxDictHg shall be 5 × OccNeighPatR + ctxIdxInc.

Table 18 — Values of ctxIdxInc for bins of the syntax element occ_histogram_index

| MSBs of binarized occ_histogram_index | BinIdx |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 |
| '000' | 0 | 1 | 2 | 3 | 4 |
| '001' | 0 | 1 | 2 | bypass | bypass |
| '01' | 0 | 1 | bypass | bypass | bypass |
| '1' | 0 | bypass | bypass | bypass | bypass |

    - Bitwise occupancy coding

      - General

Subclause 9.2.10 applies when occtree_bitwise_coding is 1.

The neighbourhood-permuted node occupancy bitmap shall be coded as a sequence of individual occupancy_bit syntax elements.  Coding uses constraints on occupancy to infer the value of certain occupancy_bit syntax elements.

Entropy coding of each coded bit is contextualized by a combination of the coded bit index, previously coded occupancy_bit syntax elements, the reduced occupied neighbourhood pattern, the number of spatially adjacent child nodes in neighbouring nodes and a ternary prediction based upon the presence of neighbouring nodes.

      - Correspondence between the node occupancy bitmap and occupancy_bit

Bits of the neighbourhood-permuted node occupancy bitmap shall be coded in the order specified by Table 19.  Each occupancy_bit[ cbIdx ] syntax element codes the bit OccBitCodingOrder[ cbIdx ].

The expression OccBitIdx[ cbIdx ] is the bit position in the node occupancy bitmap of the bit coded by occupancy_bit[ cbIdx ].  For example, when OccNeighPat is 17, occupancy_bit[ 6 ] corresponds to the second bit (b1) of the node occupancy bitmap.

OccBitIdx[cbIdx] := OccBitIdxFromNpBit[OccBitCodingOrder[cbIdx]]

The expression OccBitLocC[ cbIdx ][ 𝑘 ] is the node-relative child location represented by occupancy_bit[ cbIdx ].

OccBitLocC[cbIdx][k] := OccLocC[OccBitIdx[cbIdx]][k]

The expression OccBitMap is the node occupancy bitmap.

OccBitMap :=
  OccBitMap = 0
  for (cbIdx = 0; cbIdx < 8; cbIdx++)
    OccBitMap = OccBitMap | (occupancy_bit[cbIdx] << OccBitIdx[cbIdx])

Table 19 — Order for coding bits of the neighbourhood-permuted node occupancy bitmap as occupancy_bit[ cbIdx ]

| cbIdx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OccBitCodingOrder[ cbIdx ] | 1 | 7 | 5 | 3 | 2 | 6 | 4 | 0 |

      - Presence of occupancy_bit

An occupancy bitmap bit shall not be coded if its value can be inferred to be set or unset.  The expression OccBitPresent[ cbIdx ] specifies whether occupancy_bit[ cbIdx ] is present.

OccBitPresent[cbIdx] := ¬(OccBitInferUnset[cbIdx] || OccBitInferSet[cbIdx])

      - Inference of an unset bit

An occupancy bitmap bit shall be inferred to be 0 when either:

- the bit represents an invalid child according to the node's coded axes (9.2.2.3), or

- the bit represents a child within an unoccupied plane signalled by planar occupancy coding.

When the expression OccBitInferUnset[ cbIdx ] is equal to 1, occupancy_bit[ cbIdx ] shall be inferred to be 0.

OccBitInferUnset[cbIdx] :=
     ¬AxisCoded[0] && OccBitLocC[cbIdx][0]
  || ¬AxisCoded[1] && OccBitLocC[cbIdx][1]
  || ¬AxisCoded[2] && OccBitLocC[cbIdx][2]
  || ¬PlanarFreeAxis[0] && OccBitLocC[cbIdx][0] ^ occ_plane_pos[0]
  || ¬PlanarFreeAxis[1] && OccBitLocC[cbIdx][1] ^ occ_plane_pos[1]
  || ¬PlanarFreeAxis[2] && OccBitLocC[cbIdx][2] ^ occ_plane_pos[2]

      - Inference of a set bit

An occupancy bitmap bit occupancy_bit[ cbIdx ] shall be inferred to be 1, as specified by the expression OccBitInferSet[ cbIdx ], when:

- the bit is the last present bit and all previous coded bits are 0, or

- the bit is the penultimate present bit, all previous coded bits are 0 and the node is required to have two child nodes, or

- the bit is in a plane identified as occupied by planar occupancy coding, the bit is the last bit in the plane and all previous bits in the plane are 0.

OccBitInferSet[cbIdx] :=
    PlanarEligible[0] && PopCnt(OccKnownZero & (0x0F << 4 × OccBitLocC[cbIdx][0])) == 3
  || PlanarEligible[1] && PopCnt(OccKnownZero & (0x33 << 2 × OccBitLocC[cbIdx][1])) == 3
  || PlanarEligible[2] && PopCnt(OccKnownZero & (0x55 << 1 × OccBitLocC[cbIdx][2])) == 3
  || cbIdx == 6 && PopCnt(OccKnown) == 0 && OccMinChildren == 2
  || cbIdx == 7 && PopCnt(OccKnown) == 0

The expression OccKnownMask is a bit mask that identifies bits of the node occupancy bitmap that have a known value prior to coding occupancy_bit[ cbIdx ].

OccKnownMask :=
  OccKnownMask = 0
  for (i = 0; i < cbIdx; i++)
    OccKnownMask |= 1 << OccBitIdx[i]
  for (i = 0; i < 8; i++)
    OccKnownMask |= OccBitInferUnset[i] << OccBitIdx[i]

The expression OccKnown is the partially coded node occupancy bitmap comprising the bits coded prior to occupancy_bit[ cbIdx ].

OccKnown :=
  OccKnown = 0
  for (i = 0; i < cbIdx; i++)
    OccKnown |= occupancy_bit[i] << OccBitIdx[i]

The expression OccKnownZero is a bitmap of occupancy bits that are known to be 0.

OccKnownZero := (0xFF ^ OccKnown) & OccKnownMask

      - Contextualization

        - General

Contextualization of occupancy_bit syntax elements is a two-stage process.  First, context discriminators are used to select a demi-CPM.  Then, the demi-CPM is used to select the CPM that codes the syntax element.

A demi-CPM is an 8-bit unsigned integer that models the probability of a coded zero-valued occupancy_bit syntax element.

- The values 0, 128 and 256 represent the probability of a zero bin as impossible, equiprobable and certain, respectively.  The values 0 and 256 can never be attained due to the operation of the probability models' update process.

        - State variables

Context selection is specified in terms of the following state variable:

- The array OccCtxSel; OccCtxSel[ selNeigh ][ cbIdx ][ selSib ][ selAdj ][ selPred ] is a demi-CPM, contextualized by selNeigh, cbIdx, selSib, selAdj and selPred.

        - Initial state

The demi-CPMs shall be initialized at the start of every GDU.

When slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1, initialization shall be performed according to the parsing state restoration process (11.6.2.2).

Otherwise (slice_entropy_continuation is 0 and slice_inter_entropy_continuation is 0), all elements of OccCtxSel shall be set to 127.

        - Determination of CtxIdxOccBit for the syntax element occupancy_bit

The expression OccCtxSelVar specifies the demi-CPM for the syntax element occupancy_bit[ CbIdx ] using:

- SelNeigh, the reduced occupied neighbourhood context discriminator (9.2.10.6.6);

- SelSib, the sibling occupancy context discriminator (9.2.10.6.7);

- SelAdj, the adjacent child neighbour context discriminator (9.2.10.6.8);

- SelPred, the neighbour-predicted occupancy context discriminator (9.2.10.6.9).

OccCtxSelVar := OccCtxSel[SelNeigh][CbIdx][SelSib][SelAdj][SelPred]

The CPM index, CtxIdxOccBit, shall be the value of the demi-CPM exclusive of the bottom three bits:

CtxIdxOccBit := OccCtxSelVar >> 3

        - Update after each coded occupancy_bit syntax element

After each coded occupancy_bit syntax element, its demi-CPM shall be updated.  The update specified by Table 20 supplies a value for incrementing or decrementing the probability of a zero bin based upon the upper four bits of the demi-CPM's value:

if (OccBitPresent[CbIdx])
  if (occupancy_bit[CbIdx])
    OccCtxSelVar += OccCtxSelUpdate[255 – OccCtxSelVar >> 4]
  else
    OccCtxSelVar −= OccCtxSelUpdate[OccCtxSelVar >> 4]

Table 20 — Values of OccCtxSelUpdate[ 𝑖 ]

| 𝑖 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| OccCtxSelUpdate[ 𝑖 ] | 0 | 1 | 1 | 2 | 4 | 7 | 9 | 11 | 14 | 16 | 19 | 23 | 22 | 18 | 13 | 6 |

        - Reduced occupied neighbourhood context discriminator

The reduced occupied neighbourhood context discriminator shall distinguish between different reduced occupied neighbourhood patterns (OccNeighPatR) depending upon the coded bit index (CbIdx) as specified by Table 21 as the expression SelNeigh.

Table 21 — Discriminated values SelNeigh for occupancy_bit[ CbIdx ] and OccNeighPatR

| CbIdx | OccNeighPatR |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 0 .. 3 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 4 .. 5 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 4 | 4 |
| 6 | 0 | 1 | 1 | 2 | 2 | 1 | 2 | 2 | 2 |
| 7 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

        - Sibling occupancy context discriminator

The sibling occupancy context discriminator shall distinguish between arrangements of previously coded/inferred siblings for the node coded by occupancy_bit[ CbIdx ] as specified by the expression SelSib:

- If there are no nodes present in the occupied neighbourhood pattern, discrimination shall be by the number of present child nodes identified by the syntax elements occupancy_bit[ 𝑖 ] with 𝑖 < CbIdx.

- If there is at least one node present in the occupied neighbourhood pattern, discrimination shall be by the combination of present child nodes identified by the syntax elements occupancy_bit[ 𝑖 ] with 𝑖 < CbIdx.

SelSib := OccNeighPat ? occPrevBits : PopCnt(occPrevBits)

The expression occPrevBits is the concatenation of occupancy_bit[ 𝑖 ] for 𝑖 < CbIdx.

occPrevBits :=
  occPrevBits = 0
  for (i = 0; i < CbIdx; i++)
    occPrevBits |= occupancy_bit[i] << i

        - Adjacent child neighbour context discriminator

The adjacent child neighbour context discriminator for child the node coded by occupancy_bit[ CbIdx ] is specified by the expression SelAdj.  When adjacent child neighbour contextualization is enabled (occtree_adjacent_child_enabled is 1), it distinguishes between contexts by:

- the number of child nodes from available, previously coded nodes in the same tree level (9.2.7.2) that adjoin the coded child by a face; and

- whether any of the available, previously coded nodes in the same tree level that adjoin the coded child node do not have a child node that also adjoins the coded child.

An example is illustrated in Figure 19.  The child node C0 of the coded node N is adjoined by a single child node.  There are two available previously coded nodes that adjoin C0, one of which does not contain a child node that also adjoins C0.

SelAdj := occtree_adjacent_child_enabled
  ? 2 × Min(2, adjCntC) + ((cbIdx ≤ 4 || adjCntC == 1) && adjUnocc)
  : 0

The expression adjOccN[ 𝑘 ] identifies whether there is a spatially adjacent node along the 𝑘-th axis within the occupied neighbourhood availability window.  Values for the expressions ds, dt and dv are specified in Table 22 for each axis 𝑘.

adjOccN[k] := ¬OccBitLocC[CbIdx][k] && OccNeigh[Ns + ds][Nt + dt][Nv + dv]

The expression adjOccC[ 𝑘 ] identifies whether there is a spatially adjacent child node along the 𝑘-th axis within the occupied neighbourhood availability window.  Values for the expressions ds, dt and dv are specified in Table 22 for each axis, 𝑘.

adjOccC[k] := adjOccN[k] && OccNodePresent[Dpth + 1][cs + ds][ct + dt][cv + dv] ≠ 0
  where
    cs := NsC + OccBitLocC[CbIdx][0]
    ct := NtC + OccBitLocC[CbIdx][1]
    cv := NvC + OccBitLocC[CbIdx][2]

Table 22 — Relative neighbour locations ( ds, dt, dv ) used in the computation of adjOccN[ 𝑘 ] and adjOccC[ 𝑘 ]

| 𝑘 | ds | dt | dv |
| --- | --- | --- | --- |
| 0 | −1 | 0 | 0 |
| 1 | 0 | −1 | 0 |
| 2 | 0 | 0 | −1 |

Key

| N | Coded node |
| --- | --- |
| C0 | Contextualized child with OccBitIdx[ cbIdx ] = 0 |

Figure 19 — Example of adjacent child neighbour context discrimination.

The expressions adjCntN and adjCntC are the number of spatially adjacent nodes and child nodes, respectively, that are within the occupied neighbourhood availability window.

adjCntN := adjOccN[0] + adjOccN[1] + adjOccN[2]
adjCntC := adjOccC[0] + adjOccC[1] + adjOccC[2]

The expression adjUnocc identifies whether there exists a spatially adjacent node within the occupied neighbourhood availability window that does not have a child node spatially adjacent to the coded child.

adjUnocc := adjCntN ≠ adjCntC

        - Neighbour-predicted-occupancy context discriminator

          - General

The neighbour-predicted-occupancy context discriminator shall, for eligible nodes (9.2.10.6.9.2), distinguish between three predictions for the presence of the child node coded by occupancy_bit[ CbIdx ].  The discriminator is specified by the expression SelPred.  The three predictions are that the node is present, not present, or that it is unpredictable.

SelPred := SelPredEligible ? OccIntraPred : 0

          - Eligibility

The discriminator shall only form a prediction for eligible nodes as specified by the expression SelPredEligible.  Eligible nodes shall have both:

- three free axes and

- a maximum log2 node dimension less than occtree_intra_pred_max_nodesize_log2.

SelPredEligible :=
  OccFreeAxisCnt == 3 && MaxVec(NodeSizeLog2) < occtree_intra_pred_max_nodesize_log2

          - Occupancy prediction

Occupancy prediction generates a ternary prediction for the presence of a child node identified by occupancy_bit[ CbIdx ] of a coded node.  The prediction is specified by the expression OccIntraPred.  It is based upon how many of the nodes that neighbour the coded node also adjoin the volume of the identified child node by a face, edge or corner (as illustrated by Figure 20):

- A child node shall be predicted to be not present if there are two or fewer adjoining nodes.

- A child node shall be predicted to be present if there is at least a threshold number of adjoining nodes.  The threshold is specified by the expression OccIntraThreshold.  The threshold is four nodes unless there are more than 13 neighbouring nodes; in which case the threshold is 5 nodes.

- Otherwise, the presence is unpredictable.

- The size of the child node volume is half the size of the neighbour nodes' in each dimension.

OccIntraPred := (OccAdjCnt ≤ 2) + 2 × (OccAdjCnt ≥ OccIntraThreshold)
OccIntraThreshold := 4 + (OccNeighCnt ≥ 14)

Figure 20 — Nodes that adjoin a child node by a face, edge or corner.

The expression OccAdj[ ds ][ dt ][ dv ] identifies whether a neighbouring node with a relative tree location ( ds, dt, dv ) to the coded node would adjoin the identified child volume.

OccAdj[ds][dt][dv] := (OccBitIdx[CbIdx] & adjMask) == adjLoc
  where
    adjMask := Morton(ds ≠ 0, dt ≠ 0, dv ≠ 0)
    adjLoc  := Morton(ds > 0, dt > 0, dv > 0)

The expression OccAdjCnt is the number of neighbours that adjoin the identified child volume.

OccAdjCnt := SumN26[neighAdj]
  where
    neighAdj[ds][dt][dv] := OccNeigh[Ns + ds][Nt + dt][Nv + dv] && OccAdj[ds][dt][dv]

The expression OccNeighCnt is the number of nodes that neighbour the coded node.

OccNeighCnt := SumN26[neighRel]
  where
    neighRel[ds][dt][dv] := OccNeigh[Ns + ds][Nt + dt][Nv + dv]

The expression SumN26[ expr ] sums the result of applying expr to the relative tree location of each of the 26 possible neighbouring nodes.

SumN26[expr] :=
  SumN26 = 0
  for (ds = −1; ds ≤ 1; ds++)
    for (dt = −1; dt ≤ 1; dt++)
      for (dv = −1; dv ≤ 1; dv++)
        if (ds ≠ 0 && dt ≠ 0 && dv ≠ 0)
          SumN26 += expr[ds][dt][dv]

        - Coding occupancy bits using OBUF

The occupancy bits of octree nodes are coded as follows.

- A first buffer is created for all OBUF instances according to subclause 12.3.  The OBUF buffer size obufBufferSize is set as 20000, the buffer depth obufLeafDepth of fully deployed trees is set as 4.

- The array of OBUF ACPMs is created according to subclause 12.2.2.

- All OBUF instances are created according to subclause 12.2 by using the octree buffer created, and the OBUF instances are divided into two types, sparse OBUF instances and non-sparse OBUF instances.  Each type of OBUF instances has specific sizes nBit1 and nBit2 to specify the sizes of contextual information info1 and info2 used as input when calling the OBUF instance.

- The sizes nBit1[j][i] and nBit2[j][i] for sparse OBUF instances are set according to Table 23and the sizes nBit1[j][i] and nBit2[j][i] for non-sparse OBUF instances are set according to Table 24, 𝑘, where index i represents the child node bi in current coded node, and index j represents intra prediction method (j = 0) or inter prediction method (j = 1) for octree coding.

Table 23 — the size values (nBit1[ j][ i ], nBit2[ j][ i ]) of sparse OBUF instance

| j | i |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | (4,12) | (7,12) | (7,12) | (6,12) | (4,12) | (7,12) | (7,12) | (6,12) |
| 1 | (4,12) | (7,12) | (7,12) | (6,12) | (4,12) | (7,12) | (7,12) | (6,12) |

Table 24 — the size values (nBit1[ j][ i ], nBit2[ j][ i ]) of non-sparse OBUF instance

| j | i |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | (6,13) | (6,13) | (6,13) | (6,11) | (6,13) | (6,13) | (6,13) | (6,11) |
| 1 | (6,13) | (6,13) | (6,13) | (6,11) | (6,13) | (6,13) | (6,13) | (6,11) |

- Optionally, for each child node, if the corresponding table among Table 25 to Table 32 is provided, the context array ctxIdxMap[][] of the created OBUF instances is initialized according to the provided table. Otherwise, for this child node, ctxIdxMap[][] is initialized with all the values set as 127.

- The array ctxIdxMap[ ][ ] corresponds to 8-bit context indices pointing (after right shift by 3) to OBUF ACPMs of the array obufCtxArray[ ].

Table 25 — Initial values of ctxIdxMap[ j][0] for child node b0 under intra octree coding

| j | ctxIdxMap[ j][0] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 127 | 17 | 82 | 38 | 127 | 105 | 141 | 81 |
| 8 .. 15 | 127 | 15 | 45 | 43 | 116 | 105 | 152 | 115 |
| 16 .. 23 | 127 | 53 | 21 | 20 | 127 | 127 | 127 | 37 |
| 24 .. 31 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 32 .. 39 | 171 | 186 | 170 | 240 | 182 | 209 | 223 | 240 |
| 40 .. 47 | 44 | 101 | 101 | 74 | 65 | 66 | 134 | 199 |
| 48 .. 55 | 47 | 27 | 141 | 113 | 126 | 61 | 240 | 151 |
| 56 .. 63 | 45 | 68 | 113 | 101 | 47 | 84 | 153 | 234 |

Table 26 — Initial values of ctxIdxMap[ j][ 1]for child node b1 under intra octree coding

| j | ctxIdxMap[ j][1] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 240 | 240 | 222 | 240 | 175 | 181 | 127 | 127 |
| 8 .. 15 | 120 | 152 | 132 | 116 | 57 | 127 | 127 | 127 |
| 16 .. 23 | 105 | 185 | 127 | 87 | 105 | 116 | 65 | 69 |
| 24 .. 31 | 66 | 105 | 58 | 43 | 44 | 49 | 18 | 15 |
| 32 .. 39 | 228 | 240 | 138 | 240 | 178 | 198 | 114 | 152 |
| 40 .. 47 | 173 | 240 | 204 | 127 | 70 | 141 | 127 | 127 |
| 48 .. 55 | 184 | 192 | 105 | 116 | 121 | 181 | 35 | 46 |
| 56 .. 63 | 58 | 87 | 114 | 73 | 51 | 15 | 101 | 40 |

Table 27 — Initial values of ctxIdxMap[ j][ 2]for child node b2under intra octree coding

| j | ctxIdxMap[ j][2] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 194 | 240 | 173 | 190 | 115 | 129 | 87 | 87 |
| 8 .. 15 | 168 | 161 | 116 | 92 | 127 | 127 | 26 | 96 |
| 16 .. 23 | 160 | 106 | 96 | 127 | 86 | 109 | 105 | 127 |
| 24 .. 31 | 116 | 68 | 80 | 27 | 116 | 116 | 46 | 19 |
| 32 .. 39 | 240 | 240 | 205 | 114 | 215 | 194 | 134 | 78 |
| 40 .. 47 | 225 | 182 | 191 | 141 | 122 | 127 | 58 | 127 |
| 48 .. 55 | 200 | 214 | 124 | 89 | 188 | 161 | 91 | 59 |
| 56 .. 63 | 126 | 126 | 74 | 152 | 80 | 96 | 59 | 127 |

Table 28 — Initial values of ctxIdxMap[ j][ 3] for child node b3under intra octree coding

| j | ctxIdxMap[ j][3] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 59 | 121 | 160 | 210 | 171 | 211 | 240 | 231 |
| 8 .. 15 | 127 | 56 | 149 | 125 | 127 | 115 | 230 | 204 |
| 16 .. 23 | 55 | 127 | 78 | 192 | 127 | 182 | 197 | 218 |
| 24 .. 31 | 35 | 39 | 15 | 72 | 96 | 87 | 151 | 139 |
| 32 .. 39 | 46 | 141 | 152 | 240 | 114 | 162 | 240 | 240 |
| 40 .. 47 | 87 | 69 | 127 | 96 | 44, | 67 | 129 | 155 |
| 48 .. 55 | 53 | 105 | 141 | 73 | 96 | 105 | 198 | 128 |
| 56 .. 63 | 15 | 35 | 96 | 57 | 127 | 96 | 127 | 96 |

Table 29 — Initial values of ctxIdxMap[ j][ 4] for child node b4under intra octree coding

| j | ctxIdxMap[ j][4] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 23 | 30 | 130 | 66 | 139 | 127 | 30 | 105 |
| 8 .. 15 | 113 | 127 | 87 | 127 | 127 | 127 | 127 | 127 |
| 16 .. 23 | 166 | 146 | 70 | 15 | 209 | 116 | 141 | 90 |
| 24 .. 31 | 114 | 138 | 71 | 15 | 127 | 127 | 127 | 127 |
| 32 .. 39 | 204 | 240 | 198 | 219 | 232 | 240 | 142 | 240 |
| 40 .. 47 | 151 | 139 | 87 | 127 | 209 | 190 | 43 | 141 |
| 48 .. 55 | 141 | 181 | 116 | 127 | 240 | 210 | 88 | 127 |
| 56 .. 63 | 73 | 170 | 65 | 61 | 140 | 194 | 48 | 65 |

Table 30 — Initial values of ctxIdxMap[ j][ 5] for child node b5under intra octree coding

| j | ctxIdxMap[ j][5] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 240 | 99 | 240 | 69 | 189 | 96 | 105 | 80 |
| 8 .. 15 | 154 | 233 | 152 | 141 | 127 | 152 | 127 | 127 |
| 16 .. 23 | 166 | 48 | 57 | 15 | 97 | 41 | 43 | 15 |
| 24 .. 31 | 127 | 116 | 127 | 127 | 127 | 85 | 127 | 127 |
| 32 .. 39 | 235 | 214 | 177 | 154 | 240 | 240 | 161 | 61 |
| 40 .. 47 | 219 | 185 | 152 | 208 | 157 | 90 | 127 | 127 |
| 48 .. 55 | 117 | 138 | 69 | 30 | 154 | 80 | 62 | 15 |
| 56 .. 63 | 141 | 121 | 127 | 127 | 127 | 41 | 127 | 105 |

Table 31 — Initial values of ctxIdxMap[ j][ 6]  for child node b6under intra octree coding

| j | ctxIdxMap[ j][6] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 227 | 199 | 188 | 103 | 212 | 141 | 205 | 55 |
| 8 .. 15 | 240 | 240 | 210 | 141 | 178 | 70 | 127 | 127 |
| 16 .. 23 | 240 | 84 | 139 | 73 | 139 | 60 | 127 | 59 |
| 24 .. 31 | 161 | 127 | 127 | 127 | 80 | 65 | 127 | 127 |
| 32 .. 39 | 201 | 195 | 127 | 69 | 175 | 80 | 87 | 39 |
| 40 .. 47 | 115 | 240 | 127 | 175 | 116 | 168 | 127 | 127 |
| 48 .. 55 | 115 | 96 | 42 | 23 | 65 | 65 | 49 | 15 |
| 56 .. 63 | 96 | 141 | 127 | 127 | 105 | 127 | 127 | 127 |

Table 32 — Initial values of ctxIdxMap[ j][ 7] for child node b7under intra octree coding

| j | ctxIdxMap[ j][7] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 141 | 141 | 139 | 146 | 127 | 144 | 177 | 218 |
| 8 .. 15 | 127 | 63 | 127 | 115 | 127 | 164 | 240 | 194 |
| 16 .. 23 | 127 | 127 | 73 | 97 | 127 | 190 | 186 | 128 |
| 24 .. 31 | 73 | 16 | 15 | 88 | 116 | 127 | 80 | 161 |
| 32 .. 39 | 127 | 116 | 116 | 240 | 42 | 166 | 161 | 230 |
| 40 .. 47 | 96 | 47 | 127 | 127 | 58 | 88 | 116 | 109 |
| 48 .. 55 | 105 | 116 | 15 | 61 | 15 | 80 | 73 | 155 |
| 56 .. 63 | 15 | 15 | 15 | 45 | 36 | 73 | 57 | 121 |

- Then, the advanced occupied neighbourhood pattern OccAdvNeiPati (9.2.7.6) is used as the contextual information CI for entropy coding occupancy of child node bi of current coded node as follows.

  - CI is split into a primary part CI1 and a second part CI2 to derive the contextural information info1 and info2 for dynamic OBUF (12.5). CI1 is a first representation of occupancy information of the set of neighboring nodes of current child node and contains the first nBit1 bits of OccAdvNeiPati.  CI2 is a finer representation of the occupancy information and contains the remaining bits of OccAdvNeiPati. CI1 is set as the first contextual information info1, and CI2 is set as the second contextual information info2.

  - For example, to code child node b0 of the coded node,

    - if the occupied neighbour nodes of a child node are not sparse, the contextual information is set as below,

CI1= OccAdvNeiPat0 >> 13

CI2= OccAdvNeiPat0 & 0x1FFF

    - Otherwise, the contextual information is set as below.

CI1= OccAdvNeiPat0 >> 12

CI2= OccAdvNeiPat0 & 0x0FFF

- Then, each bit of child node occupancy of current node is decoded according to subclause 12.4 by calling the OBUF instances of octree, and using info1, info2, ctxIdxMap[ ][ ] and obufCtxArray[ ].  The type (sparse or non-sparse) of called OBUF instances depends on the first bit of OccAdvNeiPati (9.2.7.6), which indicates if the occupied neighbour nodes of coded child node are sparse or not.

    - Planar occupancy coding

      - General

Subclause 9.2.11 applies when occtree_planar_enabled is 1.

Planar occupancy coding decomposes the node occupancy bitmap into axis-aligned planes.  Each coded axis has two perpendicular planes that child nodes can occupy as illustrated by Table 33.  For each planar-eligible coded axis (9.2.11.5), planar occupancy coding specifies whether one of the two planes is unoccupied.  Plane occupancy is then used by bitwise occupancy coding to constrain and infer the coding of bits in the node occupancy bitmap.

There shall be at least one child node in each occupied plane.

- The definition of an occupancy tree node requires that at least one plane is occupied along each coded axis.

For example, if a node has three planar-eligible coded axes, there is a total of six axis-aligned planes.  Along the S axis (𝑘 = 0), information about the occupied state of the two T-V planes is coded.

Table 33 — Plane, perpendicular to each planar axis, 𝑘

| 𝑘 | Planar axis | Plane axes |
| --- | --- | --- |
| 0 | S | T-V |
| 1 | T | S-V |
| 2 | V | S-T |

      - Syntax element semantics

planar_copy_mode specifies, when present, whether (when 1) or not (when 0) the values of occ_plane_pos[ 𝑘 ] indicating the occupancy of the child nodes are copied from the corresponding node in the reference frame.

multi_planar_flag specifies, when present, whether the positions of child nodes in the node occupancy bitmap shall locate at the intersection of planes (equal to 1) or not (equal to 0).

occ_single_plane[ 𝑘 ] specifies, when present, whether (when 1) the locations of child nodes in the node occupancy bitmap shall occupy a single plane or (when 0) both planes perpendicular to the 𝑘-th axis.  When equal to 1, the location of the single plane is specified by occ_plane_pos[ 𝑘 ].  When not present, the child nodes can be located in either or both planes perpendicular to the 𝑘-th axis.  The number of occupied planes is illustrated in Table 34.

Table 34 — Interpretation of PlanarEligible[ 𝑘 ] and occ_single_plane[ 𝑘 ]

| PlanarEligible[ 𝑘 ] | occ_single_plane[ 𝑘 ] | № occupied planes | PlanarMinPlanes[ 𝑘 ] |
| --- | --- | --- | --- |
| 0 | not present | 1 or 2 | 1 |
| 1 | 0 | 2 | 2 |
| 1 | 1 | 1 | 1 |

occ_plane_pos[ 𝑘 ] specifies the node-relative location along the 𝑘-th axis for the occupied plane specified by occ_single_plane[ 𝑘 ] equal to 1.

Examples of planar occupancy are illustrated in Figure 21.  Each entry shows the child node indices for a node with three coded axes.  If an axis 𝑘 is eligible for planar coding and occ_single_plane[ 𝑘 ] is 0, the two occupied planes are marked with a dotted line.  If occ_single_plane[ 𝑘 ] is 1, the unoccupied plane is marked by hatching and a dashed (red) line, with its child indices in grey; the occupied plane is not marked for clarity.  In the case where three axes are eligible and each has occ_single_plane equal to 1, there is only a single child node present; its location is fully constrained by the planes.

Figure 21 — Example planar constraints on node occupancy bitmap.

      - Minimum number of child nodes

Planar occupancy coding can require that the coded node has a minimum of either one or two child nodes, as specified by the expression PlanarMinChildren.

A node shall have at least two child nodes if, for any planar-eligible axis 𝑘, occ_single_plane[ 𝑘 ] specifies that there shall be a minimum of two occupied planes; otherwise, the node shall have at least one child node.

PlanarMinChildren := MaxVec(PlanarMinPlanes)

      - Free axes

A free axis is a coded axis whose occupancy is not constrained to a single plane by occ_single_plane, as specified by the expression PlanarFreeAxis[ 𝑘 ].

PlanarFreeAxis[k] := ¬PlanarEligible[k] || PlanarMinPlanes[k] == 2

In Figure 21 a free axis is an axis with either two marked planes or no marked planes.  In the case where three axes are eligible, of which two have occ_single_plane equal to 1, PlanarMinChildren is 2.

      - Per-axis eligibility

        - Condition

Only certain axes are eligible for planar occupancy coding.  Eligibility for the 𝑘-th axis is specified by the expression PlanarEligible[ 𝑘 ].  Eligibility shall be determined after any applicable update to the eligibility state (9.2.11.5.2).

An axis is not eligible for planar coding when either planar occupancy coding is disabled, or the axis is not coded, or geo_planar_idcm_angular_disabled is equal to 1 and occ_direct_node is 1.  Otherwise, the determination of eligibility depends upon the use of the angular coding and whether the node is eligible for angular contextualization (9.2.13.7.2) as specified in Table 35.

PlanarEligible[k] :=
  occtree_planar_enabled && AxisCoded[k]
  && (geom_angular_enabled ? PlanarEligibleByAng[k] : PlanarEligibleByDensity[k])

PlanarEligibleByAng[k] :=
  AngularEligible ? k == 2 || k == AzimuthAxis
                  : 0

Table 35 — Method to determine eligibility for an axis

| Axis | 𝑘 | Angular coding disabled | Angular coding enabled |  |
| --- | --- | --- | --- | --- |
|  |  |  | AngularEligible == 0 | AngularEligible == 1 |
| S | 0 | PlanarEligibleByDensity[ 0 ] | Not eligible | AzimuthAxisIsS |
| T | 1 | PlanarEligibleByDensity[ 1 ] | Not eligible | AzimuthAxisIsQ |
| V | 2 | PlanarEligibleByDensity[ 2 ] | Not eligible | Eligible |

Axes whose eligibility is determined by the expression PlanarEligibeByDensity[ 𝑘 ] are eligible if:

- the density of the points in the tree level at depth dpth is sparse enough.

PlanarEligibleByDensity[k]:= PointDensity[dpth ] < 13

The expression PointDensity[dpth ] is a factor that identifies the density of the points in the tree level at depth dpth:

PointDensity[dpth ]:=	(slice_num_points_minus1 + 1 – DirectNodePointCnt) × 10                        	/ OccNodeCnt[ dpth ]

        - State variable and update

When octree_planar_neigh_prediction_enabled is 1, eligibility is specified in terms of the following state variables:

- The sparse array NodeOccMap of node occupancy bitmaps; NodeOccMap[ dpth ][ ns ][ nt ][ nv ] is the coded node occupancy bitmap of the node located at ( ns, nt, nv ) in the tree level at depth dpth.

At the end of each occupancy_tree_node syntax structure, the state shall be updated for the next tree level:

NodeOccMap[Dpth][Ns][Nt][Nv] = OccupancyMap.

      - Previous coded node for contextualization of occ_plane_pos

        - General

Subclause 9.2.11.6 does not apply when occtree_planar_buffer_disabled is 1.

Planar contextualization of occ_plane_pos[ 𝑘 ] can use the following information about the previous planar-eligible coded node that is located in the same plane (9.2.11.6.2) as the coded node:

- The zone within the plane that the node resides (9.2.11.6.3).

- The values for occ_single_plane[ 𝑘 ] and occ_plane_pos[ 𝑘 ].

        - Identification of the plane

The plane normal to the 𝑘-th axis of a coded node is identified by its location along the axis modulo 214.

PlanarNodeAxisLoc[k] := Nloc[k] & 0x3FFF

        - Zone within a plane

The plane normal to the 𝑘-th axis of a coded node is partitioned into zones according to the l∞ norm of the node location within the plane.  The expression PlanarNodeZone[ 𝑘 ] identifies the zone for the coded node.

PlanarNodeZone[k] :=
  k == 0 ? Max(Nt & 0xF8, Nv & 0xF8) >> 3 :
  k == 1 ? Max(Ns & 0xF8, Nv & 0xF8) >> 3 :
  k == 2 ? Max(Ns & 0xF8, Nt & 0xF8) >> 3 : na

Figure 22 illustrates the partitioning of an S-T plane (𝑘 = 0) according to node location.

Figure 22 — S-T plane divided into zones.

        - State variables

Information about previous planar-eligible coded nodes is specified in terms of the following state variables; the indexes 𝑘 and axisLoc identify the location of a plane along the 𝑘-th axis:

- The array PrevPlanarNodeZone; PrevPlanarNodeZone[ 𝑘 ][ axisLoc ] is the plane zone of the previous planar-eligible node in the identified plane.

- The array PrevOccSinglePlane; PrevOccSinglePlane[ 𝑘 ][ axisLoc ] is the value of occ_single_plane[ 𝑘 ] for the previous planar-eligible node in the identified plane.

- The array PrevOccPlanePos; PrevOccPlanePos[ 𝑘 ][ axisLoc ] is the value of occ_plane_pos[ 𝑘 ] for the previous planar-eligible node in the identified plane.

        - Initial state

At the start of every occupancy tree level, every element of PrevOccSinglePlane shall be initialized to 0.

        - State update at the end of each node

After each occupancy_tree_node syntax structure, the state shall be updated for each planar-eligible axis:

for (k = 0; k < 3; k++)
  if (PlanarEligible[k]) {
    PrevPlanarNodeZone[k][PlanarNodeAxisLoc[k]] = PlanarNodeZone[k]
    PrevOccSinglePlane[k][PlanarNodeAxisLoc[k]] = occ_single_plane[k]

    if (occ_single_plane[k])
      PrevOccPlanePos[k][PlanarNodeAxisLoc[k]] = occ_plane_pos[k]
  }

      - Determination of CtxIdxPlanePos for occ_plane_pos[ 𝑘 ]

        - Case for angular-ineligible axes

Contextualization of occ_plane_pos[ 𝑘 ] for nodes not eligible for angular contextualization (AngularEligible is 0) is specified by the expression CtxIdxPlanePos.  When octree_planar_neigh_prediction_enabled is 1, CtxIdxPlanePos is set equal to CtxIdxNeighPlanePos using the planar information of neighbors (9.2.11.7.2).

CtxIdxPlanePos := octree_planar_neigh_prediction_enabled ? CtxIdxNeighPlanePos:
  (occtree_planar_buffer_disabled || ¬PrevOccSinglePlane[k][PlanarNodeAxisLoc[k]]
    ? adjPlaneCtxInc
    : 12 × k + 4 × adjPlaneCtxInc + 2 × zoneCtxInc + prevPlanePosCtxInc + 3)
.

The variable OccupancyMapP for a node specifies the coded node occupancy bitmap of its parent node.

The expression adjPlaneCtxInc discriminates by whether nodes have adjoining neighbours on a single side along the 𝑘-th axis, and if so, on which of the two sides they are present.  Adjoining neighbours are:

- those along the 𝑘-th axis identified by the corresponding bits of the occupied neighbourhood pattern (adjNeighHL); and

- when the node is in the lower 𝑘-th axis plane of its parent, the sibling nodes in the corresponding upper plane (identified by OccPlaneMask[ 1 ][ 𝑘 ]).  The bit masks  OccPlaneMask[ planeLoc ][ 𝑘 ] that identify planes in an occupancy bitmap are specified by Table 36.

adjPlaneCtxInc := (adjNeighHL | sibPlaneH << 1) % 3
  where
    adjNeighHL := (OccNeighPat >> 2 × k) & 3
    sibPlaneH  := (Nloc[k] & 1) ≠ 1 && (OccupancyMapP & OccPlaneMask[1][k]) ≠ 0

- Whenever occtree_coded_axis[ Dpth − 1 ][ 𝑘 ] is 0, sibPlaneH is always 0.

Table 36 — Bit masks OccPlaneMask[ planeLoc ][ 𝑘 ] that identify planes of a node occupancy bitmap

| 𝑘 | 0 | 1 | 2 |
| --- | --- | --- | --- |
| OccPlaneMask[ 0 ][ 𝑘 ] | 0x0F | 0x33 | 0x55 |
| OccPlaneMask[ 1 ][ 𝑘 ] | 0xF0 | 0xCC | 0xAA |

If occtree_planar_buffer_disabled is 0, contextualization uses information about the previous planar-eligible node in the plane identified by the coded node location (9.2.11.6.2).

The expression zoneCtxInc discriminates by whether the coded node is within ±1 zones of the identified previous node.

zoneCtxInc := Abs(a − b) > 1
  where
    a := PrevPlanarNodeZone[k][PlanarNodeAxisLoc[k]]
    b := PlanarNodeZone[k]

The expression prevPlanePosCtxInc discriminates by the occupied plane position of the identified previous node.

prevPlanePosCtxInc := PrevOccPlanePos[k][PlanarNodeAxisLoc[k]]

        - Neighbourhood planar information based prediction

The expression adjOccC[ n] identifies the occupancy bitmaps of the spatial adjacent nodes within the availability windows.  The relative location values ds, dt and dv of the  n-th adjacent node for the expression are specified in Table 37.

adjOccC[n] := OccNodePresent[Dpth][Ns + ds][Nt + dt][Nv + dv] ≠ 0 ? NodeOccMap[Dpth][Ns + ds][Nt + dt][Nv + dv] : 0

The expression adjOccSinglePlane[ k] identifies the occupied neighbourhood planar pattern along the 𝑘-th axis. It is a linear combination of spatial adjacent nodes coded in the same tree level that are available and adjoin the coded node.  The relative location values ds, dt and dv of the  n-th adjacent node for the expression are specified in Table 37.  An occupancy tree node with no spatially adjacent nodes has an occupied neighbourhood planar pattern equal to 0.

adjOccSinglePlane[k]:= 0

for (n = 0; n < 7; n++)
  if (OccNodePresent[Dpth][Ns + ds][Nt + dt][Nv + dv] == 0)
    adjOccSinglePlane[k] <<= 1
  else {
    plane0 = (adjOccC[n] & OccPlaneMask[0][k]) ≠ 0
    plane1 = (adjOccC[n] & OccPlaneMask[1][k]) ≠ 0
    hasSinglePlane = plane0 ^ plane1
    adjOccSinglePlane[k] |= hasSinglePlane
  }

The expression adjOccPlanePos[ k] identifies the occupied neighbourhood planar position pattern along the 𝑘-th axis.  It is a linear combination of spatial adjacent nodes coded in the same tree level that are available and adjoin the coded node.  The relative location values ds, dt and dv of the  n-th adjacent node for the expression are specified in Table 37.  An occupancy tree node with no spatially adjacent nodes has an occupied neighbourhood planar position pattern equal to 0.

adjOccPlanePos[k] :=0

for (n = 0; n < 7; n++)
  if (OccNodePresent[Dpth][Ns + ds][Nt + dt][Nv + dv] == 0)
    adjOccPlanePos[k] << =1
  else{
    plane0 = (adjOccC[n] & OccPlaneMask[0][k]) ≠ 0
    plane1 = (adjOccC[n] & OccPlaneMask[1][k]) ≠ 0
    hasSinglePlane = plane0 ^ plane1
    adjOccPlanePos[k] |= hasSinglePlane & OccPlaneMask[1][k]
    adjOccPlanePos[k] <<= 1
  }

Table 37 — Relative locations (ds, dt, dv) used in the computation of adjOccC[ n], adjOccSinglePlane[ k], adjOccPlanePos[ k] and neighAvailable

| n | ds | dt | dv |
| --- | --- | --- | --- |
| 0 | −1 | 0 | 0 |
| 1 | 0 | −1 | 0 |
| 2 | 0 | 0 | −1 |
| 3 | -1 | -1 | 0 |
| 4 | -1 | 1 | -1 |
| 5 | 0 | -1 | -1 |
| 6 | -1 | -1 | -1 |

The expression adjOccNeighPat identifies the bitmap for the spatial nodes within the availability windows.  The relative location values ds, dt and dv of the  n-th adjacent node for the expression are specified in Table 38.

adjOccNeighPat: =0
for (n = 0; n < 12; n++){
  if (OccNodePresent[Dpth][Ns + ds][Nt + dt][Nv + dv] ≠ 0)
    adjOccNeighPat |= 1
  adjOccNeighPat <<= 1 
}

Table 38 — Relative locations ( ds, dt, dv ) used in the computation of adjOccNeighPat

| n | ds | dt | dv |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 |
| 2 | 0 | 0 | 1 |
| 3 | 1 | 1 | 0 |
| 4 | 1 | 0 | 1 |
| 5 | 0 | 0 | -1 |
| 6 | 1 | -1 | 0 |
| 7 | 0 | 1 | 1 |
| 8 | 0 | 1 | -1 |
| 9 | 0 | -1 | 1 |
| 10 | -1 | 1 | 0 |
| 11 | -1 | 0 | 1 |

The expression neighAvailable identifies whether there are spatial nodes within the availability windows.  The relative location values ds, dt and dv of the  n-th adjacent node for the expression are specified in Table 38.

neighAvailable := false
for (n = 0; n < 7; n++)
  if (OccNodePresent[Dpth][Ns + ds][Nt + dt][Nv + dv] ≠ 0)
    neighAvailable |= adjOccC[n] ≠ 0

The expression primaryCtx discriminates by spatial nodes’ geometry primary information.  The expression minorCtx discriminates by spatial nodes’ geometry minor information. primaryCtx and minorCtx are used to construct contextual information info1 and info2 used in Dynamic OBUF according to 9.2.11.7.3.

primaryCtx: = neighAvailable ? zoneCtxInc << 7 + prevPlanePosCtxInc << 6 + ((adjOccPlanePos[k] & 0x70) >> 4) << 3+ ((adjOccSinglePlane[k]&0x70) >> 4) :
zoneCtxInc << 6 + prevPlanePosCtxInc << 5 + adjPlaneCtxInc << 3 + ((adjOccNeighPat & 0xe00) >> 9) + (0x01 << 7)

minorCtx: = neighAvailable ? adjPlaneCtxInc << 8 + (adjOccPlanePos[k] & 0xf) << 4 + (adjOccSinglePlane[k] & 0xf) : 
(adjOccNeighPat & 0x1ff) +(0x01 << 9)

CtxIdxNeighPlanePos is derived from the Dynamic_OBUF based primaryCtx and minorCtx according to 9.2.11.7.3.

        - Coding occ_plane_pos[ 𝑘 ] using OUBF

- A first buffer is created for all OBUF instances according to 12.3.  The OBUF buffer size obufBufferSize is set as 20000, the buffer depth obufLeafDepth of fully deployed trees is set as 4.

- The array of OBUF ACPMs is created according to 12.2.2.

- All OBUF instances are created according to 12.2 by using the octree buffer created, and the OBUF instances are divided into two types, sparse OBUF instances and non-sparse OBUF instances.  Each type of OBUF instances has specific sizes nBit1 and nBit2 to specify the sizes of contextual information info1 and info2 used as input when calling the OBUF instance.

- The sizes nBit1 and nBit2 for sparse OUBF instances are set 10 and 8. For each axes, ctxIdxMap[][] is initialized with all the values set as 127.

- The array ctxIdxMap[ ][ ] corresponds to 8-bit context indices pointing (after right shift by 3) to OBUF ACPMs of the array obufCtxArray[ ].

- Then the primaryCtx and minorCtx (9.2.11.7.2) to derive the contextual information info1 and info2 for dynamic OBUF (12.5).  primaryCtx is set as the first contextual information info1, and minorCtx is set as the second contextual information info2.

- Then, occ_plane_pos[ 𝑘 ] is decoded according to 12.4 by calling the OBUF instances of octree, and using info1, info2, ctxIdxMap[ ][ ] and obufCtxArray[ ].  The type (sparse or non-sparse) of called OBUF instances depends on the primaryCtx and minorCtx (9.2.11.7.2), which indicates the spatial nodes’ geometry primary information and the minor information.

        - Case for angular-eligible axes

Contextualization of occ_plane_pos[ 𝑘 ] for nodes eligible for angular contextualization (AngularEligible is 1) is specified by 9.2.13.7.

      - Multiple axes eligibility

Eligibility for planar coding of multiple axes is specified by the expression MultiPlanarEligible.

NumEligibleAxes indicates the number of axes that are eligible for planar coding. It is used to determine the value of MultiPlanarEligible .

NumEligibleAxes = PlanarEligible[0] + PlanarEligible[1] + PlanarEligible[2]
MultiPlanarEligible = NumEligibleAxes > 1 ? 1 : 0

      - occ_single_plane inference

When multi_planar_flag is present or when planar_copy_mode is present, inferences on the value of occ_single_plane[ 𝑘 ] can be made.

PlanarInferred[ 𝑘 ] indicates that occ_single_plane[ 𝑘 ] can be inferred (equal to 1) or not (equal to 0).

When planar_copy_mode equals to 1 or multi_planar_flag equals to 1, occ_single_plane[ 𝑘 ] can be inferred if 𝑘-th axis is eligible for planar coding.

if(planar_copy_mode)
  for(k = 0; k < 3; k++)
    PlanarInferred[k] = 1
if(multi_planar_flag)
  for(k = 0; k < 3; k++)
    PlanarInferred[k] = PlanarEligible[k]

When planar_copy_mode equals to 0 and multi_planar_flag equals to 0, occ_single_plane[ 𝑘 ] can be inferred if 𝑘-th axis is eligible for planar coding and present NumEligibleAxes  – 1 occ_single_plane[ j ] which equals to 1 and j < 𝑘.

if(¬multi_planar_flag){
  count = 0
  for(k = 0; k <3; k++){
    for(j = 0; j < k; j++)
      if(PlanarEligible[j])
        count += occ_single_plane[j] ? 1 :0
    PlanarInferred[k] = (count == NumEligibleAxes – 1) && PlanarEligible[k] ? 1 :0
  }
}

If PlanarInferred[ 𝑘 ] equal to 1, occ_single_plane[ 𝑘 ] will be inferred as follows:

if(PlanarInferred[k])
  occ_single_plane[k] = planar_copy_mode ? PlaneRef[k] : multi_planar_flag

      - Determination of AllowPlanarCopyMode for planar_copy_mode

Determination of signalling of planar_copy_mode is specified by the expression AllowPlanarCopyMode.

AllowPlanarCopyMode = isInter && OccupancyIsPredictable && planarEligibile
  where 
    planarEligible = PlanarEligible[0] || PlanarEligible[1] || PlanarEligible[2]

      - occ_plane_pos inference

When planar_copy_mode is present, inferences on the value of occ_plane_pos[ 𝑘 ] may be made.

PlanarPosInferred[ 𝑘 ] indicates that occ_plane_pos [ 𝑘 ] can be inferred (equal to 1) or not (equal to 0).

When AllowPlanarCopyMode equals to 1, occ_plane_pos[ 𝑘 ] can be inferred if planar copy mode is indicated or if the 𝑘-th axis is last eligible axis for planar coding and planar coding of previous axes match that of the reference frame.

prevDirMatch = true
for(k = 0; k < 3; k++) {
  if(planar_copy_mode)
    PlanarPosInferred[k] = 1
  else if(AllowPlanarCopyMode && k == lastDirIdx && prevDirMatch && PlaneRef[k])
    PlanarPosInferred[k] = 1
  else
    PlanarPosInferred[k] = 0
  prevDirMatch = prevDirMatch & (occ_single_plane[k] == PlaneRef[k]) && (occ_plane_pos[k] == PlanePosRef[k])
}
  where
    lastDirIdx = PlanarEligible[2] ? 2 : (PlanarEligible[1] ? 1 : 0)

If PlanarPosInferred[ 𝑘 ] equal to 1, occ_plane_pos[ 𝑘 ] will be inferred as follows:

if(PlanarPosInferred[k])
  occ_plane_pos[k] = planar_copy_mode ? PlanePosRef[k] : (PlanePosRef[k] ? 0 : 1)

      - Determination of CtxIdxPlanarCopyMode for planar_copy_mode

Contextualization of planar_copy_mode is specified by the expression CtxIdxPlanarCopyMode.

CtxIdxPlanarCopyMode = 8 × (4 × (eligibilityPlanarSum – 1) + matchDirSum) + planarRefSum
  where
    eligibilityPlanarSum = PlanarEligible[0] + PlanarEligible[1] + PlanarEligible[2]
    matchedDirSum = matchedDir[0] + matchedDir[1] + matchedDir[2]
    planeRefSum = PlaneRef[0] << 2 + PlaneRef[1] << 1 + PlaneRef[0]

The variable matchedDir[] is used to determine whether the planar information in the reference frame is equal to the planar information of the closest zone in the planar buffer.

matchedDir[k] = 0
if(PlanarEligible[k])
  matchedDir[k] = PrevOccSinglePlane[k][PlanarNodeAxisLoc[k]] == PlanarRef[k] 
                  && PrevOccPlanePos[k][PlanarNodeAxisLoc[k]] == PlanePosRef[k]

      - Derivation of PlanarRef and PlanePosRef

The variables PlanarRef and PlanePosRef denote the planar information of the collocated node in the reference frame.

PlanarRef[0] = (PredOccBitMap & 0x0f) ≠ (PredOccBitMap & 0xf0)
PlanarRef[1] = (PredOccBitMap & 0x33) ≠ (PredOccBitMap & 0xcc)
PlanarRef[2] = (PredOccBitMap & 0x55) ≠ (PredOccBitMap & 0xaa)
PlanarPosRef[0] = PlanarRef[0] && (PredOccBitMap & 0xf0) > 0
PlanarPosRef[1] = PlanarRef[1] && (PredOccBitMap & 0xcc) > 0
PlanarPosRef[2] = PlanarRef[2] && (PredOccBitMap & 0xaa) > 0

    - Direct nodes

      - General

Subclause 9.2.12 applies when occtree_direct_coding_mode is not 0.

Certain occupancy tree nodes may immediately code point positions as a direct node, instead of coding a node occupancy bitmap for subsequent traversal.  A direct node can represent either two distinct point positions, or a single position that is identical for every represented point.

A direct node codes a position as a residual relative to the node position.

The number of points coded in direct nodes is counted cumulatively, DirectNodePointCnt.  The variable is initialized to 0 at the start of every slice.

Direct coding is limited to nodes that are both eligible and not prohibited by the planar direct node rate limit.  Eligibility shall be determined for each occupancy tree node based upon the degree of spatial isolation as specified in 9.2.12.3.

      - Syntax element semantics

occ_direct_node equal to 1 specifies that the occupancy tree node is a direct node that codes the position of at least one point.  When occ_direct_node is not present, it shall be inferred to be 0.

direct_point_cnt_eq2 equal to 1 specifies that the direct node codes two point positions.  direct_point_cnt_eq2 equal to 0 specifies that the occupancy tree node codes a single point position for one or more points.

direct_dup_point_cnt plus 1 specifies, when present, the number of points the direct node represents when direct_point_cnt_eq2 is 0.  When direct_dup_point_cnt is not present, it shall be inferred to be 0.

direct_joint_prefix[ 𝑘 ] specifies a sequence of identical MSBs in the 𝑘-th component of two coded position residuals.  The MSB position of the syntax element value indicates the number of position bits coded by the syntax element and does not form part of the reconstructed point position.

direct_joint_diff_bit[ 𝑘 ] specifies, when direct_joint_prefix[ 𝑘 ] is present, the value of a bit in the binary representation of the 𝑘-th component of the two coded position residuals.  The bit is the most significant non-identical bit of the two coded residual components.  Its value is that for the first point.  When direct_joint_diff_bit[ 𝑘 ] is not present, it shall be inferred to be 0.

direct_rem[ dnPt ][ 𝑘 ], direct_rem_st_ang[ dnPt ] and direct_rem_v_ang[ dnPt ] specify the remaining position bits of the dnPt-th point.  When present, direct_rem codes the 𝑘-th component, direct_rem_st_ang either the 𝑠- or 𝑡-component depending upon the node location and direct_rem_v_ang codes the 𝑣-component.  When not present, they shall be inferred to be 0.

direct_v_ang_resid_abs[ dnPt ] and direct_v_ang_resid_sign[ dnPt ] together specify, when present, the residual of the 𝑣-component of dnPt-th point, for a prediction made from a selected beam’s elevation and vertical offset.  When not present, they shall be inferred to be 0.  The residual of the 𝑣-component of dnPt-th point is specified by the expression DirectVAngResid[ dnPt ].

DirectVAngResid[dnPt] :=
    (1 − 2 × direct_v_ang_resid_sign[dnPt]) × direct_v_ang_resid_abs[dnPt]

beam_idx_resid_abs[ dnPt ] and beam_idx_resid_sign[ dnPt ] together specify the index of an enumerated beam relative to a per-node prediction.  The residual between the enumerated beam and the per node prediction is specified by the expression BeamIdxResid[ dnPt ].

BeamIdxResid[dnPt] := (1 − 2 × beam_idx_resid_sign[dnPt]) × beam_idx_resid_abs[dnPt]

The beam is used in the contextualization of the syntax elements direct_rem_st_ang[ dnPt ], direct_rem_v_ang[ dnPt ] and in 𝑣-component of dnPt-th point prediction to be used with DirectVAngResid[ dnPt ].

      - Eligibility

        - Decision for each occupancy tree node

Only certain occupancy tree nodes are eligible to be direct nodes.  They are specified by the expression DirectModeEligible.  An eligible node:

- is not the root node;

- is not the root node of a fully quantized subtree (9.2.14.2.6); and

- meets one of the following mode-dependent conditions:

  - When occtree_direct_coding_mode is 1: if there are no nodes in the occupied neighbourhood pattern of the parent node, the coded node has no siblings and the parent node has at most one sibling.

  - When occtree_direct_coding_mode is 2: if there are no nodes in the occupied neighbourhood pattern of the parent node.

  - When occtree_direct_coding_mode is 3: if the coded node has at least one sibling.

DirectModeEligible := occtree_direct_coding_mode > 0
  && Dpth > 0
  && MaxVec(QuantizedNodeSizeLog2) > 0  && ¬OccupancyIsPredictable
  && ( (interNonAngular && DirectMode1Eligible) || 
        (¬interNonAngular 
        && (occtree_direct_coding_mode ≠ 1 || DirectMode1Eligible)
        && (occtree_direct_coding_mode ≠ 2 || DirectMode2Eligible)
        && (occtree_direct_coding_mode ≠ 3 || DirectMode3Eligible) 
        )
    )
  where,
    interNonAngular = slice_inter_prediction && ¬geom_angular_enabled

DirectMode1Eligible := OccNeighPatEq0[Dpth − 1][NsP][NtP][NvP]
  &&              OccNodeChildCnt[Dpth − 1][NsP][NtP][NvP] == 1
  && (Dpth < 2 || OccNodeChildCnt[Dpth − 2][NsG][NtG][NvG] ≤ 2)

DirectMode2Eligible := OccNeighPatEq0[Dpth − 1][NsP][NtP][NvP]

DirectMode3Eligible := OccNodeChildCnt[Dpth − 1][NsP][NtP][NvP] > 1

When occtree_inter_angular_direct_coding_enabled is equal to 1, an eligible node to be direct nodes meets the condition when DnEligibleByAng is equal to 1. (9.2.13.8.1).

DirectModeEligible := DnEligibleByAng

        - Presence of occ_direct_node

The syntax element occ_direct_node shall only be present in occupancy tree nodes that are both eligible for direct coding and not prohibited by the rate limit for direct nodes that applies when planar occupancy coding is enabled.

The direct node rate limit mask is specified by the expression DnPresenceMask[ 𝑖 ], 𝑖 ∈ 0 .. 31.

DnPresenceMask[i] := occtree_planar_enabled && occtree_direct_coding_mode == 1
    ? dnRate × i % 32 + (dnRate ≥ 32)
    : 1
  where
    dnRate := occtree_direct_node_rate_minus1 + 1

The expression DirectNodePresent specifies the presence of the syntax element.

DirectNodePresent := DirectModeEligible && DnPresenceMask[(Dpth + DnEligibleCnt) % 32]

        - State variables

Eligibility is specified in terms of the following state variables; the indexes dpth, ns, nt and nv identify a node with location ( ns, nt, nv ) in the tree level at depth dpth:

- The sparse array OccNeighPatEq0; OccNeighPatEq0[ dpth ][ ns ][ nt ][ nv ] identifies whether the identified node has no nodes present in its occupied neighbourhood pattern.

- The sparse array OccNodeChildCnt; OccNodeChildCnt[ dpth ][ ns ][ nt ][ nv ] is the number of child nodes of the identified node.

- The variable DnEligibleCnt, a cumulative count of eligible nodes in a tree level.

        - Initial state

At the start of every occupancy_tree syntax structure, the OccNodeChildCnt array shall be cleared; all elements of OccNodeChildCnt are unset.

        - State update at the start of every occupancy tree level

At the start of every occupancy_tree_level syntax structure, the count of eligible nodes DnEligibleCnt shall be set to zero.

        - State update after each coded occupancy tree node

This subclause applies at the end of every occupancy_tree_node syntax structure.

The number of child nodes and the presence of any nodes in the occupied neighbourhood pattern are recorded for use in subsequent eligibility decisions.

OccNodeChildCnt[Dpth][Ns][Nt][Nv] = direct_node ? 0 : OccChildCnt
OccNeighPatEq0[Dpth][Ns][Nt][Nv] = OccNeighPat == 0

If the node is eligible for direct coding, irrespective of the presence of occ_direct_node, the count of eligible nodes shall be incremented.

if (DirectModeEligible)
  DnEligibleCnt++

      - Points represented by direct nodes

        - General

The unscaled positions for the points coded by the direct node are specified by the expression DnPtPos[ dnPt ][ 𝑘 ].  They are the concatenation of:

- the (quantized) node position,

- any bit corresponding to an occupied plane as determined by planar occupancy coding,

- any bits from joint direct position coding, and

- any remaining bits.

When geometry subtree scaling is enabled, direct nodes code partially quantized positions relative to the quantized node position.

DnPtPos[dnPt][k] := Nloc[k] << QuantizedNodeSizeLog2[k] | DnPtPosRem[dnPt][k]

DnPtPosRem[dnPt][k] := DnPlanarPos[k] | DnJointPos[dnPt][k] | DnRemPos[dnPt][k]

DnPtPosS[dnPt] := DnPtPos[dnPt][0]
DnPtPosT[dnPt] := DnPtPos[dnPt][1]
DnPtPosV[dnPt] := DnPtPos[dnPt][2]

        - Output

At the end of the direct node, the coded points shall be scaled (9.2.14.6) and appended to the output point list:

if (occ_direct_node) {
  for (dnPt = 0; dnPt ≤ direct_point_cnt_eq2; dnPt++, PointCnt++, DirectNodePointCnt++)
    for (k = 0; k < 3; k++)
      PointPos[PointCnt][k] = OccPosScaleK(k, DnPtPos[dnPt][k])

  for (i = 0; i < direct_dup_point_cnt; i++, PointCnt++, DirectNodePointCnt++)
    for (k = 0; k < 3; k++)
      PointPos[PointCnt][k] = PointPos[PointCnt − 1][k]
}

        - Planar-inferred position bits

When an axis is eligible for planar occupancy coding and it has a single occupied plane, the MSB of the position residual for that axis 𝑘 is specified by DnPlanarPos[ 𝑘 ], equal to the occupied plane location.

DnPlanarPos[k] := ¬PlanarFreeAxis[k] ? occ_plane_pos[k] << DnBitsAfterPlanar[k] : 0

The number of bits coded by each position residual exclusive of any bit derived from planar occupancy coding is specified for each component by the expression DnBitsAfterPlanar[ 𝑘 ].

DnBitsAfterPlanar[k] := QuantizedNodeSizeLog2[k] − ¬PlanarFreeAxis[k]

        - Joint coded position bits

The position residuals shall be jointly coded for an axis when the direct node codes two positions, joint coding is enabled and it has a residual bit to code.  When angular coding is enabled, components coded by direct_rem_st_ang and either direct_rem_v_ang or direct_v_ang_resid_abs and direct_v_ang_resid_sign shall not be jointly coded.

DnJointCoded[k] := occtree_direct_joint_coding_enabled && direct_point_cnt_eq2
  && DnBitsAfterPlanar[k] > 0
  && (¬geom_angular_enabled || k == (1 ^ AzimuthAxis))

The joint-coded bits for the two positions are specified by the expression DnJointPos[ dnPt ][ 𝑘 ].  For each component 𝑘, they comprise the common MSB (9.2.12.5.2) and the first divergent bit (9.2.12.5.3), if any.  Joint coding is exclusive of any planar-inferred position bit.

DnJointPos[dnPt][k] :=
  DnJointCoded[k] ? DnJointPosCommon[k] | DnJointPosDiffBit[dnPt][k] : 0

        - Remaining bits

The number of bits coded by each position residual exclusive of any bits derived from planar occupancy coding or joint coding is specified for each component by the expression DnRemBits[ 𝑘 ].

DnRemBits[k] := DnBitsAfterPlanar[k] − DnJointCommonBits[k] − DnJointDiffBits[k]

The expression DnRemPos[ dnPt ][ 𝑘 ] specifies the position bits coded by the syntax elements direct_rem, direct_rem_st_ang and direct_rem_v_ang.

DnRemPos[dnPt][k] :=
  geom_angular_enabled && k == AzimuthAxis ? direct_rem_st_ang[dnPt] :
  geom_angular_enabled && k == 2           ? occtree_angular_extension_enabled ?
  DirectRemVAngPred[dnPt] : direct_rem_v_ang[dnPt] : direct_rem[dnPt][k]

      - Joint coding of point positions

        - Point order

The coded order of two jointly coded points shall satisfy the constraint DnPtPosConstraint equal to 1:

- The 𝑠-coordinate of the first point shall be less than or equal to that of the second point.

- If the 𝑠-coordinates are equal, the 𝑡-coordinate of the first point shall be less than or equal to that of the second point.

- If both the 𝑠- and 𝑡-coordinates are equal, the 𝑣-coordinate of the first point shall be less than or equal to that of the second point

DnPtPosConstraint :=
    DnPtPosS[0] < DnPtPosS[1]
    || DnPtPosT[0] < DnPtPosT[1] && dnPtPosSameS
    || DnPtPosV[0] ≤ DnPtPosV[1] && dnPtPosSameS && dnPtPosSameT
  where
    dnPtPosSameS := DnPtPosS[0] == DnPtPosS[1]
    dnPtPosSameT := DnPtPosT[0] == DnPtPosT[1]

        - Common position bits

The number of position bits coded by direct_joint_prefix[ 𝑘 ] is specified by DnJointPrefixBits[ 𝑘 ].

DnJointPrefixBits[k] := DnJointCoded[k] ? IntLog2(direct_joint_prefix[k]) : 0

The number of bits coded by the 𝑘-th component of each position residual, exclusive of any bits derived from planar occupancy coding or a joint-coded prefix, is specified by the expression DnBitsAfterJointPrefix[ 𝑘 ].

DnBitsAfterJointPrefix[k] := DnBitsAfterPlanar[k] − DnJointCommonBits[k]

The expression DnJointPosPrefix[ 𝑘 ] specifies the value of the position bits coded by direct_joint_prefix[ 𝑘 ].

DnJointPosPrefix[k] :=
  (direct_joint_prefix[k] ^ Exp2(DnJointPrefixBits[k])) << DnBitsAfterJointPrefix[k]

        - First divergent position bit

The existence of a divergent bit in the 𝑘-th component of the two jointly coded positions is specified by the expression DnJointPosDiffBits[ 𝑘 ].  A jointly coded divergent position bit exists if direct_joint_prefix[ 𝑘 ] is present and does not complete the coding of the 𝑘-th position component.

DnJointDiffBits[k] := DnJointCoded[k] ? DnBitsAfterPlanar[k] > DnJointPrefixBits[k] : 0

The value of the jointly coded divergent bit is specified for the 𝑘-th component of each point by the expression DnJointPosDiffBit[ dnPt ][ 𝑘 ].

DnJointPosDiffBit[dnPt][k] := bit << DnRemBits[k]
  where bit :=
    DnJointDiffBitPresent[k]  ? direct_joint_diff_bit[k] ^ dnPt :
    DnJointDiffBitInferred[k] ? dnPt
                              : 0 /* DnJointDiffBits[k] is 0 */

        - Presence of the syntax element direct_joint_diff_bit

The presence of direct_joint_diff_bit[ 𝑘 ] is specified by DnJointDiffBitPresent[ 𝑘 ].

DnJointDiffBitPresent[k] := DnJointDiffBits[k] > 0 && ¬DnJointDiffBitInferred[k]

It is present when a divergent bit position exists in the 𝑘-th component of the two jointly coded positions and the bit value cannot be inferred from the ordering constraint on jointly coded positions.  The inference condition is specified by the expression DnJointDiffBitInferred[ 𝑘 ].

DnJointDiffBitInferred[k] :=
    k == 0 && DnJointDiffBits[0] == 1
  || k == 1 && DnJointDiffBits[1] == 1 && DnJointDiffBits[0] == 0
  || k == 2 && DnJointDiffBits[2] == 1 && DnJointDiffBits[0] + DnJointDiffBits[1] == 0

      - Parsing of direct_joint_prefix

The binarization of direct_joint_prefix[ 𝑘 ] interleaves the prefix and suffix bins of the binarized exp-Golomb code (11.4.3).  Every non-zero prefix bin shall be followed by a single suffix bin.

The binarization shall have no more than DnRemBits[ 𝑘 ] suffix bins.

maxVal = Exp2(DnRemBits[k])

The syntax element is parsed as follows; the variable BinIdx is the count of coded bins used for contextualization.

value = 1
for (BinIdx = 0; value < maxVal && AeReadBin() == 1; BinIdx++)
  BinIdx++
  value = (value << 1) + AeReadBin()

    - Angular coding

      - General

Subclause 9.2.13 applies when geom_angular_enabled is 1.

Angular coding in the occupancy tree specifies the contextualization for the planar occupied plane location occ_plane_pos, the direct position remainders direct_rem_st_ang and direct_rem_v_ang, the absolute value of the direct position vertical residual direct_v_ang_resid_abs and the sign of the direct position vertical residual direct_v_ang_resid_sign and the eligibility for direct coding.  The contextualization uses rays cast from the angular origin.

      - Node position relative to the angular origin

The angular-origin-relative position of the occupancy tree node position pmin is specified by the expression NposAng[ 𝑘 ].

NposAng[k] := AngPosScaleK(k, Nloc[k] << QuantizedNodeSizeLog2[k]) − AngularOrigin[k]

NposAngS := NposAng[0]
NposAngT := NposAng[1]
NposAngV := NposAng[2]

      - Azimuth coded axis

Contextualization using a beam's azimuth applies to either the S or T axis.  The index of the contextualized axis is specified by the expression AzimuthAxis.  It is determined using the angular-origin-relative node position.

AzimuthAxis := Abs(NposAngS) > Abs(NposAngT)

AzimuthAxisIsS := AzimuthAxis == 0
AzimuthAxisIsT := AzimuthAxis == 1

      - State variables

Contextualization of occ_plane_pos[ AzimuthAxis ], beam_idx_resid_abs[ dnPt ], beam_idx_resid_sign[ dnPt ] and direct_rem_st_ang is specified in terms of the following state variables; the index beamIdx identifies an SPS enumerated beam:

- The array BeamPrevPhiValid; BeamPrevPhiValid[ beamIdx ] indicates whether a value has been recorded by BeamPrevPhi[ beamIdx ].

- The array BeamPrevPhi; BeamPrevPhi[ beamIdx ] records the azimuth of the identified beam computed from the most recently coded syntax element direct_rem_st_ang.

- The array BeamPrevIdxResid; BeamPrevIdxResid[ beamIdx ] records the residual BeamIdxResid[ dnPt ] between the enumerated beam DnBeamIdx[dnPt] and the per node prediction from estimated beam index DnBeamIdxEst, as specified from the most recently coded syntax elements beam_idx_resid_abs[ dnPt ] and beam_idx_resid_sign[ dnPt ].

      - Initial state

At the start of every occupancy tree, all elements of BeamPrevPhiValid and all elements of BeamPrevIdxResid shall have the value 0.

      - Closest beam to a point

This subclause specifies the selection of the beam that can emit the closest rays to an angular-origin-relative point ( as, at, av ) by the expression BeamIdxEst[ as ][ at ][ av ].  The selection shall use rays cast from the angular origin.

When angular extension shall not be used or when performing beam selection for planar occupancy coding (9.2.13.7.3), the expression preciseBeamSelection is set equal to 0 and the selection is made without application of vertical beam displacements.

When angular extension shall be used and not performing beam selection for planar occupancy coding (9.2.13.7.3), the expression preciseBeamSelection is set equal to 1 and the selection is made by taking into account the vertical beam displacements.

BeamIdxEst[as][at][av] := num_beams_minus1 ?
    (preciseBeamSelection ? PreciseBeamIdx : BeamIdxFromGrad[aGrad]) : 0

When preciseBeamSelection is equal to 0, the selection shall be performed by comparing the gradient of a ray specified by the expression aGrad to the elevation gradients of the enumerated beams.  When preciseBeamSelection is equal to 1, the selection shall be performed by comparing for each enumerated beam the gradient of a ray specified by the expression displacedPointRayGrad[i] to the elevation gradient of the corresponding enumerated beam.  According to the expression preciseBeamSelection, the ray is cast from the angular origin and passes either (when equal to 0) through the point ( as, at, av ), or (when equal to 1) through the point to which vertical beam displacement is applied ( as, at, av  + BeamOffsetV[i]).

Unless performing beam selection for planar occupancy coding (9.2.13.7.3), the expressions aGrad and preciseBeamSelection and displacedPointRayGrad[i] shall be defined as:

aGrad := av × IntRecipSqrt(rs × rs + rt × rv) >> 14
  where
    rs := as << 8
    rt := at << 8

preciseBeamSelection := occtree_angular_extension_enabled

displacedPointRayGrad[i] := ((av << 3) + BeamOffsetV[i])
                            × IntRecipSqrt(rs × rs + rt × rt) >> 17
  where
    rs := as << 8
    rt := at << 8

When performing beam selection for planar occupancy coding (9.2.13.7.3), the expression aGrad shall be defined as:

aGrad := (2 × av − 1) × IntRecipSqrt(rs × rs + rt × rt) >> 15
  where
    rs := (as << 8) − 128
    rt := (at << 8) − 128

preciseBeamSelection := 0

When preciseBeamSelection is equal to 0, the beam search is specified by the expression BeamIdxFromGrad.  If the gradient rayGrad is half-way between that of two beams, the beam with the lower index shall be chosen.

BeamIdxFromGrad[rayGrad] :=
  for (i = 1; i < num_beams_minus1; i++)
    if (BeamElev[i] > rayGrad)
      break
  if (rayGrad − BeamElev[i − 1] ≤ BeamElev[i] − rayGrad)
    i−−
  BeamIdxFromGrad = i

When preciseBeamSelection is equal to 1, the precise beam search is specified by the expression PreciseBeamIdx.  If the smallest value of gradientDistance[i] is obtained for at least two beams, the beam with the lower index shall be chosen.

PreciseBeamIdx :=
  PreciseBeamIdx = 0
  for (i = 1; i < num_beams_minus1 + 1; i++)
    if (gradientDistance[i] < gradientDistance[PreciseBeamIdx])
      PreciseBeamIdx = i
  where
    gradientDistance[i] := abs(displacedPointRayGrad[i] - BeamElev[i])

      - Application to occupied plane location coding

        - Node dimensions and midpoint

The expression ScaledNodeSize[ 𝑘 ] specifies the scaled volume dimensions of the coded node.

ScaledNodeSize[k] := AngPosScaleK(k, Exp2(QuantizedNodeSizeLog2[k]))

ScaledNodeSizeS := ScaledNodeSize[0]
ScaledNodeSizeT := ScaledNodeSize[1]
ScaledNodeSizeV := ScaledNodeSize[2]

The expression ScaledHalfNodeSize[ 𝑘 ] specifies the midpoint within the scaled volume of the coded node.

ScaledHalfNodeSize[k] := AngPosScaleK(k, Exp2(QuantizedNodeSizeLog2[k]) >> 1)

ScaledHalfNodeSizeS := ScaledHalfNodeSize[0]
ScaledHalfNodeSizeT := ScaledHalfNodeSize[1]
ScaledHalfNodeSizeV := ScaledHalfNodeSize[2]

- When geometry scaling is disabled, ScaledNodeSize[ 𝑘 ] and ScaledHalfNodeSize[ 𝑘 ] are equal to Exp2( NodeSizeLog2[ 𝑘 ] ) and Exp2( NodeSizeLog2[ 𝑘 ] >> 1 ) respectively.

The midpoint coordinates of the coded node relative to the angular origin are specified by the expression NposAngMid[ 𝑘 ].

NposAngMid[k] := NposAng[k] + ScaledHalfNodeSize[k]

NposAngMidS := NposAngMid[0]
NposAngMidT := NposAngMid[1]
NposAngMidV := NposAngMid[2]

        - Eligibility

Only certain nodes are eligible for angular contextualization of the occupied plane location and inter direct coding.  They are specified by the expression AngularEligible.  Eligibility prevents the use of angular contextualization when a node is sufficiently large to be intersected by rays from multiple beams.

AngularEligible := geom_angular_enabled
  && (¬num_beams_minus1 || CathetusV > (ScaledHalfNodeSizeV << 26))

Eligibility is specified in terms of:

- the smallest difference in elevation gradient between any two beams, BeamMinDeltaGrad;

- the V-axis distance subtended (CathetusV) by a ray with elevation gradient BeamMinDeltaGrad, over the l1 distance in the S-T plane from the angular origin to the node midpoint; and

- the 𝑣-component of the scaled node size.

An example of an eligible node is illustrated in Figure 23.

Figure 23 — Illustration of an eligible node.

CathetusV := BeamMinDeltaGrad × ((rs + rt) >> 1)
  where
    rs := Abs((NposAngMidS << 8) − 128)
    rt := Abs((NposAngMidT << 8) − 128)

The expression BeamMinDeltaGrad is the smallest difference in elevation gradient between any two beams.

BeamMinDeltaGrad :=
  BeamMinDeltaGrad = BeamElev[0]
  for (i = 1; i ≤ num_beams_minus1; i++) {
    delta = BeamElev[i] – BeamElev[i − 1]
    if (delta < BeamMinDeltaGrad)
      BeamMinDeltaGrad = delta
  }

Only certain nodes are eligible for azimuth direct coding. They are specified by the expression AzimuthEligible.

AzimuthEligible := AngularEligible && ((HalfDeltaAziAng << 1) ≤ phiStep)
  where
    phiStep:= 6588397 / BeamStepsPerRev[PlanarBeamIdx]

HalfDeltaAziAng := Abs(azimuthLowMId - azimuthMid)
  where
    azimuthLowMId := AzimuthAxis ? IntAtan2(NposAngMidT, NposAngS)
                      : IntAtan2(NposAngT, NposAngMidS)
    azimuthMid := IntAtan2(NposAngMidT, NposAngMidS)

        - Beam selection

Every node that is eligible for angular contextualization (AngularEligible == 1) shall select a beam for that purpose.  The selected beam is specified by PlanarBeamIdx.  It is either the same beam as used for the parent node or the closest beam to the node midpoint (9.2.13.6).

PlanarBeamIdx :=
  inheritBeam ? NodeBeamIdx[Dpth − 1][NsP][NtP][NvP]
              : BeamIdxEst[NposAngMidS][NposAngMidT][NposAngMidV]

The beam selection is inherited from the parent node when both the parent node has a valid beam and the node is not too small, as specified by the expression inheritBeam.

inheritBeam :=
  NodeBeamValid[Dpth − 1][NsP][NtP][NvP] && CathetusV > (ScaledHalfNodeSizeV << 28)

        - State variables

Beam selection is specified in terms of the following state variables; the indexes dpth, ns, nt and nv identify a node with location ( ns, nt, nv ) in the tree level at depth dpth:

- The sparse array NodeBeamValid; NodeBeamValid[ dpth ][ ns ][ nt ][ nv ] indicates whether (when 1) the identified node has a valid beam selection.  Unset elements are inferred to be 0.

- The sparse array NodeBeamIdx; NodeBeamIdx[ dpth ][ ns ][ nt ][ nv ] is the selected beam, if valid, for the identified node.

        - Initial state

At the start of every occupancy tree, elements of NodeBeamValid shall be unset.

        - State update

After selecting a beam (9.2.13.7.3), the beam index and its validity shall be recorded for inheritance by child nodes.

NodeBeamIdx[Dpth][Ns][Nt][Nv] = PlanarBeamIdx
NodeBeamValid[Dpth][Ns][Nt][Nv] = 1

        - Contextualization of occ_plane_pos[ AzimuthAxis ]

The syntax element occ_plane_pos[ AzimuthAzis ] shall be contextualized according to 9.2.13.9 with the argument phiRangeMulLog2 equal to 2 and the arguments beamIdx, phiNominal, phiLow and phiHigh as specified in this subclause.

The argument beamIdx is the index of the selected beam.

beamIdx = PlanarBeamIdx

The arguments phiLow and phiHigh are the azimuthal angles for the angular-origin-relative coordinates of the lower corner of the occupancy tree node in the S-T plane for phiLow, and of the scaled node midpoint in the S-T plane for phiHigh. The argument phiNominal is a nominal azimuthal angle use during contextualization and it is set equal to phiLow.

phiLow = IntAtan2(NposAngT, NposAngS)
phiHigh = IntAtan2(NposAngMidT, NposAngMidS)
phiNominal = phiLow

        - Contextualization of occ_plane_pos[ 2 ]

The syntax element occ_plane_pos[ 2 ] shall be contextualized according to 9.2.13.10 with the argument elvIntvlGradDivLog2 equal to 2 and the arguments beamIdx, elvIntvlPosS, elvIntvlPosT, elvIntvlMidV and elvIntvlLenV as specified in this subclause.

The argument beamIdx is the index of the selected beam.

beamIdx = PlanarBeamIdx

The arguments elvIntvlPosS, elvIntvlPosT and elvIntvlMidV are the angular-origin-relative coordinates of the scaled node midpoint.  The argument elvIntvlLenV is the length of the V-axis interval represented by the syntax element

elvIntvlPosS = NposAngMidS
elvIntvlPosT = NposAngMidT
elvIntvlMidV = NposAngMidV
elvIntvlLenV = ScaledNodeSizeV

      - Application to direct node position coding

        - Eligibility

Only certain occupancy tree nodes are eligible to be direct nodes.  They are specified by the expression DirectModeEligible (9.2.12.3.1).

When occtree_inter_angular_direct_coding_enabled is equal to 1, eligibility for direct nodes shall be performed by the expression DnEligibleByAng.

DnEligibleByAng:= occtree_inter_angular_direct_coding_enabled ?
   (geom_dup_point_counts_enabled ?
   AngularEligible && AzimuthEligible: AngularEligible || AzimuthEligible): 0

Only certain occupancy tree nodes are eligible to be inter direct nodes. They are specified by the expression interPredEligible.

The reference node indicates the colocated node in the reference frameof the current node.

interNumPredPoints indicates the number of points within the reference node of the current node.

interPredMode is true, when the reference node has at most one point or all points position are same.

interPredEligible is true, when the follwing conditions are true:

- inter_prediction_enabled is true and

- interPredMode is true and

- interNumPredPoints larger than 0

interPredEligible:=
              inter_prediction_enabled && interPredMode && interNumPredPoints >=0

        - Beam selection

Context selection for direct_rem_st_ang[ dnPt ] and direct_rem_v_ang[ dnPt ] and 𝑣-component of dnPt-th point prediction to be used with DirectVAngResid[ dnPt ] shall be performed using the beam specified by  DnBeamIdx[ dnPt ]. For eligible node (9.2.12.3), an estimated beam index is determined for the dnPt-th point within the current node. The estimated beam index is specified by the expression DnBeamIdx[ dnPt ].

It is a requirement of bitstream conformance that DnBeamIdx[ dnPt ] shall be in the range 0 .. num_beams_minus1.

DnBeamIdx[dnPt] := (interPredEligible ? interDnBeamIdxEst:DnBeamIdxEst) + BeamIdxResid[dnPt]

Every direct node shall determine an estimated beam index as specified by the expression DnBeamIdxEst.  The beam is estimated from the angular-origin-relative, scaled partial point position specified by dnBeamPosAng[ 𝑘 ].

DnBeamIdxEst := BeamIdxEst[beamPosAngS][beamPosAngT][beamPosAngV]
  where
    beamPosAngS := dnBeamPosAng[0]
    beamPosAngT := dnBeamPosAng[1]
    beamPosAngV := dnBeamPosAng[2]

dnBeamPosAng[k] := NposAng[k] + AngPosScaleK(k, DnPlanarPos[k] + dnPlanarPosHalf)
  where
    dnPlanarPosHalf := Exp2(DnBitsAfterPlanar[k]) >> 1

When interPredEligible is equal to 1, every point shall determine an estimated beam index as specified by the expression interDnBeamIdxEst. The beam is estimated from the angular-origin-relative, scaled reference predictor point position specified by interDnBeamPosAng[ 𝑘 ].

inter
DnBeamIdxEst := BeamIdxEst[beamPosAngS][beamPosAngT][beamPosAngV]
  where
    beamPosAngS := interDnBeamPosAng[0]
    beamPosAngT := interDnBeamPosAng[1]
    beamPosAngV := interDnBeamPosAng[2]

interDnBeamPosAng[k]:= AngPosScaleK(k, interDnPlanarPos[k])
  where
        interDnPlanarPos:= PredGMPointCloud[predDnPt] 
        predDnPt:= dnPt< predEnd? dnPt : predEnd -1

        - Scaled angular-origin-relative point positions and partial point positions

Contextualization of direct_rem_st_ang and direct_rem_v_ang depends upon approximately scaled partially coded positions.  A partially coded position is specified in terms of the MSBs of the complete coded position.

Approximate scaling accumulates applications of geometry scaling to each bit in the partially coded position.

The expression DnPtPosAng[ dnPt ][ 𝑘 ] is the 𝑘-th component of the scaled, coded point position relative to the angular origin.

DnPtPosAng[dnPt][k] := AngPosScaleK(k, DnPtPos[dnPt][k]) − AngularOrigin[k]

DnPtPosAngS[dnPt] := DnPtPosAng[dnPt][0]
DnPtPosAngT[dnPt] := DnPtPosAng[dnPt][1]

The expression DnPartialPosAng[ dnPt ][ remBits ][ 𝑘 ] is the 𝑘-th component of the approximately scaled partially coded position for the dnPt-th coded point of the direct node.  It excludes the remBits LSBs of DnPtPos[ dnPt ][ 𝑘 ].

DnPartialPosAng[dnPt][remBits][k] :=
  DnPartialPosAng = NposAng[k]
  for (i = remBits; i < QuantizedNodeSizeLog2[k]; i++)
    DnPartialPosAng += AngPosScaleK(k, DnPtPosRem[dnPt][k] & Exp2(i))

        - Binarization of direct_rem_st_ang and direct_rem_v_ang

The direct_rem_st_ang and direct_rem_v_ang syntax elements shall be entropy coded using fixed-length binarization.  The length in bins shall be equal to the log2 quantized node size less any bit inferred by the presence of a single occupied plane.

The expression DnRemAngBitsST is the length in bins of the syntax element direct_rem_st_ang.

DnRemAngBitsST := DnBitsAfterPlanar[AzimuthAxis]

The expression DnRemAngBitsV is the length in bins of the syntax element direct_rem_v_ang.

DnRemAngBitsV := DnBitsAfterPlanar[2]

        - Contextualization of direct_rem_st_ang

Each bin, identified by BinIdx, of direct_rem_st_ang[ dnPt ] shall be contextualized according to 9.2.13.9 with the argument phiRangeMulLog2 equal to 1 and the arguments beamIdx, phiNominal, phiLow and phiHigh as specified in this subclause.

The argument beamIdx is the index of the selected beam.

beamIdx = DnBeamIdx[dnPt]

The expression PhiLowST[ remBits ] is the azimuthal angle for angular-origin-relative coordinates of the lower end of the interval in the S-T plane, according to the number of remaining bins remBits for direct_rem_st_ang[ dnPt ].  When occupancy tree angular extension is not enabled it is computed from coordinates in the S-T plane, otherwise it may be estimated.

The expression PhiHighST[ remBits ] is the azimuthal angle for angular-origin-relative coordinates of the higher end of the interval in the S-T plane, according to the number of remaining bins remBits for direct_rem_st_ang[ dnPt ].  When occupancy tree angular extension is not enabled it has no utility, otherwise it may be estimated.

The expression PhiMidST[ remBits ] is the azimuthal angle for angular-origin-relative coordinates of the mid point of the interval in the S-T plane, according to the number of remaining bins remBits for direct_rem_st_ang[ dnPt ].  When occupancy tree angular extension is not enabled it is computed from coordinates in the S-T plane, otherwise it may be estimated.

The expressions PhiPosS[ remBits ] and PhiPosT[ remBits ] are the angular-origin-relative coordinates of the lower end of the interval coded by a bin in the S-T plane, the bin index being expressed by the number of remaining bins remBits for direct_rem_st_ang[ dnPt ].

PhiPosS[remBits] := AzimuthAxisIsS ? DnPartialPosAngS[dnPt][remBits] : DnPtPosAngS[dnPt]

PhiPosT[remBits] := AzimuthAxisIsT ? DnPartialPosAngT[dnPt][remBits] : DnPtPosAngT[dnPt]

PhiLowST[remBits] :=
    occtree_angular_extension_enabled && remBits < DnRemAngBitsST ?
      (prevBitIs1 ? PhiMidST[remBits + 1] : PhiLowST[remBits + 1]) :
      IntAtan2(phiIntvlLowT, phiIntvlLowS)
  where
    prevBitIs1 := (direct_rem_st_ang[dnPt] & (1 << remBits)
    phiIntvlLowS := PhiPosS[remBits]
    phiIntvlLowT := PhiPosT[remBits]

PhiHighST[remBits] :=
    occtree_angular_extension_enabled && remBits < DnRemAngBitsST ?
      (prevBitIs1 ? PhiHighST[remBits + 1] : PhiMidST[remBits + 1]) :
      IntAtan2(phiIntvlHighT, phiIntvlHighS)
  where
    prevBitIs1 := (direct_rem_st_ang[dnPt] & (1 << remBits)
    binIdx := DnRemAngBitsST - remBits
    phiIntvlLen := AngPosScaleK(AzimuthAxis, Exp2(DnRemAngBitsST) >> binIdx)
    phiIntvlHighS = phiPosS[remBits] + (AzimuthAxisIsS ? phiIntvlLen : 0)
    phiIntvlHighT = phiPosT[remBits] + (AzimuthAxisIsT ? phiIntvlLen : 0)

PhiMidST[remBits] :=
    occtree_angular_extension_enabled
       && (Abs(PhiLowST[remBits] - PhiHighST[remBits]) < phiMidInterpThreshold ?
      PhiLowST[remBits] + PhiHighST[remBits] >> 1 :
      IntAtan2(phiIntvlMidT, phiIntvlMidS)
  where
    binIdx := DnRemAngBitsST - remBits
    phiMidInterpThreshold := 1 << 13
    phiHalfIntvlLen := AngPosScaleK(AzimuthAxis, Exp2(DnRemAngBitsST) >> 1 + binIdx)
    phiIntvlMidS = PhiPosS[remBits] + (AzimuthAxisIsS ? phiHalfIntvlLen : 0)
    phiIntvlMidT = PhiPosT[remBits] + (AzimuthAxisIsT ? phiHalfIntvlLen : 0)

The expression phiPartialSTBits is the number of remaining bins for direct_rem_st_ang[ dnPt ], including the bin identified by BinIdx.

phiPartialSTBits := DnRemAngBitsST – BinIdx

The expression phiOneFourth and phiThreeFourth are azimuthal angles estimated for angular-origin-relative coordinates of points estimated around one fourth and around three fourth of the interval in the S-T plane.

phiOneFourth := phiIntvlLow
    + ((offset0 – offset1) × (phiIntvlMid - phiIntvlLow) >> phiPartialSTBits)
phiThreeFourth := phiIntvlMid
    + ((offset0 + offset1) × (phiIntvlMid - phiIntvlLow) >> phiPartialSTBits)
  where
    phiIntvlLow := PhiLowST[ phiPartialSTBits ]
    phiIntvlMid := PhiMidST[ phiPartialSTBits ]
    offset0 := AngPosScaleK(AzimuthAxis, Exp2(DnRemAngBitsST) >> 1 + BinIdx) - 1
    offset1 := phiPartialSTBits > 1 ? phiPartialSTBits > 2 ? 0 : 1 : 2

When occupancy tree angular extension is enabled, the argument phiLow is set equal to an azimuthal angle estimated for angular-origin-relative coordinates of a point estimated around one fourth of the interval in the S-T plane, otherwise phiLow is set equal to an azimuthal angle for angular-origin-relative coordinates of the lower end of the interval in the S-T plane.  When occupancy tree angular extension is enabled, the argument phiHigh is set equal to an azimuthal angle estimated for angular-origin-relative coordinates of a point estimated around three fourth of the interval in the S-T plane, otherwise phiHigh is set equal to an azimuthal angle for angular-origin-relative coordinates of the mid point of the interval in the S-T plane.  When occupancy tree angular extension is enabled, the argument phiNominal is set equal to an azimuthal angle estimated for angular-origin-relative coordinates of the mid point of the interval in the S-T plane, otherwise phiNominal is set equal to phiLow.

phiLow = occtree_angular_extension_enabled ? phiOneFourth : PhiLowST[phiPartialSTBits]
phiHigh = occtree_angular_extension_enabled ? phiThreeFourth : PhiMidST[phiPartialSTBits]
phiNominal = occtree_angular_extension_enabled ? PhiMidST[phiPartialSTBits] : phiLow

        - State update after each direct_rem_st_ang syntax element

After each coded direct_rem_st_ang[ dnPt ] syntax element, the azimuth of the coded point shall be recorded as the azimuth of the selected beam.

BeamPrevPhi[DnBeamIdx[dnPt]] = PhiLowST[ 0]
BeamPrevPhiValid[DnBeamIdx[dnPt]] = 1

        - Contextualization of direct_rem_v_ang

Each bin, identified by BinIdx, of direct_rem_v_ang[ dnPt ] shall be contextualized according to 9.2.13.10 with the arguments beamIdx, elvIntvlPosS, elvIntvlPosT, elvIntvlLenV, elvIntvlMidV and elvIntvlGradDivLog2 as specified in this subclause.

The argument beamIdx is the index of the selected beam.

beamIdx = DnBeamIdx[dnPt]

The arguments elvIntvlPosS and elvIntvlPosT are the angular-origin-relative coordinates in the S-T plane of the approximately scaled position of the coded point.  The argument elvIntvlLenV is the length of the interval represented by the syntax element before division by Exp2( elvIntvlGradDivLog2 ).

elvIntvlPosS = AzimuthAxisIsS ? DnPartialPosAngS[dnPt][0] : DnPtPosAngS[dnPt]
elvIntvlPosT = AzimuthAxisIsT ? DnPartialPosAngT[dnPt][0] : DnPtPosAngT[dnPt]
elvIntvlLenV = AngPosScaleV(Exp2(DnRemAngBitsV))
elvIntvlGradDivLog2 = BinIdx

- When geometry scaling is disabled or the scaling step size is a power of two, DtPartialPosAng[ dnPt ][ 0 ][ 𝑘 ] is equal to DnPtPosAng[ dnPt ][ 𝑘 ].

The argument elvIntvlMidV specifies the midpoint of the V-axis interval relative to the angular origin.  The expression elvIntvlHalfLen is the half-interval range.  The start of the V-axis interval, elvPartialPosV, is the partially coded and approximately scaled point 𝑣-coordinate.

elvPartialPosV := DnPartialPosAngV[dnPt][DnRemAngBitsV − BinIdx]
elvIntvlHalfLen := AngPosScaleV(Exp2(DnRemAngBitsV) >> 1 + BinIdx)
elvIntvlMidV = elvPartialPosV + elvIntvlHalfLen

        - Binarization of direct_v_ang_resid_abs and direct_v_ang_resid_sign

When angular coding extension shall be used, direct_v_ang_resid_abs syntax element is the absolute value of a residual of a prediction made from the selected beam elevation and vertical offset and shall be entropy coded using a concatenated truncated unary and 𝑘-th order exp-Golomb code (11.4.4) and direct_v_ang_resid_sign is its sign and shall be entropy coded when the absolute value of the residual is not null.

The prediction of the 𝑣-component of dnPt-th point relative to the angular origin is specified by the expression DirectVAngPred[ dnPt ]. It clips a prediction made from the selected beam to fit in V-axis interval relative to the angular origin in which 𝑣-component of dnPt-th shall be refined.

DirectVAngPred[dnPt] :=
    Min(Max(directVAngPred0[dnPt], dnPosAng), dnPosAng + dnRemNodeSizeV - 1)
  where
    dnPosAngV := NposAng[2] + AngPosScaleK(2, DnPlanarPos[2])
    dnRemNodeSizeV := 1 << DnResVAngPredBits
    directVAngPred0[dnPt] :=
        DivExp2Fz(radius × BeamElev[beamIdx] - (BeamOffsetV[beamIdx] << 23), 26 )
      where
        radius := IntSqrt(rs × rs + rt × rt)
          where
            rs := elvIntvlPosS << 8
            rt := elvIntvlPosT << 8

The arguments elvIntvlPosS and elvIntvlPosT are the angular-origin-relative coordinates in the S-T plane of the approximately scaled position of the coded point.

elvIntvlPosS = AzimuthAxisIsS ? DnPartialPosAngS[dnPt][0] : DnPtPosAngS[dnPt]
elvIntvlPosT = AzimuthAxisIsT ? DnPartialPosAngT[dnPt][0] : DnPtPosAngT[dnPt]

The expression DnResVAngPredBits is the length in bits of the expression DirectRemVAngPred[ dnPt ].

DnResVAngPredBits := DnBitsAfterPlanar[2]

The expression DirectRemVAngPred[ dnPt ] specify the remaining position bits of the 𝑣-component of the dnPt-th point when angular coding extension shall be used.

DirectRemVAngPred[dnPt] :=
    DirectVAngPred[dnPt] + DirectVAngResid[dnPt] - NposAng[2]

It is a requirement of bitstream conformance that DirectRemVAngPred[ dnPt ] shall be in the range 0 .. Exp2(DnResVAngPredBits) – 1.

        - State update after each beam_idx_resid_abs and beam_idx_resid_sign syntax elements

When angular extension coding shall be used, after each coded beam_idx_resid_abs[ dnPt ] and beam_idx_resid_sign[ dnPt ] syntax elements, the residual between the enumerated beam and the per node prediction specified by the expression BeamIdxResid[ dnPt ] shall be recorded as the most recently coded residual of the selected beam.

if(occtree_angular_extension_enabled)
  BeamPrevIdxResid[DnBeamIdxEst] = BeamIdxResid[ dnPt ]

      - Determination of CtxIdxAngPhi for a bin according to beam azimuth

Each contextualized bin represents a half-closed interval along the azimuth coding axis AzimuthAxis located along the other S-T plane axis at a distance from the angular origin.  The bin values 1 and 0 identify the upper and lower halves of the interval respectively.

Contextualization discriminates by the intersection of a predicted ray with one of eight parameterized azimuth ranges.

The process is parameterized by:

- beamIdx, the index of the beam that casts the predicted ray;

- phiNominal represents a nominal azimuthal angle for the coded point;

- phiLow and phiHigh represents (possibly estimated) azimuthal angles for a point in the first and second half of the half-closed interval;

- phiRangeMulLog2, a syntax element dependent constant used to define the azimuth discrimination ranges.

The predicted ray azimuth is derived from the azimuth of a reference point.  The prediction quantizes the nominal azimuth to be an integer number of azimuth steps from the reference point.

The reference azimuth for the predicted ray is that of the previous direct-node-coded point recorded by the selected beam (9.2.13.8.2); if no previous point has been recorded by the selected beam, the reference azimuth is the nominal azimuth.

phiRef := BeamPrevPhiValid[beamIdx] ? BeamPrevPhi[beamIdx] : phiNominal

The expression phiPred specifies the predicted ray azimuth.

phiPred := phiRef − beamPhiStep[beamIdx] × phiSteps
  where
    phiSteps := DivExp2Up((phiRef − phiNominal) × BeamPhiStepRecip[beamIdx], 30)
    beamPhiStep := 6588397 / BeamStepsPerRev[beamIdx]
    beamPhiStepRecip := (BeamStepsPerRev[beamIdx] << 30) / 6588397

The context selection for the coded bin is specified by the expression CtxIdxAngPhi.

CtxIdxAngPhi := (a + 2 × b + 4 × c) × 3 + d
  where
    dPhiL := phiLow − phiPred
    dPhiH := phiHigh − phiPred
    a := (dPhiL ≥ 0) == (dPhiH ≥ 0)
    b := Abs(dPhiL) > Abs(dPhiH)
    c := Max(Abs(dPhiL), Abs(dPhiH)) > (Min(Abs(dPhiL),Abs(dPhiH)) << phiRangeMulLog2)
    d := occtree_angular_extension_enabled ? 
        (3 × phiStep < 4 × adPhiLH) + (phiStep < 2 × adPhiLH) : 0
      where
        phiStep := beamPhiStep[beamIdx]
        adPhiLH := Abs(phiLow – phiHigh)

      - Determination of CtxIdxAngTheta for a bin according to beam elevation

Each contextualized bin represents a half-closed V-axis interval located at a radial distance in the S-T plane from the angular origin.  The bin values 1 and 0 identify the upper and lower halves of the interval respectively.

Contextualization discriminates by the intersection of a predicted ray cast by a beam with one of four parameterized V-axis intervals.

The process is parameterized by:

- beamIdx, the index of the beam that casts the predicted ray;

- elvIntvlPosS and elvIntvlPosT, the coordinates of the V-axis interval in the S-T plane, relative to the angular origin;

- elvIntvlLenV, the length of the V-axis interval;

- elvIntvlMidV, the midpoint of the V-axis interval relative to the angular origin;

- elvIntvlGradDivLog2, the factor used to derive the sub-interval size.

The expression rRecip specifies the reciprocal radial distance of the V-axis interval in the S-T plane from the angular origin.

rRecip := IntRecipSqrt(rs × rs + rt × rt)
  where
    rs := (elvIntvlPosS << 8) − 128
    rt := (elvIntvlPosT << 8) − 128

The expression elvIntvlMidGrad specifies the gradient of a ray cast from the angular origin that intersects the midpoint of the V-axis interval.

elvIntvlMidGrad := (2 × elvIntvlMidV − 1) × rRecip / Exp2(15)

The expressions esoTopGrad and esoBotGrad specify the gradients of rays cast from the angular origin that intersect the end points of a V-axis sub-interval.

esoIntvlGrad := elvIntvlLenV × rRecip >> 18 + elvIntvlGradDivLog2
esoTopGrad := elvIntvlMidGrad + esoIntvlGrad
esoBotGrad := elvIntvlMidGrad − esoIntvlGrad

The expression elvBeamGrad specifies the gradient of a ray cast from the angular origin that would intersect the predicted ray at the given radial distance.

elvBeamGrad := BeamGrad[beamIdx] + BeamOffsetV[beamIdx] × rRecip / Exp2(17)

The context selection for the coded bin is specified by the expression CtxIdxAngTheta, comparing the gradients of rays cast from the angular origin to those of the threshold points.

CtxIdxAngTheta := a + 2 × b
  where
    a := elvBeamGrad ≥ elvIntvlMidGrad
    b := elvBeamGrad ≥ esoTopGrad || elvBeamGrad < esoBotGrad

      - Conversion from Cartesian to angular coordinates

This subclause specifies the conversion of decoded points' positions to angular coordinates.

A decoder is not required to perform the conversion unless angular coordinates are required for attribute decoding.

The angular-origin-relative position of every point is specified by the expression posAng[ ptIdx ][ 𝑘 ].

posAng[ptIdx][k] := PointPos[ptIdx][k] − AngularOrigin[k]

Every angular-origin-relative point position is converted to angular coordinates.  The radial distance is the Euclidean distance to the point in the S-T plane; the azimuth angle is the anti-clockwise angle in an S-T plane between the positive S axis and the point; the indexed elevation is the index of the beam that casts the closest rays to the point (9.2.13.6).  The conversion is:

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++){
  as = posAng[ptIdx][0]
  at = posAng[ptIdx][1]
  av = posAng[ptIdx][2]
  PointAng[ptIdx][0] = IntSqrt(as × as + at × at << 16) >> 8
  PointAng[ptIdx][1] = IntAtan2(at << 8, as << 8) + 3294199 >> 8
  PointAng[ptIdx][2] = BeamIdxEst[as][at][av]
}

    - Subtree scaling

      - Partially quantized coordinates

Subclause 9.2.14 applies when geom_scaling_enabled is 1.

Certain subtrees (9.2.14.2.1) can represent point positions using partially quantized coordinates that are parameterized by a scaling volume (9.2.14.2.2) and an applicable QP (9.2.14.5).

The partially quantized representation comprises two concatenated parts:

- An upper part that, when scaled, is the position of the scaling volume's lower corner, pmin.  Scaling is by the scaling volume size rounded down to the next power of two.

- A lower part that, when scaled, is a position relative to and within the scaling volume.  Scaling is by the QP derived scale factor.

An example of the representation is illustrated by Figure 24.  The coded point ● is represented within the scaling volume A.  The scaling volume size is 12; rounded down to the next power of two as 8 (2OccScalingVolSizeLog2).  The QP is 12; the derived scale factor (OccQpScale) is 3.  The two parts of the point's partially quantized 𝑠-coordinate are marked U and L:

- The upper part, U, scaled by 2OccScalingVolSizeLog2=23, is the pmin position of A.

- The lower part, L, scaled by OccQpScale = 3 is the pmin relative point position; for the S axis, 2 × 3 = 6.  Scaling expands L by 1 bit (OccQpScaleLog2) to be 3 bits (OccScalingVolSizeLog2) in total.

- If U were 5, then pmin would be 5 × 23 = 40, and the point's 𝑠-coordinate would be 46.

Key

| A | Scaling volume |
| --- | --- |
| U, L | Upper and lower parts of partially quantized representation |
| bi | 𝑖-th bit |

Figure 24 — Example of partially quantized coordinates.  Illustrated in 2D for the S-V plane.

      - Quantized subtrees

        - Identification

Subtrees that code partially quantized coordinates are started by:

- every node at depth OccQpSubtreeDepth;

- every direct node at a shallower depth than OccQpSubtreeDepth; in this case, the subtree consists only of the subtree root node as a direct node.

The lower parts of partially quantized point coordinates are represented by the location of a leaf node in the subtree or by a direct node that codes a subtree-relative position.

- Identification of a subtree from a direct node at a shallower depth than OccQpSubtreeDepth happens after any planar occupancy coding for that node (9.2.14.2.5).

        - Subtree scaling volume

A scaling volume is identified by every quantized subtree.  Its size depends upon the tree level in which the quantized subtree starts:

- If the quantized subtree starts at a shallower depth than OccQpSubtreeDepth and planar occupancy coding is enabled, the scaling volume dimensions are equal to those of the scaled child-node volume of the subtree root node.

- Otherwise, the scaling volume dimensions are identical to those of the scaled subtree root node volume.

The expression OccScalingVolSizeLog2[ 𝑘 ] specifies the integer log2 size of the scaling volume.  The scaled dimensions (9.2.14.2.3) are RoundUp( OccVolScale × Exp2( OccScalingVolSizeLog2[ 𝑘 ] ) ).

OccScalingVolSizeLog2[k] :=
  Dpth ≥ OccQpSubtreeDepth  ? OccLvlNodeSizeLog2[OccQpSubtreeDepth[k] :
  occtree_planar_enabled ? ChildNodeSizeLog2[k] : NodeSizeLog2[k]

For example, a quantized subtree root node identifies a volume with dimensions 32×16×32 and an applicable QP of 10 (OccQpScale = 2.5).  The quantized node size would be 16×8×16 and the scaled node size 40×20×40.

        - Scaled occupancy tree node volume dimensions

A scaled node volume is the geometric expansion of the volume associated with a node in the general occupancy tree (9.2.2.1).  The expansion is by a QP derived scale factor for the subtree, OccVolScale, and is relative to the lower corner, pmin, of the subtree scaling volume.  Scaling by OccVolScale rounds half-values up.

OccVolScale≔OccQpScale2Log2OccQpScale=OccQpScale2OccQpScaleLog2=1+OccQp mod 88

The quantized node size QuantizedNodeSize is a power-of-two contraction by OccQpScaleLog2 of the node size for ordinary nodes in the tree level.

OccQpScaleLog2 := OccQp / 8

QuantizedNodeSizeLog2[k] := Max(0, NodeSizeLog2[k] − OccQpScaleLog2)

QuantizedChildNodeSizeLog2[k] := Max(0, ChildNodeSizeLog2[k] − OccQpScaleLog2)

- The scaled node volume dimensions are equivalent to the rounded expansion of the quantized node size by the subtree scale factor OccQpScale.

        - Quantized subtree nodes

A node in a quantized subtree shall identify the presence of at least one point contained within the scaled node volume.

Leaf nodes in the quantized subtree represent indivisible volumes with dimensions equal to the unit cube scaled by the subtree scale factor.  Scaled point positions outside the unscaled subtree volume shall be clipped to be within it (hatched area in Figure 25).  For example, point ▪︎ in the figure is clipped to ▫︎.

Two example subtrees are illustrated over three tree levels by Figure 25.  The subtree root node volumes have dashed outlines; the coded nodes in each level have a thick, solid outline.  The top subtree uses QP 12, the bottom QP 0.  The position of the point ● is coded, starting from depth 𝑛, by the locations of the grey child nodes.  The top subtree identifies its leaf node at depth 𝑛 + 1, the bottom subtree at depth 𝑛 + 2.

Figure 25 — Example decomposition of two quantized subtrees.QP 12 (top) and QP 0 (bottom).

        - Direct nodes before OccQpSubtreeDepth

This subclause applies when planar occupancy coding is enabled.

Until a node is identified as a direct node through the coding of occ_direct_node, it shall not be considered to start a quantized subtree.

Coding operations that are specified to occur:

- before occ_direct_node shall use the ordinary node volume, equivalent to QP 0;

- after occ_direct_node shall use the scaled node volume derived from the direct node QP (DnQp),

i.e. the QP and scaled node volume dimensions change during the coding of the node.

This case is illustrated by Figure 26.  When planar occupancy coding is enabled (left), the scaled node dimensions are initially equal to NodeSize, but changes to OccScalingVolSize after occ_direct_node.

Figure 26 — Example subtree scaling volume for subtrees shallower than OccQpSubtreeDepth.  (Left) planar occupancy coding enabled; (Right) disabled.

        - Fully quantized subtrees

Sufficiently large subtree QPs can cause a subtree root node to have a log2 quantized node size of 0×0×0.  In this case, the subtree root node is a terminal node that:

- is ineligible to be a direct node (9.2.12.3.1)

- has its points represented by a single implicit child leaf node (9.2.6.3); the child node has the same dimensions as the root node – i.e. no axes are coded

      - Syntax element semantics

occ_subtree_qp_offset_present specifies whether (when 1) or not (when 0) per-subtree QP offsets are present in the tree level.  QP offsets can only be present in a single tree level.  When occ_subtree_qp_offset_present is not present, it shall be inferred to be 0.

occ_subtree_qp_offset_abs[ ns ][ nt ][ nv ] and occ_subtree_qp_offset_sign[ ns ][ nt ][ nv ] together specify an offset to the slice geometry QP used to scale point positions.  The offset is specified by the expression OccSubtreeQpOffset[ ns ][ nt ][ nv ].  The offset applies to all points coded in the subtree of the node located at ( ns, nt, nv ) in the tree level at depth OccQpSubtreeDepth.  When occ_subtree_qp_offset_sign[ ns ][ nt ][ nv ] is not present, it shall be inferred to be 0.

OccSubtreeQpOffset[ns][nt][nv] :=
  (1 − 2 × occ_subtree_qp_offset_sign[ns][nt][nv]) × occ_subtree_qp_offset_abs[ns][nt][nv]

      - State variables

Occupancy tree scaling is specified in terms of the following state variable:

- The variable OccQpSubtreeDepth that identifies the tree level where occ_subtree_qp_offset_present is 1.

      - The subtree QP

        - General

The subtree QP is specified by the expression OccQp.  It is determined for a coded node as:

- when geometry scaling is disabled: 0;

- when the node is a direct node at a depth shallower than OccQpSubtreeDepth: specified by 9.2.14.5.2;

- when the node is at a depth equal to or deeper than OccQpSubtreeDepth: specified by 9.2.14.5.3.

- The provisions of 9.2.14.2.5 affect evaluations of OccQp during certain nodes.

OccQp :=
  ¬geom_scaling_enabled    ? 0 :
  Dpth ≥ OccQpSubtreeDepth ? OccSubtreeQp
  occ_direct_node          ? DnQp : 0

        - Determination for a direct node before OccQpSubtreeDepth

The subtree QP is specified by the expression DnQp.  It is the slice's direct-node QP limited by the smallest log2 scaling volume dimension.

DnQp := Min(dnSliceQp, OccSubtreeQpMax)
  where
    dnSliceQp := geom_qp + occtree_direct_node_qp_offset << geom_qp_mul_log2

        - Determination for nodes at or after OccQpSubtreeDepth

The subtree QP is specified by the expression OccSubtreeQp.

OccSubtreeQp := sliceQp + (OccSubtreeQpOffset[ss][st][sv] << geom_qp_mul_log2)
  where
    sliceQp := geom_qp + slice_geom_qp_offset << geom_qp_mul_log2
    ss := OccSubtreeQpLoc[0]
    st := OccSubtreeQpLoc[1]
    sv := OccSubtreeQpLoc[2]

The expression OccSubtreeQpLoc is the subtree root node location in the tree level at depth OccQpSubtreeDepth for the node at ( Ns, Nt, Nv ).

OccSubtreeQpLoc[k] := Nloc[k] >> NodeSizeLog2[k] − OccScalingVolSizeLog2[k]

        - Maximum QP

The maximum QP for a subtree is specified by the expression OccSubtreeQpMax.

OccSubtreeQpMax := MinVec(OccScalingVolSizeLog2) × 8

It is a requirement of bitstream conformance that when occ_subtree_qp_offset_present is 1, OccSubtreeQp shall be in the range 0 .. OccSubtreeQpMax.

      - Scaling of a position component

This subclause specifies the scaling of the 𝑘-th partially quantized coordinate by the expressions:

- OccPosScaleK( 𝑘, posk ) that clips the scaled position to be within the subtree volume, and

- AngPosScaleK( 𝑘, posk ) that does not clip the scaled position.

- When either geom_scaling_enabled or OccNodeQp is 0, the expressions OccPosScaleK( 𝑘, posk ) and AngPosScaleK( 𝑘, posk ) are both equal to posk.

The expressions upperPartQ and lowerPartQ represent the upper and lower parts of the partially quantized coordinate.

upperPartQ := posk >> OccScalingVolSizeLog2[k] − OccQpScaleLog2
lowerPartQ := posk & Exp2(OccScalingVolSizeLog2[k] − OccQpScaleLog2) − 1

The 3-fractional-bit, fixed-point scale factor used to scale the low part is specified by the expression sF.

sF := 8 + (OccNodeQp & 7) << OccNodeQp / 8

The upper and lower parts are scaled as specified by the expressions upperPart and lowerPart.

upperPart := upperPartQ << OccScalingVolSizeLog2[k]
lowerPart := DivExp2Up(lowerPartQ × sF, 3)

The scaled upper and lower parts are combined to produce the scaled position component:

OccPosScaleK(k, posk) := upperPart | Min(lowerPart, Exp2(OccScalingVolSizeLog2[k]) − 1)

AngPosScaleK(k, posk) := upperPart + lowerPart

    - Reference frame generation

      - General

Subclause 9.2.15 applies when inter_prediction_enabled is true. When biprediction_enabled is 0, there are only I-frames or P-frames in the sequence. When biprediction_enabled is 1 or 2, there may be I-frames, P-frames and B-frames in the sequence.

The following frame aliases are defined: PrevReconFrame is the previous reconstructed point cloud frame, PrevReconIPFrame is the previous reconstructed I-frame or P-frame, PrevReconBFrame is the previous reconstructed B-frame and PrevPrevReconIPFrame is the reconstructed I-frame or P-frame before the previously reconstructed I-frame or P-frame.

When the previous reconstructed point cloud frame is a B-frame (or not a B-frame), prevReconFrameIsBFrame is set to 1 (or 0).

The original reference point cloud frame PredPointCloud and (when slice_biprediction is 1) the second reference point cloud frame SecondPredPointCloud are derived as follows:

| biprediction_enabled | slice_biprediction | PredPointCloud | SecondPredPointCloud |
| --- | --- | --- | --- |
| 0 | - | PrevReconFrame | - |
| 1 | 0 | PrevReconIPFrame | - |
| 1 | 1 | prevReconFrameIsBFrame ? PrevReconBFrame : PrevPrevReconIPFrame | PrevReconIPFrame |
| 2 | 1 | subclause 9.2.15.3 | subclause 9.2.15.3 |

The expression PredPointCloudBboxOrigin[ k ] is an alias for the 𝑘-th XYZ coordinate of the origin of the point cloud frame PredPointCloud.  PredPointCloudBboxSize[ k ] is an alias for the 𝑘-th XYZ component of the volume dimensions of the point cloud frame PredPointCloud.

The output of this process is the compensated reference point cloud frame CompensatedPointCloud and (when slice_biprediction is 1) the second compensated reference point cloud frame SecondCompensatedPointCloud.

When global_motion_enabled is equal to 0, CompensatedPointCloud is set equal to PredPointCloud and (when slice_biprediction is 1) SecondCompensatedPointCloud is set equal to SecondPredPointCloud. When global_motion_enabled is equal to 1, CompensatedPointCloud and (when slice_biprediction is 1) SecondCompensatedPointCloud are derived by invoking subclause 9.2.15.2.

When slice_biprediction is 1 and frame_merge_enabled is 1, the two compensated reference point cloud frames are merged into one by invoking subclause 9.2.15.4, and the merged compensated reference point cloud frame is used in inter prediction.

      - Motion compensation of reference frame

This subclause specifies the generation of a motion compensated reference frame using the global motion parameters ThresBot, ThresTop, AppliedGMRot, AppliedGMTrans and partition type. The input of this process is the original reference point cloud, which is specified by OriPointCloud. The output of this process is the motion compensated point cloud, which is specified by GMPointCloud.

For the first reference point cloud frame, OriPointCloud is set equal to PredPointCloud. ThresBot is set equal to gm_thres_bot[0], ThresTop is set equal to gm_thres_top[0], AppliedGMRot is set equal to GMMatrix and AppliedGMTrans is set equal to gm_trans[0]. After the motion compensation, CompensatedPointCloud is set equal to GMPointCloud.

For the second reference point cloud frame, OriPointCloud is set equal to SecondPredPointCloud. ThresBot is set equal to gm_thres_bot[1], ThresTop is set equal to gm_thres_top[1], AppliedGMRot is set equal to GMMatrix2 and AppliedGMTrans is set equal to gm_trans2[1]. After the motion compensation, SecondCompensatedPointCloud is set equal to GMPointCloud.

Let OriPntCnt be the number of points in the point cloud frame OriPointCloud.

When MotionPartitionType is 0, GMPointCloud is derived by invoking subclause 9.2.15.2.1. When MotionPartitionType is 1, GMPointCloud is derived by invoking subclause 9.2.15.2.2.

        - Motion compensation using road/object partitioning

This subclause specifies the generation of a motion compensated reference frame using global motion parameters and road/object partitioning.

The compensated point cloud is derived based on the thresholds ThresTop and ThresBot.

for (pointIdx=0; pointIdx < OriPntCnt; pointIdx++) {
  pt = OriPointCloud[pointIdx]
  if(pt[2] < ThresBot || pt[2] > ThresTop)
    … /* Apply global compensation to point pt and add compPt to GMPointCloud[pointIdx] (9.2.15.2.3) */_
  else
    for (i=0; i < 3; i++)
      GMPointCloud[pointIdx][i] = pt[i] >= 0 ? pt[i] : 0
}

        - Motion compensation using cuboid partitioning

gm_comp_partition_block[idx] specifies whether (when 1) or not (when 0) global motion compensation is to be applied to the idx-th partition block under cuboid partitioning.

This subclause specifies the generation of a motion compensated reference frame using global motion parameters and cuboid partitioning.

An intermediate compensated point cloud PredGMPointCloud is derived as follows:

for (pointIdx=0; pointIdx < OriPntCnt; pointIdx++) {
  pt = OriPointCloud[pointIdx]
  … /* Apply global compensation to point pt and add compPt to PredGMPointCloud[pointIdx] (9.2.15.2.3) */_
}

Let PredGMPntCnt be the number of points in the point cloud PredGMPointCloud.

The compensated point cloud is derived based on gm_comp_partition_block[blkIdx], which specifies whether motion compensation is applied for a block. When the point does not belong to a valid motion block, the value of blkIdx is -1.

compCloudPtIdx = 0
for (n=0; n < PredGMPntCnt; n++) {
  pt = PredGMPointCloud[n]
  … /* Derive motion block index for point pt and set it to blkIdx (9.2.15.2.4)*/_
  if(blkIdx ¬= -1 && gm_comp_partition_block[blkIdx])
    for (i=0; i < 3; i++)
      GMPointCloud[compCloudPtIdx++][i] = pt[i]
}
for (n=0; n < OriPnt; n++) {
  pt = OriPointCloud[n]
  … /* Derive motion block index for point pt and set it to blkIdx (9.2.15.2.4)*/_
  if(blkIdx ¬= -1 && ¬gm_comp_partition_block[blkIdx])
    for (i=0; i < 3; i++)
      GMPointCloud[compCloudPtIdx++][i] = pt[i]
}

        - Motion compensation for a point

This subclause specifies the generation of a motion compensated point compPt from point pt using the global motion matrix AppliedGMRot and global translation vector AppliedGMTrans.

for (i=0; i < 3; i++) {
  compPt[i] = divExp2RoundHalfInPositiveShift (
      rot[3×i] × offsetPt[0] + rot[3×i + 1] × offsetPt[1] + rot[3×i + 2] × offsetPt[2], 16, 1 << (15))+
      tran[i] – minPosition[i]
  compPt[i] = compPt[i] >= 0 ? compPt[i] : 0
}
  where,
    rot[m] = AppliedGMRot[m]
    tran[n] = AppliedGMTrans[n]
    offsetPt[i] = pt[i] + minPosition[i]
    where,
      minPosition[i] = motion_zero_origin ? 0 : sps.seq_origin_xyz[i]

        - Deriving motion block index of point under cuboid partitioning

The motion block index blkIdx is derived for point pt under cuboid partitioning.

for(i=0; i > 3; i++)
  idx[i] = motion_block_size[i] ? (pt[i]- PredPointCloudBboxOrigin[i]) / motion_block_size[i] : 0
if (idx[0] < 0 || idx[0] >= NumMotionBlocksPerAxis[0] || 
    idx[1] < 0 || idx[1] >= NumMotionBlocksPerAxis[1] || 
    idx[2] < 0 || idx[2] >= NumMotionBlocksPerAxis[2]) 
  blkIdx = -1
else 
  blkIdx = (idx[0] × LPUnumInAxis[1] + idx[1]) × LPUnumInAxis[2] + idx[2]

      - Reference frames generation for B-frame in Hierarchical GOF structure

This subclause specifies the generation of reference frames for B-frame in Hierarchical GOF structure. The notional frame counters of the reconstructed I-frame or P-frame before the previously reconstructed I-frame or P-frame FrameCtrPrePreIP, the previously reconstructed I-frame FrameCtrPreIP or P-frame and the current frame FrameCtr are used to derived the reference relationship among the frames.

The size of group of frames GOFsize is determined by the frame distance between the previouly reconstructed I-frames or P-frames.

GOFsize := FrameCtrPreIP – FrameCtrPrePreIP

The hierarchical reference relationship among the frames with the notional frame counter value from FrameCtrPrePreIP to FrameCtrPreIP are derived based on a hierarchical reference GOF structure determined by GOFsize.

The notional frame counter of the B-frame to be coded FrameCtrCur is derived from the notional frame counter values of the reference frames FrameCtrRef and FrameCtrSecondRef.

FrameCtrCur = (FrameCtrRef + FrameCtrSecondRef) >> 1

FrameCtrRef and FrameCtrSecondRef are initialized by FrameCtrPrePreIP to FrameCtrPreI for the frames with the notional frame counter value from FrameCtrPrePreIP to FrameCtrPreIP. After one B-frame is determined, FrameCtrSecondRef is updated by FrameCtrCur to derived next B-frame, and then FrameCtrRef is updated by FrameCtrCur to derived next B-frame of the next B-frame. This process is iterated until the notional frame counter value difference between the reference frames is less than or equal to 1. An example of reference relationship among B-frames (GOFsize is 8) is shown in Table 39.

Table 39 — Example of reference relationship among point cloud frames in a Hierarchical GOF

| Notional frame counter | a | a +1 | a +2 | a +3 | a +4 | a +5 | a +6 | a +7 | a +8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Notional frame counters of first reference frame | × | a | a | a+2 | a | a+4 | a+4 | a+6 | a |
| Notional frame counters ofsecond reference frame | × | a+2 | a+4 | a+4 | a+8 | a+6 | a+8 | a+8 | a |
| Frame type (I/B/P) | I | B | B | B | B | B | B | B | P |

An example of decoding order of B-frames (GOFsize is 8) is shown in Table 40.

Table 40 — Example of decoding order of B-frames in a Hierarchical GOF

| Notional frame counter of frame in decoding order | a | a +8 | a +4 | a +2 | a +1 | a +3 | a +6 | a +5 | a +7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame type (I/B/P) | I | P | B | B | B | B | B | B | B |

For each B-frame, the reference frames are generated based on the derived reference relationship which is indicated by the notional frame counter values.

      - Frame merge mode for bi-prediction

When slice_biprediction is 1 and frame_merge_enabled is 1, SecondCompensatedPointCloud is merged with CompensatedPointCloud. CompPntCnt and SecondCompPntCnt specify the numbers of points in CompensatedPointCloud and SecondCompensatedPointCloud before appending respectively.

for (pointIdx = 0; pointIdx < SecondCompPntCnt; pointIdx++) {
  pt = SecondCompensatedPointCloud[pointIdx] 
  CompensatedPointCloud[pointIdx + CompPntCnt] = pt
}

After merging the two compensated reference point cloud frames, only CompensatedPointCloud is used in inter prediction.

  - Predictive tree

    - General

Subclause 9.3 specifies the reconstruction of point positions from parsed predictive trees.  It applies when geom_tree_type is 1.

The slice geometry can be represented by a sequence of one or more predictive trees of predictive tree nodes.  Every tree node specifies a single position for one or more points.  Traversal of a predictive tree is in depth-first order.

    - Syntax element semantics

      - Predictive tree

slice_ptree_qp_period_log2_offset specifies an offset to the GPS-signalled period at which predictive tree QP offsets are signalled.  When slice_ptree_qp_period_log2_offset is not present, it shall be inferred to be 0.

A QP offset is signalled once every Exp2( PtnQpInterval ) nodes.

PtnQpInterval := Exp2(ptree_qp_period_log2 + slice_ptree_qp_period_log2_offset)

ptn_resid_abs_log2_bits[ 𝑘 ] specifies the number of bins used for the fixed-length binarization of ptn_resid_abs_log2[ 𝑘 ].

ptn_radius_min specifies the minimum angular radius coordinate for nodes where ptn_pred_mode is 0.

ptree_end_of_slice specifies whether (when 0) or not (when 1) there is a subsequent predictive tree in the DU.

      - Predictive tree node

Nodes in the predictive trees are numbered and coded according to their position in the depth-first traversal order.  The semantics specified in this subclause are for the nodeIdx-th coded node.

The syntax structure parameter depth is the tree depth of the node.

ptn_qp_offset_abs[ nodeIdx ] and ptn_qp_offset_sign[ nodeIdx ] together specify, when present and in accordance with PtnQpOffset[ nodeIdx ], an offset to the slice geometry QP used to scale point positions.  ptn_qp_offset_sign specifies whether (when 0) the offset's sign is positive or (when 1) negative.  When ptn_qp_offset_sign is not present, it shall be inferred to be 0.

PtnQpOffset[nodeIdx] :=
  (1 − 2 × ptn_qp_offset_sign[nodeIdx]) × ptn_qp_offset_abs[nodeIdx]

The QP for a node is specified by the expression PtnQp[ nodeIdx ].

PtnQp[nodeIdx] :=
    ¬geom_scaling_enabled   ? 0 :
    nodeIdx % PtnQpInterval ? PtnQp[nodeIdx − 1]
                            : sliceQp + PtnQpOffset[nodeIdx] << geom_qp_mul_log2
  where
    sliceQp := geom_qp + slice_geom_qp_offset

It is a requirement of bitstream conformance that PtnQp[ 𝑖 ] shall be in the range 0 .. 167 for each node index 𝑖.

ptn_dup_point_cnt[ nodeIdx ] plus 1 specifies the number of points represented by the node.  When ptn_dup_point_cnt[ nodeIdx ] is not present, it shall be inferred to be 0.

ptn_child_cnt_xor1[ nodeIdx ] xor 1 specifies the number of child nodes.

ptn_inter_flag[ nodeIdx ] equal to 1 specifies that position of the node is coded with inter prediction. When ptn_inter_flag[ nodeIdx ] is not present, it shall be inferred to be 0.

ptn_pred_direction[ nodeIdx ] specifies whether (when 0) the first reference frame or (when 1) the second reference frame is used for inter prediction. When ptn_pred_direction[ nodeIdx ] is not present, it shall be inferred to be 0.

ptn_inter_pred_mode[ nodeIdx ] specifies the method used to predict the node’s coded point position using inter prediction.

ptn_pred_mode[ nodeIdx ] specifies the method used to predict the node's coded point position.

When adaptive quantization step size of the predictive geometry azimuth angle residuals is provided (ptree_ang_azimuth_scaling_enabled is 1), ptn_pred_mode[ nodeIdx ] is not present and shall be inferred to be 1.

ptn_pred_idx[ nodeIdx ] specifies the predictor index in a prediction list for angular coordinates to be used to predict the node's coded point position when adaptive quantization step size of the predictive geometry azimuth angle residuals is provided (ptree_ang_azimuth_scaling_enabled is 1).

ptn_phi_mul_abs_prefix[ nodeIdx ], ptn_phi_mul_abs_minus2[ nodeIdx ], ptn_phi_mul_abs_minus9[ nodeIdx ] and ptn_phi_mul_sign[ nodeIdx ] together specify, in accordance with PtnPhiMul[ nodeIdx ], an integer number of ptree_ang_azimuth_step_minus1 + 1 steps that form an azimuth prediction residual.  ptn_phi_mul_sign specifies whether (when 0) the step change is in the anticlockwise or (when 1) clockwise direction.  Any of ptn_phi_mul_sign, ptn_phi_mul_abs_minus2 or ptn_phi_mul_abs_minus9 that are not present shall be inferred to be 0.

PtnPhiMul[nodeIdx] := (1 − 2 × ptn_phi_mul_sign[nodeIdx]) × absVal
  where
    absVal := ptn_phi_mul_abs_prefix[nodeIdx]
            + ptn_phi_mul_abs_minus2[nodeIdx] + ptn_phi_mul_abs_minus9[nodeIdx]

When adaptive quantization step size of the predictive geometry azimuth angle residuals is provided (ptree_ang_azimuth_scaling_enabled is 1), the expression BoundPhiResid[ nodeIdx ] provides for the point being coded a maximum boundary for the absolute value of the azimuthal angle residual.  This value is determined in accordance with the radius coordinate residual residAng[ 0 ] and the predicted radius coordinate PtnPred[0 ] for that point, the azimuthal angle step size ptree_ang_azimuth_step_minus1 and the azimuthal angle precision ptree_ang_azimuth_pi_bits_minus11.

BoundPhiResid[nodeIdx] := DivExp2Fz(scaledR × phiStep, azimuthTwoPiLog2 + 1)
  where
    scaledR := (PtnPred[0] + residAng[0]) × 8
    phiStep := ptree_ang_azimuth_step_minus1 + 1
    azimuthTwoPiLog2 := ptree_ang_azimuth_pi_bits_minus11 + 12

When adaptive quantization step size of the predictive geometry azimuth angle residuals is provided (ptree_ang_azimuth_scaling_enabled is 1), the expression PtnPhiStep[ nodeIdx ] provides a dynamically adjusted azimuthal angle step for the point being coded.  When ptree_ang_azimuth_scaling_enabled is 0 it provides a constant azimuthal angle step.  This value is determined in accordance with the unitary azimuthal angle step size obtained from ptree_ang_azimuth_step_minus1 using the expression BoundPhiResid[ nodeIdx ], the radius coordinate residual residAng[ 0 ] and the predicted radius coordinate PtnPred[0 ] for that point.

PtnPhiStep[nodeIdx] :=
  (ptree_ang_azimuth_scaling_enabled &&
    radius && BoundPhiResid[nodeIdx] == 0 && (azimuthPiLog2 - phiStepArcLog2) > 0) ?
      (ptree_ang_azimuth_step_minus1 + 1) << (azimuthPiLog2 - phiStepArcLog2) :
      (ptree_ang_azimuth_step_minus1 + 1)
    where
       radius := PtnPred[0] + residAng[0]
       phiStepArcLog2 := IntLog2(radius × (ptree_ang_azimuth_step_minus1 + 1))
       azimuthPiLog2 := ptree_ang_azimuth_pi_bits_minus11 + 11

ptn_radius_resid_abs[ nodeIdx ], and ptn_radius_resid_sign[ nodeIdx ] together specify, in accordance with PtnRadiusResidual[ nodeIdx ], the first component of the first coordinate-prediction residual, when adaptive quantization step size of the predictive geometry azimuth angle residuals is provided.  ptn_radius_resid_sign[ nodeIdx] specifies whether (when 0) the residual's sign is positive or (when 1) negative.  When ptn_radius_resid_sign[ nodeIdx ]is not present, it shall be inferred to be 0.

PtnRadiusResidual[nodeIdx] := (1 − 2 × ptn_radius_resid_sign[nodeIdx]) × absVal[k]
  where
    absVal[k] := ptn_radius_resid_abs[nodeIdx]

ptn_phi_resid_abs_gt0[ nodeIdx ], ptn_phi_resid_sign[ nodeIdx ], ptn_phi_resid_abs_gt1[ nodeIdx ] and ptn_phi_resid_abs_rem[ nodeIdx ] together specify, in accordance with PtnPhiResidual[ nodeIdx ], the 2nd component of the first coordinate-prediction residual, when adaptive quantization step size of the predictive geometry azimuth angle residuals is provided.  ptn_phi_resid_sign[ nodeIdx] specifies whether (when 0) the residual's sign is positive or (when 1) negative.  Any of ptn_phi_resid_abs_gt0[ nodeIdx ], ptn_phi_resid_sign[ nodeIdx ], ptn_phi_resid_abs_gt1[ nodeIdx ] or ptn_phi_resid_abs_rem[ nodeIdx ] that are not present shall be inferred to be 0.

PtnPhiResidual[nodeIdx] := (1 − 2 × ptn_phi_resid_sign[nodeIdx]) × absVal[k]
  where
    absVal[k] := ptn_phi_resid_abs_gt0[nodeIdx]
               + ptn_phi_resid_abs_gt1[nodeIdx]
               + ptn_phi_resid_abs_rem[nodeIdx]

ptn_resid_abs_gt0[ nodeIdx ][ 𝑘 ], ptn_resid_abs_log2[ nodeIdx ][ 𝑘 ], ptn_resid_abs_rem[ nodeIdx ][ 𝑘 ] and ptn_resid_sign[ nodeIdx ][ 𝑘 ] together specify, in accordance with PtnResidual[ nodeIdx ][ 𝑘 ], the 𝑘-th component of the first coordinate-prediction residual.  ptn_resid_sign[ nodeIdx][ 𝑘 ] specifies whether (when 0) the residual's sign is positive or (when 1) negative.  Any of ptn_resid_abs_gt0[ nodeIdx ][ 𝑘 ], ptn_resid_sign[ nodeIdx ][ 𝑘 ], ptn_resid_abs_log2[ nodeIdx ][ 𝑘 ] or ptn_resid_abs_rem[ nodeIdx ][ 𝑘 ] that are not present shall be inferred to be 0.

PtnResidual[nodeIdx][k] := (1 − 2 × ptn_resid_sign[nodeIdx][k]) × absVal[k]
  where
    absVal[k] := ptn_resid_abs_gt0[nodeIdx][k]
               + (Exp2(ptn_resid_abs_log2[nodeIdx][k]) >> 1)
               + ptn_resid_abs_rem[nodeIdx][k]

ptn_sec_resid_abs[ nodeIdx ][ 𝑘 ] and ptn_sec_resid_sign[ nodeIdx ][ 𝑘 ] together specify, in accordance with PtnSecResidual[ nodeIdx ][ 𝑘 ], the 𝑘-th component of the second coordinate-prediction residual that applies after conversion from angular to Cartesian coordinates.  ptn_sec_resid_sign[ nodeIdx ][ 𝑘 ] specifies whether (when 0) the residual's sign is positive or (when 1) negative.  If ptn_sec_resid_sign[ nodeIdx ][ 𝑘 ] is not present, it shall be inferred to be 0.

PtnSecResidual[nodeIdx][k] :=
  (1 − 2 × ptn_sec_resid_sign[nodeIdx][k]) × ptn_sec_resid_abs[nodeIdx][k]

    - Tree traversal for reconstruction of point positions

      - State variables

The reconstruction of point positions from predictive tree nodes is specified in terms of the following state variables:

- The array PtnStack, a stack of ancestor node indexes; PtnStack[ dpth ] is the node index of the ancestor at depth dpth in the predictive tree for the current node.

The array PtnPredList, a prediction list for angular coordinates; PtnPredList[ ptn_pred_idx[ nodeIdx ]][k] is used for predicting point's k-th angular coordinate when geom_angular_enabled is 1 and when adaptive quantization step size of the predictive geometry azimuth angle residuals is provided.  The prediction list is dynamically updated after angular coordinates of points are coded (9.3.3.7).

- The variable PtnDepth, indicating the size of the stack of ancestor node indexes and equivalent to the depth of the current node in its predictive tree.

- The variable PtnIdx, the node index of the current node in the canonical traversal order.

- The variable PtnCnt, a count of nodes parsed from the bitstream.

- The variable PrevInterFrameRefIdx, providing the reference frame buffer index of the last inter decoded point.

- The variable PrevPhiResidSign, providing the sign of the last decoded non-null azimuthal angle residual.

- The variable PrevPhiMul, providing the value of the preceding decoded azimuthal angle step multiplier.

- The variable PrevRadiusResidSign, providing the sign of the last decoded non-null radius residual.

      - Initial state

The state variables PrevInterFrameRefIdx, PrevPhiResidSign, PrevPhiMul, PrevRadiusResidSign and InterFlagHist shall be initialized at the start of every GDU.

When slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1, initialization shall be performed by the parsing state restoration process (11.6.2.2).

Otherwise (slice_entropy_continuation is 0 and slice_inter_entropy_continuation is 0), the state variables PrevInterFrameRefIdx, PrevPhiResidSign, PrevPhiMul, PrevRadiusResidSign and InterFlagHist shall be set to 0.

      - Decoding a sequence of predictive trees

Each predictive tree is decoded according to the recursive application of 9.3.3.4, starting from its root node:

- At the start of each tree, the stack of ancestor node indexes is empty, PtnDepth = 0, and the array PtnPredList shall be initialized to 0.

- The root node of the first tree has node index PtnIdx = 0.

- The index of each subsequent tree's root node is the successor to the index of the last node in the preceding tree.

Predictive trees are decoded while PtnIdx is less than PtnCnt.

for (PtnIdx = 0; PtnIdx < PtnCnt; PtnIdx++) {
  PtnDepth = 0
  for (predIdx = 0; predIdx ≤ ptree_ang_max_pred_index; predIdx++)
    for (k = 0; k < 2; k++)
      PtnPredList[predIdx][k] = 0
  … /* decode predictive tree with root node index PtnIdx; see 9.3.3.3 */
}

      - Recursive decoding of a subtree

This subclause specifies the reconstruction of a node with index PtnIdx and the remainder of its subtree:

- The point positions for the node are reconstructed and appended to the output point lists as specified by 9.3.4.

- When adaptive quantization step size of the predictive geometry azimuth angle is provided (when ptree_ang_azimuth_scaling_enabled is 1), the prediction list for angular coordinates shall be updated as specified by 9.3.3.7.

- When inter prediction is enabled (inter_prediction_enabled is 1), the inter prediction reference buffer shall be updated as specified by 9.3.3.10.

- The node's index is appended to the stack of ancestor node indexes.

- The subtrees for every child node are reconstructed in turn by the recursive application of this subclause: the index of the first child node is PtnIdx + 1; each subsequent child node index is the successor to the last node index of the preceding subtree.

PtnStack[PtnDepth++] = PtnIdx /* push */

ptnChildCnt = ptn_child_cnt_xor1[PtnNodeIdx] ^ 1
for (i = 0; i < ptnChildCnt; i++) {
  PtnIdx++
  … /* Recursively reconstruct the i−th child subtree (9.3.3.4) */
}

After the node's subtree has been reconstructed, the node is removed from the stack of ancestor node indexes.

PtnDepth−− /* pop */

      - Update of the state variable PrevPhiResidSign and PrevInterFrameRefIdx after each coded ptn_phi_resid_sign syntax element

After each coded ptn_phi_resid_sign syntax element, the state variable PrevPhiResidSign shall be set equal to the coded value of ptn_phi_resid_sign.

After each coded ptn_phi_resid_sign syntax element, the state variable PrevInterFrameRefIdx shall be set equal to (ptn_inter_pred_mode > 1) when ptn_inter_flag is equal to 1.

      - Update of the state variables PrevPhiMul and PrevRadiusResidSign after each coded ptn_resid_sign[ nodeIdx ][0 ] syntax element

When ptree_ang_azimuth_scaling_enabled is 1, after each coded ptn_resid_sign[ nodeIdx ][0 ] syntax element (i.e. after each code radius residual sign), the state variable PrevPhiMul shall be set equal to the expression PtnPhiMul[ nodeIdx ], and the state variable PrevRadiusResidSign shall be set equal to the coded value of ptn_resid_sign[ nodeIdx ][0 ].

      - Dynamic update of the prediction list for angular coordinates

After the point position for a node is decoded, the prediction list for angular coordinates shall be updated according to the value of the decoded radius residual residAng[ 0 ] and the threshold ptree_ang_pred_list_radius_resid_threshold:

if (ptree_ang_azimuth_scaling_enabled) {
  for (predIdx = predIdxRemoved; predIdx > 0; predIdx--)
    for(k = 0; k < 2; k ++)
      PtnPredList[predIdx][k] = PtnPredList[predIdx - 1][k]
  for(k = 0; k < 2; k ++)
    PtnPredList[0][k] = PtnAng[k]
}
  where
    predIdxRemoved := flagNewObject ? ptree_ang_max_pred_index : ptnPredIdx
      where
        ptnPredIdx := ptn_pred_idx[PtnIdx]
        flagNewObject := Abs(residAng[0]) > ptree_ang_pred_list_radius_resid_threshold

      - Update of the state variable InterFlagHist after each coded ptn_inter_flag[ nodeIdx ] syntax element

When slice_inter_prediction is 1, after each coded ptn_inter_flag[ nodeIdx ] syntax element, the state variable InterFlagHist shall be set equal to the expression InterFlagHist << 1 & ptn_inter_flag[ nodeIdx ].

      - Generate inter prediction list for angular coordinates

After the point position for a node is decoded, the inter prediction list for angular coordinates shall be updated according to the value of the decoded azimuth and beam ID. The subclause 9.3.3.9.1 is invoked with PtnRefFrame and (when slice_inter_frame_ref_gmc[0] is 1) PtnAltRefFrame as input and inter predictor list PtnInterPredList as output. When slice_biprediction is 1, subclause 9.3.3.9.1 is invoked with PtnSecondRefFrame and (when slice_inter_frame_ref_gmc[1] is 1) PtnSecondAltRefFrame as input and inter predictor list PtnSecondInterPredList as output.

        - Update of inter prediction list

The input to this subclause are reference frames RefFrame and in some case AltRefFrame and the output is the inter predictor list InterPredList.

cnt = 0
for (qAzim = stAzim; qAzim <= MaxQAzim && cnt < 2; qAzim++)
  if (PtnRefFrame[beamId][qAzim][0] != -1) {
    for (k = 0; k < 3; k++)
      InterPredList[cnt][k] = RefFrame[beamId][qAzim][k]
    cnt++
  }
while(cnt < 2)
  InterPredList[cnt++] = na
if (global_motion_enabled)
  for (qAzim = stAzim; qAzim <= MaxQAzim && cnt < 4; qAzim++)
    if (AltRefFrame[beamId][qAzim][0] != -1) {
      InterPredList[cnt][0] = AltRefFrame[beamId][qAzim][0]
      if(slice_inter_frame_ref_gmc){
        deltaPhi = AltRefFrame[beamId][qAzim][1] – phiP
        InterPredList[cnt][1] = phiP
        if(deltaPhi >= (phiStep >> 1) ||
           deltaPhi <= -(phiStep >> 1)){
          phiMul = Div((deltaPhi) + (phiStep >> 1), phiStep, 0)
          InterPredList[cnt][1] += phiMul × phiStep
        }
      }
      else
        InterPredList[cnt][1] = AltRefFrame[beamId][qAzim][1]
      InterPredList[cnt][2] = AltRefFrame[beamId][qAzim][2]
      cnt++
    }
  while(cnt < 4)
    InterPredList[cnt++] = na
  where
    beamId := PtnAng[2]
    stAzim := DivExp2Fz(PtnAng[1], inter_azim_scale_log2)
    phiStep := ptree_ang_azimuth_step_minus1 + 1
    phiP := PointAng[ptIdx][1]
    where
      ptIdx := PtnStack[PtnDepth – 1]

      - Generation of derived reference frame

Subclause 9.3.3.10 applies to generate the derived reference frame(s).

The original reference frame is specified by PtnOriRefFrame.

When biprediction_enabled is 0, PtnOriRefFrame is set equal to PtnCurrFramePos.

When biprediction_enabled is 1 or 2, the reconstructed frame PtnCurrFramePos is indicated by the notional frame counter to be used as the reference frame for subsequent point cloud frames. PtnOriRefFrame is determined as follows:

- when slice_biprediction is 0, PtnOriRefFrame is set equal to the previously reconstructed I-frame or P-frame;

- when biprediction_enabled is 1 and slice_biprediction is 1, if the previously reconstructed point cloud frame is a B-frame, PtnOriRefFrame is set equal to the previously reconstructed B-frame; otherwise, PtnOriRefFrame is set equal to the I-frame or P-frame before the previously reconstructed I-frame or P-frame;

- when biprediction_enabled is 2 and slice_biprediction is 1, PtnOriRefFrame is set equal to the first reference frame derived by invoking subclause 9.2.15.3.

The first derived reference frame PtnRefFrame and (when global_motion_enabled is 1) the second derived reference frame PtnAltRefFrame are derived as follows:

- when global_motion_enabled is 0, PtnRefFrame is set equal to PtnOriRefFram;

- when global_motion_enabled is 1 and slice_inter_frame_ref_gmc is 0, PtnAltRefFrame is set equal to PtnRefFrame and then PtnRefFrame is set equal to PtnOriRefFrame;

- when global_motion_enabled is 1 and slice_inter_frame_ref_gmc is 1, the global motion compensation is applied to points in PtnOriRefFrame as follows:

if (global_motion_enabled)
  for(beamId = 0; beamId <= num_beams_minus1; beamId++)
    for(qAzim = MinQAzim; qAzim <= MaxQAzim; qAzim++)
      if(PtnOriRefFrame[beamId][qAzim][0] != -1) {      
        … /* Apply global compensation to point pt and add compensated point to PtnGlobFrame (9.3.3.12) */_
      }
  where
    pt[k] = PtnOriRefFrame[beamId][qAzim][k]

When resampling_enabled is 1, the radius values of points in PtnOriRefFrame are updated using global motion compensated points as follows:

for(beamId = 0; beamId <= num_beams_minus1; beamId++)
  for(qAzim = MinQAzim; qAzim <= MaxQAzim; qAzim++)
    if(PtnOriRefFrame[beamId][qAzim][0] != -1) {
        … /* Update radius of point pt in PtnOriRefFrame (9.3.3.13) */_
    }
  where
    pt[k] = PtnOriRefFrame[beamId][qAzim][k]

PtnRefFrame is set equal to PtnOriRefFrame and PtnAltRefFrame is set equal to PtnGlobFrame.

      - Generation of derived reference frame for the second reference frame

When biprediction_enabled is 1 or 2, and slice_biprediction is 1, the second original reference frame PtnSecondOriRefFrame is determined as follows.

- when biprediction_enabled is 1, PtnSecondOriRefFrame is set equal to the previously reconstructed I-frame or P-frame;

- when biprediction_enabled is 2, PtnSecondOriRefFrame is set equal to the second reference frame derived by invoking subclause 9.2.15.3.

The corresponding first derived reference frame PtnSecondRefFrame and (when global_motion_enabled is 1) the second derived reference frame PtnSecondAltRefFrame are derived as follows:

- when global_motion_enabled is 0, PtnSecondRefFrame is set equal to PtnSecondOriRefFram;

- when global_motion_enabled is 1 and slice_inter_frame_ref_gmc2 is 0, PtnSecondAltRefFrame is set equal to PtnSecondRefFrame and then PtnSecondRefFrame is set equal to PtnSecondOriRefFrame;

- when global_motion_enabled is 1 and slice_inter_frame_ref_gmc2 is 1, the global motion compensation is applied to points in PtnSecondOriRefFrame as follows:

if (global_motion_enabled)
  for(beamId = 0; beamId <= num_beams_minus1; beamId++)
    for(qAzim = MinQAzim; qAzim <= MaxQAzim; qAzim++)
      if(PtnSecondOriRefFrame[beamId][qAzim][0] != -1) {      
        … /* Apply global compensation to point pt and add compensated point to PtnSecondGlobFrame (9.3.3.12) */_
      }
  where
    pt[k] = PtnSecondOriRefFrame[beamId][qAzim][k]

When resampling_enabled is 1, the radius values of points in PtnSecondOriRefFrame are updated using global motion compensated points as follows:

for(beamId = 0; beamId <= num_beams_minus1; beamId++)
  for(qAzim = MinQAzim; qAzim <= MaxQAzim; qAzim++)
    if(PtnSecondOriRefFrame[beamId][qAzim][0] != -1) {
        … /* Update radius of point pt in PtnSecondOriRefFrame (9.3.3.13) */_
    }
  where
    pt[k] = PtnSecondOriRefFrame[beamId][qAzim][k]

PtnSecondRefFrame is set equal to PtnSecondOriRefFrame and PtnSecondAltRefFrame is set equal to PtnGlobFrame.

      - Global motion compensation

The spherical to cartesian coordinate conversion of point is applied as follows, the slice-relative angular origin in STV coordinates of the reference frame is specified by the expression AngularOriginRef[ 𝑘 ]:

ptCart[k] := AngularOriginRef[k] + (
            k == 0 ? DivExp2Fz(ρ × IntCos(φ, ptree_ang_azimuth_pi_bits_minus11 + 11), 24):
            k == 1 ? DivExp2Fz(ρ × IntSin(φ, ptree_ang_azimuth_pi_bits_minus11 + 11), 24):
            k == 2 ? DivExp2Fz(DivExp2Fz(BeamElev[i] × ρ, 15) − BeamOffsetV[i], 3): na)
            where
              ρ := pt[0] << ptree_ang_radius_scale_log2
              φ := pt[1]
              i := pt[2]

The global motion compensation of a point is applied based on the threshold values ThresBot and ThresTop, global motion AppliedGMMatrix and AppliedGMTrans as follows:

if(ptCart[2] < ThresBot || ptCart[2] > ThresTop)
  for(k = 0; k < 3; k++)
    ptCartRef[k] = DivExp2Fz(ptCartComp[k], 16) + AppliedGMTrans[k]
      where
        ptCartComp[k] := AppliedGMMatrix[k][0] × ptCart[0] + AppliedGMMatrix[k][1] × ptCart[1] + AppliedGMMatrix[k][2] × ptCart[2]

For the first reference frame, ThresBot is set equal to gm_thres_bot, ThresTop is set equal to gm_thres_top, AppliedGMMatrix is set equal to GMMatrix and AppliedGMTrans is set equal to gm_trans. When slice_biprediction is 1, ThresBot is set equal to gm_thres_bot2, ThresTop is set equal to gm_thres_top2, AppliedGMMatrix is set equal to GMMatrix2 and AppliedGMTrans is set equal to gm_trans2.

The compensated point is then converted back into angular domain as follows:

ptSph[0] = IntSqrt(ptC[0] × ptC[0] + ptC[1] × ptC[1] << 16) >> 8
ptSph[1] = IntAtan2(ptC[1] << 8, ptC[0] << 8)
ptSph[1] = (((ptSph[1] + 3294199) × 5340354 + off) >> sh) - (1 << azimLog2)
ptSph[2] = BeamIdxEst[ptC[0]][ptC[1]][ptC[2]]
  where
    ptC[k] = ptCartRef[k]
    azimLog2 = ptree_ang_azimuth_pi_bits_minus11 + 11
    sh = 44 – azimLog2
    off = 1 << sh – 1

The compensated point is then updated to buffer PtnGlobFrame as follows:

if (PtnGlobFrame[beamId][qAzim][0] == -1 || PtnGlobFrame[beamId][qAzim][0] > ptSph[0])
  for(k = 0; k < 3; k ++)
    PtnGlobFrame[beamId][qAzim][k] = ptSph[k]

      - Update radius after compensation

The final derived reference frame is generated as follows:

if (PtnGlobFrame[beamId][qAzim][0] != -1) {
  for(k = 0; k < 2; k++) {
    ptA[k] = PtnGlobFrame[beamId][qAzim][k]
    ptB[k] = PtnGlobFrame[beamId][qAzim][k]
  }
  if (ptA[1] < pt[1]) {
    for(qA = qAzim + 1; qA <= MaxQAzim; qA++)
      if (PtnGlobFrame[beamId][qA][0] != -1) {
        for(k = 0; k < 2; k++)
          ptB[k] = PtnGlobFrame[beamId][qAzim][k]
        break
      }
  } else if (ptA[1] > pt[1])
    for(qB = qAzim - 1; qB >= MinQAzim; qB--)
      if(PtnGlobFrame[beamId][qB][0] != -1) {
        for(k = 0; k < 2; k++)
          ptB[k] = PtnGlobFrame[beamId][qAzim][k]
        break
      }
  }
} else {
  for(qA = qAzim + 1; qA <= MaxQAzim; qA++)
    if (PtnGlobFrame[beamId][qA][0] != -1) {
      for(k = 0; k < 2; k++)
        ptA[k] = PtnGlobFrame[beamId][qAzim][k]
      break
    }
  for(qB = qAzim - 1; qB >= MinQAzim; qB--)
    if(PtnGlobFrame[beamId][qB][0] != -1) {
      for(k = 0; k < 2; k++)
        ptB[k] = PtnGlobFrame[beamId][qAzim][k]
      break
    }
}

When both ptA and ptB are not initialized (there is no point PtnGlobFrame[beamId] corresponding to beamId) the radius is not updated. When only one of ptA and ptB is initialized, both ptA and ptB are set equal to the initialized point as follows:

if (qA == MaxQAzim + 1 && qB != MinQAzim – 1)
  ptA = ptB
else if (qA != MaxQAzim + 1 && qB == MinQAzim – 1)
  ptB = ptA

Once points ptA and ptB are obtained, the radius is updated as follows:

if (!del0 || !del1)
  pt[0] = ptA[0]
else
  pt[0] = ptA[0] + (1 – 2×sgn) × (abs(nr) + (abs(dr) >> 1)) / abs(dr)
  where
    nr = del0 × (pt[1] – ptA[1])
    dr = del1
    sgn = !((nr > 0 && dr > 0) || (nr < 0 && dr < 0))
    where
      del0 = ptA[0] – ptB[0]
      del1 = ptA[1] – ptB[1]

    - Reconstruction of point coordinates

      - General

Subclause 9.3.4 specifies the reconstruction of the point positions for a node with index PtnIdx at depth PtnDepth of a predictive tree.

      - Reconstructed STV coordinates

The node's reconstructed STV coordinates are specified by the expression PtnPos[ 𝑘 ].  They are the sum of a prediction (predStv) and a scaled residual (residStv), with negative coordinates clipped to 0.  The predicted position is:

- when angular geometry coding is disabled: derived from the reconstructed STV coordinates of ancestor nodes (9.3.4.6);

- when angular geometry coding is enabled: a conversion from the node's reconstructed angular coordinates (9.3.4.5).

PtnPos[k] := Max(0, predStv[k] + residStv[k])
  where
    predStv[k] := geom_angular_enabled ? PtnAngStv[k] : PtnPred[k]

The scaled STV coordinate residuals, specified by residStv[ 𝑘 ], are:

- 0, when second coordinate residual coding is disabled; or otherwise

- coded by the node's first residual when angular geometry coding is disabled, and by its second residual when enabled; and

- scaled by the 3-fractional-bit, fixed-point, geometry scale factor specified by the expression sf for the node's QP; scaling shall round to the nearest integer with half-values rounded up.

residStv[k] := DivExp2Up(resid[k] × sf, 3)
  where
    sf := 8 + (PtnQp[PtnIdx] & 7) << PtnQp[PtnIdx] / 8
    resid[k] := geom_angular_enabled ? ptree_sec_resid_disabled ? 0
                                     : PtnSecResidual[PtnIdx][k]
                                     : PtnResidual[PtnIdx][k]

      - Reconstructed RPI node coordinates

When angular geometry coding is enabled, the node's reconstructed RPI coordinates are specified by the expression PtnAng[ 𝑘 ].  They are the sum of an angular coordinate prediction (9.3.4.6) and a residual (residAng).

It is a requirement of bitstream conformance that PtnAng[ 2 ] shall be in the range 0 .. num_beams_minus1.

PtnAng[k] := PtnPred[k] + residAng[k]

The residual RPI coordinates specified by residAng[ 𝑘 ] are the sum of a 𝜑-component offset, phiOffset and values PtnResiAng of primary residual.

residAng[k] := (k == 1 ? phiOffset : 0) + PtnResiAng[k]
  where
    phiOffset := phiOffset0 + phiOffset1
    PtnResiAng[k] :=
        k == 0 && ptree_ang_azimuth_scaling_enabled ? PtnRadiusResidual[PtnIdx] :
        k == 1 && ptree_ang_azimuth_scaling_enabled ? scaledPhiAng :
        PtnResidual[PtnIdx][k]
      where
        phiOffset0 := PtnPhiMul[PtnIdx] × PtnPhiStep[ nodeIdx ]
        phiOffset1 := ptree_ang_azimuth_scaling_enabled ? (
                      PtnResiAng[k] + phiOffset0 ≥ azimuthPi ? –2 × azimuthPi :
                      PtnResiAng[k] + phiOffset0 < -azimuthPi ? 2 × azimuthPi : 0) : 0
          where
            azimuthPi := 1 << (ptree_ang_azimuth_pi_bits_minus11 + 11)

When adaptive quantization step size of the predictive geometry azimuth angle residuals is provided, scaledPhiAng is the scaled value of the quantized primary residual.

scaledPhiAng := DivExp2Tz(PtnPhiResidual[PtnIdx] × invR, invRFracBits - azimuthTwoPiLog2)
  where
    azimuthTwoPiLog2 := ptree_ang_azimuth_pi_bits_minus11 + 12
    (invR, invRFracBits) := IntRecip(scaledR)
      where
        scaledR := (PtnPred[0] + residAng[0]) > 0 ? (PtnPred[0] + residAng[0]) × 8 : 1

      - Points represented by a node

The reconstructed STV and, if applicable, RPI point coordinates are appended to the output point lists PointPos and PointAng for each point represented by the node.

for (i = 0; i ≤ ptn_dup_point_cnt[PtnIdx]; i++, PointCnt++)
  for (k = 0; k < 3; k++) {
    PointPos[PointCnt][k] = PtnPos[k]
    if (geom_angular_enabled)
      PointAng[PointCnt][k] = PtnAng[k]
  }

The decoded point position is output for each point represented by the current node.

for (i = 0; i ≤ ptn_dup_point_cnt[curNodeIdx]; i++, PointCnt++)
  for (k = 0; k < 3; k++) {
    PointPos[PointCnt][k] = nodePos[k]
    if (geom_angular_enabled)
      PointAng[PointCnt][k] = nodeAng[k]
  }

      - Predicted STV coordinates for angular coded geometry

When angular geometry coding is enabled, the node's RPI coordinates are converted to Cartesian STV coordinates, as specified by PtnAngStv[ 𝑘 ], for prediction of the coded position.

PtnAngStv[k] := AngularOrigin[k] + (
    k == 0 ? DivExp2Fz(ρ × IntCos(φ, ptree_ang_azimuth_pi_bits_minus11 + 11), 24) :
    k == 1 ? DivExp2Fz(ρ × IntSin(φ, ptree_ang_azimuth_pi_bits_minus11 + 11), 24) :
    k == 2 ? DivExp2Fz(DivExp2Fz(BeamElev[i] × ρ, 15) − BeamOffsetV[i], 3)        : na)
  where
    ρ := PtnAng[0] << ptree_ang_radius_scale_log2
    φ := PtnAng[1]
    i := PtnAng[2]

      - Prediction from ancestor nodes

Node coordinates can be predicted from up to three ancestor nodes.  Depending upon whether angular geometry is enabled or disabled, the prediction is for either RPI or STV coordinates.  The coordinates of the parent, grandparent and great-grandparent nodes are specified by ptnP[ 𝑘 ], ptnG[ 𝑘 ] and ptnU[ 𝑘 ], respectively.

ptnP[k] := PtnRef[1][k]
ptnG[k] := PtnRef[2][k]
ptnU[k] := PtnRef[3][k]

PtnRef[ancestor][k] := geom_angular_enabled ? PointAng[ptIdx][k] : PointPos[ptIdx][k]
  where
    ptIdx := PtnStack[PtnDepth − ancestor]

When the adaptive quantization step size of the predictive geometry azimuth angle residuals is not provided, the predicted coordinates, specified by PtnPred[ 𝑘 ], are:

- when the prediction mode is 0 and if angular geometry is:

  - disabled, the origin ( 0, 0, 0 );

  - enabled, a coordinate specified by predAngMode0[ 𝑘 ] whose 𝜌-component is ptn_radius_min, and whose 𝜑- and 𝑖-components are the same as the parent node, or zero if the predicted node is the root node of a predictive tree.

- when the prediction mode is 1, the coordinates of the parent node;

- when the prediction mode is 2, the coordinates of the parent node, translated by the vector from the second to first ancestor;

- when the prediction mode is 3, the coordinates of the parent node, translated by the vector from the third to the second ancestor.

When adaptive quantization step size of the predictive geometry azimuth angle residuals is provided, the prediction mode is always 1 and the predicted coordinates, specified by PtnPred[ 𝑘 ], are:

- the coordinates specified by predAngMode0[ 𝑘 ] if the predicted node is the root node of a predictive tree;

- derived according to the prediction list for angular coordinates, for the two first predicted coordinates PtnPred[0 ] and PtnPred[1]; and the third coordinate of the parent node for the third predicted coordinate PtnPred[2 ].

PtnPred[k] :=
    interFlag ? (interPredDir ? PtnSecondInterPredList[interModeIdx] : 
    PtnInterPredList[interModeIdx]) :
    predMode == 0 && geom_angular_enabled ? predAngMode0[k] :
    predMode == 0 ? 0 :
    predMode == 1 && PtnIdx == 0 ? predAngMode0[k] :
    predMode == 1 && usePredList ? PtnPredList[ptnPredIdx][k] + PtnPredAdjust :
    predMode == 1 ? ptnP[k] :
    predMode == 2 ? ptnP[k] + ptnP[k] − ptnG[k] :
    predMode == 3 ? ptnP[k] + ptnG[k] − ptnU[k] : na
  where
    interFlag := ptn_inter_flag[PtnIdx]
    interPredDir := ptn_pred_direction[PtnIdx]
    interModeIdx:= ptn_inter_pred_mode[PtnIdx]
    predMode := ptn_pred_mode[PtnIdx]
    ptnPredIdx := ptn_pred_idx[nodeIdx]
    usePredList := ptree_ang_azimuth_scaling_enabled && k < 2 && ptnPredIdx > 0
    PtnPredAdjust :=
        k == 1 && abs(deltaPhi) >= phiStep ? phiMul × deltaPhi : 0
      where
        deltaPhi := ptnP[1] - PtnPredList[ptnPredIdx][1]
        phiStep := ptree_ang_azimuth_step_minus1 + 1
        phiMul := Div(deltaPhi, phiStep, 0)

predAngMode0[k] :=
  k == 0       ? ptn_radius_min :
  PtnDepth > 0 ? ptnP[k]        : 0

  - TriSoup

    - General

Subclause 9.4 specifies the parsing and the reconstruction of point positions from the TriSoup process. It applies when geom_tree_type is 0 and when trisoup_enabled is 1.

At first, the occupancy tree process described in subclause 9.2 is applied to obtain an occupancy tree at depth occtreeMaxDepthMinus1. The log2 node dimensions at depth occtreeMaxDepthMinus1 is equal to the value trisoup_node_size_log2_minus2 obtained from the geometry data unit header.

Then, the set of all occupied leaf nodes of the occupancy tree at depth occtreeMaxDepthMinus1 constitutes the set of TriSoup nodes. The slice geometry is represented by TriSoup edge vertices (9.4.3.1) located on the edges of the cuboid volumes associated with the occupied leaf nodes and, optionally, by residual values of centroid vertices determined from TriSoup edge vertices, and, optionally, by face vertices created from two centroid vertices in the cuboid volumes and in the adjacent nodes.

The point positions are reconstructed by generating (9.4.3.2) a set of TriSoup triangles from the TriSoup edge vertices, TriSoup face vertices and centroid vertices, and by voxelization (9.4.3.3) of the TriSoup triangles into points by a ray tracing process.

      - Definition and ordering of TriSoup nodes

TriSoup nodes are the occupied leaf nodes at maximum coded tree level occtreeMaxDepthMinus1, obtained from the decoding process of the occupancy tree as described in subclause 9.2. The TriSoup log2 node dimension is OccLvlNodeSizeLog2[ occtreeMaxDepthMinus1+1 ][ 𝑘 ] along the 𝑘-th component. The occupancy tree is coded such that the three TriSoup log2 node dimensions are all equal to trisoup_node_size_log2_minus2 + 2. Consequently, the volumes associated with TriSoup nodes are cubes having edge length equal to

TriSoupCubeSize := Exp2( trisoup_node_size_log2_minus2 + 2 )

The edge length TriSoupNodeSize[ nodeIdx ][ k ] of the nodeIdx-th TriSoup node is set as follows:

TriSoupNodeSize[nodeIdx][k] := TriSoupCubeSize

The TriSoup node ordering is inherited from the occupancy tree node ordering, as described in clause 9.2.2.2 applied to the leaf nodes of the occupancy tree. This order is obtained from a breadth-first traversal of the occupancy tree and, at each level, child nodes of a node are ordered following a Morton order. This results in a traversal in the ascending Morton order of node location.

The number of TriSoup nodes is numberTriSoupNodes. In subclause 9.4 nodeIdx is a node index between 0 and numberTriSoupNodes – 1 that refers to the nodeIdx-th TriSoup node.

The location TriSoupNodeLoc[ nodeIdx ][ k ] of the nodeIdx-th TriSoup node lower corner in the slice coordinate system is obtained as described in subclause 9.2.2.1 by

TriSoupNodeLoc[nodeIdx][k] = nodeLoc[k] × TriSoupCubeSize

where nodeLoc[ 𝑘 ] is obtained recursively on the occupancy tree levels as described in clause 9.2.2.3.

When one or both of trisoup_non_cubic_node_start_edge_present and trisoup_non_cubic_node_end_edge_present are equal to 1, TriSoupNodeLoc[ nodeIdx ][ 𝑘 ] and TriSoupNodeSize[ nodeIdx ][ 𝑘 ] are modified as follows:

if(trisoup_slice_bb_pos_bits > 0 && TriSoupNodeLoc[nodeIdx][k] < TriSoupSliceBbPos[k]){
  TriSoupNodeSize[nodeIdx][k] := 
    TriSoupNodeLoc[nodeIdx][k] + TriSoupNodeSize[nodeIdx][k] − TriSoupSliceBbPos[k]
  TriSoupNodeLoc[nodeIdx][k] := TriSoupSliceBbPos[k]
}

if(trisoup_slice_bb_width_bits > 0 && 
   TriSoupSliceBbBoundary[k]) < TriSoupNodeLoc[nodeIdx][k] + TriSoupNodeSize[nodeIdx][k])
  TriSoupNodeSize[nodeIdx][k] := 
    TriSoupSliceBbBoundary[k] − TriSoupNodeLoc[nodeIdx][k] + 1

      - Definition and ordering of TriSoup edges

The cuboid volume associated with the nodeIdx-th TriSoup node is depicted on Figure 27. Twelve edges are obtained from the boundaries of the volume. Each edge is defined by its start position edgePosStart and its end position edgePosEnd.

Figure 27 — Volume and edges associated with a TriSoup node

The start and end positions of the twelve edges are obtained from Table 41 as below:

for (edgeTag = 0; edgeTag < 12; edgeTag ++)
  for (k = 0; k < 3; k++) {
    edgePosStart[nodeIdx][edgeTag][k] = TriSoupNodeLoc[nodeIdx][k]
      + edgePosStartShift[edgeTag][k] × TriSoupNodeSize[nodeIdx][k]
    edgePosEnd[nodeIdx][edgeTag][k] = edgePosStart[nodeIdx][edgeTag][k]
      + edgePosEndShift[edgeTag][k] × TriSoupNodeSize[nodeIdx][k]
  }

By generating the twelve edges of the numberTriSoupNodes TriSoup nodes, a collection of 12×numberTriSoupNodes edges is obtained. An edge is uniquely defined by its start and end positions. Consequently, an edge may present multiple times in the collection of 12×numberTriSoupNodes edges because adjacent TriSoup nodes can share one or more common edges. By removing duplicated edges, a set of unique numberTriSoupEdges TriSoup edges is obtained.

Table 41 — Twelve edges' positions relative to the TriSoup node location

| edge number (edgeTag) | edgePosStartShift[edgeTag][k] |  |  | edgePosEndShift[edgeTag][k] |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | k =0 | k =1 | k =2 | k =0 | k =1 | k =2 |
| 0 | 0 | 0 | 0 | 1 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 1 | 0 |
| 2 | 0 | 1 | 0 | 1 | 0 | 0 |
| 3 | 1 | 0 | 0 | 0 | 1 | 0 |
| 4 | 0 | 0 | 0 | 0 | 0 | 1 |
| 5 | 0 | 1 | 0 | 0 | 0 | 1 |
| 6 | 0 | 1 | 1 | 0 | 0 | 1 |
| 7 | 1 | 0 | 0 | 0 | 0 | 1 |
| 8 | 0 | 0 | 1 | 1 | 0 | 0 |
| 9 | 0 | 0 | 1 | 0 | 1 | 0 |
| 10 | 0 | 1 | 1 | 1 | 0 | 0 |
| 11 | 1 | 0 | 1 | 0 | 1 | 0 |

In the following subclause 9.4 edgeIdx is an edge index between 0 and numberTriSoupEdges – 1 that refers to the edgeIdx -th TriSoup edge in the set of unique edges. uniqueEdgePosStart[ edgeIdx ] and uniqueEdgePosEnd[ edgeIdx ] specify the start and end position of edgeIdx-th unique edge. The set of unique TriSoup edges is ordered lexicographically, first in the three-component start position and second in the three-component end position of the edges as described in 9.4.1.3.

Ordering method of TriSoup edges

When there are two unique edges with index i and j, order of edges is determined as follows.

- Derive startPos_i and startPos_j as follows.

startPos_i = uniqueEdgePosStart[i][0] << 42
  + uniqueEdgePosStart[i][1] << 21
  + uniqueEdgePosStart[i][2]
startPos_j = uniqueEdgePosStart[j][0] << 42 
  + uniqueEdgePosStart[j][1] << 21 
  + uniqueEdgePosStart[j][1]

- If startPos_i is less than startPos_j, i shall be less than j.

- If startPos_i is greater than startPos_j, i shall be greater than j.

- Otherwise, endPos_i and endPos_j are derived as follows.

endPos_i = uniqueEdgePosEnd[i][0] << 42
  + uniqueEdgePosEnd[i][1] << 21
  + uniqueEdgePosEnd[i][2]
endPos_j = uniqueEdgePosEnd[j][0] << 42 
  + uniqueEdgePosEnd[j][1] << 21 
  + uniqueEdgePosEnd[j][1]

  - If endPos_i is less than endPos_j, i shall be less than j.

  - Otherwise, i shall be greater than j.

    - Syntax element semantics

      - Semantics of TriSoup syntax elements present in the geometry data unit header

trisoup_node_size_log2_minus2 plus 2 specifies the size of a TriSoup node.

trisoup_sampling_value_minus1 plus 1 specifies the sampling distance of each rayOrigin used by the ray tracing process to generate decoded points.

trisoup_num_unique_segments_bits_minus1 plus 1 specifies the length in bits of trisoup_num_unique_segments_minus1 syntax element.

trisoup_num_unique_segments_minus1 plus 1 specifies the number of TriSoup edges.

trisoup_vertex_number_bits specifies the number of bits for a decoded TriSoup vertex. The values VertexPrecisionLog2 and bitDropped specify the precision and the quantization scale of a decoded TriSoup vertex position.

VertexPrecisionLog2 := trisoup_vertex_number_bits > 0 ? trisoup_vertex_number_bits : trisoup_node_size_log2_minus2 + 2

bitDropped := max(0,  trisoup_node_size_log2_minus2 + 2  - VertexPrecisionLog2)

trisoup_centroid_vertex_residual_present specifies whether (when 1) or not (when 0) the residuals of centroid positions are present in the TriSoup syntax structure.

trisoup_face_vertex_enabled equal to 1 specifies that the eligibility of a face vertex is judged for each TriSoup node as specified in subclause 9.4.2.4 and that the face vertex may be present on the face of TriSoup nodes. When trisoup_face_vertex_enabled equal to 0 specifies that the eligibility of face vertex is not judged for any TriSoup node and that the face vertex is not present on the face of any TriSoup node. When trisoup_face_vertex_enabled is not present, it shall be inferred to be 0.

trisoup_halo_enabled specifies whether (when 1) or not (when 0) halo is applied in the ray tracing process.

trisoup_adaptive_halo_enabled specifies whether (when 1) or not (when 0) adaptive halo is applied in the ray tracing process. When trisoup_adaptive_halo_enabled is not present, it shall be inferred to be 0.

trisoup_vertex_merge_enabled specifies whether (when 1) or not (when 0) the vertex merge process that replaces multiple vertices near the corner of TriSoup nodes by a single vertex is applied.

trisoup_slice_bb_pos_bits specifies the length in bits of each trisoup_slice_bb_pos_xyz syntax element. When trisoup_slice_bb_pos_bits is not present, it shall be inferred to be 0.

trisoup_slice_bb_pos_xyz[ k] and trisoup_slice_bb_pos_log2_scale together specify the lower corner k-th XYZ coordinates of the slice bounding box in the slice coordinate system. The lower corner of the slice bounding box in STV coordinates is specified by the expression TriSoupSliceBbPos[ k]. When trisoup_slice_bb_pos_bits is 0, trisoup_slice_bb_pos_xyz[k ] and trisoup_slice_bb_pos_log2_scale shall be inferred to be 0.

TriSoupSliceBbPos[k] := 
trisoup_slice_bb_pos_xyz[StvToXyz[k]] << trisoup_slice_bb_pos_log2_scale

trisoup_slice_bb_width_bits specifies the length in bits of each trisoup_slice_bb_width_xyz syntax element. When trisoup_slice_bb_width_bits is not present, it shall be inferred to be 0.

trisoup_slice_bb_width_xyz[ k ] and trisoup_slice_bb_width_log2_scale together specify the k-th XYZ component of the slice bounding box dimensions in the slice coordinate system. When trisoup_slice_bb_width_bits is not 0, the upper boundary of the slice bounding box in STV coordinates is specified by the expression TriSoupSliceBbBoundary[k ].

TriSoupSliceBbBoundary[k] := TriSoupSliceBbPos[k] + 
trisoup_slice_bb_width_xyz[StvToXyz[k]] << trisoup_slice_bb_width_log2_scale

      - Semantics of TriSoup syntax elements associated with TriSoup edges

vertex_present[edgeIdx] specifies whether (when 1) or not (when 0) a TriSoup vertex exists on edgeIdx-th TriSoup edge.

has_vertex[nodeIdx][edgeTag] specifies whether (when 1) or not (when 0) a TriSoup vertex exists on edgeTag-th TriSoup edge of nodeIdx-th TriSoup node. has_vertex[nodeIdx][edgeTag] is set by corresponding vertex_present[edgeIdx] as defined in 9.4.1.1.

vertex_position[edgeIdx][bit] specifies a bit-th value of quantized one-dimensional vertex position according to the direction of each TriSoup edge with a vertex quantizedVertexPos[edgeIdx]. When vertex_present[edgeIdx] is equal to 0, quantizedVertexPos[edgeIdx] is set to -1. quantizedVertexPos[edgeIdx] is derived as follows.

quantizedVertexPos[edgeIdx] = 0
if (vertex_present[edgeIdx])
  for (bit = 0; bit < trisoup_vertex_number_bits; bit++)
    quantizedVertexPos[edgeIdx] += vertex_position[edgeIdx][bit] << bit
else
  quantizedVertexPos[edgeIdx] = -1

edgeVertex[nodeIdx][edgeTag] specifies a quantized one-dimensional vertex position according to the direction of edgeTag-th TriSoup edge of nodeIdx-th TriSoup node. edgeVertex[nodeIdx][edgeTag] is set by corresponding quantizedVertexPos[edgeIdx] as defined in 9.4.1.1.

When trisoup_vertex_merge_enabled is equal to 1, the value vertexNumOfCorner[ns][nt][nv] specifies the number of vertices around a node corner at ( ns, nt, nv ) and the threshold value thVertexMerge derived as follows.

thVertexMerge := min(2048, TriSoupCubeSize << 6)
for (edgeIdx = 0; edgeIdx < numberTriSoupEdges; edgeIdx++){
  if (vertex_present[edgeIdx]) {
    ns = uniqueEdgePosStart[edgeIdx][0]
    nt = uniqueEdgePosStart[edgeIdx][1]
    nv = uniqueEdgePosStart[edgeIdx][2]
    vertexNumOfCorner[ns][nt][nv] = 0
    
    ns = uniqueEdgePosEnd[edgeIdx][0]
    nt = uniqueEdgePosEnd[edgeIdx][1]
    nv = uniqueEdgePosEnd[edgeIdx][2]
    vertexNumOfCorner[ns][nt][nv] = 0
  }
}
for (edgeIdx = 0; edgeIdx < numberTriSoupEdges; edgeIdx++){
  if (vertex_present[edgeIdx]) {
    relativePos = (quantizedVertexPos[edgeIdx] << (8 + bitDropped)) + (128 << bitDropped)
    if (relativePos < thVertexMerge) {
      ns = uniqueEdgePosStart[edgeIdx][0]
      nt = uniqueEdgePosStart[edgeIdx][1]
      nv = uniqueEdgePosStart[edgeIdx][2]
      vertexNumOfCorner[ns][nt][nv]++
    }
    if ((TriSoupCubeSize << 8) - relativePos < thVertexMerge) {
      ns = uniqueEdgePosEnd[edgeIdx][0]
      nt = uniqueEdgePosEnd[edgeIdx][1]
      nv = uniqueEdgePosEnd[edgeIdx][2]
      vertexNumOfCorner[ns][nt][nv]++
    }
  }
}

      - Semantics of TriSoup syntax elements associated with TriSoup nodes

centroid_residual_is_zero[nodeIdx] specifies whether (when 1) or not (when 0) a centroid residual of nodeIdx-th node is 0.

centroid_residual_is_zero[nodeIdx], centroid_residual_magnitude[nodeIdx][bit], and centroid_residual_sign[nodeIdx] together specify a quantized residual of each centroid. When centroid_residual_is_zero[nodeIdx] is not present, it shall be inferred to be 1. When centroid_residual_magnitude[nodeIdx] and centroid_residual_sign[nodeIdx] are not present, they both shall be inferred to be 0.

The value driftQ[nodeIdx] specifies a quantized centroid residual.

driftQ[nodeIdx] := (2 × centroid_residual_sign[nodeIdx] - 1) × (¬centroid_residual_is_zero[nodeIdx] + magBits[nodeIdx])

has_face_vertex[nodeIdx][fvIdx] equal to 1 specifies that a face vertex is added on the nodeIdx-th TriSoup node’s face which is orthogonal to the direction of fvIdx-th component and located far side from the slice origin. has_face_vertex[nodeIdx][fvIdx] equal to 0 specifies that a face vertex is not added on the nodeIdx-th TriSoup node’s face which is orthogonal to the direction of fvIdx-th component and located far side from the slice origin. When has_face_vertex[nodeIdx][fvIdx] is not present, it shall be inferred to 0.

      - Presence of has_face_vertex

When trisoup_face_vertex_enabled is equal to 1, the presence of has_face_vertex[nodeIdx][fvIdx] is specified by the expression FaceEligible[nodeIdx][fvIdx]. The syntax element has_face_vertex[nodeIdx][fvIdx] shall be present in the TriSoup syntax when FaceEligible[nodeIdx][fvIdx] is equal to 1.

Centroid[nodeIdx] is the result of the process specified in 9.4.3.2.5. FaceVertex[nodeIdx][fvIdx] is the result of the process specified in 9.4.3.2.6. Let adjNode[nodeIdx][fvIdx] be the node index of the adjacent TriSoup node in the direction of fvIdx-th component from the nodeIdx-th TriSoup node.

When all the conditions listed below are true, FaceEligibleCond1 is set to 1.

- centroid_residual_is_zero[nodeIdx] is equal to 0, Centroid[nodeIdx][k] is greater than or equal to 0 and Centroid[nodeIdx][k] is less than or equal to TriSoupNodeSize[nodeIdx][k] for each k (k = 0, 1, 2), and

- adjNode[nodeIdx][fvIdx]-th TriSoup node is available, and

- centroid_residual_is_zero[adjNode[nodeIdx][fvIdx]] is equal to 0, Centroid[adjNode[nodeIdx][fvIdx]][k] is greater than or equal to 0 and Centroid[adjNode[nodeIdx][fvIdx]][k] is less than or equal to TriSoupNodeSize[adjNode[nodeIdx][fvIdx]][k] for each k (k = 0, 1, 2), and

When all the conditions listed below are true, FaceEligible[nodeIdx][fvIdx] is set to 1.

- FaceEligibleCond1 is 1, and

- ((fvIdx < 2) ? (has_vertex[nodeIdx][3 − fvIdx] + has_vertex[nodeIdx][6 − fvIdx] + has_vertex[nodeIdx][7 − fvIdx] + has_vertex[nodeIdx][11 − fvIdx]) : (has_vertex[nodeIdx][8] + has_vertex[nodeIdx][9] + has_vertex[nodeIdx][10] + has_vertex[nodeIdx][11])) is equal to either 2 or 3, and

- Both dp0 and dp1, which are derived as follows, are greater than 0.

evCnt = numTriSoupVertices[nodeIdx]
distMin = 0x7fffffff
for(evIdx = 0; evIdx < ((evCnt == 3) ? 1 : evCnt); evIdx++){
  ev0 = evIdx
  ev1 = evIdx + 1
  if( ev1 >= evCnt )
    ev1 −= evCnt
  evCoord0 = sortedVertices[nodeIdx][ev0] + 128
  evCoord1 = sortedVertices[nodeIdx][ev1] + 128
  if( ( TriSoupNodeSize[nodeIdx][fvIdx] ≠ evCoord0[fvIdx] ) || 
      ( TriSoupNodeSize[nodeIdx][fvIdx] ≠ evCoord1[fvIdx] ) )
    continue;
  middlePoint = ( evCoord0 + evCoord1 ) / 2
  distVec = (middlePoint – FaceVertex[nodeIdx][fvIdx]) >> 8
  dist = distVec[0] × distVec[0] + distVec[1] × distVec[1] + 
         distVec[2] × distVec[2]
  if( distMin > dist ) {
    evIdxMin[nodeIdx][fvIdx][0] = ev0
    evIdxMin[nodeIdx][fvIdx][1] = ev1
    distMin = dist
  }
}
eeVec = sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][1]] − 
        sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][0]]
eeVecNorm = IntSqrt(eeVec[0] × eeVec[0] + eeVec[1] × eeVec[1] + 
                    eeVec[2] × eeVec[2])
eUnitVec = eeVecNorm ? ( ( eeVec << 8 ) / eeVecNorm ) : 0
enVec = (faceVertex[nodeIdx][fvIdx] − 
                       sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][0]]) × 
         eUnitVec >> 8
nfVec = faceVertex[nodeIdx][fvIdx] − 
        sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][0]] − 
        ((enVec × eUnitVec) >> 8)
dp0 = DriftVec[nodeIdx] × nfVec
dp1 = DriftVec[adjNode[nodeIdx][fvIdx]] × nfVec

Otherwise, when all the conditions listed below are true, FaceEligible[nodeIdx][fvIdx] is set to 1.

- FaceEligibleCond1 is 1, and

- numTriSoupVertices[nodeIdx] is equal to 2 and numTriSoupVertices[adjNode[nodeIdx][fvIdx]] is equal to 2.

Otherwise, FaceEligible[nodeIdx][fvIdx] is set to 0.

    - TriSoup decoding process

TriSoup decoding process consists on the following three processes.

- Decoding of TriSoup edge vertices located on TriSoup edges (9.4.3.1)

- Derivation of TriSoup vertices in TriSoup nodes (9.4.3.2)

- Determination of decoded points by the voxelization of TriSoup triangles (9.4.3.3)

      - Decoding of TriSoup edge vertices located on TriSoup edges

Vertex flag (segment indicator) and vertex position of each TriSoup edge are decoded as follows.

- determination of neighbouring information (9.4.3.1.1)

- determination of contexts and decoding of vertex flag and vertex position using dynamic OBUF (9.4.3.1.2)

- initialization and decoding TriSoup vertices bits using OBUF (9.4.3.1.3)

        - Determination of Neighbouring Information

Mask information with 16 bits length neighbourMask[edgeIdx] is set for all unique TriSoup Edges (edgeIdx = 0, 1,…, numberTriSoupEdges) as follows.

- Assume edgeStartPos of edgeIdx-th unique TriSoup edge is (k0, k1, k2).

- A variable TCS is set to TriSoupCubeSize.

TCS := TriSoupCubeSize

- edgeDirection of edgeIdx-th unique TriSoup Edge is derived as follows,

  - If edgeEndPos of i-th unique TriSoup Edge is (k0+TCS, k1, k2), edgeDirection is set to 0.

  - Otherwise, if edgeEndPos of edgeIdx-th unique TriSoup Edge is (k0, k1+TCS, k2), edgeDirection is set to 1.

  - Otherwise, edgeDirection is set to 2.

- Construct neighbourMask[edgeIdx] as defined in 9.4.3.1.4.

Then edgePattern[edgeIdx] is derived for all unique TriSoup edges (edgeIdx = 0, 1,…, numberTriSoupEdges). edgePattern[edgeIdx] consists of unique edge indexes of nine neighbouring edges a through edgeIdx in the Table 42. When the corresponding edge of variable j (j  = 0, 1,…, 8) exists, edgePattern[edgeIdx][j] is set to the unique edge index of the corresponding edge. Otherwise, when the corresponding edge of variable j does not exist, edgePattern[edgeIdx][j] is set to -1.

Table 42 — edgeStartPos and edgeEndPos of neighbour edges

| j | edge | edgeStartPos, edgeEndPos |  |  |
| --- | --- | --- | --- | --- |
|  |  | edgeDirection = 0 | edgeDirection = 1 | edgeDirection = 2 |
| - | E | (k0, k1, k2), (k0 + TCS, k1, k2) | (k0, k1, k2), (k0, k1 + TCS, k2) | (k0, k1, k2), (k0, k1, k2 + TCS) |
| 0 | a | (k0 - TCS, k1, k2), (k0, k1, k2) | (k0, k1 - TCS, k2), (k0, k1, k2) | (k0, k1, k2 - TCS), (k0, k1, k2) |
| 1 | b | (k0, k1, k2 - TCS), (k0, k1, k2) | (k0, k1, k2 - TCS), (k0, k1, k2) | (k0, k1 - TCS, k2), (k0, k1, k2) |
| 2 | c | (k0, k1 - TCS, k2), (k0, k1, k2) | (k0 - TCS, k1, k2), (k0, k1, k2) | (k0 - TCS, k1, k2), (k0, k1, k2) |
| 3 | d | (k0, k1, k2), (k0, k1, k2 + TCS) | (k0, k1, k2), (k0, k1, k2 + TCS) | (k0 - TCS, k1, k2 + TCS), (k0, k1, k2 + TCS) |
| 4 | e | (k0, k1, k2), (k0, k1 + TCS, k2) | (k0 - TCS, k1 + TCS, k2), (k0, k1 + TCS, k2) | (k0, k1 - TCS, k2 + TCS), (k0, k1, k2 + TCS) |
| 5 | f | (k0, k1 - TCS, k2), (k0 + TCS, k1 - TCS, k2) | (k0 - TCS, k1, k2), (k0 - TCS, k1 + TCS, k2), | (k0 - TCS, k1, k2), (k0 - TCS, k1, k2 + TCS) |
| 6 | g | (k0, k1, k2 - TCS), (k0 + TCS, k1, k2 - TCS) | (k0, k1, k2 - TCS), (k0, k1 + TCS, k2 - TCS) | (k0, k1 - TCS, k2), (k0, k1 - TCS, k2 + TCS) |
| 7 | h | (k0, k1 - TCS, k2), (k0, k1 - TCS, k2 + TCS) | (k0, k1, k2 - TCS), (k0 + TCS, k1, k2 - TCS), | (k0, k1 - TCS, k2), (k0 + TCS, k1 - TCS, k2), |
| 8 | i | (k0, k1, k2 - TCS), (k0, k1 + TCS, k2 - TCS) | (k0 - TCS, k1 + TCS, k2 + TCS), (k0, k1 + TCS, k2 + TCS) | (k0 - TCS, k1 + TCS, k2 + TCS), (k0, k1 + TCS, k2 + TCS) |

        - Determination of Contexts and Decoding of Vertex Flag and Vertex Position

The following process is applied to all unique edges.

- Derive common information for vertex flag and position as defined in 9.4.3.1.5.

- Derive context ctxMap1 and ctxMap2 for vertex flag.

ctxMap1 = min(nclosestPattern, 2) × 15 × 2 +  (neighbEdge-1) × 2 + ((ctx1 == 4))
ctxMap2 = neighbEnd << 11
ctxMap2 |= (patternClose & (0b00000110)) << 9 - 1
ctxMap2 |= direction << 7
ctxMap2 |= (patternClose & (0b00011000))<< 5 - 3 
ctxMap2 |= (patternClose & (0b00000001))<< 4 
orderedPclosePar = 
  (((pattern >> 5) & 3) << 2) + (!!(pattern & 128) << 1) + !!(pattern & 256)
ctxMap2 |= orderedPclosePar

- Decode vertex flag vertex_present[edgeIdx] as shown in 9.4.3.1.3.

- If vertex_present[edgeIdx] is equal to 1, the following processes are applied to decode vertex_position[edgeIdx][bit].

- Derive context ctxMap1 and ctxMap2 for the first bit of position.

ctxFullNbounds =
(4 × (ctx0 <= 1 ? 0 : (ctx0 >= 3 ? 2 : 1)) + (max(1, ctx1) - 1)) × 2 + (ctxE == 3)
ctxMap1 = ctxFullNbounds × 2 + (nclosestStart > 0)
ctxMap2 = missedCloseStart << 8
ctxMap2 |= (patternClosest & 1) << 7
ctxMap2 |= direction << 5
ctxMap2 |= patternClose & (0b00011111)

- Decode the first bit of position vertex_position[edgeIdx][0] as shown in 9.4.3.1.3 and decoded bit is set as v.

- When trisoup_vertex_number_bits is greater than 1, the following processes are applied.

  - derive context for the second bit of position

ctxMap1 = ctxFullNbounds × 2 + (nclosestStart > 0)
ctxMap1 = (ctxMap1 << 1) + v
ctxMap2 = missedCloseStart << 8
ctxMap2 |= (patternClose & 1) << 7
ctxMap2 |= (patternClosest & 1) << 6
ctxMap2 |= direction << 4
ctxMap2 |= (patternClose & (0b00011111)) >> 1
orderedPclosePar = (((patternClose >> 5) & 3) << 2) + (!!(patternClose & 128) << 1) + !!(patternClose & 256)
ctxMap2 = (ctxMap2 << 4) + orderedPclosePar

  - Decode the second bit of position vertex_position[edgeIdx][1] as shown in 9.4.3.1.3 and variable v is updated.

v = (v << 1) + vertex_position[edgeIdx][1]

  - When trisoup_vertex_number_bits is greater than 2, the following processes are applied.

    - derive context for the third bit of position

ctxBits3 = (6 × (ctx0 >> 1) + missedCloseStart) × 2 + (ctxE == 3)
ctxBits3 = 4 × ctxBits3 + v

    - Decode the third bit of position vertex_position[edgeIdx][2] as shown in 9.4.3.1.3 and variable v is updated.

v = (v << 1) + vertex_position[edgeIdx][2]

    - When trisoup_vertex_number_bits is greater than 3, decode remaining bits of vertex_position[edgeIdx][bit] (bit=3,…,trisoup_vertex_number_bits-1) by bypass decoding and variable v is updated.

        - Initialization and Decoding TriSoup Vertices Bits Using OBUF

The following processes are applied to initialize OBUF instances.

- To decode vertex position bits of TriSoup nodes, a first buffer (TriSoup buffer) is created for all OBUF instances to be later used by TriSoup coding according to 12.3, and the OBUF buffer size obufBufferSize and the buffer depth obufLeafDepth of fully deployed trees are set as follows.

obufBufferSize := 20000
obufLeafDepth := 4

- The context arrays OBUF ACPMs are created according to 12.2.2.

- All OBUF instances used by TriSoup coding are created and initialized according to 12.2. The size nBit1 and nBit2 are set as according to Table 43.

  - When i is equal to 0, the OBUF instance TOI[ i ] is used for coding vertex presence flag for an edge.

  - When i is equal to 1, the OBUF instance TOI[ i ] is used for coding the first bit of vertex position.

  - When i is equal to 2, the OBUF instance TOI[ i ] is used for coding the second bit of vertex position.

- The array ctxIdxMap[ j][i ] is initialized according to Table 44, Table 45 and Table 46.

The following processes are applied to decode TrisSoup vertices bits.

- Each bit of vertex position of coded TriSoup nodes is decoded according to 12.4 by using two contextual information ctxMap1 and ctxMap2, which are set as the first contextual information info1 and the second contextual information info2, and calling the OBUF instances of TriSoup coding to get the decoded bin.

- During updating of dynamic OBUF trees, the primary part remains unchanged (is never reduced) and the dynamic of secondary part may be reduced by applying a OBUF tree updating process according to the 12.4.1.2.

Table 43 —  the size values (nBit1,nBit2) of TriSoup OBUF instance TOI[ i ]

|  | i |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| (nBit1,nBit2) | (7,15) | (6,11) | (7,15) |

Table 44 — Initial values of CtxIdxMap[ j][0] for coding vertex presence flag of an edge

| j | CtxIdxMap[ j][0] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 15 | 15 | 15 | 15 | 15 | 15 | 15 | 15 |
| 8 .. 15 | 15 | 15 | 42 | 96 | 71 | 37 | 15 | 15 |
| 16 .. 23 | 22 | 51 | 15 | 15 | 30 | 27 | 15 | 15 |
| 24 .. 31 | 64 | 15 | 48 | 15 | 224 | 171 | 127 | 24 |
| 32 .. 39 | 127 | 34 | 80 | 46 | 141 | 44 | 66 | 49 |
| 40 .. 47 | 127 | 116 | 140 | 116 | 105 | 39 | 127 | 116 |
| 48 ..55 | 114 | 46 | 172 | 109 | 60 | 73 | 181 | 161 |
| 56 ..63 | 112 | 65 | 240 | 159 | 127 | 127 | 127 | 87 |
| 64 .. 71 | 183 | 127 | 116 | 116 | 195 | 88 | 152 | 141 |
| 72 .. 79 | 228 | 141 | 127 | 80 | 127 | 127 | 160 | 92 |
| 80 .. 87 | 224 | 167 | 129 | 135 | 240 | 183 | 240 | 184 |
| 88 .. 95 | 240 | 240 | 127 | 127 | 127 | 127 | 127 | 127 |
| 96 ..103 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 104 ..111 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 112 ..119 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 120 ..127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |

Table 45 — Initial values of CtxIdxMap[ j][1] for coding the first bit of vertex position

| j | CtxIdxMap[ j][1] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 116 | 127 | 118 | 15 | 104 | 56 | 97 | 15 |
| 8 .. 15 | 96 | 15 | 29 | 15 | 95 | 15 | 46 | 15 |
| 16 .. 23 | 196 | 116 | 182 | 53 | 210 | 104 | 163 | 69 |
| 24 .. 31 | 169 | 15 | 114 | 15 | 121 | 15 | 167 | 63 |
| 32 .. 39 | 240 | 127 | 184 | 92 | 240 | 163 | 197 | 77 |
| 40 .. 47 | 239 | 73 | 179 | 59 | 213 | 48 | 185 | 108 |
| 48 ..55 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 56 ..63 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |

Table 46 — Initial values of CtxIdxMap[ j][2] for coding the second bit of vertex position

| j | CtxIdxMap[ j][2] |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 .. 7 | 141 | 127 | 127 | 127 | 189 | 81 | 36 | 127 |
| 8 .. 15 | 143 | 105 | 103 | 116 | 201 | 60 | 38 | 116 |
| 16 .. 23 | 116 | 127 | 15 | 127 | 153 | 59 | 15 | 116 |
| 24 .. 31 | 69 | 105 | 15 | 127 | 158 | 93 | 36 | 79 |
| 32 .. 39 | 141 | 161 | 116 | 127 | 197 | 102 | 53 | 127 |
| 40 .. 47 | 177 | 125 | 88 | 79 | 209 | 75 | 102 | 28 |
| 48 ..55 | 95 | 74 | 72 | 56 | 189 | 62 | 78 | 18 |
| 56 ..63 | 88 | 116 | 28 | 45 | 237 | 100 | 152 | 35 |
| 64 .. 71 | 141 | 240 | 127 | 127 | 208 | 133 | 101 | 141 |
| 72 .. 79 | 186 | 210 | 168 | 98 | 201 | 124 | 138 | 15 |
| 80 .. 87 | 195 | 194 | 103 | 94 | 229 | 82 | 167 | 23 |
| 88 .. 95 | 92 | 197 | 112 | 59 | 185 | 87 | 156 | 79 |
| 96 ..103 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 104 ..111 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 112 ..119 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |
| 120 ..127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 | 127 |

        - Construction of neighbour mask

Construct neighbourMask[edgeIdx] for each TriSoup edge corresponding to edgeIdx as follows based on Table 47.

- Initialize neighbourMask[edgeIdx] as 0.

- When node e1 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= 1

- When node e2 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 1)

- When node e3 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 2)

- When node e4 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 3)

- When node a1 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 4)

- When node a2 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 5)

- When node a3 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 6)

- When node a4 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 7)

- When node a1 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 8)

- When node a2 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 9)

- When node a3 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 10)

- When node a4 exists, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 11)

- When edgeDirection is equal to 1, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 13)

- When edgeDirection is equal to 2, neighbourMask[edgeIdx] is updated as follows.

neighbourMask[edgeIdx] |= (1 << 14)

Table 47 — Locations of neighbour nodes

| Node | TriSoupNodeLoc |  |  |
| --- | --- | --- | --- |
|  | edgeDirection = 0 | edgeDirection = 1 | edgeDirection = 2 |
| e1 | (k0, k1, k2) | (k0, k1, k2) | (k0, k1, k2) |
| e2 | (k0, k1 - TCS, k2) | (k0 - TCS, k1, k2) | (k0, k1 - TCS, k2) |
| e3 | (k0, k1, k2 - TCS) | (k0, k1, k2 - TCS) | (k0 - TCS, k1 - TCS, k2) |
| e4 | (k0, k1 - TCS, k2 - TCS) | (k0 - TCS, k1, k2 - TCS) | (k0 - TCS, k1, k2) |
| a1 | (k0 - TCS, k1, k2) | (k0, k1 - TCS, k2) | (k0, k1, k2 - TCS) |
| a2 | (k0 - TCS, k1 - TCS, k2) | (k0 - TCS, k1 - TCS, k2) | (k0, k1 - TCS, k2 - TCS) |
| a3 | (k0 - TCS, k1, k2 - TCS) | (k0, k1 - TCS, k2 - TCS) | (k0 - TCS, k1 - TCS, k2 - TCS) |
| a4 | (k0 - TCS, k1 - TCS, k2 - TCS) | (k0 - TCS, k1 - TCS, k2 - TCS) | (k0 - TCS, k1, k2 - TCS) |
| b1 | (k0 + TCS, k1, k2) | (k0, k1 + TCS, k2) | (k0, k1, k2 + TCS) |
| b2 | (k0 + TCS, k1 - TCS, k2) | (k0 - TCS, k1 + TCS, k2) | (k0, k1 - TCS, k2 + TCS) |
| b3 | (k0 + TCS, k1, k2 - TCS) | (k0, k1 + TCS, k2 - TCS) | (k0 - TCS, k1 - TCS, k2 + TCS) |
| b4 | (k0 + TCS, k1 - TCS, k2 - TCS) | (k0 - TCS, k1 + TCS, k2 - TCS) | (k0 - TCS, k1, k2 + TCS) |

        - Derive common information for vertex flag and position

Derive common information to decode vertex flag and position as follows.

- Derive ctxE, ctx0, ctx0, and direction as follows.

ctxE = 
(!!(neighbourMask[edgeIdx] & 1)) + (!!(neighbourMask[edgeIdx] & 2))
+ (!!(neighbourMask[edgeIdx] & 4)) + (!!(neighbourMask[edgeIdx] & 8)) - 1

ctx0 = 
(!!(neighbourMask[edgeIdx] & 16)) + (!!(neighbourMask[edgeIdx] & 32))
+ (!!(neighbourMask[edgeIdx] & 64)) + (!!(neighbourMask[edgeIdx] & 128))

ctx1 = (!!(neighbourMask[edgeIdx] & 256)) + (!!(neighbourMask[edgeIdx] & 512))
+ (!!(neighbourMask[edgeIdx] & 1024)) + (!!(neighbourMask[edgeIdx] & 2048))

direction = neighbourMask[edgeIdx] >> 13

- Derive pattern, patternClose, patternClosest, and nclosestPattern.

for(v = 0; v < 9; v++){
  if (segind[edgePattern[v]]){
    pattern |= 1 << v
    vertexPos2bits = 
      vertices[correspondanceSegment2V[idxEdge]] >> max(0, nbitsVertices - 2)
    if (towardOrAway[v18])
      vertexPos2bits = max2bits - vertexPos2bits // reverses for away
    if (vertexPos2bits >= mid2bits)
      patternClose |= 1 << v
    if (vertexPos2bits >= max2bits)
      patternClosest |= 1 << v
    nclosestPattern += vertexPos2bits >= max2bits && v <= 4
  }
}

- Derive missedCloseStart and nclosestStart.

missedCloseStart = !(pattern & 2) + !(pattern & 4)
nclosestStart =
  !!(patternClosest & 1) + !!(patternClosest & 2) + !!(patternClosest & 4)

- Derive neighbEdge, neighbEnd, and neighbStart.

neighbEdge = (neighbourMask[edgeIdx] >> 0) & 15
neighbEnd = (neighbourMask[edgeIdx] >> 4) & 15
neighbStart = (neighbourMask[edgeIdx] >> 8) & 15
if (direction == 2) {
  neighbEdge = ((neighbourMask[edgeIdx] >> 0 + 0) & 1)
  neighbEdge += ((neighbourMask[edgeIdx] >> 0 + 3) & 1) << 1
  neighbEdge += ((neighbourMask[edgeIdx] >> 0 + 1) & 1) << 2
  neighbEdge += ((neighbourMask[edgeIdx] >> 0 + 2) & 1) << 3

  neighbEnd = ((neighbourMask[edgeIdx] >> 4 + 0) & 1)
  neighbEnd += ((neighbourMask[edgeIdx] >> 4 + 3) & 1) << 1
  neighbEnd += ((neighbourMask[edgeIdx] >> 4 + 1) & 1) << 2
  neighbEnd += ((neighbourMask[edgeIdx] >> 4 + 2) & 1) << 3

  neighbStart = ((neighbourMask[edgeIdx] >> 8 + 0) & 1)
  neighbStart += ((neighbourMask[edgeIdx] >> 8 + 3) & 1) << 1
  neighbStart += ((neighbourMask[edgeIdx] >> 8 + 1) & 1) << 2
  neighbStart += ((neighbourMask[edgeIdx] >> 8 + 2) & 1) << 3
}

      - Derive TriSoup vertices corresponding each TriSoup node

TriSoup vertices for each TriSoup node shall be derived as following steps:

- Derive information related to vertex merge (9.4.3.2.1)

- Derive TriSoup vertices corresponding each TriSoup edge (9.4.3.2.2)

- Derive initial position of centroid vertices (9.4.3.2.3),

- Determine the dominant axis and sort vertices (9.4.3.2.4),

- Derive refined centroid position (9.4.3.2.5),

- Derive TriSoup face vertices corresponding each TriSoup node (9.4.3.2.6),

- Insert TriSoup face vertices to corresponding sorted TriSoup vertices (9.4.3.2.7)

        - Derive information related to vertex merge

When trisoup_vertex_merge_enabled is equal to 1, expressions activeNode[nodeIdx] and activeCorner[nodeIdx][n] for each TriSoup node corresponding to nodeIdx are derived as follows.

The expression activeNode[nodeIdx] indicates whether the TriSoup node corresponding to nodeIdx need to be processed.

numVertex = 0
for (edgeTag = 0; edgeTag < 12; edgeTag++) {
  if (edgeVertex[nodeIdx][edgeTag] >= 0) {
    numVertex++
  }
}
if (numVertex >= 6) {
  activeNode[nodeIdx] = 1
} else {
  activeNode[nodeIdx] = 0
}

The expression activeCorner[nodeIdx][n] indicates whether the corner corresponding to n (n=0,…,7) of the TriSoup node corresponding to nodeIdx need to be dealt with.

for (n = 0; n < 8; n++) {
  ns = TriSoupNodeLoc[nodeIdx][0] + TriSoupNodeSize[nodeIdx][0] × Bit(n, 0)
  nt = TriSoupNodeLoc[nodeIdx][1] + TriSoupNodeSize[nodeIdx][1] × Bit(n, 1)
  nv = TriSoupNodeLoc[nodeIdx][2] + TriSoupNodeSize[nodeIdx][2] × Bit(n, 2)
  if (vertexNumOfCorner[ns][nt][nv] >= 4) {
    activeCorner[nodeIdx][n] = 1
  } else {
    activeCorner[nodeIdx][n] = 0
  }
}

        - Derive TriSoup vertices corresponding each TriSoup edge

Vertices vertex[nodeIdx][vIdx][k] for each TriSoup node corresponding to nodeIdx are derived as follows.

The variable vIdx is initialized by 0.

For all edges (edgeTag=0,…,11) on a TriSoup node corresponding to nodeIdx, the edge direction and the expression edgeVertexModify[nodeIdx][edgeTag], which indicates whether the edge indexed by edgeTag on the TriSoup node corresponding to nodeIdx need to be modified are derived as follows.

- When edgeVertex[nodeIdx][edgeTag] is greater than or equal to 0, the following processes are applied.

  - Derive direction[k] (k=0, 1, 2):

for (k = 0; k < 3; k++) {
  direction[k] = edgePosEnd[nodeIdx][edgeTag][k] - edgePosStart[nodeIdx][edgeTag][k]
}

  - The expression edgeVertexModify[nodeIdx][edgeTag] is initialized by 0.

  - When trisoup_vertex_merge_enabled is equal to 1 and activeNode[nodeIdx] is equal to 1, the expression edgeVertexModify[nodeIdx][edgeTag] is modified as follows.

relativePos = (edgeVertex[nodeIdx][edgeTag] << (8 + bitDropped))
              + (128 << bitDropped)
n = edgePosStartShift[edgeTag][0]
    + (edgePosStartShift[edgeTag][1] << 1)
    + (edgePosStartShift[edgeTag][2] << 2)
if (relativePos < thVertexMerge && activeCorner[nodeIdx][n]) {
  edgeVertexModify[nodeIdx][edgeTag] = 1
}

n = edgePosEndShift[edgeTag][0]
    + (edgePosEndShift[edgeTag][1] << 1)
    + (edgePosEndShift[edgeTag][2] << 2)
if ((TriSoupCubeSize << 8) - relativePos < thVertexMerge
      && activeCorner[nodeIdx][n]) {
  edgeVertexModify[nodeIdx][edgeTag] = 1
}

  - When edgeVertexModify[nodeIdx][edgeTag] is equal to 0, dequantized vertex positions vertex[nodeIdx][vIdx][k] is derived.

for (k = 0; k < 3; k++) {
  vertex[nodeIdx][vIdx][k] =
    (edgePosStart[nodeIdx][edgeTag][k] - TriSoupNodeLoc[nodeIdx][k]) << 8
  vertex[nodeIdx][vIdx][k] -= 128
  if (direction[k] > 0){
    vertex[nodeIdx][vIdx][k] +=
      (edgeVertex[nodeIdx][edgeTag] << (bitDropped + 8)) + (128 << bitDropped)
  }
}
vIdx++

When trisoup_vertex_merge_enabled is equal to 1 and activeNode[nodeIdx] is equal to 1, vertex[nodeIdx][vIdx][k] is added as follows.

for (n = 0; n < 8; n++){
  if (activeCorner[nodeIdx][n]){
    vertex[nodeIdx][vIdx][0] = ((TriSoupNodeSize[nodeIdx][0] × Bit(n, 0)) << 8) – 128
    vertex[nodeIdx][vIdx][1] = ((TriSoupNodeSize[nodeIdx][1] × Bit(n, 1)) << 8) - 128
    vertex[nodeIdx][vIdx][2] = ((TriSoupNodeSize[nodeIdx][2] × Bit(n, 2)) << 8) - 128
    vIdx++
  }
}

Number of vertices numTriSoupVertices[nodeIdx] is set by vIdx.

numTriSoupVertices[nodeIdx] = vIdx

        - Derive initial position of centroid vertices

The initial position of centroid vertices Centroid[nodeIdx][k] for each TriSoup node nodeIdx is derived as follows as a weighted sum. Firstly, weights weights[i] associated with edge vertices of the node are initialized to zero.

for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
  weights[i] = 0
}

A total weight wTotal is also initialized to zero.

wTotal = 0

Then, weights weights[i] and total weight wTotal are derived as follows.

for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
  i2 = i + 1
  if (i2 >= numTriSoupVertices[nodeIdx])
    i2 -= numTriSoupVertices[nodeIdx]
  w = abs(vertex[nodeIdx][i][0] - vertex[nodeIdx][i2][0])
  w += abs(vertex[nodeIdx][i][1] - vertex[nodeIdx][i2][1])
  w += abs(vertex[nodeIdx][i][2] - vertex[nodeIdx][i2][2])

  weights[i] += w
  weights[i2] += w
  wTotal += 2 × w
}

The position Centroid[nodeIdx][] of the centroid is obtained as a weighted sum of edge vertices.

Centroid[nodeIdx][0] = 0
Centroid[nodeIdx][1] = 0
Centroid[nodeIdx][2] = 0
for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
  Centroid[nodeIdx][0] += weights[i] × vertex[nodeIdx][i][0]
  Centroid[nodeIdx][1] += weights[i] × vertex[nodeIdx][i][1]
  Centroid[nodeIdx][2] += weights[i] × vertex[nodeIdx][i][2]
}
Centroid[nodeIdx][0] = Centroid[nodeIdx][0] / wTotal
Centroid[nodeIdx][1] = Centroid[nodeIdx][1] / wTotal
Centroid[nodeIdx][2] = Centroid[nodeIdx][2] / wTotal

        - Determine the dominant axis and sort TriSoup vertices

When numTriSoupVertices[nodeIdx] is greater than 3, the dominant axis and sorted TriSoup vertices are derived as follows. Otherwise, the dominant axis is set to 0.

- The sum of triangle areas for each axis k (0, 1, 2) is derived as follows.

  - Derive two axes to calculate triangle areas based on Table 48.

Table 48 — Corresponding axes S1 and S2 with k

| Axis k | S1 | S2 |
| --- | --- | --- |
| 0 | 2 | 1 |
| 1 | 2 | 0 |
| 2 | 1 | 0 |

  - The score to sort each vertex is calculated in anti-clockwise order according to the edges of the projected square as follows:

for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
  if(vertex[nodeIdx][i][S1] >= TriSoupNodeSize[nodeIdx][S1]){
    Score[i] = vertex[nodeIdx][i][S2]
  } else if (vertex[nodeIdx][i][S2] >= TriSoupNodeSize[nodeIdx][S2]){
    Score[i] = TriSoupNodeSize[nodeIdx][S2] + TriSoupNodeSize[nodeIdx][S1] - 
            vertex[nodeIdx][i][S1]
  } else if (vertex[nodeIdx][i][S1] <= 0) {
    Score[i] = 2 × TriSoupNodeSize[nodeIdx][S2] + TriSoupNodeSize[nodeIdx][S1] - 
            vertex[nodeIdx][i][S2]
  } else {
    Score[i] = 2 × TriSoupNodeSize[nodeIdx][S2] + TriSoupNodeSize[nodeIdx][S1] + 
            vertex[nodeIdx][i][S1]
  }
}

  - Sort vertices by ascending order of scores.

  - Calculate the sum of triangle areas as follows

for (Axis = 0; Axis < 3; Axis++){
  Area = 0
  for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
    i2 = i + 1
    if (i2 >= numTriSoupVertices[nodeIdx])
      i2 -= numTriSoupVertices[nodeIdx]
    for (k = 0; k < 3; k++){
      v1[k] = sortedVertices[nodeIdx][i][k] – Centroid[nodeIdx][k]
      v2[k] = sortedVertices[nodeIdx][i2][k] – Centroid[nodeIdx][k]
    }
    cP = CrossProduct(v1,v2)
    Area += abs(cP[Axis]) 
  }
}

- The axis with the largest area is determined as the dominant axis, and sorted vertices associated with the dominant axis is retained.

        - Derive refined centroid position

When all the following conditions are true, centroid position Centroid[nodeIdx][k] for each TriSoup node shall be refined.

- trisoup_centroid_vertex_residual_present is equal to 1,

- At least one of the following conditions are true,

  - numTriSoupVertices[nodeIdx] is greater than 3,

  - All the conditions listed in 9.4.3.2.8 are true.

Otherwise, output of this process is the same as the output of 9.4.3.2.3.

Refinement process of centroid position is as follows.

- Derive a normal vector by retained dominant axis and sorted vertices derived in 9.4.3.2.4.

accumNormal = 0
if (numTriSoupVertices[nodeIdx] ≠  2){
  for (i = 0; i < numTriSoupVertices[nodeIdx]; i++){
    i2 = i + 1
    if (i2 >= numTrisoupVertices[nodeIdx])
      i2 -= numTriSoupVertices[nodeIdx]
    accumNormal = CrossProduct (sortedVertices[nodeIdx][i] – Centroid[nodeIdx],
                                sortedVertices[nodeIdx][i2] – Centroid[nodeIdx])
  }
} else {
  accumNormal = Centroid[nodeIdx] - TriSoupNodeSize[ adjNode[nodeIdx][fvIdx] ] -
                Centroid[adjNode[nodeIdx][fvIdx]]
}
absNormal = isqrt(accumNormal[0] × accumNormal[0] + 
                  accumNormal[1] × accumNormal[1] +
                  accumNormal[2] × accumNormal[2])
for (k = 0; k < 3; k++){
  normal[k] = (accumNormal[k] << 8) / absNormal
}

- Derive context indexes ctxMinMax for ctxDrift0 (centroid_residual_is_zero[nodeIdx]) and context indexes ctxIdxDritSign, lowS and highS for ctxDriftSign (centroid_residual_sign[nodeIdx]) as defined in 9.4.3.2.9.

- Derive dequantized centroid position residual driftDQ from decoded centroid position residual driftQ.

driftDQ = 0
if (driftQ){
  driftDQ = abs(driftQ) << bitDropped + 6
  half = 1 << 5 + bitDropped
  driftDQ += 2 × half / 3 – half
  if (driftQ < 0)
    driftDQ = -driftDQ
}

- Derive drift vector DriftVec[nodeIdx][].

for (k = 0; k < 3; k++) {
  DriftVec[nodeIdx][k] = (driftDQ × normal[k]) >> 6
}

- Refine centroid position.

for (k = 0; k < 3; k++) {
  Centroid[nodeIdx][k] += DriftVec[nodeIdx] [k]
}

- Bound centroid position.

for (k = 0; k < 3; k++) {
  Centroid[nodeIdx][k] = max(-128, Centroid[nodeIdx][k])
  Centroid[nodeIdx][k] = 
      min(((TriSoupCubeSize - 1) << 8) + 127, Centroid[nodeIdx][k])
}

        - Derive TriSoup face vertices corresponding each TriSoup node

When trisoup_face_vertex_enabled is equal to 1, for each TriSoup node, TriSoup face vertex faceVertex[nodeIdx][fvIdx] is set to the intersection of the face, which is orthogonal to the direction of fvIdx-th component and located far side from the slice origin, and the line segment between the centroid vertex of nodeIdx-th TriSoup node and that of adjNode[nodeIdx][fvIdx]-th TriSoup node for each fvIdx (fvIdx = 0, 1, 2) as follows:

c0facePos = (TriSoupNodeSize[nodeIdx][fvIdx] << 8 ) − 128
c0 = Centroid[nodeIdx]
c1 = Centroid[adjNode[nodeIdx][fvIdx]]
c1[fvIdx] += TriSoupNodeSize[nodeIdx][fvIdx] << 8
denom = c1[fvIdx] − c0[fvIdx]
t = denom ? (((c0facePos − c0[fvIdx]) << 8) / denom) : 0
faceVertex[nodeIdx][fvIdx] = c0 + ((t × (c1 − c0) + 128) >> 8)
faceVertex[nodeIdx][fvIdx][fvIdx] = c0facePos

        - Insert TriSoup face vertices to corresponding sorted TriSoup vertices

For each TriSoup node, when has_face_vertex[nodeIdx][fvIdx] is equal to 1 and numTriSoupVertices[nodeIdx] is not equal to 2, faceVertex[nodeIdx][fvIdx] is added to sortedVertices[nodeIdx] and sortedVertices[adjNode[nodeIdx][fvIdx]] by the following steps for each fvIdx (fvIdx = 0, 1, 2):

- faceVertex[nodeIdx][fvIdx] is inserted between sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][0]] and sortedVertices[nodeIdx][evIdxMin[nodeIdx][fvIdx][1]].

- Face vertex adjFaceVertex to be added to sortedVertices[adjNode[nodeIdx][fvIdx]] is derived as follows:

adjFaceVertex = faceVertex[nodeIdx][fvIdx]
adjFaceVertex[fvIdx] = −128

- Indices of sortedVertices[adjNode[nodeIdx][fvIdx]] adjEvIdxMin0 and adjEvIdxMin1 are derived as follows:

evCnt = numTriSoupVertices[adjNode[nodeIdx][fvIdx]]
distMin = 0x7fffffff
for(evIdx = 0; evIdx < ((evCnt == 3) ? 1 : evCnt); evIdx++){
  ev0 = evIdx
  ev1 = evIdx + 1
  if( ev1 >= evCnt )
    ev1 −= evCnt
  evCoord0 = sortedVertices[adjNode[nodeIdx][fvIdx]][ev0] + 128
  evCoord1 = sortedVertices[adjNode[nodeIdx][fvIdx]][ev1] + 128
  if( (evCoord0[fvIdx] ≠ 0) || (evCoord1[fvIdx] ≠ 0) )
    continue
  middlePoint = ( evCoord0 + evCoord1 ) / 2
  distVec = (middlePoint – adjFaceVertex) >> 8
  dist = distVec[0] × distVec[0] + distVec[1] × distVec[1] + 
         distVec[2] × distVec[2]
  if( distMin > dist ) {
    adjEvIdxMin0 = ev0
    adjEvIdxMin1 = ev1
    distMin = dist
  }
}

- adjFaceVertex is inserted between sortedVertices[adjNode[nodeIdx][fvIdx]][adjEvIdxMin0] and sortedVertices[adjNode[nodeIdx][fvIdx]][adjEvIdxMin1].

For each TriSoup node, when has_face_vertex[nodeIdx][fvIdx] is equal to 1 and numTriSoupVertices[nodeIdx] is equal to 2 (there are only 2 vertices in sortedVertices), faceVertex[nodeIdx][fvIdx] is added to sortedVertices[nodeIdx] by the following steps for each fvIdx (fvIdx = 0, 1, 2):

- If face_vertex[nodeIdx][fvIdx] shares the same face as the first TriSoup edge vertex in sortedVertices[nodeIdx], insert it ahead of the first element.

- If face_vertex[nodeIdx][fvIdx] shares the same face as the second TriSoup edge vertex in sortedVertices[nodeIdx], insert it after the last element.

        - Applicable conditions of non-closed surface reconstruction

The following conditions are applicable conditions of non-closed surface reconstruction for a TriSoup node corresponding to nodeIdx.

- trisoup_face_vertex_enabled is equal to 1.

- numTrisoupVertices[nodeIdx] is equal to 2.

- onSameFace, which indicates that edge vertices vertex[nodeIdx][0] and vertex[nodeIdx][1] are positioned on the same node face, is true.

onSameFace = false
for (k = 0; k < 3; k++) {
  if (vertex[nodeIdx][0][k] == vertex[nodeIdx][1][k]) {
    onSameFace = true
  }
}

- notOnSameEdge, which indicates that edge vertices vertex[nodeIdx][0] and vertex[nodeIdx][1] are not positioned on the same TriSoup edge, is true.

notOnSameEdge = true
counter = 0
for (k = 0; k < 3; k++) {
  if (vertex[nodeIdx][0][k] == vertex[nodeIdx][1][k] &&
      (vertex[nodeIdx][0][k] < 0 ||
        vertex[nodeIdx][0][k] >= (TriSoupNodeSize[nodeIdx] << 8) – 128)) {
    counter++
  }
}
if (counter >= 2){
  notOnSameEdge = false
}

        - Derive context indexes for refined centroid position

Derive context indexes for refined centroid position as follows.

- Derive context indexes ctxMinMax for ctxDrift0 (centroid_residual_is_zero[nodeIdx]).

minPos = sortedVertices[nodeIdx][0][dominantAxis]
maxPos = sortedVertices[nodeIdx][0][dominantAxis]
for (i = 1; i < numTriSoupVertices[nodeIdx]; i++){
  if (sortedVertices[nodeIdx][i][dominantAxis] < minPos)
    minPos = sortedVertices[nodeIdx][i][dominantAxis]
  if (sortedVertices[nodeIdx][i][dominantAxis] > maxPos)
    maxPos = sortedVertices[nodeIdx][i][dominantAxis]
}
ctxIdxDrift0 = min(8, (maxPos - minPos) >> (8 + bitDropped))

- Derive expressions highBound[nodeIdx] and lowBound[nodeIdx], context indexes ctxIdxDritSign, lowS and highS for ctxDriftSign (centroid_residual_sign[nodeIdx]).

halfDropped = bitDropped == 0 ? 0 : 1 << (bitDropped - 1)
bound = (TriSoupNodeSize[nodeIdx][dominantAxis] – 1) << 8
for (m = 1; m < TriSoupNodeSize[nodeIdx][dominantAxis]; m++){
  temp = Centroid[nodeIdx] + m × normal
  if (temp[0] < 0 || temp[0] > bound 
      || temp[1] < 0 || temp[1] > bound
      || temp[2] < 0 || temp[2] > bound)
    break
}
highBound[nodeIdx] = (m – 1) + halfDropped >> bitDropped

for (m = 1; m < TriSoupNodeSize[nodeIdx][dominantAxis]; m++){
  temp = Centroid[nodeIdx] - m × normal
  if (temp[0] < 0 || temp[0] > bound 
      || temp[1] < 0 || temp[1] > bound
      || temp[2] < 0 || temp[2] > bound)
    break
}
lowBound[nodeIdx] = (m – 1) + halfDropped >> bitDropped

lowBoundSurface = ((Centroid[nodeIdx][dominantAxis] – minPos) + 128 >> 8) + halfDropped >> bitDropped
highBOundSurface = ((maxPos – Centroid[nodeIdx][dominantAxis]) + 128 >> 8) + halfDropped >> bitDropped

lowS = min(7, lowBoundSurface)
highS = min(7, highBoundSurface)
ctxIdxDriftSign = lowBound[nodeIdx] == highBound[nodeIdx] ?
                                      0 : 1 + (lowBound[nodeIdx] < highBound[nodeIdx])

      - Determination of decoded points by the voxelization of TriSoup triangles

In this process, decoded points by the voxelization of TriSoup triangles are derived and added to PointPos as follows.

- Set PointCnt is equal to 0.

- Apply the following processes for each TriSoup nodes with nodeIdx

  - Initialize numDecPointsInNode as 0.

  - Point positions corresponding to TriSoup edge vertices are derived and added to decPoints[nodeIdx][] as defined in 9.4.3.3.1.

  - When numTriSoupVertices[nodeIdx] is greater than or equal to 3, or all the conditions listed in 9.4.3.2.8 are true, following processes are applied.

    - When numTriSoupVertices[nodeIdx] is greater than 3, add centroid to decoded points as decPoints[nodeIdx][numDecPointsInNode] and numDecPointsInNode is incremented.

    - Construct TriSoup triangles as defined in 9.4.3.3.2.

    - Derive point positions corresponding to each triangle and add the points to decode points as defined in 9.4.3.3.3.

  - Eliminate duplicate points in decPoints[nodeIdx][]. numDecPointsInNode is updated to the number of decoded points in the node nodeIdx after elimination of duplicated points. Because list of points decPoints[nodeIdx][] are disjoint by construction, elimination of duplicated points can be performed locally for each node nodeIdx individually independently on other nodes.

  - Add decoded points to pointPos.

for (i = 0; i < numDecPointsInNode; PointCnt++, i++){
  for (k = 0; k < 3; k++){
    pointPos[PointCnt][k] = decPoints[nodeIdx][numDecPointsInNode][k]
  }
}

        - Adding edge vertices to decoded points

The following processes applied for all vIdx (vIdx = 0,…,numTriSoupVertices[nodeIdx] - 1).

- Point position d_point[k] is derived:

for (k = 0; k < 3; k++) {
  d_point[k] = vertex[nodeIdx][vIdx][k] + 128 >> 8
}

- When all the following conditions are true, d_point is added as decPoints[nodeIdx][numDecPointsInNode] and numDecPointsInNode is incremented.

- d_point belongs to the node nodeIdx. The condition for the d_point belongs to the node nodeIdx is, for all k (k=0, 1, 2), d_point[k] is greater than or equal to TriSoupNodeLoc[nodeIdx][k] and lower than or equal to TriSoupNodeLoc[nodeIdx] [k] + TriSoupCubeSize – 1.

- bitDropped is greater than 0 or samplingValue is greater than 1.

if (pointWithinNode(d_point) && (bitDropped > 0 || samplingValue > 1)){
  for (k = 0; k < 3; k++) {
    decPoints[nodeIdx][numDecPointsInNode][k] = d_point[k]
  }
  numDecPointsInNode++
}
where
  pointWithinNode(point) :=
    point[0] >= nodeLoc[0]
    && point[0] <= nodeLoc[0] + TrisoupCubeSize – 1
    && point[1] >= nodeLoc[1]
    && point[1] <= nodeLoc[1] + TrisoupCubeSize – 1
    && point[2] >= nodeLoc[2]
    && point[2] <= nodeLoc[2] + TrisoupCubeSize – 1
  where
    nodeLoc = TriSoupNodeLoc[nodeIdx]

        - Construct TriSoup Triangles

Construct TriSoup Triangles for each TriSoup node corresponding to nodeIdx as follows.

- The number of sorted vertices numSortedVertices is set as the number of vertices in sortedVertices[nodeIdx].

- Set numTriangles as follows.

  - numTriangles = 1 (if numSortedVertices  is equal to 3),

  - numTriangles = numSortedVertices (otherwise).

- triangles[i][j][k] are derived as follows.

for (i = 0; i < numTriangles; i++){
  for (k = 0; k < 3; k++){
    triangles[i][0][k] = sortedVertices[nodeIdx][i][k]
  }
  i2 = i + 1
  if (numTriSoupVertices[nodeIdx]==2 && i2 >= numTriSoupVertices[nodeIdx])
    break
  else if (i2 >= numTriSoupVertices[nodeIdx])
    i2 -= numTriSoupVertices[nodeIdx]
  for (k = 0; k < 3; k++){
    triangles[i][1][k] = sortedVertices[nodeIdx][i2][k]
    triangles[i][2][k] = numTriangles == 1 ? 
      sortedVertices[nodeIdx][i + 2][k] : Centroid[nodeIdx][k]
  }
}

        - Derive point positions corresponding to each triangle and add to decoded points

The following processes operated for each triangle with index i (=0,…,numTriangles).

- In this process, edge1[], edge2[], rayVector[], v0[], preC[4], h and a shall be computed into 64 bits signed integer registers if letting the possibly of computing them into 64 bits signed integer registers.

- Derive edge vectors of a triangle.

edge1 = triangles[i][1] - triangles[i][0]
edge2 = triangles[i][2] - triangles[i][0]

- Determine the two directions for ray tracing.

minVal = 1 << 28
directionExcluded = 0
h = CrossProduct(edge1, edge2) >> 8
for (k = 0; k < 3; k++){
  rayVector = {0, 0, 0}
  rayVector[k] = 256
  a = (rayVector × h) >> 8
  if (abs(a) < minVal){
    minVal = abs(a)
    directionExcluded = k
}

- minRange and maxRange are derived as follows.

for (k = 0; k < 3; k++){
  minPos = min(triangles[i][0][k], min(triangles[i][1][k], triangles[i][2][k]))
  minRange[k] = max(0, minPos + 128 >> 8)
  maxPos = max(triangles[i][0][k], max(triangles[i][1][k], triangles[i][2][k]))
  maxRange[k] = min(TriSoupNodeSize[nodeIdx][k], maxPos + 128 >> 8)
}

- precompute preC[4] as follows.

v0 = triangles[i][0]
preC[0] = edge1[1] × edge2[2] - edge1[2] × edge2[1]
preC[1] = edge1[2] × edge2[0] - edge1[0] × edge2[2]
preC[2] = edge1[0] × edge2[1] - edge1[1] × edge2[0]
tmp = CrossProduct(v0, edge1) >> 8
preC[3] = tmp[0] × edge2[0] + tmp[1] × edge2[1] + tmp[2] × edge2[2]

- ray tracing is applied to each direction except for the direction corresponding to directionExcluded. decPoints[nodeIdx][] and numDecPointsInNode are updated as described in 9.4.3.3.4.

        - Ray tracing

In this process, decoded points are generated from a TriSoup triangle by ray tracing as follows.

- In the processes defined in 9.4.3.3.4, 9.4.3.3.5, 9.4.3.3.6, and 9.4.3.3.7, rayOrigin[], c[4], a1, a2, a3, deltaT1, deltaT2, V0[2], V1[2], V2[2], S and P[2] shall be computed into 64 bits signed integer registers if letting the possibly of computing them into 64 bits signed integer registers.

- Initialize variables rayVector[3], h, a, startposG1, startposG2, endposG1, endposG2, and rayOrigin[3] as defined in 9.4.3.3.5.

- haloTriangle, haloThickness and haloTriangle2D are set as defined in 9.4.3.3.6.

- isVisible[129][129] is initialized as follows.

for (i = startposG1; i <= endposG1; i += samplingValue)
  for (j = startposG2; j <= endposG2; j += samplingValue)
    isVisible[i][j] = false

- A variable g1 is initialized as startposG1.

- When g1 is less than or equal to endposG1, the following processes applied.

  - rayOrigin is updated.

rayOrigin[g1pos[direction]] = g1 << 8

  - A variable g2 is initialized as startposG2.

  - When g2 is less than or equal to endposG2, the following processes applied.

    - rayOrigin is updated.

rayOrigin[g2pos[direction]] = g2 << 8

    - P[2] is initialized.

P = {g1 << 8, g2 << 8}

    - isVisible[g1][ g2] is updated.

isVisible[g1][g2] = isPointInTriangle(P, V0, V1, V2, haloTriangle2D) ? true : false
where
  isPointInTriangle(P, V0, V1, V2, haloTriangle2D) :=
    ((P[0] - V0[0]) × (V1[1] - V0[1]) - (P[1] - V0[1]) × (V1[0] - V0[0])
    >= haloTriangle2D)
    && ((P[0] - V1[0]) × (V2[1] - V1[1]) - (P[1] - V1[1]) × (V2[0] - V1[0])
    >= haloTriangle2D)
    && ((P[0] - V2[0]) × (V0[1] - V2[1]) - (P[1] - V2[1]) × (V0[0] - V2[0])
    >= haloTriangle2D)

    - intersection[3] is initialized.

intersection[0] = rayOrigin[0]
intersection[1] = rayOrigin[1]
intersection[2] = rayOrigin[2]

    - intersection is updated.

if ((g2 - samplingValue) >= startposG2 && isVisible[g1][g2 - samplingValue])
  tBuffer[g1][g2] = tBuffer[g1][g2 - samplingValue] + deltaT2
else if ((g1 - samplingValue) >= startposG1 && isVisible[g1 - samplingValue][g2])
  tBuffer[g1][g2] = tBuffer[g1 - samplingValue][g2] + deltaT1
else
  tBuffer[g1][g2] = ((P[0] × a1) >> kTriSoupFpBits)+ ((P[1] × a2) >> 
  kTriSoupFpBits) + a3
intersection[direction] = intersection[direction] + tBuffer[g1][g2]

    - When isVisible[g1][ g2] is true, decPoints[nodeIdx][] and numDecPointsInNode are updated as defined in 9.4.3.3.7.

    - g2 is updated to search the next point.

g2 += samplingValue

  - g1 is updated to search the next point.

g1 += samplingValue

        - Initialize ray tracing parameters

Parameters related to the ray tracing process are initialized as follows.

- Initialize rayVector[3].

rayVector[0] = 0
rayVector[1] = 0
rayVector[2] = 0
rayVector[direction] = 1 << 8

- Calculate a cross product of rayVector and edge2 as h.

h = CrossProduct(rayVector, edge2) >> 8

- Calculate an inner product of edge1 and h as a.

a = InnerProduct(edge1, h) >> 8

- If a is greater than 256, the following processes are applied. Otherwise, the ray tracing process is completed for the input triangle and the direction.

- Starting positions startposG1 and startposG2, and ending positions endposG1 and endposG2 are derived.

g1pos[3] = { 1, 0, 0 }
g2pos[3] = { 2, 2, 1 }
startposG1 = minRange[g1pos[direction]]
startposG2 = minRange[g2pos[direction]]
endposG1 = maxRange[g1pos[direction]]
endposG2 = maxRange[g2pos[direction]]

- rayOrigin[3] is initialized.

rayOrigin[0] = minRange[direction][0] << 8
rayOrigin[1] = minRange[direction][1] << 8
rayOrigin[2] = minRange[direction][2] << 8

        - Set halo parameters

Parameters related to halo are set as follows.

- haloTriangle is set.

  - If trisoup_halo_enabled is equal to 0, haloTriangle is set as 0.

  - Otherwise, if samplingValue is strictly greater than 1, the value of haloTriangle is obtained depending on trisoup_adaptive_halo_enabled and samplingValue by

haloTriangle = trisoup_adaptive_halo_enabled? 50 × samplingValue : 50
haloTriangle = haloTriangle > 100 ? 100 : haloTriangle

  - Otherwise, if samplingValue is equal to 1, the value of haloTriangle is obtained depending on bitDropped by

haloBit = (((1 << bitDropped) - 1) << 8) / TriSoupCubeSize
haloBit = (haloBit × 24) / 32
haloTriangle = haloBit > 40 ? 40 : haloBit

- haloThickness is set depending on samplingValue.

haloThickness = samplingValue > 1 ? 16 : 32

- c[4], a1, a2, a3, deltaT1 and deltaT2 are calculated as follows.

c[0] = preC[0] / a
c[1] = preC[1] / a
c[2] = preC[2] / a
c[3] = preC[3] / a

a1 = c[g1pos[direction]]
a2 = c[g2pos[direction]]
a3 =((c[direction] × rayOrigin[direction]) >> 8) - c[3]

deltaT1 = a1 × samplingValue
deltaT2 = a2 × samplingValue

- V0[2], V1[2], V2[2] are set.

V0 = {triangles[i][0][g1pos[direction]], triangles[i][0][g2pos[direction]]}
V1 = {triangles[i][1][g1pos[direction]], triangles[i][1][g2pos[direction]]}
V2 = {triangles[i][2][g1pos[direction]], triangles[i][2][g2pos[direction]]}

- A variable S is calculated as follows.

S = (V0[0] - V1[0]) × (V2[1] - V1[1]) - (V0[1] - V1[1]) × (V2[0] - V1[0])

- Determine whether to swap V0 and V2 according to the value of S as follows.

if (S < 0)
  swap(V0,V2)

- 2D halo parameter haloTriangle2D is calculated as follows.

haloTriangle2D = -(((abs(S) + 128) >> 8) × haloTriangle)

        - Point position reconstruction

Point positions based on triangles are reconstructed and added to the decoded point cloud as follows.

- Voxel position foundvoxel[3] is calculated as follows.

for (k = 0; k < 3; k++)
  foundvoxel[k] = (TriSoupNodeLoc[nodeIdx][k] + intersection[k] + 128) >> 8

- Voxel position foundvoxelUp[3] is calculated as follows.

for (k = 0; k < 3; k++)
  intersectionUp[k] = intersection[k]
intersectionUp[direction] += haloThickness
for (k = 0; k < 3; k++){
  foundvoxelUp[k] = 
      (TriSoupNodeLoc[nodeIdx][k] + intersectionUp[k] + 128) >> 8
}

- Voxel position foundvoxelDown[3] is calculated as follows.

for (k = 0; k < 3; k++)
  intersectionDown[k] = intersection[k]
intersectionDown[direction] -= haloThickness
for (k = 0; k < 3; k++){
  foundvoxelDown[k] = 
      (TriSoupNodeLoc[nodeIdx][k] + intersectionDown[k] + 128) >> 8
}

- foundvoxel, foundvoxelUp and foundvoxelDown are added as decPoints[nodeIdx][numDecPointsInNode] and numDecPointsInNode is incremented respectively, under the condition they belong the TriSoup node nodeIdx. The condition for a point voxel belongs to the node nodeIdx is, for all k (k=0, 1, 2), voxel[k] is greater than or equal to TriSoupNodeLoc[nodeIdx] [k] and lower than or equal to TriSoupNodeLoc[nodeIdx][k] + TriSoupCubeSize – 1.

if(pointWithinNode(foundvoxel)){
  for (k = 0; k < 3; k++)
    decPoints[nodeIdx][numDecPointsInNode][k] = foundvoxel[k]
  numDecPointsInNode++
}
if(pointWithinNode(foundvoxelUp)){
  for (k = 0; k < 3; k++)
    decPoints[nodeIdx][numDecPointsInNode][k] = foundvoxelUp[k]
  numDecPointsInNode++
}
if(pointWithinNode(foundvoxelDown)){
  for (k = 0; k < 3; k++)
    decPoints[nodeIdx][numDecPointsInNode][k] = foundvoxelDown[k]
  numDecPointsInNode++
}
where
  pointWithintNode(point) :=
    point[0] >= nodeLoc[0]
    && point[0] <= nodeLoc[0] + TrisoupCubeSize – 1
    && point[1] >= nodeLoc[1]
    && point[1] <= nodeLoc[1] + TrisoupCubeSize – 1
    && point[2] >= nodeLoc[2]
    && point[2] <= nodeLoc[2] + TrisoupCubeSize – 1
  where
    nodeLoc = TriSoupNodeLoc[nodeIdx]

Slice attributes

  - General

Clause 10 specifies the reconstruction of a single slice attribute for the coded slice geometry.  The reconstructed attribute values are stored in the array PointAttr.

  - Point coordinates

    - General

Attribute coding can use either the slice's reconstructed STV point positions or the points' scaled angular coordinates.

The expression AttrPos[ ptIdx ][ 𝑘 ] specifies the coordinates of each point for attribute coding:

- When attr_coord_conv_enabled is 0 and attr_inter_prediction_enabled is 0, AttrPos is equivalent to PointPos, which is the slice geometry in the slice’s coordinate system.

- When attr_coord_conv_enabled is 0 and attr_inter_prediction_enabled is 1, AttrPos is equivalent to the slice geometry translated to the coding coordinate system by the addition of the slice origin, SliceOrigin.

- Otherwise, AttrPos[ ptIdx ][ 𝑘 ] are angular point coordinates as specified by 10.2.2.

AttrPos[ptIdx][k] := attr_coord_conv_enabled
  ? AttrPosAng[ptIdx][k]
  : (attr_inter_prediction_enabled 
  ? PointPos[ptIdx][k] + SliceOrigin[k]
  : PointPos[ptIdx][k])

    - Conversion to scaled angular coordinates

The conversion is specified by the expression AttrPosAng[ ptIdx ][ 𝑘 ].

When geom_tree_type is equal to 1 and  slice_attr_inter_prediction is equal to 1, the point’s angular coordinates shall be offset by the minimum value between the minimum angular coordinates of the current slice and previously applied offset value.  The minimum angular coordinates of the current slice are specified by the expression MinCurAng[k].  The previously applied offset value is specified by the expression MinRefAng[k].  The minimum value between MinRefAng and MinCurAng is specified by the expression MinAng[k].

MinCurAng[k] := geom_tree_type == 1 && k == 1
  ? −Exp2(ptree_ang_azimuth_pi_bits_minus11 + 10) : 0

MinAng[k] := geom_tree_type == 1 && slice_attr_inter_prediction == 1
  ? Min(MinCurAng[k], MinRefAng[k]) : MinCurAng[k]

Otherwise, the point's angular coordinates shall be offset by the minimum angular coordinates.

The offset coordinates shall be scaled.  Any negative coordinate after conversion shall be clipped to 0.

AttrPosAng[ptIdx][k] := DivExp2Up(relPos × attr_coord_conv_scale[k], 8)
  where
    relPos := Max(0, PointAng[ptIdx][k] – MinAng[k])

It is a requirement of bitstream conformance that attr_coord_conv_scale shall not cause any converted coordinate, AttrPosAng[ ptIdx ][ 𝑘 ], to be greater than Exp2( MaxSliceDimLog2 ) − 1.

When geom_tree_type is equal to 1, after the coordinate’s conversion of a slice, MinRefAng shall be set equal to MinAng.

  - Syntax element semantics

      - Attribute data unit coefficients

The array AttrCoeff, with elements AttrCoeff[ coeffIdx ][ 𝑐 ], contains transform coefficient values.  Elements of the array shall be initialized to zero.

zero_run_length_prefix, zero_run_length_minus3_div2, zero_run_length_minus3_mod2 and zero_run_length_minus11 together specify, in accordance with the expression ZeroRunLength, the number of consecutive transform coefficient tuples with all components equal to zero.  Any of zero_run_length_minus3_div2, zero_run_length_minus3_mod2 and zero_run_length_minus11 that are not present shall be inferred to be 0.

ZeroRunLength := zero_run_length_prefix
  + 2 × zero_run_length_minus3_div2 + zero_run_length_minus3_mod2
  + zero_run_length_minus11

      - Attribute coefficient tuples

Attribute coefficient values are signalled for a coeffIdx-th coefficient tuple when at least one component is not equal to zero.

coeff_abs[ 𝑐 ] and coeff_sign[ 𝑐 ] together specify the 𝑐-th transform coefficient component AttrCoeff[ coeffIdx ][ 𝑐 ].  coeff_sign[ 𝑐 ] specifies whether (when 0) the coefficient's sign is positive or (when 1) negative.  If coeff_sign[ 𝑐 ] is not present, it shall be inferred to be 0.

The coefficients of the coeffIdx-th tuple are specified by the derivation of AttrCoeff:

for (c = 0; c < AttrDim; c++){
  absVal = coeff_abs[c]
  if (c == AttrDim − 1)
    if (AttrDim == 1
      || AttrDim == 2 && coeff_abs[0] == 0
      || AttrDim == 3 && coeff_abs[0] == 0 && coeff_abs[1] == 0)
      absVal++
  AttrCoeff[coeffIdx][(c + 1) % AttrDim] = (1 − 2 × coeff_sign[c]) × absVal
}

- When a point is eligible for direct prediction, the LSBs of coeff_abs encode the direct predictor mode.

      - Raw attribute values

raw_attr_component_length, when present, specifies the length in bytes of each syntax element raw_attr_value.

raw_attr_value[ ptIdx ][ 𝑐 ] specifies the attribute value for the 𝑐-th component of the ptIdx-th point in canonical decoding order.  The length in bits of each syntax element is specified by the expression RawAttrValueBits.

RawAttrValueBits := raw_attr_width_present
  ? 8 × raw_attr_component_length
  : AttrBitDepth

  - Raw attribute decoding

This subclause applies when attr_coding_type is 3.

Attribute values shall be set equal to the corresponding raw_attr_value syntax elements.

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  for (c = 0; c < AttrDim; c++)
    PointAttr[ptIdx][c] = raw_attr_value[ptIdx][c]

  - Attribute decoding using the region-adaptive hierarchical transform

    - General

The region-adaptive hierarchical transform specified by 10.5 is a recursive two-point transform.  It applies when attr_coding_type is 0.

The transform constructs a spatial tree of 3D transform blocks using the slice geometry (10.5.2).  Basis vectors are calculated for each application of the transform, weighted in proportion to the significance of each coefficient.

When lossless_coding_enabled is 1, a transform domain prediction process predicts AC coefficients from the DC coefficients of certain adjoining blocks.  When slice_attr_inter_prediction is 1, a spatial tree of reference 3D transform blocks is constructed using the reference slice geometry (10.5.2.6).  The transform domain inter prediction process predicts the DC coefficient of the transform block of the root node in the spatial tree from the DC coefficient of certain reference block.  For layers other than the root, the transform domain inter prediction process modifies the prediction of AC coefficients from the AC coefficients of certain reference block.

When lossless_coding_enabled is 0, a sample domain prediction process predicts DC coefficients of the children nodes from the DC coefficients of certain adjoining blocks.  When slice_attr_inter_prediction is 1, a spatial tree of reference 3D transform blocks is constructed using the reference slice geometry (10.5.2.6).  The sample domain inter prediction process predicts the DC coefficient of the root node in the spatial tree from the DC coefficient of certain reference block.  For layers other than the root,  the sample domain inter prediction predicts the DC coefficients of the children nodes.

The reconstruction process is specified for a single attribute component Cidx ∈ 0 .. AttrDim − 1. It is skipped in transform level Lvl when:

- Lvl is greater than 0 and the number of blocks in this level is equal to the number of blocks in the previous transform level Lvl + 1, or

- Lvl is equal to 0 and the number of blocks in this transform level is equal to the number of coded points.

The reconstruction process starts by:

- mapping coded coefficients to the transform tree (10.5.3) and

- scaling the coded coefficients (10.5.4).

Then in turn for each level, starting from the root of the transform tree (Lvl = RahtRootLvl) and proceeding down the tree until completing level 0, :

- performing transform domain prediction (10.5.5) when lossless_coding_enabled is 1 or performing sample domain prediction (10.5.6) when lossless_coding_enabled is 0,

- applying the inverse transform (10.5.7),

- adding sample domain prediction (10.5.7) when lossless_coding_enabled is 0

The reconstructed attribute values are specified by 10.5.8.

    - Transform tree

      - General

The tree of transform bocks is defined recursively:

- In tree level 0, each block groups together points with identical attribute coordinates.

- Each subsequent tree level 𝑙 shrinks the preceding level 𝑙 − 1 by a factor of two in each dimension; each 2×2×2 block groups together up to eight blocks from the preceding level.

An example tree is illustrated in Figure 28.  The points a to f are grouped into blocks according to their attribute coordinates: C groups together c, d and e.  The weight of each block is the number of points spanned by the block (10.5.2.3): C has a weight of 3; B with a weight of 5 groups together C, b and f; A with a weight of 6 groups together B and a.

Key

| a to f | Points |
| --- | --- |
| A to C | Transform blocks |
| × | Transform coefficients at input to inverse transform for labelled block |
| ○ | Inverse transformed coefficient |
| ● | Inherited DC coefficient (See RahtDcCoeff) |
| 1 to 6 | Coefficient weights (See RahtCoeffWeightM) |

Figure 28 — Example RAHT tree, block weights and transform structure

      - State variables

The RAHT tree is specified in terms of the following state variables:

- The sparse array RahtCoeff of transform block coefficients; RahtCoeff[ lvl ][ bs ][ bt ][ bv ][ 𝑖 ] is the 𝑖-th coefficient for the block located at ( bs, bt, bv ) in transform level lvl.  Unset elements shall be inferred to be 0.

- The array RahtBlkLoc of transform block locations; RahtBlkLoc[ lvl ][ nIdx ][ 𝑘 ] is the location of the nIdx-th coded block in transform level lvl.

- The array RahtBlkCnt of node counts per tree level; RahtBlkCnt[ lvl ] is the number of blocks in transform level lvl.

- The variable RahtLvlCnt, the number of transform levels.

      - Transform block weight

The weight of the DC transform coefficient for a block located at ( bs, bt, bv ) in transform level lvl is specified by the expression RahtBlkWeight[ lvl ][ bs ][ bt ][ bv ].  It is equal to the number of points that the coefficient applies to.

  - The sum of all block weights in any transform level is equal to the number of coded points (PointCnt).

  - A block's weight is equal to the sum of its child block weights.

RahtBlkWeight[lvl][bs][bt][bv] :=
    RahtBlkWeight = 0
    for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
      RahtBlkWeight += isPointInSubtree[ptIdx]
  where
    isPointInSubtree[ptIdx] :=
      bs == AttrPos[ptIdx][0] >> lvl
      && bt == AttrPos[ptIdx][1] >> lvl
      && bv == AttrPos[ptIdx][2] >> lvl

      - Number of transform levels and per-level block order

The root node of the transform tree is the lowest block in the tree with a DC coefficient that spans the entire geometry; i.e. it has a weight equal to the number of coded points, PointCnt.

The tree level containing the root node is RahtRootLvl:

RahtRootLvl := RahtLvlCnt − 1

Within a transform level, blocks are ordered for coefficient coding by ascending Morton-coded block location, as specified by the derivation of RahtBlkLoc.  Empty blocks are ignored.

for (RahtLvlCnt = 0; !done; RahtLvlCnt++)
  for (mIdx = 0, nIdx = 0, wSum = 0; wSum < PointCnt; mIdx++) {
    (bs, bt, bv) = FromMorton(mIdx)
    wSum += RahtBlkWeight[RahtLvlCnt][bs][bt][bv]
    if (RahtBlkWeight[RahtLvlCnt][bs][bt][bv] == 0)
      continue
    RahtBlkCnt[RahtLvlCnt]++
    RahtBlkLoc[RahtLvlCnt][nIdx][0] = bs
    RahtBlkLoc[RahtLvlCnt][nIdx][1] = bt
    RahtBlkLoc[RahtLvlCnt][nIdx][2] = bv
    nIdx++
    done = RahtBlkWeight[RahtLvlCnt][bs][bt][bv] == PointCnt
  }

      - 2×2×2 transform block coefficient weights

Transform coefficient weights are specified for each directional stage of the two-point transform and inverse transform for 2×2×2 transform blocks by the expression RahtCoeffWeightM[ lvl ][ stage ][ bs ][ bt ][ bv ][ 𝑚 ]; the parameter(s):

- bs, bt and bv specify a transform block location in tree level lvl, lvl > 0;

- 𝑚 specifies the transform coefficient index in forward transform stage or inverse transform stage stage.

RahtCoeffWeightM[lvl][stage][bs][bt][bv][m] := RahtCoeffWeight[lvl][stage][s][t][v]
  where
    s := 2 × bs + FromMorton[m][0]
    t := 2 × bt + FromMorton[m][1]
    v := 2 × bv + FromMorton[m][2]

Within a block, coefficient weights are determined iteratively starting from its child block weights (stage 0) to the transform block coefficient weights of stage 3.  At each stage, for each pair of inverse-transformed values 𝑎 and 𝑏, the weight for the DC (wL) and AC (wH) coefficient is the sum of the weights for a and b.  If the weight for either 𝑎 or 𝑏 is 0, the AC coefficient weight is 0.

The expression RahtCoeffWeight[ lvl ][ stage ][ 𝑠 ][ 𝑡 ][ 𝑣 ] specifies the derivation of a weight in transform stage stage for the coefficient corresponding to the block located at ( 𝑠, 𝑡, 𝑣 ) in tree level lvl − 1.

RahtCoeffWeight[lvl][stage][s][t][v] :=
    stage == 0 ? RahtBlkWeight[lvl − 1][s][t][v] :
    stage == 1 ? v % 2 == 0 ? wL[0][0][1] : wH[0][0][−1] :
    stage == 2 ? t % 2 == 0 ? wL[0][1][0] : wH[0][−1][0] :
    stage == 3 ? s % 2 == 0 ? wL[1][0][0] : wH[−1][0][0] : na
  where
    wL[ds][dt][dv] := wSum[ds][dt][dv]
    wH[ds][dt][dv] := wSum[ds][dt][dv] × wHnz[ds][dt][dv]
    wSum[ds][dt][dv] := wP[s][t][v] + wP[s + ds][t + dt][v + dv]
    wHnz[ds][dt][dv] := wP[s][t][v] × wP[s + ds][t + dt][v + dv] > 0
    wP[s][t][v] := RahtCoeffWeight[lvl][stage − 1][s][t][v]

- RahtBlkWeight[ lvl ][ 𝑠 ][ 𝑡 ][ 𝑣 ] ≡ RahtCoeffWeight[ lvl ][ 3 ][ 2 × 𝑠 ][ 2 × 𝑡 ][ 2 × 𝑣 ]; lvl > 0.

In the example of Figure 28, block B has stage 0 coefficient weights of 1, 3 and 1; stage 1 and 2 weights of 1, 4 and 1; and stage 3 weights of 5, 4 and 5.  RahtCoeffWeight[ 1 ][ 1 ][ 3 ][ 0 ][ 2 ] would be 4.

      - Reference transform tree

The tree of reference transform blocks is defined recursively, applying the same process as the tree of transform blocks and using the reference slice geometry RefAttrPos:

- In tree level 0, each block groups together points with identical attribute coordinates.

- Each subsequent tree level 𝑙 shrinks the preceding level 𝑙 − 1 by a factor of two in each dimension; each 2×2×2 block groups together up to eight blocks from the preceding level.

The weight of the DC transform coefficient for a reference block located at ( bs, bt, bv ) in transform level lvl is specified by the expression RahtBlkWeightRef[ lvl ][ bs ][ bt ][ bv ].  It is equal to the number of points that the coefficient applies to, which is derived from the RefAttrPos in a similar way as RahtBlkWeight in 10.5.2.3.

The sum of attributes of points within each reference block is recorded when defining the tree of reference transform blocks.  The sum of attributes of points within a reference block located at ( bs, bt, bv ) in transform level lvl is specified by the expression RahtBlkSumAttrRef[ lvl ][ bs ][ bt ][ bv ], which is derived from RefPointAttr.

Transform coefficient weights are specified for each directional stage of the two-point transform for reference 2×2×2 transform blocks by the expression RahtCoeffWeightMRef[ lvl ][ stage ][ bs ][ bt ][ bv ][ 𝑚 ], which is derived from RahtBlkWeightRef in a similar way as RahtCoeffWeightM in 10.5.2.5; the parameter(s):

- bs, bt and bv specify a transform block location in tree level lvl, lvl > 0;

- 𝑚 specifies the transform coefficient index in forward transform stage stage.

    - Coefficient order

      - General

Subclause 10.5.3 specifies the correspondence between coded transform coefficients and the transform tree.

Starting from the root of the transform tree and proceeding in breadth-first order, coefficients are coded for each transform block; all transform blocks within one tree level are coded before those of the next level.  Within a tree level, blocks shall be traversed in ascending Morton order of block location.

The order of coefficients within a transform block is specified by 10.5.3.2 for 2×2×2 blocks (tree levels greater than 0) and 10.5.3.3 for blocks of co-located points (tree level 0).

The mapping from the coded order to the transform tree is specified in terms of the following variables:

- Lvl, the index of the mapped transform level.

- CoeffIdx, the index into the decoded coefficient array AttrCoeff for the next mapped coefficient.

CoeffIdx = 0
for (Lvl = RahtLvlCnt; Lvl ≥ 0; Lvl−−) {
  if (Lvl > 0)
    if (RahtBlkCnt[Lvl] != RahtBlkCnt[Lvl-1]) {
      … /* See 10.5.3.2 */
    } 
  else 
    if (RahtBlkCnt[Lvl] != PointCnt) {
      … /* See 10.5.3.3 */
    }
}

      - Mapping for a tree level of 2×2×2 transform blocks

This subclause applies to tree levels greater than 0.

For each 2×2×2 transform block, up to 7 AC coefficients are mapped from the bitstream to coefficient indexes within the block.  In the case of the root transform block, the DC coefficient is additionally mapped.

Table 49 specifies the order in which transform block coefficients are coded; RahtCoeffOrder[ 𝑖 ] is the block index of the 𝑖-th coded coefficient.

Only coefficients with a non-zero transform coefficient weight are coded.

Table 49 — 2×2×2 RAHT coefficient coding order

| 𝑖 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RahtCoeffOrder[ 𝑖 ] | 0 | 4 | 2 | 1 | 6 | 5 | 3 | 7 |

for (nIdx = 0; nIdx < RahtBlkCnt[Lvl]; nIdx++) {
  bs = RahtBlkLoc[Lvl][nIdx][0]
  bt = RahtBlkLoc[Lvl][nIdx][1]
  bv = RahtBlkLoc[Lvl][nIdx][2]
  for (i = 0; i < 8; i++) {
    /* skip the DC coefficient that will be inherited */
    if (i == 0 && Lvl < RahtRootLvl)
      continue
    if (RahtCoeffWeightM[Lvl][3][bs][bt][bv][RahtCoeffOrder[i]] > 0)
      RahtCoeff[Lvl][bs][bt][bv][RahtCoeffOrder[i]] = AttrCoeff[CoeffIdx++][Cidx]
  }
}

      - Mapping for co-located points

This subclause applies to the final tree level (Lvl == 0), after all other tree levels have been mapped.

Each transform block with a node weight 𝑤 greater than 1 codes 𝑤 − 1 AC coefficients with the same attribute coordinates.

for (nIdx = 0; nIdx < RahtBlkCnt[0]; nIdx++) {
  ns = RahtBlkLoc[0][nIdx][0]
  nt = RahtBlkLoc[0][nIdx][1]
  nv = RahtBlkLoc[0][nIdx][2]
  for (i = 1; i < RahtBlkWeight[0][ns][nt][nv]; i++)
    RahtCoeff[0][ns][nt][nv][i] = AttrCoeff[CoeffIdx++][Cidx]
}

    - Coefficient scaling

      - General

Subclause 10.5.4 specifies the scaling of coded coefficients for a block located at ( Bs, Bt, Bv ) in tree level Lvl.  It shall be applied to every block in every tree level in any order.

If a regional QP offset is present (i.e. attr_region_cnt > 0), a tree (10.5.4.4) is specified that blends QP offsets along region boundaries according to the structure of the RAHT tree.

      - For a transform block

Within a transform block, coded coefficients shall be scaled according to a per-coefficient QP.  The DC coefficient is not scaled except when coded in the root node of the transform tree.

mCnt := Lvl > 0 ? 8 : RahtBlkWeight[0][Bs][Bt][Bv]
for (m = 0; m < mCnt; m++) {
  /* skip the DC coefficient that will be inherited */
  if (m == 0 && Lvl < RahtRootLvl)
    continue
  RahtCoeff[Lvl][Bs][Bt][Bv][m] = RahtCoeffScaled[Lvl][Bs][Bt][Bv][m]
}

The scaling of the 𝑚-th coded coefficient of a transform block located at ( bs, bt, bv ) in tree level lvl is specified by the expression RahtCoeffScaled[ lvl ][ bs ][ bt ][ bv ][ 𝑚 ]: it is scaled by the fixed-point step size AttrQstep[ qp ] (10.7.4) and represented as a 15 fractional-bit, fixed-point coefficient value.

RahtCoeffScaled[lvl][bs][bt][bv][m] := coeff × AttrQstep[qp] << 7
  where
    coeff := RahtCoeff[lvl][bs][bt][bv][m]
    qp :=  lossless_coding_enabled == 0 ? RahtCoeffQp[lvl][bs][bt][bv][m] : 4

      - Per coefficient QP

The expression RahtCoeffQp[ lvl ][ bs ][ bt ][ bv ][ 𝑚 ] specifies the QP for the 𝑚-th coefficient of the transform block located at ( bs, bt, bv ) in tree level lvl for the Cidx-th attribute component:

- rgnOffset[ qc ] is the per-coefficient offset from the region-dependent QP offset tree.

- dpth is the depth of the transform block in the RAHT tree.

- ACqpoffset is the per-coefficient AC coefficient QP offset.

RahtCoeffQp[lvl][bs][bt][bv][m] := AttrQp[dpth][rgnOffset][Cidx > 0] + ACqpoffset
  where
    dpth := RahtRootLvl − lvl
    isACoffset = dpth <= attr_AC_qp_layer_cnt_minus1 && m > 0
    ACqpoffsetP = isACoffset ? attr_AC_qp_offset[ dpth ][0][m – 1] : 0
    ACqpoffsetS = isACoffset ? attr_AC_qp_offset[ dpth ][1][m – 1] : 0
    Acqpoffset := Cidx > 0 ? ACqpoffsetS : ACqpoffsetP
    rgnOffset[qc] := RahtTreeQpOffsetM[lvl][bs][bt][bv][m][qc]

      - Region-dependent QP offset tree

The integer, averaged region-dependent QP offset for each coefficient of a 2×2×2 transform block is specified by the expression RahtTreeQpOffsetM[ lvl ][ bs ][ bt ][ bv ][ 𝑚 ].  The parameter(s):

- bs, bt and bv specify a transform block location in tree level lvl;

- 𝑚 specifies the transform coefficient index from the final forward transform stage.

RahtTreeQpOffsetM[lvl][bs][bt][bv][m] :=
  lvl == 0 ? RahtTreeQpOffset[  0][3][bs][bt][bv][qc] >> 4
  : RahtTreeQpOffset[lvl][3][ms][mt][mv][qc] >> 4
  where
    ms := 2 × bs + FromMorton[m][0]
    mt := 2 × bt + FromMorton[m][1]
    mv := 2 × bv + FromMorton[m][2]

The fixed-point, region-dependent QP offset tree is structurally identical to the transform tree.  It is specified recursively for a QP component qc by RahtTreeQpOffset[ lvl ][ 𝑠 ][ 𝑡 ][ 𝑣 ][ qc ]:

- For tree level 0, the offset is the regional QP offset for a point with attribute coordinates ( 𝑠, 𝑡, 𝑣 ).

- For each subsequent tree level 𝑙, a 2×2×2 block of QP offsets is averaged for each transform stage in turn.  Each QP offset in a block at stage 0 is that of the DC transform block coefficient for a child block in the preceding level 𝑙 − 1.

Within a block, each subsequent transform stage averages, along the transformed axis, adjacent pairs of QP offsets from the preceding stage that have a non-zero transform coefficient weight.  For a pair of QPs 𝑎 and 𝑏 with respective weights wa and wb:

- The weight for the corresponding DC coefficient is 𝑎 + 𝑏 divided by 2 if wa and wb are non-zero, or 1 otherwise.

- The weight for the corresponding AC coefficient is 𝑏 if wa and wb are non-zero, or 0 otherwise.

Averages shall be calculated using four fractional bits.

RahtTreeQpOffset[lvl][stage][s][t][v][qc] :=
  lvl == 0 ? RahtRegionQpOffset[qs][qt][qv][qc] << 4 :
  stage == 0 ? RahtTreeQpOffset[lvl − 1][3][2 × s][2 × t][2 × v][qc] :
  stage == 1 ? v % 2 == 0 ? qpL[0][0][1] : qpH[0][0][−1] :
  stage == 2 ? t % 2 == 0 ? qpL[0][1][0] : qpH[0][−1][0] :
  stage == 3 ? s % 2 == 0 ? qpL[1][0][0] : qpH[−1][0][0] : na
  where
    qpL[ds][dt][dv] := qpSum[ds][dt][dv] >> qpHnz[ds][dt][dv]
    qpH[ds][dt][dv] := qpC[s][t][v] × qpHnz[ds][dt][dv]
    qpSum[ds][dt][dv] := qpC[s][t][v] + qpC[s + ds][t + dt][v + dv]
    qpHnz[ds][dt][dv] :=  wC[s][t][v] ×  wC[s + ds][t + dt][v + dv] > 0
    qpC[s][t][v] := RahtTreeQpOffset[lvl][stage − 1][s][t][v][qc]
    wC[s][t][v]  :=  RahtCoeffWeight[lvl][stage − 1][s][t][v]

An example tree is illustrated in Figure 29 for the transform tree of Figure 28.  The hatched area has a regional QP offset of +6; co-located points c, d and e have an offset of +6; points b and f, 0.  In block A, the stage 3 QPs are used to scale the transform coefficients.  For the coefficient at 𝑚 = 0, 3¾, the QP is the mean of the stage 2 QPs 1½ and 6; for the coefficient at 𝑚 = 4, it is 1½.  Scaling of coefficient uses the integer part of the fractional QP.

Key

| b to f | Points (See Figure 28) |
| --- | --- |
| A to C | Transform blocks (See Figure 28) |
| × | Transform coefficients at input to inverse transform for labelled block |
| ○ | Inverse transformed coefficient |
| ● | Inherited DC coefficient (See RahtDcCoeff) |
| 0 to 6 | QP values (See RahtTreeQpOffset) |

Figure 29 — Example region-dependent QP offset tree

    - Transform domain prediction

      - General

Subclauses 10.5.5.2 - 10.5.5.6 apply when raht_prediction_enabled_flag is 1 and lossless_coding_enabled is 1, subclauses 10.5.5.7 - 10.5.5.9 apply when slice_attr_inter_prediction is 1 and lossless_coding_enabled is 1.  These specify the transform domain prediction for a block located at ( Bs, Bt, Bv ) in tree level Lvl.  These shall be performed for every eligible block for intra prediction (10.5.5.2) or inter prediction (10.5.5.7)  in the tree level, in any order, by:

- generating a prediction block (10.5.5.3) and/or;

- generating a inter prediction block and resampling the inter prediction block (10.5.5.8);

- applying the forward transform for the intra prediction block (10.5.5.5) and/or;

- applying the forward transform for the inter prediction block (10.5.5.9);

- modifying the prediction block from the inter prediction block;

- adding the resulting AC transform coefficients to the coefficient residuals in the coefficient tree.

The prediction block and its transform are specified in terms of the eight-element array RahtPredBlk; RahtPredBlk[ 𝑚 ] is the prediction block value for the Morton-coded location 𝑚. The inter prediction block and its transform are specified in terms of the eight-element array RahtInterPredBlk; RahtInterPredBlk[ 𝑚 ] is the inter prediction block value for the Morton-coded location 𝑚.

if (RahtPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++)
    RahtPredBlk[m] = (RahtPredW[m] >> 15} << 15

  … /* in−place, forward transform of RahtPredBlk (10.5.5.5) */

} 
if (RahtInterPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++)
    RahtInterPredBlk[m] = RahtInterPredW[m]

  … /* in−place, forward transform of RahtInterPredBlk （10.5.5.9） */

  for (m = 0; m < 8; m++){
    if (m == 0 && Lvl < RahtRootLvl)
      continue
    RahtPredBlk[m] = RahtInterPredBlk[m]
  }
}
if (RahtPredEligible[Lvl][Bs][Bt][Bv] || RahtInterPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++){
    if (m == 0 && Lvl < RahtRootLvl)
      continue
    RahtCoeff[Lvl][Bs][Bt][Bv][m] += RahtPredBlk[m]
  }
}

      - Eligibility

When enabled, transform domain prediction shall be performed for 2×2×2 transform blocks unless the block:

- is the root of the transform tree; or

- has only one non-empty child block; or

- is adjoined (10.5.5.3) by fewer than raht_prediction_samples_min non-empty blocks; or

- has an ancestor, except the root block, that is adjoined by fewer than raht_prediction_subtree_min non-empty blocks.

The expression RahtBlkChildCnt[ lvl ][ bs ][ bt ][ bv ] is the number of non-empty child blocks of the block located at ( bs, bt, bv ) in tree level lvl .

RahtBlkChildCnt[lvl][bs][bt][bv] :=
  RahtBlkChildCnt = 0
  for(m = 0; m ≤ 8; m++){
    if(RahtCoeffWeightM[lvl][0][bs][bt][bv][m] > 0)
      RahtBlkChildCnt ++ 
  }

The expression RahtPredEligible[ lvl ][ bs ][ bt ][ bv ] specifies whether the transform block located at ( bs, bt, bv ) in tree level lvl is eligible.

RahtPredEligible[lvl][bs][bt][bv] := raht_prediction_enabled_flag
  && lvl > 0
  && lvl < RahtRootLvl
  && !DisabledBySendingMode
  && RahtBlkChildCnt[lvl][bs][bt][bv] > 1
  && RahtNeighCnt[lvl][bs][bt][bv] ≥ raht_prediction_samples_min
  && RahtNeighCntMinAncestor[lvl][bs][bt][bv] ≥ raht_prediction_subtree_min
  where
    DisabledBySendingMode := (raht_intra_layer_code_enabled && depth < layer_code_depth &&
      depth ≥ 0) ? !slice_raht_intra_layer_code_mode[depth] : 0
    depth := RahtRootLevel – lvl – 1

      - Generation of prediction block from adjoining blocks

A prediction block is generated from up to 19 transform blocks that contain a DC coefficient: the co-located block and those that adjoin the predicted block by a face or an edge.

The expression RahtNeighCnt[ lvl ][ bs ][ bt ][ bv ] is the number of non-empty blocks that can be used to predict the block located at ( bs, bt, bv ), which is defined by (RahtBlkLoc[ lvl ][ nIdx ][ 0 ], RahtBlkLoc[ lvl ][ nIdx ][ 1 ], RahtBlkLoc[ lvl ][ nIdx ][ 2]), is the location of the nIdx-th coded block in tree level lvl.  When the block located at ( bs, bt, bv ) in tree level lvl has only one non-empty child block, the number of non-empty blocks that can be used to predict that block is set as 19.

RahtNeighCnt[lvl][bs][bt][bv] := 
  RahtBlkChildCnt[lvl][bs][bt][bv] == 1 ? 19 : SumN19[neighWeightGt0]
  where
    neighWeightGt0[ds][dt][dv] := RahtBlkWeight[lvl][bs + ds][bt + dt][bv + dv] > 0 &&
      ¬isNeighOutsideSearchRange
    isNeighOutsideSearchRange :=
      Morton(bs + ds, bt + dt, bv + dv) < Morton(bs, bt, bv) ?
      Morton(bs + ds, bt + dt, bv + dv) < Morton(LE(0), LE(1), LE(2)) :
      Morton(bs + ds, bt + dt, bv + dv) > Morton(BE(0), BE(1), BE(2))
    sr := raht_prediction_search_range
    LE(k) := RahtBlkLoc[lvl][Max(nIdx - sr, 0)][k]
    BE(k) := RahtBlkLoc[lvl][Min(nIdx + sr, RahtBlkCnt[lvl] - 1)][k]

The expression RahtNeighCntMinAncestor[ lvl ][ bs ][ bt ][ bv ] is lowest value of RahtNeighCnt for any ancestor of the block located at ( bs, bt, bv ) in tree level lvl.  In determining eligibility, the root node shall be considered to have 19 adjoining blocks.

RahtNeighCntMinAncestor[lvl][bs][bt][bv] :=
    lvl >= RahtRootLvl − 1 ? 19 : Min(neighCntP, minAncestorCnt)
  where
    neighCntP := RahtNeighCnt[lvl + 1][bs / 2][bt / 2][bv / 2]
    minAncestorCnt := RahtNeighCntMinAncestor[lvl + 1][bs / 2][bt / 2][bv / 2]

The expression SumN19[ expr ] sums the result of applying expr to the relative tree location of each of the 19 possible adjacent blocks.

SumN19[expr] :=
  SumN19 = 0
  for (ds = −1; ds ≤ 1; ds++)
    for (dt = −1; dt ≤ 1; dt++)
      for (dv = −1; dv ≤ 1; dv++)
        if (Abs(ds) + Abs(dt) + Abs(dv) < 3)
          SumN19 += expr[ds][dt][dv]

      - Upsampling

        - Normalized DC values

The samples used to generate an upsampled prediction block are transform-block DC coefficients (10.5.7.2) normalized by their weight as specified by RahtDcNorm, that is a 15 fractional-bit, fixed-point value; RahtDcNorm[ lvl ][ bs ][ bt ][ bv ] is the sample value for the block located at ( bs, bt, bv ) in tree level lvl.

RahtDcNorm[lvl][bs][bt][bv] := raht_buffer_extension_flag == 0 ?
  (lossless_coding_enabled == 0 ? DivExp2Fz((coeff >> wShift) × (IntRecipSqrt(w) >> 25 −
  wShift), 30) : DivExp2Fz(coeff, 15)) :
  (lossless_coding_enabled == 0 ? DivExp2Fz((coeff >> wShift) × (IntRecipSqrt(w) >> 25 −
  wShift), 15) : DivExp2Fz(coeff, 0))
  where
    w := RahtBlkWeight[lvl][bs][bt][bv]
    coeff := RahtDcCoeff[lvl][bs][bt][bv]
    wShift := w > 1024 ? IntLog2(w − 1) >> 1 : 0

        - Exclusion of adjoining blocks

Adjoining blocks shall be excluded from the upsampling process if either their weight is zero or the normalized DC value for their primary attribute component is:

- less than or equal to 0,2 times that of co-located block; or

- greater than or equal to 2,5 times that of the co-located block.

The expression RahtPredExcluded[ ds ][ dt ][ dv ] specifies whether the block with relative location ( ds, dt, dv ) is excluded from contributing to the upsampled prediction of the block ( Bs, Bt, Bv ).

RahtPredExcluded[ds][dt][dv] :=
  Cidx == 0 ? empty || sample ≤ limitMin || sample ≥ limitMax
  : … /* Value of RahtPredExcluded[ds][dt][dv] for Cidx == 0 */
  where
    empty := RahtBlkWeight[Lvl][Bs][Bt][Bv] == 0
    sample := 10 × RahtSample[Lvl][Bs + ds][Bt + dt][Bv + dv]
    limitMin :=  2 × RahtSample[Lvl][Bs][Bt][Bv]
    limitMax := 25 × RahtSample[Lvl][Bs][Bt][Bv]

        - Eligibility of replacing adjoining blocks with child blocks

Adjoining block shall be replaced by its one child block to calculate the sample value of the prediction block for the Morton-coded sample location m in the block ( Bs, Bt, Bv ) when the eligibility conditions are meet.  The child block to replace the adjoining block is the child block with relative location ( dsc, dtc, dvc ) to the 𝑚-th child block in tree level lvl - 1.  The replacement shall be performed when the following eligibility conditions are true:

- raht_subnode_prediction_enabled is 1;

- The Morton code of the Adjoining block is less than the Morton code of the block;

- The Adjoining block adjoin the block by a face and it has a non-empty child block that adjoin the 𝑚-th child block of the block by a face; or the Adjoining block adjoin the block by an edge and it has a non-empty child block that adjoin the 𝑚-th child block of the block by an edge.

The expression IsReplaced[ ds ][ dt ][dv][ 𝑚 ] specifies whether the adjoining block with relative location ( ds, dt, dv ) is replaced by its child block to calculate the sample of the prediction block for the 𝑚-th child block of the block ( Bs, Bt, Bv ).

IsReplaced[ds][dt][dv][m] :=
  isAdj && isEarly && (RahtBlkWeight[lvl-1][bsc + dsc][btc + dtc][bvc + dvc] > 0)
  where
    adjMask := Morton(ds ≠ 0, dt ≠ 0, dv ≠ 0)
    adjLoc  := Morton(ds > 0, dt > 0, dv > 0)
    isAdj := (m & adjMask) == adjLoc
    isEarly := Morton(Bs + ds, Bt + dt, Bv + dv) < Morton(Bs, Bt, Bv)
    bsc := 2 × Bs + FromMorton[m][0]
    btc := 2 × Bt + FromMorton[m][1]
    bvc := 2 × Bv + FromMorton[m][2]
    dsc := ds
    dtc := dt
    dvc := dv

        - Upsampled prediction block

The samples of the 2x2x2 prediction block are the weighted averages of the adjoining blocks' normalized DC values when raht_subnode_prediction_enabled is 0.  For each sample, the weight for the adjoining block depends upon the relative positions of the block and the sample location.

The expression RahtPred[ 𝑚 ], that is a 15 fractional-bit, fixed-point value, specifies the value for the Morton-coded sample location m; where:

- SumN19[ 𝑤 ] is sum of the weight of each adjoining block;

- SumN19[ wNeigh ] is the 15 fractional-bit, fixed-point the sum of the weighted normalized DC values for each adjoining block;

- the 15 fractional-bit, fixed-point reciprocal of the sum of weights, RahtPredRecipW[ 𝑥 ] is specified by Table 50.

- Only samples that correspond to child blocks with non-zero block weights need to be calculated.

RahtPred[m] := (raht_buffer_extension_flag == 0 ? SumN19[wNeigh] << 15 : SumN19[wNeigh])
  × RahtPredRecipW[SumN19[w]]
  where
    w[ds][dt][dv] := RahtPredExcluded[ds][dt][dv] && Abs(Morton(bs + ds, bt + dt, bv +
      dv)- Morton(bs, bt, bv)) <= raht_prediction_search_range ? 0 :
      RahtPredWeight[ds][dt][dv][m]
    wNeigh[ds][dt][dv] := w[ds][dt][dv] × RahtDcNorm[Lvl][Bs + ds][Bs + dt][Bs + dv]

The expression RahtPredWeight[ ds ][ dt ][ dv ][ 𝑚 ] is the weight to be applied to the normalized DC value of the block with relative tree location ( ds, dt, dv ) for the Morton-coded prediction block sample location 𝑚.  The weight shall be 4 for the co-located block, 2 for blocks that adjoin by a face and 1 for blocks that adjoin by only an edge.

RahtPredWeight[ds][dt][dv][m] := (m & adjMask) == adjLoc ? weight : 0
  where
    weight := 4 >> (ds ≠ 0) + (dt ≠ 0) + (dv ≠ 0)
    adjMask := Morton(ds ≠ 0, dt ≠ 0, dv ≠ 0)
    adjLoc := Morton(ds > 0, dt > 0, dv > 0)

Table 50 — Values of RahtPredRecipW[ 𝑥 ]

| 𝑥 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RahtPredRecipW[ 𝑥 ] | 8 192 | 6 554 | 5 461 | 4 681 | 4 096 | 3 641 | 3 277 | 2 979 | 2 731 | 2 521 |

        - Replacement of adjoining blocks with child blocks

Replacement of adjoining blocks with their child blocks shall be performed when raht_subnode_prediction_enabled is 1.  The child blocks’ normalized DC values and weights are to replace the adjoining blocks’ normalized DC values and weights when the eligibility conditions are meet.

RahtPred[m] := (raht_buffer_extension_flag == 0 ? SumN19[wNeigh] << 15 : SumN19[wNeigh])
  × RahtPredRecipWC[SumN19[w]-1]
  where
    s := 2 × Bs + FromMorton[m][0]
    t := 2 × Bt + FromMorton[m][1]
    v := 2 × Bv + FromMorton[m][2]
    RahtPredRealWeight[ds][dt][dv][m] := 
      IsReplaced [ds][dt][dv][m] ? RahtPredWeightC[ds][dt][dv][m]
      : RahtPredWeightB[ds][dt][dv][m]
    w[ds][dt][dv] := RahtPredExcluded[ds][dt][dv] ? 0 : RahtPredRealWeight[ds][dt][dv][m]
    RahtDcN := IsReplaced [ds][dt][dv][m] ? RahtDcNorm[lvl-1][s + ds][t + dt][v + dv] 
      : RahtDcNorm[Lvl][Bs + ds][Bt + dt][Bv + dv]
    wNeigh[ds][dt][dv] := w[ds][dt][dv] × RahtDcN

The expression RahtPredWeightB[ ds ][ dt ][ dv ][ 𝑚 ] is the weight to be applied to the normalized DC value of the not replaced block with relative tree location ( ds, dt, dv ) for the Morton-coded prediction block sample location 𝑚.  The weight shall be specified by raht_prediction_weights[0] for the co-located block, raht_prediction_weights[1] for blocks that adjoin by a face and raht_prediction_weights[2] for blocks that adjoin by only an edge.

RahtPredWeightB[ds][dt][dv][m] := (m & adjMask) == adjLoc ? weight : 0
  where
    idx := (ds ≠ 0) + (dt ≠ 0) + (dv ≠ 0)
    weight := raht_prediction_weights[idx]
    adjMask := Morton(ds ≠ 0, dt ≠ 0, dv ≠ 0)
    adjLoc := Morton(ds > 0, dt > 0, dv > 0)

The expression RahtPredWeightC[ ds ][ dt ][ dv ][ 𝑚 ] is the weight to be applied to the normalized DC value of the child block of the replaced block with relative tree location ( ds, dt, dv ) for the Morton-coded prediction block sample location 𝑚.  The weight shall be specified by raht_prediction_weights[3] for child blocks that adjoin by a face and raht_prediction_weights[4] for child blocks that adjoin by only an edge.

RahtPredWeightC[ds][dt][dv][m] := IsReplaced[ds][dt][dv][m] ? weight : 0
  where
    idx := (ds ≠ 0) + (dt ≠ 0) + (dv ≠ 0)
    weight := raht_prediction_weights[2+idx]

The 15 fractional-bit, fixed-point reciprocal of the sum of weights, RahtPredRecipWC[ 𝑥 ] is specified by Table 51.

Table 51 — Values of RahtPredRecipWC[ 𝑥 ]

| 𝑥 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RahtPredRecipWC[ 𝑥 ] | 32768 | 16384 | 10923 | 8192 | 6554 | 5461 | 4681 | 4096 | 3641 | 3277 |
| 𝑥 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| RahtPredRecipWC[ 𝑥 ] | 2979 | 2731 | 2521 | 2341 | 2185 | 2048 | 1928 | 1820 | 1725 | 1638 |
| 𝑥 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 |
| RahtPredRecipWC[ 𝑥 ] | 1560 | 1489 | 1425 | 1365 | 1311 | 1260 | 1214 | 1170 | 1130 | 1092 |
| 𝑥 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| RahtPredRecipWC[ 𝑥 ] | 1057 | 1024 | 993 | 964 | 936 | 910 | 886 | 862 | 840 | 819 |
| 𝑥 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| RahtPredRecipWC[ 𝑥 ] | 799 | 780 | 762 | 745 | 728 | 712 | 697 | 683 | 669 | 655 |
| 𝑥 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| RahtPredRecipWC[ 𝑥 ] | 643 | 630 | 618 | 607 | 596 | 585 | 575 | 565 | 555 | 546 |
| 𝑥 | 60 | 61 | 62 | 63 |  |  |  |  |  |  |
| RahtPredRecipWC[ 𝑥 ] | 537 | 529 | 520 | 512 |  |  |  |  |  |  |

      - Forward transform for a 2×2×2 block prediction block

The forward transform for a 2×2×2 prediction block comprises transforming pairs of coefficients along each axis.

First, along the V axis:

rahtFwd1D[2][0][1]
rahtFwd1D[2][2][3]
rahtFwd1D[2][4][5]
rahtFwd1D[2][6][7]

Second, along the T axis:

rahtFwd1D[1][0][2]
rahtFwd1D[1][1][3]
rahtFwd1D[1][4][6]
rahtFwd1D[1][5][7]

Third, along the S axis:

rahtFwd1D[0][0][4]
rahtFwd1D[0][1][5]
rahtFwd1D[0][2][6]
rahtFwd1D[0][3][7]

The expression rahtFwd1D[ 𝑘 ][ aIdx ][ bIdx ] specifies the invocation of the in-place, forward, two-point transform for the aIdx-th and bIdx-th coefficients along the 𝑘-th axis.

rahtFwd1D[k][aIdx][bIdx] := RahtFwd(aCoeff, bCoeff, wa, wb)
  where
    aCoeff := RahtPredBlk[aIdx]
    bCoeff := RahtPredBlk[bIdx]
    wa := RahtCoeffWeightM[Lvl][stage][Bs][Bt][Bv][aIdx]
    wb := RahtCoeffWeightM[Lvl][stage][Bs][Bt][Bv][bIdx]
    stage := 2 − k

      - Forward two-point transform

This subclause specifies the in-place, forward, two-point transform RahtFwd( aCoeff, bCoeff, wa, wb ).  Its parameters are:

- the expressions aCoeff and bCoeff that identify the coefficients to be transformed in-place;

- the weights wa and wb that are the coefficient weights for aCoeff and bCoeff, respectively.

- The specification of the forward two-point transform applies only to prediction blocks for the reconstruction of point attributes.

The transform basis vectors 15-fractional-bit, fixed-point coefficients 𝑎 and 𝑏:

a := IntSqrt(wa << 30) × IntRecipSqrt(wa + wb) >> 40
b := IntSqrt(wb << 30) × IntRecipSqrt(wa + wb) >> 40

If both wa or wb are 0, the transform result is:

if (wa == 0 && wb == 0)
  yl = yh = 0

If either wa or wb is 0, the transform result is:

if (wa == 0 || wb == 0) {
  yl = wa ≠ 0 ? aCoeff : bCoeff
  yh = 0
}

- If either wa or wb is zero, the respective opposing coefficient 𝑏 or 𝑎 is not necessarily 215.

Otherwise (both wa and wb are greater than 0), the transform result is:

if (wa ≠ 0 && wb ≠ 0) {
  yh = bCoeff − aCoeff
  yl = aCoeff + ((yh >> 16) << 15)
}

The transform result replaces the input coefficients:

aCoeff = yl
bCoeff = yh

      - Eligibility for inter prediction

When inter prediction is enabled, inter prediction shall be performed for 2×2×2 transform blocks unless:

- inter prediction is disabled for current level; or

- the block has no reference block which share the same location in the reference transform tree.

The expression RahtInterPredEligible[ lvl ][ bs ][ bt ][ bv ] specifies whether the transform block located at ( bs, bt, bv ) in tree level lvl is eligible for inter prediction.

RahtInterPredEligible[lvl][bs][bt][bv] := slice_attr_inter_prediction
  && lvl > 0
  && lvl > (RahtRootLvl – raht_inter_layer_depth_minus1 - 1)
  && !DisabledBySendingMode
  && RahtBlkWeightRef[lvl][bs][bt][bv] > 0
  where
    DisabledBySendingMode := (raht_inter_layer_code_enabled && depth < layer_code_depth &&
      depth ≥ 0) ? !slice_raht_inter_layer_code_mode[depth] : 0
    depth := RahtRootLevel – lvl – 1

      - Generation of inter prediction block

The samples used to generate an inter prediction block are the sum of point attributes within each child block of the reference block as specified by RahtInterPred; RahtInterPred[ lvl  - 1][ bsc ][ btc ][ bvc ] is the sample value for the child block located at ( bsc, btc, bvc ) in tree level lvl  - 1.

RahtInterPred[lvl][bs][bt][bv] := RahtBlkSumAttrRef[ lvl ][ bs ][ bt ][ bv ]

Each sample in the prediction block is specified by RahtInterPredW[ m ] for the m-th child block located at ( bsc, btc, bvc ) in tree level lvl  - 1.

- If one child block is with zero weight, the sample used to generate the inter prediction block shall be set to 0;

- Else, if the corresponding child block of the reference block is with zero weight, the sample used to generate the inter prediction block shall be the sum of point attributes within the reference block;

- Else, the corresponding child block of the reference block is with non-zero weight, the sample used to generate the inter prediction block shall be the sum of point attributes within the corresponding child block of the reference block.

RahtInterPredW[ m ] := RahtBlkWeight[lvl - 1][bsc][btc][bvc] ?
  (RahtBlkWeightRef[lvl - 1][bsc][btc][bvc] ? RahtInterPred[lvl – 1][bsc][btc][bvc] :
  RahtInterPred[lvl][bs][bt][bv]) : 0

      - Forward transform for a 2×2×2 block inter prediction block

The forward transform for a reference 2×2×2 prediction block comprises transforming pairs of coefficients along each axis.

First, along the V axis:

rahtFwd1D[2][0][1]
rahtFwd1D[2][2][3]
rahtFwd1D[2][4][5]
rahtFwd1D[2][6][7]

Second, along the T axis:

rahtFwd1D[1][0][2]
rahtFwd1D[1][1][3]
rahtFwd1D[1][4][6]
rahtFwd1D[1][5][7]

Third, along the S axis:

rahtFwd1D[0][0][4]
rahtFwd1D[0][1][5]
rahtFwd1D[0][2][6]
rahtFwd1D[0][3][7]

The expression rahtFwd1Dref[ 𝑘 ][ aIdx ][ bIdx ] specifies the invocation of the in-place, forward, two-point transform for the aIdx-th and bIdx-th coefficients along the 𝑘-th axis.

rahtFwd1Dref[k][aIdx][bIdx] := RahtFwd(aCoeffRef, bCoeffRef, waRef, wbRef)
  where
    aCoeffRef := RahtInterPredBlk[aIdx]
    bCoeffRef := RahtInterPredBlk[bIdx]
    waRef := RahtCoeffWeightMRef[Lvl][stage][Bs][Bt][Bv][aIdx]
    wbRef := RahtCoeffWeightMRef[Lvl][stage][Bs][Bt][Bv][bIdx]
    stage := 2 − k

    - Sample domain prediction and last component prediction

      - General

Subclauses 10.5.5.2 - 10.5.5.4, subclause 10.5.6.2 and  apply when raht_prediction_enabled_flag is 1 and lossless_coding_enabled is 0, subclause 10.5.5.7 and subclauses 10.5.6.3 - 10.5.6.4  apply when slice_attr_inter_prediction is 1 and lossless_coding_enabled is 0, subclause 10.5.6.5 applies when last_comp_pred_enabled is 1 and lossless_coding_enabled is 0.  These specify the sample domain prediction and last component prediction in transform domain for a block located at ( Bs, Bt, Bv ) in tree level Lvl. These shall be performed for every eligible block for intra prediction (10.5.5.2) or inter prediction (10.5.5.7)  in the tree level, in any order, by:

- generating a prediction block (10.5.5.3) and/or;

- generating a inter prediction block (10.5.6.3);

- weighting the values of the prediction block (10.5.6.2);

- resampling and weighting the inter prediction block (10.5.6.3);

- if the DC values of the current block and the reference block are similar, applying temporal filtering (10.5.6.4) for the inter prediction block and modifying the prediction block from the inter prediction block;

- applying the last component prediction in transform domain(10.5.6.5);

The prediction block is specified in terms of the eight-element array RahtPredBlk; RahtPredBlk[ 𝑚 ] is the prediction block value for the Morton-coded location 𝑚.  The inter prediction block is specified in terms of the eight-element array RahtInterPredBlk; RahtInterPredBlk[ 𝑚 ] is the inter prediction block value for the Morton-coded location 𝑚.

if (RahtPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++)
    RahtPredBlk[m] = RahtPredW[m]
} 
if (RahtInterPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++)
    RahtInterPredBlk[m] = RahtInterPredW[m]
  if (RahtDcCoeffRef[lvl][Bs][Bt][Bv] < RahtDcCoeff[lvl][Bs][Bt][Bv] × 0.5 || RahtDcCoeffRef[lvl][Bs][Bt][Bv] > RahtDcCoeff[lvl][Bs][Bt][Bv] × 2)
    continue (not inter-eligible)

  … /* in-place, temporal filtering of RahtInterPredBlk （10.5.6.4） */

  for (m = 0; m < 8; m++){
    if (m == 0 && Lvl < RahtRootLvl)
      continue
    RahtPredBlk[m] = RahtInterPredBlk[m]
  }
}

/* in-place, last component prediction for RahtCoeff（10.5.6.5） */

      - Weighted prediction block

The upsampled prediction block shall be weighted with each sample weighted by the square root of the corresponding transform coefficient weight.  The weighted sample value is specified by RahtPredW[ 𝑚 ] for the block located at ( Bs, Bt, Bv ) in tree level Lvl.

RahtPredW[m] := lossless_coding_enabled == 0 ? DivExp2Fz(RahtPred[m] × w, 15) :
  (RahtPred[m] >> 15) << 15
  where
    w := IntSqrt(RahtCoeffWeightM[Lvl][0][Bs][Bt][Bv][m] << 30)

      - Resampling and Weighted inter prediction block

The samples used to generate an inter prediction block are the sum of point attributes within each child block of the reference block as specified by RahtInterPred; RahtInterPred[ lvl  - 1][ bsc ][ btc ][ bvc ] is the sample value for the child block located at ( bsc, btc, bvc ) in tree level lvl  - 1.

RahtInterPred[lvl][bs][bt][bv] := RahtBlkSumAttrRef[ lvl ][ bs ][ bt ][ bv ]

The weighted sample value is specified by RahtInterPredW[ m ] for the m-th child block located at ( bsc, btc, bvc ) in tree level lvl  - 1.

- If one child block is with zero weight, the sample used to generate the inter prediction block shall be set to 0;

- Else, if the corresponding child block of the reference block is with zero weight, the sample used to generate the inter prediction block shall be the sum of point attributes within the reference block and be weighted by the transform coefficient weight of the reference block;

- Else, the corresponding child block of the reference block is with non-zero weight, the sample used to generate the inter prediction block shall be the sum of point attributes within the corresponding child block of the reference block and be weighted by its corresponding transform coefficient weight.

RahtInterPredW[ m ] := RahtBlkWeight[lvl - 1][bsc][btc][bvc] ?
  (RahtBlkWeightRef[lvl - 1][bsc][btc][bvc] ?
  DivExp2Fz((RahtInterPred[lvl – 1][bsc][btc][bvc] >> wShift ) × rsqrtW, 15) :
  DivExp2Fz((RahtInterPred[lvl][bs][bt][bv] >> wShiftP ) × rsqrtWP, 15)) : 0
  where
    w := RahtBlkWeightRef[lvl - 1][bsc][btc][bvc]
    wShift := w > 1024 ? IntLog2(w - 1) >> 1 : 0
    rsqrtW := IntRecipSqrt(w) >> (25 – wShift)
    w := RahtBlkWeightRef[lvl][bs][bt][bv]
    wShiftP := wP > 1024 ? IntLog2(wP - 1) >> 1 : 0
    rsqrtWP := IntRecipSqrt(wP) >> (25 – wShiftP)

      - RAHT temporal filtering

When the tree level lvl  of the current block is smaller than or equal to RahtRootLvl – raht_inter_skip_layers, the temporal filtering is enabled for the transformed inter prediction block.  The transformed inter-predicted block is filtered by the derived filter as specified by RahtInterFilter.

RahtInterPredBlk[m] = lvl <= (RahtRootLvl – raht_skip_layers) ? (RahtInterFilter ×
  RahtInterPredBlk[m]) >> 7 : RahtInterPredBlk[m]

RahtInterFilter is determined for each tree level.  If raht_send_filters is equal to 0, RahtInterFilter is determined by a set of stored filters as specified by FixedRahtFilters, based on the tree level lvl.  If raht_send_filters is equal to 1, raht_inter_filter_qidx[dpth - raht_skip_layers] is inverse quantized and subtracted from 128 to derive RahtInterFilter.

if(raht_send_filters)
  RahtInterFilter = 128 – (raht_inter_filter_qidx[dpth - raht_skip_layers] ×
  AttrQstep[qp]>> 7)
else
  RahtInterFilter = FixedRahtFilters[LvlIdx]
  where
    FixedRahtFileters := [128, 128, 128, 127, 125, 121, 115]
    LvlIdx := dpth > 6 ? 6 : dpth
    dpth := RahtRootLevel – lvl

      - Last component prediction for RAHT

When attr_coding_type is 0 and raht_last_comp_pred_enabled is 1, the third attribute coefficient component is, if present, a residual to a prediction by the second scaled coefficient component.  The expression LcpScale[lvl][nIdx] specifies the scale factor applied at the nIdx -th coded block in transform level lvl to the second coefficient components to predict third coefficient components.  The scale factor LcpScale[lvl][nIdx] is calculated using the second and third components of the coefficients of its preceding 128 blocks.

if (AttrDim == 3) {
  bs = RahtBlkLoc[Lvl][nIdx][0]
  bt = RahtBlkLoc[Lvl][nIdx][1]
  bv = RahtBlkLoc[Lvl][nIdx][2]
  LcpScale[lvl][nIdx] = 0;
  for(n = 0; n < Min(nIdx,128); n++){
    ns = RahtBlkLoc[Lvl][nIdx - n][0]
    nt = RahtBlkLoc[Lvl][nIdx - n][1]
    nv = RahtBlkLoc[Lvl][nIdx - n][2]
    for(m = 0; m < 8; m++){
      if(m == 0 && Lvl < RahtRootLvl)
        continue；
      sumk1k1 += RahtCoeffLcp[lvl][ns][nt][nv][m][1] ×
        RahtCoeffLcp[lvl][ns][nt][nv][m][1];
      sumk1k2 += RahtCoeffLcp[lvl][ns][nt][nv][m][1] ×
        RahtCoeffLcp[lvl][ns][nt][nv][m][2];
    }
  }
  if (sumk1k2 && sumk1k1) {
    LcpScale[lvl][nIdx] = Clip3(-16, 16, Div(sumk1k2, sumk1k2,4))
  }
  RahtCoeffLcp[lvl][bs][bt][bv][m][2] += DivExp2Floor(LcpScale[lvl][nIdx] ×
    RahtCoeffLcp[lvl][bs][bt][bv][m][1],4)
}

when AttrDim is 3, RahtCoeffLcp[ lvl ][ bs ][ bt ][ bv ][ m][ Cidx] specifies the coefficient residuals for the Cidx component of RahtCoeff[ lvl ][ bs ][ bt ][ bv ][ m], where Cidx ∈ 0 .. AttrDim – 1.

    - Inverse transform

      - General

Subclause 10.5.7 specifies the inverse transform for a block located at ( Bs, Bt, Bv ) in tree level Lvl.  It shall be applied to every block in the tree level in any order.

When sample domain prediction is applied to the transform block, the prediction is added to the inverse transformed block.

if (RahtPredEligible[Lvl][Bs][Bt][Bv] || RahtInterPredEligible[Lvl][Bs][Bt][Bv]) {
  for (m = 0; m < 8; m++){
    if (m == 0 && Lvl < RahtRootLvl)
      continue
    RahtCoeff[Lvl][Bs][Bt][Bv][m] += RahtPredBlk[m]
  }
}

      - DC transform coefficient inheritance

Each block other than the root node of the transform tree shall inherit its DC coefficient from the corresponding inverse-transformed coefficient in its parent block.

When sample domain prediction is not applied to the transform block, the DC coefficient is obtained as follows:

If raht_buffer_extension_flag is equal to 0, the inherited coefficient shall be rounded to retain two fractional bits, with half values rounded away from zero.

if (Lvl < RahtRootLvl)
  RahtCoeff[Lvl][Bs][Bt][Bv][0] = raht_buffer_extension_flag == 0 ?
    DivExp2Fz(RahtDcCoeff[Lvl][Bs][Bt][Bv], 13) << 13 : RahtDcCoeff[Lvl][Bs][Bt][Bv]

For a block located at ( bs, bt, bv ) in tree level lvl, the corresponding coefficient in the parent block is specified by RahtDcCoeff[ lvl ][ bs ][ bt ][ bv ].

RahtDcCoeff[lvl][bs][bt][bv] := lvl < RahtRootLvl ?
  RahtCoeff[lvl + 1][bs / 2][bt / 2][bv / 2][mP] : 0
  where
    mP := Morton[bs & 1][bt & 1][bv & 1]

When sample domain prediction is applied to the transform block, the DC coefficient is inferred as the difference between the inherited coefficient and the DC coefficient of the prediction block.

if (Lvl < RahtRootLvl)
  RahtCoeff[Lvl][Bs][Bt][Bv][0] = RahtDcCoeff[Lvl][Bs][Bt][Bv] –
    RahtPredDcCoeff[Lvl][Bs][Bt][Bv]

For a block located at ( bs, bt, bv ) in tree level lvl, the corresponding DC coefficient of the prediction block is specified by RahtPredDcCoeff[ lvl ][ bs ][ bt ][ bv ].

RahtPredDcCoeff[ lvl ][ bs ][ bt ][ bv ] = 0
rsqrtWeightSum = IntRecipSqrt(RahtBlkWeight[Lvl][bs][bt][bv])
for(m = 0; m < 8; m++){
  bcs = bs << 1 + m & 1
  bct = bt << 1 + m & 2
  bcv = bv << 1 + m & 4
  sqrtWeight = IntSqrt(RahtBlkWeight[Lvl - 1][bcs][bct][bcv] << 30)
  normSqrtWeight = RahtBlkWeight[Lvl - 1][bcs][bct][bcv] == 1 ? rsqrtWeightSum >> 25 :
    rsqrtWeightSum × sqrtWeight >> 40
  predDC = normSqrtWeight * RahtInterPredBlk[m] >> 15
  RahtPredDcCoeff[ lvl ][ bs ][ bt ][ bv ] += RahtBlkWeight[Lvl - 1][bcs][bct][bcv] ? predDC : 0
}

      - For a 2×2×2 transform block

The inverse transform for a 2×2×2 block located at ( Bs, Bt, Bv ) in tree level Lvl comprises transforming pairs of coefficients along each axis.

First, along the S axis:

rahtInv1D[0][0][4]
rahtInv1D[0][1][5]
rahtInv1D[0][2][6]
rahtInv1D[0][3][7]

Second, along the T axis:

rahtInv1D[1][0][2]
rahtInv1D[1][1][3]
rahtInv1D[1][4][6]
rahtInv1D[1][5][7]

Third, along the V axis:

rahtInv1D[2][0][1]
rahtInv1D[2][2][3]
rahtInv1D[2][4][5]
rahtInv1D[2][6][7]

The expression rahtInv1D[ 𝑘 ][ aIdx ][ bIdx ] specifies the invocation of the in-place inverse transform for the aIdx-th and bIdx-th coefficients along the 𝑘-th axis of the block.

rahtInv1D[k][aIdx][bIdx] := RahtInv(aCoeff, bCoeff, wa, wb)
  where
    aCoeff := RahtCoeff[Lvl][Bs][Bt][Bv][aIdx]
    bCoeff := RahtCoeff[Lvl][Bs][Bt][Bv][bIdx]
    wa := RahtCoeffWeightM[Lvl][stage][Bs][Bt][Bv][aIdx]
    wb := RahtCoeffWeightM[Lvl][stage][Bs][Bt][Bv][bIdx]
    stage := 2 − k

      - For co-located points

The inverse transform for a block of co-located points located at ( Bs, Bt, Bv ) in tree level 0 comprises iteratively transforming the block's DC coefficient paired with each successive block coefficient.

for (i = 1; i < RahtBlkWeight[0][Bs][Bt][Bv]; i++)
  rahtInvDup[i]

The expression rahtInvDup[ 𝑖 ] specifies the invocation of the in-place inverse transform for the 𝑖-th coefficient.

rahtInvDup[i] := RahtInv(aCoeff, bCoeff, wa, 1)
  where
    aCoeff := RahtCoeff[0][Bs][Bt][Bv][0]
    bCoeff := RahtCoeff[0][Bs][Bt][Bv][i]
    wa := RahtBlkWeight[0][Bs][Bt][Bv] − i

      - Inverse two-point transform

This subclause specifies the in-place, inverse, two-point transform RahtInv( aCoeff, bCoeff, wa, wb ).  Its parameters are:

- the expressions aCoeff and bCoeff that respectively identify low- and high-frequency transform coefficients;

- the weights wa and wb that are the respective coefficient weights for aCoeff and bCoeff.

The transform basis vectors use 15-fractional-bit, fixed-point coefficients 𝑎 and 𝑏:

a := IntSqrt(wa << 30) × IntRecipSqrt(wa + wb) >> 40
b := IntSqrt(wb << 30) × IntRecipSqrt(wa + wb) >> 40

If either wa or wb is 0, the transform result is:

if (wa == 0 || wb == 0) {
  if (lossless_coding_enabled == 0) {
    ya = wa ≠ 0 ? aCoeff : 0
    yb = wb ≠ 0 ? aCoeff : 0
  }
  else {
    ya = aCoeff – ((bCoeff >> 16) << 15)
    yb = bCoeff + ya
  }
}

- If either wa or wb is zero, the respective opposing coefficient 𝑏 or 𝑎 is not necessarily 215.

Otherwise (both wa and wb are greater than 0), the transform result is:

if (wa ≠ 0 && wb ≠ 0) {
  ya = DivExp2Fz(aCoeff × a, 15) − DivExp2Fz(bCoeff × b, 15)
  yb = DivExp2Fz(bCoeff × a, 15) + DivExp2Fz(aCoeff × b, 15)
}

    - Reconstructed attribute values

Reconstructed attribute values are specified by the expression RahtRecon[ 𝑠 ][ 𝑡 ][ 𝑣 ][ 𝑖 ] for an 𝑖-th co-located point with attribute coordinates ( 𝑠, 𝑡, 𝑣 ).  They are:

- extracted from inverse transformed blocks in the bottom two tree levels; unique points from tree level 1; duplicate points from tree level 0; then

- rounded to discard the 15 fractional bits of the fixed-point representation, with halve values rounded away from zero; then

- clipped to be within the attribute value range [ 0, AttrMaxVal ].

RahtRecon[s][t][v][i] := Clip3(0, AttrMaxVal, DivExp2Fz(value, 15))
  where
    value := RahtBlkWeight[0][s][t][v] == 1
      ? RahtDcCoeff[0][s][t][v]
      : RahtCoeff[0][s][t][v][i]

The mapping of the slice geometry to reconstructed attribute values shall map points with identical attribute coordinates to successive elements 𝑖 of RahtRecon[ 𝑠 ][ 𝑡 ][ 𝑣 ][ 𝑖 ] in canonical point order.  i.e. the 𝑖-th element shall be the 𝑖-th instance of the attribute coordinates ( 𝑠, 𝑡, 𝑣 ) from the start of AttrPos.

The following is specified in terms of the sparse array dupPtIdx; dupPtIdx[ 𝑠 ][ 𝑡 ][ 𝑣 ] is the cumulative count of points with attribute coordinates ( 𝑠, 𝑡, 𝑣 ).  Unset elements of dupPtIdx shall be inferred to be 0.

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++) {
  s = AttrPos[ptIdx][0]
  t = AttrPos[ptIdx][1]
  v = AttrPos[ptIdx][2]
  i = dupPtIdx[s][t][v]
  dupPtIdx[s][t][v]++
  PointAttr[ptIdx][Cidx] = RahtRecon[s][t][v][i]
}

  - Attribute decoding using levels of detail

    - General

The attribute decoding processes specified by 10.6 are distance-based prediction schemes that use a hierarchical level-of-detail representation of the slice geometry.  They apply when attr_coding_type is either 1 or 2.

Detail levels are defined by an iterative subsampling process (10.6.5).  The finest detail level comprises all points in the slice geometry.  With each iteration, a coarser detail level is generated from the previous coarsest detail level.

Every detail level comprises a list of points present in the detail level, and is associated with a list of refinement points.  A refinement point is a point that is present in a detail level and not present in any coarser detail level; the refinement points for detail level lvl, when combined with the coarser detail level lvl + 1, form detail level lvl.

For each refinement point, a set of neighbouring points is determined (10.6.6) using inter-detail-level, intra-detail-level and inter-frame searches.  The neighbouring points form a predictor set that is used to predict attribute/transform coefficient values.

Attribute reconstruction (10.6.7) proceeds from the coarsest to the finest detail level.  Transform coefficients are coded in the same order.

A coded transform coefficient is associated with each refinement point.  The transform (10.6.12) comprises two operations: an update step that modifies the predicting points and a prediction step that adds the transform coefficient to a predicted attribute/coefficient value.

Figure 30 — Example of points in three detail levels and their spatial arrangement.

An example level-of-detail hierarchy is illustrated in Figure 30.  LoD0 is the finest detail level and corresponds to all points in the slice.  The generation of subsequent detail levels is performed using periodic subsampling with samplingPeriod equal to 4.  The number of detail levels is limited to 3.  The points in LoD0 that are not assigned to LoD1 are in refinement list R0.  The attribute value for the marked point ● in R0 for LoD0 is predicted from a set of spatially neighbouring points found using an inter-detail-level search of LoD1 and an intra-detail-level search of points earlier in R0.  Transform coefficient values are associated with each refinement point and coded from R2 to R0.  The refinement list R2 comprises all points in LoD2.

    - Syntax element semantics

lod_dist_log2_offset specifies an offset to the APS-specified finest detail level block size lod_initial_dist_log2 for LoD generation and predictor searches.  When lod_dist_log2_offset is not present, it shall be inferred to be 0.

    - Reconstruction process

The reconstruction of point attribute values comprises:

- deriving a set of detail levels from the slice geometry (10.6.5);

- searching for point predictors (10.6.6);

- determining transform coefficient weights (10.6.11) or point quantization weights (10.6.13); and

- reconstructing attribute values from coded coefficients (10.6.7).

The reconstructed values are stored in the array PointAttr.

    - State variables

Levels of detail are specified in terms of the following state variables; the index lvl identifies a detail level:

- The variable LodCnt, a count of detail levels generated from the slice geometry.

- The array LodPtCnt, the size of each detail level; LodPtCnt[ lvl ] is the number of points in the identified detail level.

- The array LodPtIdx, identifying points in each detail level by their index in the canonical decoding order; LodPtIdx[ lvl ][ 𝑖 ] is the AttrPos index of the 𝑖-th point in the identified detail level.

- The array LodRfmtPtCnt, the size of each detail level's refinement list; LodRfmtPtCnt[ lvl ] is the number of points in the refinement list for the identified detail level.

- The array LodRfmtPtIdx, identifying points in each refinement list by their index in the canonical decoding order; LodRfmtPtIdx[ lvl ][ 𝑖 ] is the AttrPos index of the 𝑖-th refinement point for the identified detail level.

Point predictors are specified in terms of the following state variables; the index ptIdx identifies a point by its index into AttrPos:

- The array PredCnt; PredCnt[ ptIdx ] is the size of the predictor set for the identified point.

- The array PredPtIdx, identifies point predictors by their index in the canonical decoding order; PredPtIdx[ ptIdx ][ ni ] is the AttrPos index or RefAttrPos index of the ni-th point in the predictor set for the identified point.

- The array PredPtRef of the flags to specify whether the point predictors are searched from the reference slice. When PredPtRef[ ptIdx ][ ni ] is equal to 1, the ni-th point in the predictor set is specified to be searched from the reference slice for the predictor identified by PredPtIdx[ ptIdx ][ ni ]; When PredPtRef[ ptIdx ][ ni ] is equal to 0, the ni-th point in the predictor set is specified to be searched from the current slice for the predictor identified by PredPtIdx[ ptIdx ][ ni ].

- The array PredWeight of point predictor weights; PredWeight[ ptIdx ][ ni ] is the prediction weight for the predictor identified by PredPtIdx[ ptIdx ][ ni ].

- The array CoeffWeight of transform coefficient weights; CoeffWeight[ ptIdx ] is the normalization weight for the transform coefficients associated with the identified point.

- The array QuantWeight of point quantization weights; QuantWeight[ ptIdx ] is the quantization weight for the transform coefficients associated with the identified point.

    - Levels of detail

      - General generation process

The effect of this process is to represent the LoD structure in the state variables LodCnt, LodPtCnt, LodPtIdx, LodRfmtPtCnt and LodRfmtPtIdx.

The finest detail level shall contain the entire slice geometry (10.6.5.2).  It is identified by the detail level index 0.

Detail levels shall be iteratively subsampled (10.6.5.4), starting from the finest detail level, until either a single point remains or lod_max_levels_minus1 subsampled detail levels have been produced.  The variable Lvl identifies the detail level to be subsampled.

Lvl = 0
for (; Lvl < lod_max_levels_minus1; Lvl++) {
  if (LodPtCnt[Lvl] == 1)
    break
  … /* subsample LodPtIdx[Lvl] */
}
LodCnt = Lvl + 1

The coarsest detail level is identified by the detail level index LodCnt − 1.  All points in the coarsest detail level shall be assigned to the coarsest level's refinement list (10.6.5.3).

      - The finest detail level

The AttrPos point indexes of the finest detail level shall have an initial one-to-one correspondence with the canonical decoding order of the slice geometry.

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  LodPtIdx[0][ptIdx] = ptIdx
LodPtCnt[0] = PointCnt

The point indexes of the finest detail level shall be sorted by group in ascending order of their respective Morton-coded attribute coordinates.  The variable maxPtsPerSort identifies the max group size when sorting by group.

maxPtsPerSort = !attr_canonical_order_enabled && !max_points_per_sort_log2_plus1 ?
  LodPtCnt[0] : 1 << (max_points_per_sort_log2_plus1 - 1)

The sorted order shall be identical for the decoding of all attributes in a single slice with identical attribute coordinate arrays (AttrPos).

- Performing a stable sort for each attribute, or reusing the reordered points would satisfy the requirement for identical orders.

An example (inefficient) sorting process is:

for (benIdx = 0; benIdx < LodPtCnt[0]; benIdx += maxPtsPerSort) {
  endIdx = Min(benIdx + maxPtsPerSort, LodPtCnt[0])
  for (i = benIdx; i < endIdx; i++)
    for (j = i + 1; j < endIdx; j++) {
      iPtIdx = LodPtIdx[0][i]
      jPtIdx = LodPtIdx[0][j]
      iMorton = Morton(AttrPos[iPtIdx][0], AttrPos[iPtIdx][1], AttrPos[iPtIdx][2])
      jMorton = Morton(AttrPos[jPtIdx][0], AttrPos[jPtIdx][1], AttrPos[jPtIdx][2])
      if (iMorton > jMorton)
        Swap(LodPtIdx[0][i], LodPtIdx[0][j])
    }
}

      - The coarsest detail level

After generation of the LoD hierarchy, all points in the coarsest detail level shall be assigned to its refinement list.

for (i = 0; i < LodPtCnt[LodCnt − 1]; i++)
  LodRfmtPtIdx[LodCnt − 1][i] = LodPtIdx[LodCnt − 1][i]

      - Generation of a single detail level

The coarser detail level Lvl + 1 shall be produced by subsampling the points of detail level Lvl.

The following definitions are used in the specification of the subsampling processes:

- The expression InLodPtCnt is an alias for LodPtCnt[ Lvl ], the number of points in the input detail level.

- The expression InLodPtIdx[ 𝑖 ] is an alias for LodPtIdx[ Lvl ][ 𝑖 ], the point indexes of the input detail level.

- The expression OutLodPtCnt is an alias for LodPtCnt[ Lvl + 1 ], the number of points in the output detail level.

- The expression OutLodPtIdx[ 𝑖 ] is an alias for LodPtIdx[ Lvl + 1 ][ 𝑖 ], the point indexes of the output detail level.

- The expression OutRfmtPtIdx[ 𝑖 ] is an alias for LodRfmtPtIdx[ Lvl ][ 𝑖 ], the point indexes of the refinement list for detail level Lvl.

Subsampling partitions points in the input detail level into an output detail level and the refinement list for the input detail level.  The partitioning process shall preserve the relative ordering of points in the input detail level.

Subsampling shall proceed according to:

- block-based subsampling (10.6.5.8) if lod_scalability_enabled is 1, or lod_decimation_mode is 2;

- periodic subsampling (10.6.5.5) if lod_decimation_mode is 1; or

- distance-based subsampling (10.6.5.6) otherwise.

      - Periodic subsampling

When aps_extension_present is 1, geom_tree_type is 1, and attr_canonical_order_enabled is 0, the variable CanonicalLodSubsampling is set to 1. Otherwise, CanonicalLodSubsampling is set to 0.

Periodic subsampling generates a subsampled output detail level by sampling every one-in-sampling-period points in the input detail level.

If CanonicalLodSubsampling is equal to 0, periodic subsampling generates a subsampled output detail level by:

The sampling period for the current detail level is subsamplingPeriod.

samplingPeriod := 2 + lod_sampling_period_minus2[Lvl]

Input points shall be assigned to either the output detail level or the refinement list according to their index in the input detail level modulo the sampling period:

OutLodPtCnt = outRfmtPtCnt = 0
for (i = 0; i < InLodPtCnt; i++) {
  if (i % samplingPeriod)
    OutRfmtPtIdx[outRfmtPtCnt++] = InLodPtIdx[i]
  else
    OutLodPtIdx[OutLodPtCnt++] = InLodPtIdx[i]
}

If CanonicalLodSubsampling is equal to 1, canonical periodic subsampling generates a subsampled output detail level by:

The sampling period for the current detail level is subsamplingPeriod.

samplingPeriod := (Lv1==0 ? 1 : canonical_samplingPeriod[Lvl-1]) * (2 +
  lod_sampling_period_minus2[Lvl])

Input points shall be assigned to either the output detail level or the refinement list according to their index in the input detail level modulo the sampling period:

OutLodPtCnt = outRfmtPtCnt = 0
for (i = 0; i < InLodPtCnt; i++) {
  if (InLodPtIdx[i] % samplingPeriod)
    OutRfmtPtIdx[outRfmtPtCnt++] = InLodPtIdx[i]
  else
    OutLodPtIdx[OutLodPtCnt++] = InLodPtIdx[i]
}
canonical_samplingPeriod[Lvl] = samplingPeriod

      - Distance-based subsampling

Distance-based subsampling generates a subsampled output detail level by:

- spatially partitioning the input detail level into a lattice of 2BlkSizeLog2 sized cubic blocks; and

- assigning at most one point from each block to the subsampled detail level.

BlkSizeLog2 := lod_initial_dist_log2 + lod_dist_log2_offset + Lvl + 1

The subsampling process is specified in terms of the following state variables; the indexes bs, bt and bv identify the block location ( bs, bt, bv ):

- The sparse array MapSub; MapSub[ bs ][ bt ][ bv ] equal to 1 indicates that the identified block contains a single point previously assigned to the subsampled detail level.  Unset elements of MapSub are inferred to be 0.

- The sparse array MapPtIdx, identifies points assigned to the subsampled detail level.  When MapSub[ bs ][ bt ][ bv ] is 1, MapPtIdx[ bs ][ bt ][ bv ] is the AttrPos index of the point assigned to the subsampled detail level.

The points in the input detail level shall be processed sequentially.  For each input point:

- The variable PtIdx is the AttrPos index of the point.

- The block location ( Bs, Bt, Bv ) is determined from the point's attribute coordinates.

- Depending upon the result of a per-point test (10.6.5.7), the point shall be assigned to either the output detail level or the refinement list.  The result of the test is the variable IsSubsampledPoint.

OutLodPtCnt = outRfmtPtCnt = 0
for (i = 0; i < InLodPtCnt; i++) {
  PtIdx = InLodPtIdx[i]
  Bs = AttrPos[PtIdx][0] >> BlkSizeLog2
  Bt = AttrPos[PtIdx][1] >> BlkSizeLog2
  Bv = AttrPos[PtIdx][2] >> BlkSizeLog2

  … /* IsSubsampledPoint = result of per−point test (10.6.5.7) */

  if (MapSub[Bs][Bt][Bv] || ¬IsSubsampledPoint)
    OutRfmtPtIdx[outRfmtPtCnt++] = PtIdx
  else {
    OutLodPtIdx[OutPtCnt++] = PtIdx
    MapSub[Bs][Bt][Bv] = 1 
    MapPtIdx[Bs][Bt][Bv] = PtIdx
  }
}

      - Per-point decision for distance-based subsampling

The derivation of IsSubsampledPoint specifies whether the point shall be assigned to the output detail level.

A point shall be assigned to the output detail level unless the squared distance between it and any previously assigned point from a set of adjacent blocks within an availability window is less than or equal to sqRadius.  Each availability window shall be a 128×128×128 block volume identified by ( Bs >> 7, Bt >> 7, Bv >> 7 ).

sqRadius = 3 << 2 × (BlkSizeLog2 − 1)

Figure 31 — Example decisions using distance-based subsampling.

Example per-point decisions are illustrated in Figure 31.  Subsampling generates three detail levels.  The points assigned to LoD1 by subsampling LoD0 (when Lvl = 0) are not within the shaded radius sqRadius of any other point in LoD1.  The block size used to subsample LoD0 is BlkSizeLog2 = 1.  All points are within a single availability window.

The array neighPtIdx is a neighCnt-element list of AttrPos indexes of points present in the adjacent blocks of the output detail level that are within the availability window.  Table 52 specifies the relative locations of the adjacent blocks.

neighCnt = 0
for (i = 0; i < 19; i++) {
  ns = Bs + adjBlkOffset[i][0]
  nt = Bt + adjBlkOffset[i][1]
  nv = Bv + adjBlkOffset[i][2]
  unavailable = (ns ^ Bs) >> 7 || (nt ^ Bt) >> 7 || (nv ^ Bv) >> 7
  if (unavailable)
    continue
  if (MapSub[ns][nt][nv])
    neighPtIdx[neighCnt++] = MapPtIdx[ns][nt][nv]
}

Table 52— Adjacent block coordinates, adjBlkOffset[ 𝑖 ][ 𝑘 ], relative to ( Bs, Bt, Bv )

| 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  | 0 | 1 | 2 |  | 0 | 1 | 2 |  | 0 | 1 | 2 |
| 0 | −1 | 0 | 0 | 5 | −1 | 1 | 0 | 10 | −1 | 0 | −1 | 15 | 1 | −1 | −1 |
| 1 | 0 | −1 | 0 | 6 | 0 | 1 | −1 | 11 | −1 | −1 | 0 | 16 | −1 | 1 | −1 |
| 2 | 0 | 0 | −1 | 7 | 1 | 0 | −1 | 12 | −1 | 1 | 1 | 17 | −1 | −1 | 1 |
| 3 | 0 | −1 | 1 | 8 | 1 | −1 | 0 | 13 | 1 | −1 | 1 | 18 | −1 | −1 | −1 |
| 4 | −1 | 0 | 1 | 9 | 0 | −1 | −1 | 14 | 1 | 1 | −1 |  |  |  |  |

The point's attribute coordinates shall be compared to those of each point identified by the neighPtIdx array to determine the value of IsSubsampledPoint.

IsSubsampledPoint = 1
for (i = 0; i < neighCnt; i++) {
  sqDist = 0
  for (k = 0; k < 3; k++) {
    d = AttrPos[neighPtIdx[i]][k] − AttrPos[PtIdx][k]
    sqDist += d × d
  }
  if (sqDist ≤ sqRadius)
    IsSubsampledPoint = 0
}

      - Block-based subsampling

Block-based subsampling generates a subsampled output detail level by:

- spatially partitioning the input detail level into a lattice of 2BlkSizeLog2 sized cubic blocks;

- grouping together blocks, in Morton order, according to the number of points they contain; and

- assigning one point from each block group to the subsampled detail level.

BlkSizeLog2 := lod_initial_dist_log2 + lod_dist_log2_offset + Lvl + 1

- Under certain conditions, blocks correspond to nodes of the occupancy tree.  For instance, when lod_scalability_enabled is 1.

A list of block groups shall be generated by traversing the input detail level in canonical order.  Consecutive blocks shall be grouped together until the group spans at least minGrpPts points.

minGrpPts := lod_scalability_enabled ? 0 : 2 + lod_sampling_period_minus2[Lod]

The array grpBdry, with elements grpBdry[ grpIdx ], identifies block group boundaries as indexes into the input detail level array InLodPtIdx.

for (i = 1, grpStart = 0; i < InLodPtCnt; i++) {
  ptIdx = InLodPtIdx[i]
  ptIdxPrev = InLodPtIdx[i − 1]
  bdryS = (AttrPos[ptIdx][0] ^ AttrPos[ptIdxPrev][0]) >> BlkSizeLog2
  bdryT = (AttrPos[ptIdx][1] ^ AttrPos[ptIdxPrev][1]) >> BlkSizeLog2
  bdryV = (AttrPos[ptIdx][2] ^ AttrPos[ptIdxPrev][2]) >> BlkSizeLog2
  if (bdryS | bdryT | bdryV)
    if (i − grpStart ≥ minGrpPts)
      grpBdry[grpCnt++] = grpStart = i
}
grpBdry[grpCnt++] = InLodPtCnt

For each group of blocks, a test (10.6.5.9) shall be performed to determine the index of the point to be assigned to the output detail level.  All other points shall be assigned to the refinement list.  The variables GrpStart and GrpEnd identify the start and end of a block group.  The result of the test is the variable IdxOfSubsampledPoint.

OutLodPtCntSize = outRfmtPtCnt = 0
for (GrpStart = grpIdx = 0; grpIdx < grpCnt; GrpStart = grpBdry[grpIdx++]) {
  GrpEnd = grpBdry[grpIdx]

  … /* IdxOfSubsampledPoint = result of per−point test (10.6.5.9) */

  for (i = GrpStart; i < GrpEnd; i++) {
    if (IdxOfSubsampledPoint == i)
      OutLodPtIdx[OutLodPtCnt++] = InLodPtIdx[i]
    else
      OutRfmtPtIdx[outRfmtPtCnt++] = InLodPtIdx[i]
  }
}

      - Per block-group decision for block-based subsampling

The derivation of IdxOfSubsampledPoint specifies the input detail level index of the point in the block group that shall be assigned to the output detail level.

The distance to the block group centroid shall be used to select the point assigned to the output detail level.  The block group centroid and point distances shall be calculated using attribute coordinates quantized by Exp2( BlkSizeLog2 − 1 ).  The distance metric shall be the Manhattan distance.

Figure 32 — Example decisions using block group subsampling with minGrpPts = 3.

Example per-point decisions are illustrated in Figure 32.  Subsampling generates two detail levels.  To subsample LoD0, the points are grouped into block groups containing a minimum of three points.  In this example, the block size for LoD0 (Lvl = 0) is BlkSizeLog2 = 1.  The first block group (solid shading) comprises four points from three blocks each with one, one and two points, respectively.  The first point that is the closest to the centroid of the points in the block group is assigned to LoD1.

The block group centroid shall be the sum of all quantized attribute coordinates, centroidSum, divided by the number of points in the block group, numPtsInGrp.

numPtsInGrp := GrpEnd − GrpStart
for (k = 0; k < 3; k++)
  centroidSum[k] = 0
for (i = 0; i < numPtsInGrp; i++) {
  ptIdx = InLodPtIdx[GrpStart + i]
  for (k = 0; k < 3; k++)
    centroidSum[k] += AttrPos[ptIdx][k] >> BlkSizeLog2 − 1
}

The array ptDist maps the index of each point in the block group to the distance between it and the centroid.

for (i = 0; i < numPtsInGrp; i++) {
  ptIdx = InLodPtIdx[GrpStart + i]
  ptDist[i] = 0
  for (k = 0; k < 3; k++) {
    posk = AttrPos[ptIdx][k] >> BlkSizeLog2 − 1
    ptDist[i] += Abs(posk × numPtsInGrp − centroidSum[k])
  }
}

The point closest to the block group centroid shall be assigned to the output detail level.  In the case that the block group contains multiple closest points, the selected point is the closest point with:

- when lod_scalability_enabled is 1 and Lvl is odd: the greatest InLodPtIdx index;

- when lod_scalability_enabled is 0 or Lvl is even: the lowest InLodPtIdx index.

last := lod_scalability_enabled ? Lvl & 1 : 1
minIdx = 0
for (i = 1; i < numPtsInGrp; i++)
  if (last ? dist[i] ≤ dist[minIdx] : dist[i] < dist[minIdx])
    minIdx = i

IdxOfSubsampledPoint = GrpStart + minIdx

      - The finest detail level of the reference slice

When slice_attr_inter_prediction is equal to 1, the finest detail level of the reference slice is specified in terms of the following state variables:

- The variable refLodPtCnt, the size of the finest detail level.

- The array RefLodPtIdx, identifying points in the finest detail level by their index in the canonical decoding order; RefLodPtIdx [ 𝑖 ] is the RefAttrPos index of the 𝑖-th point in the finest detail level.

The RefAttrPos point indexes of the finest detail level shall have an initial one-to-one correspondence with the canonical decoding order of the reference slice geometry.

for (ptIdx = 0; ptIdx < refPointCnt; ptIdx++)
  LodPtIdx[ptIdx] = ptIdx
refLodPtCnt = refPointCnt

The point indexes of the finest detail level shall be sorted by group in ascending order of their respective Morton-coded attribute coordinates.  The sorted order shall be identical for the decoding of all attributes in a single slice with identical attribute coordinate arrays (RefAttrPos).

- Performing a stable sort for each attribute, or reusing the reordered points would satisfy the requirement for identical orders.

An example (inefficient) sorting process is:

for (benIdx = 0; benIdx < RefLodPtCnt; benIdx += maxPtsPerSort) {
  endIdx = Min(benIdx + maxPtsPerSort, RefLodPtCnt)
  for (i = benIdx; i < endIdx; i++)
    for (j = i + 1; j < endIdx; j++) {
      iPtIdx = RefLodPtIdx[i]
      jPtIdx = RefLodPtIdx[j]
      iMorton = Morton(RefAttrPos[iPtIdx][0], RefAttrPos[iPtIdx][1],
        RefAttrPos[iPtIdx][2])
      jMorton = Morton(RefAttrPos[jPtIdx][0], RefAttrPos[jPtIdx][1],
        RefAttrPos[jPtIdx][2])
      if (iMorton > jMorton)
        Swap(RefLodPtIdx[i], RefLodPtIdx[j])
    }
}

    - Predictor search

      - General process

The points used to predict the refinement points of each detail level shall be determined by a search (10.6.6.3).

The effect of this process is to represent the point predictors in the state variables PredCnt, PredPtIdx, PredPtRef and PredWeight.

When attr_coding_type is 2, no searches shall be performed for the refinement points of the coarsest detail level.

maxLvl = LodCnt − (attr_coding_type == 2)
for (Lvl = 0; Lvl < maxLvl; Lvl++)
  for (RfmtIdx = 0; RfmtIdx < LodRfmtPtCnt[Lvl]; RfmtIdx++) {
    … /* find predictors (10.6.6.2) of the current point */
  }

      - Minimum reference detail level for inter-level predictor searches

The variable MinInterRefLvl identifies the finest detail level that shall be used as a reference for inter-detail level prediction.  When lod_scalability_enabled is 1, it shall be the finest detail level with fewer refinement points than the total number of refinement points associated with all finer detail levels.

MinInterRefLvl = 1
if (lod_scalability_enabled) {
  for (lvl = 1; lvl < LodCnt − 1; lvl++) {
    if (LodRfmtPtCnt[lvl] < slice_num_points_minus1 − LodPtCnt[lvl])
      break
    MinInterRefLvl++
  }
}

      - Predictor search for a single refinement point

For a refinement point with index RfmtIdx in detail level Lvl, a search shall be performed to find the closest neighbouring points from a set of candidate neighbours.

The search process is specified in terms of the following variables:

- The variable PtIdx, the AttrPos index of the refinement point.

- The variable RefLvl, the reference detail level used for inter-level predictor searches.

PtIdx = LodRfmtPtIdx[RfmtIdx]
RefLvl = Max(Lvl + 1, MinInterRefLvl)

An inter-detail-level search shall be performed prior to any intra-level search.  Except for the coarsest detail level, the following inter-level searches shall be performed:

- An initial search (10.6.6.6).

- If fewer than three predictors are found (PredCnt[ PtIdx ] < 3), an extended search (10.6.6.7).

When Lvl is greater than or equal to pred_intra_min_lod, an intra-detail-level search (10.6.6.8) shall be performed.

When slice_attr_inter_prediction is equal to 1, an initial inter-frame search (10.6.6.9) and an extended inter-frame search (10.6.6.10) shall be performed.

After completing the searches, weights shall be calculated for each predictor (10.6.6.11), during which the predictor set is pruned and re-sorted.  When pred_blending_enabled is 1, predictor weights shall be blended (10.6.6.12).

Figure 33 — Example of searches performed for a single refinement point.

An example predictor search when slice_attr_inter_prediction is equal to 0 is illustrated in Figure 33.  Searches are performed for the refinement point 𝛾 in R0 of LoD0; other points in R0 are denoted 𝛼, 𝛽 and 𝛿.  Points in the next coarsest detail level, LoD1, are marked a to f.  After the initial inter-level search, the predictor set is { c, b }.  Then, since fewer than three predictors were found, an extended inter-level search is performed over ±pred_inter_lod_search_range points in the LoD1 point list.  This search adds predictor d to the predictor set { c, b, d }.  Finally, an intra- level search is performed over pred_intra_lod_search_range previous points in R0.  The final predictor set for γ is { c, 𝛽, 𝛼 }.

      - Inclusion of a candidate point in the predictor set (InsertPredictor)

This subclause defines the function InsertPredictor( candPtIdx  , candRef) that conditionally inserts a candidate point into the predictor set of the current refinement point.  Each candidate shall be tested against the refinement point's predictor set to determine if and where it is to be inserted.

The parameter candPtIdx is the AttrPos index or the RefAttrPos index of the candidate point. The parameter candRef specifies whether the candidate point is searched from the reference slice.

A candidate shall only be inserted into the point's neighbour set once.  If candPresent is 1, the candidate is not inserted into the predictor set.

candPresent = 0
for (i = 0; i < PredCnt[PtIdx]; i++)
  candPresent |= PredPtIdx[PtIdx][i] == candPtIdx
    && (slice_attr_inter_prediction
    ? PredPtRef[PtIdx][i] == candRef
    : 1)

Otherwise (the candidate is not already present), the following shall be used to decide the inclusion in the predictor set.

- the spatial distance between the candidate and the refinement point.

- the relative spatial location of the candidate to the current point.

The distance shall be calculated as the biased l1 norm weighted by PredBias.

dist = BiasedNorm1(PtIdx, candPtIdx, 0, candRef)

The point shall be inserted into the predictor set, with elements ordered according to the biased l1 distance to the refinement point.  When the size of predictor set is less than 6 and prediction_with_distribution_enabled is 1, the point is inserted only when the distance between the point and the current point is equal to the distance between the third predictor in the predictor set and the current point.  Points at the same distance shall be ordered by insertion order, with earlier members being ordered before later members.

for (i = 0; i < PredCnt[PtIdx]; i++)
  if (dist < BiasedNorm1(PtIdx, PredPtIdx[PtIdx][i], 0, PredPtRef[PtIdx][i]))
    break
  if(i < 3 || ((dist == BiasedNorm1(PtIdx, PredPtIdx[PtIdx][2])) && PredCnt[PtIdx] < 6)
    for (j = PredCnt[PtIdx]; j > i; j−−){
      PredPtIdx[PtIdx][j] = PredPtIdx[PtIdx][j − 1]
      if (slice_attr_inter_prediction)
        PredPtRef[PtIdx][j] = PredPtRef[PtIdx][j - 1]
    }
  PredPtIdx[PtIdx][i] = candPtIdx
  PredPtRef[PtIdx][i] = slice_attr_inter_prediction ? candRef : 0

The size of the predictor set shall be limited to three elements (when prediction_with_distribution_enabled is 0) or six elements (when prediction_with_distribution_enabled is 1) by discarding the furthest predictor if necessary.

PredCnt[PtIdx] = Min(prediction_with_distribution_enabled ? 6 : 3, PredCnt[PtIdx] + 1)

      - Distance computation using the biased L1 norm (BiasedNorm1)

This subclause defines the function BiasedNorm1( ptIdxA, ptIdxB, ptA, ptB) that is the weighted Manhattan distance between two points.

When pcA is equal to 0, the parameter, the parameters ptIdxA is an AttrPos index. When pcA is equal to 1, the parameter, the parameters ptIdxA is a RefAttrPos index.

When pcB is equal to 0, the parameter, the parameters ptIdxB is an AttrPos index. When pcB is equal to 1, the parameter, the parameters ptIdxB is a RefAttrPos index.

The result of this function is specified by the expression BiasedNorm1.  The expression posA[ ptIdx ][ 𝑘 ] and posB[ ptIdx ][ 𝑘 ] represent the attribute coordinates used to calculate the distance: when lod_scalability_enabled is 1, coordinates shall be quantized according to the detail level.

BiasedNorm1(ptIdxA, ptIdxB, pcA, pcB) := dist[0] + dist[1] + dist[2]
  where
    dist[k] := Abs(posA[ptIdxA][k] − posB[ptIdxB][k]) × PredBias[k]
    posA[ptIdx][k] := pcA ?
      RefAttrPos[ptIdxA][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxA][k] >> Lvl) << Lvl
      : AttrPos[ptIdxA][k])
    posB[ptIdx][k] := pcB ?
      RefAttrPos[ptIdxB][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxB][k] >> Lvl) << Lvl
      : AttrPos[ptIdxB][k])

      - Initial inter-level predictor search

The initial inter-level search shall be performed by spatially partitioning the reference detail level into a lattice of 2BlkSizeLog2 sized cubic blocks.  Only blocks adjacent to the block containing the refinement point that are within an availability window shall be searched.

BlkSizeLog2 := lod_initial_dist_log2 + lod_dist_log2_offset + Lvl + 1

The block location ( bs, bt, bv ) identifies the block containing the refinement point.

bs := AttrPos[PtIdx][0] >> BlkSizeLog2
bt := AttrPos[PtIdx][1] >> BlkSizeLog2
bv := AttrPos[PtIdx][2] >> BlkSizeLog2

The availability window shall be a 128×128×128 block volume identified by ( bs >> 7, bt >> 7, bv >> 7 ).

The search shall proceed over the search blocks in the order specified by Table 53.  Within each search block, points shall be searched in ascending order of index within the reference detail level.

for (si = 0; si < 27; si++) {
  ss = bs + searchBlkOffsets[si][0]
  st = bt + searchBlkOffsets[si][1]
  sv = bv + searchBlkOffsets[si][2]
  unavailable = (ss ^ bs) >> 7 || (st ^ bt) >> 7 || (sv ^ bv) >> 7
  if (unavailable)
    continue
  for (i = 0; i < LodPtCnt[RefLvl]; i++) {
    candPtIdx = LodPtIdx[RefLvl][i]
    cs = AttrPos[candPtIdx][0]
    ct = AttrPos[candPtIdx][1]
    cv = AttrPos[candPtIdx][2]
    inSblk  = cs ≥ (ss << BlkSizeLog2) && cs < (ss + 1 << BlkSizeLog2)
    inSblk &= ct ≥ (st << BlkSizeLog2) && ct < (st + 1 << BlkSizeLog2)
    inSblk &= cv ≥ (sv << BlkSizeLog2) && cv < (sv + 1 << BlkSizeLog2)
    if (inSblk)
      InsertPredictor(candPtIdx, 0)
  }
}

- For each search block, the indices 𝑖 for which inSblk is true are consecutive.

Table 53 — Search block coordinates, searchBlkOffsets[ 𝑖 ][ 𝑘 ], relative to ( bs, bt, bv )

| 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  | 𝑖 | 𝑘 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 |  | 0 | 1 | 2 |  | 0 | 1 | 2 |  | 0 | 1 | 2 |
| 0 | 0 | 0 | 0 | 7 | 0 | 1 | 1 | 14 | 1 | 0 | −1 | 21 | 1 | −1 | 1 |
| 1 | −1 | 0 | 0 | 8 | 1 | 0 | 1 | 15 | 1 | −1 | 0 | 22 | 1 | 1 | −1 |
| 2 | 0 | −1 | 0 | 9 | 1 | 1 | 0 | 16 | 0 | −1 | −1 | 23 | 1 | −1 | −1 |
| 3 | 0 | 0 | −1 | 10 | 0 | −1 | 1 | 17 | −1 | 0 | −1 | 24 | −1 | 1 | −1 |
| 4 | 1 | 0 | 0 | 11 | −1 | 0 | 1 | 18 | −1 | −1 | 0 | 25 | −1 | −1 | 1 |
| 5 | 0 | 1 | 0 | 12 | −1 | 1 | 0 | 19 | 1 | 1 | 1 | 26 | −1 | −1 | −1 |
| 6 | 0 | 0 | 1 | 13 | 0 | 1 | −1 | 20 | −1 | 1 | 1 |  |  |  |  |

      - Extended inter-level search

The extended inter-level search evaluates predictor candidates over a span of indexes in the reference detail level.

The span shall be centred around the index centre in the reference detail level.  It shall be the index of:

- if at least one predictor has been found for the current point: the first predictor; or

if (PredCnt[PtIdx])
  for (centre = 0; centre < LodPtCnt[RefLvl] − 1; centre++)
    if (LodPtIdx[RefLvl][centre] == PredPtIdx[PtIdx][0])
      break

- otherwise (no predictors have been found): the first point with Morton-coded attribute coordinates greater than those of the current point.

if (PredCnt[PtIdx] == 0) {
  mortonCurPt = Morton[AttrPos[PtIdx]]
  for (centre = 0; centre < LodPtCnt[RefLvl] − 1; centre++) {
    mortonCentre = Morton[AttrPos[LodPtIdx[RefLvl][centre]]]
    if (mortonCurPt < mortonCentre)
      break
  }
}

The search range for extended inter-level search is specified by the variable interLodSearchRange.

if (slice_attr_inter_prediction)
  interLodSearchRange = attr_inter_prediction_search_range
else
  interLodSearchRange = pred_inter_lod_search_range

The extended search shall proceed over each index offset 𝑖 of the following, in order: 0, +1, −1, +2, −2, +3 .. interLodSearchRange, and −( 3 .. interLodSearchRange ).

A predictor candidate shall be evaluated for each valid search index centre + 𝑖 that is within the range specified by interLodSearchRange and does not exceed the bounds of the reference detail level.

if (Abs(i) ≤ interLodSearchRange)
  if (centre + i ≥ 0 && centre + i < LodPtCnt[RefLvl])
    InsertPredictor(LodPtIdx[RefLvl][centre + i], 0)

      - Intra-level search

The intra-level search evaluates predictor candidates over a span of indexes in the refinement list of the current detail level.  Intra-level predictor candidates shall precede the refinement point in the refinement list.

The search range for intra-level search is specified by the variable intraLodSearchRange.

if (slice_attr_inter_prediction)
  intraLodSearchRange = attr_inter_prediction_search_range
else
  intraLodSearchRange = pred_intra_lod_search_range

A predictor candidate shall be evaluated for each valid search index offset − 𝑖 from the refinement point, for 𝑖 = 1 .. intraLodSearchRange, that does not exceed the bounds of the refinement list.

for (i = 1; i ≤ Min(RfmtIdx, intraLodSearchRange); i++)
  InsertPredictor(LodRfmtPtIdx[Lvl][RfmtIdx − i], 0)

      - Initial inter-frame predictor search

The initial inter-frame search shall be performed by spatially portioning the reference slice into a lattice of 2BlkSizeLog2 sized cubic blocks.  Only blocks adjacent to the block containing the refinement point that are within an availability window shall be searched.

BlkSizeLog2 := lod_initial_dist_log2 + lod_dist_log2_offset + Lvl + 1

The block location ( bs, bt, bv ) identifies the block containing the refinement point.

bs := AttrPos[PtIdx][0] >> BlkSizeLog2
bt := AttrPos[PtIdx][1] >> BlkSizeLog2
bv := AttrPos[PtIdx][2] >> BlkSizeLog2

The availability window shall be an 8×8×8 block volume identified by ( bs >> 3, bt >> 3, bv >> 3 ).

The search shall proceed over the search blocks in the order specified by Table 53.  Within each search block, points shall be searched in ascending order of index within the reference slice.

for (si = 0; si < 27; si++) {
  ss = bs + searchBlkOffsets[si][0]
  st = bt + searchBlkOffsets[si][1]
  sv = bv + searchBlkOffsets[si][2]
  unavailable = (ss ^ bs) >> 3 || (st ^ bt) >> 3 || (sv ^ bv) >> 3
  if (unavailable)
    continue
  for (i = 0; i < RefLodPtCnt; i++) {
    candPtIdx = RefLodPtIdx[i]
    cs = RefAttrPos [candPtIdx][0]
    ct = RefAttrPos [candPtIdx][1]
    cv = RefAttrPos [candPtIdx][2]
    inSblk  = cs ≥ (ss << BlkSizeLog2) && cs < (ss + 1 << BlkSizeLog2)
    inSblk &= ct ≥ (st << BlkSizeLog2) && ct < (st + 1 << BlkSizeLog2)
    inSblk &= cv ≥ (sv << BlkSizeLog2) && cv < (sv + 1 << BlkSizeLog2)
    if (inSblk)
      InsertPredictor(candPtIdx, 1)
  }
}

- For each search block, the indices 𝑖 for which inSblk is true are consecutive.

      - Extended inter-frame search

The inter-frame search evaluates predictor candidates over a span of indexes in the finest detail level of the reference slice.

The span shall be centred around the index centreRef in the finest detail level of the reference slice.  It shall be the index of the first point with Morton-coded attribute coordinates greater than or equal to those of the current point.

mortonCurPt = Morton[AttrPos[PtIdx]]
for (centreRef = 0; centreRef < RefLodPtCnt − 1; centreRef++) {
  mortonCentre = Morton[RefAttrPos[RefLodPtIdx[centreRef]]]
  if (mortonCurPt <= mortonCentre)
    break
}

The search range for intra-level search is specified by the variable interFrameSearchRange.

interFrameSearchRange := (slice_biprediction && slice_attr_inter_prediction &&
  slice_attr_inter_prediction2) ? attr_inter_prediction_search_range >> 1 :
  attr_inter_prediction_search_range

The inter-frame search shall proceed over each index offset 𝑖 of the following, in order: 0, +1, −1, +2, −2, +3 .. interFrameSearchRange, and −( 3 .. interFrameSearchRange ).

A predictor candidate shall be evaluated for each valid search index centreRef + 𝑖 that is within the range specified by interFrameSearchRange and does not exceed the bounds of the finest detail level of the reference slice.

if (Abs(i) ≤ interFrameSearchRange)
  if (centreRef + i ≥ 0 && centreRef + i < RefLodPtCnt)
    InsertPredictor(RefLodPtIdx[centreRef + i], 1)

      - Predictor set pruning and generation of prediction weights

After the predictor search for a refinement point is complete, its predictor set shall be pruned, weights computed for each qualifying predictor and the predictors ordered according to weight.

The size of the predictor set shall be limited to pred_set_size_minus1 + 1 elements by discarding the furthest predictors if necessary.

PredCnt[PtIdx] = Min(pred_set_size_minus1 + 1, PredCnt[PtIdx])

When cross_attr_prediction_enabled_this_type is 0, predictor weights shall be calculated using the biased squared distance between each predictor and the current point.

for (ni = 0; ni < PredCnt[PtIdx]; ni++)
  dist[ni] = BiasedNorm2(PtIdx, PredPtIdx[PtIdx][ni], 0, PredPtRef[PtIdx][ni])
    + PredPtRef[PtIdx][ni] ? 1 : 0

If the first predictor is spatially coincident with the current point, all other predictors shall be discarded.

if (dist[0] == 0)
  PredCnt[PtIdx] = 1

When lod_scalability_enabled is 1, predictors with an unbiased squared distance greater than a threshold shall be discarded.

if (lod_scalability_enabled) {
  threshold = 3 × (pred_max_range_minus1 + 1) << 2 × Lvl
  for (ni = 1; ni < PredCnt[PtIdx]; ni++)
    if (Norm2(ptIdx, PredPtIdx[PtIdx][ni], 0, 0) > threshold) {
      PredCnt[PtIdx] = ni
      break
    }
}

When cross_attr_prediction_enabled_this_type is 1, predictor weights shall be calculated using the overall distance between each predictor and the current point

for (ni = 0; ni < PredCnt[PtIdx]; ni++
  dist[ni] = overAllDist[ni]

The overall distance is a weighted combination of the geometry distance and the attribute distance and shall be stored in array overAllDist[ni], ni = 0 .. PredCnt[ PtIdx ] − 1.  The geometry distance is defined as the spatial distance which shall be calculated as the biased l1 norm weighted by PredBias.  The attribute distance is defined as the sum of the absolute difference in attribute value for each component.

overAllDist[ni] = geomDis + attrWeight * attrDis
  where
    geomDis = BiasedNorm1(PtIdx, PredPtIdx[PtIdx][ni], 0, 0)
    attrDis = 0
    for(i = 0; i < attr_components_minus1[refAttrIdx] + 1; i++)
      attrDis += Abs(RecCloudAttr[ ptIdx ][ refAttrIdx][i] -
        RecCloudAttr[PredPtIdx[PtIdx][ni]][ refAttrIdx][i])

The attribute weight attrWeight which is defined by the weight of attribute distance shall be determined as:

attrWeight = lambda * maxGeom / maxAttr
  where
    lambda = attr_label[refAttrIdx] ? (4011 – 67 * (attr_primary_qp_minus4 + 4)) >> 15 :
      (1939 – 33 * (attr_primary_qp_minus4 + 4)) >> 12
    for (k = 0; k < 3; j++)
      maxGeom += RefCloudAttrBboxSize[k]
    maxAttr = (attr_components_minus1[refAttrIdx] + 1) *
      1 << (attr_bitdepth_minus1[refAttrIdx] + 1)

where maxGeom represents the summation of the length, width and height of the slice bounding box and maxAttr represents the possible maximum value of the encoded attribute that is used for decoding the current attribute.

If the first predictor is same as the current point, all other predictors shall be discarded.

if (dist[0] == 0)
  PredCnt[PtIdx] = 1

When cross_attr_prediction_enabled_this_type is 1, the predictors shall be reordered in ascending order according to their overall distance to the current point; when cross_attr_prediction_enabled_this_type is 0, the predictors shall be reordered according to their biased squared distance to the current point:

- An array order shall have elements such that dist[ order[ 𝑖 ] ], for 𝑖 = 0 .. PredCnt[ PtIdx ] − 1, is an ascending stable sorting of the array dist.

- The members of the predictor set and the dist array shall be permuted according to the elements of the array order.

The predictor distances shall be normalized by the smallest distance to produce initial weights.

n = Max(0, IntLog2(dist[0] − 8))
for (ni = 0; ni < PredCnt[PtIdx]; ni++)
  weight[ni] = DivExp2Up(dist[ni], n)

Any predictors with a weight 256 times greater than or equal to the smallest weight shall be discarded.

if (PredCnt[PtIdx] == 3 && weight[2] ≥ 256 × weight[0])
  PredCnt[PtIdx] = 2

if (PredCnt[PtIdx] == 2 && weight[1] ≥ 256 × weight[0])
  PredCnt[PtIdx] = 1

The final weights shall be derived as:

if (PredCnt[PtIdx] == 1)
  PredWeight[PtIdx][0] = 256

if (PredCnt[PtIdx] == 2) {
  PredWeight[PtIdx][1] = Div(weight[0], weight[0] + weight[1], 8)
  PredWeight[PtIdx][0] = 256 − PredWeight[PtIdx][1]
}

if (PredCnt[PtIdx] == 3) {
  d1d2 = weight[1] × weight[2]
  d0d2 = weight[0] × weight[2]
  d0d1 = weight[0] × weight[1]
  sum = d1d2 + d0d2 + d0d1
  PredWeight[PtIdx][2] = Div(d0d1, sum, 8)
  PredWeight[PtIdx][1] = Div(d0d2, sum, 8)
  PredWeight[PtIdx][0] = 256 − PredWeight[PtIdx][1] − PredWeight[PtIdx][2]
}

      - Blending of predictor weights

When a point has three predictors in its predictor set and pred_blending_enabled is 1, the predictor weights shall be blended according to the distance between the predicting points.

The squared distance between each of the three predictors shall be determined:

distA := Norm2(PredPtIdx[PtIdx][0], PredPtIdx[PtIdx][1], PredPtRef[PtIdx][0],
  PredPtRef[PtIdx][1])
distB := Norm2(PredPtIdx[PtIdx][0], PredPtIdx[PtIdx][2], PredPtRef[PtIdx][0],
  PredPtRef[PtIdx][2])
distC := Norm2(PredPtIdx[PtIdx][1], PredPtIdx[PtIdx][2], PredPtRef[PtIdx][1],
  PredPtRef[PtIdx][2])

Blending weights shall be selected according to distance:

b1 := distA ≤ distB ? 1 : 5
b2 := distA ≤ distC ? 5 : 1
b3 := distB ≤ distC ? 1 : 5

The predictor weights shall be blended and updated:

if (PredCnt[PtIdx] == 3 && pred_blending_enabled) {
  w0 = PredWeight[PtIdx][0]
  w1 = PredWeight[PtIdx][1]
  w2 = PredWeight[PtIdx][2]
  w0p = w0 × 10 + w1 × (6 − b2) + w2 × b3 >> 4
  w1p = w0 × b1 + w2 × (6 − b3) + w1 × 10 >> 4
  PredWeight[PtIdx][0] = w0p
  PredWeight[PtIdx][1] = w1p
  PredWeight[PtIdx][2] = 256 − w0p − w1p
}

      - Distance computation using the biased L2 norm (BiasedNorm2)

This subclause defines the function BiasedNorm2( ptIdxA, ptIdxB  , pcA, pcB) that is the weighted squared distance between two points.

When pcA is equal to 0, the parameter, the parameters ptIdxA is an AttrPos index.  When pcA is equal to 1, the parameter, the parameters ptIdxA is a RefAttrPos index.

When pcB is equal to 0, the parameter, the parameters ptIdxB is an AttrPos index.  When pcB is equal to 1, the parameter, the parameters ptIdxB is a RefAttrPos index.

The result of this function is specified by the expression BiasedNorm2.  The expression posA[ ptIdx ][ 𝑘 ] and posB[ ptIdx ][ 𝑘 ] represent the attribute coordinates used to calculate the distance: when lod_scalability_enabled is 1, coordinates shall be quantized according to the detail level.

BiasedNorm2(ptIdxA, ptIdxB, pcA, pcB) := dist2[0] + dist2[1] + dist2[2]
  where
    dist[k] := Abs(posA[ptIdxA][k] − posB[ptIdxB][k]) × PredBias[k]
    dist2[k] := dist[k] × dist[k]
    posA[ptIdx][k] := pcA ?
      RefAttrPos[ptIdxA][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxA][k] >> Lvl) << Lvl
      : AttrPos[ptIdxA][k])
    posB[ptIdx][k] := pcB ?
      RefAttrPos[ptIdxB][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxB][k] >> Lvl) << Lvl
      : AttrPos[ptIdxB][k])

      - Distance computation using the unbiased L2 norm (Norm2)

This subclause defines the function Norm2( ptIdxA, ptIdxB , pcA, pcB ) that is the squared distance between two points.

When pcA is equal to 0, the parameter, the parameters ptIdxA is an AttrPos index.  When pcA is equal to 1, the parameter, the parameters ptIdxA is a RefAttrPos index.

When pcB is equal to 0, the parameter, the parameters ptIdxB is an AttrPos index.  When pcB is equal to 1, the parameter, the parameters ptIdxB is a RefAttrPos index.

The result of this function is specified by the expression Norm2.  The expression posA[ ptIdx ][ 𝑘 ] and posB[ ptIdx ][ 𝑘 ] represent the attribute coordinates used to calculate the distance. when lod_scalability_enabled is 1, coordinates shall be quantized according to the detail level.

Norm2(ptIdxA, ptIdxB, pcA, pcB) := dist2[0] + dist2[1] + dist2[2]
  where
    dist[k] := Abs(posA[ptIdxA][k] − posB[ptIdxB][k])
    dist2[k] := dist[k] × dist[k]
    posA[ptIdx][k] := pcA ?
      RefAttrPos[ptIdxA][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxA][k] >> Lvl) << Lvl
      : AttrPos[ptIdxA][k])
    posB[ptIdx][k] := pcB ?
      RefAttrPos[ptIdxB][k]
      : (lod_scalability_enabled
      ? (AttrPos[ptIdxB][k] >> Lvl) << Lvl
      : AttrPos[ptIdxB][k])

      - Reduction of predictor set

Subclause 10.6.6.15 applies when prediction_with_distribution_enabled is 1.

When the size of the predictor set is larger than 3, the third point in the predictor set may be replaced by one of the following points in the predictor set according to their relative spatial locations.

The array PtDirection represents the relative direction of a point in the predictor set with respect to the current point.

The space around the current point is separated into eight octants indexed from 0 to 7 along the three axes using the current point as the origin.  The relative direction of a point in the predictor set with respect to the current point is defined as the index of the octant in which the point is located.

for (j = 0; j < PredCnt[PtIdx];j++)
  for (k = 0; k < 3; ++k) {
    dist = ((AttrPos[PtIdx][k] - AttrPos[PredPtIdx[PtIdx][j]][k])
    PtDirection[j] |= (dist >= 0) << k
  }

Strict opposite octants and loose opposite octants are defined as strictOpposite and looseOpposite, respectively, where strictOpposite[ 𝑖 ][ 𝑘 ] (or looseOpposite[ 𝑖 ][ 𝑘 ]) denote whether the k-th octant is a strict (or loose) opposite octant of the i-th octant (Table 54 and Table 55).

Table 54— Strictly opposite octants, strictOpposite[ 𝑖 ][ 𝑘 ]

| 𝑖 | 𝑘 |  |  |  |  |  |  |  | 𝑖 | 𝑘 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 4 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 5 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 6 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |
| 3 | 0 | 0 | 0 | 0 | 1 | 0 | 0 | 0 | 7 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Table 55— Loose opposite octants, looseOpposite[ 𝑖 ][ 𝑘 ]

| 𝑖 | 𝑘 |  |  |  |  |  |  |  | 𝑖 | 𝑘 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| 0 | 0 | 0 | 0 | 1 | 0 | 1 | 1 | 0 | 4 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 | 1 | 5 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 0 |
| 2 | 0 | 1 | 0 | 0 | 1 | 0 | 0 | 1 | 6 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 0 |
| 3 | 1 | 0 | 0 | 0 |  | 1 | 1 | 0 | 7 | 0 | 1 | 1 | 0 | 1 | 0 | 0 | 0 |

The first three points in the predictor set are referred to as initial predictor points.

The variables equal01, equal02 and equal12 indicate whether the pairs of initial predictor points are in the same octant.

equal01 = PtDirection[0] == PtDirection[1]
equal02 = PtDirection[0] == PtDirection[2]
equal12 = PtDirection[1] == PtDirection[2]

The predictor set needs no replacement if any two points of the initial predictor points are in strict opposite octants of each other.

Otherwise, the third point in the predictor set may be replaced by a candidate predictor point.  A distance threshold dist1 is derived below that is used in the determination of replacement of the third point.

dist1 = (BiasedNorm1(PtIdx, PredPtIdx[PtIdx][2]) × (pred_replace_weight_minus32 + 32))
  >> 5

The points in the predictor set that have index value 3 or more, and whose distance to the current point is less than dist1 are referred as candidate predictor points of the predictor set.If a candidate predictor point is at the strict opposite direction of the first or the second initial predictor point, that point is used to replace the third point in the predictor set.

for (i = 3; i < PredCnt[PtIdx]; i++){
  if (dist1 > BiasedNorm1(PtIdx, PredPtIdx[PtIdx][i]) 
    && (strictOpposite[i][0] || strictOpposite[i][1])){
    PredPtIdx[PtIdx][2] =  PredPtIdx[PtIdx][i]
    break
  }
}

If the third point in the predictor set is not replaced by the above process, and if the all the initial predictor points are in the same octant p, the third point in the predictor set is be replaced by a candidate point that is in loose opposite direction of octant p as below.

if(equal01 && (equal02 || equal12))
  for (i = 3; i < PredCnt[PtIdx]; i++){
    if (dist1 > BiasedNorm1(PtIdx, PredPtIdx[PtIdx][i]) && looseOpposite[i][0]){
      PredPtIdx[PtIdx][2] =  PredPtIdx[PtIdx][i]
      break
    }
  }

If the third point of the predictor set is not replaced by the above process, the replacement process continues.  If the first and second initial predictors are not in the same octant and not loose opposite octants, and if the third initial predictor is in the same octant as the first or the second predictor, a candidate predictor point that is not in the same octant as the first or the second initial predictor, that candidate predictor point is used to replace the third point in the predictor set.

if(¬equal01 && (equal02 || equal12) && ¬looseOpposite[0][1])
  for (i = 3; i < PredCnt[PtIdx]; i++){
    if(dist1 > BiasedNorm1(PtIdx, PredPtIdx[PtIdx][i]) 
      && PtDirection[i] ≠ PtDirection[0]
      && PtDirection[i] ≠ PtDirection[1]){
      PredPtIdx[PtIdx][2] =  PredPtIdx[PtIdx][i]
      break
    }
  }

If the third point in the predictor set is not replaced by the above process, the replacement process continues.  If the first and second initial predictors are in the same octant and the third initial predictor is not in loose opposite octant as the first predictor, a candidate predictor point that is at the loose opposition direction to the first initial predictor is used to replace the third point in the predictor set.

if(equal01 && ¬looseOpposite[0][2])
  for (i = 3; i < PredCnt[PtIdx] ; i++){
    if(dist2 > BiasedNorm1(PtIdx, PredPtIdx[PtIdx][i] && looseOpposite[i][0]){
      PredPtIdx[PtIdx][2] =  PredPtIdx[PtIdx][i]
      break
    }
  }

After the above processes, the size of the predictor set shall be reduced to three elements by discarding the furthest predictor.

PredCnt[PtIdx] = Min(3, PredCnt[PtIdx] + 1)

    - Reconstruction of attribute values

      - General process

Each detail level shall be processed in turn, proceeding from the coarsest to the finest level, according to attr_coding_type (10.6.7.3, 10.6.7.4).  The variable Lvl is the index of the current detail level.

for (Lvl = LodCnt − 1; Lvl ≥ 0; Lvl−−)
  … /* process a detail level */

      - Coefficient processing order within a detail level

Within a detail level, processing proceeds in coded coefficient order.  The variable PtIdx is the AttrPos index of the current coefficient.  The variable CoeffIdx is the AttrCoeff array index of the current coefficient.

for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[Lvl]; rfmtIdx++) {
  PtIdx = LodRfmtPtIdx[Lvl][rfmtIdx]
  CoeffIdx = LodPtCnt[Lvl] − rfmtIdx
  … /* process current coefficient */
}

      - Processing a detail level (attr_coding_type = 1)

When attr_coding_type is 1, the following operations shall be performed in turn for each coefficient in the coefficient processing order of the current detail level:

- Prediction mode information is decoded from an encoded coefficient tuple (10.6.8.1).  The result is the variable PredMode.

- The unencoded coefficient tuple is scaled (10.6.9.1) to produce transform coefficients.

- Transform coefficient components are divided by 256 with half-values rounded up.

for (c = 0; c < AttrDim; c++)
  PointAttr[PtIdx][c] = DivExp2Up(PointAttr[PtIdx][c], 8)

- Transform coefficient are scaled by the point quantization weights (10.6.13.3).

- Transform coefficient components are predicted using inter-component prediction (10.6.10.2) to form prediction residuals.

- The attribute value is predicted and combined with the prediction residual (10.6.12).

- The reconstructed attribute value is clipped.

for (c = 0; c < AttrDim; c++)
  PointAttr[PtIdx][c] = Clip3(0, AttrMaxVal, PointAttr[PtIdx][c])

      - Processing a detail level (attr_coding_type = 2)

When attr_coding_type is 2, the following operations shall be performed in turn, each over all the coefficients in the detail level:

- Coefficient tuples are scaled (10.6.9.1) to produce transform coefficients.

- Transform coefficient components are predicted using last-component prediction (10.6.10.3).

- Transform coefficients are weighted by transform coefficient weights (10.6.11.4).

If Lvl is less than LodCnt − 1, the transform shall be applied (10.6.12):

- Attribute values predicted from the coarser detail level, Lvl + 1, are modified by the transform update operator (10.6.12.1).

- Attribute values corresponding to coefficients in the current detail level are predicted and combined with the scaled transform coefficient to produce the detail level output (10.6.12.1).

When Lvl is 0, the reconstructed attributes values shall be divided by 256 with half-values rounded away from zero and clipped to the maximum attribute value:

if (Lvl == 0)
  for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
    for (c = 0; c < AttrDim; c++)
      PointAttr[ptIdx][c] = Clip3(0, AttrMaxVal, DivExp2Fz(PointAttr[ptIdx][c], 8))

    - Prediction mode coding

      - General

Subclause 10.6.8 specifies the conditional coding of the prediction mode PredMode in coefficient tuples.  It applies when pred_direct_max_idx_plus1 is greater than zero; when pred_direct_max_idx_plus1 is 0, PredMode shall be 0.

A per-transform-coefficient test (10.6.8.2) shall be performed to determine whether the coefficient tuple encodes a prediction mode.  The result of the test is the variable PredModePresent.

If PredModePresent is:

- false, PredMode shall be 0;

- true, the coded prediction mode (PredModeCoded) shall be decoded according to the number of attribute components (10.6.8.3, 10.6.8.4) and the maximum codable prediction mode PredModeMax.  As a side-effect of decoding the prediction mode, the coefficient tuple (in AttrCoeff) is updated.

PredModeMax := pred_direct_max_idx_plus1 + ¬pred_direct_avg_disabled

The prediction mode shall be derived from the coded prediction mode:

PredMode = PredModePresent ? PredModeCoded + pred_direct_avg_disabled : 0

      - Presence of an encoded direct prediction mode

The derivation of PredModePresent specifies the presence of an encoded direct prediction mode:

- A direct prediction mode shall not be coded when disabled, or for refinement points with fewer than two predictors.

if (PredCnt[PtIdx] < 2 || pred_direct_max_idx_plus1 == 0)
  PredModePresent = 0

- Otherwise, a prediction mode shall be coded for a refinement point if, for any component, the absolute difference in attribute value between any of its predictors exceeds a bit-depth adjusted threshold.

for (ni = 0; ni < PredCnt[PtIdx]; ni++) {
  ptIdx  = PredPtIdx[PtIdx][ni]
  for (c = 0; c < AttrDim; c++) {
    minVal[c] = ni ? Min(minVal[c], PointAttr[ptIdx][c]) : PointAttr[ptIdx][c]
    maxVal[c] = ni ? Max(maxVal[c], PointAttr[ptIdx][c]) : PointAttr[ptIdx][c]
  }
}

maxDiff = 0
for (c = 0; c < AttrDim; c++)
  maxDiff = Max(maxDiff, maxVal[c] − minVal[c])

threshold = pred_direct_threshold << Max(0, AttrBitDepth − 8)
PredModePresent = maxDiff ≥ threshold

      - Decoding process for single component attributes

For single component attributes (AttrDim == 1), the prediction mode PredModeCoded is encoded by the LSBs of the coefficient magnitude:

PredModeCoded = 0
absCoeff = Abs(AttrCoeff[CoeffIdx][0])

if (PredModeMax == 4){
  PredModeCoded = absCoeff & 3
  absCoeff >>= 2
}

if (PredModeMax == 3) {
  PredModeCoded = absCoeff & 1
  absCoeff >>= 1
  if (PredModeCoded){
    PredModeCoded += absCoeff & 1
    absCoeff >>= 1
  }
}

if (PredModeMax == 2){
  PredModeCoded = absCoeff & 1
  absCoeff >>= 1
}

After decoding the prediction mode, the coefficients shall be updated.

AttrCoeff[CoeffIdx][0] = Sign(AttrCoeff[CoeffIdx][0]) × absCoeff

      - Decoding process for multi-component attributes

For multi-component attributes (AttrDim > 1), the prediction mode PredModeCoded is encoded by the LSB of the last two component's coefficient magnitude:

PredModeCoded = 0
absCoeffA = Abs(AttrCoeff[CoeffIdx][AttrDim − 2])
absCoeffB = Abs(AttrCoeff[CoeffIdx][AttrDim − 1])

if (PredModeMax == 4) {
  PredModeCoded = ((absCoeffA & 1) << 1) + (absCoeffB & 1)
  absCoeffA >>= 1
  absCoeffB >>= 1
}

if (PredModeMax == 3) {
  PredModeCoded = absCoeffA & 1
  absCoeffA >>= 1
  if (PredModeCoded) {
    PredModeCoded += absCoeffB & 1
    absCoeffB >>= 1
  }

if (PredModeMax == 2) {
  PredModeCoded = absCoeffA & 1
  absCoeffA >>= 1
}

After decoding the prediction mode, the coefficients shall be updated.

sgnCoeffA = Sign(AttrCoeff[CoeffIdx][AttrDim − 2])
sgnCoeffB = Sign(AttrCoeff[CoeffIdx][AttrDim − 1])

AttrCoeff[CoeffIdx][AttrDim − 2] = sgnCoeffA × absCoeffA
AttrCoeff[CoeffIdx][AttrDim − 1] = sgnCoeffB × absCoeffB

    - Scaling

      - Derivation of per-point QP

The QP for a point depends upon the detail level and its attribute coordinates as specified by the expression LodCoeffQp[ qc ] for a QP component qc:

- rgnOffset[ qc ] is the per-coefficient offset from the region-dependent QP offset tree.

- dpth is the depth of detail level in the LoD hierarchy.

LodCoeffQp[qc] := AttrQp[dpth][rgnOffset][Cidx > 0]
  where
    s := AttrPos[PtIdx][0]
    t := AttrPos[PtIdx][1]
    v := AttrPos[PtIdx][2]
    dpth := LodCnt − 1 − Lvl
    rgnOffset[qc] := AttrRegionQpOffset[s][t][v][qc]

      - Scaling by quantization step size

The coefficient tuple shall be scaled by the quantization step size (10.7.4) for the primary and secondary attribute components.

for (c = 0; c < AttrDim; c++)
  PointAttr[PtIdx][c] = AttrCoeff[CoeffIdx][c] × AttrQstep[LodCoeffQp[c > 0]]

    - Coefficient prediction

      - Syntax element semantics

last_comp_pred_coeff_diff[ dpth ] specifies in accordance with LastCompPredCoeff[ dpth ] the two-fractional-bit, fixed-point scale factor applied at depth dpth of the LoD hierarchy to second coefficient components to predict third coefficient components.  The syntax element codes the scale factor relative to LastCompPredCoeffPrev[ dpth ].

LastCompPredCoeff[dpth] := last_comp_pred_enabled
  ? LastCompPredCoeffPrev[dpth] + last_comp_pred_coeff_diff[dpth]
  : 0

LastCompPredCoeffPrev[dpth] := dpth == 0 ? 4 : LastCompPredCoeff[dpth − 1]

It is a requirement of bitstream conformance that LastCompPredCoeff[ dpth ] shall be in the range −128 .. 127 for dpth ∈ 0 .. lod_max_levels_minus1.

inter_comp_pred_coeff_diff[ dpth ][ 𝑐 ] specifies in accordance with InterCompPredCoeff[ dpth ][ 𝑐 ] the two-fractional-bit, fixed-point scale factor applied at depth dpth of the LoD hierarchy to first coefficient components to predict 𝑐-th coefficient components.  The syntax element codes the scale factor relative to InterCompPredCoeffPrev[ dpth ][ 𝑐 ].

InterCompPredCoeff[dpth][c] := inter_comp_pred_enabled
  ? predCoeff + inter_comp_pred_coeff_diff[dpth][c]
  : 0

InterCompPredCoeffPrev[dpth][c] := dpth == 0 ? 4 : InterCompPredCoeff[dpth − 1][c]

It is a requirement of bitstream conformance that InterCompPredCoeff[ dpth ][ 𝑐 ] shall be in the range −128 .. 127 for dpth ∈ 0 .. lod_max_levels_minus1.

      - Inter-component prediction

When attr_coding_type is 1 and inter_comp_pred_enabled is 1, secondary attribute coefficient components are residuals to a prediction by the first scaled coefficient component.  The predicted value shall round the two fractional bits from the scale factor, with half-values rounded up.

for (c = 1; c < AttrDim; c++) {
  icpCoeff = InterCompPredCoeff[LodCnt − 1 − Lvl][c]
  PointAttr[PtIdx][c] += DivExp2Up(icpCoeff × PointAttr[PtIdx][0], 2)
}

      - Last component prediction

When attr_coding_type is 2 and last_comp_pred_enabled is 1, the third attribute coefficient component is, if present, a residual to a prediction by the second scaled coefficient component.  The predicted value shall round the two fractional bits from the scale factor towards negative infinity.

if (AttrDim == 3) {
  lcpCoeff = LastCompPredCoeff[LodCnt − 1 − Lvl]
  PointAttr[PtIdx][2] += DivExp2Floor(lcpCoeff × PointAttr[PtIdx][1], 2)
}

    - Transform coefficient weights

      - General

Coefficient weights represent the relative significance of a coefficient.  Coefficients with larger weights have a greater influence on the decoded attribute values.

The array CoeffWeight is initialized by setting all elements to 256.

for (i = 0; i < PointCnt; i++)
  CoeffWeight[i] = 256

The derivation of coefficient weights depends upon whether LoD scalability is enabled.

      - Non-scalable case

When lod_scalability_enabled is 0, coefficient weights are calculated accumulatively, proceeding from the finest to the coarsest detail level.

The accumulated coefficient weight of each refinement point in a detail level shall be distributed to the points in its predictor set.  The distribution is proportional to the respective predictor weights:

for (lvl = 0; lvl < LodCnt − 1; lvl++)
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++) {
    ptIdx = LodRfmtPtIdx[lvl][rfmtIdx]
    coeffW = CoeffWeight[ptIdx]
    for (ni = 0; ni < PredCnt[ptIdx]; ni++) {
      if (!PredPtRef[ptIdx][ni]){
        predW = PredWeight[ptIdx][ni]
        CoeffWeight[PredPtIdx[ptIdx][ni]] += DivExp2Up(coeffW × predW, 8)
      }
    }
  }

      - Scalable case

When lod_scalability_enabled is 1, a single weight shall be assigned to all refinement points within a detail level:

for (lvl = 1; lvl < LodCnt − 1; lvl++) {
  weight = (slice_num_points_minus1 + 1) / LodPtCnt[lvl]
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++)
    CoeffWeight[LodRfmtPtIdx[lvl][rfmtIdx]] = weight × 256
}

      - Application to coefficient scaling

Transform coefficients shall be scaled by the integer reciprocal square root of their coefficient weight and divided by 236 with half-values rounded away from zero.

weight = IntRecipSqrt(CoeffWeight[PtIdx])
for (c = 0; c < AttrDim; c++)
  PointAttr[PtIdx][c] = DivExp2Fz(PointAttr[PtIdx][c] × weight, 36)

    - Transform

      - Update operation

When attr_coding_type is 2, the transform update operator shall redistribute coefficient values to predicting points in the coarser detail level.

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  updateN[ptIdx] = updateD[ptIdx] = 0

for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[Lvl]; rfmtIdx++) {
  rfmtPtIdx = LodRfmtPtIdx[Lvl][rfmtIdx]
  coeffW = CoeffWeight[rfmtPtIdx]
  for (ni = 0; ni < PredCnt[rfmtPtIdx]; ni++) {
    if (!PredPtRef[ptIdx][ni]){
      nPtIdx = PredPtIdx[rfmtPtIdx][ni]
      nWeight = DivExp2Up(PredWeight[rfmtPtIdx][ni] × coeffW, 8)
      updateD[nPtIdx] += nWeight
      for (c = 0; c < AttrDim; c++)
        updateN[nPtIdx][c] += nWeight × PointAttr[rfmtPtIdx][c]
    }
  }
}

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  if (updateD[ptIdx])
    PointAttr[ptIdx] −= Div(updateN[ptIdx], updateD[ptIdx], 0)

      - Direct prediction

When attr_coding_type is 1 and PredMode for a refinement point is greater than zero, its value shall be predicted to be the same as the point with predictor set index PredMode − 1.  If the indicated predictor is invalid, prediction shall not be performed.

It is a requirement of bitstream conformance that PredMode shall be less than or equal to PredCnt[ CoeffIdx ].

if (PredMode && PredMode ≤ PredCnt[PtIdx])
  for (c = 0; c < AttrDim; c++)
    PointAttr[PtIdx][c] += PredPtRef[PtIdx][PredMode - 1] ? 
      RefPointAttr[PredPtIdx[PtIdx][PredMode − 1]][c] : 
      PointAttr[PredPtIdx[PtIdx][PredMode − 1]][c]

      - Average prediction

When attr_coding_type is 2 or PredMode for a refinement point is 0, the weighted average of the predictor set shall predict the value of the refinement point:

if (PredMode == 0)
  for (c = 0; c < AttrDim; c++) {
    sum = 0
    for (ni = 0; ni < PredCnt[PtIdx]; ni++)
      sum += PredWeight[PtIdx][ni] × PredPtRef[PtIdx][ni] ? 
        RefPointAttr[PredPtIdx[PtIdx][ni]][c] : 
        PointAttr[PredPtIdx[PtIdx][ni]][c]
    PointAttr[PtIdx][c] += DivExp2Fz(sum, 8)
  }

    - Point quantization weights

      - General

Point quantization weights represent the predictive relationships between points.  Points with larger quantization weights have a greater influence on the prediction process.

The array QuantWeight is initialized by setting all elements to 256.

for (i = 0; i < PointCnt; i++)
  QuantWeight[i] = 256

      - Quantization weights derivation

Quantization weights are calculated accumulatively, proceeding from the finest to the coarsest detail level.  The quantization weight of each point is updated by the points in its predictor set.  The weights of the points in the predictor set to update the quantization weights are specified by quant_neigh_weight:

for (lvl = 0; lvl < LodCnt − 1; lvl++)
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++) {
    ptIdx = LodRfmtPtIdx[lvl][rfmtIdx]
    coeffW = QuantWeight[ptIdx]
    for (ni = 0; ni < PredCnt[ptIdx]; ni++) {
      if (!PredPtRef[ptIdx][ni]){
        QuantWeight[PredPtIdx[ptIdx][ni]] +=
          DivExp2Inf(coeffW × quant_neigh_weight[ni], 8)
      }
    }
  }

      - Application to coefficient scaling

Transform coefficients shall be scaled by quantization step size (10.7.4) or quantization weights.

weight = Min(QuantWeight[PtIdx], AttrQstep[qp]) >> 8
for (c = 0; c < AttrDim; c++)
  PointAttr[PtIdx][c] /= weight

  - Attribute quantization parameters

    - Syntax element semantics

attr_qp_offset[ qc ] specifies per-slice offsets used to derive QPs for the primary (qc = 0) and any secondary (qc = 1) attribute components.  When attr_qp_offset[ qc ] is not present, it shall be inferred to be 0.

attr_qp_layers_present specifies whether (when 1) or not (when 0) per-transform-layer QP offsets are present in the ADU.

attr_qp_layer_cnt_minus1 plus 1 specifies, when present, the number of levels in the LoD hierarchy or RAHT tree for which QP offsets are signalled.

attr_qp_layer_offset[ dpth ][ qc ] specifies QP offsets used for the primary (qc = 0) and any secondary (qc = 1) attribute components.  Each offset applies to transform coefficients at depth dpth of the LoD hierarchy or RAHT tree.  If the LoD hierarchy or RAHT tree has a greater number of levels than attr_qp_layer_cnt_minus1 + 1, attr_qp_layer_offset[ attr_qp_layer_cnt_minus1 ][ qc ] also specifies the QP offsets for transform coefficients at a depth greater than attr_qp_layer_cnt_minus1.

The expression AttrQpLayerOffset[ dpth ][ qc ] specifies the per layer QP offsets at depth dpth of the LoD hierarchy or RAHT tree.

AttrQpLayerOffset[dpth][qc] := attr_qp_layers_present > 0
  ? attr_qp_layer_offset[Min(attr_qp_layer_cnt_minus1, dpth)][qc]
  : 0

attr_qp_region_cnt specifies the number of spatial regions within the slice that have a region QP offset signalled.

- In profiles specified in this version of this document, all but the first region are ignored.

attr_qp_region_bits_minus1 plus 1 specifies the length in bits of each syntax element attr_qp_region_origin_xyz, attr_qp_region_size_minus1_xyz, attr_qp_region_origin_rpi and attr_qp_region_size_minus1_rpi.

attr_qp_region_origin_xyz[ 𝑖 ][ 𝑘 ] and attr_qp_region_size_minus1_xyz[ 𝑖 ][ 𝑘 ] specify, when present, the 𝑖-th spatial region in the slice where attr_qp_region_offset[ 𝑖 ][ qc ] applies.  The region is a bounding box in the slice coordinate system with lower corner XYZ coordinates attr_qp_region_origin_xyz[ 𝑖 ][ 𝑘 ] and dimensions attr_qp_region_size_minus1_xyz[ 𝑖 ][ 𝑘 ] + 1.

attr_qp_region_origin_rpi[ 𝑖 ][ 𝑘 ] and attr_qp_region_size_minus1_rpi[ 𝑖 ][ 𝑘 ] specify, when present, the 𝑖-th spatial region in the slice where attr_qp_region_offset[ 𝑖 ][ qc ] applies.  The region is a bounding box in the scaled angular coordinate system (10.2.2) used for attribute coding with lower corner RPI coordinates attr_qp_region_origin_rpi[ 𝑖 ][ 𝑘 ] and dimensions attr_qp_region_size_minus1_rpi[ 𝑖 ][ 𝑘 ] + 1.

The expressions AttrRegionQpOrigin[ 𝑖 ][ 𝑘 ] and AttrRegionQpSize[ 𝑖 ][ 𝑘 ] specify the 𝑘-th component of the bounding box origin and size for the 𝑖-th QP region in attribute coordinates.

AttrRegionQpOrigin[i][k] = attr_coord_conv_enabled
  ? attr_qp_region_origin_rpi[i][k]
  : attr_qp_region_origin_xyz[i][StvToXyz[k]]

AttrRegionQpSize[i][k] = attr_coord_conv_enabled
  ? attr_qp_region_size_minus1_rpi[i][k] + 1
  : attr_qp_region_size_minus1_xyz[i][StvToXyz[k]] + 1

A constraint on the bounds of a region is specified by expression AttrRegionSizeConstraint[ 𝑖 ][ 𝑘 ].  It is a requirement of bitstream conformance that AttrRegionSizeConstraint[ 𝑖 ][ 𝑘 ] shall be true for every component 𝑘 of each region 𝑖.

AttrRegionSizeConstraint[i][k] :=
  AttrRegionQpOrigin[i][k] + AttrRegionQpSize[i][k] < Exp2(MaxSliceDimLog2)

attr_qp_region_offset[ 𝑖 ][ qc ] specifies offsets used to derive the QPs for the primary (qc = 0) and any secondary (qc = 1) attribute components of points positioned within the region defined by AttrRegionQpOrigin[ 𝑖 ] and AttrRegionQpSize[ 𝑖 ].  When attr_qp_region_offset[ 𝑖 ][ qc ] is not present, it shall be inferred to be 0.

attr_AC_qp_offset_present specifies whether (when 1) or not (when 0) per-RAHT-layer AC transform coefficient QP offsets are present in the ADU.  When attr_AC_qp_offset_present is not present, it shall be inferred to be 0.

attr_AC_qp_layer_cnt_minus1 plus 1 specifies, when present, the number of levels in the  RAHT tree for which AC QP offsets are present in the ADU.  When attr_AC_qp_layer_cnt_minus1 is not present, it shall be inferred to be -1.

attr_AC_qp_offset[ dpth ][ qc][ACcompidx] specifies the per AC coefficient (ACcompidx = 0,1,..6) QP offsets for the primary (qc = 0) and any secondary (qc = 1) attribute components at depth dpth of the RAHT tree.

    - Per-point regional QP offset

The region-dependent QP offset for a point with attribute coordinates ( 𝑠, 𝑡, 𝑣 ) is specified by the expression AttrRegionQpOffset[ 𝑠 ][ 𝑡 ][ 𝑣 ][ qc ].

AttrRegionQpOffset[s][t][v][qc] :=
  isPointInRegion[0][s][t][v] ? attr_qp_region_offset[0][qc] : 0

The expression isPointInRegion[ rgnIdx ][ 𝑠 ][ 𝑡 ][ 𝑣 ] specifies whether the coordinates ( 𝑠, 𝑡, 𝑣 ) are within the rgnIdx-th region.

isPointInRegion[rgnIdx][s][t][v] :=
  rIdx < attr_qp_region_cnt
  && s ≥ AttrRegionQpOrigin[rIdx][0] && s < regionEnd[0]
  && t ≥ AttrRegionQpOrigin[rIdx][1] && t < regionEnd[1]
  && v ≥ AttrRegionQpOrigin[rIdx][2] && v < regionEnd[2]
  where
    regionEnd[k] := AttrRegionQpOrigin[rIdx][k] + AttrRegionQpSize[rIdx][k]

    - Attribute coefficient QP

An attribute coefficient QP is specified by the expression AttrQp[ dpth ][ rgnOffset ][ qc ], parameterized by:

- dpth, the depth of a level in the LoD hierarchy or RAHT tree;

- rgnOffset, an expression that when applied to an argument qc is a region-dependent QP offset; and

- qc, indicating a primary or secondary QP component.

The expressions qpP and qpS are the QPs for the primary and secondary attribute components.  Attribute QPs shall be clipped to the bit-depth dependent range [ 4, qpMax ].

AttrQp[dpth][rgnOffset][qc] := qc == 0 ? qpP : qpS
  where
    qpMax := 51 + 6 × (AttrBitDepth − 8)
    qpP := Clip3(4, qpMax,       attr_primary_qp_minus4 + 4 + qpOffset[qc])
    qpS := Clip3(4, qpMax, qpP + attr_secondary_qp_offset + qpOffset[qc])
    qpOffset[qc] := attr_qp_offset[qc] + AttrQpLayerOffset[dpth][qc] + rgnOffset[qc]

    - Definition of AttrQstep

This subclause specifies the expression AttrQstep[ qp ] that is the attribute scale factor for the attribute quantization parameter qp.

AttrQstep[qp] := AttrLevelScale[qp % 6] << (qp / 6)

Values of AttrLevelScale are specified by Table 56.

Table 56 — Values of AttrLevelScale[ 𝑖 ]

| 𝑖 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| AttrLevelScale[ 𝑖 ] | 161 | 181 | 203 | 228 | 256 | 287 |

  - Reference slice generation for attribute

    - General

Subclause 10.8 applies when attr_inter_prediction_enabled is equal to 1.

After the decoding of a frame, subclause 10.8.3 applies to generate the reference frame for the attribute coding of the next point cloud frame to be decoded.

After the generation of point coordinates of the current slice as specified by subclause 10.2, subclause 10.8.4 applies to generate the reference slice.

The output of this process are the reference slice attribute and the reference slice geometry for attribute coding.  The expression RefPointAttr[ptIdx][c] specifies the attribute values of each point in the reference slice.  The expression RefAttrPos[ptIdx][k] specifies the coordinates of each point for attribute coding in the reference slice.

    - Reconstructed frame generation

The coordinates for attribute coding of the previously reconstructed point cloud frame are stored in the array CloudAttrPos[ptIdx][k].  AttrPos[ptIdx][k] is an alias into the geometry coordinates array for attribute coding for the points in the slice.

AttrPos[ptIdx][k] := 
CloudAttrPos[RecCloudPointCnt + ptIdx][k]

The coordinates of points for attribute coding shall be generated as specified by subclause 10.2.

    - Reference frame generation

The geometry coordinates for attribute coding of the reference frame are stored in the array RefCloudAttrPos[ptIdx][k].  The attributes of the reference frame are stored in the array RefCloudAttr[ptIdx][k].  The number of points in the reference frame is specified by the variable refCloudPointCnt.

- When biprediction_enabled is 0, subclause 10.8.3.1 applies to generate the reference frame for uni-prediction.

- Otherwise, subclause 10.8.3.2 applies to generate the reference frame for bi-prediction.

The expression RefCloudAttrBboxOrigin[ k ] is an alias for the 𝑘-th XYZ coordinate of the origin of the attribute reference frame RefCloudAttrPos.  RefCloudAttrBboxSize[ k ] is an alias for the 𝑘-th XYZ component of the volume dimensions of the attribute reference frame RefCloudAttrPos.

      - Reference frame for uni-prediction

When geom_tree_type is 0, global_motion_enabled is equal to 0 or attr_coord_conv_enabled is equal to 1, RefCloudAttrPos is set equal to CloudAttrPos, and RefCloudAttr is set equal to RecCloudAttr.  When geom_tree_type is 0, global_motion_enabled is equal to 1 and attr_coord_conv_enabled is equal to 0, RefCloudAttrPos is set equal to GMPointCloud (GMPointCloud is derived by invoking subclause 9.2.15.2) and RefCloudAttr is set equal to RecCloudAttr.

When geom_tree_type is 1 and attr_coord_conv_enabled is equal to 1, RefCloudAttrPos and RecCloudAttr are derived as follows:

ptIdx = 0
for(beamId = 0; beamId <= num_beams_minus1; beamId++)
  for(qAzim = MinQAzim; qAzim <= MaxQAzim; qAzim++)
    for(j = 0; j <= maxPointsPerEntryMinus1; j++)
      if (PtnCurrFramePos[beamId][qAzim][j][0] ¬= -1) {
        for(k = 0; k < 3; k ++)
          RefCloudAttrPos[ptIdx][k] = PtnCurrFramePos[beamId][qAzim][j][k]
        RefCloudAttr[ptIdx][0] = PtnCurrFramePos[beamId][qAzim][j][0]
        ptIdx++
      }
refCloudPointCnt = ptIdx

      - Reference frame for bi-prediction

When biprediction_enabled is 1 or 2,  the geometry coordinates for attribute coding CloudAttrPos and the reconstructed attributes RecCloudAttr are indicated by the notional frame counter to be used as the reference frame for subsequent point cloud frames.

The geometry coordinates for attribute coding of the first original reference frame are stored in the array RefCloudAttrPosFirst[ptIdx][k].  The attributes of the first original reference original frame are stored in the array  RefCloudAttrFirst[ptIdx][k].  The number of points in the first original reference frame is specifies by the variable refCloudPointCntFirst.  The geometry coordinates for attribute coding of the second original reference frame are stored in the array RefCloudAttrPosSecond[ptIdx][k].  The attributes of the second original reference frame are stored in the array  RefCloudAttrSecond[ptIdx][k].  The number of points in the second original reference frame is specifies by the variable refCloudPointCntSecond.

The two original reference frames are determined as follows:

- when slice_biprediction is 0, RefCloudAttrPosFirst and RefCloudAttrFirst are set to the geometry coordinates for attribute coding and the reconstructed attributes of the previously reconstructed I-frame or P-frame;

- when biprediction_enabled is 1 and slice_biprediction is 1, RefCloudAttrPosSecond and RefCloudAttrSecond are set to the geometry coordinates for attribute coding and the reconstructed attributes of the previously reconstructed I-frame or P-frame.

  - If the previously reconstructed point cloud frame is a B-frame, RefCloudAttrPosFirst and RefCloudAttrFirst are set to the geometry coordinates for attribute coding and the reconstructed attributes of the previously reconstructed B-frame;

  - otherwise, RefCloudAttrPosFirst and RefCloudAttrFirst are set to the geometry coordinates for attribute coding and the reconstructed attributes of the previously reconstructed I-frame or P-frame.

- when biprediction_enabled is 2 and slice_biprediction is 1, RefCloudAttrPosFirst, RefCloudAttrFirst, RefCloudAttrPosSecond and RefCloudAttrSecond are set to the geometry coordinates for attribute coding and the reconstructed attributes of the reference frames determined by invoking subclause 9.2.15.3.

RefCloudAttrPos and RefCloudAttr are determined as follows:

- when slice_biprediction is 1, slice_attr_inter_prediction2 is 1 and slice_attr_inter_prediction is 0, RefCloudAttrPos is set equal to RefCloudAttrPosSecond, and RefCloudAttr is set equal to RefCloudAttrSecond;

- when slice_biprediction is 1, slice_attr_inter_prediction2 is 1 and slice_attr_inter_prediction is 1, the reference frame is set equal to the fusion of the two original reference frames:

refCloudPointCnt = refCloudPointCntFirst + refCloudPointCntSecond
for (i = 0; i < refCloudPointCntFirst; i++){
  for (c = 0; c < AttrDim; c++)
    RefCloudAttr[i][c] = RefCloudAttrFirst[i][c]
  for (k = 0; k < 3; k++)
    RefCloudAttrPos[i][k] = RefCloudAttrPosFirst[i][k]
}
for (i = 0; i < refCloudPointCntSecond; i++){
  for (c = 0; c < AttrDim; c++)
    RefCloudAttr[i + refCloudPointCntFirst][c] = RefCloudAttrSecond[i][c]
  for (k = 0; k < 3; k++)
    RefCloudAttrPos[i + refCloudPointCntFirst][k] = RefCloudAttrPosSecond[i][k]
}

- otherwise, RefCloudAttrPos is set equal to RefCloudAttrPosFirst, and RefCloudAttr is set equal to RefCloudAttrFirst.

    - Reference slice generation

The attribute values and coordinates of the reference slice are derived based on the bounding box of the coordinates of the current slice.  The number of points in the reference slice is specified by the variable refPointCnt.

refPointCnt = 0
for (i = 0; i < refCloudPointCnt; i++)
  if (ptInBox[i]){
    for (c = 0; c < AttrDim; c++)
      RefPointAttr[refPointCnt][c] = RefCloudAttr[i][c]
    for (k = 0; k < 3; k++)
      RefAttrPos[refPointCnt][k] = RefCloudAttrPos[i][k]
    refPointCnt++
  }
where,
  ptInBox[i] = pos[0] < bMax[0] && pos[0] > bMin[0] && pos[1] < bMax[1]
    && pos[1] > bMin[1] && pos[2] < bMax[2] && pos[2] > bMin[2]
  where,
    pos = RefCloudAttrPos[i]
    bMax = RefCloudAttrBboxOrigin + RefCloudAttrBboxSize
    bMin = RefCloudAttrBboxOrigin

Parsing process

  - General

Syntax elements are parsed according to the processes corresponding to the syntax element’s descriptor and name as specified in Table 57 to Table 59.

Table 57 — Descriptor parsing processes

| Descriptor | Parsing process | Arguments | Channel read method (readBit) |
| --- | --- | --- | --- |
| u(𝑛) | 11.4.1 | maxBins = 𝑛 | DuNextBit (11.2.5) |
| u(v) | 11.4.1 | See Table 58 | DuNextBit (11.2.5) |
| ue(v) | 11.4.3 | 𝑘 = 0 | DuNextBit (11.2.5) |
| s(𝑛) | 11.4.2 | maxBins = 𝑛 | DuNextBit (11.2.5) |
| s(v) | 11.4.2 | See Table 58 | DuNextBit (11.2.5) |
| se(v) | 11.4.3, 11.4.6 | 𝑘 = 0 | DuNextBit (11.2.5) |
| oid(v) | 11.4.7 |  | DuNextBit (11.2.5) |
| ae(v) | See Table 59 | See Table 59 | AeReadBin (11.5.2) |
| de(v) | 9.2.9 |  | na |

Table 58 — Syntax element specific parsing processes (non-ae(v))

| Syntax element | Parsing process | Arguments |
| --- | --- | --- |
| attr_coord_conv_scale[ ] | 11.4.1 (FL) | numBins = attr_coord_conv_scale_bits_minus1[ 𝑘 ] + 1 |
| attr_default_value[ attrIdx ][ ] | 11.4.1 (FL) | numBins = attr_bitdepth_minus1[ attrIdx ] + 1 |
| attr_offset[ ] | 11.4.2 (FL+S) | numBins = attr_offset_bits |
| attr_qp_region_origin_rpi[ ] | 11.4.1 (FL) | numBins = attr_qp_region_bits_minus1 + 1 |
| attr_qp_region_origin_xyz[ ] | 11.4.1 (FL) | numBins = attr_qp_region_bits_minus1 + 1 |
| attr_qp_region_size_minus1_rpi[ ] | 11.4.1 (FL) | numBins = attr_qp_region_bits_minus1 + 1 |
| attr_qp_region_size_minus1_xyz[ ] | 11.4.1 (FL) | nmBins = attr_qp_region_bits_minus1 + 1 |
| attr_scale_minus1[ ] | 11.4.1 (FL) | numBins = attr_scale_bits |
| defattr_value[ ] | 11.4.1 (FL) | numBins = AttrBitDepth |
| fbdu_frame_ctr_lsb | 11.4.1 (FL) | numBins = fbdu_frame_ctr_lsb_bits |
| frame_ctr_lsb | 11.4.1 (FL) | numBins = frame_ctr_lsb_bits |
| fsap_frame_ctr_lsb | 11.4.1 (FL) | numBins = fsap_frame_ctr_lsb_bits |
| gps_angular_origin_xyz[ ] | 11.4.1 (FL) | numBins = gps_angular_origin_bits_minus1 + 1 |
| raw_attr_value[ ][ ] | 11.4.1 (FL) | numBins = RawAttrValueBits |
| seq_bbox_size_minus1_xyz[ ] | 11.4.1 (FL) | numBins = seq_bbox_size_bits |
| seq_coded_scale_mantissa | 11.4.1 (FL) | numBins = seq_coded_scale_mantissa_bits |
| seq_origin_xyz[ ] | 11.4.2 (FL+S) | numBins = seq_origin_bits |
| slice_angular_origin_xyz[ ] | 11.4.2 (FL+S) | numBins = slice_angular_origin_bits_minus1 + 1 |
| slice_geom_origin_xyz[ ] | 11.4.1 (FL) | numBins = slice_geom_origin_bits_minus1 + 1 |
| slice_tag | 11.4.1 (FL) | numBins = slice_tag_bits |
| ti_frame_ctr_lsb | 11.4.1 (FL) | numBins = ti_frame_ctr_lsb_bits |
| ti_origin_xyz[ ] | 11.4.2 (FL+S) | numBins = ti_origin_bits_minus1 + 1 |
| tile_id | 11.4.1 (FL+S) | numBins = tile_id_bits |
| tile_origin_xyz[ ][ ] | 11.4.2 (FL) | numBins = tile_origin_bits_minus1 + 1 |
| tile_size_minus1_xyz[ ][ ] | 11.4.1 (FL) | numBins = tile_size_bits_minus1 + 1 |
| subgroup_bbox_origin_xyz[ ] | 11.4.2 (FL) | numBins = subgroup_bbox_origin__bits_minus1 + 1 |
| subgroup_bbox_size_xyz[ ] | 11.4.1 (FL) | numBins = subgroup_bbox_size__bits_minus1 + 1 |

Table 59 — Syntax element specific parsing processes (ae(v))

| Syntax element | Parsing process | Arguments |
| --- | --- | --- |
| beam_idx_resid_abs[ ] | 11.4.4 (TU+EGk) | maxOffset = 3, 𝑘 = 1 |
| beam_idx_resid_sign[ ] | 11.4.1 (FL) | numBins = 1 |
| coeff_abs | 11.4.4 (TU+EGk) | maxOffset = 2, 𝑘 = 1 |
| coeff_sign | 11.4.1 (FL) | numBins = 1 |
| direct_dup_point_cnt | 11.4.4 (TU+EGk) | maxOffset = 2, 𝑘 = 0 |
| direct_joint_diff_bit[ ] | 11.4.1 (FL) | numBins = 1 |
| direct_joint_prefix[ ] | 9.2.12.6 |  |
| direct_point_cnt_eq2 | 11.4.1 (FL) | numBins = 1 |
| direct_rem_st_ang[ ] | 11.4.1 (FL) | numBins = DnRemAngBitsST (9.2.13.8.3) |
| direct_rem_v_ang[ ] | 11.4.1 (FL) | numBins = DnRemAngBitsV (9.2.13.8.3) |
| direct_rem[ 𝑘 ] | 11.4.1 (FL) | numBins = DnRemBits[ 𝑘 ] (9.2.12.4.5) |
| direct_v_ang_resid_abs[ ] | 11.4.4 (TU+EGk) | maxOffset = 3, 𝑘 = 1 |
| direct_v_ang_resid_sign[ ] | 11.4.1 (FL) | numBins = 1 |
| occ_direct_node | 11.4.1 (FL) | numBins = 1 |
| occ_dup_point_cnt[ ] | 11.4.4 (TU+EGk) | maxOffset = 1, 𝑘 = 0 |
| occ_histogram_hit | 11.4.1 (FL) | numBins = 1 |
| occ_histogram_index | 11.4.1 (FL) | numBins = 5 |
| planar_copy_mode | 11.4.1 (FL) | numBins = 1 |
| occ_plane_pos[ ] | 11.4.1 (FL) | numBins = 1 |
| occ_recent_hit | 11.4.1 (FL) | numBins = 1 |
| occ_recent_index | 11.4.1 (FL) | numBins = 4 |
| occ_single_child | 11.4.1 (FL) | numBins = 1 |
| occ_single_plane[ ] | 11.4.1 (FL) | numBins = 1 |
| occ_subtree_qp_offset_abs | 11.4.4 (TU+EGk) | maxOffset = 1, 𝑘 = 0 |
| occ_subtree_qp_offset_present | 11.4.1 (FL) | numBins = 1 |
| occ_subtree_qp_offset_sign | 11.4.1 (FL) | numBins = 1 |
| occ_symbol_escape | 11.4.1 (FL) | numBins = 8 |
| occtree_end_of_entropy_stream | 11.4.1 (FL) | numBins = 1 |
| occupancy_bit[ ] | 11.4.1 (FL) | numBins = 1 |
| occupancy_byte | 9.2.9 |  |
| occupancy_idx[ ] | 11.4.1 (FL) | numBins = 1 |
| gm_comp_partition_block[] | 11.4.1 (FL) | numBins = 1 |
| ptn_child_cnt_xor1[ ] | 11.4.5 (TU) | maxVal = 3 |
| ptn_dup_point_cnt | 11.4.4 (TU+EGk) | maxOffset = 1, 𝑘 = 0 |
| ptn_phi_mul_abs_minus2 | 11.4.1 (FL) | numBins = 3 |
| ptn_phi_mul_abs_minus9 | 11.4.2 (EGk) | 𝑘 = 0 |
| ptn_phi_mul_abs_prefix | 11.4.5 (TU) | maxVal = 2 |
| ptn_phi_mul_sign | 11.4.1 (FL) | numBins = 1 |
| ptn_radius_resid_abs | 11.4.1 (TU+EGk) | maxOffset = 3, 𝑘 = 2 |
| ptn_radius_resid_sign | 11.4.1 (FL) | numBins = 1 |
| ptn_phi_resid_abs_gt0 | 11.4.1 (FL) | numBins = 1 |
| ptn_phi_resid_sign | 11.4.1 (FL) | numBins = 1 |
| ptn_phi_resid_abs_gt1 | 11.4.1 (FL) | numBins = 1 |
| ptn_phi_resid_abs_rem | 11.4.2 (EGk) | 𝑘 = 1 |
| ptn_inter_flag | 11.4.1 (FL) | numBins = 1 |
| ptn_pred_direction | 11.4.1 (FL) | numBins = 1 |
| ptn_inter_pred_mode | 11.4.1 (FL) | numBins = global_motion_enabled ? 2: 1 |
| ptn_pred_mode[ ] | 11.4.1 (FL) | numBins = 2 |
| ptn_pred_idx[ ] | 11.4.1 (TU) | maxVal = ptree_ang_max_pred_index |
| ptn_qp_offset_abs | 11.4.4 (TU+EGk) | maxOffset = 1, 𝑘 = 0 |
| ptn_qp_offset_sign | 11.4.1 (FL) | numBins = 1 |
| ptn_resid_abs_gt0[ ] | 11.4.1 (FL) | numBins = 1 |
| ptn_resid_abs_log2[ 𝑘 ] | 11.4.1 (FL) | numBins = ptn_resid_abs_log2_bits[ 𝑘 ] |
| ptn_resid_abs_rem[ 𝑘 ] | 11.4.1 (FL) | numBins = ptn_resid_abs_log2[ 𝑘 ] − 1 |
| ptn_resid_sign[ ] | 11.4.1 (FL) | numBins = 1 |
| ptn_sec_resid_abs[ ] | 11.4.4 (TU+EGk) | maxOffset = 2, 𝑘 = 0 |
| ptn_sec_resid_sign[ ] | 11.4.1 (FL) | numBins = 1 |
| ptree_end_of_slice | 11.4.1 (FL) | numBins = 1 |
| zero_run_length_prefix | 11.4.5 (TU) | maxVal = 3 |
| zero_run_length_minus3_div2 | 11.4.5 (TU) | maxVal = 4 |
| zero_run_length_minus3_mod2 | 11.4.1 (FL) | numBins = 1 |
| zero_run_length_minus11 | 11.4.2 (EGk) | 𝑘 = 2 |

  - Data unit buffer

    - General

The parsing of syntax elements is specified as operations on a DU buffer.  The DU buffer represents the coded DU as a sequence of unencapsulated bytes as provided by an encapsulation format such as that specified by Annex B or by another application-specific means.

    - State

The DU buffer is specified in terms of the following state variables:

- The array DataUnitBytes, representing the DU buffer; DataUnitBytes[ 𝑖 ] is the 𝑖-th byte of the data unit.

- The variable DataUnitLength, equal to the length of the DU in bytes.

- The variable DataUnitReadIdx, equal to the byte index and bit position of the next bit to be read from the DU buffer.

    - Initialization at the start of parsing a data unit

At the start of every DU, parsing shall commence at the first bit of the DU buffer.

DataUnitReadIdx = 0

    - Initialization at the start of parsing a geometry data unit footer

The parsing of a geometry_data_unit_footer syntax structure shall commence at an offset from the end of the DU buffer.  The length of the GDU footer is specified by the expression DuFooterLen.  The expression DuIsGdu is equal to 1 when the DU is a GDU.

GduFooterLen := 3 × (1 + occtree_point_cnt_list_present × occtreeMaxDepthMinus1)

DuFooterLen := DuIsGdu ? GduFooterLen : 0
DataUnitReadIdx = 8 × (DataUnitLength − DuFooterLen)

    - Definition of DuNextBit

This subclause specifies the reading of a single bit from the DU buffer by the expression DuNextBit.  Each evaluation of DuNextBit returns the next unread bit from the buffer.

duStreamByte[bitIdx] := DataUnitBytes[bitIdx >> 8]
duStreamBit[bitIdx] := Bit(duStreamByte[bitIdx], 7 − (bitIdx & 7))

DuNextBit := duStreamBit[DataUnitReadIdx++]

  - Chunked bytestream parsing

    - General

This subclause applies to GDUs and ADUs that contain syntax elements with ae(v) descriptors when bypass_stream_enabled is 1.

- An ADU with attr_coding_type equal to 3 does not contain any ae(v) syntax elements.

The CBS representation conveys two multiplexed data streams as a sequence of chunks: a stream of arithmetic-coded bytes (AeBits) and a stream of bits that bypass the arithmetic decoding engine (BpBits).  Every chunk is a block of 256 bytes, with the exception of the final chunk which may be shorter.

An example CBS is illustrated in Figure 34.  It starts with two ChunkLen length chunks.  From the CBS, two subtreams, AeBits and BpBits are extracted.

Figure 34 — Multiplexed data streams in a chunked bytestream.

When occtree_stream_cnt_minus1 is greater than 0, each of an occupancy tree's entropy streams shall be conveyed in a separate CBS (11.3.12).  Consecutive CBSs shall be spliced together (11.3.11) such that the last chunk of each CBS is merged with the first chunk of the next.  Splicing pads the last chunk of a CBS to 256 bytes.

    - Chunk syntax

| ae_chunk( ) { | Descriptor |
| --- | --- |
| chunk_ae_len | u(8) |
| for( 𝑖 = 0; 𝑖 < chunk_ae_len; 𝑖++ ) |  |
| chunk_ae_byte[ 𝑖 ] | u(8) |
| for( 𝑖 = 0; 𝑖 < ChunkPadLen; 𝑖++ ) |  |
| chunk_splice_byte[ 𝑖 ] |  |
| if( chunk_ae_len < ChunkLen − 1 − ChunkPadLen ) { |  |
| chunk_bypass_5bits | u(5) |
| chunk_bypass_flushed_bits | u(3) |
| } |  |
| for( 𝑖 = 0; 𝑖 < ChunkLen − 2 − chunk_ae_len − ChunkPadLen; 𝑖++) |  |
| chunk_bypass_byte[ 𝑖 ] | u(8) |
| } |  |

    - Chunk semantics

chunk_ae_len specifies the number of chunk_ae_byte syntax elements present in the chunk.  It is a requirement of bitstream conformance that chunk_ae_len shall be less than ChunkLen.

chunk_ae_byte[ 𝑖 ] specifies the 𝑖-th arithmetic-coded byte conveyed by the chunk.

chunk_splice_byte[ 𝑖 ] specifies a padding byte used to pad the last chunk of a CBS.  The padding bytes shall consist of bytes moved from the start of the next CBS.

chunk_bypass_byte[ 𝑖 ], chunk_bypass_5bits and chunk_bypass_flushed_bits together specify the bypass-coded bits conveyed by the chunk.  Within a chunk, the bits are in reverse order, as specified by the unpacking process (11.3.8).

    - State

The CBSs are specified in terms of the following state variables:

- The 256 byte array ChunkBuf, a buffer used to merge and unpack chunks.

- The array AeBits of unpacked arithmetic-coded bits; each element is a single bit.  The variable AeBitsLen is the length of the array.

- The array BpBits of unpacked bypass-coded bits; each element is a single bit.  The variable BpBitsLen is the length of the array.

- The variables AeBitsReadIdx and BpBitsReadIdx, indexes of the next element to be read from the AeBits and BpBits arrays, respectively.

    - Span of chunked bytestream data within a data unit

Each applicable DU comprises a header, the CBS data and a footer (if present).

The CBS data starts at the byte aligned position at or prior to the first ae(v) coded syntax element.

When a DU footer is present, the CBS data ends immediately prior to the first non-ae(v) coded syntax element of the fixed-length footer.  Otherwise, the end shall coincide with the end of the DU buffer.

The number of bytes remaining in the CBS data is specified by the expression ChunkDuRem.

ChunkDuRem := DataUnitLength − (DataUnitReadIdx >> 3) − DuFooterLen

    - The chunk buffer

Immediately prior to parsing a chunk, the chunk buffer is populated with the bytes of the next chunk from the CBS data span.

Unless 11.3.11 applies, the chunk buffer is populated by the next ChunkLen unparsed bytes from the DU buffer.  Every chunk shall be either 256 bytes in length, or as long as the remaining bytes in the CBS data span, whichever is shorter.

ChunkLen = Min(256, ChunkDuRem)
for (i = 0; i < ChunkLen; i++) {
  ChunkBuf[i] = DataUnitBytes[DataUnitReadIdx >> 3]
  DataUnitReadIdx += 8
}

    - State update at the start of every CBS

No unpacked data shall be preserved across CBSs.  Immediately prior to unpacking the first chunk of a CBS, the unpacked arithmetic- and bypass-coded bit buffers and their respective read positions shall be cleared.

AeBitsLen = BpBitsLen = 0
AeBitsReadIdx = BpBitsReadIdx = 0

    - Unpacking a single chunk

Unpacking a single chunk comprises parsing the contents of the chunk buffer and appending the per-stream data to the unpacked streams.  Parsing shall be performed according to the syntax and semantics of the ae_chunk syntax structure with ChunkPadLen assumed to be 0.

Any arithmetic-coded data are appended to the unpacked array AeBits.

for (i = 0; i < chunk_ae_len; i++)
  for (b = 7; b ≥ 0; b−−)
    AeBits[AeBitsLen++] = Bit(chunk_ae_byte[i], b)

Any bypass data are appended to the unpacked array BpBits.  Bypass data are appended in reverse order of the chunk data.  The last chunk_bypass_flushed_bits are excluded.

numChunkBypassBytes := Max(0, ChunkLen − 2 − chunk_ae_len)

for (j = numChunkBypassBytes − 1; j ≥ 0; j−−)
  for (b = 7; b ≥ 0; b−−)
    BpBits[BpBitsLen++] = Bit(chunk_bypass_byte[j], b)

for (b = 4; b ≥ 0; b−−)
  BpBits[BpBitsLen++] = Bit(chunk_bypass_5bits, b)
BpBitsLen −= chunk_bypass_flushed_bits

    - Definition of ChunkNextAeBit

This subclause specifies the reading of a single bit from the arithmetic-coded bitstream by the expression ChunkNextAeBit.  Each evaluation of the expression returns the next unread bit from the stream.

Prior to reading a bit from the stream, if there are no unread bits left in the stream buffer, subsequent chunks shall be unpacked as specified by 11.3.8 until an unread bit is available.

ChunkNextAeBit :=
  while (AeBitsReadIdx ≥ AeBitsLen) {
    … /* unpack chunk as specified by 11.3.6 */
  }
  ChunkNextAeBit = AeBits[AeBitsReadIdx++]

    - Definition of ChunkNextBpBit

This subclause specifies the reading of a single bit from the bypass-coded bitstream by the expression ChunkNextBpBit.  Each evaluation of the expression returns the next unread bit from the stream.

Prior to reading a bit from the stream, if there are no unread bits left in the stream buffer, subsequent chunks shall be unpacked as specified by 11.3.8 until an unread bit is available.

ChunkNextBpBit :=
  while (BpBitsReadIdx ≥ BpBitsLen) {
    … /* unpack chunk as specified by 11.3.6 */
  }
  ChunkNextBpBit = BpBits[BpBitsReadIdx++]

    - Boundary between spliced chunked bytestreams

This subclause applies when occtree_stream_cnt_minus1 is greater than 0.

Multiple CBSs shall be spliced to form a contiguous span of CBS data.  Splicing pads the last chunk of each CBS to maintain a fixed-length chunk size within that CBS.  To pad a CBSi the padding data shall consist of bytes moved from the start of the next CBS, CBSi+1.  If the length of CBSi+1 is less than 256 bytes, then it shall first be spliced with CBSi+2, if existent, prior to the splicing of CBSi with CBSi+1.

  - The definition of splicing is recursive.  For example, if the splicing of two CBSs A and B is denoted by A :: B, then splicing four CBSs, A to D, is performed as A :: ( B :: ( C :: D ) ).

  - The padding process permits the start of the bypass data in the last chunk of any CBS to be located after the parsing of that CBS has commenced.

Figure 35 — Extraction of the first chunk from a spliced CBS.

At the boundary between two CBSs, the padding data from the preceding CBS, CBSi shall form the initial part of the first chunk of the next CBS, CBSi+1, as illustrated by chunk_splice_byte[  ] in Figure 35.  The rest of the first chunk shall follow the last chunk of the preceding CBS.

The length of the padding data shall be derived from the unconsumed length of the bypass-coded bitstream specified by ChunkPadLen.  The length of padding data includes the bits used to code chunk_bypass_flushed_bits and the number of bits discarded.

ChunkPadLen = (BpBitsLen − BpBitsReadIdx + chunk_bypass_flushed_bits + 3) / 8

  - The initial parsing of the chunk (11.3.8) assumes that there are no padding bytes present.  For the last chunk in a CBS, this assumption might be wrong and the parsed values of chunk_bypass_5bits and chunk_bypass_flushed_bits are meaningless.

To recover the first chunk of the next CBS, the last chunk is re-parsed according to the syntax and semantics of the ae_chunk syntax structure with the determined value of ChunkPadLen.  Any padding data is moved to the start of the chunk buffer, and the remainder of the chunk buffer populated by the next unparsed bytes from the CBS data span.

for (i = 0; i < ChunkPadLen; i++)
  ChunkBuf[i] = chunk_splice_byte[i]

ChunkPartBLen := Min(256 − ChunkPadLen, ChunkDuRem)

ChunkLen = ChunkPadLen + ChunkPartBLen
for (i = ChunkPadLen; i < ChunkLen; i++) {
  ChunkBuf[i] = DataUnitBytes[DataUnitReadIdx >> 3]
  DataUnitReadidx += 8
}

After populating the chunk buffer, the parsing state shall be updated for the start of the next CBS (11.3.7) and the first chunk shall be unpacked (11.3.8).

    - Location of chunked bytestream boundaries

This subclause applies when occtree_stream_cnt_minus1 is greater than 0.

An additional CBS shall commence at the start of each occupancy_tree_level( dpth ) syntax structure where dpth is greater than OcctreeEntropyStreamDepth.

  - General inverse binarization processes

    - Parsing unsigned fixed-length codes (FL)

Parsing is parameterized by:

- numBits, the number of bits that represent the syntax element;

- readBit, the channel read method expression.

The result is the unsigned syntax element value PartValue, parsed and constructed as:

PartVal = 0
for (BinIdx = 0; BinIdx < numBits; BinIdx++)
  PartVal = (PartVal << 1) + readBit

    - Parsing signed fixed-length codes (FL+S)

Parsing is parameterized by:

- numBits, the number of bits that represent the absolute syntax element value;

- readBit, the channel read method expression.

The unsigned syntax element magnitude is parsed:

PartVal = 0
for (BinIdx = 0; BinIdx < numBits; BinIdx++)
  PartVal = (PartVal << 1) + readBit

The result is the signed syntax element value val, parsed and constructed as:

sign = readBit
val = sign ? −PartVal : PartVal

    - Parsing 𝑘-th order exp-Golomb codes (EGk)

Parsing is parameterized by:

- 𝑘, the order of the exp-Golomb code;

- readBit, the channel read method expression.

First, a unary encoded prefix is parsed as:

prefix = 0
for (BinIdxPfx = 0; readBit ≠ 0; BinIdxPfx++)
  prefix++

Then, a suffix comprising 𝑘 + prefix bins is parsed:

suffix = 0
for (BinIdxSfx = 0; BinIdxSfx < k + prefix; BinIdxSfx++)
  suffix = (suffix << 1) + readBit

The result is the unsigned syntax element value val, constructed as:

val = Exp2(prefix + k) + suffix − Exp2(k)

    - Parsing concatenated truncated unary and 𝑘-th order exp-Golomb codes (TU+EGk)

Parsing is parameterized by:

- maxOffset, the limit for the truncated unary offset encoding;

- 𝑘, the order of the exp-Golomb code;

- readBit, the channel read method expression.

First, a truncated unary encoded offset is parsed:

offset = 0
for (BinIdxTu = 0; offset < maxOffset && readBit == 1; BinIdxTu++)
  offset++

Second, if the value of offset is equal to maxOffset, a unary encoded prefix is parsed:

prefix = 0
if (offset == maxOffset)
  for (BinIdxPfx = 0; readBit ≠ 0; BinIdxPfx++)
    prefix++

Then, if the value of offset is equal to maxOffset, a suffix comprising 𝑘 + prefix bins is parsed:

suffix = 0
if (offset == maxOffset)
  for (BinIdxSfx = 0; BinIdxSfx < k + prefix; BinIdxSfx++)
    suffix = (suffix << 1) + readBit

The result is the unsigned syntax element value val, constructed as:

val = offset + Exp2(prefix + k) + suffix − Exp2(k)

    - Parsing truncated unary codes (TU)

Parsing is parameterized by:

- maxVal, the limit for the encoding;

- readBit, the channel read method expression.

The result is the unsigned syntax element value PartVal, parsed and constructed as:

PartVal = 0
for (BinIdxTu = 0; PartVal < maxVal && readBit == 1; BinIdxTu++)
  PartVal++

    - Mapping process for signed codes

The signed value of a syntax element parsed according to the descriptor se(v) shall be converted from its unsigned, parsed value.  If the parsed value val is:

- even, the signed syntax element value is − ( val >> 1 );

- odd, the signed syntax element value is val + 1 >> 1.

Examples of the conversion are shown in Table 60.

Table 60 — Conversion of unsigned values for signed syntax elements

| Unsigned value | 0 | 1 | 2 | 3 | 4 | 5 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Signed value | 0 | 1 | −1 | 2 | −2 | 3 | −3 |

    - Parsing ASN.1 object identifiers

      - Object identifier syntax

| oid( ) { | Descriptor |
| --- | --- |
| oid_forbidden_zero_bit | u(1) |
| oid_length | u(7) |
| for( 𝑖 = 0; 𝑖 < oid_length; 𝑖++ ) |  |
| oid_contents_octets[ 𝑖 ] | u(8) |
| } |  |

      - Object identifier semantics

The coded representation of an ASN.1 object identifier shall follow the ASN.1 distinguished encoding rules specified in Rec. ITU-T X.690﻿ |‌ ISO/IEC 8825-1.

oid_forbidden_zero_bit shall be 0.

oid_length specifies the number of octets present in oid_contents_octets.

oid_contents_octets[ 𝑖 ] is the 𝑖-th contents octet of an object identifier value encoding as specified in Rec. ITU-T X.690﻿ |‌ ISO/IEC 8825-1.

  - CABAC parsing processes

    - Initialization

The arithmetic decoding engine and CPMs shall be initialized according to 11.5.4.3 and 11.5.3.2 at the start of the following syntax structures:

- occupancy_tree (7.3.3.4);

- occupancy_tree_level( dpth ) (7.3.3.5) when dpth is greater than OcctreeEntropyStreamDepth;

- predictive_tree (7.3.3.8);

- attribute_coeffs (7.3.4.3).

    - Definition of AeReadBin

This subclause specifies the reading of a single arithmetic-coded bin as the expression AeReadBin.  Each evaluation reads a single bin, parameterized by the name of the coded syntax element.

A CPM identified by the expression Ctx shall be selected according to 11.5.3.4.

If the value of Ctx is neither equal to 'bypass' nor 'terminate':

- The value of the decoded bin shall be determined in accordance with 11.5.4.5 for a single arithmetic-coded bin with Ctx as the argument prob0.

- The selected CPM shall then be updated in accordance with 11.5.3.3 using the decoded bin value as the argument binVal.

If the value of Ctx is 'bypass', the value of the decoded bin shall be determined:

- When bypass_stream_enabled is 0, in accordance with 11.5.4.6 for an arithmetic-coded bypass bin.

- When bypass_stream_enabled is 1, by evaluating the expression ChunkNextBpBit (11.3.10).

If the value of Ctx is 'terminate':

- The arithmetic decoder shall be flushed in accordance with 11.5.4.8.

    - Contextual probability models

      - General

A CPM is a 16-bit unsigned integer value that models the probability of a zero bin.

- The values 0, 215 and 216 represent the probability of a zero bin as impossible, equiprobable and certain respectively.  The values 0 and 216 can never be attained due to the operation of the context update process.

The array Contexts, with elements Contexts[ ctxTbl ][ ctxIdx ], represents individual adaptive CPMs used by the CABAC parsing process.

      - Initialization

When slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1, initialization shall be performed by the parsing state restoration process (11.6).

Otherwise (slice_entropy_continuation is 0 and slice_inter_entropy_continuation is 0), all CPMs shall be initialized to 215.

      - Update after each coded bin

After each bin coded using an adaptive CPM, the modelled probability shall be updated.

The parameter binVal is the value of the coded bin and the expression Ctx identifies the CPM used to arithmetically code it (11.5.3.4).

The update shall increase or decrease the modelled probability of a zero-valued bin according to the known value of the coded bin, the upper eight bits of the modelled probability and the channel model specified by Table 61:

if (binVal)
  Ctx −= CtxUpdateDelta[Ctx >> 8]
else
  Ctx += CtxUpdateDelta[255 − (Ctx >> 8)]

Table 61 — Values of CtxUpdateDelta[ 𝑖 + 𝑗 ]

| 𝑗 | 𝑖 |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| 0 | 0 | 2 | 5 | 8 | 11 | 15 | 20 | 24 | 29 | 35 | 41 | 47 |
| 12 | 53 | 60 | 67 | 74 | 82 | 89 | 97 | 106 | 114 | 123 | 132 | 141 |
| 24 | 150 | 160 | 170 | 180 | 190 | 201 | 211 | 222 | 233 | 244 | 256 | 267 |
| 36 | 279 | 291 | 303 | 315 | 327 | 340 | 353 | 366 | 379 | 392 | 405 | 419 |
| 48 | 433 | 447 | 461 | 475 | 489 | 504 | 518 | 533 | 548 | 563 | 578 | 593 |
| 60 | 609 | 624 | 640 | 656 | 672 | 688 | 705 | 721 | 738 | 754 | 771 | 788 |
| 72 | 805 | 822 | 840 | 857 | 875 | 892 | 910 | 928 | 946 | 964 | 983 | 1 001 |
| 84 | 1 020 | 1 038 | 1 057 | 1 076 | 1 095 | 1 114 | 1 133 | 1 153 | 1 172 | 1 192 | 1 211 | 1 231 |
| 96 | 1 251 | 1 271 | 1 291 | 1 311 | 1 332 | 1 352 | 1 373 | 1 393 | 1 414 | 1 435 | 1 456 | 1 477 |
| 108 | 1 498 | 1 520 | 1 541 | 1 562 | 1 584 | 1 606 | 1 628 | 1 649 | 1 671 | 1 694 | 1 716 | 1 738 |
| 120 | 1 760 | 1 783 | 1 806 | 1 828 | 1 851 | 1 874 | 1 897 | 1 920 | 1 935 | 1 942 | 1 949 | 1 955 |
| 132 | 1 961 | 1 968 | 1 974 | 1 980 | 1 985 | 1 991 | 1 996 | 2 001 | 2 006 | 2 011 | 2 016 | 2 021 |
| 144 | 2 025 | 2 029 | 2 033 | 2 037 | 2 040 | 2 044 | 2 047 | 2 050 | 2 053 | 2 056 | 2 058 | 2 061 |
| 156 | 2 063 | 2 065 | 2 066 | 2 068 | 2 069 | 2 070 | 2 071 | 2 072 | 2 072 | 2 072 | 2 072 | 2 072 |
| 168 | 2 072 | 2 071 | 2 070 | 2 069 | 2 068 | 2 066 | 2 065 | 2 063 | 2 060 | 2 058 | 2 055 | 2 052 |
| 180 | 2 049 | 2 045 | 2 042 | 2 038 | 2 033 | 2 029 | 2 024 | 2 019 | 2 013 | 2 008 | 2 002 | 1 996 |
| 192 | 1 989 | 1 982 | 1 975 | 1 968 | 1 960 | 1 952 | 1 943 | 1 934 | 1 925 | 1 916 | 1 906 | 1 896 |
| 204 | 1 885 | 1 874 | 1 863 | 1 851 | 1 839 | 1 827 | 1 814 | 1 800 | 1 786 | 1 772 | 1 757 | 1 742 |
| 216 | 1 727 | 1 710 | 1 694 | 1 676 | 1 659 | 1 640 | 1 622 | 1 602 | 1 582 | 1 561 | 1 540 | 1 518 |
| 228 | 1 495 | 1 471 | 1 447 | 1 422 | 1 396 | 1 369 | 1 341 | 1 312 | 1 282 | 1 251 | 1 219 | 1 186 |
| 240 | 1 151 | 1 114 | 1 077 | 1 037 | 995 | 952 | 906 | 857 | 805 | 750 | 690 | 625 |
| 252 | 553 | 471 | 376 | 255 |  |  |  |  |  |  |  |  |

      - Selection

A CPM shall be selected for each bin of the coded syntax element as specified by the expression Ctx.  The values CtxTbl and CtxIdx shall be determined according to the entries for the syntax element in Table 62 (GDU) and Table 63 (ADU).  Entries qualified by Offset, Prefix or Suffix individually apply when selecting a CPM for a bin of that part of the binarized syntax element.

Ctx := CtxIdx ≠ 'bypass' && CtxIdx ≠ 'terminate' ? Contexts[CtxTbl][CtxIdx] : CtxIdx

Table 62 — Values of CtxTbl and CtxIdx for binarized ae(v) coded GDU syntax elements

| Syntax element | CtxTbl | CtxIdx |  | Count |
| --- | --- | --- | --- | --- |
| beam_idx_resid_abs[ ] | 1 | Offset | 3 × (BeamPrevIdxResid[ DnBeamIdxEst] != 0) + BinIdxTu | 6 |
|  |  | Prefix | 6 | 1 |
|  |  | Suffix | bypass | 0 |
| beam_idx_resid_sign[ ] | 2 | 2 × (BeamPrevIdxResid[DnBeamIdxEst] < 0) + (BeamPrevIdxResid[DnBeamIdxEst] > 0) |  | 3 |
| direct_dup_point_cnt | 3 | Offset | BinIdxTu | 2 |
|  |  | Prefix | 2 | 1 |
|  |  | Suffix | bypass | 0 |
| direct_joint_diff_bit[ ] | na | bypass |  | 0 |
| direct_joint_prefix[ 𝑘 ] | 4 | BinIdx & 1 ? 'bypass' : 5 × 𝑘 + Min( 4, BinIdx / 2 ) |  | 15 |
| direct_point_cnt_eq2 | 5 | 0 |  | 1 |
| direct_rem_st_ang[ ] | 6 | CtxIdxAngPhi (9.2.13.8.4) |  | 24 |
| direct_rem_v_ang[ ] | 7 | CtxIdxAngTheta (9.2.13.8.6) |  | 4 |
| direct_rem[ ] | na | bypass |  | 0 |
| direct_v_ang_resid_abs[ ] | 41 | Offset | BinIdxTu | 3 |
|  |  | Prefix | 3 | 1 |
|  |  | Suffix | bypass | 0 |
| direct_v_ang_resid_sign[ ] | 42 | 0 |  | 1 |
| occ_direct_node | 8 | 0 |  | 1 |
| occ_dup_point_cnt | 3 | Offset | 0 | 1 |
|  |  | Prefix | 2 | 1 |
|  |  | Suffix | bypass | 0 |
| occ_histogram_hit | 9 | NeighPatR |  | 9 |
| occ_histogram_index | 10 | CtxIdxDictHg (9.2.9.11) |  | 45 |
| planar_copy_mode | TBA | CtxIdxPlanarCopyMode (9.2.11.12) |  | 128 |
| occ_plane_pos[ ], when ¬AngularEligible | 11 | CtxIdxPlanePos (9.2.11.7) |  | 51 |
| occ_plane_pos[ 𝑘 ], when AngularEligble && 𝑘 < 2 | 12 | CtxIdxAngPhi (9.2.13.7.7) |  | 8 |
| occ_plane_pos[ 𝑘 ], when AngularEligble && 𝑘 == 2 | 13 | CtxIdxAngTheta (9.2.13.7.8) |  | 4 |
| occ_recent_hit | 14 | NeighPatR |  | 9 |
| occ_recent_index | na | bypass |  | 0 |
| occ_single_child | 15 | 0 |  | 1 |
| occ_single_plane[ 𝑘 ] | 16 | 𝑘 + PlaneRef[𝑘] ? (3 * PlanePosRef[𝑘] + 1 : 0) |  | 9 |
| occ_subtree_qp_offset_abs | 17 | Offset | 0 | 1 |
|  |  | Prefix | 1 | 1 |
|  |  | Suffix | bypass | 0 |
| occ_subtree_qp_offset_present | na | bypass |  | 0 |
| occ_subtree_qp_offset_sign | 18 | 0 |  | 1 |
| occ_symbol_escape | 19 | NeighPatR |  | 9 |
| occtree_end_of_entropy_stream | na | terminate |  | 0 |
| occupancy_bit[ ] | 20 | CtxIdxOccBit (9.2.10.6) |  | 32 |
| occupancy_idx[ ] | na | bypass |  | 0 |
| gm_comp_partition_block[] | 48 | 0 |  | 1 |
| ptn_child_cnt_xor1[ ] | 21 | BinIdxTu |  | 3 |
| ptn_dup_point_cnt | 22 | Offset | 0 | 1 |
|  |  | Prefix | 1 | 1 |
|  |  | Suffix | bypass | 0 |
| ptn_phi_mul_abs_minus2 | 23 | 16 × ptn_inter_flag[ nodeIdx ] + 8 x (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) + Exp2( BinIdx ) + PartVal − 1 |  | 32 |
| ptn_phi_mul_abs_minus9 | 24 | Prefix | 2 ×ptn_inter_flag[ nodeIdx ] + (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) | 4 |
|  |  | Suffix | bypass | 0 |
| ptn_phi_mul_abs_prefix | 25 | 4 × ptn_inter_flag[ nodeIdx ] + 2 x (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) + BinIdxTu |  | 8 |
| ptn_phi_mul_sign | 26 | 2 ×ptn_inter_flag[ nodeIdx ] + (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) |  | 4 |
| ptn_pred_mode[ ] | 27 | Exp2( BinIdx ) + PartVal − 1 |  | 3 |
| ptn_qp_offset_abs | 28 | Offset | 0 | 1 |
|  |  | Prefix | 1 | 1 |
|  |  | Suffix | bypass | 0 |
| ptn_qp_offset_sign | 29 | 0 |  | 1 |
| ptn_resid_abs_gt0[ 𝑘 ] | 30 | 𝑘 +3 × ptn_inter_flag[ nodeIdx ] |  | 3 |
| ptn_resid_abs_log2[ 𝑘 ], when 𝑘 == 0 ¦¦ geom_angular_enabled | 31 + 𝑘 | Exp2( BinIdx ) + PartVal – 1 +186 × ptn_inter_flag[ nodeIdx ] |  | 62 |
| ptn_resid_abs_log2[ 𝑘 ], when 𝑘 > 0 && ¬geom_angular_enabled | 31 + 𝑘 | Min( 4, ptn_resid_abs_log2[ 0 ] + 1 >> 1 ) × 31 + Exp2( BinIdx ) + PartVal − 1 |  | 155 |
| ptn_resid_abs_rem[ ] | na | bypass |  | 0 |
| ptn_resid_sign[ 𝑘 ] | 34 | 𝑘 +3 × ptn_inter_flag[ nodeIdx ] |  | 6 |
| ptn_sec_resid_abs[ 𝑘 ] | 35 | Offset | 𝑘 × BinIdxTu | 6 |
|  |  | Prefix | 6 + 9 × 𝑘 + Min( 4, BinIdxPfx ) | 15 |
|  |  | Suffix | 11 + 9 × 𝑘 + Min( 3, BinIdx Sfx) | 12 |
| ptn_sec_resid_sign[ 𝑘 ] | 36 | 𝑘 |  | 3 |
| ptree_end_of_slice | 37 | 0 |  | 1 |
| ptn_radius_resid_abs | 38 | Offset | ( ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) + 2 × (ptn_inter_flag[ nodeIdx ] ? Abs(PtnPhiMul[ nodeIdx ]) > 2 : Abs(PtnPhiMul[ nodeIdx ]) > thQphi) + 4 × BinIdxTu +12 × ptn_inter_flag[ nodeIdx ] | 24 |
|  |  | Prefix | 10 × ( ( ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] ≠ 0) + 2 × (ptn_inter_flag[ nodeIdx ] ? Abs(PtnPhiMul[ nodeIdx ]) > 2 : Abs(PtnPhiMul[ nodeIdx ] ) > thQphi) ) + Min( 9, BinIdxPfx ) + 40 × ptn_inter_flag[ nodeIdx ] | 80 |
|  |  | Suffix | 10 × ( ( ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] ≠ 0) + 2 × (ptn_inter_flag[ nodeIdx ] ? Abs(PtnPhiMul[ nodeIdx ]) > 2 : Abs(PtnPhiMul[ nodeIdx ]) > thQphi) ) + Min( 9, BinIdxSfx ) +40 × ptn_inter_flag[ nodeIdx ] | 80 |
| ptn_radius_resid_sign | 39 | PrevRadiusResidSign + 2 × (PtnPhiMul[ nodeIdx ] ≠ 0) + 4 × (PrevPhiMul ≠ 0) + 8 × (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) +16 × (ptn_inter_flag[ nodeIdx ] ? 2 : InterFlagHist & 0x1) |  | 48 |
| ptn_phi_resid_abs_gt0 | 41 | (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) + 2 × ptn_inter_flag[ nodeIdx ] |  | 4 |
| ptn_phi_resid_sign | 42 | (ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ( ptn_pred_idx[ nodeIdx ] == 0) ) × 5 + (ptn_inter_flag[ nodeIdx ] ? 4 : (InterFlagHist & 0x1 ? PrevInterFrameRefIdx + 2 : PrevPhiResidSign) ) |  | 10 |
| ptn_phi_resid_abs_gt1 | 43 | ( ptn_inter_flag[ nodeIdx ] ? ptn_inter_pred_mode[ nodeIdx ] > 1 : ptn_pred_idx[ nodeIdx ] == 0) + 2 × ptn_inter_flag[ nodeIdx ] |  | 4 |
| ptn_phi_resid_abs_rem | 44 | Prefix | Min( 3, BinIdxPfx ) + 4 × (ptn_inter_flag[ nodeIdx ] ? (ptn_inter_pred_mode[ nodeIdx ] > 1 ) + 1 : 0) | 24 |
|  |  | Suffix | Min( 3, BinIdx Sfx) + 4 × (ptn_inter_flag[ nodeIdx ] ? (ptn_inter_pred_mode[ nodeIdx ] > 1 ) + 1 : 0) | 24 |
| ptn_pred_idx | 45 | BinIdxTu |  | 7 |
| ptn_inter_flag | 46 | InterFlagHist & 1F |  | 32 |
| ptn_pred_direction | 49 | 0 |  | 1 |
| ptn_pred_inter_mode | 47 | Exp2( BinIdx ) + PartVal − 1 |  | 3 |
| - The syntax elements occ_dup_point_cnt and direct_dup_point_cnt use the same context table despite using different binarizations. |  |  |  |  |

Table 63 — Values of CtxTbl and CtxIdx for binarized ae(v) coded ADU syntax elements

| Syntax element | CtxTbl | CtxIdx |  | Count |
| --- | --- | --- | --- | --- |
| coeff_abs[ 0 ] when aps_extension_present == 0 | 38 | Offset | BinIdxTu | 2 |
|  |  | Prefix | 4 + Min( 4, BinIdx Pfx) | 3 |
|  |  | Suffix | 9 + Min( 2, BinIdx Sfx) | 3 |
| coeff_abs[ 1 ] when aps_extension_present == 0 | 38 | Offset | For BinIdxTu == 0: 2 + ( coeff_abs[ 0 ] ≠ 0 ) For BinIdxTu == 1: 4 + ( coeff_abs[ 0 ] ≤ 1 ) | 4 |
|  |  | Prefix | 4 + Min( 4, BinIdx Pfx ) | 3 |
|  |  | Suffix | 9 + Min( 2, BinIdxSfx ) | 3 |
| coeff_abs[ 2 ] when aps_extension_present == 0 | 39 | Offset | For BinIdxTu == 0: ( coeff_abs[ 0 ] ≠ 0 ) + 2 × ( coeff_abs[ 1 ] ≠ 0 ) For BinIdxTu == 1: 4 + ( coeff_abs[ 0 ] ≤ 1 ) + 2 × ( coeff_abs[ 1 ] ≤ 1 ) | 8 |
|  |  | Prefix | 6 + Min( 4, BinIdx Pfx ) | 3 |
|  |  | Suffix | 11 + Min( 2, BinIdx Sfx) | 3 |
| coeff_abs[ 0 ], when aps_extension_present == 1 | 38 | Offset | BinIdxTu | 2 |
|  |  | Prefix | 4 + Min( 13, BinIdx Pfx) | 12 |
|  |  | Suffix | 18 + Min( 11, BinIdx Sfx) | 12 |
| coeff_abs[ 1 ], when aps_extension_present == 1 | 38 | Offset | For BinIdxTu == 0: 2 + ( coeff_abs[ 0 ] ≠ 0 ) For BinIdxTu == 1: 4 + ( coeff_abs[ 0 ] ≤ 1 ) | 4 |
|  |  | Prefix | 4 + Min( 13, BinIdx Pfx ) | 12 |
|  |  | Suffix | 18 + Min( 11, BinIdxSfx ) | 12 |
| coeff_abs[ 2 ], when aps_extension_present == 1 | 39 | Offset | For BinIdxTu == 0: ( coeff_abs[ 0 ] ≠ 0 ) + 2 × ( coeff_abs[ 1 ] ≠ 0 ) For BinIdxTu == 1: 4 + ( coeff_abs[ 0 ] ≤ 1 ) + 2 × ( coeff_abs[ 1 ] ≤ 1 ) | 8 |
|  |  | Prefix | 6 + Min( 13, BinIdx Pfx ) | 12 |
|  |  | Suffix | 19 + Min( 11, BinIdx Sfx) | 12 |
| coeff_sign[ ] | na | bypass |  | 0 |
| zero_run_length_prefix | 40 | BinIdxTu |  | 3 |
| zero_run_length_minus3_div2 | 40 | 3 |  | 1 |
| zero_run_length_minus3_mod2 | na | bypass |  | 0 |
| zero_run_length_minus11 | 40 | Prefix | 4 | 1 |
|  |  | Suffix | bypass | 0 |
| slice_raht_inter_layer_code_mode[ lvl ] | na | bypass |  | 0 |
| slice_raht_intra_layer_code_mode[ lvl] | na | bypass |  | 0 |
| - The prefix and suffix bins of the syntax elements coeff_abs[ 0 ] and coeff_abs[ 1 ] use the same values of CtxIdx and CtxTbl. |  |  |  |  |

    - Arithmetic decoding engine

      - General

The arithmetic decoding engine is a context-adaptive, binary arithmetic decoder, performing binary renormalization and producing binary outputs.

  - An arithmetic encoding engine that complements this decoding engine is described in Annex C.

  - The arithmetic decoding engine is related to that of SMPTE VC-2.

      - State variables

The arithmetic decoder is specified in terms of the following state variables:

- IvlLow, representing the beginning of the 16-bit coding interval.

- IvlRange, representing the size of the 16-bit coding interval.

- IvlCode, a codeword within the interval [ IvlLow, IvlLow + IvlRange − 1 ], updated from the arithmetic-coded bitstream.

      - Initial state

The arithmetic decoding state variables shall be initialized as follows; and 16 bits shall be read from the arithmetic-coded bitstream:

IvlLow = 0
IvlRange = 0xFFFF
IvlCode = 0
for (i = 0; i < 16; i++) {
  IvlCode <<= 1
  IvlCode += NextAeStreamBit
}

      - Arithmetic-coded bitstream

The next bit to be consumed as input to the arithmetic decoder is specified by the expression NextAeStreamBit.

NextAeStreamBit := bypass_stream_enabled ? ChunkNextAeBit : DuNextBit

      - Decoding a single binary symbol

Decoding is parameterized by the probability prob0 that the decoded binary symbol is zero-valued.

The decoded binary value binVal is determined and the state variables IvlRange and IvlCode are updated:

rangeTimesProb = IvlRange × prob0 >> 16
binVal = rangeTimeProb ≤ IvlCode − IvlLow
if (¬binVal)
  IvlRange = rangeTimesProb
else {
  IvlLow += rangeTimesProb
  IvlRange −= rangeTimesProb
}

      - Decoding a single binary bypass symbol

If bypass_bin_coding_prob_update_disabled is 0, the decoded binary value binVal is determined and the state variables IvlRange and IvlCode are updated:

rangeTimesProb = IvlRange >> 1
binVal = rangeTimeProb ≤ IvlCode − IvlLow
if (¬binVal)
  IvlRange = rangeTimesProb
else {
  IvlLow += rangeTimesProb
  IvlRange −= rangeTimesProb
}

if bypass_bin_coding_prob_update_disabled is 1, The decoded binary value binVal is determined and the state variables IvlLow and IvlCode are updated:

IvlCode <<= 1
IvlLow <<= 1
binVal = IvlRange ≤ IvlCode − IvlLow
if (binVal)
  IvlLow -= IvlRange
}

      - Arithmetic decoder state renormalization

Renormalization stops the arithmetic decoding engine from losing accuracy.  Renormalization shall be applied while the size of the coding interval is less than or equal to a quarter of the total available 16-bit range.  Each renormalization doubles the interval and reads a bit into the codeword.

If IvlRange is less than or equal to 214, the state variables IvlRange, IvlLow and IvlCode are updated:

if ((IvlLow + IvlRange − 1) ^ IvlLow ≥ 0x8000) {
  IvlCode ^= 0x4000
  IvlLow ^= 0x4000
}
IvlRange <<= 1
IvlLow = (IvlLow << 1) & 0xFFFF
IvlCode = ((IvlCode << 1) | NextAeStreamBit) & 0xFFFF

If IvlRange remains less than or equal to 214, the process shall be repeated until it is not.

      - Arithmetic decoder flushing process

The arithmetic decoder shall be flushed at the end of each occupancy tree entropy stream.

Flushing shall repeatedly perform state renormalization until IvlRange is greater than 214, and then discard bits from the arithmetic-coded bitstream until it is byte aligned.

while (IvlRange ≤ 0x4000) {
  NextAeStreamBit
  IvlRange <<= 1
}

/* byte−align */
while (ReadAeStreamIdx % 8)
  NextAeStreamBit

  - Parsing state memorization and restoration

    - General

Subclause 11.6 applies when either entropy_continuation_enabled is 1, slice_inter_entropy_continuation is 1 or occtree_stream_cnt_minus1 is greater than 0.

At certain moments, the entropy parsing state is recorded and later, used as the initial state for parsing other DUs or occupancy tree entropy streams.

The entropy parsing state shall comprise:

- for a GDU, the CABAC CPMs (11.5.3), the demi-CPMs for bitwise occupancy coding (9.2.10.6), the dictionary codec state for bytewise occupancy coding (9.2.9.4), the planar occupancy coding state (9.2.11.5.2) and the state of the variables PrevInterFrameRefIdx, PrevPhiResidSign, PrevPhiMul and PrevRadiusResidSign (9.3.3.1);

- for an ADU, the CABAC CPMs only (11.5.3).

The entropy parsing state shall be recorded and restored independently according to DU type (ADU versus GDU) and for each different value of ADU AttrIdx.  For example, a coded point cloud sequence with num_attributes equal to 2 would require storage for three sets of entropy parsing state.

At the start of any GDU with slice_entropy_continuation equal to 0 and slice_inter_entropy_continuation equal to 0, all previously recorded GDU and ADU entropy parsing state shall be discarded.

    - Geometry data units

      - Memorization

The GDU entropy parsing state shall be recorded at:

- the end of every geometry_data_unit syntax structure (7.3.3.1); and

- the end of every occupancy_tree_level( dpth ) syntax structure (7.3.3.5) where dpth is equal to OcctreeEntropyStreamDepth − 1.

Memorization shall record the elements and values of the GDU entropy parsing state for restoration by the restoration process (11.6.2.2).

      - Restoration

The GDU entropy parsing state shall be restored at:

- the start of a geometry_data_unit syntax structure (7.3.3.1) when slice_entropy_continuation is 1; and

- the start of a geometry_data_unit syntax structure (7.3.3.1) when slice_entropy_continuation is 0, slice_inter_entropy_continuation is 1; and

- the start of every occupancy_tree_level( dpth ) syntax structure (7.3.3.5) where dpth is greater than OcctreeEntropyStreamDepth.

Restoration shall restore the elements and values of the GDU entropy parsing state to those previously recorded by the memorization process (11.6.2.1).  At the start of a geometry_data_unit syntax structure, restoration shall exclude the planar occupancy coding state.

    - Attribute data units

      - Memorization

The ADU entropy parsing state shall be recorded at the end of every attribute_data_unit syntax structure (7.3.4.1).

Memorization shall record the elements and values of the ADU entropy parsing state for restoration by the restoration process (11.6.3.2).  The state shall be recorded separately for each value of AttrIdx.

      - Restoration

The ADU entropy parsing state shall be restored at the start of each attribute_data_unit syntax structure (7.3.4.1) when either slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1.  The restoration shall be from the state recorded by the memorization process (11.6.3.1) with the same value of AttrIdx.

    - Defaulted attribute data units

The recorded ADU entropy parsing state for the attribute identified by AttrIdx shall be initialized at the start of each defaulted attribute data unit when slice_entropy_continuation is 0 or slice_inter_entropy_continuation is 0.  The initialization shall be according to 11.5.3.2 as if the data unit contained a syntax element with the descriptor ae(v).

- While a defaulted attribute data unit does not use arithmetic coding, it is necessary to record the initialized ADU entropy parsing state when slice_entropy_continuation is 0 or slice_inter_entropy_continuation is 0 so that ADUs in any following slices where slice_entropy_continuation is 1 or slice_inter_ entropy_continuation is 1  do not have an indeterminate ADU parsing state.

- OBUF parsing process

  - General

The acronym OBUF stands for “Optimal Binary coder with Update on the Fly”.  An OBUF instance decodes information from the bitstream to obtain a bit bin.

An OBUF instance is called with two contextual information info1 and info2 as input and follows the steps of

- obtaining, based on internal statistics, an index ctxIdx pointing to an element of a table of OBUF adaptive context probability models (ACPMs) as defined in 12.2.2,

- decoding an arithmetic-coded bin bin by using the CABAC decoder with the pointed OBUF ACPM (12.5),

- and updating internal statistics.

Internal statistics are modeled by a memory channel that evolves (is updated) after the decoding of each bin. The memory channel is stored into three elements:

- an OBUF tree constituted of three arrays kShift[ ][ ], nVisit[ ][ ] and ctxIdxMap[ ][ ],

- a buffer obufBuffer[ ][ ] of OBUF tree leaves,

- and an array obufCtxArray[ ] of OBUF ACPMs.

  - Creation of an OBUF instance

An OBUF instance is a set made of the three above elements, namely an OBUF tree, a buffer of OBUF tree leaves and an array of OBUF ACPMs.  The OBUF tree and the array of OBUF ACPMs are uniquely attached to the OBUF instance.  However, one buffer of OBUF tree leaves can be shared among several OBUF instances.

Creating an OBUF instance follows the steps of

- associating a buffer of OBUF tree leaves, that has been previously created according to clause 12.3, with the instance,

- creating and initializing OBUF trees according to clause 12.2.1,

- and creating and initializing an array of OBUF ACPMs according to clause 12.2.2.

A buffer of OBUF tree leaves is created before creating an OBUF instance because a buffer can be shared by multiple OBUF instances.

The OBUF instance is created based on the given of

- an OBUF buffer obufBuffer[ ][ ] of OBUF tree leaves of depth obufLeafDepth,

- a first size nBit1 in bits corresponding to the size of first contextual information info1 used as input when calling the OBUF instance,

- a second size nBit2 in bits corresponding to the size of second contextual information info2 used as input when calling the OBUF instance,

- and optionally an initialization array initObufArray.

Creation and initialization of the OBUF trees is performed according to clause 12.2.1 based on nBit1, nBit2, obufLeafDepth and initObufArray.  Creation and initialization of the array of OBUF ACPMs is performed according to clause 12.2.2.

    - Creation and initialization of OBUF trees

The sizes s1 and s2 of the two contextual information as well as the size obufTreeSize2 of the s1 OBUF trees are determined by

s1 = 1 << nBit1
s2 = 1 << nBit2
obufTreeSize2 = 1 << nBit2 – obufLeafDepth

Three double-entry 8-bit arrays kShift[ ][ ], nVisit[ ][ ] and ctxIdxMap[ ][ ] are created with size s1 along the first entry and size obufTreeSize2 along the second entry for a total size of s1 × obufTreeSize2 each.

The array kShift[ ][ ] is initialized with all s1 × obufTreeSize2 values set to s2. The array nVisit [ ][ 0] is initialized with all of the s1 values set to 0.  The array ctxIdxMap [ ][ 0] is initialized with all of the s1 values set to 127.

The array kShift[ ][ ] corresponds to the number of erased bits of a secondary information info2 when the instance is called.  The initialising to s2 indicates that all bits of any second information info2 are erased at initial state of the OBUF instance.

The array nVisit[ ][ ] corresponds to the number of visits of nodes of the OBUF trees.  The initialising to 0 indicates that root nodes of the OBUF trees have not been visited yet at initial state of the OBUF instance.

The array ctxIdxMap[ ][ ] corresponds to 8-bit context indices pointing (after right shift by 3) to OBUF ACPMs of the array obufCtxArray[ ].  The initialising to 127 indicates pointing to the OBUF ACPM with associated probability 0.5 at initial state of the OBUF instance.

When the optional initialization array initObufArray of size s1 is provided, the array ctxIdxMap[ ][ ] is further initialized by

for (j = 0; j < s1; j++)
  ctxIdxMap[j][0] = initObufArray[j]

This further initialising provides an initial statistical model for the root nodes of the s1 OBUF trees.

    - Creation and initialization of an array of OBUF ACPMs

An array of OBUF ACPMS is an extension of an array of ACPMs. It is made of

- a 16-bit array obufCtxArray[ ] of size 32 of ACPMs as defined in 11.5.3,

- a 16-bit array obufCtxProbaBounds[ ] of size 33 of probability bounds associated with the 32 models of the array of ACPMs such that the probability obufCtxArray[k ] remains between the two bounds obufCtxProbaBounds[k ] and obufCtxProbaBounds[k+1 ] before decoding a bin.

The array obufCtxArray[ ] is initialized by the table ObufInitProba [ ] defined by Table 64, and the array obufCtxProbaBounds[ ] is initialized by the table ObufInitProbaBounds [ ] defined by Table 65.

Table 64 — Values of ObufInitProba[ k ]

| k | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| value | 65461 | 65160 | 64551 | 63637 | 62426 | 60929 | 59163 | 57141 |
| k | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| value | 54884 | 52413 | 49753 | 46929 | 43969 | 40899 | 37750 | 34553 |
| k | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| value | 31338 | 28135 | 24977 | 21893 | 18914 | 16067 | 13382 | 10883 |
| k | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| value | 8596 | 6542 | 4740 | 3210 | 1967 | 1023 | 388 | 75 |

Table 65 — Values of ObufInitProbaBound[ k ]

| k | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| value | 65535 | 65388 | 64933 | 64169 | 63105 | 61747 | 60112 | 58214 |  |
| k | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |  |
| value | 56069 | 53699 | 51128 | 48379 | 45480 | 42458 | 39340 | 36160 |  |
| k | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |  |
| value | 32946 | 29730 | 26541 | 23413 | 20374 | 17454 | 14681 | 12083 |  |
| k | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 |
| value | 9684 | 7509 | 5575 | 3905 | 2515 | 1419 | 627 | 150 | 0 |

  - Creation of a buffer of OBUF tree leaves

A buffer of OBUF tree leaves is a rolling buffer whose elements are fully deployed trees that can be attached to a leaf node of an OBUF tree to prolong the latter.

A buffer of OBUF tree leaves is created based on the given of

- a buffer size obufBufferSize,

- and a depth obufLeafDepth of fully deployed trees constituting each element of the buffer.

The number deployedTreeSize of leaf nodes of fully deployed trees is

deployedTreeSize = 1 << obufLeafDepth

The buffer is composed of

- one 8-bit array obufBuffer[ ][ ] of size obufBufferSize along its first component and of size deployedTreeSize along its second component.  The k-th OBUF tree leaf in the buffer is the array obufBuffer[k ][ ] of size deployedTreeSize,

- a buffer element index nextUsableIdx indicating the position of the next OBUF tree leaf obufBuffer[usableIdx] [ ] that is usable for attaching to a leaf node of an OBUF tree,

- and a rolling flag obufBufferRolled that indicates if the buffer element index has rolled back to the start of the buffer at least once.

An array obufBuffer[k ][ ] is made of 8-bit context indices pointing (after right shift by 3) to OBUF ACPMs of the array obufCtxArray[ ] associated to an OBUF instance.  An array obufBuffer[k][] is used to prolong the array ctxIdxMap[ ][ ] of the OBUF instance once attached to a leaf node of the OBUF tree of the OBUF instance.

The buffer element index nextUsableIdx and the rolling flag obufBufferRolled are initialized to 0.  However, the array obufBuffer[ ][ ] does not require initialization as each OBUF tree leaf obufBuffer[k][ ] will be initialized when attached to a leaf node of an OBUF tree.

  - Call and update of an OBUF instance

An OBUF instance is called with input

- a first contextual information info1 of size nBit1 bits,

- and a second contextual information info2 of size nBit2 bits.

The output is a decoded bin bin.

A reduced second contextual information info2Red and the number nErasedBit of bits to be erased from the second contextual information are obtained by

info2Red = info2 >> obufLeafDepth
nErasedBit = kShift[ info1][ info2Red]

Depending on the value of nErasedBit, the decoding and update process is performed according to either the OBUF trees or the buffer of OBUF tree leaves.  In case nErasedBit >= obufLeafDepth , the process continues to clause 12.4.1; otherwise, the process continues to clause 12.4.2.

    - Decode and update according to OBUF trees

      - Decode of a bin and context index update

The second contextual information info2Erased with erased bits, according to the number of erased nErasedBit bits, is computed by

nErasedBitTree = nErasedBit – obufLeafDepth
info2Erased = (info2Red >> nErasedBitTree) << nErasedBitTree

An 8-bit context index ctxIdx is obtained from the array ctxIdxMap[ ][ ].

ctxIdx = ctxIdxMap[info1 ][ info2Erased]

The decoded bin bin is obtained by applying clause 12.5.

Depending on the value of the decoded bin bin, the obtained context index is updated in the array ctxIdxMap[ ][ ] by using ObufCtxIdxDelta[ ] as defined in Table 66.

if (bin) 
  ctxIdxMap[info1 ][ info2Erased] += ObufCtxIdxDelta[(255 - ctxIdx) >> 4] 
else
  ctxIdxMap[info1 ][ info2Erased] -= ObufCtxIdxDelta[ctxIdx >> 4]

Table 66 — Values of ObufCtxIdxDelta[ k ]

| k | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| value | 0 | 1 | 1 | 2 | 4 | 7 | 9 | 11 |
| k | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| value | 14 | 16 | 19 | 23 | 22 | 22 | 20 | 15 |

The number of visits nVisit is incremented by one unit

nVisit[info1 ][ info2Erased]++

and a threshold thVisit on the number of visits is obtained by

thVisit = 3 + abs((ctxIdx-127) >> 4)

If the number of visits nVisit is strictly lower than the threshold thVisit then the call to the OBUF instance is finished. Otherwise, an updating process is performed, and the process continues to clause 12.4.1.2 if nErasedBit > 0 or continues to clause 12.4.1.3 otherwise.

      - OBUF tree update

The node of OBUF trees referenced by info1 and info2Erased is split into two new nodes with no visit for each.

nVisit[ info1][info2Erased] = 0 /* first new node */
nVisit[ info1][info2Erased + (1 << nErasedBit -1)] = 0 /* second new node */

The context index ctxIdxMap[ info1][info2Erased] of the first of the two new nodes is automatically inherited from the node.  The context index of the second of the two new nodes is obtained by copying the context index of the node.

ctxIdxMap[ info1][info2Erased + (1 << nErasedBit -1)] = ctxIdxMap[ info1][info2Erased]

The number of erased bits is decreased by one unit for all entries of kShift[info1 ][i ] where i corresponds to the two new nodes.

for (i = 0; i < (1<< nErasedBit); i++) 
  kShift[info1][info2Erased + i]--

This terminates the call of the OBUF instance.

      - Attaching of a buffer element

A buffer element of the buffer of OBUF leave is attached to the node of the OBUF tree referenced by info1 and info2Erased.  Attaching depends on the availability of a non-used buffer element.  This availability is obtained from the value of the rolling flag obufBufferRolled.

When the rolling flag obufBufferRolled is equal to 0, the buffer element obufBuffer[nextUsableIdx][ ] is initialized by the context index of the node

for (i = 0; i < deployedTreeSize; i++) 
  obufBuffer[nextUsableIdx][ i] = ctxIdx

and the pointer nextUsableIdx pointing to the attached element of the buffer of OBUF tree leaves is stored as pointer hidden information in the two arrays ctxIdxMap[ ][] and nVisit[ ][] by

nVisit[ info1][info2Erased] = nextUsableIdx & 255
ctxIdxMap[ info1][info2Erased] = nextUsableIdx >> 8

The pointer nextUsableIdx is incremented.

nextUsableIdx++

Otherwise, when the rolling flag obufBufferRolled is equal to 1, there is no non-used buffer element left.  A search for the best element to attach within a search window between nextUsableIdx and nextUsableIdx + 20 is performed.

distMin = 256
idxMin =  nextUsableIdx
mask = (1 << obufLeafDepth) – 1

for (b = nextUsableIdx; b < nextUsableIdx + 20; b++) {
  d = abs(ctxIdx - obufBuffer[b][info2 & mask]) 
  if (d < dmin) {
    distMin = d
    idxMin = b
  }

}

The pointer idxMin points to the best element of the buffer of OBUF tree leaves to be attached and this pointer is stored as pointer hidden information in the two arrays ctxIdxMap[ ][] and nVisit[ ][] by

nVisit[ info1][info2Erased] = idxMin & 255
ctxIdxMap[ info1][info2Erased] = idxMin >> 8

The pointer nextUsableIdx is then modified by pointing to the element next to the attached element.

nextUsableIdx = idxMin + 1

After attaching a buffer element, independently on the value of obufBufferRolled, if the index nextUsableIdx is greater than or equal to the buffer size obufBufferSize, the buffer is rolled: the index is reset to 0 and the rolling flag obufBufferRolled is set to 1.

The number of erased bits of the node is decremented.

kShift[info1][info2Erased]--

This terminates the call of the OBUF instance.

    - Decode and update according to an element of the buffer of OBUF tree leaves

A 16-bit pointer leafIdx to an element of the buffer of OBUF tree leaves is obtained from the pointer hidden information (as created in clause 12.4.1.3) in ctxIdxMap[ ][] and nVisit[ ][].

leafIdx = (ctxIdxMap[ info1][info2Erased] << 8) + nVisit[ info1][info2Erased]

An 8-bit context index ctxIdx is obtained from the pointed OBUF tree leaf obufBuffer[leafIdx][ ].

mask = (1 << obufLeafDepth) - 1
ctxIdx = obufBuffer[leafIdx][info2 & mask ]

The decoded bin bin is obtained by applying clause 12.5.

Depending on the value of the decoded bin bin, the obtained context index is updated in the array obufBuffer[ ][ ] by using ObufCtxIdxDelta[ ] as defined in Table 66.

if (bin) 
  obufBuffer[leafIdx][info2 & mask ] += ObufCtxIdxDelta[(255 - ctxIdx) >> 4] 
else
  obufBuffer[leafIdx][info2 & mask ] -= ObufCtxIdxDelta[ctxIdx >> 4]

This terminates the call of the OBUF instance.

  - Decode of a bin based on an OBUF ACPM

A selected context  SelCtx is obtained from the 8-bit context index ctxIdx and the array obufCtxArray[ ] of OBUF ACPMs by

idx = ctxIdx >> 3
SelCtx = obufCtxArray[idx ]

A sanity check and correction of the 16-bit probability prob0 associated with the selected context SelCtx are performed (12.5.1) before decoding a bin (12.5.2) by using CABAC with the corrected selected context SelCtx as input.

    - enProbability correction based on probability bounds

Lower and upper bounds of probability for the selected context are obtained by

lowProba = obufCtxProbaBounds[idx+1] 
upProba = obufCtxProbaBounds[idx]

In case the probability prob0 of the selected context SelCtx is not within the bounds, this probability is corrected toward the bounds and the bounds are adjusted by using the table CtxUpdateDelta[  ] defined by Table 61.

if (prob0 > upProba) {
  prob0 = upProba
  upProba += CtxUpdateDelta[255 - (upProba >> 8)] >> 2
  if (idx > 0 && upProba > obufCtxProbaBounds [idx - 1]) {
    upProba = obufCtxProbaBounds [idx - 1] 
  }
}

if (prob0 < lowProba) {
  prob0 = lowProba
  lowProba -= CtxUpdateDelta[lowProba >> 8] >> 2
  if (idx < 31 && lowProba > obufCtxProbaBounds [idx + 2]) {
    lowProba = obufCtxProbaBounds [idx + 2] 
  }
}

    - Decoding of a bin using CABAC

A bin bin of information is decoded according to the clause 11.5.4.5 by using the corrected probability prob0 of the selected context SelCtx. Then, the probability evolves according to clause 11.5.3.3.

- (normative)
Profiles and levels

This Annex will describe the profiles and levels.

  - Permitted ranges for syntax elements

Table A.1 to Table A.14 specify constraints on coded syntax element values in bitstreams conforming to this version of this document.  Other constraints specified in this document may further constrain their permitted ranges.

Unless otherwise specified in this document, a decoder conforming to this version of this document may reject bitstreams containing syntax elements outside the permitted ranges.

Table A.1 — Permitted ranges for sequence parameter set syntax elements

| Syntax element | Range |
| --- | --- |
| reserved_profile_18bits | 0 |
| sps_seq_parameter_set_id | 0 |
| seq_origin_bits | 0 .. 31 |
| seq_origin_log2_scale | 0 .. 31 |
| seq_bbox_size_bits | 0 .. MaxSeqBboxDimLog2 |
| seq_unit_numerator_minus1 | 0 .. 31 |
| seq_unit_denominator_minus1 | 0 .. 31 |
| seq_coded_scale_exponent | 0 .. 31 |
| seq_coded_scale_mantissa_bits | 0 .. 31 |
| num_attributes | 0 .. 63 |
| attr_components_minus1 | 0 .. 3 |
| attr_instance_id | 0 .. 63 |
| attr_bitdepth_minus1 | 0 .. 63 |
| attr_label | 0 .. 6 |
| attr_property_cnt | 0 .. 16 |
| sps_extension_present | 0 .. 1 |

Table A.2 — Permitted ranges for attribute parameter syntax elements

| Syntax element | Range |
| --- | --- |
| attr_cicp_colour_primaries | 0 .. 255 |
| attr_cicp_transfer_characteristics | 0 .. 255 |
| attr_cicp_matrix_coeffs | 0 .. 255 |
| attr_offset_bits | 0 .. 64 |
| attr_scale_bits | 0 .. 16 |
| attr_frac_bits | 0 .. 31 |

Table A.3 — Permitted ranges for tile inventory syntax elements

| Syntax element | Range |
| --- | --- |
| ti_seq_parameter_set_id | 0 |
| tile_origin_bits_minus1 | 0 .. 30 |
| tile_size_bits_minus1 | 0 .. 30 |
| ti_origin_bits_minus1 | 0 .. 30 |
| ti_origin_log2_scale | 0 .. 31 |

Table A.4 — Permitted ranges for geometry parameter set syntax elements

| Syntax element | Range |
| --- | --- |
| gps_seq_parameter_set_id | 0 |
| gps_geom_origin_log2_scale | 0 .. 31 |
| occtree_intra_pred_max_nodesize_log2 | 0 .. MaxSliceDimLog2 |
| occtree_planar_threshold | 0 .. 127 |
| gps_angular_origin_bits_minus1 | 0 .. 31 |
| ptree_ang_azimuth_pi_bits_minus11 | 0 .. 9 |
| ptree_ang_azimuth_step_minus1 | 0 .. 2ptree_ang_azimuth_pi_bits_minus11 + 12-1 |
| ptree_ang_radius_scale_log2 | 0 .. 31 |
| num_beams_minus1 | 0 .. 254 |
| beam_elevation_init | ±218 |
| beam_voffset_init | ±218 |
| beam_steps_per_rotation_init_minus1 | 0 .. 6 588 396 |
| beam_elevation_diff[ 𝑖 ] | ±219 |
| beam_voffset_diff[ 𝑖 ] | ±219 |
| beam_steps_per_rotation_diff[ 𝑖 ] | ±6 588 396 |
| geom_qp | 0 .. 167 |
| ptree_qp_period_log2 | 0 .. 21 |
| occtree_direct_node_qp_offset | ±167 |
| gps_extension_present | 0 .. 1 |
| biprediction_enabled | 0 .. 2 |
| ptree_ang_azimuth_scaling_enabled | 0 .. 1 |
| ptree_ang_max_pred_index | 0 .. 7 |
| ptree_ang_pred_list_radius_resid_threshold | xxx .. xxx |
| ptree_ang_radius_resid_context_qphi_threshold | 0 .. (ptree_ang_azimuth_pi_bits_minus11+12) / (ptree_ang_azimuth_step_minus1+1) |

Table A.5 — Permitted ranges for attribute parameter set syntax elements

| Syntax element | Range |
| --- | --- |
| aps_seq_parameter_set_id | 0 |
| attr_coding_type | 0 .. 3 |
| attr_primary_qp_minus4 | 0 .. 95 |
| attr_secondary_qp_offset | ±95 |
| raht_prediction_subtree_min | 0 .. 19 |
| raht_prediction_samples_min | 0 .. 19 |
| pred_set_size_minus1 | 0 .. 2 |
| pred_inter_lod_search_range | 0 .. MaxSlicePoints − 1 |
| pred_dist_bias_minus1_xyz | 0 .. 210-2 |
| pred_max_range_minus1 | 0 .. 2MaxSliceDimLog2-1 |
| lod_max_levels_minus1 | 0 .. MaxSliceDimLog2 – 1 |
| lod_decimation_mode | 0 .. 2 |
| lod_sampling_period_minus2 | 0 .. MaxSlicePoints − 2 |
| lod_initial_dist_log2 | 0 .. MaxSliceDimLog2 |
| pred_direct_max_idx_plus1 | 0 .. pred_set_size_minus1 + 1 |
| pred_intra_min_lod | 0 .. lod_max_levels_minus1 + 1 |
| pred_intra_lod_search_range | 0 .. MaxSlicePoints − 1 |
| aps_extension_present | 0 .. 1 |
| attr_inter_prediction_search_range | 0 .. MaxSlicePoints − 1 |
| refAttrIdx | 0 .. num_attributes − 1 |
| raht_inter_layer_depth_minus1 | 0 .. MaxSliceDimLog2 − 1 |
| raht_inter_skip_layers | 0 .. raht_inter_layer_depth_minus1 + 1 |
| raht_prediction_weights[ ] | 0 .. 63 a |
| max_points_per_sort_log2_plus1 | 0 .. IntLog2(MaxSlicePoints) + 2 |
| raht_prediction_search_range | 0 .. MaxSlicePoints − 1 |
| a The following condition should be satisfied: raht_prediction_weights[0 ] + 3 × Max(raht_prediction_weights[1 ], raht_prediction_weights[3 ]) + 3 × Max(raht_prediction_weights[2 ], raht_prediction_weights[4 ]) ≤ 63 |  |

Table A.6 — Permitted ranges for frame-specific attribute properties syntax elements

| Syntax element | Range |
| --- | --- |
| fsap_seq_parameter_set_id | 0 |
| fsap_sps_attr_idx | 0 .. num_attributes – 1 |
| fsap_num_props | 0 .. 15 |

Table A.7 — Permitted ranges for geometry data unit syntax elements

| Syntax element | Range |
| --- | --- |
| gdu_temporal_id | 0 .. 7 |
| slice_id | 0 .. 216 − 1 |
| prev_slice_id | 0 .. 216 − 1 |
| slice_geom_origin_log2_scale | 0 .. 31 |
| slice_geom_origin_bits_minus1 | 0 .. MaxSeqBboxDimLog2 − 1 |
| slice_angular_origin_bits_minus1 | 0 .. MaxSeqBboxDimLog2 − 1 |
| occtree_depth_minus1 | 0 .. MaxSliceDimLog2 + 3 |
| occtree_stream_cnt_minus1 | 0 .. occtree_depth_minus1 |
| slice_geom_qp_offset | ±167 |
| slice_ptree_qp_period_log2_offset | ±21 |
| ptn_radius_min | 0 .. 221 |
| occ_subtree_qp_offset_abs[ ][ ][ ] | 0 .. 167 |
| occ_dup_point_cnt[ ] | 0 .. MaxSlicePoints − 1 |
| direct_dup_point_cnt | 0 .. MaxSlicePoints − 1 |
| beam_idx_resid_abs[ ] | 0 .. 254 |
| ptn_qp_offset_abs[ ] | 0 .. 167 |
| ptn_dup_point_cnt[ ] | 0 .. MaxSlicePoints − 1 |
| ptn_child_cnt_xor1[ ] | 0 .. 3 |
| ptn_inter_pred_mode[ ] | 0 .. global_motion_enabled ? 3 : 1 |
| ptn_pred_mode[ ] | 0 .. Min( 3, dpth ) ab |
| ptn_phi_mul_abs_minus9[ ][ ] | 0 .. 220 |
| ptn_sec_resid_abs[ ][ ] | 0 .. 2MaxSeqBboxDimLog2-1 |
| a where dpth is per predictive_tree_node( dpth, nodeIdx ) b ptn_pred_mode[ ] may be 1 when dpth is 0 if ptree_ang_azimuth_scaling_enabled is 1 |  |

Table A.8 — Permitted ranges for attribute data unit syntax elements

| Syntax element | Range |
| --- | --- |
| adu_temporal_id | 0 .. 7 |
| adu_sps_attr_idx | 0 .. num_attributes – 1 |
| adu_slice_id | 0 .. 216 − 1 |
| last_comp_pred_coeff_diff | ±255 |
| inter_comp_pred_coeff_diff | ±255 |
| lod_dist_log2_offset | ±21 |
| attr_qp_offset | ±95 |
| attr_qp_layer_cnt_minus1 | 0 .. MaxSliceDimLog2 − 1 |
| attr_qp_layer_offset[ 𝑖 ][ 𝑐 ] | ±95 |
| attr_qp_region_cnt | 0 .. 1 |
| attr_qp_region_bits_minus1 | 0 .. MaxSliceDimLog2 − 1 |
| attr_qp_region_offset[ 𝑖 ][ 𝑐 ] | ±95 |
| zero_run_length_minus11 | 0 .. MaxSlicePoints − 11 |
| coeff_abs | 0 .. 231 − 1 |
| attr_AC_qp_layer_cnt_minus1 | 0 .. MaxSliceDimLog2 − 1 |
| attr_AC_qp_offset[ 𝑖 ][ 𝑐 ][ a] | ±95 |
| num_inter_filters | Min(raht_inter_layer_depth_minus1+1, RahtRootLvl) – raht_send_inter_filters |
| raht_inter_filter_qidx | ±233 |

Table A.9 — Permitted ranges for defaulted attribute data unit syntax elements

| Syntax element | Range |
| --- | --- |
| defattr_seq_parameter_set_id | 0 |
| defattr_reserved_zero_3bits | 0 |
| defattr_sps_attr_idx | 0 .. num_attributes − 1 |
| defattr_slice_id | 0 .. 216 − 1 |
| defattr_value | 0 .. AttrMaxVal |

Table A.10 — Permitted ranges for FGS parameter syntax elements

| Syntax element | Range |
| --- | --- |
| subgroup_bbox_origin_bits_minus1 | 0 .. 28 − 1 |
| subgroup_bbox_size_bits_minus1 | 0 .. 28 − 1 |

Table A.11 — Permitted ranges for FGS layer-group structure inventory syntax elements

| Syntax element | Range |
| --- | --- |
| lgsi_seq_parameter_set_id | 0 |
| lgsi_slice_id[] | 0 .. 216 − 1 |
| lgsi_subgroup_bbox_origin_bits_minus1 | 0 .. 28 − 1 |
| lgsi_subgroup_bbox_size_bits_minus1 | 0 .. 28 − 1 |
| lgsi_origin_bits_minus1 | 0 .. 30 |
| lgsi_origin_xyz[] | 0 .. 30 |
| lgsi_origin_log2_scale | 0 .. 31 |

Table A.12 — Permitted ranges for FGS dependent geometry data unit header syntax elements

| Syntax element | Range |
| --- | --- |
| dgdu_slice_id | 0 .. 216 − 1 |

Table A.13 — Permitted ranges for FGS attribute data unit parameter syntax elements

| Syntax element | Range |
| --- | --- |
| subgroup_weight_adj_coeff_a | 0 .. 263 − 1 |
| subgroup_weight_adj_coeff_b | 0 .. 263 − 1 |

Table A.14 — Permitted ranges for FGS dependent attribute data unit header syntax elements

| Syntax element | Range |
| --- | --- |
| dadu_sps_attr_idx | 0 .. num_attributes – 1 |
| dadu_slice_id | 0 .. 216 − 1 |
| last_comp_pred_coeff_diff[ ] | ±255 |
| inter_comp_pred_coeff_diff[ ][ ] | ±255 |
| attr_qp_offset[ ] | ±95 |
| attr_qp_layer_cnt_minus1 | 0 .. MaxSliceDimLog2 − 1 |
| attr_qp_layer_offset[ ][ ] | ±95 |

(normative)
Type-length-value encapsulated bytestream format

General

This annex specifies the syntax and semantics of a bytestream format for use by applications that deliver DUs as an ordered stream of bytes without any requirement for further encapsulation in a file format.

The bytestream format comprises a sequence of type-length-value encapsulation structures that each represent a single coded DU syntax structure.

  - Syntax and semantics

    - Syntax

| tlv_encapsulation( ) { | Descriptor |
| --- | --- |
| tlv_type | u(8) |
| tlv_num_payload_bytes | u(32) |
| for( 𝑖 = 0; 𝑖 < tlv_num_payload_bytes; 𝑖++ ) |  |
| tlv_payload_byte[ 𝑖 ] | u(8) |
| } |  |

    - Semantics

The order of tlv_encapsulation structures shall follow the decoding order for the encapsulated syntax structures.

tlv_type identifies the syntax structure represented by tlv_payload_byte[ ] as specified by Table B.1.

Table B.1 — Mapping of tlv_type and associated data unit to syntax tables

| tlv_type | Syntax table | Description |
| --- | --- | --- |
| 0 | 7.3.2.1 | Sequence parameter set data unit |
| 1 | 7.3.2.5 | Geometry parameter set data unit |
| 2 | 7.3.3.1 | Geometry data unit |
| 3 | 7.3.2.6 | Attribute parameter set data unit |
| 4 | 7.3.4.1 | Attribute data unit |
| 5 | 7.3.2.4 | Tile inventory data unit |
| 6 | 7.3.2.8 | Frame boundary marker data unit |
| 7 | 7.3.5 | Defaulted attribute data unit |
| 8 | 7.3.2.7 | Frame-specific attribute properties data unit |
| 9 | 7.3.2.9 | User data data unit |
| 10 | 7.3.3.1 | Geometry data unit unused for reference |
| 11 | 7.3.4.1 | Attribute data unit unused for reference |

tlv_num_payload_bytes specifies the length in bytes of the syntax element array tlv_payload_byte[ ].

tlv_payload_byte[ 𝑖 ] is the 𝑖-th byte of payload data.

It is a requirement of bitstream conformance that when a TLV structure containing coded slice geometry has tlv_type equal to 10, the TLV structure containing the coded slice attributes of the coded slice geometry shall have a tlv_type equal to 11.

It is a requirement of bitstream conformance that all the geometry TLV data units for all slices within a frame must have the same TLV type, all the attribute TLV data units for all slices within a frame must have the same TLV type.

  - Parsing process

The decoder repeatedly parses tlv_encapsulation structures until the end of the bytestream is encountered (as determined by unspecified means) and the last tlv_encapsulation structure in the bytestream has been decoded.

After parsing each tlv_encapsulation structure:

- The array DataUnitBytes is set equal to tlv_payload_byte[ ].

- The variable DataUnitLength is set to tlv_num_payload_bytes.

- The parsing process for the syntax structure corresponding to tlv_type as specified in Table B.1 is performed.

- (informative)
Arithmetic encoding engine

This annex does not form an integral part of this document.

  - General

This annex describes an arithmetic encoding engine that complements the arithmetic decoding engine specified by 11.5.4.  The encoding engine is essentially symmetric with the decoding engine, i.e. its complementary processes are performed in the same order.  Table C.1 illustrates the correspondence between decoding and encoding processes.

Table C.1 — Correspondence between decoder and encoder arithmetic coding processes

| Process | Decoder | Encoder |
| --- | --- | --- |
| Initialization | 11.5.4.3 | C.3 |
| Symbol coding | 11.5.4.4 | C.4 |
| Renormalization | 11.5.4.7 | C.5 |
| Termination | 11.5.4.8 | C.6 |

  - State variables

The arithmetic encoding engine is described in terms of the following state variables:

- IvlLow, indicating the bottom of the 16-bit encoding interval.

- IvlRange, indicating the size of the 16-bit encoding interval.

- IvlCarry, a count of unresolved straddle conditions during renormalization.

  - Initial state

The arithmetic encoding state is initialized before encoding the first binary symbol for an entropy stream:

IvlLow = 0
IvlRange = 0xFFFF
IvlCarry = 0

With 16-bit accuracy, 0xFFFF corresponds to an interval width value of (almost) 1.

  - Encoding process for a single binary symbol

Encoding is parameterized by the binary symbol binVal and its associated contextual probability prob0 of it being zero-valued.

The binary symbol is encoded by updating the encoding interval bounds [ IvlLow, IvlLow + IvlRange ] according to the symbol value and the contextual probability:

rangeTimesProb = (IvlRange × prob0) >> 16
if (¬binVal)
  IvlRange = rangeTimesProb
else {
  IvlLow += rangeTimesProb
  IvlRange −= rangeTimesProb
}

After encoding the symbol, the interval is renormalized and any available entropy stream bits output according to C.5.

  - Arithmetic encoder state renormalization process

Renormalization causes IvlLow and IvlRange to be modified exactly as for the decoder.  It is performed when IvlRange is less than or equal to 214.

If, during renormalization, IvlLow and IvlLow + IvlRange straddle 215, a carry is recorded.

Bits are output to the entropy stream when IvlLow and IvlLow + IvlRange do not straddle 215.  The output bits include any accumulated carries.

if (IvlRange ≤ 0x4000) {
  if ((IvlLow + IvlRange − 1) ^ IvlLow ≥ 0x8000) {
    IvlLow ^= 0x4000
    IvlCarry++
  } else {
    writeBit(Bit(IvlLow, 15))
    for (; IvlCarry > 0; IvlCarry−−)
      writeBit(¬Bit(IvlLow, 15))
  }
  IvlRange <<= 1
  IvlLow <<= 1
  IvlLow &= 0xFFFF
}

If IvlRange remains less than or equal to 214, the process is repeated until it is not.

  - Arithmetic encoding engine termination process

After encoding all binary symbols, there might be insufficient bits written to the entropy stream for a decoder to determine the final encoded symbols; partly because further renormalization is required – for example, MSBs might agree but the range is still larger than 214 – and partly because there may be unresolved carries.

The following four-stage process adequately flushes the encoder by outputting remaining resolved MSBs, resolving remaining straddle conditions, flushing carry bits and finally byte aligning the output with padding bits.

while ((IvlLow + IvlRange − 1) ^ IvlLow < 0x8000) {
  writeBit(Bit(IvlLow, 15))
  for (; IvlCarry > 0; IvlCarry−−)
    writeBit(¬Bit(IvlLow, 15))
  IvlRange <<= 1
  IvlLow <<= 1
  IvlLow &= 0xFFFF
}

while ((IvlLow & 0x4000) && ((IvlLow + IvlRange − 1) & 0x4000)) {
  carry++
  IvlLow ^= 0x4000
  IvlLow &= 0x7FFF
  IvlLow <<= 1
  IvlRange <<= 1
}

writeBit(Bit(IvlLow, 15))
for (; IvlCarry > 0; IvlCarry−−)
  writeBit(¬Bit(IvlLow, 15))

byte_align()

- (normative)
Partial decoding and spatial scalability

  - General

A decoder may decode and reconstruct slice geometry coded by the occupancy tree at a lower precision than specified by 9.2.  When used in conjunction with level of detail attribute scalability (lod_scalability_enabled is 1), attributes may be decoded and reconstructed for the lower precision geometry.

This annex specifies the lower precision slice geometry a decoder shall output for a partially decoded occupancy tree that excludes nodes smaller than a minimum node size (D.2).  This annex shall only apply when the minimum node size is greater than the unit cube.

If LoD attribute scalability is enabled, attribute values shall be determined by partially decoding the slice attribute data in accordance with D.3.  Otherwise, attribute values shall not be output by a decoder.

  - Partial slice geometry

    - General

A decoder shall generate a lower-precision slice geometry that is equivalent to that specified by subclause D.2 at the end of the geometry decoding specified by Clause 9.

- While the lower-precision slice geometry is specified as a post-process, it is equivalent to halting the decoding of an occupancy tree at the start of the tree level where NodeSizeLog2, is equal to MinNodeSizeLog2, quantizing the positions of any points coded by direct nodes and eliminating any coincident points.

The lower-precision slice geometry is specified in terms of the following variables:

- The variable MinNodeSizeLog2, an application-specific minimum occupancy tree node size that specifies the precision of output points.

- The array PartialPtIdx that maps points in the lower-precision slice geometry to point indexes in PointPos; PartialPtIdx[ idx ] is a point's index into the array PointPos.

- The variable PartialPtCnt, a cumulative count of points in the lower-precision slice geometry.

    - Selection of partial point positions

To select the points that form the lower-precision slice geometry:

- Spatially partition the full slice geometry into a lattice 2MinNodeSizeLog2 sized cubic blocks.

- Select one point from each occupied block, recording its PointPos index.

In the following, the sparse array blkPtCnt identifies the blocks of the partitioned geometry; blkPtCnt[ ps ][ pt ][ pv ] greater than 0 indicates that the quantized point position ( ps, pt, pv ) is already present in the output partial slice geometry.  Unset elements of blkPtCnt are inferred to be 0.

PartialPtCnt = 0
for (ptIdx = 0; ptIdx < PointCnt; ptIdx++) {
  ps = PointPos[ptIdx][0] >> MinNodeSizeLog2
  pt = PointPos[ptIdx][1] >> MinNodeSizeLog2
  pv = PointPos[ptIdx][2] >> MinNodeSizeLog2

  blkPtCnt[ps][pt][pv]++
  if (blkPtCnt[ps][pt][pv] > 1)
    continue

  PartialPtIdx[PartialPtCnt++] = ptIdx
}

    - Partial point positions

The points selected for the lower-precision slice geometry shall be quantized according to the minimum node size:

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  for (k = 0; k < 3; k++)
    PointPos[ptIdx][k] = (PointPos[ptIdx][k] >> MinNodeSizeLog2) << MinNodeSizeLog2

If MinNodeSizeLog2 is greater than 1, points shall be centred within their corresponding block:

for (ptIdx = 0; ptIdx < PointCnt; ptIdx++)
  for (k = 0; k < 3; k++)
    PointPos[ptIdx][k] |= (MinNodeSizeLog2 > 1) << MinNodeSizeLog2 − 1

    - Output points

Partial decoding shall be equivalent to only outputting the selected points; i.e. PartialPtIdx[ 𝑖 ], 𝑖 ∈ 0 .. PartialPtCnt − 1.

  - Partial attribute decoding

    - General

Slice attributes with lod_scalability_enabled equal to 1 shall be reconstructed in accordance with Clause 10; but:

- using the lower-precision slice geometry (D.2.3);

- constructing the finest detail level in accordance with D.3.2 instead of 10.6.5.2.

- This is equivalent to not reconstructing LoDs with Lvl < MinNodeSizeLog2.

    - The finest detail level

The finest detail level shall comprise the lower-precision slice geometry specified by D.2:

for (i = 0; i < PartialPtCnt; i++)
  LodPtIdx[0][i] = PartialPtIdx[i]
LodPtCnt[0] = PartialPointCnt

The point indexes of the finest detail level shall be sorted in ascending order of their respective Morton-coded attribute coordinates.

- (normative)
Fine granularity slices

  - General

This annex specifies the fine granularity slices decoder.  This annex shall only apply when fgs_layer_group_enabled equals to 1.

  - Coded point cloud format

    - Fine granularity slices

A slice can comprise FGSs, where each FGS is mapped one-to-one to geometry or attribute of a subgroup in a layer-group.

The slice of FGSs is identified by a common slice identifier (slice_id).

An FGS is identified by a pair consisting of a layer-group index (layer_group_idx) and a subgroup index (subgroup_idx).  When FGSs have the same pair of indexes, they shall be either the geometry or attribute of the subgroup indicated by the layer-group index and the subgroup index.

Every FGS shall include a GDU or DGDU that codes the partial slice geometry, or ADUs or DADUs that code the partial slice attributes.

The first FGS in a slice shall be FGS of GDU.  This FGS may be followed by FGSs of DGDU that depends on the previously decoded GDU and DGDU.

The first FGS in a slice attribute shall be FGS of ADU. This FGS may be followed by FGSs of DADU that depends on the previously decoded ADU and DADU.  FGSs of ADUs and DADUs shall be occur after FGSs of GDU and DGDU.

    - Layer-group structure

In the layer-group structure, nodes in the occupancy tree are grouped into a layer-group and a subgroup.

A layer-group is a group of consecutive tree levels, where each tree levels shall belong to only one layer-group.  The minimum depth of a subgroup shall be the maximum depth of its parent subgroup plus 1, or 0 for the root layer-group.  The maximum depth of a subgroup shall be the minimum depth of its child subgroup minus 1, or the greatest depth of the occupancy tree for the last layer-group.  A layer-group is identified by a layer-group index (layer_group_idx).

A subgroup is a spatial subset of a layer-group, where a node in a tree level shall belong to only one subgroup in a layer-group.  The range of the position of the nodes in a subgroup shall be described by a bounding box, which must not overlap with the bounding boxes of other subgroups in the same layer-group.  The set of the nodes in all subgroups in a layer-group shall be identical to the set of the nodes in the layer-group. There is only one subgroup for the root layer-group.  A subgroup in a layer-group is identified by a subgroup index (subgroup_idx).

In Figure E.1 a), a layer-group structure of an occupancy tree with maximum depth of 8 is depicted.  In this example, three layer-groups are defined and each layer-group comprises tree levels from depth 0 to 3, 4 to 6, and 7 and 8, respectively.  Except for the root layer-group, layer-groups may comprise subgroups.  A subgroup is indicated by the pair consisting of the layer-group index and the subgroup index.  For example, the root layer-group is indicated by (0, 0).

In Figure E.1 b), the spatial region of subgroups from Figure E.1 a) are depicted by a rectangular bounding box in a xy-plane.  When the bounding box of a subgroup in a layer-group is a superset of the bounding box of one or more subgroups in the next layer-group, the subgroups in adjacent layer-groups are in a parent and child relationship.  In this example, subgroup (0,0) is the parent of subgroups (1,0) and (1,1). Similarly, subgroups (2,0) and (2,1) are children of subgroup (1,0).  Each subgroup, indicated by a pair of layer-group index and subgroup index, is in different FGSs.

Key

| Sl,n | Subgroup 𝑛, associated with layer-group l |
| --- | --- |
| di | Occupancy tree depth i |

Figure E.1 —A layer-group structure of an occupancy tree

  - Syntax and semantics for fine granularity slices

    - Syntax in tabular form

      - General

The fine granularity slice syntax structures and the syntax elements within these structures are specified in E.3.1.  Any values that are not specified in the tables shall not be present in the bitstream unless otherwise specified in this document.

      - FGS parameter sets

        - FGS attribute parameter syntax

| fgs_attr_parameter( ) { | Descriptor |
| --- | --- |
| fgs_attr_ref_id_present | u(1) |
| } |  |

        - FGS layer-group structure inventory syntax

| fgs_layer_group_structure_inventory( ) { | Descriptor | Semantics |
| --- | --- | --- |
| lgsi_seq_parameter_set_id | u(4) | E.3.2.2.3 |
| lgsi_frame_ctr_lsb_bits | u(5) | E.3.2.2.3 |
| lgsi_frame_ctr_lsb | u(v) | E.3.2.2.3 |
| lgsi_num_slice_minus1 | u(16) | E.3.2.2.3 |
| if(lgsi_num_slices_minus1 ≥ 0) { |  |  |
| for(sId = 0; sId ≤ lgsi_num_slices_minus1; sId++) { |  |  |
| lgsi_slice_id[sId] | ue(v) | E.3.2.2.3 |
| lgsi_num_layer_groups_minus1[sId] | u(8) | E.3.2.2.3 |
| lgsi_subgroup_bbox_origin_bits_minus1 | ue(v) | E.3.2.2.3 |
| lgsi_subgroup_bbox_size_bits_minus1 | ue(v) | E.3.2.2.3 |
| for(gId = 0; gId ≤ lgsi_num_layer_groups_minus1[sId]; gId++) { |  |  |
| lgsi_layer_group_id[sId][gId] | u(8) | E.3.2.2.3 |
| lgsi_num_layers_minus1[sId][gId] | u(8) | E.3.2.2.3 |
| lgsi_num_subgroups_minus1[sId][gId] | u(16) | E.3.2.2.3 |
| for(sgId = 0; sgId ≤ lgsi_num_subgroups_minus1[sId][gId]; sgId++) { |  |  |
| lgsi_subgroup_id[sId][gId][sgId] | u(16) | E.3.2.2.3 |
| lgsi_parent_subgroup_id[sId][gId][sgId] | u(16) | E.3.2.2.3 |
| for(k = 0; k < 3; k++) |  |  |
| lgsi_subgroup_bbox_origin_xyz[sId][gId][sgId][k] | u(v) | E.3.2.2.3 |
| for(k = 0; k < 3; k++) |  |  |
| lgsi_subgroup_bbox_size_xyz[sId][gId][sgId][k] | u(v) | E.3.2.2.3 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| lgsi_origin_bits_minus1 | ue(v) | E.3.2.2.3 |
| for(k = 0; k < 3; k++) |  |  |
| lgsi_origin_xyz[k] | se(v) | E.3.2.2.3 |
| lgsi_origin_log2_scale | ue(v) | E.3.2.2.3 |
| byte_alignment( ) |  |  |
| } |  |  |

      - FGS geometry data unit

        - FGS geometry data unit parameter syntax

| fgs_geometry_data_unit_parameter( ) { | Descriptor | Semantics |
| --- | --- | --- |
| num_layer_groups_minus1 | u(8) | E.3.2.3.2 |
| for( i = 0; i ≤ num_layer_groups_minus1; i++ ) { |  |  |
| layerGroupIdx = i |  |  |
| num_layers_minus1[ i ] | u(8) | E.3.2.3.2 |
| subgroup_enabled[ i ] | u(1) | E.3.2.3.2 |
| } |  |  |
| fgs_subgroup_enabled | u(1) | E.3.2.3.2 |
| if(fgs_subgroup_enabled ) { |  |  |
| subgroup_bbox_origin_bits_minus1 | ue(v) | E.3.2.3.2 |
| subgroup_bbox_size_bits_minus1 | ue(v) | E.3.2.3.2 |
| } |  |  |
| for( k = 0; k ≤ 3; k++ ) |  |  |
| root_subgroup_bbox_size_log2[ k] | u(8) | E.3.2.3.2 |
| for( i = 1; i ≤ num_layer_groups_minus1; i++ ) |  |  |
| num_subsequent_subgroups[ i ] | u(8) | E.3.2.3.2 |
| if( occtree_planar_enabled && ¬geom_angular_enabled) { |  |  |
| for( i = 0; i ≤ num_layers_minus1[ 0 ]; i++ ) |  |  |
| subgroup_planar_eligibility_by_density[ i ] | u(1) | E.3.2.3.2 |
| } |  |  |
| } |  |  |

        - FGS occupancy tree syntax

| fgs_occupancy_tree( startDepth, endDepth) { | Descriptor | Semantics |
| --- | --- | --- |
| OccQpSubtreeDepth = occtree_depth_minus1 + 1 |  | 9.2.14.4 |
| for( Dpth = startDepth; Dpth ≤ endDepth; Dpth++ ) { |  |  |
| occupancy_tree_level( Dpth ) |  |  |
| if( Dpth + 1 > OcctreeEntropyStreamDepth ) |  | 9.2.3 |
| occtree_end_of_entropy_stream | ae(v) | 9.2.3 |
| } |  |  |
| } |  |  |

        - FGS dependent geometry data unit syntax

| dependent_geometry_data_unit( ) { | Descriptor |
| --- | --- |
| dependent_geometry_data_unit_header( ) |  |
| fgs_occupancy_tree(startDepth, endDepth) |  |
| geometry_data_unit_footer(occtreeMaxDepthMinus1 ) |  |
| } |  |

        - FGS dependent geometry data unit header syntax

| dependent_geometry_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| dgdu_geometry_parameter_set_id | u(4) | E.3.2.3.4 |
| dgdu_slice_id | ue(v) | E.3.2.3.4 |
| layer_group_id | u(8) | E.3.2.3.4 |
| if( subgroup_enabled[ layer_group_id] ) { |  |  |
| subgroup_id | u(8) | E.3.2.3.4 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| subgroup_bbox_origin_xyz[ 𝑘 ] | u(v) | E.3.2.3.4 |
| for( 𝑘 = 0; 𝑘 < 3; 𝑘++ ) |  |  |
| subgroup_bbox_size_xyz[ 𝑘 ] | u(v) | E.3.2.3.4 |
| } |  |  |
| ref_layer_group_id | u(8) | E.3.2.3.4 |
| if( subgroup_enabled[ layer_group_id] ) |  |  |
| ref_subgroup_id | u(8) | E.3.2.3.4 |
| subgroup_context_reference_indication_enabled | u(1) | E.3.2.3.4 |
| if( subgroup_context_reference_indication_enabled) |  |  |
| for( i = 1; i ≤ num_layer_groups_minus1; i++ ) |  |  |
| num_subsequent_subgroups[ i ] | u(8) | E.3.2.3.2 |
| if( occtree_planar_enabled && ¬geom_angular_enabled) { |  |  |
| for( i = 0; i ≤ num_layers_minus1[ layer_group_id ]; i++ ) |  |  |
| subgroup_planar_eligibility_by_density[ i ] | u(1) | E.3.2.3.4 |
| } |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

      - FGS attribute data unit

        - FGS attribute data unit parameter syntax

| fgs_attribute_data_unit_parameter( ) { | Descriptor | Semantics |
| --- | --- | --- |
| for( i = 1; i ≤ num_layer_groups_minus1; i++ ) |  |  |
| num_subsequent_subgroups[ i ] | u(8) | E.3.2.3.2 |
| subgroup_weight_adjustment_enabled | u(1) | E.3.2.4.2 |
| if( subgroup_weight_adjustment_enabled) { |  |  |
| subgroup_weight_adj_coeff_a | se(v) | E.3.2.4.2 |
| subgroup_weight_adj_coeff_b | se(v) | E.3.2.4.2 |
| } |  |  |
| } |  |  |

        - FGS dependent attribute data unit syntax

| dependent_attribute_data_unit( ) { | Descriptor |
| --- | --- |
| dependent_attribute_data_unit_header( ) |  |
| if( attr_coding_type == 1 ¦¦ attr_coding_type == 2 ) |  |
| attribute_coeffs( ) |  |
| byte_alignment( ) |  |
| } |  |

        - FGS dependent attribute data unit header syntax

| dependent_attribute_data_unit_header( ) { | Descriptor | Semantics |
| --- | --- | --- |
| dadu_attr_parameter_set_id | u(4) | E.3.2.4.4 |
| dadu_sps_attr_idx | ue(v) | E.3.2.4.4 |
| dadu_slice_id | ue(v) | E.3.2.4.4 |
| dadu_layer_group_id | u(8) | E.3.2.4.4 |
| if(subgroup_enabled[dadu_layer_group_id]) |  |  |
| dadu_subgroup_id | u(8) | E.3.2.4.4 |
| if(fgs_ attr_ref_id_present ) |  |  |
| attr_ref_layer_group_id | u(8) | E.3.2.4.4 |
| if(subgroup_enabled[dadu_layer_group_id]) |  |  |
| if(fgs_ attr_ref_id_present) { |  |  |
| attr_ref_subgroup_id | u(8) | E.3.2.4.4 |
| attr_subgroup_context_reference_indication_enabled | u(1) | E.3.2.4.4 |
| if(attr_ subgroup_context_reference_indication_enabled) |  |  |
| for( i = 1; i ≤ num_layer_groups_minus1; i++ ) |  |  |
| num_subsequent_subgroups[ i ] | u(8) | E.3.2.3.2 |
| } |  |  |
| if( last_comp_pred_enabled && AttrDim == 3) |  |  |
| for( dpth = 0; dpth ≤ num_layers_minus1[dadu_layer_group_id]; dpth++ ) |  |  |
| last_comp_pred_coeff_diff[ dpth ] | se(v) | 10.6.10.1 |
| if( inter_comp_pred_enabled ) |  |  |
| for( dpth = 0; dpth ≤ num_layers_minus1[dadu_layer_group_id]; dpth++ ) |  |  |
| for( 𝑐 = 1; 𝑐 < AttrDim; 𝑐++) |  |  |
| inter_comp_pred_coeff_diff[ dpth ][ 𝑐 ] | se(v) | 10.6.10.1 |
| if( attr_qp_offsets_present ) |  |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++) |  |  |
| attr_qp_offset[ qc ] | se(v) | 10.7.1 |
| attr_qp_layers_present | u(1) | 10.7.1 |
| if( attr_qp_layers_present ) { |  |  |
| attr_qp_layer_cnt_minus1 | ue(v) | 10.7.1 |
| for( dpth = 0; dpth ≤ attr_qp_layer_cnt_minus1; dpth++ ) |  |  |
| for( qc = 0; qc < Min( 2, AttrDim ); qc++) |  |  |
| attr_qp_layer_offset[ dpth ][ qc ] | se(v) | 10.7.1 |
| } |  |  |
| subgroup_weight_adjustment_enabled | u(1) | E.3.2.4.4 |
| if( subgroup_weight_adjustment_enabled) { |  |  |
| subgroup_weight_adj_coeff_a | se(v) | E.3.2.4.4 |
| subgroup_weight_adj_coeff_b | se(v) | E.3.2.4.4 |
| } |  |  |
| byte_alignment( ) |  |  |
| } |  |  |

    - Semantics

      - General

The semantics associated with the fine granularity slice syntax structures and with the syntax elements within these structures are specified either in E.3.2 or in the subclause identified by the semantics column of the syntax table.

When the semantics of a syntax element are specified in tabular form, any values that are not specified in the table(s) shall not be present in the bitstream unless otherwise specified in this document.

General constraints on syntax element values are specified in Annex A.

      - FGS parameter sets

        - FGS attribute parameter semantics

fgs_attr_ref_id_present specifies whether (when 1) or not (when 0) the context references of the attribute FGS are present in the DADU header.  When fgs_attr_ref_id_present equals to 1 indicates the context reference of the attribute FGSs will be indicated by the attr_ref_layer_group_id and attr_ref_subgroup_id, and the context state usage in the followed attribute slices is indicated by the attr_subgroup_context_reference_indication_enabled in the DADU header.  When fgs_attr_ref_id_present is equal to 0, the context reference and the context state usage shall be inherited from the geometry FGS whose layer_group_id and subgroup_id are identical to dadu_layer_group_id and dadu_subgroup_id of the current dependent attribute FGS, respectively.

        - FGS layer-group structure inventory semantics

A layer-group structure inventory, when present, contains metadata that defines the spatial region and parent-child relationship of each enumerated layer-groups and subgroups.  Each layer-group and subgroup is identified by either an implicit or explicit layer-group id and subgroup id.

A layer-group structure inventory shall apply from the next coded point cloud frame that follows the layer-group structure inventory.  It shall remain valid until it is replaced by another layer-group structure inventory.

A layer-group structure inventory shall occur before the first GDU of the coded point cloud frame from which it applies.  It shall not occur before the last DU of any coded point cloud frame that precedes that from which it applies in data unit order.

lgsi_seq_parameter_set_id identifies that active SPS by its sps_seq_parameter_set_id.

lgsi_frame_ctr_lsb_bits specifies the length in bits of the syntax element lgsi_frame_ctr_lsb.  It is a requirement of bitstream conformance that lgsi_frame_ctr_lsb_bits shall be equal to frame_ctr_lsb_bits of the active SPS.

lgsi_frame_ctr_lsb specifies the lgsi_frame_ctr_lsb_bits least significant bits of FrameCtr from which the group structure inventory is valid.  A layer-group structure inventory remains valid until it is replaced by another layer-group structure inventory.

lgsi_num_slices_minus1 plus 1 specifies the number of slices present in the layer-group structure inventory.

lgsi_slice_id[sId] specifies the slice id of the sId-th slice within the layer-group structure inventory.

lgsi_num_layer_groups_minus1[sId] plus 1 specifies the number of layer-group in the sId-th slice.

lgsi_subgroup_bbox_origin_bits_minus1[sId] plus 1 is the length in bits of the syntax elements lgsi_subgroup_bbox_origin_xyz in the sId-th slice.

lgsi_subgroup_bbox_size_bits_minus1[sId] plus 1 is the length in bits of the syntax elements lgsi_subgroup_bbox_size_xyz in the sId-th slice.

lgsi_layer_group_id[sId][gId] specifies the indicator of a layer-group.  The range of lgsi_layer_group_id shall be in the range of 0 to lgsi_num_layer_groups_minus1.  It is a requirement of bitstream conformance that all values of lgsi_layer_group_id of gId-th layer-group in the sId-th slice are unique within a layer-group structure inventory.

lgsi_num_layers_minus1[sId][gId] plus 1 specifies the number of layers in the layer-group of gId-th layer-group in the sId-th slice.

lgsi_num_subgroups_minus1[sId][gId] plus 1 specifies the number of subgroups in the gId-th layer-group in the sId-th slice.

lgsi_subgroup_id[sId][gId][sgId] specifies the indicator of a subgroup. The range of lgsi_subgroup_id shall be in 0 to lgsi_num_subgroups_minus1[sId][gId].

lgsi_parent_subgroup_id[sId][gId][sgId] specifies the indicator of a parent subgroup of the sgId-th subgroup in the gId-th layer-group in the sId-th slice.  The range of lgsi_parent_subgroup_id shall be in 0 to lgsi_num_subgroups_minus1[sId][gId-1].

lgsi_subgroup_bbox_origin_xyz[sId][gId][sgId][ 𝑘 ] specifies the 𝑘-th XYZ coordinate of the bounding box of the sgId-th subgroup in the gId-th layer-group in the sId-th slice.

lgsi_subgroup_bbox_size_xyz[sId][gId][sgId][ 𝑘 ] specifies the 𝑘-th XYZ dimension of the bounding box of the sgId-th subgroup in the gId-th layer-group in the sId-th slice.

lgsi_origin_bits_minus1 plus 1 specifies the length in bits of the lgsi_origin_xyz syntax elements.

lgsi_origin_xyz[ 𝑘 ] indicate the XYZ origin of the sequence.  The value of lgsi_origin_xyz[k] should be equal to seq_origin_xyz[ 𝑘 ].

lgsi_origin_log2_scale indicates a scaling factor to scale components of lgsi_origin_xyz.  The value of lgsi_origin_log2_scale should be equal to seq_origin_log2_scale.

      - FGS geometry data unit

        - FGS geometry data unit semantics

When fgs_layer_group_enabled is equal to 1, a GDU conveys the partial slice geometry of a root layer-group and associated slice information.  A GDU of fine granularity slices comprises a GDU header, geometry coded using an occupancy tree, and a GDU footer.  layer_group_id for a GDU of fine granularity slices shall be inferred to be 0.

        - FGS geometry data unit parameter semantics

num_layer_groups_minus1 plus 1 specifies the number of layer-groups where the layer-group represents a group of consecutive tree levels within the occupancy tree.  num_layer_groups_minus1 shall be in the range of 0 to the number of coding tree layers.

num_layers_minus1[ i ] plus 1 specifies the number of tree levels in the i-th layer-group where i represents layer-group index of the i -th layer-group.  The total number of layer-groups shall be derived by adding all (num_layers_minus1[ i ] + 1) for i equal to 0 to num_layer_groups_minus1[ i ].

subgroup_enabled[ i ] equals to 1 specifies that the i-th layer-group comprises two or more subgroups where i represents layer-group index of the i -th layer-group.  subgroup_enabled[ i ]  equals to 0 specifies that the i-th layer-group comprises a subgroup.

When subgroup_enabled[ i ] equals to 1, the aggregation of the nodes in each subgroup in the i-th layer-group shall be identical to the set of nodes in the layer-group.

When subgroup_enabled[ i ] is equal to 1, subgroup_enabled[ j ] shall equal to 1 when j is greater than i.

fgs_subgroup_enabled equals to 1 specifies that any layer-group in FGS comprises two or more subgroups.  fgs_subgroup_enabled equals to 0 specifies that all layer-groups comprise a subgroup.

fgs_subgroup_enabled = 0
for (
i := 0; i <= num_layer_groups_minus1; i++)
  fgs_subgroup_enabled |= subgroup_enabled[i]

subgroup_bbox_origin_bits_minus1 plus 1 specifies the length in bits of the syntax elements subgroup_ bbox_origin.

subgroup_bbox_size_bits_minus1 plus 1 is the length in bits of the syntax elements subgroup_bbox_size.

root_subgroup_bbox_size_log2[ k ] specifies the size of the bounding box of the root subgroup of the coded occupancy tree.  MaxVec(root_subgroup_bbox_size_log2) shall be the number of coded tree layers from root to the leaf layer.

When fgs_layer_group_enabled is equal to 1, the value of occtreeMaxDepthMinus1 is set as

occtreeMaxDepthMinus1 = MaxVec(root_subgroup_bbox_size_log2)- 1

The number of missing layers of a subgroup shall be the difference between occtreeMaxDepthMinus1 plus 1 and number of levels of decoded occupancy tree.  The number of missing layers of a subgroup shall be used to derive the sampling direction in the subgroup LoD generation (E.6.3.3.1) or to compensate the geometry position of a node in the intermediate layers (E.8.2.3).

num_subsequent_subgroups[ i ] specifies the number of the subsequent dependent data units belong to the i-th layer-group which reference the context state of the current data unit.  When not present, the value of num_subsequent_subgroups[ i ] is inferred to be zero.

subgroup_planar_eligibility_by_density[ i ] equals to 1 indicates that planar eligibility is enabled for the (i + startDepth)-th depth of the current subgroup.  subgroup_planar_eligibility_by_density[ i ] equals to 0 indicates that the planar eligibility is disabled for the (i + startDepth)-th depth of the current subgroup.  When not present, subgroup_planar_eligibility_by_density[ i ] is inferred to 0.

        - FGS dependent geometry data unit semantics

A DGDU conveys partial slice geometry and associated fine granularity slice information such as a pair of layer-group index and subgroup index, subgroup bounding box, and context state reference index.  A DGDU comprises a DGDU header, geometry coded using an occupancy tree, and a GDU footer.

When fgs_layer_group_enabled equals to 1 and layer_group_id greater than 0, slice_num_points_minus1 plus 1 shall equal to the number of nodes at the maximum depth of tree levels in the DGDU.

        - FGS dependent geometry data unit header semantics

dgdu_geometry_parameter_set_id specifies the active GPS indicated by gps_geom_parameter_set_id. The value of dgdu_geometry_parameter_set_id shall be identical to the value of gdu_geometry_parameter_set_id in the same slice.

dgdu_slice_id specifies the slice to which the current dependent geometry data unit belongs.

layer_group_id specifies the indicator of a layer-group in the layer-group structure related to the slice.  The range of layer_group_id shall be in the range 0 .. num_layer_groups_minus1.  When not present, layer_group_id shall be inferred to be 0.

subgroup_id specifies the indicator of the subgroup in the layer-group referred by layer_group_id.  The range of subgroup_id shall be in the range 0 .. num_subgroups_minus1[layer_group_id] where the subgroup_id represent a partial region in a layer-group specified by layer_group_id.  When not present, subgroup_id shall be inferred to be 0.

subgroup_bbox_origin_xyz[ 𝑘 ] specifies the minimum 𝑘-th XYZ component position of the subgroup bounding box of the subgroup indicated by the pair of layer-group index layer_group_id and the subgroup index subgroup_id.

subgroup_bbox_size_xyz[ 𝑘 ] specifies the 𝑘-th XYZ size component of the subgroup bounding box of the subgroup indicated by the pair of layer-group index layer_group_id and the subgroup index subgroup_id.

ref_layer_group_id specifies the indicator of the layer-group identifier of the context reference of the current dependent data unit.  The range of the ref_layer_group_id shall be in the range 0 .. num_layer_groups_minus1.

ref_subgroup_id specifies the indicator of the subgroup identifier of the context reference of the current dependent data unit.  When not present, ref_subgroup_id is inferred to be 0.

The reference context state is identified by the pair of layer-group index ref_layer_group_id and subgroup index ref_subgroup_id.

subgroup_context_reference_indication_enabled equals to 1 indicates that the context state of the current data unit will be used to initialize one or more subsequent data units.  subgroup_context_reference_indication_enabled equals to 0 indicates that the context state of the current data unit will not be used to initialize the subsequent data units.  When not present, subgroup_context_reference_indication_enabled is inferred to be 1.

subgroup_planar_eligibility_by_density[ i ] equals to 1 indicates that the planar eligibility is enabled for the i-th depth of the current subgroup.  subgroup_planar_eligibility_by_density equals to 0 indicates that the planar eligibility is disabled for the i-th depth of the current subgroup.  When not present, subgroup_planar_eligibility_by_density[ i ] is inferred to 0.

      - FGS attribute data unit

        - FGS attribute data unit semantics

An ADU codes attribute values for a single attribute in a slice or an FGS.  ADU is described in Subclause 7.4.4.1.

        - FGS attribute data unit parameter semantics

subgroup_weight_adjustment_enabled equals to 1 indicates that the subgroup weight adjustment coefficients subgroup_weight_adj_coeff_a and subgroup_weight_adj_coeff_b are present for the current subgroup corresponding to the current FGS.  subgroup_weight_adjustment_enabled equals to 0 indicates that the subgroup weight adjustment coefficients are not present and subgroup_weight_adj_coeff_a and subgroup_weight_adj_coeff_b are inferred to be 0.

subgroup_weight_adj_coeff_a and subgroup_weight_adj_coeff_b indicate the coefficient of subgroup weight adjustment.

        - FGS dependent attribute data unit semantics

A DADU codes attribute values for a single attribute in an FGS.  It comprises a DADU header and either attribute coefficients (attribute_coeffs) when transform coding is equal to 1 or 2.

        - FGS dependent attribute data unit header semantics

dadu_attr_parameter_set_id specifies the active APS by its aps_attr_parameter_set_id.

dadu_sps_attr_idx identifies the coded attribute by its index into the active SPS attribute list.

At the start of every DADU, the variable AttrIdx is set to dadu_sps_attr_idx:

AttrIdx = dadu_sps_attr_idx

The attribute coded by the DADU shall have at most three components.

dadu_slice_id specifies the value of the preceding GDU slice_id.

dadu_layer_group_id specifies the indicator of a layer-group in the layer-group structure related to the slice. The range of dadu_layer_group_id shall be in the range 0 .. num_layer_groups_minus1.  When not present, dadu_layer_group_id shall be inferred to be 0.

dadu_subgroup_id specifies the indicator of the subgroup in the layer-group referred by dadu_layer_group_id.  The range of dadu_subgroup_id shall be in the range 0 .. num_subgroups_minus1[dadu_layer_group_id] where the dadu_subgroup_id represent a partial region in a layer-group specified by dadu_layer_group_id.  When not present, dadu_subgroup_id shall be inferred to be 0.

attr_ref_layer_group_id specifies the indicator of the layer-group identifier of the context reference of the current DADU.  The range of the attr_ref_layer_group_id shall be in the range of 0 to the dadu_layer_group_id of the current DADU.  When not present, attr_ref_layer_group_id is inferred to be ref_layer_group_id of the geometry FGS whose layer_group_id and subgroup_id in DGDU are identical to dadu_layer_group_id and dadu_subgroup_id of the current attribute FGS.

attr_ref_subgroup_id specifies the indicator of the reference subgroup of the layer-group indicated by attr_ref_layer_group_id.  The range of the attr_ref_subgroup_id shall be in the range of 0 to num_subgroup_id_minus1 of the layer-group indicated by attr_ref_layer_group_id.  When not present, attr_ref_subgroup_id is inferred to be ref_subgroup_id of the geometry FGS whose layer_group_id and subgroup_id in DGDU are identical to dadu_layer_group_id and dadu_subgroup_id of the current attribute FGS.

attr_subgroup_context_reference_indication_enabled equals to 1 indicates that the context state of the current DADU will be used to initialize one or more subsequent data units.  attr_subgroup_context_reference_indication_enabled equals to 0 indicates that the context state of the current attribute data unit will not be used to initialize the subsequent data units.  When not present, attr_subgroup_context_reference_indication_enabled is inferred to be 1.

  - Decoding process

    - General

The decoding process is described in Clause 8.  This annex specifies the difference in the decoding process for the fine granularity slice.

    - Fine granularity slice decoding process

      - General

When fgs_layer_group_enabled equals to 1, an FGS in a coded point cloud frame shall be decoded as follows:

- Point positions are decoded from one GDU and zero or more DGDUs as specified by E.4.2.3.

- Default attribute values are set for each attribute as specified by 8.3.4.

- Point attributes are decoded from one ADU and zero or more DADUs as specified by E.4.2.4.  The ADU and DADUs shall be decoded after decoding of the GDU and DGDUs those are indicated by the identical pair of subgroup index layer_group_id and subgroup_id.

- The decoded point positions are offset and the output point count incremented as specified by 8.3.6.

      - State variables

FGS decoding is specified in terms of the following state variables:

- The variable startDepth, a start depth of tree level in a data unit

- The variable endDepth, an end depth of tree level in a data unit

      - FGS geometry decoding process

The GDU shall be decoded before all of the DGDUs in a slice. DGDUs of child subgroups shall be decoded after DGDU of the parent subgroup.

When decoding a GDU, startDepth and  endDepth are set to 0 and (num_layers_minus1[ 0] + 1), respectively.

When decoding a DGDU, startDepth is set to accumulated values of (num_layers_minus1[ 𝑘 ] + 1) where 𝑘 is in the range of 0 .. layer_group_id – 1.  endDepth is set to accumulated value of (num_layers_minus1[ 𝑘 ] + 1) where 𝑘 is in the range of 0 .. layer_group_id.

The expression SubgroupNodePos[ layerGroupIdx ][ subgroupIdx ][ nodeIdx ][ 𝑘 ] is an alias into the nodes at endDepth-th depth of a subgroup identified by layerGroupIdx and subgroupIdx, where layerGroupIdx is equal to layer_group_id and subgroupIdx is equal to subgroup_id.

SubgroupNodePos[layerGroupIdx ][ subgroupIdx ][nodeIdx][k] := OccNodeLoc[ endDepth ][nodeIdx][k]

The expression SubgroupNodeCnt[ layerGroupIdx ][ subgroupIdx ] is an alias into the number of nodes at endDepth-th depth of a subgroup identified by layerGroupIdx and subgroupIdx.

SubgroupNodeCnt[layerGroupIdx ][ subgroupIdx ] := OccNodeCnt[endDepth]

The expression SubgroupBBoxMin[ layerGroupIdx ][ subgroupIdx ] and SubgroupBBoxMax [ layerGroupIdx ][ subgroupIdx ] are alias into the minimum and maximum point position of the bounding box of a subgroup identified by layerGroupIdx and subgroupIdx, respectively.

SubgroupBBoxMin[layerGroupIdx][subgroupIdx][k] := subgroup_bbox_origin_xyz[k]
SubgroupBBoxMax[layerGroupIdx][subgroupIdx][k] := subgroup_bbox_origin_xyz[k] + subgroup_bbox_size[k]

The sparse array SubgroupOccNeighPatEq0[ CurrLayerGroupIdx ][ CurrSubgroupIdx ][ ns ][ nt ][ nv ] identifies whether the identified node of the parent subgroup has no nodes present in its occupied neighbourhood pattern.

The sparse array SubgroupOccNodeChildCnt[ CurrLayerGroupIdx ][ CurrSubgroupIdx ][ k ][ ns ][ nt ][ nv ] identifies the number of child node of the parent subgroup.

The expression SubgroupDirectNodePointCnt[ layerGroupIdx ][ subgroupIdx ] is an alias into the number of the points of the direct nodes at endDepth-th depth of a subgroup identified by layerGroupIdx and subgroupIdx.

When layer_group_id is equal to num_layer_groups_minus1, the position of nodes in the subgroup are copied to the output point cloud.

if (layer_group_id == num_layer_groups_minus1) {
  for (i = 0; i < SubgroupNodeCnt[layerGroupIdx ][ subgroupIdx ]; i++, PointCnt++)
    for (k = 0; k < 3; k++)
      PointPos[PointCnt][k] = SubgroupNodePos[layerGroupIdx ][ subgroupIdx ][i][k]
}

Node positions shall be decoded and reconstructed as specified by  E.5.

      - FGS attribute decoding process

The ADU shall be decoded before all of the DADUs in a slice.  DADUs of child subgroups shall be decoded after DADU of the parent subgroup.

The ADU and DADU shall be decoded and the reconstructed attribute values stored in the corresponding output point cloud attribute or the leaf nodes of the occupancy tree in the subgroup.

When decoding a ADU, startDepth is set to 0.

When decoding a DADU, startDepth is set to accumulated values of (num_layers_minus1[ 𝑘 ] + 1) where 𝑘 is in the range of 0 .. layer_group_id – 1.

DirectNodePointCnt is set to the number of coded points in direct nodes in each subgroup.

DirectNodePointCnt := SubgroupDirectNodePointCnt[layerGroupIdx][subgroupIdx]

The expression SubgroupNodeAttr[ layerGroupIdx ][ subgroupIdx ][ ptIdx ][ c ] is an alias into the output point cloud attribute array for the points in a subgroup identified by layerGroupIdx and subgroupIdx, where layerGroupIdx is equal to layer_group_id and subgroupIdx is equal to subgroup_id.

When layer_group_id is equal to num_layer_groups_minus1, the expression PointAttr[ ptIdx ][ 𝑐 ] is an alias into the output point cloud attribute array for the points in the FGS.

PointAttr[ptIdx][c] := RecCloudAttr[RecCloudPointCnt + ptIdx][AttrIdx][c]

Otherwise, the expression PointAttr[ ptIdx ][ 𝑐 ] is an alias into the decoded points or the leaf nodes of the occupancy tree in the subgroup.

If ptIdx < DirectNodePointCnt:

PointAttr[ptIdx][c] := RecCloudAttr[RecCloudPointCnt + ptIdx][AttrIdx][c]

Otherwise :

PointAttr[ptIdx][c] := SubgroupNodeAttr[layerGroupIdx][subgroupIdx][ptIdx- DirectNodePointCnt][c]

Point attributes shall be decoded and reconstructed as specified by E.6.

  - Fine granularity slice geometry

    - General

Slice geometry shall be parsed and reconstructed point position from a coded occupancy tree in accordance with Clause 9.  This annex specifies the difference of the fine granularity slice geometry decoder for parsing and reconstruction of point position.

When fgs_layer_group_enabled equals to 1, the reconstructed geometry of each subgroup is stored in the arrays SubgroupNodePos (E.4.2.3).

    - Partial tree of occupancy tree

      - General

When geom_tree_type is 0, slice geometry shall be parsed and reconstructed point position from a coded occupancy tree in accordance with Subclause 9.2.

When fgs_layer_group_enabled equals to 1, a partial occupancy tree represents the fine granularity slice geometry as a partial tree of occupancy tree nodes in terms of spatial region and depth.  Fine granularity slice geometry shall be parsed or reconstructed point position from a coded partial occupancy tree as specified by E.5.2.

      - Coded occupancy tree

        - General tree structure

An occupancy tree node shall identify the presence of at least one point contained within the volume of an axis-aligned cuboid.

When fgs_layer_group_enabled equals to 1, an occupancy tree node shall identify the presence of at least one point contained within the volume of a cube.  The volume is defined in the slice's coordinate system by an inclusive lower corner pmin and an exclusive upper corner pmax.  The volume edge lengths are non-negative integer powers of two.  A node's size, nodeSize, is synonymous with the volume dimensions pmax-pmin.

        - Tree traverse order

The coded occupancy tree shall be traversed in breadth-first order.  Traversal shall start from the top tree level.  All nodes in a tree level shall be sequentially traversed before proceeding to the next level.  Within a tree level, nodes shall be traversed in ascending Morton order of node location.

When fgs_layer_group_enabled equals to 1, the coded partial occupancy tree shall be traversed in breadth-first order in a subgroup.  Traversal shall start from the minimum depth of the subgroup.  All nodes in a tree level of the subgroup shall be sequentially traversed before proceeding to the next level.  Within a tree level of the subgroup, nodes shall be traversed in ascending Morton order of node location.

      - State representation

        - State variables

Traversal of the occupancy tree is specified in terms of the following state variables:

- The array OccNodeCnt; OccNodeCnt[ dpth ] is the cumulative count of nodes present at depth dpth. When fgs_layer_group_enabled equals to 1, OccNodeCnt[ dpth ] represents the cumulative count of nodes present at depth dpth of a subgroup.

- The array OccNodeLoc; OccNodeLoc[ dpth ][ nodeIdx ][ 𝑘 ] identifies the 𝑘-th location component of the nodeIdx-th coded node in the traversal order of the tree level at depth dpth. When fgs_layer_group_enabled equals to 1, OccNodeLoc[ dpth ][ nodeIdx ][ 𝑘 ] identifies the 𝑘-th location component of the nodeIdx-th coded node in the traversal order of the tree level at depth dpth of a subgroup.

      - Initialization

At the start of the occupancy tree syntax structure in GDU and DGDU, all elements of OccNodePresent, OccNodeLoc and OccNodeCnt are cleared.

When layer_group_id equals to 0, the arrays OccNodePresent, OccNodeLoc and OccNodeCnt are initialized by 9.2.5.2.

When layer_group_id is greater than 0, the arrays OccNodePresent, OccNodeLoc and OccNodeCnt are initialized by E.5.2.4.1 and E.5.2.4.2 to decode the occupancy tree continuous to the parent subgroup.

        - Parent subgroup detection

The layer-group index PrtLayerGroupIdx and the subgroup index PrtSubgroupIdx of a parent subgroup are inferred by using the parent-child subgroup relationship of SubgroupBBoxMin and SubgroupBBoxMax as follows.

CurrLayerGroupIdx := layer_group_id
PrtLayerGroupIdx 
:= CurrLayerGroupIdx – 1
CurrSubgroupIdx := subgroup_id

for (i=0; i<numSubgroups[PrtLayerGroupIdx]; i++) 
  if (pL[0] ≤ cL[0]) && (pL[1] ≤ cL[1]) && (pL[2] ≤ cL[2]) &&
      (pR[0] > cL[0]) && (pR[1] > cL[1]) && (pR[2] > cL[2]) {
    PrtSubgroupIdx = i
    break
  }
  where
    pL := SubgroupBBoxMin[PrtLayerGroupIdx][i]
    pR := SubgroupBBoxMax[PrtLayerGroupIdx][i]
    cL := SubgroupBBoxMin[CurrLayerGroupIdx][CurrSubgroupIdx]

        - Inheritance of parent subgroup output nodes

Given parent subgroup index and the bounding box information of decoded subgroups, OccNodeLoc[ startDepth ] is set as a subset of the parent subgroup output nodes SubgroupNodePos by selecting nodes in a region within the bounding box of the current subgroup depicted by SubgroupBBoxMin[ CurrLayerGroupIdx ][ CurrSubgroupIdx ] and SubgroupBBoxMax[ CurrLayerGroupIdx ][ CurrSubgroupIdx ].

count = 0
numMissingLayers = occtree_depth_minus1 + 1 – endDepth

for (i = 0; i < SubgroupNodeCnt[PrtLayerGroupIdx][PrtSubgroupIdx]; i++) {
  for (k = 0; k < 3; k++)
    nodePos[k] = SubgroupNodePos[ layerGroupIdx ][ subgroupIdx ][i][k] << numMissingLayers
  if ((nodePos[0] ≥ cL[0]) && (nodePos[1] ≥ cL[1]) && (nodePos[2] ≥ cL[2]) &&
      (nodePos[0] < cR[0]) && (nodePos[1] < cR[1]) && (nodePos[2] < cR[2]) ) {
    for(k = 0; k < 3; k++)
      OccNodeLoc[ startDepth ][count][k] = 
         SubgroupNodePos[ PrtLayerGroupIdx ][PrtSubgroupIdx ][i][k] 
    count++
  }
}
  where
    cL := SubgroupBBoxMin[CurrLayerGroupIdx][CurrSubgroupIdx]
    cR := SubgroupBBoxMax[CurrLayerGroupIdx][CurrSubgroupIdx]

OccNodeCnt[ startDepth ] is set as the number of nodes in OccNodeLoc[ startDepth ].

OccNodeCnt[ startDepth ] = count

OccNodePresent[ startDepth ][ ns ][ nt ][ nv ] is set to 1 at a tree location ( ns, nt, nv )  equal to OccNodeLoc[ startDepth ][ nodeIdx ].

for (nodeIdx = 0; nodeIdx < OccNodeCnt[ startDepth ]; nodeIdx ++) {
  ns = OccNodeLoc[startDepth][nodeIdx][0]
  nt = OccNodeLoc[startDepth][nodeIdx][1]
  nv = OccNodeLoc[startDepth][nodeIdx][2]
  OccNodePresent[startDepth][ns][nt][nv] = 1
}

      - Dictionary coding of occupancy_byte

        - Initial state

The dictionary state shall be initialized at the start of every GDU.

At the start of every DGDU, the dictionary state shall be initialized by the parsing state restoration process (11.5.3.2).

      - Bitwise occupancy coding

        - Contextualization

          - Initial state

The demi-CPMs shall be initialized at the start of every GDU.

At the start of every DGDU, the demi-CPMs shall be initialized according to the parsing state restoration process (E.7.3.1.2).

      - Planar occupancy coding

        - Per-axis eligibility

          - Condition

The expression PointDensity[dpth ] is a factor that identifies the density of the points in the tree level at depth dpth:

PointDensity[dpth ] := (slice_num_points_minus1 + 1 – DirectNodePointCnt) × 10 / OccNodeCnt[ dpth ]

When fgs_layer_group_enabled equals to 1, PointDensity[dpth ] is specified by subgroup_planar_eligibility_by_density in the GDUH or DGDUH.

PointDensity[dpth] = subgroup_planar_eligibility_by_density[dpth -  startDepth ]

      - Direct nodes

        - General

The number of points coded in direct nodes is counted cumulatively, DirectNodePointCnt.  The variable is initialized to 0 at the start of fine granularity slice.

        - Eligibility

          - Initial state

At the start of every occupancy_tree syntax structure, the OccNodeChildCnt array shall be cleared; all elements of OccNodeChildCnt are unset.

When fgs_layer_group_enabled equals to 1, at the start of every occupancy_tree syntax structure, the initial states of OccNeighPatEq0 and OccNodeChildCnt are set as follows.

if (layer_group_enabled && dpth == startDepth) {
    OccNeighPatEq0[ Dpth-1 ]
      = SubgroupOccNeighPatEq0[ PrtLayerGroupIdx ][ PrtSubgroupIdx ]
    OccNodeChildCnt[ Dpth-1 ] 
      = SubgroupOccNodeChildCnt[ PrtLayerGroupIdx ][ PrtSubgroupIdx ][0]
    OccNodeChildCnt[ Dpth-2 ]
      = SubgroupOccNodeChildCnt[ PrtLayerGroupIdx ][ PrtSubgroupIdx ][1]
}

          - State update after each coded occupancy tree node

This subclause applies at the end of every occupancy_tree_node syntax structure.

The number of child nodes and the presence of any nodes in the occupied neighbourhood pattern are recorded for use in subsequent eligibility decisions.

OccNodeChildCnt[Dpth][Ns][Nt][Nv] = direct_node ? 0 : OccChildCnt
OccNeighPatEq0[Dpth][Ns][Nt][Nv] = OccNeighPat == 0

If the node is eligible for direct coding, irrespective of the presence of occ_direct_node, the count of eligible nodes shall be incremented.

if (DirectModeEligible)
  DnEligibleCnt++

When fgs_layer_group_enabled equals to 1 and dpth is equal to endDepth, the SubgroupOccNeighPatEq0 and SubgroupOccNodeChildCnt are set as follows.

if (layer_group_enabled && dpth == endDepth){
    SubgroupOccNeighPatEq0[ LayerGroupIdx ][ SubgroupIdx ][ Ns ][ Nt ][ Nv ]
      = OccNeighPatEq0[ Dpth ][ Ns ][ Nt ][ Nv ]
    SubgroupOccNodeChildCnt[ LayerGroupIdx ][SubgroupIdx ][0][ Ns ][ Nt ][ Nv ]
      = OccNodeChildCnt[ Dpth ][ Ns ][ Nt ][ Nv ]
    SubgroupOccNodeChildCnt[ LayerGroupIdx ][ SubgroupIdx ][1][ NsP ][ NtP ][ NvP ]
      = OccNodeChildCnt[ Dpth-1 ][ NsP ][ NtP ][ NvP ]
}

  - Fine granularity slice attributes

    - General

Slice attributes shall be reconstructed attributes from a coded slice geometry in accordance with Clause 10.  E.6 specifies the reconstruction of an FGS attribute for the coded FGS geometry.  This annex specifies the difference of the fine granularity slice attribute decoder.

When fgs_layer_group_enabled equals to 1, the reconstructed attributes of each subgroup is stored in the arrays SubgroupNodeAttr (E.4.2.4).

    - Point coordinates

      - General

When fgs_layer_group_enabled is 1, attr_coord_conv_enabled is 0 and attr_inter_prediction_enabled is 0.  Attribute coding can use the FGS geometry's reconstructed STV point positions.

The expression AttrPos[ ptIdx ][ 𝑘 ] specifies the coordinates of each point for attribute coding.  AttrPos is equivalent to SubgroupNodePos, which is the FGS geometry in the slice’s coordinate system.

AttrPos[ptIdx][k] := SubgroupNodePos[ptIdx][k]

    - Attribute decoding using levels of detail

      - General

When attr_coding_type is either 1 or 2, slice attributes shall be reconstructed the attributes from a coded geometry in accordance with Subclause 10.6.  The different or additional processes for the fine granularity slice’s attribute decoder are specified in E.6.3.

The attribute decoding processes specified by Subclause 10.6 and E.6.3 are distance-based prediction schemes that use a hierarchical level-of-detail representation of the slice geometry.

Detail levels are defined by an iterative subsampling process (10.6.5).  The finest detail level comprises all points in the slice geometry or the FGS geometry.  With each iteration, a coarser detail level is generated from the previous coarsest detail level.

Every detail level comprises a list of points present in the detail level, and is associated with a list of refinement points.  A refinement point is a point that is present in a detail level and not present in any coarser detail level; the refinement points for detail level lvl, when combined with the coarser detail level lvl + 1, form detail level lvl.

For each refinement point, a set of neighbouring points is determined (10.6.6) using inter-detail-level, intra-detail-level and inter-frame and inter-layer-group searches.  The neighbouring points form a predictor set that is used to predict attribute/transform coefficient values.

The index variable lvl identifies a detail level. The index lvl starts from 0. When fgs_layer_group_enabled is equal to 1, the lvl value is initialized to LodMinLevel, which is set to log2 quantized node size of the leaf nodes of the FGS geometry.

      - State variables

Point predictors are specified in terms of the following state variables; the index ptIdx identifies a point by its index into AttrPos:

- The array PredPtIdx, identifies point predictors by their index in the canonical decoding order; PredPtIdx[ ptIdx ][ ni ] is the AttrPos index of the ni-th point in the predictor set for the identified point.  When fgs_layer_group_enabled equals to 1, PredPtIdx[ ptIdx ][ ni ] is the RefAttrPos index of the ni-th point in the predictor set for the identified point.

- The array PredPtRef of the flags to specify whether the point predictors are searched from the reference slice. When PredPtRef[ ptIdx ][ ni ] is equal to 1, the ni-th point in the predictor set is specified to be searched from the reference slice for the predictor identified by PredPtIdx[ ptIdx ][ ni ];  When PredPtRef[ ptIdx ][ ni ] is equal to 0, the ni-th point in the predictor set is specified to be searched from the current slice for the predictor identified by PredPtIdx[ ptIdx ][ ni ].  When fgs_layer_group_enabled equals to 1 and PredPtRef[ ptIdx ][ ni ] is equal to 1, the ni-th point in the predictor set is specified to be searched from the parent FGS for the predictor identified by PredPtIdx[ ptIdx ][ ni ];  When fgs_layer_group_enabled equals to 1 and PredPtRef[ ptIdx ][ ni ] is equal to 0, the ni-th point in the predictor set is specified to be searched from the current FGS for the predictor identified by PredPtIdx[ ptIdx ][ ni ].

When fgs_layer_group_enabled is equal to 1 and layer_group_id is greater than 0, the finest detail level of the parent FGS is specified in the array ParentLodPtIdx, identifying points in the finest detail level by their index in the canonical decoding order; ParentLodPtIdx [ 𝑖 ] is the RefAttrPos index of the 𝑖-th point in the finest detail level.

The variable LodMinLevel identifies a minimum level of detail levels generated from the FGS geometry.

      - Levels of detail

When fgs_layer_group_enabled is 1, general LoD generation process is specified by E.6.3.3.1.

        - General generation process

When fgs_layer_group_enabled is 1, the minimum level of LoD is specified in the variable LodMinLevel. The maximum level of the LoD is set to overlap with the minimum level of the parent FGS's LOD.

The finest detail level is identified by the detail level index LodMinLevel. LodMinLevel is set to log2 quantized node size of the leaf nodes of the FGS geometry.

Detail levels shall be iteratively subsampled (E.6.3.3.4), starting from the finest detail level, until either a single point remains or LodMaxLevel subsampled detail levels have been produced.  The variable Lvl identifies the detail level to be subsampled. When layer_group_id is 0, LodMaxLevel is set to log2 quantized node size of the root nodes of the FGS geometry. Otherwise, LodMaxLevel is set to plus 1 for log2 quantized node size of the root nodes of the FGS geometry.

LodMinLevel = occtreeMaxDepthMinus1 – (startDepth-1)
LodMaxLevel = LodMinLevel+num_layers_minus1[layer_group_id]+1 
if(layer_group_id>0)
  LodMaxLevel++

Lvl = LodMinLevel 
for (; Lvl < LodMaxLevel; Lvl++) {
  if (LodPtCnt[Lvl] == 1)
    break
  … /* subsample LodPtIdx[Lvl] */
}
LodCnt = Lvl + 1

The coarsest detail level is identified by the detail level index LodCnt − 1.  All points in the coarsest detail level shall be assigned to the coarsest level's refinement list (10.6.5.3).

        - The finest detail level

The AttrPos point indexes of the finest detail level shall have an initial one-to-one correspondence with the canonical decoding order of the FGS geometry.

subgroupPointCnt = SubgroupNodeCnt[layerGroupIdx][subgroupIdx] + SubgroupDirectNodePointCnt[layerGroupIdx][subgroupIdx]
for (ptIdx = 0; ptIdx < subgroupPointCnt; ptIdx++)
  LodPtIdx[LodMinLevel][ptIdx] = ptIdx
LodPtCnt[LodMinLevel] = subgroupPointCnt

The point indexes of the finest detail level shall be sorted by group in ascending order of their respective Morton-coded attribute coordinates.  The variable maxPtsPerSort identifies the max group size when sorting by group.

maxPtsPerSort = !attr_canonical_order_enabled && !max_points_per_sort_log2_plus1 ? LodPtCnt[LodMinLevel] : 1 << (max_points_per_sort_log2_plus1 - 1)

The sorted order shall be identical for the decoding of all attributes in an FGS with identical attribute coordinate arrays (AttrPos).

- Performing a stable sort for each attribute, or reusing the reordered points would satisfy the requirement for identical orders.

An example (inefficient) sorting process is:

for (benIdx = 0; benIdx < LodPtCnt[0]; benIdx += maxPtsPerSort) {
  endIdx = Min(benIdx + maxPtsPerSort, LodPtCnt[LodMinLevel])
  for (i = benIdx; i < endIdx; i++)
    for (j = i + 1; j < endIdx; j++) {
      iPtIdx = LodPtIdx[LodMinLevel][i]
      jPtIdx = LodPtIdx[LodMinLevel][j]
      iMorton = Morton(AttrPos[iPtIdx][0], AttrPos[iPtIdx][1], AttrPos[iPtIdx][2])
      jMorton = Morton(AttrPos[jPtIdx][0], AttrPos[jPtIdx][1], AttrPos[jPtIdx][2])
      if (iMorton > jMorton)
        Swap(LodPtIdx[LodMinLevel][i], LodPtIdx[LodMinLevel][j])
    }
}

        - The finest detail level of the parent FGS

The expression RefAttrPos [ptIdx][k] specifies the coordinates of each point for attribute coding in the parent FGS.  The RefAttrPos point indexes of the finest detail level shall have an initial one-to-one correspondence with the canonical decoding order of the parent FGS geometry.  The variable parentPointCnt, the size of the number of points in the parent FGS.

for (ptIdx = 0; ptIdx < parentPointCnt; ptIdx++)
  LodPtIdx[ptIdx] = ptIdx
parentLodPtCnt = parentPointCnt

The point indexes of the finest detail level shall be sorted by group in ascending order of their respective Morton-coded attribute coordinates.  The sorted order shall be identical for the decoding of all attributes in a single FGS with identical attribute coordinate arrays (RefAttrPos).

- Performing a stable sort for each attribute, or reusing the reordered points would satisfy the requirement for identical orders.

An example (inefficient) sorting process is:

for (benIdx = 0; benIdx < parentPointCnt; benIdx += maxPtsPerSort) {
  endIdx = Min(benIdx + maxPtsPerSort, parentPointCnt)
  for (i = benIdx; i < endIdx; i++)
    for (j = i + 1; j < endIdx; j++) {
      iPtIdx = ParentLodPtIdx[i]
      jPtIdx = ParentLodPtIdx[j]
      iMorton = Morton(RefAttrPos[iPtIdx][0], RefAttrPos[iPtIdx][1], 
        RefAttrPos[iPtIdx][2])
      jMorton = Morton(RefAttrPos[jPtIdx][0], RefAttrPos[jPtIdx][1], 
        RefAttrPos[jPtIdx][2])
      if (iMorton > jMorton)
        Swap(ParentLodPtIdx[i], ParentLodPtIdx[j])
    }
}

        - Generation of a single detail level

The coarser detail level Lvl + 1 shall be produced by subsampling the points of detail level Lvl.

When fgs_layer_group_enabled is equal to 1, block-based subsampling (10.6.5.8) shall proceed.

      - Predictor search

        - General process

The index variable lvl value is initialized to LodMinLevel.

maxLvl = LodCnt − (attr_coding_type == 2)
for (Lvl = LodMinLevel; Lvl < maxLvl; Lvl++)
  for (RfmtIdx = 0; RfmtIdx < LodRfmtPtCnt[Lvl]; RfmtIdx++) {
    … /* find predictors (10.6.6.2) of the current point */
  }

        - Minimum reference detail level for inter-level predictor searches

MinInterRefLvl = LodMinLevel+1
if (lod_scalability_enabled) {
  for (lvl = LodMinLevel+1; lvl < LodCnt − 1; lvl++) {
    if (LodRfmtPtCnt[lvl] < slice_num_points_minus1 − LodPtCnt[lvl])
      break
    MinInterRefLvl++
  }
}

        - Predictor search for a single refinement point

When fgs_layer_group_enabled is equal to 1 and layer_group_id is greater than or equal to 1, only an inter-layer-group search (E.6.3.4.4) shall be performed for the coarsest detail level.

        - Inter-layer-group predictor search

The inter-layer-group search shall be performed by finding the parent point which a Morton code matches.

candPtIdx = ParentLodPtIdx[RfmtIdx]
PredCnt[PtIdx] = 1
PredPtIdx[PtIdx][i] = candPtIdx
PredPtRef[PtIdx][i] = 1

      - Reconstruction of attribute values

        - General process

Each detail level shall be processed in turn, proceeding from the coarsest to the finest level, according to attr_coding_type (10.6.7.3, 10.6.7.4).  The variable Lvl is the index of the current detail level.

for (Lvl = LodCnt − 1; Lvl ≥ LodMinLevel; Lvl−−)
  … /* process a detail level */

        - Processing of a detail level (attr_coding_type = 2)

When Lvl is LodMinLevel, the reconstructed attributes values shall be divided by 256 with half-values rounded away from zero and clipped to the maximum attribute value:

if (Lvl == LodMinLevel) {
  subgroupPointCnt = SubgroupNodeCnt[layerGroupIdx][subgroupIdx] + 
    SubgroupDirectNodePointCnt[layerGroupIdx][subgroupIdx]
  for (ptIdx = 0; ptIdx < subgroupPointCnt; ptIdx++)
    for (c = 0; c < AttrDim; c++)
      PointAttr[ptIdx][c] = Clip3(0, AttrMaxVal, DivExp2Fz(PointAttr[ptIdx][c], 8))
}

      - Transform coefficient weights

        - Fine granularity slice case

When fgs_layer_group_enabled is equal to 1, coefficient weights are calculated accumulatively, proceeding from the finest to the coarsest detail level of an FGS.

The accumulated coefficient weight of each refinement point in a detail level shall be distributed to the points in its predictor set.  The distribution is proportional to the respective predictor weights:

for (lvl = LodMinLevel; lvl < LodCnt − 1; lvl++)
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++) {
    ptIdx = LodRfmtPtIdx[lvl][rfmtIdx]
    coeffW = CoeffWeight[ptIdx]
    for (ni = 0; ni < PredCnt[ptIdx]; ni++) {
      if (!PredPtRef[ptIdx][ni]){
        predW = PredWeight[ptIdx][ni]
        CoeffWeight[PredPtIdx[ptIdx][ni]] += DivExp2Up(coeffW × predW, 8)
      }
    }
  }

      - Transform coefficient weights

        - Quantization weights derivation

The index variable lvl value is initialized to LodMinLevel.

for (lvl = LodMinLevel; lvl < LodCnt − 1; lvl++)
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++) {
    ptIdx = LodRfmtPtIdx[lvl][rfmtIdx]
    coeffW = QuantWeight[ptIdx]
    for (ni = 0; ni < PredCnt[ptIdx]; ni++) {
      if (!PredPtRef[ptIdx][ni]){
        QuantWeight[PredPtIdx[ptIdx][ni]] +=
          DivExp2Inf(coeffW × quant_neigh_weight[ni], 8)
      }
    }
  }

When fgs_layer_group_enabled equals to 1 and subgroup_weight_adjustment_enabled equals to 1, quantization weights of a subgroup is adjusted to alleviate the missing information out of the layer-group boundary.  The array NumRefNodes is the number of nodes which refer a node as a neighbor.  The array NumRefNodes is initialized by setting all elements to 0

maxRefNodes = 0
numPointsInSubgroup = SubgroupNodeCnt[dadu_layer_group_id ][ dadu_subgroup_id ] + SubgroupDirectNodePointCnt[dadu_layer_group_id ][ dadu_subgroup_id ]
for (i = 0; i < numPointsInSubgroup; i++) {
  NumRefNodes[i] = 0
}
for (lvl = LodMinLevel; lvl < LodCnt − 1; lvl++)
  for (rfmtIdx = 0; rfmtIdx < LodRfmtPtCnt[lvl]; rfmtIdx++) {
    ptIdx = LodRfmtPtIdx[lvl][rfmtIdx] 
    for (ni = 0; ni < PredCnt[ptIdx]; ni++) {
      if (!PredPtRef[ptIdx][ni]) {
        NumRefNodes[PredPtIdx[ptIdx][ni]]++
          if (maxRefNodes < NumRefNodes[PredPtIdx[ptIdx][ni]])
            maxRefNodes = NumRefNodes[PredPtIdx[ptIdx][ni]]
      }
    }
  }
if(maxRefNodes > 0) {
  for(i = 0; i < numPointsInSubgroup; i++){
    QuantWeight[i] += (NumRefNodes[i]* subgroup_weight_adj_coeff_a)/maxRefNodes + subgroup_weight_adj_coeff_b
  }
}

    - Parent FGS generation for attributes

      - General

When fgs_layer_group_enabled is equal to 1 and layer_group_id is greater than 1, E.6.4.2 shall be applied.

After the point coordinates generation of the current FGS as specified by E.6.2, the parent FGS generation is proceeded.

The output of this process are the parent FGS attribute and the parent FGS geometry for attribute coding.  The expression RefPointAttr [ptIdx][c] specifies the attribute values of each point in the parent FGS.  The expression RefAttrPos [ptIdx][k] specifies the coordinates of each point for attribute coding in the parent FGS.

      - Parent FGS generation

The attribute values and attribute coordinates of the parent FGS are derived based on the bounding box of the coordinates of the current FGS.  The number of points in the parent FGS is specifies by the variable parentPointCnt.

parentPointCnt = 0
nodeSizeLog2OfParent = occtreeMaxDepthMinus1 - startDepth
parentNodeCnt := SubgroupNodeCnt[layerGroupIdx-1][prtSubgroupIdx] 
parentNodePos[i][k] := SubgroupNodePos[layerGroupIdx-1][prtSubgroupIdx][i][k]
parentNodeAttr[i][c] := SubgroupNodeAttr[layerGroupIdx-1][ prtSubgroupIdx][i][AttrIdx][c]

for (i = 0; i < parentNodeCnt; i++){
  parentNodePosShift = parentNodePos[i] << nodeSizeLog2OfParent
  if ((p[0] < pR[0]) && (p[1] < pR[1]) && (p[2] < pR[2]) && (p[0] >= pL[0]) && (p[1] >= pL[1]) && (p[2] >= pL[2])) {
    for (c = 0; c < AttrDim; c++)
      RefPointAttr [parentPointCnt][c] = parentNodeAttr[i][c]
    for (k = 0; k < 3; k++)
      RefAttrPos [parentPointCnt][k] = parentNodePos[i][k]
    parentPointCnt++
  }
}
  where
    p := parentNodePosShift
    pR := SubgroupBBoxMax[layerGroupIdx-1][prtSubgroupIdx]
    pL := SubgroupBBoxMin [layerGroupIdx-1][prtSubgroupIdx]

  - Parsing process

    - General

The parsing process of syntax elements is described in Clause 11.  This annex specifies additional processes for the fine granularity slice.

    - Data unit buffer

      - Initialization at the start of parsing a geometry data unit footer

The parsing of a geometry_data_unit_footer syntax structure shall commence at an offset from the end of the DU buffer.  The length of the GDU footer is specified by the expression DuFooterLen.  The expression DuIsGdu is equal to 1 when the DU is a GDU or DGDU.

GduFooterLen := 3 × (1 + occtree_point_cnt_list_present × occtreeMaxDepthMinus1)

DuFooterLen := DuIsGdu ? GduFooterLen : 0
DataUnitReadIdx = 8 × (DataUnitLength − DuFooterLen)

    - CABAC parsing processes

      - Contextual probability models

        - General

A CPM is a 16-bit unsigned integer value that models the probability of a zero bin.

The array Contexts, with elements Contexts[ ctxTbl ][ ctxIdx ], represents individual adaptive CPMs used by the CABAC parsing process.

When fgs_layer_group_enabled equals to 1, the array SubgroupContexts and SubgroupContextsForAttributes are used.

The array SubgroupContexts[ layerGroupIdx ][ subgroupIdx ] represents the array of Contexts of a subgroup indicated by the layer-group index layerGroupIdx and the subgroup index subgroupIdx.

The array SubgroupContextsForAttributes[ AttrIdx ][ layerGroupIdx ] [ subgroupIdx ] represents the array of Contexts of a subgroup attributes indicated by the attribute index AttrIdx, and the layer-group index layerGroupIdx and the subgroup index subgroupIdx.

        - Initialization

When fgs_layer_group_enabled equals to 1, initialization shall be performed by the parsing state restoration process (E.7.4).

    - Parsing state memorization and restoration

      - General

E.7.3 applies when fgs_layer_group_enabled is 1.

At certain moments, the entropy parsing state is recorded and later, used as the initial state for parsing other DUs or occupancy tree entropy streams.

The entropy parsing state shall comprise:

- for a GDU or a DGDU, the CABAC CPMs (E.7.3.1), the demi-CPMs for bitwise occupancy coding (9.2.10.6), the dictionary codec state for bytewise occupancy coding (9.2.9.4), the planar occupancy coding state (9.2.11.5.2) and the state of the variables PrevInterFrameRefIdx, PrevPhiResidSign, PrevPhiMul and PrevRadiusResidSign (9.3.3.1);

- for an ADU or DADU, the CABAC CPMs only (E.7.3.1).

The entropy parsing state shall be recorded and restored independently according to DU type (ADU and DADU versus GDU and DGDU) and for each different value of ADU AttrIdx.  For example, a coded point cloud sequence with num_attributes equal to 2 would require storage for three sets of entropy parsing state.

At the start of any GDU with slice_entropy_continuation equal to 0 or inter_entropy_continuation_enabled equal to 0, all previously recorded GDU, ADU, DGDU and DADU entropy parsing state shall be discarded.

      - Geometry data units

        - Memorization

The GDU or DGDU entropy parsing state shall be recorded at:

- the end of every dependent_geometry_data_unit syntax structure (E.3.1.3.4) when subgroup_context_reference_indication_enabled equals to 1 and layer_group_id is less than num_layer_groups_minus1.

When fgs_layer_group_enabled equals to 1, memorization shall record the elements and values of the GDU or DGDU entropy parsing state for restoration by the restoration process (E.7.4.2.2).  When subgroup_context_reference_indication_enabled equals to 1 and layer_group_id is less than num_layer_groups_minus1, the state shall be recorded to the subgroup array context SubgroupContexts[ layerGroupIdx ][ subgroupIdx ] of layer-group index layerGroupIdx equals to layer_group_id and subgroup index subgroupIdx equals to subgroup_id.

SubgroupContexts[ layer_group_id ][ subgroup_id ] = Contexts

- A decoder may release SubgroupContexts[ layerGroupIdx ][ subgroupIdx ] during decoding process of fine granularity slices.  For the context buffer release, SubgroupContextsCounter[ layerGroupIdx ][ subgroupIdx ] may defined as an integer counter of SubgroupContexts[ layerGroupIdx ][ subgroupIdx ].  SubgroupContextsCounter is initialized by the number of subsequent subgroups referencing to the context state of the current subgroup.  When GDU or DGDU entropy parsing state identified by the subgroup context reference identifiers ref_layer_group_id and ref_subgroup_id is restored, SubgroupContextsCounter[ layerGroupIdx ][ subgroupIdx ] is decreased by 1.  When the integer counter SubgroupContextsCounter[ layerGroupIdx ][ subgroupIdx ] is equal to 0, the corresponding context state SubgroupContexts[ layerGroupIdx ][ subgroupIdx ] may be released from the context buffer.

        - Restoration

The GDU entropy parsing state shall be restored at:

- the start of a geometry_data_unit syntax structure (7.3.3.1) when slice_entropy_continuation is 1; and

- the start of a geometry_data_unit syntax structure (7.3.3.1) when slice_entropy_continuation is 0, slice_inter_prediction is 1 and inter_entropy_continuation_enabled is 1; and

- the start of every occupancy_tree_level( dpth ) syntax structure (7.3.3.5) where dpth is greater than OcctreeEntropyStreamDepth.

The GDU or DGDU entropy parsing state shall be restored at:

- the start of a dependent_geometry_data_unit syntax structure (E.3.1.3.4).

When fgs_layer_group_enabled equals to 1, restoration shall restore the elements and values of the GDU entropy parsing state to those previously recorded by the memorization process (E.7.4.2.1).  At the start of a geometry_data_unit syntax structure, restoration shall exclude the planar occupancy coding state.

Restoration shall restore the elements and values of the DGDU entropy parsing state to those previously recorded by the memorization process (E.7.4.2.1) of the layer-group index ref_layer_group_id and subgroup index ref_subgroup_id.

      - Attribute data units

        - Memorization

The ADU entropy parsing state shall be recorded at :

- the end of every attribute_data_unit syntax structure (7.3.4.1).

When fgs_layer_group_enabled is 1, the DADU entropy parsing state shall be recorded at :

- the end of every dependent_attribute_data_unit syntax structure (E.3.1.4.2) when attr_subgroup_context_reference_indication_enabled equals to 1 and dadu_layer_group_id is less than num_layer_groups_minus1.

Memorization shall record the elements and values of the ADU entropy parsing state for restoration by the restoration process (11.6.3.2).  The state shall be recorded separately for each value of AttrIdx. When fgs_layer_group_enabled is 1, memorization shall record the elements and values of the DADU entropy parsing state for restoration by the restoration process (E.7.4.3.2).  The state shall be recorded separately for each value of AttrIdx, dadu_layer_group_id and dadu_subgroup_id.

When attr_subgroup_context_reference_indication_enabled equals to 1 and dadu_layer_group_id is less than num_layer_groups_minus1, the state shall be recorded to the subgroup array context for attribute SubgroupContextsForAttributes[ AttrIdx ][ layerGroupIdx ][ subgroupIdx ] of layer-group index layerGroupIdx equals to dadu_layer_group_id and subgroup index subgroupIdx equals to dadu_subgroup_id.

SubgroupContextsForAttributes[AttrIdx][ dadu_layer_group_id ][ dadu_subgroup_id ] = Contexts

        - Restoration

The ADU entropy parsing state shall be restored at :

- the start of each attribute_data_unit syntax structure (7.3.4.1) when either slice_entropy_continuation is 1 or slice_inter_entropy_continuation is 1.  The restoration shall be from the state recorded by the memorization process (11.6.3.1) with the same value of AttrIdx.

When fgs_layer_group_enabled is 1, the DADU entropy parsing state shall be restored at :

- the start of each dependent_attribute_data_unit syntax structure (E.3.1.4.2).  The restoration shall be from the state recorded by the memorization process (E.7.4.3.1) which is indicated by AttrIdx, ref_layer_group_id and ref_subgroup_id.

When fgs_layer_group_enabled equals to 1, restoration shall restore the elements and values of the ADU or DADU entropy parsing state to those previously recorded by the memorization process (E.7.4.3.1) of the layer-group index attr_ref_layer_group_id, subgroup index attr_ref_subgroup_id, and the same value of AttrIdx.

Contexts = SubgroupContextsForAttributes[AttrIdx][ attr_ref_layer_group_id ][attr_ref_ subgroup_id ]

  - Partial decoding

    - General

A decoder may decode and reconstruct part of fine granularity slices. A slice which is segmented by fine granularity slices supports partial decoding in terms of density (E.8.2) and/or spatial region (E.8.3).  When partial decoding is used, the unnecessary data units to produce the partial output shall be filtered before decoding occupancy tree or attribute coefficients.

    - Partial density decoding

      - General

A decoder shall generate a lower density slice point cloud.

The lower density FGS point cloud is specified in terms of the following variables:

- The variable SkippedLayerGroup, an application-specific number of skipped layer-groups for partial decoding in the direction of the density.  The value of SkippedLayerGroup shall be in the range 0 .. num_layer_groups_minus1.

- The variable MinNodeSizeLog2, a minimum occupancy tree node size that is specified by the SkippedLayerGroup.

- The arrays SubgroupNodePos[ layerGroupIdx ][ subgroupIdx ][ ptIdx ][ 𝑘 ], the subgroup output nodes of the layer-group index layerGroupIdx and the subgroup index subgroupIdx.

- The arrays SubgroupNodeCnt[ layerGroupIdx ][ subgroupIdx ], the number of nodes in the subgroup output nodes of the layer-group index layerGroupIdx and the subgroup index subgroupIdx

      - Selection of FGS

If SkippedLayerGroup is greater than 0, the layer-groups whose index is in the range 0 .. OutLayerGroup is selected to be decoded.  The maximum value of the layer-group index of partial decoding OutLayerGroup is specified by the total number of layer-groups minus SkippedLayerGroup.

OutLayerGroup := num_layer_groups_minus1 – SkippedLayerGroup

if (layer_group_id == 0)
  … /*decode GDU or ADU*/
else if (layer_group_id ≤ OutLayerGroup)
  … /*decode DGDU or DADU*/
else
  … /*skip DGDU or DADU*/

Consequently, the depth of the geometry occupancy tree of the partial decoding PartialDepth is inferred by the sum of number of layers in each layer-group whose index is in the range 0 .. OutLayerGroup.

PartialDepth = 0
for (i=0; I ≤ OutLayerGroup; i++)
  PartialDepth += num_layers_minus1[i] + 1

      - Geometry position compensation

The maximum depth of the geometry occupancy tree when decoding all layer-groups is inferred by the sum of number of layers in each layer-group whose index is in the range 0 .. num_layer_groups_minus1.

TotalDepth = 0
for (i=0; i< num_layer_groups_minus1; i++)
  TotalDepth += num_layers_minus1[i] + 1

The MinNodeSizeLog2 is inferred by the difference between occtreeMaxDepthMinus1 and PartialDepth.

MinNodeSizeLog2 = occtreeMaxDepthMinus1 + 1 - PartialDepth

If MinNodeSizeLog2 is greater than 1, points shall be centred within their corresponding block:

for (ptIdx = 0; ptIdx < SubgroupNodeCnt[ layerGroupIdx ][ subgroupIdx ]; ptIdx++)
  for (k = 0; k < 3; k++)
    SubgroupNode[ layerGroupIdx ][ subgroupIdx ][ ptIdx ][ 𝑘 ] |= (MinNodeSizeLog2 > 1) << (MinNodeSizeLog2 – 1)

    - Partial region decoding

      - General

A decoder shall generate the point cloud of a partial region of a slice.

The partial region FGS point cloud is specified in terms of the following variables:

- The arrays RoiBBoxMin and RoiBBoxMax, an application-specific array which specifies the region of interest as the minimum and the maximum position of the bounding box.

- The arrays SubgroupNodePos[ layerGroupIdx ][ subgroupIdx ], the subgroup output nodes of the layer-group index layerGroupIdx and the subgroup index subgroupIdx.

- The arrays SubgroupNodeCnt[ layerGroupIdx ][ subgroupIdx ], the number of nodes in the subgroup output nodes of the layer-group index layerGroupIdx and the subgroup index subgroupIdx

      - Selection of FGS

If RoiBBoxMin and RoiBBoxMax are present, the subgroups whose subgroup bounding box is overlapped with the bounding box of region of interest is selected to be decoded.

if (layerGroupIdx == 0)
  … /*decode GDU or ADU*/
else if ((RoiBBoxMin[0] < SubgroupBBoxMax[layerGroupIdx][subgroupIdx][0] && 
        RoiBBoxMin[1] < SubgroupBBoxMax[layerGroupIdx][subgroupIdx][1] &&
        RoiBBoxMin[2] < SubgroupBBoxMax[layerGroupIdx][subgroupIdx][2]) &&
        (RoiBBoxMax[0] > SubgroupBBoxMin[layerGroupIdx][subgroupIdx][0] &&
        RoiBBoxMax[1] > SubgroupBBoxMin[layerGroupIdx][subgroupIdx][1] &&
        RoiBBoxMax[2] > SubgroupBBoxMin[layerGroupIdx][subgroupIdx][2]))
  … /*decode DGDU or DADU*/
else
  … /*skip DGDU or DADU*/

Bibliography

- Society of Motion Picture and Television Engineers ST 2042-1 (2017), VC-2 Video Compression

Index of named expressions and variables

AeBits, 11.3.4

AeBitsReadIdx, 11.3.4

AeReadBin, 11.5.2

AngPosScaleK, 9.2.14.6

AngularEligible, 9.2.13.7.2

AngularOrigin, 7.4.3.2

AttrBitDepth, 7.4.2.1.3

AttrCoeff, 10.3.1.1

AttrDim, 7.4.2.1.3

AttrIdx, 7.4.4.2, 7.4.5

AttrLevelScale, 10.7.4

AttrMaxVal, 7.4.2.1.3

AttrPos, 10.2.1

AttrPosAng, 10.2.2

AttrQp, 10.7.3

AttrQpLayerOffset, 10.7.1

AttrQstep, 10.7.4

AttrRegionQpOffset, 10.7.2

AttrRegionQpOrigin, 10.7.1

AttrRegionQpSize, 10.7.1

AttrRegionSizeConstraint, 10.7.1

AxisCoded, 9.2.6.6

AzimuthAxis, 9.2.13.3

AzimuthAxisIsS/T, 9.2.13.3

BeamElev, 7.4.2.5.2

BeamIdxEst, 9.2.13.6

BeamIdxFromGrad, 9.2.13.6

BeamMinDeltaGrad, 9.2.13.7.2

BeamOffsetV, 7.4.2.5.2

BeamStepsPerRev, 7.4.2.5.2

BiasedNorm1, 10.6.6.5

BiasedNorm2, 10.6.6.12

BinIdxPfx, 11.4.3, 11.4.4

BinIdxSfx, 11.4.3, 11.4.4

BinIdxTu, 11.4.4, 11.4.5

BlkSizeLog2, 10.6.5.6, 10.6.5.8, 10.6.6.6

BpBits, 11.3.4

BpBitsReadIdx, 11.3.4

CanonicalLodSubsampling, 10.6.5.5

CathetusV, 9.2.13.7.2

ChildNodeSizeLog2, 9.2.6.4

ChunkBuf, 11.3.4

ChunkDuRem, 11.3.5

ChunkLen, 11.3.6

ChunkNextAeBit, 11.3.9

ChunkNextBpBit, 11.3.10

ChunkPadLen, 11.3.8, 11.3.11

ChunkPartBLen, 11.3.11

CloudAttrPos, 10.8.2

CodedAxisCnt, 9.2.6.6

CoeffIdx, 10.5.3.1

CoeffWeight, 10.6.4, 10.6.11.1

Contexts, 11.5.3.1

Ctx, 11.5.3.4

CtxIdx, 11.5.3.4

CtxIdxAngPhi, 9.2.13.9

CtxIdxAngTheta, 9.2.13.10

CtxIdxDictHg, 9.2.9.11

CtxIdxOccBit, 9.2.10.6.4

CtxIdxPlanePos, 9.2.11.7.1

CtxTbl, 11.5.3.4

CtxUpdateDelta, 11.5.3.3

DataUnitBytes, 11.2.2

DataUnitLength, 11.2.2

DataUnitReadIdx, 11.2.2

DictsHistogram, 9.2.9.4

DictsMostProb, 9.2.9.4

DictsMostProbAge, 9.2.9.4

DictsMostProbMaxAge, 9.2.9.4

DictsNextEvictIdx, 9.2.9.4

DictsRecent, 9.2.9.4

DirectMode1Eligible, 9.2.12.3.1

DirectMode2Eligible, 9.2.12.3.1

DirectMode3Eligible, 9.2.12.3.1

DirectModeEligible, 9.2.12.3.1

DirectNodePresent, 9.2.12.3.2

DnBeamIdx, 9.2.13.8.1

DnBeamIdxEst, 9.2.13.8.1

DnBitsAfterJointPrefix, 9.2.12.5.2

DnBitsAfterPlanar, 9.2.12.4.3

DnEligibleCnt, 9.2.12.3.3

DnJointCoded, 9.2.12.4.4

DnJointDiffBitInferred, 9.2.12.5.4

DnJointDiffBitPresent, 9.2.12.5.4

DnJointDiffBits, 9.2.12.5.3

DnJointPos, 9.2.12.4.4

DnJointPosDiffBit, 9.2.12.5.3

DnJointPosPrefix, 9.2.12.5.2

DnJointPrefixBits, 9.2.12.5.2

DnPartialPosAng, 9.2.13.8.2

DnPlanarPos, 9.2.12.4.3

DnPresenceMask, 9.2.12.3.2

DnPtPos, 9.2.12.4.1

DnPtPosAng, 9.2.13.8.2

DnPtPosAngS, 9.2.13.8.2

DnPtPosConstraint, 9.2.12.5.1

DnPtPosRem, 9.2.12.4.1

DnPtPosS/T/V, 9.2.12.4.1

DnQp, 9.2.14.5.2

DnRemAngBitsST, 9.2.13.8.3

DnRemAngBitsV, 9.2.13.8.3

DnRemBits, 9.2.12.4.5

DnRemPos, 9.2.12.4.5

Dpth, 9.2.6.1

DuFooterLen, 11.2.4

DuIsGdu, 11.2.4

DuNextBit, 11.2.5

ExpMovAvg, 9.2.11.5.3

FixedRahtFilters, 10.5.5.11

FrameCtr, 8.2.2

GduFooterLen, 11.2.4

IdxOfSubsampledPoint, 10.6.5.9

InLodPtCnt, 10.6.5.4

InLodPtIdx, 10.6.5.4

InterCompPredCoeff, 10.6.10.1

InterCompPredCoeffPrev, 10.6.10.1

IsSubsampledPoint, 10.6.5.7

IsReplaced, 10.5.5.4.3

IvlCarry, C.2

IvlCode, 11.5.4.2

IvlLow, 11.5.4.2, C.2

IvlRange, 11.5.4.2, C.2

LastCompPredCoeff, 10.6.10.1

LastCompPredCoeffPrev, 10.6.10.1

LcpScale, 10.5.5.12

Length, 7.2

LodCnt, 10.6.4

LodCoeffQp, 10.6.9.1

LodPtCnt, 10.6.4

LodPtIdx, 10.6.4

LodRfmtPtCnt, 10.6.4

LodRfmtPtIdx, 10.6.4

Lvl, 10.5.3.1, 10.6.5.1

MapPtIdx, 10.6.5.6

MaxPtsPerSort, 10.6.5.2

MapSub, 10.6.5.6

MaxSeqBboxDimLog2, A.4.1

MaxSliceDimLog2, A.4.1

MaxSlicePoints, A.4.1

MinAng, 10.2.2

MinCurAng, 10.2.2

MinInterRefLvl, 10.6.6.2

MinNodeSizeLog2, D.2.1

MinRefAng, 10.2.2

Morton, 5.10.7

NextAeStreamBit, 11.5.4.4

Nloc, 9.2.6.3

NlocC, 9.2.6.3

NlocG, 9.2.6.3

NlocP, 9.2.6.3

NodeBeamIdx, 9.2.13.7.4

NodeBeamValid, 9.2.13.7.4

NodeIdx, 9.2.6.1

NodeOccMap, 9.2.11.5.2

NodeSizeLog2, 9.2.6.4

Norm2, 10.6.6.13

NposAng, 9.2.13.2

NposAngMid, 9.2.13.7.1

NposAngMidS/T/V, 9.2.13.7.1

NposAngS/T/V, 9.2.13.2

Ns/Nt/Nv, 9.2.6.3

NsC/NtC/NvC, 9.2.6.3

NsG/NtG/NvG, 9.2.6.3

NsP/NtP/NvP, 9.2.6.3

NumRefNodes, E.6.3.7.1

ObufCtxIdxDelta, 12.4.1.1

ObufInitProba, 12.2.2

ObufInitProbaBounds, 12.2.2

OccAdj, 9.2.10.6.9.3

OccAdjCnt, 9.2.10.6.9.3

OccAvailWinLog2, 9.2.7.2

OccBitCodingOrder, 9.2.10.2

OccBitIdx, 9.2.10.2

OccBitIdxFromNpBit, 9.2.8

OccBitInferSet, 9.2.10.5

OccBitInferUnset, 9.2.10.4

OccBitLocC, 9.2.10.2

OccBitMap, 9.2.10.2

OccBitPresent, 9.2.10.3

OccChildCnt, 9.2.6.11

OccCtxSel, 9.2.10.6.2

OccCtxSelUpdate, 9.2.10.6.5

OccCtxSelVar, 9.2.10.6.4

OccDictHistogram, 9.2.9.6

OccDictMostProb, 9.2.9.6

OccDictMostProbAge, 9.2.9.6

OccDictMostProbMaxAge, 9.2.9.6

OccDictMostProbMaxAgeNext, 9.2.9.9

OccDictNextEvictIdx, 9.2.9.6

OccDictRecent, 9.2.9.6

OccFreeAxis, 9.2.6.6

OccFreeAxisCnt, 9.2.6.6

OccFromNpOcc, 9.2.8

OccIntraPred, 9.2.10.6.9.3

OccKnown, 9.2.10.5

OccKnownMask, 9.2.10.5

OccKnownZero, 9.2.10.5

OccLocC, 9.2.2.3

OccLvlNodeSizeLog2, 9.2.4

OccMapPresent, 9.2.6.9

OccMaxChildren, 9.2.6.7

OccMaybeSingleChild, 9.2.6.8

OccMinChildren, 9.2.6.7

OccNeigh, 9.2.7.3

OccNeighAvail, 9.2.7.2

OccNeighCnt, 9.2.10.6.9.3

OccNeighPat, 9.2.7.4

OccNeighPatEq0, 9.2.12.3.3

OccNeighPatR, 9.2.7.5, 9.2.10.6.6

OccNodeChildCnt, 9.2.12.3.3

OccNodeCnt, 9.2.5.1

OccNodeLoc, 9.2.5.1

OccNodePresent, 9.2.5.1

OccPlaneMask, 9.2.11.5.5

OccPosScaleK, 9.2.14.6

OccQp, 9.2.14.5.1

OccQpScaleLog2, 9.2.14.2.3

OccQpSubtreeDepth, 9.2.14.4

OccScalingVolSizeLog2, 9.2.14.2.2

OccSubtreeQp, 9.2.14.5.3

OccSubtreeQpLoc, 9.2.14.5.3

OccSubtreeQpMax, 9.2.14.5.4

OccSubtreeQpOffset, 9.2.14.3

OcctreeEntropyStreamDepth, 9.2.3

OccupancyDensity, 9.2.11.5.2

OccupancyMapP, 9.2.11.5.5

OccVolScale, 9.2.14.2.3

OutLodPtCnt, 10.6.5.4, 10.6.5.6

OutLodPtIdx, 10.6.5.4

OutRfmtPtIdx, 10.6.5.4

PartialPtCnt, D.2.1

PartialPtIdx, D.2.1

PartVal, 11.4.5

PartValue, 11.4.1

PlanarBeamIdx, 9.2.13.7.3

PlanarEligible, 9.2.11.5.1

PlanarEligibleByAng, 9.2.11.5.1

PlanarEligibleByRate, 9.2.11.5.1

PlanarFreeAxis, 9.2.11.4

PlanarMinChildren, 9.2.11.3

PlanarMinPlanes, 9.2.11.2

PlanarNodeAxisLoc, 9.2.11.6.2

PlanarNodeZone, 9.2.11.6.3

PlanarRate, 9.2.11.5.2

PlanarRateThreshold, 9.2.11.5.1

PlaneThresholdIdx, 9.2.11.5.1

PointAng, 8.3.2

PointAttr, 8.3.5

PointCnt, 8.3.2

PointPos, 8.3.3

PredBias, 7.4.2.6.3

PredCnt, 10.6.4

PredMode, 10.6.8.1

PredModeCoded, 10.6.8.3, 10.6.8.4

PredModeMax, 10.6.8.1

PredModePresent, 10.6.8.2

PredPointCloudBboxOrigin, 9.2.15.1

PredPointCloudBboxSize, 9.2.15.1

PredPtIdx, 10.6.4

PredPtRef, 10.6.4

PredWeight, 10.6.4

PrevOccPlanePos, 9.2.11.6.4

PrevOccSinglePlane, 9.2.11.6.4

PrevPlanarNodeZone, 9.2.11.6.4

PtnAng, 9.3.4.3

PtnAngStv, 9.3.4.5

PtnCnt, 9.3.3.1

PtnDepth, 9.3.3.1

PtnIdx, 9.3.3.1

PtnPhiMul, 9.3.2.2

PtnPos, 9.3.4.2

PtnPred, 9.3.4.6

PtnQp, 9.3.2.2

PtnQpInterval, 9.3.2.1

PtnQpOffset, 9.3.2.2

PtnRef, 9.3.4.6

PtnResidual, 9.3.2.2

PtnSecResidual, 9.3.2.2

PtnStack, 9.3.3.1

QuantizedChildNodeSizeLog2, 9.2.14.2.3

QuantizedNodeSizeLog2, 9.2.14.2.3

QuantWeight, 10.6.4, 10.6.13.1

RahtBlkCnt, 10.5.2.2

RahtBlkChildCnt, 10.5.5.2

RahtBlkLoc, 10.5.2.2

RahtBlkSumAttrRef, 10.5.2.6

RahtBlkWeight, 10.5.2.3

RahtBlkWeightRef, 10.5.2.6

RahtCoeff, 10.5.2.2

RahtCoeffLcp, 10.5.5.12

RahtCoeffOrder, 10.5.3.2

RahtCoeffQp, 10.5.4.3

RahtCoeffScaled, 10.5.4.2

RahtCoeffWeight, 10.5.2.5

RahtCoeffWeightM, 10.5.2.5

RahtCoeffWeightMRef, 10.5.2.6

RahtDcCoeff, 10.5.6.2

RahtPredDcCoeff, 10.5.7.2

RahtDcNorm, 10.5.5.4.1

RahtFwd, 10.5.5.7

RahtInterFilter, 10.5.5.11

RahtInterPred, 10.5.5.9

RahtInterPredBlk, 10.5.5.1

RahtInterPredEligible, 10.5.5.8

RahtInterPredW, 10.5.5.9

RahtInv, 10.5.6.5

RahtLvlCnt, 10.5.2.2

RahtNeighCnt, 10.5.5.3

RahtNeighCntMinAncestor, 10.5.5.3

RahtPred, 10.5.5.4.4

RahtPredBlk, 10.5.5.1

RahtPredEligible, 10.5.5.2

RahtPredExcluded, 10.5.5.4.2

RahtPredRecipW, 10.5.5.4.4

RahtPredRecipWC, 10.5.5.4.5

RahtPredW, 10.5.5.5

RahtPredWeight, 10.5.5.4.4

RahtPredWeightB, 10.5.5.4.5

RahtPredWeightC, 10.5.5.4.5

RahtRecon, 10.5.7

RahtRootLvl, 10.5.2.4

RahtTreeQpOffset, 10.5.4.4

RahtTreeQpOffsetM, 10.5.4.4

RawAttrValueBits, 10.3.1.3

RecCloudAttr, 6.5.6

RecCloudPointCnt, 6.5.6

RecCloudPos, 6.5.6

RefAttrPos, 10.8.1

RefCloudAttrPos, 10.8.3

RefCloudAttrPosFirst, 10.8.3.2

RefCloudAttrPosSecond, 10.8.3.2

RefCloudAttr, 10.8.3

RefCloudAttrBboxOrigin, 10.8.3

RefCloudAttrFirst, 10.8.3.2

RefCloudAttrSecond, 10.8.3.2

RefCloudAttrBboxSize, 10.8.3

RefLodPtIdx, 10.6.5.10

RefPointAttr, 10.8.1

ScaledHalfNodeSize, 9.2.13.7.1

ScaledHalfNodeSizeS/T/V, 9.2.13.7.1

ScaledNodeSize, 9.2.13.7.1

ScaledNodeSizeS/T/V, 9.2.13.7.1

SelAdj, 9.2.10.6.8

SeCtx, 12.5

SelNeigh, 9.2.10.6.6

SelPred, 9.2.10.6.9.1

SelPredEligible, 9.2.10.6.9.2

SelSib, 9.2.10.6.7

SeqCodedScale, 7.4.2.1.2

SeqOrigin, 7.4.2.1.2

SeqUnit, 7.4.2.1.2

SliceOrigin, 7.4.3.2

StvToXyz, 7.4.2.1.2

SubgroupBBoxMin, E.4.2.3

SubgroupContexts, E.7.3.1.1

SubgroupContextsCounter, E.7.4.2.1

SubgroupContextsForAttributes, E.7.3.1.1

SubgroupDirectNodePointCnt, E.4.2.3

SubgroupNodeAttr, E.4.2.4

SubgroupNodeCnt, E.4.2.3

SubgroupNodePos, E.4.2.3

SubgroupOccNeighPatEq0, E.4.2.3

SubgroupOccNodeChildCnt, E.4.2.3

SumN19, 10.5.5.3

SumN26, 9.2.10.6.9.3

TerminalNode, 9.2.6.5

TileInventoryOrigin, 7.4.2.4

ZeroRunLength, 10.3.1.1