# ISO/IEC JTC 1/SC 29/WG 3 N1520

ISO/IEC JTC 1/SC 29/WG 3

MPEG Systems 
Convenorship: KATS (Korea, Republic of)

Document type:	Output Document

Title:	EE Description for Avatar representation formats

Status:	Approved

Date of document:	2025-04-11

Source:	ISO/IEC JTC 1/SC 29/WG 3

Expected action:	ACT

Action due date:                 2025-04-11

No. of pages:	57 (with cover page)

Email of Convenor:	young.L@samsung.com

Committee URL:	https://isotc.iso.org/livelink/livelink/open/jtc1sc29wg3

INTERNATIONAL ORGANISATION FOR STANDARDISATION

ORGANISATION INTERNATIONALE DE NORMALISATION

ISO/IEC JTC 1/SC 29/WG 3

CODING OF MOVING PICTURES AND AUDIO

ISO/IEC JTC 1/SC 29/WG 3 N01520

Online– March 2025

| Source | WG03 (MPEG Systems) |
| --- | --- |
| Title | Exploration Experiments Description for MPEG-I Avatar Representation Formats |
| Editor | Anthony Trioux (Xidian Univ.) |
| MPEG number | 25056 |

1	EE1: Compression for Animation Streams and avatar components (ongoing)	2

2	EE2: Integrating on Geometry Data Components for Avatar Data (closed)	48

3	EE3: Other Animation Sample Formats for Animation Stream (ongoing)	63

4	EE4: Content Discovery and Partial Access (ongoing)	85

5	EE5: Animation Controllers (ongoing)	97

6	EE6: Seamless Asset Linking with Animation Capabilities (opened)	105

1	EE1: Compression For Animation Streams and Avatar Components (ongoing)

EE opened at MPEG #148. Please see WG03 N01520 for the latest description of this EE.

2	EE2: Integrating On Geometry Data Components For Avatar Data (closed)

EE opened at MPEG #148 and closed at MPEG#150. Please see WG03 N01520 for the latest description of this EE.

3	EE3: Other Animation Sample Formats For Animation Stream (ongoing)

EE opened at MPEG #148. Please see WG03 N01520 for the latest description of this EE.

4	EE4 ARF Manifest (ongoing):

EE opened at MPEG #149. Please see WG03 N01520 for the latest description of this EE.

5	EE5 Animation Controllers (ongoing):

EE opened at MPEG #149. Please see WG03 N01520 for the latest description of this EE.

6	EE6 Assets Linking (opened):

EE opened at MPEG #150. Please see WG03 N01520 for the latest description of this EE.

EE1: Compression for Animation Streams and avatar components (ongoing)

## NOTE: It was advised by AG02 to move compression related aspects to WG07.  WG03 agreed that they will address the potential transition of compression related aspects in EE1 to WG07 as part of the AHG period. For this, WG03 added a mandate to the AHG to address this issue and scheduled a dedicated call (scheduled for May 26) to discuss this matter. WG03 expect to finalize this transition during MPEG#151 based on the proposal shown in the WG03 plenary, but want to simplify processes as much as possible and also minimize any risk of delay of the project.

## 1.1 Introduction

This Exploration Experiment (EE) aims to evaluate and develop methods for compressing animation streams for avatars, focusing on both facial and body animations. The experiment will explore compression for various animation data types, including blendshapes, facial landmarks, animation controllers, and joint transforms, with the goal of establishing a set of standardized approaches that enable efficient and flexible encoding and decoding of avatar animation data across platforms. In addition, the EE will evaluate and develop methods for compressing avatar components from the ARF container, one example is the compression of blendshapes faces.

## 1.2 Problem statement

The following statement on Compression for Animation streams are identified:

- Compression of Facial Animation Streams: Develop and test methods for compressing diverse facial animation data types, including blendshapes, landmarks, and animation controllers, focusing on preserving expression fidelity and temporal consistency. The exploration should address various compression levels (both lossless and lossy) to accommodate different bandwidth and latency requirements for real-time applications.

- Compression of Body Animation Streams: Address compression techniques for body animation streams, with a particular emphasis on joint transforms used for skeletal motion. The goal is to preserve the accuracy and natural flow of body movements while reducing the amount of data. This includes evaluating compression mechanisms for different joint hierarchies and motion patterns and examining trade-offs between compression efficiency (both lossless and lossy)  and articulation fidelity.

- Standardization of Avatar Animation Data Formats: Propose a standardized format for compressed avatar animation data that is interoperable across platforms and supports a wide range of animation components. The format should be extensible to accommodate new animation parameters as they emerge, and must support compatibility with existing frameworks, such as Mediapipe, ARKit, OpenXR, etc.

- Evaluation of Compression Techniques on Realism and Responsiveness: Establish metrics and testing methodologies to assess the impact of compression on avatar realism and responsiveness, with an emphasis on user perception. The evaluation process will examine how different compression levels affect the perceived quality of facial expressions and body movement, including potential delays, artifacts, or loss of motion fidelity. Objective and subjective testing methods will be developed to validate the compression approaches.

The EE will focus on developing compression tools for avatar animation streams including but not limited to : blendshapes, joints, controllers, landmarks, etc. and support their integration into the new part related to Avatar representation format (Part 39).

## 1.3 Use cases relevant for the EE

The work of the EE is based on the requirements defined in N00359 and use cases identified in m64008 .

The requirements defined in N00359 are currently organized into five different categories including: Avatar Representation, Coding format, Transport/Synchronization/Carriage, Storage and Privacy and Security requirements.

The requirements related to the “Avatar Representation” are summarized as follows:

- Interoperability: allow converting to and from other avatar representation.

- 3D Scenes: to allow integration into scene description solution.

- Attributes: requirements applicable to the features supported by the avatar, such as, CGI features as textures, geometries, skeletons, accessories, semantic definitions.

The requirements related to the “Coding Format” are summarized as follows:

- General: requirements applicable to the whole format and its use

- Geometry: requirements applicable to the geometry of the base avatar

- Interaction: requirements that relate to the interaction between Avatars and between Avatars and other objects in the scene

- Animation and control: requirements on the facial and body animation

Transport/Synchronization/Carriage: requirements related to the transport of the base avatar model and the associated animation streams

- Storage: requirements that pertain to the storage of the base avatar model

- Security and privacy: requirements on the protection of the avatar in different scenarios

The requirements and use cases explicit mention the interoperability concept to allow converting to and from other representation and the representation of all components of an avatar including but not exclusive to geometries, textures, skeletons, accessories and semantics.

## 1.4 Related (WG2) and Extracted (new) Requirements

## 1.5 Relation to other activities (EE, requirements, etc…)

## 1.6 Mandates

## 1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | L |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | P |
| Interdigital | Joao Regateiro | joao.regateiro@interdigital.com | P |

(P = proponent, L = leader)

## 1.8 Information about proposed technologies

The following contributions on Compression for Animation Stream have been submitted:

Meeting #148

m70195 [SD] Avatar facial animation stream format and coding scheme

Meeting #149

m71290 [SD][Avatar][ARF] Animation Bitstream and Compression with Landmarks

This proposal has been accepted for inclusion in the EE1 and is documented below.

- Introduction

In the MPEG#148 meeting, a new part (Part 39) of MPEG System workgroup (WG03) has been initiated with an initial output document WG03N01398 [1] and, consequently, three Exploratory Experiments [2] have also been initiated to continue the effort on delivering a new avatar format.

The content of this contribution addresses EE1 on “Compression for Animation Streams” and EE3 on “Other animation sample formats for animation stream” by providing an animation bitstream as the basis support for compression algorithms to be investigated and further proposed.

The initial proposal to address the issue of animation data streaming is present as an alternative in the current specification working draft [1]. While the proposed solution addresses the requirements of the storage and transport of animation streams, it is container-specific (i.e., it is mainly based on ISOBMFF and ZIP containers) and may not be applicable to delay-sensitive use cases.

This contribution presents a generic bitstream format for avatar animation streams that is compatible with the Avatar Representation Format (ARF) and can be encapsulated using different media containers or packetization mechanisms. This is based on our previous contribution m70326 [3] that was presented in the last meeting and included in the EE. It should also be noted that the proposed bitstream is also compatible with the ISOBMFF-based carriage approach with few modifications.

- Avatar Animation Bitstream

An avatar animation bitstream is composed of a sequence of avatar animation units (AAUs), which may also be referred to as NAL units.

The general syntax structure for an AAU is shown in Table 1, where the data types used for the definition of different fields in the syntax structures are as follows.

- uimsbf: Unsigned integer with most significant bit first.

- vlc8: Variable length character string. Contains string data stored as a character array encoded in UTF-8.

- boolean: A single bit that represents a Boolean value.

- float32: A 32-bit floating point value represented according to the IEEE 754 specification.

Each avatar animation unit (AAU) contains a header and a payload. An AAU header contains at least a field that indicates the unit type and a field that indicates the AAU payload. The contents of the payload depend on the type of the AAU, where ByteAlignment is a padding with up to seven bits set to 0 for the AAU payload to be byte-aligned.

Figure 1 - Illustration of binary structure using keyframe and non-keyframe AAUs.

A keyframe AAU represents the main structure of the bitstream format including detailed information about the frame being transmitted e.g., quantization of the data of unit_type different from configuration type (AAU_CONFIG), and other properties of the payload defined in this document. A non-keyframe contains the same properties of the payload as for the keyframe with the exception that the data being transmitted is reconstructed relative to the previous keyframe AAU. To this end, the data of a non-keyframe is different between the current frame and the last keyframe, using the aau_quantization as a precision parameter to reconstruct the current frame.

Table 1 – Syntax of avatar_animation_unit()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| avatar_animation_unit() |  |  |
| { |  |  |
| aau_header(); |  |  |
| if (aau_keyframe) |  |  |
| aau_quantization | 16 | uimsbf |
| aau_payload(); |  |  |
| ByteAlignement | 0-7 | uimsbf |
| } |  |  |

- aau_quantization: Indicates the type of the AAU signaled by aau_unit_type is not a keyframe therefore it carries a quantization scale for the carried data.

The syntax structure of the AAU header is as shown in Table 2.

Table 2 – Syntax of aau_header().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_header() |  |  |
| { |  |  |
| aau_unit_type; | 7 | uimsbf |
| aau_unit_length; | 32 | uimsbf |
| aau_keyframe; | 1 | boolean |
| if (aau_keyframe) |  |  |
| aau_quantization | 16 | uimsbf |
| reserved | 1 | uimsbf |
| } |  |  |

The aau_header() syntax construct contains the following syntax elements:

- aau_unit_type: indicates the type of the AAU. The possible values are described in Table 3.

- aau_unit_length: indicates the size of the AAU payload in bytes.

- aau_keyframe: indicates whether the AAU payload is a keyframe.

- aau_quantization: Indicates the type of the AAU signaled by aau_unit_type is not a keyframe therefore it carries a quantization scale for the carried data. The quantization processing model is described in Section 2.5.

Table 3 – Avatar Animation Unit type codes and corresponding payloads.

| aau_unit_type | Name of AAU type | Content of AAU payload |
| --- | --- | --- |
| 0 | AAU_CONFIG | aau_config_unit() |
| 1 | AAU_ANIMATION | aau_animation_unit() |
| 2 | AAU_JOINT | aau_joint_unit() |
| 3 | AAU_LANDMARK | aau_landmark_unit() |
| 4..10 | AAU_RSV_4 AAU_RSV_10 | Reserved AAU types. |
| 11..127 | AAU_UNSPEC_11 AAU_UNSPEC_127 | Unspecified AAU types. |

The aau_payload() is defined as shown in Table 4.

Table 4 – Syntax of aau_payload().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_payload() |  |  |
| { |  |  |
| aau_timestamp; | 32 | uimsbf |
| if (aau_unit_type == AAU_CONFIG) |  |  |
| aau_config_unit() |  |  |
| else if (aau_unit_type == AAU_ANIMATION) |  |  |
| aau_animation_unit() |  |  |
| else if (aau_unit_type == AAU_JOINT) |  |  |
| aau_joint_unit() |  |  |
| else if (aau_unit_type == AAU_LANDMARK) |  |  |
| aau_landmark_unit() |  |  |
| } |  |  |

The aau_payload() syntax construct contains the following syntax elements:

- aau_timestamp: is the timestamp of the AAU in ticks. The timestamp in seconds can be calculated as timestamp/timescale, where timescale is signalled in the configuration AAU.

- ByteAlignment: is a padding with up to seven bits set to 0 for the AAU payload to be byte-aligned.

  - Configuration avatar animation unit

A configuration AAU is an AAU with aau_unit_type set to AAU_CONFIG. The payload of such AAU is defined as shown in Table 5.

Table 5 – Syntax of aau_config_unit().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_config_unit() |  |  |
| { |  |  |
| acu_profile_length; | 8 | uimsbf |
| acu_animation_profile; | acu_profile_length * 8 | vlc8 |
| acu_timescale; | 32 | uimsbf |
| acu_target_avatar_info_flag; | 1 | boolean |
| if (acu_target_avatar_info_flag) { |  |  |
| acu_avatar_id; | 3 | uimsbf |
| acu_lod_id; | 3 | uimsbf |
| } |  |  |
| acu_animation_correspondence_flag; | 1 | boolean |
| if (acu_animation_correspondence_flag) { |  |  |
| acu_animation_set_id; | 3 | uimsbf |
| acu_animation_count; | 10 | uimsbf |
| for (i=0; i<acu_animation_count; i++) { |  |  |
| acu_target_animation_index[i]; | 10 | uimsbf |
| } |  |  |
| } |  |  |
| acu_joint_correspondence_flag; | 1 | boolean |
| if (acu_joint_correspondence_flag) { |  |  |
| acu_skeleton_id; | 3 | uimsbf |
| acu_joint_count; | 10 | uimsbf |
| for (i=0; i<acu_joint_count; i++) { |  |  |
| acu_target_joint_index[i]; | 10 | uimsbf |
| } |  |  |
| } |  |  |
| acu_landmark_correspondence_flag; | 1 | boolean |
| if (acu_landmark_correspondence_flag) { |  |  |
| acu_landmarkset_id; | 3 | uimsbf |
| acu_landmark_count; | 10 | uimsbf |
| for (i=0; i<acu_landmark_count; i++) { |  |  |
| acu_target_landmark_index[i]; | 10 | uimsbf |
| } |  |  |
| } |  |  |
| acu_reserved_correspondence_flags; | 6 |  |
| acu_reserved_unspecified_flags; | 22 |  |
| } |  |  |

The aau_config_unit() syntax construct contains the following syntax elements:

- acu_profile_length: is the number of characters in the profile string signalled by acu_animation_profile.

- acu_animation_profile: is a character string with the name of the profile that generated stream conforms to.

- acu_timescale: is the number of ticks per second.

- acu_target_avatar_info_flag: is a flag indicating whether target avatar information is signalled in this configuration AAU. Value 0 indicates that no target avatar information is present. Value 1 indicates that target avatar information is present in the configuration AAU.

- acu_avatar_id: is an integer identifying the avatar to animate.

- acu_lod_id: is an integer identifying the level-of-detail (LoD) of the avatar to animate.

- acu_animation_correspondence_flag: is a flag indicating whether animation correspondence information is signalled in this configuration AAU. Value 0 indicates that no correspondence information is present. Value 1 indicates that animation correspondence information is present in the configuration AAU.

- acu_animation_set_id: is an integer identifying the animation set of the avatar to animate.

- acu_target_animation_index[i]: is the index of the target animation corresponding to the source avatar’s i-th animation.

- acu_joint_correspondence_flag: is a flag indicating whether joint correspondence information is signalled in this configuration AAU. Value 0 indicates that no correspondence information is present. Value 1 indicates that joint correspondence information is present in the configuration AAU.

- acu_skeleton_id: is an integer identifying the skeleton of the avatar to animate.

- acu_target_joint_index[i]: is the index of the target avatar skeleton joint corresponding to the source avatar’s i-th joint.

- acu_landmarks_correspondence_flag: is a flag indicating whether landmark correspondence information is signalled in this configuration AAU. Value 0 indicates that no correspondence information is present. Value 1 indicates that landmarks correspondence information is present in the configuration AAU.

- acu_landmarkset_id: is an integer identifying which collection of landmarks is used from the avatar.

- acu_target_landmark_index[i]: is the index of the target avatar skeleton joint corresponding to the source avatar’s i-th joint.

- acu_reserved_correspondence_flags: are reserved flags for the correspondences of future avatar components.

- acu_reserved_unspecified_flags: are flags for unspecified features.

  - Animation target avatar animation unit

An animation target AAU is an AAU whose aau_unit_type field is set to AAU_ANIMATION. The payload of such AAU is defined as shown in Table 6.

Table 6 – Syntax of aau_animation_unit().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_animation_unit() |  |  |
| { |  |  |
| amu_animation_target_count_minus1; | 10 | uimsbf |
| for (i=1; i<amu_animation_target_count_minus1 + 1; i++) { |  |  |
| amu_animation_source_index[i]; | 10 | uimsbf |
| amu_animation_target_index[i]; | 10 | uimsbf |
| amu_animation_target_weight[i]; | 32 | float32 |
| } |  |  |
| } |  |  |

The aau_animation _unit() syntax construct contains the following syntax elements:

- amu_animation_target_count_minus1: plus 1 indicates the number of animation targets whose weights are signalled in this animation target AAU.

- amu_animation_source_index[i]: is the index of the i-th animation source whose weight is signalled in the animation AAU.

- amu_animation_target_index[i]: is the index of the i-th animation target whose weight is signalled in the animation AAU.

- amu_animation_target_weight[i]: is the weight of the i-th animation target whose index is signalled by the field amu_animation_target_index[i].

  - Joint avatar animation unit

A joint AAU is an AAU whose aau_unit_type field is set to AAU_JOINT. The payload of such AAU is defined as shown in Table 7.

Table 7 – Syntax of aau_joint_unit().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_joint_unit() |  |  |
| { |  |  |
| aju_joint_count_minus1; | 10 | uimsbf |
| for (i=1; i<aju_joint_count_minus1 + 1; i++) { |  |  |
| aju_source_joint_index[i]; | 10 | uimsbf |
| aju_target_joint_index[i]; | 10 | uimsbf |
| aju_joint_transform[i]; | 16 * 32 | float32 |
| } |  |  |
| } |  |  |

The aau_joint_unit() syntax construct contains the following syntax elements:

- aju_joint_count_minus1: plus 1 indicate the number of joint transformations signalled in the joint transform AAU.

- aju_source_joint_index[i]: indicates the source joint index for the i-th joint signalled in the joint transform AAU.

- aju_target_joint_index[i]: indicates the target joint index for the i-th joint signalled in the joint transform AAU.

- aju_joint_transform[i]: is the transformation matrix for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

Alternatively, the aju_joint_transform[i] field may be replaced by an encoding of optional translation, rotation, scale, velocity, inertia and uncertainty as presented in Table 8.

Table 8 – Syntax of aau_joint_unit() with translation, rotation, and scale.

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_joint_unit() |  |  |
| { |  |  |
| aju_joint_count_minus1; | 10 | uimsbf |
| for (i=1; i<aju_joint_count_minus1 + 1; i++) { |  |  |
| aju_source_joint_index[i]; | 10 | uimsbf |
| aju_target_joint_index[i]; | 10 | uimsbf |
| aju_joint_translation_flag[i]; | 1 | boolean |
| if (aju_joint_translation_flag[i]) { |  |  |
| aju_joint_translation[i]; | 3 * 32 | float32 |
| } |  |  |
| aju_joint_rotation_flag[i]; | 1 | boolean |
| if (aju_joint_rotation_flag[i]) { |  |  |
| aju_joint_quaternion_flag[i]; | 1 | boolean |
| if (aju_joint_quaternion_flag[i]) { |  |  |
| aju_joint_quaternion[i]; | 4 * 32 | float32 |
| }else { | 4 * 32 | float32 |
| aju_joint_rotation[i]; | 4 * 32 | float32 |
| } |  |  |
| aju_joint_quaternion_flag[i]; | 1 | boolean |
| if (aju_joint_quaternion_flag[i]) { |  |  |
| aju_joint_quternion[i]; | 4 * 32 | float32 |
| } |  |  |
| aju_joint_scale_flag[i]; | 1 | boolean |
| if (aju_joint_scale_flag[i]) { |  |  |
| aju_joint_scale[i]; | 3 * 32 | float32 |
| } |  |  |
| aju_joint_velocity_flag[i]; | 1 | boolean |
| if (aju_joint_velocity_flag[i]) { |  |  |
| aju_joint_velocity[i]; | 16 * 32 | float32 |
| } |  |  |
| aju_joint_inertia_flag[i]; | 1 | boolean |
| if (aju_joint_inertia_flag[i]) { |  |  |
| aju_joint_inertia[i]; | 6 * 32 | float32 |
| } |  |  |
| aju_joint_uncertainty_flag[i]; | 1 | boolean |
| if (aju_joint_uncertainty_flag[i]) { |  |  |
| aju_joint_uncertainty[i]; | 32 | float32 |
| } |  |  |
| } |  |  |
| } |  |  |

- aju_joint_translation_flag[i]: is a flag indicating whether a translation is signalled for the i-th joint.

- aju_joint_translation[i]: is the translation for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_joint_rotation_flag[i]: is a flag indicating whether a rotation is signalled for the i-th joint.

- aju_joint_ rotation [i]: is the rotation for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_joint_quaternion_flag[i]: is a flag indicating whether a quaternion type of rotation is signalled for the i-th joint.

- aju_joint_quaternion [i]: is the quaternion type of rotation for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_joint_scale_flag[i]: is a flag indicating whether a scale is signalled for the i-th joint.

- aju_joint_scale[i]: is the scale for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_velocity_flag[i]: is a flag indicating whether a velocity is signalled for the i-th joint.

- aju_joint_velocity[i]: is the velocity for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_inertia_flag[i]: is a flag indicating whether an inertia is signalled for the i-th joint.

- aju_joint_inertia[i]: is the inertia for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

- aju_uncertainty_flag[i]: is a flag indicating whether a uncertainty is signalled for the i-th joint.

- aju_joint_uncertainty [i]: is the uncertainty of the prediction for the animation parameter for the target joint corresponding to the source joint whose index is signalled by the field aju_source_joint_index[i].

  - Landmark avatar animation unit

A landmark AAU is an AAU whose aau_unit_type field is set to AAU_LANDMARK. The payload of such AAU is defined as shown in Table 9.

Table 9 – Syntax of aau_landmark_unit().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_landmark_unit() |  |  |
| { |  |  |
| alu_landmark_count_minus1; | 10 | uimsbf |
| for (i=1; i<alu_landmark_count_minus1 + 1; i++) { |  |  |
| alu_source_landmark_index[i]; | 10 | uimsbf |
| alu_target_landmark_index[i]; | 10 | uimsbf |
| alu_landmark_transform[i]; | 16 * 32 | float32 |
| } |  |  |
| } |  |  |

The aau_landmark_unit() syntax construct contains the following syntax elements:

- alu_landmark_count_minus1: plus 1 indicate the number of landmark transformations signalled in the landmark animation AAU.

- alu_source_landmark_index[i]: indicates the source landmark index for the i-th landmark signalled in the landmark animation AAU.

- alu_target_landmark_index[i]: indicates the target landmark index for the i-th landmark signalled in the landmark animation AAU.

- alu_landmark_transform[i]: is the transformation matrix for the target landmark corresponding to the source landmark whose index is signalled by the field aju_source_ landmark_index[i].

Alternatively, the alu_landmark_transform[i] field may be replaced by an encoding of optional translation, rotation, scale, velocity, inertia and uncertainty as presented in Table 10.

Table 10 – Syntax of aau_landmark_unit() with translation, rotation, and scale.

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_landmark_unit() |  |  |
| { |  |  |
| alu_landmark_count_minus1; | 10 | uimsbf |
| for (i=1; i<alu_landmark_count_minus1 + 1; i++) { |  |  |
| alu_source_landmark_index[i]; | 10 | uimsbf |
| alu_target_landmark_index[i]; | 10 | uimsbf |
| alu_landmark_translation_flag[i]; | 1 | boolean |
| if (alu_landmark_translation_flag[i]) { |  |  |
| alu_landmark_translation[i]; | 3 * 32 | float32 |
| } |  |  |
| alu_landmark_position_flag[i]; | 1 | boolean |
| if (alu_landmark_position_flag[i]) { |  |  |
| alu_landmark_position[i]; | 3 * 32 | float32 |
| } |  |  |
| alu_landmark_uncertainty_flag[i]; | 1 | boolean |
| if (alu_landmark_uncertainty_flag[i]) { |  |  |
| alu_landmark_uncertainty[i]; | 32 | float32 |
| } |  |  |
| } |  |  |
| } |  |  |

- alu_landmark_translation_flag[i]: is a flag indicating whether a translation is signalled for the i-th landmark.

- alu_landmark_translation[i]: is the translation for the target landmark corresponding to the source landmark whose index is signalled by the field alu_source_landmark_index[i].

- alu_landmark_position_flag[i]: is a flag indicating whether a position is signalled for the i-th landmark.

- alu_landmark_position [i]: is the absolute position for the target landmark corresponding to the absolute position of the source landmark whose index is signalled by the field alu_source_landmark_index[i].

- alu_uncertainty_flag[i]: is a flag indicating whether an uncertainty is signalled for the i-th landmark.

- alu_landmark_uncertainty [i]: is the uncertainty of the prediction for the animation parameter for the target landmark corresponding to the source landmark whose index is signalled by the field alu_source_landmark_index[i].

  - Quantization processing model

This section describes the process of using quantization to code and compress the bitstream format. The quantization flag in one embodiment can be fixed and normative with a defined default value for all the data parameters transmitted in the payload.

On a different embodiment the quantization flag is transmitted and unique to all data parameters in the payload, which means all data in the payload is quantified to an integer value to the same number of bits e.g., quantization of 10 bits.

On another embodiment the quantization flag is transmitted per keyframe and different for every data parameter transmitted in the payload, making the bitstream size variable depending on the type of data being transmitted in the payload.

Figure 2 - Processing model for the avatar animation unit using quantization and compression.

Figure 2 initiates with a complete AAU containing an aau_header and an aau_payload function, if the headed contains an aau_keyframe the process applies the following quantization:

dataq=data(1≪QS)

Where “data” can be represented by any data floating value present in the “aau_payload” with a “aau_type” equals to “AAU_ANIMATION”, “AAU_JOINT” or “AAU_LANDMARK” e.g., “amu_animation_target_weights[i]” that represent 32 bits of type float32, but not limited to. The QS can be defined by any variable integer value e.g., QS = 5, or QS = 10 or any positive number.

The resulted dataq in one embodiment is used to define the number of bits for the data that needs to be compressed and transmitted, and so the bitstream is either positive or positive the size of dataq.

The residual computation is performed as: if the “aau_header” contains “aau_keyframe=1”, then the final result is as follows:

datares=round(dataq)

This permits to transform the data to integers type. : if the “aau_header” contains “aau_keyframe=0”, then the final result is as follows:

data'=dataprev-data

The data of the previous frame is subtracted from the current fame, which results in applying quantization to the difference between two frames, being the previous frame a keyframe. The

Finally with the data’ is passed through the previous quantization function to achieve a positive or negative integer value.

Finally, the last component is to use a compression mechanism of integers, this can typically be entropy encoding, but not limited too.

# Proposal

It is proposed to adopt the proposed bitstream format in section 2 of this contribution as an MPEG avatar animation bitstream into the working draft (WD) of ISO/IEC 23090-39.

# References

[1] WG03N01398, “Avatar Representation Format”, MPEG#147, Kemer, Turkey, November 2024.

[2] WG03N01379, “EE Description of Avatar representation formats”, MPEG#147, Kemer, Turkey, November 2024.

[3] m70326, “[SD][Avatar] MPEG Avatar Animation Stream”, MPEG#147, Kemer, Turkey, November 2024.

m71323 [ARF][EE1] Assessing Compressed Blendshape-Based Avatar Facial Animation: Objective Metrics vs. Subjective Scores

This proposal has been accepted for inclusion in the EE1 and is documented below.

2	Introduction

The MPEG 148 meeting discussed the relevant proposals for the avatar WD (working draft) stage, focusing on two aspects: 1) avatar representation format design and 2) avatar animation stream format. This meeting established three exploratory experiments (EEs) on digital humans, among which EE1[1] was about "Compression for Animation Streams". m70195[2] proposed a compression coding scheme for digital human animation parameters and the corresponding bitstream structure. The compression method used inter-frame prediction (residual calculation of adjacent frames), followed by quantization and entropy coding. Experimental results showed that the compression scheme achieved valuable bitrate reduction (around 10 times reduction) while ensuring good visual quality. However, only subjective scores as well as bitrate computation were provided. Objective scores are essential as they provide a fast, consistent, and scalable alternative to subjective evaluations, which are often time-consuming and resource-intensive.

3	Experimental setup

In what follows, we evaluate the ability of widely used objective metrics to predict the observed MOS scores. For this evaluation, we consider popular metrics in the pixel domain, including Peak Signal-to-Noise Ratio (PSNR) [3], PSNR with consideration for the Human Visual System (PSNR-HVS) [4], Structural Similarity Index Measure (SSIM) [5], Multi-Scale Structural Similarity Index Measure (MS-SSIM) [6], Visual Information Fidelity (VIF) [7], Detail Loss Measure (DLM) [8], and Video Multi-method Assessment Fusion (VMAF) [9]. In addition, we also consider direct computation of PSNR in the BS and Head Pose (HP) domain. We first define the equation of the MSE in the BS and HP domain as follows:

where, MSEBS(t) and MSEHP(t) is the mean square error for each time index (frames) t, of the BS and HP weights, respectively. K and L is the number of BS, HP weights used in the application, respectively. BSorii(t) and BSdisi(t) are the ith original and reconstructed BS at frame t.

Then, for each time index, one can obtain two PSNR values:

RBS is the peak-to-peak range of the BS weights and RHP is the peak-to-peak range of the HP weights.

Finally, the PSNR for the complete sequence can be obtained by averaging all PSNR per frame together:

where F is the number of frames in the sequence. Note that one can also get a single value either per frame or per sequence by averaging the PSNR-BS and PSNR-HP together.

The above metrics are inspired by the PSNR-FAP [10], which was developed during MPEG-4 FBA. The equation is as follows:

,

where NFAP is the number of FAP's used for a sequence, Nframes is the number of frames of the sequence, Ri the peak-to-peak range of the ith FAP, MSEi is the mean square error along the sequence for each decoded FAP, and fapori[i][j] and faprecon[i][j] are the original and decoded value of FAP i for frame j.

However, there is a critical distinction between the two metrics: in our case, the MSE computation is performed across all BS/HP for each time index or frame, rather than per FAP over the temporal axis. This adjustment addresses two key issues with PSNR-FAP: (1) it mitigates the disproportionate impact of sparse or inactive BS, whose low MSE values can dominate the metric, and (2) it retains frame-level granularity, capturing temporal variations in reconstruction fidelity. As a result, PSNR-BS+HP provides a more stable and perceptually aligned measure of blendshape compression quality as shown below in Table I.

The evaluation considers three key aspects: accuracy, monotonicity, and consistency. These are measured using the following metrics: Pearson Correlation Coefficient (PCC) and Root Mean Squared Error (RMSE) for accuracy, Spearman Rank Order Correlation Coefficient (SROCC) for monotonicity, and Outlier Ratio (OR) for consistency [11]. The metrics are computed following the application of a non-linear regression to the data, as described in [12].

4	Experimental results

Table 1: SROCC, PCC, OR and RMSE results. Best values are shown in bold, second-best values in bold and underlined, and third-best values in italic bold.

|  | PCC↑ | SROCC↑ | OR↓ | RMSE↓ |
| --- | --- | --- | --- | --- |
| PSNR[3] | 0.548 | 0.538 | 0.146 | 0.822 |
| PSNR HVS[4] | 0.552 | 0.540 | 0.135 | 0.819 |
| SSIM[5] | 0.500 | 0.499 | 0.156 | 0.851 |
| MS-SSIM[6] | 0.507 | 0.499 | 0.167 | 0.847 |
| VIF[7] | 0.561 | 0.547 | 0.146 | 0.813 |
| DLM[8] |  | 0.611 | 0.601 | 0.135 | 0.778 |
| VMAF[9] | 0.598 | 0.588 | 0.135 | 0.787 |
| PSNR-FAP[10] | 0.265 | 0.272 | 0.208 | 0.947 |
| PSNR-BS | 0.323 | 0.287 | 0.198 | 0.930 |
| PSNR-HP | 0.601 | 0.614 | 0.177 | 0.785 |
| PSNR-BS+HP | 0.573 | 0.562 | 0.156 | 0.805 |

Results displayed in Table I highlight the limitations of traditional objective metrics in accurately predicting perceived quality after blendshape compression. Among the evaluated metrics, DLM, VMAF, and PSNR-HP demonstrate the highest correlation with the MOS values, with correlation coefficients reaching approximately 0.6 for both Pearson and Spearman correlations scores. This illustrates the needs of developing better objective metrics tailored blendshape-based facial avatar animation.

5	Conclusion

In this proposal, we have proposed a comprehensive evaluation of traditional objective metrics and highlighted their resulting correlations scores with subjective evaluations. Results reveal the limitations of traditional metrics to accurately predict perceived quality, with DLM, VMAF, PSNR-HP and PSNR-BS+HP achieving the highest correlation with MOS (∼ 0.6). Relying on the PSNR computed in the BS domain is proven to be a robust choice, as it directly evaluates the accuracy of the blendshape representation without being influenced by potential distortions introduced during the rendering process. Unlike PSNR in the pixel domain, which may account for such distortions, this metric focuses solely on the quality of the blendshape compression. Furthermore, it offers a more targeted evaluation of the encoding process, making it particularly relevant for applications prioritizing efficient transmission and reconstruction of facial animations. We recommend to consider the proposed metrics to evaluate future proposals in the Avatar working group.

References

[1] WG03 N01379, “EE Description for Avatar representation formats”, MPEG#148, Kemer, TK, November 2024.

[2] WG03 m70195, “Avatar facial animation stream format and coding scheme”, MPEG#148, Kemer, TK, November 2024.

[3]	Q. Huynh-Thu and M. Ghanbari, “Scope of validity of PSNR in image/video quality assessment,” Electronics Letters, vol. 44, no. 13, pp. 800–801, 2008.

[4]	Karen Egiazarian, Igor Kovalchuk, and Mikhail M. Gabbouj, “New full reference quality metrics based on hvs,” in Proceedings of the Second International Workshop on Video Processing and Quality Metrics. 2006, vol. 4, pp. 1–6, IST/SPIE.

[5]	Zhou Wang, Alan C. Bovik, Hamid R. Sheikh, and Eero P. Simoncelli, “Image quality assessment: from error visibility to structural similarity,” IEEE Transactions on Image Processing, vol. 13, no. 4, pp. 600–612, 2004.

[6]	Zhou Wang, Eero P. Simoncelli, and Alan C. Bovik, “Multi-scale structural similarity for image quality assessment,” Conference Record of the Thirty-Seventh Asilomar Conference on Signals, Systems and Computers, pp. 1398–1402, 2003.

[7]	Hamid R Sheikh and Alan C Bovik, “Image information and visual quality,” IEEE Transactions on image processing, vol. 15, no. 2, pp. 430–444, 2006.

[8]	Songnan Li, Fan Zhang, Lin Ma, and King Ngi Ngan, “Image quality assessment by separately evaluating detail losses and additive impairments,” IEEE Transactions on Multimedia, vol. 13, no. 5, pp. 935–949, Oct. 2011.

[9]	C. G. Bampis, P. K. Yadav, A. M. B. Barbosa, S. S. R. K. M. K. A., and J. A. V. de Lemos, “Spatiotemporal feature integration and model fusion for full reference video quality assessment,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 29, no. 8, pp. 2256– 2270, 2019.

[10] Hai Tao, Homer H Chen, Wei Wu, and Thomas S Huang, “Compression of MPEG-4 facial animation parameters for transmission of talking heads,” IEEE Transactions on circuits and systems for video technology, vol. 9, no. 2, pp. 264–276, 1999.

[11] International Telecommunication Union, “Methodologies for the subjective assessment of the quality of television images,” Recommendation ITU-R BT.500-15, May 2023.

[12] Video Quality Experts Group (VQEG), “Final report from the video quality experts group on the validation of objective models of video quality assessment, phase ii,” Tech. Rep., Video Quality Experts Group (VQEG), 2003.

m71324 [ARF][EE1] Compression Methods and Stream Format for Blend Shape-based Avatar Facial Animation

This proposal has been accepted for inclusion in the EE1 and is documented below.

Introduction

The MPEG 148 meeting discussed the relevant proposals for the avatar WD (working draft) stage, focusing on two aspects: 1) avatar representation format design and 2) avatar animation stream format. This meeting established three exploratory experiments (EEs) on digital humans, among which EE1[1] was about "Compression for Animation Streams". m70195[2] proposed a compression coding scheme for digital human animation parameters and the corresponding bitstream structure. The compression method used inter-frame prediction (residual calculation of adjacent frames), followed by quantization and entropy coding. Experimental results showed that the compression scheme achieved valuable bitrate reduction (around 10 times reduction) while ensuring good visual quality. However, the performance of this scheme considering lossless compression and/or all-intra mode was not considered in the last meeting.

In this proposal, we:

1. Add support for lossless coding and provide resulting bitrate for both lossless coding and all-intra mode;

2. Improve the prediction module, by adding an "intra-frame prediction" to further improve the compression rate;

3. Adjust the code stream structure to support more flexible and efficient transmission.

A notable feature of this contribution concerns the correlation analysis of animation parameters within the same frame, which allow to propose an “intra-frame prediction” mode relying on right/left blendshapes correlation to achieve further compression of the bitstream.

The application scenarios met by this contribution are daily conversation scenarios, which are applicable to the "3.1. Video Conferencing / Remote Calls" and "3.2. Telepresence / Immersive Experiences" use cases in N00301[3].

3	Coding framework

Compared to m70195, a new "scaling" module is added prior to any prediction mode. The overall coding framework consists of scaling, prediction, quantization, and entropy coding modules. The overall coding process is shown in Figure 1.

- ‘Original data’ corresponds to the BlendShape weights value, ranging from 0 to 1, with p effective digits.

- ‘Scaling’ scales up the original decimal value into an integer and retains all valid digits;

- In the ‘Prediction module’, different prediction modes (intra/inter prediction) can be selected to obtain the predicted value of the current one and compute the residual information;

- In the ‘Quantization’ module, it is possible to quantize the residual. Quantization will cause loss when rounding off the data. The value directly input to the entropy coding module without quantization is the predicted value itself.

- ‘Entropy coding’ module uses ‘Exponential Golomb’ codes to obtain the final bitstream.

Note: The dotted border in the block diagram

Fig.1 BlendShape coding flow                   indicates optional modules.

3.1 Introduction to scaling

The scale-up module multiplies the original value (the decimal BS weight value of [0~1]) by an integer power of 10 to enlarge the small data and retain the precision. This operation also satisfies the implementation of lossless coding:

Taking exponential Golomb coding as an example, it requires integers at the input, so the goal of the "scaling" process module is to retain all the decimal digits, and then send the resulting integer to the exponential Golomb encoder for further encoding. The formula is as follows:

is the obtained integer value after scaling, v is the original data, and p is the scaling factor, which is determined by the number of digits after the decimal point of the original data.

3.2 Multiple prediction modes

(1) Intra prediction:

Because each animation parameter in a frame represents the animation of different parts of the digital human, when different animation parameters represent the movement of the same organ, there will be a certain correlation between them. For example, for the two animation parameters "EyeBlinkLeft" and "EyeBlinkRight", when the left eye blinks, the right eye often makes the same movement, and there is a strong correlation between the two. Therefore, for intra-frame prediction, we can use some animation parameters as a reference for the corresponding parameters of other part(s):

BSi is the animation parameter to be encoded in the current frame, BSj is its corresponding reference animation parameter in the same frame, with i ≠ j and pred_intra is the resulting residual in all-intra mode.

Intra-frame prediction requires a correspondence table to reference such existing relationships between BlendShapes, as shown in Table 1.

Table 1. Blend shape reference correspondence table

| Can be referenced (BS*) | Need to refer to BS* |
| --- | --- |
| BS1 | BS2 |
| BS3 | BS4 |
| … | … |
| BSm | BSn |

Note: The reference mapping in this table is related to the scene type of facial animation. This proposal considers the "normal dialogue" scene as use-case, which can meet most "non-extreme" other facial animation scenes.

(2) Inter prediction:

Inter-frame prediction is a time domain redundancy removal process that calculates the residual of two consecutive frames:

is the animation parameter to be encoded in the current frame,  is the animation parameter of the previous frame, and pred_inter is the resulting residual of the animation parameter i in the time domain.

(3) Joint prediction:

For the animation parameter BSi to be encoded in the current frame, two predictions can also be superimposed at the same time:

pred_intra_inter is the residual obtained by intra prediction and then inter prediction.

Table 2. Prediction mode correspondance table

| Predictive mode | Description |
| --- | --- |
| 00 | No prediction |
| 01 | Intra prediction |
| 10 | Inter prediction |
| 11 | Intra-frame prediction followed by inter-frame prediction |

Note: The solution supports a total of 4 prediction modes.

3.3 Quantization module

The quantization module performs scalar quantization on the "prediction residual" data output by the prediction module according to the quantization step size:

Y is the output of the prediction module, Qstep is the quantization step size,  is the quantized value that will be input to the Golomb encoding module, and round() is the rounding operation to the nearest integer.

Different compression levels (or different error losses) are determined by the quantization step. The quantization step is generally described by a quantization parameter, similar to the 'QP' used in video coding. The quantization parameter 'QPBS' of the blend shape can be set. The corresponding relationship between QPBS and Qstep is as follows:

Quantification can lead to loss, however when QPBS = 0, i.e., Qstep =1, the Y (prediction residual) will not be quantized, the encoded value is therefore lossless.

3.4 Stream format

This solution adopts a hierarchical structure design, extracting the common parameters of the entire video sequence frame separately to form a sequence parameter set.

Fig.2 Hierarchical structure of code stream

The parameter set is sent at the beginning of the session; The parameter set and data frame belong to two independent channels, and the parameter set can be reliably transmitted and retransmitted through the connection oriented TCP protocol to ensure the correct reception of the parameter set; Data frames can use UDP protocol to meet real-time requirements.

Fig.3 I frame structure of code stream

Fig.4 P frame structure of code stream

The header information and index information of the p-frame refer to the previous I-frame.

4	Animation stream syntax and semantics

The stream structure includes the syntax and semantics of the sequence set and BlendShape parameters for each frame.

4.1 Sequence Set syntax and semantics

The parameter set information includes the common parameters of the video sequence, and there can be many types of parameter sets, which are referenced by video frames based on their IDs.

Syntax

| aligned(8) class SeqParameterSet(){ | Descriptor |
| --- | --- |
| bs_seq_set_id; | u(4) |
| bs_scale; | u(4) |
| reference_flag; | u(1) |
| reserved; | f(7) |
| blend_shape_set; | u(8) |
| if(reference_flag) |  |
| BS_REFERENCE_ TABLE； | ue(v) |
| } |  |

Semantics

- bs_seq_set_id: 4-bit unsigned integer, specifies the sequence parameter set number to be used.

- bs_scale: 4-bit unsigned integer, scales up the original BS animation parameters.

- reference_flag: 1-bit unsigned integer, whether to include the reference BS information table BS_REFERENCE_TABLE.

- reserved: A 7-bit general frame information reserved field for future extension of defined parameters.

- blend_shape_set: 8-bit unsigned integer, corresponding to the identifier of the BlendShape set. The default value 0 represents the blend shape set of MORGAN.

- BS_REFERENCE_ TABLE: Reference BS information table, as described in Table 1. " Blend shape reference correspondence table ".

4.2 Data Frame syntax and semantics

Header

The header information contains the metadata required to decode each frame of animation, including the frame's timestamp, parameter set number, prediction mode identifier, and some necessary control information identifiers.

Syntax

| aligned(8) class BlendShapeHeader(){ | Descriptor |
| --- | --- |
| timestamp; | u(32) |
| predictive_mode; | u(2) |
| if(predictive_mode == 0 ¦¦ predictive_mode == 1){ |  |
| bs_seq_set_id; | u(4) |
| blend_shape_count_enabled; | u(1) |
| reserved; | f(1) |
| QPBS | u(8) |
| if (blend_shape_count_enabled) { |  |
| blend_shape_count; | u(16) |
| } |  |
| } |  |
| else: |  |
| reserved; | f(6) |

Semantics

- timestamp: 32-bit unsigned integer, specifies the timestamp of the current frame, used for synchronization between frames.

- predictive_mode: 2-bit unsigned integer, the first bit is 0/1 to indicate inter-frame prediction disabled/enabled; the second bit is 0/1 to indicate intra-frame prediction disabled/enabled.

- bs_seq_set_id: 4-bit unsigned integer, specifies the sequence parameter set number used.

- blend_shape_count_enabled: 1-bit Boolean value, specifies whether to enable the selection of the number of blend shapes. If 1, it allows the selection of non-default number of blend shapes.

- QPBS: 8-bit unsigned integer, blend shape weight quantization parameter.

- blend_shape_count: 16-bit unsigned integer, when blend_shape_count_enabled is 1, specifies the total number of blend shapes, and the value does not exceed blend_shape_set.

- reserved: General frame information reserved bit.

Payload

The payload specifies the element values ​​to be encoded according to different prediction modes and control parameters. First, there are 4 cases according to the prediction mode. In each prediction mode, the number of BSs can be freely selected.

Syntax

| aligned(8) class BlendShapeFormat { | Descriptor |
| --- | --- |
| if(!predictive_mode) { // 00, No predictions |  |
| if(blend_shape_set & blend_shape_count_enabled) { |  |
| for (unsigned int i = 0; i < blend_shape_count; i++){ |  |
| blend_shape _index[i]; | ue(v) |
| } |  |
| for (unsigned int i = 0; i < blend_shape_count; i++) { |  |
| blend_shape_weight [i]; | se(v) |
| } |  |
| } |  |
| if(blend_shape_set & !blend_shape_count_enabled) { |  |
| for (int i = 0; i < num(blend_shape_set); i++){ |  |
| blend_shape_weight [i]; | se(v) |
| } |  |
| } |  |
| } |  |
| else if(predictive_mode == 1){ // 01, Intra prediction |  |
| if (blend_shape_set & blend_shape_count_enabled){ |  |
| for (unsigned int i = 0; i < blend_shape_count; i++){ |  |
| blend_shape _index[i]; | ue(v) |
| } |  |
| for (unsigned int i = 0; i < blend_shape_count; i++) { |  |
| if(blend_shape _index[i] in BS_REFERENCE_TABLE ){ |  |
| blend_shape_reference_weight; | se(v) |
| pred_intra; | se(v) |
| } |  |
| blend_shape_ weight[i]; | se(v) |
| } |  |
| } |  |
| if(blend_shape_set & !blend_shape_count_enabled) { |  |
| for (int i = 0; i < num(blend_shape_set); i++){ |  |
| if (blend_shape _index[i] in BS_REFERENCE_TABLE ){ |  |
| blend_shape_reference_weight; | se(v) |
| pred_intra; | se(v) |
| } |  |
| blend_shape_weight[i]; | se(v) |
| } |  |
| } |  |
| } |  |
| else if(predictive_mode == 2){ // 10, Inter prediction |  |
| if (blend_shape_set & blend_shape_count_enabled){ |  |
| for (unsigned int i = 0; i < blend_shape_count; i++){ |  |
| delta_blend_shape_weight[i]; | se(v) |
| } |  |
| } |  |
| if (blend_shape_set & !blend_shape_count_enabled){ |  |
| for (int i = 0; i < num(blend_shape_set); i++){ |  |
| delta_blend_shape_weight[i]; | se(v) |
| } |  |
| } |  |
| }else{ // 11, Inter after intra |  |
| if (blend_shape_set & blend_shape_count_enabled){ |  |
| for (unsigned int i = 0; i < blend_shape_count; i++) { |  |
| for ( blend_shape _index[i] in BS_REFERENCE_TABLE ){ |  |
| delta_blend_shape_reference_weight; | se(v) |
| pred_intra_inter; | se(v) |
| } |  |
| delta_blend_shape_weight [i]; | se(v) |
| } |  |
| } |  |
| if(blend_shape_set & !blend_shape_count_enabled) { |  |
| for (int i = 0; i < num(blend_shape_set); i++){ |  |
| for ( int i: BS_REFERENCE_TABLE ){ |  |
| delta_blend_shape_reference_weight; | se(v) |
| pred_intra_inter; | se(v) |
| } |  |
| delta_blend_shape_ weight [i]; | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |

Semantics

- blend_shape _index[i] : integer array, the index value of the i-th blend shape weight in the current frame, indicating the index represented by the current i-th blend shape.

- blend_shape_weight[i]: Float array, the value of the i-th blend shape in the current frame.

- delta_blend_shape_weight[i]: floating point array, the difference between the current frame blend_shape_weight[i] and the previous frame blend_shape_weight[i] reconstruction value.

- blend_shape_reference_weight: the value of the blend shape weight in the BS_REFERENCE_TABLE of the current frame.

- BS_REFERENCE_TABLE: Configuration information table - BS reference mapping table.

- pred_intra: The residual value obtained by intra-frame prediction of the current frame BS according to blend_shape_reference_weight.

- delta_blend_shape_reference_weight: the temporal difference of the blend shape weight of the current frame BS in BS_REFERENCE_TABLE.

- pred_intra_inter: The temporal difference of the residual value obtained by intra-frame prediction of the current frame BS according to blend_shape_reference_weight.

5	Experimental results

We used a camera to collect a large amount of BlendShape original data from real male and female users considering “dialogue” scenes use-cases, and divide the data into different sub-sequences. Each sub-sequence duration is about 20s and the frame rate is 30fps. According to the compression algorithm designed above, the compression coding of the original data is made, and the rate-distortion performance of one of the sub-sequence of the “maleTalking” type is as follows.

- Bitrate reduction:

Table 3. Bitrate results of 4 QPBS considering the different prediction modes

| Prediction modes Different QPBS | 00 | 01 | 10 | 11 | IEEE 754 (float32) |
| --- | --- | --- | --- | --- | --- |
|  | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) |
| QPBS_11_maleTalking | 15778.86 | 13624.23 | 6346.988 | 5529.088 | 49920 |
| QPBS_13_maleTalking | 11232.91 | 9443.442 | 3914.355 | 3464.923 | 49920 |
| QPBS_14_maleTalking | 9070.43 | 7556.489 | 3086.954 | 2793.666 | 49920 |
| QPBS_15_maleTalking | 7086.816 | 5888.468 | 2500.792 | 2353.735 | 49920 |

The yellow highlight in Table 3 corresponds to the best bitrate reduction, i.e., the "best prediction mode (intra+inter)", which can achieve 9 to 20 times compression at four quantization parameter scales compared with the IEEE 754[4]. The rate-distortion analysis is performed in the following subsections, while the video rendering quality will be shown during the oral presentation.

Fig.5 Bitrate results of 4 QPBS vs IEE754

- Lossless performance:

Table 4 shows the lossless compression (QPBS=0) performance of the scheme. The data shown in the table is the bit rate when bs_scale is 6 (that is, the original data is accurate to 6 decimal digits). In the best prediction mode, the compression ratio achieved is nearly twice that of IEEE 754.

Table 4. Lossless results in different prediction modes

| Prediction modes Different QPBS | 00 | 01 | 10 | 11 | IEEE 754 (float32) |
| --- | --- | --- | --- | --- | --- |
|  | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) | bit_rate (bps) |
| QPBS_0_maleTalking | 41753.803 | 39897.831 | 30156.557 | 28740.516 | 49920 |
| QPBS_11_maleTalking | 15778.86 | 13624.23 | 6346.988 | 5529.088 | 49920 |
| QPBS_13_maleTalking | 11232.91 | 9443.442 | 3914.355 | 3464.923 | 49920 |
| QPBS_15_maleTalking | 7086.816 | 5888.468 | 2500.792 | 2353.735 | 49920 |

- Note1: The yellow highlight in Table 4 is the bit rate comparison between lossless coding and IEEE 754 coding, and the bold data is the best performance of lossless coding of this scheme.

- Note2: When using lower precision values ​​for the capture or renderer, higher lossless compression ratio can be achieved by reducing the value of bs_scale.

Fig.6 Lossless vs Lossy performance

- Rate-distortion performance

This section shows the loss of reconstructed data and original data of blend shape weights under the above four QPBS, which is reflected by calculating the MSE and PSNR of blend shape data.

The definition and related equation(s) of the BS-PSNR can be found in our other proposal m71323 “[ARF][EE1] Assessing Compressed Blendshape-Based Avatar Facial Animation: Objective Metrics vs. Subjective Scores”[5].

Table 5. Rate-distortion performance under 4 QPBS with 4 prediction modes

| Name | Bitrate | PSNR | MSE |
| --- | --- | --- | --- |
| QPBS_11_maleTalking00 | 15778.86 | 65.82 | 2.65E-07 |
| QPBS_11_maleTalking01 | 13624.23 | 65.82 | 2.65E-07 |
| QPBS_11_maleTalking10 | 6346.99 | 65.82 | 2.65E-07 |
| QPBS_11_maleTalking11 | 5529.09 | 65.82 | 2.65E-07 |
| QPBS_13_maleTalking00 | 11232.91 | 53.81 | 4.21E-06 |
| QPBS_13_maleTalking01 | 9443.44 | 53.81 | 4.21E-06 |
| QPBS_13_maleTalking10 | 3914.36 | 53.57 | 4.47E-06 |
| QPBS_13_maleTalking11 | 3464.92 | 53.57 | 4.47E-06 |
| QPBS_14_maleTalking00 | 9070.43 | 47.79 | 1.68E-05 |
| QPBS_14_maleTalking01 | 7556.49 | 47.79 | 1.68E-05 |
| QPBS_14_maleTalking10 | 3086.95 | 47.40 | 1.87E-05 |
| QPBS_14_maleTalking11 | 2793.67 | 47.40 | 1.87E-05 |
| QPBS_15_maleTalking00 | 7086.82 | 41.51 | 7.14E-05 |
| QPBS_15_maleTalking01 | 5888.47 | 41.51 | 7.14E-05 |
| QPBS_15_maleTalking10 | 2500.79 | 41.69 | 7.07E-05 |
| QPBS_15_maleTalking11 | 2353.74 | 41.69 | 7.07E-05 |

Fig.7 R-D Curves under 4 QPBS with 4 prediction modes

6	Conclusion

In this proposal, we have further explored the compression of Blend shape data for facial animation. Specifically, we proposed and implemented a coding scheme with the relevant bitstream structure for Blend shape data facial animation, presented detailed experimental results, and analyzed the performance comparison between the different proposed prediction modes. This coding scheme can be applied to the application scenarios of "3.1. Video Conferencing / Remote Calls" and "3.2. Telepresence / Immersive Experiences" in N00301, and other facial animation scenarios can be adapted accordingly by adjusting the Blend shape reference correspondance table. The experimental results show that this scheme achieves good compression performance while preserving good quality of the animated avatar.

References

[1] WG03 N01379, “EE Description for Avatar representation formats”, MPEG#148, Kemer, TK, November 2024.

[2] WG03 m70195, “Avatar facial animation stream format and coding scheme”, MPEG#148, Kemer, TK, November 2024.

[3] WG02 N00301, “Use Cases for MPEG Avatar,” MPEG#143, Geneva, Switzerland, July 2023.

[4] Kahan W. IEEE standard 754 for binary floating-point arithmetic[J]. Lecture Notes on the Status of IEEE, 1996, 754(94720-1776): 11.

[5] WG03 m71323, “[ARF][EE1] Assessing Compressed Blendshape-Based Avatar Facial Animation: Objective Metrics vs. Subjective Scores”, MPEG#149, Geneva January 2025.

m71325 [SD] Storage of blendshapes in ARF container

This proposal has been accepted for inclusion in the EE1 and is documented below.

The Avatar Representation Format (ARF) as defined in ISO/IEC 23090-39 has adopted a blendshape-based approach for facial animation. The user’s facial expressions are tracked, e.g. by an XR runtime, as a vector of weights, each of which represents the amount a particular blendshape contributes to the user’s current facial expression.

To enable receivers to properly animate the user’s avatar independently of which blendshape set is being used, the ARF container stores the base avatar model’s blendshapes, potentially also in different levels of detail.

The following figures depict some examples of these blendshapes:

The following table shows some selected set of facial tracking frameworks and their associated number of blendshapes:

| Framework | Number of Blendshapes |
| --- | --- |
| Apple ARKit | 52 |
| HTC Vive OpenXR Facial Tracking | 38 |
| Meta (Facebook) OpenXR Face Tracking | 63 and 70 |
| Android OpenXR Face Tracking Extension | 70 |
| Qualcomm QFace | 84 |

As can be noted from the table, a range of 52-84 blendshapes would be required to achieve accurate facial expressions in Avatar communications. This poses significant challenges on the size of the ARF container, especially when multiple levels of detail are also supported.

Blendshape compression

To address this issue, we propose to introduce a simple blendshape compression scheme to be supported in the Avatar Representation Format specification. This scheme is specifically designed for blendshapes and takes into account that the blendshapes only contain updated vertex positions compared to the base mesh (which is the face in neutral expression).

The ARF blendshape compressor follows the following architecture:

The following sections give details about each of these blocks.

Input Validation

Input validation is the first step of the compression pipeline, ensuring that the base mesh and the target blendshape meshes are compatible for processing. This involves loading the base mesh and blendshape meshes and verifying that their topologies match. Topology preservation is confirmed by checking that both meshes have the same number of vertices, the same number of faces, and identical face indices. If any discrepancies are found during these checks, the compression process stops.

Procrustes Transformation

The Procrustes transformation step involves the computation of an optimal transformation for aligning the base and target blendshape meshes to minimize positional differences. First, the centroids of both meshes are calculated by averaging their vertex positions. The centroid is then subtracted from each vertex to center the meshes at the origin.

centroid=1Ni=1Nvertexi

Next, the scales of the meshes are computed by taking the square root of the sum of squared distances of vertices from the centroid. The scale ratio between the meshes is calculated and applied to normalize them as follows:

scale=blendshapeRMSbaseRMS

Subsequently, the rotation matrix is computed using the covariance matrix derived from the centered and scaled vertices. Singular Value Decomposition (SVD) is performed on the covariance matrix to extract the optimal rotation matrix as follows:

H=basenormalizedT.blendshapenormalized
H=U.Σ.VT
R=VT.UT
ifdetR<0, flip sign of last column of R

Transform of Base Mesh and Delta Computation

The determined transform operations (translation, scale, and rotation) are applied to the base mesh to align it with the target blendshape.

The difference or delta computation is then performed. After applying the optimal transformation, the vertex positions of the transformed base mesh are subtracted from the corresponding vertex positions of the target blendshape.

These differences are stored as an Nx3 matrix, where N represents the number of vertices. This matrix represents the deltas, or displacements, that define the deformation of the base mesh into the target shape.

Sparse Encoding

Sparse encoding is used to achieve compression efficiency by encoding only significant vertex deltas. This is driven by the fact that only a small subset of the vertices from the base mesh are deformed to achieve a specific facial expression of the target blendshape.

A threshold, typically set to a small value such as 10-5, is used to determine which deltas are considered significant. Deltas below this threshold are treated as negligible and discarded. The indices of vertices with significant deltas are then stored using a predictive encoding scheme to further reduce storage overhead.

Optional Quantization

Quantization is an optional step that reduces data precision to achieve higher compression ratios. The desired bit depth, such as 8, 12, or 16 bits, is chosen based on a tradeoff between compression efficiency and fidelity to the original mesh. Per-coordinate statistics, such as minimum, maximum, and range values, are calculated to normalize the data. The deltas are scaled to fit within the quantization range, and the scaling parameters are stored for reconstruction. Currently, we use a linear quantization process.

normalized_value=value-minmax-min

quantizedvalue=roundnormalizedvalue* 2bits-1

Bitstream Writing and Parsing

Bitstream writing and parsing involves writing and reading the compressed blendshape bitstream and extracting compressed data and metadata required for the reconstruction of the blendshape. Metadata includes the number of vertices, the transformation matrix, and quantization parameters, if applicable. The compressed data consists of indices and values of the deltas. This step prepares the data for the reconstruction process by ensuring all necessary information is correctly interpreted.

In the writing process, a Zlib encoding (Huffman entropy coding) algorithm is applied to the sparse encoded position data to further reduce the size of the bitstream.

Attribute Re-computation

Certain attributes of the blendshape are changed as part of the deformation from the base mesh. The main example of such an attribute are the normals. Fortunately, the normals of the blendshape don’t have to be carried as part of the compressed blendshape. Instead, they can be easily recomputed after the decompression process.

The process for recomputing normal is as follows:

- Average the normals of all adjacent faces for each vertex.

- Normalize these vectors to unit length.

Results

The following results have been generated using the Alexis blendshape set:

The size savings are depicted in the following figure:

Source Code

The source code will be made available on GitLab. It includes the MeshCompressor as well as scripts for encoding/decoding and evaluating the performance of the compression algorithm.

Proposal

We propose to consider adding a basic compression scheme for blendshapes to reduce the total size of the ARF container to a reasonable size.

Meeting #150

m72256 [Avatar][EE1] Baseline Uncompressed Avatar Animation Bitstream

This proposal has been accepted for inclusion in the output document N1510 Potential improvement of ISO/IEC 23090-39 Avatar representation formats with the following comment: Integration will take some efforts. Author and editor will collaborate.

m72282 [ARF][EE1]Description of the evaluation process of compressed animated streams for avatars

This proposal has been accepted for inclusion in the EE document with the following note: This is only a starting point, more details are needed and we expect anyways that this work will continue in a separate effort in WG07.  The current content is described below:

1 Introduction

This contribution introduces the workflow and details of BlendShape facial animation compression coding, covering the entire process from BS data acquisition, compression encoding/decoding, to rendering and quality evaluation (as shown in Figure 1). This contribution is intended to explain the specific implementation of the animation compression coding experiment in MPEG 149 meeting's proposal m71324 [1] and to provide a reference test scenario for facial animation compression coding as well as quality evaluation aspects.

Figure 1. Overall process of facial animation compression coding experiment

2	Data collection

We previously conducted tests comparing the use of MediaPipe to capture BlendShape data from a regular camera without depth sensing versus using the iPhone's TrueDepth front camera. The results showed that the latter provided superior data accuracy and rendered visual quality. Therefore, in our previous proposals, we ultimately adopted TrueDepth in combination with the Unreal-compatible LiveLinkFace [2] to capture BlendShape data under the ARKit framework [3]. Note that extracted data through OpenXR can follow the same procedure as long as the data respect the format shown in Table 1.

Table 1. Format of BlendShape animation data collected by LiveLinkFace

| StartTimecode | BlendShape Count | BS1 | BS2 | … | BS3 |
| --- | --- | --- | --- | --- | --- |
| 07:55:19:20.720 | 61 | 0.043679033 | 0.207646146 | … | 0.043679033 |
| 07:55:19:21.721 | 61 | 0.028177874 | 0.143405675 |  | 0.047394778 |
| … | … | … | … | … | … |

Note: 1) The meaning of startTimecode: System timer. The format is: Hour: minute: second: frame : offset. 2) We excluded 6 eye movements and 3 head rotation data because they were based on joint movements, and finally obtained the remaining 52 BSs.

Data collection steps:

Step 1: Download LiveLinkFace from the Apple Store and select the capture mode as LiveLink (ARKit).

Step 2: Click the record button to capture animation parameters (you can perform face calibration before recording, set the target frame rate, and enable or disable head rotation).

Step 3: Once the recording is complete, you can find the stored ZIP file in the files directory and export it. The ZIP file contains a JSON file with recorded configuration information and a CSV sequence of BlendShape animation parameters.

3	BS Coding framework

The raw data is processed through the compression framework designed in m71324. After performing intra-frame and inter-frame redundancy reduction, the residual data is quantized and entropy coded to generate the final binary bitstream. The decoding process takes a binary file as input and outputs the decoded CSV data. In our latest implementation, we provide a visualized UI interface (as shown in Figure 3), allowing users to customize the number of BlendShapes, QP, reference mapping table (required for intra-frame prediction), and prediction mode parameter configurations.

Figure 2 BS compression encoding and decoding process

Figure 3 User defined parameter setting UI

3	Animation stream syntax and semantics

The binary ‘.bin’ file undergoes inverse quantization and inverse prediction to reconstruct data that matches the original count. The figure below illustrates a comparison between the reconstructed data and the original data. Additionally, we provide MSE and PSNR-BS metrics to evaluate the quality loss in data reconstruction.

Table 2. Raw BS Collection Data

| Timecode | BlendshapeCount | EyeBlinkLeft | EyeLookDownLeft | EyeLookInLeft |
| --- | --- | --- | --- | --- |
| 01:38:51:14.001 | 52 | 0.106404528 | 0.117343955 | 0.20256798 |
| 01:38:51:15.001 | 52 | 0.104117289 | 0.114088826 | 0.205764115 |
| 01:38:51:16.001 | 52 | 0.098501429 | 0.109629251 | 0.209680691 |
| 01:38:51:17.001 | 52 | 0.09403608 | 0.101047948 | 0.215120524 |
| 01:38:51:18.002 | 52 | 0.085975409 | 0.098150984 | 0.221841201 |
| 01:38:51:19.001 | 52 | 0.078614622 | 0.096047677 | 0.224572867 |
| 01:38:51:20.002 | 52 | 0.067380451 | 0.092128657 | 0.228263468 |

Table 3. Reconstruction values for quantization parameter=11 and prediction mode=11

| Timecode | BlendshapeCount | EyeBlinkLeft | EyeLookDownLeft | EyeLookInLeft |
| --- | --- | --- | --- | --- |
| 01:38:51:14.001 | 52 | 0.106496 | 0.116736 | 0.202752 |
| 01:38:51:15.001 | 52 | 0.104448 | 0.114688 | 0.2048 |
| 01:38:51:16.001 | 52 | 0.098304 | 0.110592 | 0.208896 |
| 01:38:51:17.001 | 52 | 0.094208 | 0.100352 | 0.21504 |
| 01:38:51:18.002 | 52 | 0.086016 | 0.098304 | 0.221184 |
| 01:38:51:19.001 | 52 | 0.077824 | 0.096256 | 0.22528 |
| 01:38:51:20.002 | 52 | 0.067584 | 0.09216 | 0.227328 |

Table 4. Reconstruction Quality Loss

| rec_QP11_Pre11 |  |
| --- | --- |
| PSNR-BS | MSE-BS |
| 64.58118768 | 3.48242E-07 |
| 65.88146479 | 2.58139E-07 |
| 65.89137334 | 2.57551E-07 |
| 65.74540627 | 2.66354E-07 |
| 65.80145275 | 2.62939E-07 |
| 65.75955015 | 2.65488E-07 |
| 66.99720445 | 1.99655E-07 |

4	Rendering platform scene construction

Regarding the selection of the rendering platform, we considered various platforms such as Unity and Unreal Engine (UE). Ultimately, we chose in our previous proposal, UE due to its more realistic MetaHuman digital human model and better compatibility with the animation data format of LiveLinkFace. Note that the current MPEG Pyrenderer could also be used following similar procedures as described below to output videos for further quality evaluation.

Below are the steps for setting up the test scene in UE:

(1) Environment Resource Configuration

-Step 1: Download and install Epic Games Launcher, install Unreal Engine 5.4.4, and create a new project.

-Step 2: Install plugins: In UE5, open "Plugins" (Edit > Plugins). Search for Live Link and enable the Live Link and Live Link Face plugins. Install and enable the MetaHuman plugin.

(2) MetaHuman Model Selection

-Step 3: Import MetaHuman model: Use Quixel Bridge to download MetaHuman. Open Quixel Bridge and log in. In the MetaHuman tab, select the desired character and download it. After downloading, click "Export," and Quixel Bridge will automatically import it into UE5. In the Content Browser, find the MetaHuman character and drag it into the scene.

(3) Recording Studio Scene Setup

-Step 4: Build the "recording studio" scene. One can search for "Metahuman Lighting[4]" in the Epic Games Marketplace and add it to the library. This add-on will provide a better and easier digital human rendering scene. One can view all the assets in the Epic Fab Library.

The configured scene is shown below. One can drag the MetaHuman character model into this scene, adjust its position, and configure the camera and lighting parameters.

Then one can configure a camera for the scene, in our previous proposals we have chosen the CineCameraActor. After configuring the camera parameters (position, focal length, etc.), one can click to preview the rendering angle effect under that camera.

(4) Configure LiveLinkFace animation

-Step5: Enable the LiveLinkFaceImporter plugin to be able to drag-in captured CSV files and drive character models using locally stored captured and compressed animation data. Drag the CSV file into the Content Browser to get the BlendShape sequence, then double-click on it.

-Step6:  Set up Live Link for real-time facial animation. Drag the avatar and camera from the Outliner directory on the right to the left side of the sequence window, so as to get the bound head and body controllers. Note that in our case, the controller for the body has been deleted, as Facial BlendShape data cannot control the body.

Finally, in the Live Link ARKit Face Subj of the avatar detail panel, one can select his CSV data as the driver source and choose Use ARKit Face.

Then one can drag the slider to check the animation effect of each frame.

(5) Rendering and exporting

-Step7:  Click on the animation to view the animation effect, confirm it is correct, and then click on render settings to output a video. One can set the rendering resolution, container format to store, and save directory.

5	Quality evaluation

Once the rendered facial animation videos have been exported (lossless format shall be employed for valid comparison), one may compute standard video-based quality metrics (e.g., Peak Signal-to-Noise Ratio (PSNR) [5], PSNR with consideration for the Human Visual System (PSNR-HVS) [6], Structural Similarity Index Measure (SSIM) [7], Multi-Scale Structural Similarity Index Measure (MS-SSIM) [8], Video Multi-method Assessment Fusion (VMAF) [9], etc.) between the reference and compressed animations.

Alternatively, one can also conduct subjective quality evaluation experiments to calculate the MOS score. However, rendering high-resolution avatar animations for each sequence and each compression configuration is time-consuming and resource-intensive, especially when evaluating multiple QP levels, prediction modes, and bitrates. Furthermore, video rendering can introduce additional noise or artifacts—such as lighting inconsistencies, mesh instabilities, or camera motion—that are not intrinsic to the compression method itself, thus negatively impacting the evaluation.

To address these challenges, an alternative and more efficient approach is to compute distortion metrics directly in the data domain, specifically in the BlendShape (BS) and Head Pose (HP) parameter space. As described in our previous contribution [10], we calculate per-frame and sequence-level MSE and PSNR values across the BS and HP weights. The process is recall below for completeness. We first define the equation of the MSE in the BS and HP domain as follows:

where, MSEBS(t) and MSEHP(t) is the mean square error for each time index (frames) t, of the BS and HP weights, respectively. K and L is the number of BS, HP weights used in the application, respectively. BSorii(t) and BSdisi(t) are the ith original and reconstructed BS at frame t.

Then, for each time index, one can obtain two PSNR values:

RBS is the peak-to-peak range of the BS weights and RHP is the peak-to-peak range of the HP weights.

Finally, the PSNR for the complete sequence can be obtained by averaging all PSNR per frame together:

where F is the number of frames in the sequence. Note that one can also get a single value either per frame or per sequence by averaging the PSNR-BS and PSNR-HP together.

The above metrics are inspired by the PSNR-FAP [11], which was developed during MPEG-4 FBA. The equation is as follows:

,

where NFAP is the number of FAP's used for a sequence, Nframes is the number of frames of the sequence, Ri the peak-to-peak range of the ith FAP, MSEi is the mean square error along the sequence for each decoded FAP, and fapori[i][j] and faprecon[i][j] are the original and decoded value of FAP i for frame j.

However, there is a critical distinction between the two metrics: in our case, the MSE computation is performed across all BS/HP for each time index or frame, rather than per FAP over the temporal axis. This adjustment addresses two key issues with PSNR-FAP: (1) it mitigates the disproportionate impact of sparse or inactive BS, whose low MSE values can dominate the metric, and (2) it retains frame-level granularity, capturing temporal variations in reconstruction fidelity.

These above data-domain metrics offer a precise measure of the fidelity of the reconstructed animation parameters, isolating the compression performance from rendering-related effects.

Compared to pixel-domain metrics, BS-domain evaluation offers the following key advantages:

- Efficiency: Avoids the need to render and export videos, significantly reducing evaluation time and computational burden.

- Clarity: Eliminates confounding factors introduced during rendering (lighting, skin shaders, camera setup), focusing the evaluation purely on compression accuracy.

- Granularity: Retains per-frame analysis capability, enabling fine-grained temporal assessment of fidelity (e.g., frame-by-frame drops or artifacts).

- Alignment with Source Domain: Evaluates the quality of what is actually transmitted and reconstructed—the BS weights—not the downstream visual representation that depends on the rendering pipeline.

- Reproducibility: Independent of rendering engine or MetaHuman versions, which may vary across users or platforms, leading to inconsistent visual outputs.

These advantages make BS-domain evaluation particularly well-suited for early-stage codec development, algorithmic benchmarking, and bitrate-distortion analysis.

Moreover, correlation studies between objective metrics and Mean Opinion Scores (MOS) from subjective tests show that BS/HP domain metrics like PSNR-BS and PSNR-BS+HP outperform traditional video metrics (e.g., PSNR, SSIM) in predicting perceived animation quality, with DLM, VMAF, and PSNR-BS+HP achieving the highest correlations (∼0.6)[10]. These findings underscore the perceptual relevance and technical robustness of PSNR-BS as a core evaluation metric.

6	Conclusion

In this proposal, we review the whole evaluation process of compressed animated streams for avatars, illustrated considering the ARKIT and Unreal Engine platform, while being compatible with any other BS extraction process and/or rendering platform. We recommend that future proposals related to animation streams compression and submitted to the MPEG Avatar Representation Format adopt PSNR-BS+HP and other data-domain metrics as primary quality indicators, complemented by rendered-based evaluation only when assessing end-user perception or when evaluating rendering fidelity itself.

7	Appendix

Table 5. Data collection and encoding/decoding

| Dataset collection |  | CODEC |  |
| --- | --- | --- | --- |
| Software | Hardware | Software | Hardware |
| LiveLinkFcae ARKit | iPhone14pro | Python3.8 Conda | CPU Intel(R) Core(TM) i7-9700KF CPU @ 3.60GHz; x64 ; System Windows 11 Pro (22H2； 22621.4169)； GPU：NVIDIA GeForce RTX 2060(6GB), Driver Version:561.09, CUDA Version:12.6 |

Table 6. Rendering platform and evaluation

| Render platform Unreal engine 5.4.4 |  |  |
| --- | --- | --- |
| Plug-in Dependencies | Lighting scene | Evaluation |
| Live Link; Live Link Control Rig; Live Link Curve Debug UI; LiveLinkFaceImporter; metahuman. | Metahuman Lighting[5] | VMAF-3.0.0[12] FFmpeg-7.1.0[13] |
|  |  | Xiaomi display (RMMNT27NQ, 2K resolution,27-inch) |

8	References

[1] WG03 m71324, “[ARF][EE1] Compression Methods and Stream Format for BlendShape-based Avatar Facial Animation”, MPEG#149, Geneva January 2025.

[2] https://apps.apple.com/us/app/live-link-face/id1495370836

[3] https://developer.apple.com/documentation/arkit/arfaceanchor/blendshapelocation

[4] https://www.fab.com/listings/52f008f2-bfd2-4db1-b9f5-94c5b1512b8a

[5] Q. Huynh-Thu and M. Ghanbari, “Scope of validity of PSNR in image/video quality assessment,” Electronics Letters, vol. 44, no. 13, pp. 800–801, 2008.

[6] Karen Egiazarian, Igor Kovalchuk, and Mikhail M. Gabbouj, “New full reference quality metrics based on hvs,” in Proceedings of the Second International Workshop on Video Processing and Quality Metrics. 2006, vol. 4, pp. 1–6, IST/SPIE.

[7] Zhou Wang, Alan C. Bovik, Hamid R. Sheikh, and Eero P. Simoncelli, “Image quality assessment: from error visibility to structural similarity,” IEEE Transactions on Image Processing, vol. 13, no. 4, pp. 600–612, 2004.

[8] Zhou Wang, Eero P. Simoncelli, and Alan C. Bovik, “Multi-scale structural similarity for image quality assessment,” Conference Record of the Thirty-Seventh Asilomar Conference on Signals, Systems and Computers, pp. 1398–1402, 2003.

[9] C. G. Bampis, P. K. Yadav, A. M. B. Barbosa, S. S. R. K. M. K. A., and J. A. V. de Lemos, “Spatiotemporal feature integration and model fusion for full reference video quality assessment,” IEEE Transactions on Circuits and Systems for Video Technology, vol. 29, no. 8, pp. 2256– 2270, 2019.

[10] WG03 m71323, “[ARF][EE1] Assessing Compressed Blendshape-Based Avatar Facial Animation: Objective Metrics vs. Subjective Scores”, MPEG#149, Geneva January 2025.

[11] Hai Tao, Homer H Chen, Wei Wu, and Thomas S Huang, “Compression of MPEG-4 facial animation parameters for transmission of talking heads,” IEEE Transactions on circuits and systems for video technology, vol. 9, no. 2, pp. 264–276, 1999.

[12] https://github.com/Netflix/vmaf/blob/master/resource/doc/windows.md

[13] https://ffmpeg.org/download.html

m72367 Compression of joint animation data

This proposal has been accepted for inclusion in the EE document to be further studied as part of the EE discussion.

1	Introduction

Avatar animation in virtual and augmented reality applications requires efficient representation of skeletal motion to enable real-time transmission and rendering. The animation data typically consists of transformation matrices for each joint in the skeleton, which define the position, orientation, and scale of the joint relative to its parent or in world space. These transformations are represented as 4×4 matrices, with each matrix containing 16 floating-point values. For complex avatars with many joints, this representation can consume significant bandwidth and storage.

This document presents a joint compression scheme that leverages the hierarchical structure of skeletal animation to achieve efficient compression while maintaining visual quality.

2	Joint Animation Sample Compression

2.1	Overview

Each joint transformation is typically represented as a 4×4 matrix T that combines rotation, translation, and sometimes scale:

T=R11R12R13txR21R22R23tyR31R32R33tz0001

where R is a 3×3 rotation matrix and tx,ty,tz is a translation vector. The rotation matrix R can also incorporate scale and shear, although pure rotations are more common in skeletal animation.

For a skeleton with n joints, the complete pose requires n such transformation matrices, resulting in 16n floating-point values. At 32 bits per float, this amounts to 512n bits per frame. For a complex avatar with 70 joints animated at 60 frames per second, this results in approximately 2.15 MB of data per second, which is prohibitively large for real-time transmission in bandwidth-constrained environments.

The proposed compression scheme consists of the following key components:

- Scale-Rotation Decomposition: Separating scale and rotation components from the transformation matrix for more efficient representation. We propose to modify the joint animation sample to incorporate this, which would also align with the format of the XrPosef structure from OpenXR.

- Quaternion Conversion: More efficient representation of quaternions.

- Hierarchical Encoding: Representing child joint transformations relative to their parent joints.

- Component Quantization: Mapping floating-point values to integers within a fixed range for rotation, translation, and scale components.

- Delta Encoding: Representing transformations as differences from previous frames.

2.2 	Scale-Rotation Decomposition

The first step in the compression process is to decompose the 4×4 transformation component of each joint matrix into separate scale and rotation components. This decomposition is important because it allows us to handle scale and rotation separately, which leads to more efficient compression and better quality.

A general 4×4 transformation matrix M can include rotation, scale, and potentially shear components. We extract the scale factors by computing the magnitude of each column vector:

sx=M112+M212+M312

sy=M122+M222+M322

sz=M132+M232+M332

Once we have the scale factors, we can normalize the matrix to obtain a pure rotation matrix R:

Rij=Mijsj

This decomposition allows us to represent the transformation as a pair of a pure rotation and a scale vector, where each can be compressed more efficiently than the original matrix.

2.3 	Quaternion Conversion and Optimization

After extracting the pure rotation matrix, we convert it to a quaternion representation. The conversion from a rotation matrix R to a quaternion q=w,x,y,z is performed using the following algorithm:

- Compute the trace of the rotation matrix: trace=R11+R22+R33

- If trace>0, compute the quaternion as:

s=0.5/trace+1.0

w=0.25/s

x=R32-R23×s

y=R13-R31×s

z=R21-R12×s

- Otherwise, determine which diagonal element has the greatest value and compute the quaternion accordingly.

Once the quaternion is obtained, we exploit the unit constraint to reduce the storage requirement. Since w2+x2+y2+z2=1, we can omit one component and reconstruct it during decompression. To minimize the reconstruction error, we choose to omit the component with the largest absolute value, as this component can be reconstructed with the highest precision.

Let qmax be the component with the largest absolute value. We store the index of this component along with the other three components. During decompression, we reconstruct qmax as:

qmax=1-q12-q22-q32

where q1,q2,q3 are the three stored components. The sign of qmax is determined by ensuring that the quaternion has a non-negative largest component, which is achieved by negating all components if necessary during compression.

2.4 Component Quantization

To reduce the data size, we quantize the quaternion components, translation vector, and scale vector to fixed-point integers. The quantization process maps a floating-point value v in the range vmin,vmax to an integer i in the range 0,2b-1, where b is the number of bits:

i=roundv-vminvmax-vmin×2b-1

During decompression, the original value is approximated as:

v'=i2b-1×vmax-vmin+vmin

For each component type, we use different ranges:

- Quaternion Components: The range is -1,1, as quaternion components are always within this range.

- Translation Components: The range is -trange,trange, where trange is a parameter that can be adjusted based on the expected range of translations in the animation. For intra-frame compression, we typically use a larger range (e.g., 5.0 units) to accommodate the full range of possible positions. For inter-frame compression, we can use a smaller range (e.g., 1.0 unit) since the delta between consecutive frames is usually small.

- Scale Components: The range is 1-srange,1+srange, where srange is a parameter that can be adjusted based on the expected range of scale factors. We center the range around 1.0 because scale factors are typically close to 1.0 (no scaling). For intra-frame compression, we might use a larger range (e.g., 2.0), while for inter-frame compression, we can use a smaller range (e.g., 1.0) for the scale delta.

The number of bits b for each component type is a parameter that controls the trade-off between compression ratio and precision:

- Rotation Components: typically 8-16 bits, with higher values providing better angular precision.

- Translation Components: typically 8-16 bits, with higher values providing better positional precision.

- Scale Components: typically 8 bits, as scale factors usually don’t require as much precision as rotation or translation.

These bit allocations can be adjusted based on the specific requirements of the application, with higher values providing better precision but resulting in larger data sizes.

2.5 	Hierarchical Encoding

The hierarchical encoding approach exploits the parent-child relationships in the skeleton to improve compression efficiency. Instead of encoding each joint transformation in world space, we encode child joint transformations relative to their parent joints.

For a joint with index j and parent index p, the local transformation Tjlocal is computed as:

Tjlocal=Tpworld-1×Tjworld

where Tjworld is the world-space transformation of joint j and Tpworld is the world-space transformation of its parent.

The local transformation typically has smaller translation components than the world-space transformation, allowing for more efficient quantization. During decompression, the world-space transformation is reconstructed as:

Tjworld=Tpworld×Tjlocal

The hierarchical encoding process requires processing joints in a specific order: parents must be processed before their children. This is achieved by sorting the joints based on their depth in the hierarchy, where the depth of a joint is the number of ancestors it has.

2.6 	Delta Encoding

For animations with high temporal coherence, delta encoding can provide additional compression by representing transformations as differences from previous frames. In the inter-frame compression mode, we encode the difference between the current transformation and the previous transformation.

For quaternions, the delta is computed as:

qdelta=qprev-1×qcurr

where qprev is the quaternion from the previous frame and qcurr is the quaternion from the current frame. The inverse of a quaternion q=w,x,y,z is its conjugate normalized to unit length: q-1=w,-x,-y,-z for unit quaternions.

For translations, the delta is computed as:

tdelta=RprevT×tcurr-tprev

where tprev and tcurr are the translation vectors from the previous and current frames, respectively, and Rprev is the rotation matrix from the previous frame. This formula rotates the translation difference into the local space of the previous frame, which typically results in smaller values that can be quantized more efficiently.

For scale factors, the delta is computed as a ratio:

sdelta=scurrsprev

where sprev and scurr are the scale vectors from the previous and current frames, respectively. This ratio representation is more appropriate for scale factors than a simple difference, as scale factors are multiplicative in nature. The ratio is typically close to 1.0 for small changes in scale, which allows for efficient quantization.

During decompression, the current transformation is reconstructed as:

qcurr=qprev×qdelta

tcurr=tprev+Rprev×tdelta

scurr=sprev×sdelta

2.7	Bit Representation

The compressed representation of a single joint transformation consists of the following components:

- Quaternion Index: 2 bits to indicate which quaternion component was omitted (0 for w, 1 for x, 2 for y, 3 for z).

- Quaternion Components: 3 × br bits for the three stored quaternion components, where br is the number of bits per rotation component.

- Translation Components: 3 × bt bits for the three translation components, where bt is the number of bits per translation component.

- Scale Components: 3 × bs bits for the three scale components, where bs is the number of bits per scale component.

The total size for a single joint transformation is 2+3br+3bt+3bs bits. For a skeleton with n joints, the total size per frame is n×2+3br+3bt+3bs bits.

In our implementation, we typically use: br=8 to 16 bits for rotation components bt=8 to 16 bits for translation components bs=8 bits for scale components

This results in a total of 50 to 98 bits per joint, compared to 512 bits for the uncompressed representation (16 floats × 32 bits), achieving a compression ratio of 5.2× to 10.2×.

3	Proposal

This contribution outlined a compression scheme for joint animation data. We propose to document this scheme as part of the EE on animation data compression and to perform the following:

- Collect animation data for the evaluation

- Develop metrics for the evaluation of the joint animation compression scheme and quality

- Evaluate the compression scheme

## 1.9 Extracted from TuC

## 1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case can be with live content while another in the on-demand content.

## 1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria #1 | Description | The technical solution should minimize/optimize … |

## 1.12 Timeline

MPEG #150: Document solutions and evaluation results and recommend changes to the CD text.

## 1.13 References

[1] WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] WG03 N0XXXX, “WD of ISO/IEC 23090-38 Avatar representation formats”, Kemer, Turkey, November 2024.

# [4] WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.

EE2: Integrating on Geometry Data Components for Avatar Data (closed)

EE-2 was closed at MPEG#150 as all information is moved to the updated standard.

## 1.1 Introduction

Currently, there are several formats (FBX, USD or glTF) for encoding 3D content including mesh rigs, e.g., deformable 3D objects with coefficient parametrizations. None of these formats are dedicated to avatar data representation. In most cases the definition and parsing of the avatar data structure is given to the application has a task, such as parsing existing formats.

The non existing of a standardized avatar format motivate for the initiation of the current work and the combined effort to produce a format that is interoperable between application and compatible with existing frameworks.

The goal of the EE is thus to specify the integration of avatar data information into a container format that is accessible in the avatar data model. The objective is to improve interoperability, avoid the exclusive used of external limited formats and provide a complete representation of an avatar format in MPEG-I. The EE will study the integration of the data components and based on this integration and the possible identified technical gaps; the EE aims at defining the necessary data components to the data structure of the avatar representation format.

## 1.2 Problem statement

An interoperable avatar representation format should be capable of allowing other formats to decode, understand, map and utilize its data model structures for individual applications. In the current days an avatar format is a combination of several formats (illustrated in Figure X).

Figure 1 - Ecosystem of avatar asset creation.

This is impractical and not conformed with the claim for an interoperable format.

An interoperable format requires:

- Representation and definition of basic structures e.g., meshes, joints, textures and external file references.

## 1.3 Use cases relevant for the EE

The work of the EE is based on the requirements defined in N00359 and use cases identified in m64008 .

The requirements defined in N00359 are currently organized into five different categories including: Avatar Representation, Coding format, Transport/Synchronization/Carriage, Storage and Privacy and Security requirements.

The requirements related to the “Avatar Representation” are summarized as follows:

- Interoperability: allow converting to and from other avatar representation.

- 3D Scenes: to allow integration into scene description solution.

- Attributes: requirements applicable to the features supported by the avatar, such as, CGI features as textures, geometries, skeletons, accessories, semantic definitions.

The requirements related to the “Coding Format” are summarized as follows:

- General: requirements applicable to the whole format and its use

- Geometry: requirements applicable to the geometry of the base avatar

- Interaction: requirements that relate to the interaction between Avatars and between Avatars and other objects in the scene

- Animation and control: requirements on the facial and body animation

Transport/Synchronization/Carriage: requirements related to the transport of the base avatar model and the associated animation streams

- Storage: requirements that pertain to the storage of the base avatar model

- Security and privacy: requirements on the protection of the avatar in different scenarios

The requirements and use cases explicit mention the interoperability concept to allow converting to and from other representation and the representation of all components of an avatar including but not exclusive to geometries, textures, skeletons, accessories and semantics.

## 1.4 Related (WG2) and Extracted (new) Requirements

## 1.5 Relation to other activities (EE, requirements, etc…)

## 1.6 Mandates

## 1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Interdigital | Joao Regateiro | joao.regateiro@interdigital.com | L |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | P |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | P |

(P = proponent, L = leader)

## 1.8 Information about proposed technologies

The following contributions on XXXX have been submitted:

Meeting #148

m69421 Avatar JSON Interchangeable Format

Meeting #149

m71292 [SD][Avatar][ARF] Avatar Representation Format Data Types.

This proposal has been accepted for inclusion in the EE2 and is documented below.

- Introduction

In the MPEG#148 meeting, a new part 39 of the ISO/IEC 23090 standard on Coded Representation of Immersive Media was initiated in output document N01398 [1], targeting the definition of an Avatar Representation Format (ARF). In this scope, three Exploratory Experiments (EE) [2] were launched to investigate potential improvements to the initial version of the standard Working Draft (WD).

This contribution addresses EE2 on “Integrating on Geometry Data components for Avatar Data”. It proposes to add enrich the Data component with new types that encode the representation of low-level geometry and texture features of the avatar within ARF. It thereby addresses the following high-priority requirements in [3]:

| Category | Req # | Description | Priority |
| --- | --- | --- | --- |
| Avatar Representation | 194 | A suitable exchange format for conversion between avatar representation formats in market | High |
|  | 196a | Representation format includes description of body, skeletal, etc. | High |
|  | 199 | Mesh-based format for representation and animation | High |
| Storage | 235 | Base avatar stored in single file | High |
|  | 236 | Partial access | High |

Table 11: requirements from [3] addressed by the contribution

- Rationale for embedding a mesh representation format in ARF

In the current version of the ARF WD [1], the data that define the geometry and texture of the avatar model, e.g., vertex coordinates and UV maps, are represented within the Mesh component as Data items. Essentially, as shown on Table 12, a Data item references through the uri property an encoding of the geometry and texture of the avatar in an external representation format such as glTF binary. This external encoding format is specified as a MIME type by the type property.

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| name | string | M | a string that defines the name of this data. |
| type | string | M | a string that provides the mime type of the data. |
| uri | string | M | a string that defines the data content or reference to the data content depending on type. |
| offset | integer | O | defines the number of bytes used as offset into the data content as pointed to by uri. |
| byteLength | integer | O | defines the number of bytes to use in data content. |
| compression | string | O | an identifier of the compressor used to compress this LoD representation of the mesh. |
| protection | number | O | an identifier of the protection configuration that is applied to encrypt this LoD representation of the mesh. |

Table 12: Data component specification in the current ARF WD [1]

Platforms

First, updates of the external avatar geometry and texture encoding formats used in the ARF description, such as glTF, currently needs to be managed by the platforms receiving the description. This puts a burden on the platforms by requiring that they can cope with several signaling ways of all the geometry and texture encoding formats they support.

Not Avatar-Format

Second, a mesh representation relying on external encoding formats that are not avatar-specific are likely to lack avatar modeling attributes that will be needed in future versions of ARF to improve on the functionality and realism of the avatar representation.

Skinning

Third, the ARF WD includes a skin.weights property that references an array of floating-point weights associated with every vertex of the mesh. It is unclear how this array of weights can be represented as a Data item. A candidate MIME type would be application/octet-stream, but this type does not provide any information on the structure or typing (e.g., float32, float64) of the data. It is also unclear how a GLB buffer could be used to represent ARF skinning weights, since in glTF the skinning weights and the joints to which they refer are attributes of a mesh primitive. Hence, the GLB buffer would need to contain both mesh, skeleton and skinning information.

Indexing

Fourth, there is no robust solution when indexing is required between external data and ARF content. For instance, if the vertices are expected to be ordered in a specific way (like for the storage of skin weights), there are no rule nor guarantee that the vertex order of the mesh in an external file (like .glb) is as expected.

For these reasons, we propose to enrich ARF with an internal representation format, e.g., for the avatar geometry and texture, as an alternative to referencing an external representation format.

- Proposed ARF representation format for avatar geometry and texture

To maintain consistency with the current ARF WD scheme, we propose to represent avatar geometry and texture properties as a hierarchy of JSON objects and encode them as ARF Data items. To this purpose, these objects will be assigned types that will be registered by the Internet Assigned Numbers Authority (IANA) as subtypes of a new model/arf MIME type.

Data types

The objects representing the geometry and appearance of the avatar reference data buffers for storing, e.g., the 3D coordinates of the mesh vertices or the pixel intensities of the avatar texture images. To comply with the current design in the ARF WD, these buffers must be encoded as ARF Data objects with dedicated type values. Accordingly, we propose to extend the ARF Data component with new data types with additional type-specific properties, highlighted in green in Table 13 below.

The dense data type represents regular multi-dimensional arrays.

The image data type represents an image, i.e., a 2D array of pixel intensities encoded in a given color space.

The tuples data type represents a list of tuples, each tuple having 0 or more values. The tuples in the list may have different value counts, but the data type of all values in all tuples e.g., 16-bit IEEE 754 floating point numbers, must be the same. The tuples type could be used, for instance, to encode the faces of a mesh composed of a mix of triangles and quads, as a list containing 3-tuples (for vertex indices of triangular faces) and 4-tuples (for vertex indices of quad faces).

The sparse data type represents a sparse tensor encoded as a list of 2-tuples (index, value), where index is the location of the value in the tensor. An index is the 1D indexing of cells in a tensor. For tensors with more than one dimension, the 1D indexing is obtained by “flattening” the tensor to a 1D tensor. Sparse tensors could be used, for instance, to encode the skinning weights of the mesh vertices. Indeed, many vertices are bound to fewer than the maximum allowed number of joints, hence have 0 weight entries. As a result, the skinning weight tensor has a large number of 0 entries.

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| name | string | M | a string that defines the name of this data. |
| type | string | M | a string that provides the mime type of the data. |
| uri | string | M | a string that defines the data content or reference to the data content depending on type. |
| offset | integer | O | defines the number of bytes used as offset into the data content as pointed to by uri. |
| byteLength | integer | O | defines the number of bytes to use in data content. |
| compression | string | O | an identifier of the compressor used to compress this LoD representation of the mesh. |
| protection | number | O | an identifier of the protection configuration that is applied to encrypt this LoD representation of the mesh. |
| If type is “model/arf.dense” { |  |  |  |
| dims | integer [1-*] | M | A list of integers that define the dimension of the tensor e.g., dimension of [2, 7, 4] refers to a tensor with 2 * 7 * 4 = 56 values, where the first element of the tensor has dimension 2, the second element has dimension 7 and the last element has dimension 4. |
| dtype | string | M | A string that defines the type of values e.g., “f32”, “f64”, “i8”. |
| } |  |  |  |
| If type is “model/arf.sparse” { |  |  |  |
| dims | integer [1-*] | M | A list of integers that define the dimension of the tensor e.g., dimension of [2, 7, 4] refers to a tensor with 2 * 7 * 4 = 56 values, where the first element of the tensor has dimension 2, the second element has dimension 7 and the last element has dimension 4. |  |
| count | integer | M | Number of a 2-tuple (index, value) in data content |  |
| dtype | string | M | A string that defines the type of values e.g., “f32”, “f64”, “i8”. |  |
| itype | string | M | A string that defines the type of values e.g., “i32”, “i64”. |  |
| } |  |  |  |
| If type is “model/arf.image” { |  |  |  |
| mimeType | string | O | Image mime type, such as, “image/png” |  |
| dims | integer [2] | O | An array of integers with size of 2 that represents the image size (width, height). |  |
| ctype | string | O | A string that defines the type of colors e.g., “RGB” or “YUV”. |  |
| dtype | string |  | A string that defines the type of values e.g., “f32”, “f64”, “i8”. |  |
| } |  |  |  |
| If type is “model/arf.tuples” { |  |  |  |
| count | integer | O | An integer that defines the total number of tuples. |  |
| dtype | string | M | A string that defines the type of values e.g., “f32”, “f64”, “i8”. |  |
| itype | string | M | A string that defines the type of values e.g., “i32”, “i64”. |  |
| } |  |  |  |

Table 13: proposed extension of the Data component for the representation of data types. The added properties are highlighted in green.

Representation of avatar geometry and appearance

Similarly to the encodings as ARF Data objects of the new data types presented in the previous section, we propose to introduce in the ARF description new properties for describing the geometry and appearance of an avatar as ARF Data objects with dedicated MIME types. For clarity, these new properties are described below as objects. However, it should be understood that these objects are meant to be encoded, similarly to the new data types in the previous section, as ARF Data items with additional object-specific properties that are listed below. This encoding of new object properties as ARF Data items is illustrated in the JSON ARF example description provided in section 0 of this contribution.

At the top of the object hierarchy describing the geometry and appearance of an avatar is the Geometry object. This object is encoded as an ARF Data component with type set to “model/arf.geometry+json” and appended to the properties of the ARF Mesh, as illustrated on Table 14Table 14: proposed extension of the Mesh component for the representation of avatar geometry and texture data. The added geometry property is highlighted in green.

Similarly to the encodings as ARF Data objects of the new data types presented in the previous section, we propose to introduce in the ARF Data new objects for describing the geometry and appearance of an avatar as ARF Data objects with dedicated MIME types.

For clarity, these new properties are described below as objects. However, it should be understood that these objects are meant to be encoded, similarly to the new data types in the previous section, as ARF Data items with additional object-specific properties that are listed below. This encoding of new object properties as ARF Data items is illustrated in the JSON ARF example description provided in section 0 of this contribution.

At the top of the object hierarchy describing the geometry and appearance of an avatar is the Geometry object. This object is encoded as an ARF Data component with type set to “model/arf.geometry+json” and appended to the properties of the ARF Mesh.

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| name | string | M | The name of the mesh. |
| id | number | M | The identifier of the mesh. |
| path | string | M | A string that represents a hierarchical path that can be used to associate the mesh with a node in the external scene graph. |
| data | array(Data) | M | A reference into a data item that contains the mesh data. |

Table 14: proposed extension of the Mesh component for the representation of avatar geometry and texture data. The added geometry property is highlighted in green.

Table 15 defines the semantics of the Geometry object. The vertices and normals properties reference tensors holding respectively the 3D coordinates of the mesh vertices and the vertex normals. The uvs property is a list of tensors holding the UV coordinates of the mesh vertices in the set of UV maps associated with the textures that model the avatar appearance.

| Name | Type | Description | Required |
| --- | --- | --- | --- |
| vertices | integer | Index of the corresponding item in the “data” collection. The referenced item must be a float tensor [number of vertices, 3]. | ü Yes |
| normals | integer | Index of the corresponding item in the “data” collection. The referenced item must be a float tensor [number of vertices, 3]. | No |
| uvs | integer [1-*] | List of indexes to corresponding items in the “data” collection. The ith referenced item must be a float tensor [number of uvs(i), 2]. | No |
| faces | Faces | A Faces object. | No |
| textures | Texture [1-*] | List of Texture. | No |

Table 15: specification of the Geometry object

The faces property in Table 15 references a Faces object that defines the vertex indices for each face of the mesh, as well as the corresponding normal vectors and UV coordinates. The Faces object is described in Table 16. It is encoded in the ARF description as a Data object with type set to “model/arf.faces+json”.

| Name | Type | Description | Required |
| --- | --- | --- | --- |
| vertices | integer | References an item in the “data” collection. The referenced item must be a float tensor [number of faces, 3-256] or a list of tuples, where tuples must have between 3 and 256 values. | No |
| normals | integer | References an item in the “data” collection. The referenced item must be a float tensor [number of faces, 3-256] or a list of tuples, where tuples must have between 3 and 256 values. | No |
| uvs | integer [1-*] | List of references of items in the “data” collection. Each referenced item must be a float tensor [number of faces, 3-256] or a list of tuples, where tuples must have between 3 and 256 values. | No |

Table 16: specification of the Faces object

The textures property in Table 15 references a list of Texture objects that define the textures mapped to the avatar mesh geometry. The specification of the Texture object is shown on Table 17. The Texture object is encoded in the ARF description as a Data object with type set to “model/arf.texture+json”.

| Name | Type | Description | Required |
| --- | --- | --- | --- |
| type | string | Texture type, such as “albedo”, “specular”, etc. | ü Yes |
| uvs | integer | References one of the uvs item in the “Data” collection. | ü Yes |
| image | integer | References an item in the “data” collection. The referenced item must be an image. | ü Yes |

Table 17: specification of the Texture object

Mapping to MIME types

The new ARF types introduced above for the encoding of the avatar geometry and appearance will be mapped to subtypes of a new model/arf MIME type, as described in Table 18.

| Object type | MIME type |
| --- | --- |
| Geometry | model/arf.geometry+json |
| Faces | model/arf.faces+json |
| Texture | model/arf.texture+json |
| dense | model/arf.dense |
| sparse | model/arf.sparse |
| image | model/arf.image |
| tuples | model/arf.tuples |

Table 18: mapping of object types introduced in this contribution to future MIME types.

- Example JSON description

An example ARF avatar description building on the new geometry and texture Data items proposed in this contribution is provided below. For conciseness, the parts of the ARF description that do not pertain to the elements introduced in the contribution have been omitted.

{ […] “components”: { “skeletons”: [], “skins”: [], “meshes”: [ { “name”: “full_body”, “id”: 42, “path”: “”, “data”: [5] } ], “nodes”: […], “blendshapes”: […] }, “data”: [ { “name”: “my_avatar_mesh_vertices”, “type”: “model/arf.dense”, “uri”: “my_avatar_mesh_vertices.bin”, “offset”: 0, “byteLength”: 180000, “dims”: [15000, 3], “dtype”: “f32” }, { “name”: “my_avatar_vertex_normals”, “type”: “model/arf.dense”, “uri”: “my_avatar_vertex_normals.bin”, “offset”: 0, “byteLength”: 180000, “dims”: [15000, 3], “dtype”: “f32” }, { “name”: “my_avatar_uvs0”, “type”: “model/arf.dense”, “uri”: “my_avatar_uvs0.bin”, “offset”: 0, “byteLength”: 120000, “dims”: [15000, 2], “dtype”: “f32” }, { “name”: “my_avatar_faces”, “type”: “model/arf.dense”, “uri”: “my_avatar_faces.bin”, “offset”: 0, “byteLength”: 252000, “dims”: [21000, 3], “dtype”: “u32” }, { “name”: “my_avatar_albedo”, “type”: “model/arf.image”, “uri”: “my_avatar_albedo.png”, “offset”: 0, “byteLength”: 414720, “mimeType”: “image/png”, “dims”: [720, 576], “ctype”: “RGB”, “dtype”: “i8” }, { “name”: “my_avatar_geometry_description”, “type”: “model/arf.geometry+json”, “uri”: “my_avatar_geometry_description.json”, “offset”: 0, “byteLength”: 680, “vertices”: 0, “normals”: 1, “uvs”: 2, “faces”: 6, “textures”: [7] }, { “name”: “my_avatar_faces_description”, “type”: “model/arf.faces+json”, “uri”: “my_avatar_faces_description.json”, “offset”: 0, “byteLength”: 148, “vertices”: 3 }, { “name”: “my_avatar_texture_description”, “type”: “model/arf.texture+json”, “uri”: “my_avatar_texture_description.json”, “offset”: 0, “byteLength”: 312, “type”: “albedo”, “uvs”: 2, “image”: 4 } ] }

The listing starts with the ”components” component of the ARF description. This component holds a “meshes” array that is composed of a single Mesh item. The “data” item of this single Mesh item contains a single ARF Data item that is item 5 in the “data” array, i.e., the Data item with type set to “model/arf.geometry+json” that describes the avatar geometry and texture properties.

This geometry Data item references Data items 6 and 7 that correspond to the Faces and Texture objects introduced in the contribution. The geometry, Faces and Texture Data items reference dense tensors encoded as Data items with indices 0 through 3 that represent, respectively, the coordinates of the mesh vertices in the avatar geometry, the components of the vertex normals, the UV coordinates of the mesh vertices in the avatar texture image, and the indices of the vertices that define each face of the mesh. The Texture Data item references the albedo texture image encoded as the Data item with index 4 in the ”data” array.

5. Conclusion

It is proposed to add the descriptive elements of avatar geometry and appearance proposed in section 3 of this contribution to the Working Draft of ISO/IEC 23090-39.

6. References

[1] N01398, “Avatar Representation Format”, MPEG#148, Kemer, Turkey, November 2024.

[2] N01379, “EE Description of Avatar representation formats”, MPEG#148, Kemer, Turkey, November 2024.

[3] N01370, “Procedures and Tests Formats for Avatars Representation Formats”, MPEG#148, Kemer, Turkey, November 2024.

## 1.9 Extracted from TuC

## 1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case  can be with live content while another in the on-demand content.

## 1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria #1 | Description | The technical solution should minimize/optimize … |

## 1.12 Timeline

## MPEG #150: Document solutions and evaluation results and recommend changes to the CD text.

## 1.13 References

[1] WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] WG03 N0XXXX, “WD of ISO/IEC 23090-38 Avatar representation formats”, Kemer, Turkey, November 2024.

[4] WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.

Meeting #150

# m72258 [Avatar][EE2] ARF Sparse Data Type

# This proposal has been accepted in principle (minoring some small revision) to be added to the output document N1510 Potential improvement of ISO/IEC 23090-39 Avatar representation formats with the following comment. The current version is described below.

# 1 Introduction

# In the MPEG#148 meeting, a new part 39 of the ISO/IEC 23090 standard on Coded Representation of Immersive Media was initiated in output document N01398 [1], targeting the definition of an Avatar Representation Format (ARF). In this scope, three Exploratory Experiments (EE) [2] were launched to investigate potential improvements to the initial version of the standard Working Draft (WD).

# This contribution addresses EE2 on “Integrating on Geometry Data components for Avatar Data”. Within this EE, it was decided in MPEG meeting #149 in Geneva to add an ARF Data type for dense tensors. This allows the encoding in ARF of data arrays needed to represent elements of an avatar animation model. Examples of such elements are skinning weights and inverse bind matrices.

# This contribution proposes to enrich the ARF specification with a new Data type for sparse tensors and a memory-efficient representation of this Data type, in order to reduce the volume of data needed to encode the description of the avatar.

# 2 Problem statement

# For high-resolution avatar models, some of the data arrays in the avatar representation can be quite bulky. For instance, the array encoding the skinning weights contains scalar weights that bind every vertex of the avatar mesh to a subset of joints in the avatar skeleton. The dimension of this subset is often fixed to 4. This is the case, e.g., in the glTF specification [3]. The High-Resolution version of the MORGAN reference avatar model of MPEG-I SD [4] has 53,695 vertices. Thus, for this model, assuming 4 joint weights per vertex, the skinning weights array holds more than 200K floating point values.

# In practice, it is observed that while some vertices do require binding to as many as 4 joints of the animation skeleton (see vertex A on Figure 1), a large proportion of vertices in an avatar skinning model need not be bound to more than 2 joints (vertex B on Figure 1) hence have at most 2 non-zero skinning weights. As a result, the skinning weights array often contains a large number of zero entries.

# Figure 1: two example vertex to joints bindings in an avatar skinning model.
Vertex A is bound to 4 joints, vertex B to only 2 joints.

# 3 Proposed Solution

# To reduce the volume of data needed to store or transmit an ARF avatar model, we propose to add a new Data type for sparse tensors in the ARF specification.

# In line with the specification of the dense data type in annex E.1 of [1], we propose to define the syntax of a sparse data type as described on Table 1. The sparse data type represents in general a multi-dimensional tensor. Its dimensions are defined by the “dims” property and its number of dimensions by the “num_of_dims” property.

# The content of the sparse tensor is encoded as a list of “valueCount” entries representing the non-zero values in the tensor. Each entry consists of an (index, value) pair. “index” is a scalar that specifies the position of the entry in a flattened version of the multi-dimensional sparse tensor. “value” refers to the value of the tensor entry at this position. The flattening of the data tensor is performed in row-major order.

# For illustration, assume the sparse tensor has dimensions [4,7,2]. For this tensor, num_of_dims=3 and dims=[4,7,2]. Flattening the sparse tensor yields a 1D array of 4x7x2=56 elements. Since flattening is performed in row-major order, assuming zero-based numbering, the first 5 elements of the flattened tensor have indexes (0,0,0), (0,0,1), (0,1,0), (0,1,1), (0,2,0) in the original 3-dimensional tensor.

# The ”itype” property specifies, as a glTF integer componentType encoding, the data type of the “index” value of the (index, value) pairs describing the non-zero entries of the sparse tensor. Similarly, the “dtype” property specifies as a glTF componentType encoding, the data type of the “value” value of the (index, value) pairs.

| # Name | # Type | # Use | # Description |
| --- | --- | --- | --- |
| # num_of_dims | # Int32 | # M | # Provides the number of dimensions for the data tensor. |
| # dims | # int32 [num_of_dims] | # M | # A list of integers that define the dimension sizes of the tensor e.g., dimension of [4, 7, 2] refers to a tensor with 4 x 7 x 2 = 56 values, where the first element of the tensor has dimension 4, the second element has dimension 7 and the last element has dimension 2. |
| # valueCount | # number | # M | # Number of non-zero entries in the sparse tensor. The tensor entries are encoded as a list of (index, value) pairs where indexes are scalars representing the position of each entry in a 1D-flattened version of the tensor. |
| # itype | # enum | # M | # A number that describes the data type of the index element of the (index, value) pairs representing non-zero tensor entries. The allowed data types correspond to the integer glTF 2.0 component types, as specified in glTF 2.0 [3] clause 5.1.3. |
| # dtype | # enum | # M | # A number that describes the data type of the value element of the (index, value) pair representing entries. The allowed data types correspond to the glTF 2.0 component types, as specified in glTF 2.0 [3] clause 5.1.3. |

# Table 1: definition of the sparse Data type

# To comply with the definition of Data items in ARF, the new sparse tensor Data type will need to be registered as a MIME type, for instance as “application/mpeg.arf.sparse”.

# MIME Type Registration

# The MIME type for the sparse tensor data as defined in this document shall be “application/mpeg.arf.sparse”.

# Registration Form

# Type name:					application

# Subtype name:					mpeg.arf.sparse

# Required parameters:

# Optional parameters:

# Encoding considerations:

# Security considerations:

# Interoperability considerations:

# Published specification:			ISO/IEC 23090-39

# Applications that use this media type:		Avatar Communications

# Fragment identifier considerations:

# Additional information:

# Deprecated alias names for this type:

# Magic number(s):

# File extension(s):

# Macintosh file type code(s):

# Person & email address to contact for further information:

# Intended usage:

# (One of COMMON, LIMITED USE, or OBSOLETE.)

# Restrictions on usage:

# (Any restrictions on where the media type can be used go here.)

# Author:

# Change controller:

# Provisional registration? (standards tree only):

# (Any other information that the author deems interesting may be

# added below this line.)

# 4 Recommendation

# It is proposed to add a sparse tensor Data type, as proposed in this contribution, to the specification of the Avatar Representation Format [1] on a new Annex F.

EE3: Other Animation Sample Formats for Animation Stream (ongoing)

## 1.1 Introduction

This Exploration Experiment (EE) aims to develop structures for animation streams for avatars, focusing on both facial and body animations. The experiment will explore animation sample to be streamed and transmitted for various animation data types, including blendshapes, facial landmarks, animation controllers, and joint transforms, with the goal of establishing a set of standardized approaches that enable efficient and flexible encoding and decoding of avatar animation data across platforms.

## 1.2 Problem statement

The following statement for Animation samples are identified:

- Standardization of Avatar Animation Data Formats: Propose a standardized format for avatar animation data that is interoperable across platforms and supports a wide range of animation components. The format should be extensible to accommodate new animation parameters as they emerge, and must support compatibility with existing frameworks, such as Mediapipe, ARKit, OpenXR, etc.

## 1.3 Use cases relevant for the EE

The work of the EE is based on the requirements defined in N00359 and use cases identified in m64008 .

The requirements defined in N00359 are currently organized into five different categories including: Avatar Representation, Coding format, Transport/Synchronization/Carriage, Storage and Privacy and Security requirements.

The requirements related to the “Avatar Representation” are summarized as follows:

- Interoperability: allow converting to and from other avatar representation.

- 3D Scenes: to allow integration into scene description solution.

- Attributes: requirements applicable to the features supported by the avatar, such as, CGI features as textures, geometries, skeletons, accessories, semantic definitions.

The requirements related to the “Coding Format” are summarized as follows:

- General: requirements applicable to the whole format and its use

- Geometry: requirements applicable to the geometry of the base avatar

- Interaction: requirements that relate to the interaction between Avatars and between Avatars and other objects in the scene

- Animation and control: requirements on the facial and body animation

Transport/Synchronization/Carriage: requirements related to the transport of the base avatar model and the associated animation streams

- Storage: requirements that pertain to the storage of the base avatar model

- Security and privacy: requirements on the protection of the avatar in different scenarios

The requirements and use cases explicit mention the interoperability concept to allow converting to and from other representation and the representation of all components of an avatar including but not exclusive to geometries, textures, skeletons, accessories and semantics.

## 1.4 Related (WG2) and Extracted (new) Requirements

## 1.5 Relation to other activities (EE, requirements, etc…)

## 1.6 Mandates

## 1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | L |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | P |
| Interdigital | Joao Regateiro | joao.regateiro@interdigital.com | P |

(P = proponent, L = leader)

## 1.8 Information about proposed technologies

The following contributions on XXXX have been submitted:

Meeting #148

m70326 [SD][Avatar] MPEG Avatar Animation Stream

m70195 Avatar facial animation stream format and coding scheme

Meeting #149

m71326 [SD] Support for eye animation in the Avatar Representation Format

This proposal has been accepted for inclusion in the EE3 and is documented below.

# 1	Introduction

In this contribution, we provide an overview of the existing eye and gaze tracking functionality in popular XR frameworks and APIs.

# 2	Overview of Eye and Gaze Tracking

Eye and gaze tracking technologies play a crucial role in modern augmented reality (AR) and virtual reality (VR) systems, enabling enhanced user interaction and social presence. Several key extensions and frameworks provide tools for developers to incorporate these functionalities into their applications. This paper delves into the XR_EXT_eye_gaze_interaction, XR_FB_eye_tracking_social, XR_ANDROID_avatar_eyes, and ARKit frameworks, focusing on their sample syntax and semantics to illustrate their implementation.

## 2.1	XR_EXT_eye_gaze_interaction

The XR_EXT_eye_gaze_interaction extension in OpenXR facilitates the integration of eye-gaze data into applications, allowing developers to create natural and intuitive interaction mechanisms. By tracking the user's eye movement, applications can infer the user's focus point and enable gaze-based actions, such as selection and cursor positioning.

### 2.1.1	Sample Syntax and Semantics

The core structure of this extension includes:

typedef struct XrEyeGazeSampleTimeEXT {

XrStructureType type;    // Specifies the type of this structure.

const void* next;        // Pointer to the next structure in a chain.

XrTime time;             // Timestamp of the eye-gaze sample.

} XrEyeGazeSampleTimeEXT;

- type: Identifies the structure type (XR_TYPE_EYE_GAZE_SAMPLE_TIME_EXT).

- next: A pointer for extension-specific structures or NULL.

- time: Represents the precise time of the eye gaze sample.

To retrieve gaze data, the function xrGetEyeGazeSampleTimeEXT is used. This function queries the latest available eye gaze sample time and returns it in the specified structure. Developers use this data to synchronize eye movements with system events or other inputs.

## 2.2	XR_FB_eye_tracking_social

The XR_FB_eye_tracking_social extension, created by Meta, is specifically tailored for social VR experiences. It provides detailed data on each eye's gaze, enabling lifelike avatar animations and realistic eye movements.

### 2.2.1	Sample Syntax and Semantics

The key structure for this extension is:

typedef struct XrEyeGazesFB {

XrStructureType type;                // Type of this structure.

void* next;                          // Pointer to the next structure in a chain.

XrEyeGazeFB gaze[XR_EYE_POSITION_COUNT_FB]; // Array holding gaze data for each eye.

XrTime time;                         // Timestamp for the gaze data.

} XrEyeGazesFB;

- type: Indicates the structure type (XR_TYPE_EYE_GAZES_FB).

- next: Pointer for extension-specific structures or NULL.

- gaze: An array of XrEyeGazeFB structures, holding gaze direction and other data for each eye.

- time: A timestamp indicating when the gaze data was captured.

The XrEyeGazeFB structure includes the gaze direction and position for each eye, which are crucial for animating avatars or enabling gaze-based interactions in social VR.

## 2.3	XR_ANDROID_avatar_eyes

The XR_ANDROID_avatar_eyes extension supports avatar eye representation by providing information on eye pose, tracking state, and non-tracked states like blinking. It is designed to make avatars more lifelike in AR/VR applications.

### 2.3.1	Sample Syntax and Semantics

The key structures include:

typedef struct XrEyeStateANDROID {

XrBool32 isTracked;                  // Indicates if the eye is being tracked.

XrBool32 isBlinking;                 // Indicates if the eye is blinking.

} XrEyeStateANDROID;

typedef struct XrEyePoseANDROID {

XrPosef pose;                        // Pose of the eye in the base space.

XrEyeStateANDROID state;             // State of the eye (tracked, blinking, etc.).

} XrEyePoseANDROID;

- isTracked: A boolean indicating if the eye tracking is active.

- isBlinking: A boolean indicating if the eye is blinking.

- pose: Contains the position and orientation of the eye within the base space.

Applications use these structures to query eye state and pose data, enabling realistic animation of avatar eyes, including blinking and gaze tracking.

## 2.4	ARKit Eye Tracking

Apple’s ARKit framework offers robust facial and eye tracking capabilities, enabling developers to create immersive applications that respond to real-time facial expressions and gaze direction. This framework is deeply integrated with Apple’s hardware, ensuring high fidelity.

### Sample Syntax and Semantics

ARKit provides high-level APIs to access eye tracking data. A key component is the ARFaceAnchor class, which includes blend shape coefficients and eye tracking properties.

if let faceAnchor = anchor as? ARFaceAnchor {

let leftEyeTransform = faceAnchor.leftEyeTransform

let rightEyeTransform = faceAnchor.rightEyeTransform

let lookAtPoint = calculateLookAtPoint(leftEye: leftEyeTransform, rightEye: rightEyeTransform)

}

- leftEyeTransform and rightEyeTransform: Matrices representing the position and orientation of each eye.

- lookAtPoint: A calculated point representing the user’s gaze direction based on eye transforms.

ARKit also supports blend shapes for eye openness, allowing developers to track whether the eyes are open or closed. This data can be used for avatar animation or gaze-based interaction.

# 3	Summary and Proposal

By investigating the existing frameworks for eye and gaze tracking, we can identify the key features that the eye animation sample should contain. All these formats share a common pose for each eye. Additionally, a state of the eye, e.g. blinking, may also be provided.

We propose to work as part of the EE#3 core experiment on defining a sample format for eye animation based on the existing tracking frameworks described above.

# 4	References

[1]	Khronos, OpenXR 1.1 Specification, https://registry.khronos.org/OpenXR/specs/1.1/html/xrspec.html

[2]	Android XR, XR_ANDROID_avatar_eyes OpenXR extension, XR_ANDROID_avatar_eyes OpenXR extension  |  Android XR  |  Android Developers

[3] 	Apple ARKit, Tracking and Visualizing Faces | Apple Developer Documentation

Meeting #150

m72260 [Avatar][EE3] Gaze and Eye Animation in Avatar Representation Format

This proposal has been accepted for inclusion in the EE document to be further studied as part of the EE discussion, with the following comment: we expect that we need some sort of eye animation, this is also part of the study.

1 Introduction

This contribution presents the encoding of avatar gaze and eye animations in MPEG Avatar Representation Format (ARF). Thanks to received parameters like rotations or a position to look at, the encoded animations allow the update of the avatar gaze and eyes with no unwanted artifacts like clipping.

2 Animation streaming format

The gaze and eye animation parameters are expected to respect a specific format, like XR_FB_eye_tracking_social. These parameters can be low-level like joint rotations or high-level like angles or a position to look at. In all cases, the application needs to know which animation stream format is supported as for face and hands animations. As a result, a new “eyeAnimations” property in the “supportedAnimation” property of the “preamble” root property:

Table 1 - Properties of “supportedAnimation” in “preamble” root property

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| faceAnimations | array(uri) | O | Lists the supported face animation types. Each item in the array is a string representing a supported face animation type. Each identifier should be formatted as a URN that includes an identifier of the framework, followed by an identifier of the facial blendshape set. |
| bodyAnimations | array(uri) | O | Lists the supported body animation types. Each item in the array is a string representing a supported body animation type. Each identifier should be formatted as a URN that includes an identifier of the body animation/tracking framework, followed by an identifier of the body joint set. |
| handAnimations | array(uri) | O | Lists the supported hand animation types. Each item in the array is a string representing a supported hand animation type. Each identifier should be formatted as a URN that includes an identifier of the body animation/tracking framework, followed by an identifier of the body joint set. |
| landmarkAnimations | array(uri) | O | Lists the supported landmark animation types. Each item in the array is a string representing a supported landmark animation type. Each identifier should be formatted as a URN that includes an identifier of the landmark animation/tracking framework, followed by an identifier of the landmark set. |
| eyeAnimations | array(uri) | O | Lists the supported eye and gaze animation types. Each item in the array is a string representing a supported eye or gaze animation type. Each identifier should be formatted as a URN that includes an identifier of the eye or gaze animation/tracking framework. |
| proprietaryAnimations | array( ProprietaryAnimation) | O | A list of proprietary animation descriptions, which may be used to animate assets in the ARF container. |

Eye sets in ARF

The encoding of eye sets is required to bring the parameters which enable the different eye and gaze features. As for other features like blendshapes and landmarks, the eye sets are signaled in the “components” root property:

Table 2 – ARF structure with new “eyeSets” component

Table 3 - Properties of “components” root property

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| skeletons | array(Skeleton) | O | A list of skeletons used to describe the avatar skeletal asset. |
| skins | array(Skin) | O | A list of assets that are stored in this ARF container. |
| meshes | array(Mesh) | M | A list of geometries used to describe the avatar asset. |
| nodes | array(Node) | O | A list of nodes used to organize, merge and describe and transform the avatar components. |
| blendshapeSets | array(BlendshapeSet) | O | A list of blend shape sets used to describe the blend shape-based animations. |
| landmarkSets | array(LandmarkSet) | O | A list of landmark sets used to describe landmark-based animation. |
| eyeSets | array(EyeSet) | O | A list of eye sets used to describe eyes and gaze animation. |

Each item of “eyeSets” contains eye and gaze properties (more details below) and is linked to an animation stream listed in “eyeAnimations” of “supportedAnimation” in “preamble” root property.

Similarly to other components, a LOD (Level Of Details) in the “lods” list of an Asset in the “assets” list of the “structure” root property signals the eye sets being used by the LOD:

Table 4 - Properties of a LOD with new “eyeSets” references

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| name | string | M | The name of the LOD. |
| skins | array(number) | O | List of references to all skins that are part of this asset. |
| meshes | array(number) | M | List of non-skinned meshes that are part of this asset. |
| skeletons | array(number) | O | List of references to skeletons in the ARF container. |
| blendshapeSets | array(number) | O | List of references to blendshape sets in the ARF container. |
| eyeSets | array(number) | O | List of references to eye sets in the ARF container. |

The “eyeSets” list contains references (e.g., values of “id” property in EyeSet) to eye sets in the “eyeSets” list in “components” root property.

Eye set in ARF

Each item of the “eyeSets” list in “components” defines the properties of an eye set:

Table 5 - Properties of an EyeSet

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| name | string | M | The name of the eye set. |
| id | integer | M | A unique identifier of the eye set. |
| animationInfo | array(AnimationLink) | M | Establishes a link to the supported animation and tracking frameworks that this eye set can be used with. |
| eyes | array(Eye) | M | The list of eyes. |
| gaze | Gaze | O | Eye gaze definition. |

The “name” property describes the eye set.

The “id” property uniquely identifies the eye set and is used to reference it (like in a LOD).

The “animationInfo” indicates the animation streaming format the eye set can support (like XR_FB_eye_tracking_social).

The “eyes” lists properties for each eye. These properties enable features like automatically correcting the eye when the shape of the avatar face changes.

The “gaze” property defines properties shared by all eyes, like how to animate the gaze/angles of all eyes given up to three angles.

Eye in ARF

Each item of the “eyes” list in an item of “eyeSets” in “components” defines the property of an eye:

Table 6 - Properties of Eye

| Name | Type |  | Use |  | Description |
| --- | --- | --- | --- | --- | --- |
| name | string |  | M |  | The name of the eye. |
| id | integer |  | M |  | A unique identifier of the eye. |
| front | number[3] |  | M |  | A unit vector defining the eye front direction. |
| correctionType | string |  | M |  | Eye correction method, which can be “none”, “animation”, “controller” or “simple”. |
| if (correctionType == “simple”) { |  |  |  |  |  |
| mesh | number |  | M |  | References a Mesh item |
| translationIndices | number |  | M |  | References a Data item with a list of vertex indices for computing the corrective translation |
| scaleIndices | number |  | M |  | References a Data item with a list of vertex indices for computing the corrective scale |
| else if (correctionType == “controller”) { |  |  |  |  |  |
| controllerBlendshapes | array(ControllerBlendshape) | M |  | List mappings between blendshapes and controllers |  |
| } |  |  |  |  |  |
| eyelidCorrection | string |  | M |  | Eyelid correction method, can be “none” or “front”. |
| if (eyelidCorrection == “front” ¦¦ eyelidCorrection == “normals”) { |  |  |  |  |  |
| eyelidMesh | number |  | M |  | References a Mesh item |
| eyelidIndices | number |  | M |  | References a Data item with a list of eyelid vertex indices |
| alpha | number |  | M |  | The amount of space to ensure between the eye and the eyelid. |
| } |  |  |  |  |  |

The “name” property defines the name of the avatar eye.

The “id” property defines a unique integer for identifying the avatar eye.

Eye correction. The “correctionType” property, if different from “none”, enables the automatic correction of the eye when the shape of the avatar changes.

If “transformType” is “simple”, then the correction presented in Section 5.1 is used. In this case the following properties are mandatory:

- “mesh” references the mesh with anchor vertices (denoted vis and vit in Section 5.1)

- “translationIndices” contains the list of vertex indices to use for computing the corrective translation (denoted vit in Section 5.1).

- “scaleIndices” contains the list of vertex indices used for computing the corrective scale (denoted vis in Section 5.1).

If “transformType” is “controller”, then the correction presented in Section 5.2 is used. In this case the “controllerBlendshapes” property is mandatory and contains mappings between blendshapes and controllers:

Table 7 - Properties of ControllerBlendshape

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| blendshapeSet | number | M | References a blendshape set |
| blendshapeIndex | number | M | The index of a blendshape in the blendshape set |
| controllerSet | number | M | References a controller set |
| controllerIndex | string | M | The index of a controller in the controller set |

If a blendshape is not listed in “controllerBlendshapes”, then no controller for eye correction is used when the weight of this blendshape changes.

Eyelid correction. The “eyelidCorrection” property, if different from “none”, enables the automatic correction of the eyelid when the shape of the avatar changes.

If “eyelidCorrection” is “front”, then the method in Section 6.1 is used. If “eyelidCorrection” is “normals”, then the method in Section 6.2 is used. In both cases, the following properties are mandatory:

- “eyelidMesh” references the mesh with eyelid.

- “eyelidIndices” contains the list of vertex indices of the eyelid.

- “alpha” is a number that defines the minimal space between the eye and the eyelid (e.g., the α variable in each methods).

Gaze in ARF

The “gaze” property defines properties shared by all eyes, like how to animate the gaze/angles of all eyes given up to three angles. In allows an application to animate the gaze (possibly in a complex fashion) only using a horizontal, vertical and optionally a roll angle.

Table 8 - The Gaze properties

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| style | string | M | The gaze style, which defines how eyes move. |
| speed | number | M | Defines the speed at which the eyes move. |
| horizontal | GazeAnimation | M | Gaze animation properties for the horizontal axis. |
| vertical | GazeAnimation | M | Gaze animation properties for the vertical axis. |
| roll | GazeAnimation | O | Gaze animation properties for the roll angle. |

The “style” property defines rules or hints about the usage of the gaze information. For example, if “style” is “instantaneous”, then eyes are immediately rotated from the current state to a new one, with no delay. Another value for “style” could be “linear”, in which case eyes rotates linearly towards to the look at point, at a speed defined by the “speed” property.

The “speed” property defines how fast the eye moves from their current state to a new state given by new angles. The interpretation of “speed” may depend on the “style” property.

The “horizontal” property defines the animation for the yaw axis.

The “vertical” property defines the animation for the pitch axis.

The “roll” property defines the animation for the roll axis.

Gaze Animation in ARF

A GazeAnimation object defines how an eye can be animated along a given axis. An application can deduce from the values of this object an animation from a minimal angle. Any animation is possible, the application does not have to wonder which part of the avatar are actually animated: it only runs the animation given a gaze angle, and the avatar should change its gaze accordingly. For instance, if a GazeAnimation object is dedicated to the horizontal axis, then the minimum angle will let the avatar looks to the right and the maximum angle to the left.

Table 9 - The GazeAnimation property

| Name |  | Type | Use | Description |  |
| --- | --- | --- | --- | --- | --- |
| range |  | number[2] | M | Minimum and maximum angles of the rotation. |
| type |  | enum | M | Type of gaze animation (GA_JOINT, GA_BLENDSHAPE, GA_CONTROLLER) |
| if (type == GA_JOINT) { |  |  |  |
| joint | integer | M | The identifier of a joint. |
| } else if (type == GA_BLENDSHAPE) { |  |  |  |
| blendshapeSet | number | M | The identifier of a blendshape set. |
| blendshapeShape | number | M | The index of a shape in the blendshape set. |
| } else if (type == GA_CONTROLLER) { |  |  |  |
| controllerSet | integer | M | The identifier of a controller set. |
| controller | integer | M | The identifier of a controller in the controller set. |
| } |  |  |  |

The “range” property defines the minimum and maximum angles of the rotation.

The “type” property defines the type of animation.

If “type” is GA_JOINT, the “joint” property is mandatory and indicates the joint to rotate given the angle associated with this gaze animation. The rotation axis is deduced from the direction associated with this gaze animation and the value of the “front” property in the Eye property that contains this gaze animation.

If “type” is GA_BLENDSHAPE, then “blendshapeSet” and blendshapeShape” are mandatory and indicates the blendshape to update given the angle associated with this gaze animation. The weight of this blendshape is the value of the angle associated with this gaze animation.

If “type” is GA_CONTROLLER, then “controllerSet” and “controller” are mandatory and indicates the controller to update given the angle associated with this gaze animation. The weight of this controller is the value of the angle associated with this gaze animation.

Automatic eye correction

The eye encoding presented in the following section allows the signaling of methods for an automatic eye correction when the avatar changes.

Figure 1 illustrates the problem solved by such correction, where the avatar changes after the use of a blendshape. Note that the chosen blendshape is an extreme case to ease understanding; similar problems arise using expression blendshapes that changes the shape and location of eye sockets.

Figure 1 - Example of automatic eye correction

In Figure 1 (a), the head of the avatar is in a neutral state, without the use of dynamic features like blendshapes or skinning. The eyes are correctly located and scaled.

In Figure 1 (b), the head of the avatar is in a neutral state, except that a blendshape was applied to close the eyes. The eyes are correctly located and scaled.

In Figure 1 (c), the head of the avatar is no longer in a neutral state, with a blendshape being applied to change the shape of the face. The eyes are no longer correctly located and scaled.

In Figure 1 (d), the head of the avatar is in the same state as in Figure 1 (c), except that one of the proposed solutions was applied to correct the location and scale of the eyes. These solutions are presented in the following.

Two solutions are proposed: a simple one, based on the translation and scaling of the eye, and a more complex one, based on a corrective blendshape or controller. The first one is easier to implement but limited to eyeballs.

Simple correction

The main purpose of this solution is to correctly locate and scale an eye. It is based on the computation of a corrective scale factor cs and corrective translation ct for the eye. Every time the avatar head changes (for instance, using blendshapes or skinning), the corrective scale factor is first computed and used to scale the eye. Then, the corrective translation is computed and used to translate the eye.

This solution is the easiest to implement but may not cover all cases. Indeed, this solution assumes that the eye is close to a sphere (like an eyeball), and that it is inside a cavity (like the eye socket). As a result, it works for humanoid avatars, but not for non-humanoid avatars like the robot Wall-E (the main character in the Pixar movie).

Corrective scale. The computation of the corrective scale is based on two or more anchor vertices on the avatar. For instance, a vertex to the left of the eye socket and a vertex to the right of the eye socket can be chosen. These anchor vertices are identified by their index, denoted vis where 1≤i<m and m>0 is the number of anchor vertices.

In an initial step, the XYZ coordinates of the anchor vertices are saved in some variable sis. Then, the smallest sphere of center ps and radius rs which includes all anchor vertices sis is computed.

Every time the avatar head changes, the XYZ coordinates eis of the anchor vertices are computed. Then, the smallest sphere of center qs and radius ls which includes all anchor vertices eis is computed. The corrective scale is then the ratio between the current and initial enclosing spheres: cs=lsrs.

Figure 2 illustrates this process:

Figure 2 - Corrective scale.

Corrective translation. The computation of the corrective translation is based on one or more anchor vertices on the avatar. For instance, a vertex deep in the eye socket can be chosen. These anchor vertices are identified by their index, denoted vit where 1≤i<n and n>0 is the number of anchor vertices.

In an initial step, the XYZ coordinates of the anchor vertices are saved in some variable sit.

Every time the avatar head changes, the XYZ coordinates eit of the anchor vertices are computed. Then, the corrective translation is the average of the differences between the current and initial coordinates is computed: ct=1n1neit-sit.

Blendshape-based correction

The main purpose of this solution is to correctly locate and scale the eye according to the weight of a blendshape. Using this solution, every time a blendshape is applied with a weight w∈wmin,wmax, a controller is run with weight t=w-wminwmax-wmintmax-tmin+tmin, where controller weight is in range tmin,tmax. The controller must be designed such that, for each possible weight of the blendshape, the eye is correctly transformed.

For example, the shape avatar in Figure 1 (b) is changed using a blendshape weight to get the result in Figure 1 (c). From the initial step (blendshape weight wmin=0) to the last step (blendshape weight is wmax=1), the right eye socket is translated towards to the bottom and a bit to the left and is also reduced. In order to ensure that the right eye is correct whatever the weight w of the blendshape, a controller is created from weight tmin=0 to weight tmax=0, using translation and scale transforms on the right eye. A translation tr0=(0,0,0) and a first scale sc0=(1,1,1) are defined in the controller for  t=tmin  (they do not change the eye). A translation tr1 and scale sc1 are defined for  t=tmax to ensure that the eye is correct. Then, a linear interpolation is chosen in the controller so all transform and scale transforms are interpolated for tmin<t<tmax. Finally, the controller is applied with t=w every time the weight w of the blendshape changes.

Automatic eyelids correction

The eye encoding presented in the following section allows the signaling of methods for an automatic eyelids correction when the avatar changes.

Figure 3 illustrates the problem solved by such correction, where the avatar changes after the use of a blendshape. Note that the chosen blendshape is an extreme case to ease the understanding; similar problems arise using expression blendshapes that changes the shape and location of eye sockets.

Figure 3 - Example of automatic eyelid correction

In Figure 3 (a), the head of the avatar is in a neutral state, without the use of dynamic features like blendshapes or skinning.

In Figure 3 (b), the head of the avatar is in a neutral state, except that a blendshape was applied to close the eyes. The eyelid correctly hides the eyes.

In Figure 3 (c), the head of the avatar is no more in a neutral state, with a blendshape being applied to change the shape of the face, and another other to close the right eye. The eyes are corrected using a method presented in Section 5 and are correctly located and scaled. However, the right eyelid does not cover the eye.

In Figure 3 (d), the head of the avatar is in the same state as in Figure 3 (c), except that one of the proposed solutions was applied to correct the eyelids. These solutions are presented in the following.

Two solutions are proposed: both moves the vertices of the eyelids, the first one using a forward/front direction, and the second one the normals of the eye.

Correction using a front direction

The main purpose of this solution is to ensure that the eyelid is always in front of the eyes. It assumes that a front direction f is defined, and the eyelid vertices vi are identified.

In an initial step, the XYZ coordinates of the eyelid vertices vi are saved in some variable si.

Every time the avatar head changes, the XYZ coordinates ei of the eyelid vertices is the result of the avatar current transform applied on the initial coordinates si (in other words, coordinates ei are the eyelid vertices “as if” no eyelids correction was made before).

Then, the projection pi of each point ei on the eye following the front direction f is computed:

- If there is no eye point in front of ei, nothing is done.

- If there is a single point pi in front of ei, pi is used in the following processing.

- If there is more than one point in front of ei, the farthest one pi from the eyelid is used in the following processing.

If pi exists, then the eyelid vertex is moved a little in front of the eye, e.g. ei⟵pi+αf, where α>0 is a number and f is a unit vector along the front direction of the eyelid vertex. Figure 4 illustrates this process.

Figure 4 - Eyelids correction.

Correction using normal

The main purpose of this solution is to ensure that the eyelid is always on the contour of the eye and structure and shape of the eyelid is preserved.

In an initial step, the XYZ coordinates of the eyelid vertices vi are saved in some variable si.

Every time the avatar head changes, the XYZ coordinates ei of each vertex vi are computed as the result si of the avatar current transform applied on the initial coordinates (in other words, coordinates ei are the eyelid vertices “as if” no eyelids correction was made before).

The outward normal ni of each vertex vi is also computed, using the coordinates ei.

Then, the projection pi on the eye following the corresponding outward normal direction ni of each point ei is computed:

- If there is at least one point pi ahead of ei along the outward normal ni, i.e., pi is located towards the front of the face with respect to ei on ni, the farthest point pi from the eye is moved slightly in front of the eye, e.g. ei⟵pi+αui, where α>0 is a pre-determined number and ui is the normal vector on ni.

- If there is no eye point ahead of ei along the outward normal ni,, the intersection between faces of the eyelid and eye are computed. If there is at least one face fi which intersects the eye, the farthest one fi from the eye is moved slightly in front of the eye, such as the distance between the eye and the face is a predetermined number α>0.

These steps are repeated until no eyelid vertex and face are inside the eye.

Figure 5 illustrates this process.

Figure 5 - Eyelids correction using normal direction.

References

[1] ISO/IEC 23090-39:2025 “Information technology — Coded representation of immersive media — Part 39: Avatar Representation Format”.

m72275 [Avatar] Parametric Texture Animation in ARF Avatar Representation Format

This proposal has been accepted for inclusion into the revised CD with the following comment : We agree to add this to the revised CD with a note that animation stream or tracking framework still needs to be provided. The document needs to be updated to address the typos.

m72370 Landmark animation sample format

This proposal has been accepted for inclusion into the output document N1510 Potential improvement of ISO/IEC 23090-39 Avatar representation formats in clause 8.4. with the following comment: Needs to be checked with potential overlap in #836. Editor and authors work together. The current version of the content is described below.

1	Introduction

During the previous meeting, the ARF specification [1] was enhanced with support for Landmark sets. However, there is no corresponding landmark animation sample defined in clause 8.

In this contribution, we propose an animation sample format for landmarks.

2	Landmark Animation Sample Format

The landmark animation sample shall follow the format specified in the following table:

| landmark_animation_sample() { | Descriptor |
| --- | --- |
| timestamp | int(64) |
| landmark_set_id | int(16) |
| velocity_present | int(1) |
| confidence_present | int(1) |
| is_3D | int(1) |
| reserved | int(5) |
| num_landmarks |  |
| for (i=0;i<num_landmarks;i++) { |  |
| if (is_3D) { |  |
| coordinates[3] | float(32) |
| } else { |  |
| coordinates[2] | float(32) |
| } |  |
| if (velocity_present) { |  |
| velocity | float(32) |
| } |  |
| if (confidence_present) { |  |
| confidence | float(32) |
| } |  |
| } |  |
| } |  |

3	Proposal

We propose to add the content of section 2 to the ISO/IEC 23090-39 specification in clause 8.3.

m72382 [SD][AVATAR] Carriage of Avatar Animation Streams in ISOBMFF ARF Containers

This proposal has been accepted for inclusion into the output document N1510 | Potential improvement of ISO/IEC 23090-39 Avatar representation formats with the following comments: Author works with editor for integration in standard. The current version is documented below:

- Introduction

In the MPEG#137 meeting, the MPEG Systems workgroup (WG03) has agreed on creating a new specification (ISO/IEC 23090-39) for defining an MPEG Avatar Representation format. One of the aspects covered by this specification, and one of its objectives, is the definition of an avatar animation stream that enables delay-sensitive transport of timed animation data and can be integrated into the MPEG-I Scene Description architecture ‎[1].

In the latest committee draft (CD) of ISO/IEC 23090-39 [2], animation data are carried as a list of samples in a ZIP-based ARF container and sequence of samples in an ISOBMFF-based ARF container. However, the sample format varies from one sample to another based on the type of animation being carried in the sample. This approach makes it difficult for a player to parse the animation samples in a unified manner as the sample structure is animation-dependent and not clearly signalled or directly identifiable. To address this issue, a unified animation data bitstream format was proposed in [3] and [4] and is currently being investigated as part of the exploration experiments conducted by the group (EE1 and EE3) [5].

This contribution focuses on ISOBMFF-based ARF containers and proposes an approach to carry avatar animation data in a more unified and structured approach as avatar animation tracks in the ISOBMFF file.

- Background

In [2], avatar animation streams as stored as part of the ARF container by creating one or more metadata tracks carrying the avatar animation samples. The following requirements are described:

- 'meta' handler type shall be used in the HandlerBox of the MediaBox

- The sample entry format shall be 'urim'

- Independent animation samples shall be marked as sync samples

- The URI identifying the type of the metadata is 'urn:mpeg:avatar:animation'

The sample entry for the animation timed metadata track is defined as follows:

aligned(8) class AvatarAnimationSampleEntry() extends DataSampleEntry('anim') { unsigned int(3) precision; bits(5) reserved; unit(8)[256] animation_profile; float timescale; uint(8) avatar_id; uint(8) lod_id; }

The following semantics apply

- precision specifies the length in bytes of the correspondence values within each sample. The value of precision shall be greater than 0 and smaller or equal to 4.

- animation_profile is a character string with the name of the profile that generated stream conforms to.

- timescale is the number of ticks per second.

- control_precision_minus1 plus 1 specifies the size in bytes of the target avatar index in control AAUs. The value of this field shall be greater than 0 and smaller than or equal to 3.

- avatar_id is an integer identifying the avatar to animate.

- lod_id is an integer identifying the level-of-detail (LoD) of the avatar to animate.

In the avatar animation bitstream described in [x], each avatar animation unit (AAU) contains a header and a payload. An AAU header contains at least a field that indicates the unit type and a field that indicates the AAU payload. The contents of the payload depend on the type of the AAU.

Figure 1 - Illustration of the structure of an AAU.

Table 1 – Syntax of avatar_animation_unit()

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| avatar_animation_unit() |  |  |
| { |  |  |
| aau_header(); |  |  |
| aau_payload(); |  |  |
| ByteAlignement | 0-7 | uimsbf |
| } |  |  |

The syntax structure of the AAU header is as shown in Table 2, where:

- aau_unit_type indicates the type of the AAU. The possible values are described in Table 3.

- aau_unit_length indicates the size of the AAU payload in bytes.

Table 2 – Syntax of aau_header().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_header() |  |  |
| { |  |  |
| aau_unit_type; | 7 | uimsbf |
| aau_unit_length; | 32 | uimsbf |
| reserved | 1 | uimsbf |
| } |  |  |

Table 3 – Avatar Animation Unit type codes and corresponding payloads.

| aau_unit_type | Name of AAU type | Content of AAU payload |
| --- | --- | --- |
| 0 | AAU_CONFIG | aau_config_unit() |
| 1 | AAU_ANIMATION | aau_animation_unit() |
| 2 | AAU_JOINT | aau_joint_unit() |
| 3 | AAU_LANDMARK | aau_landmark_unit() |
| 4..10 | AAU_RSV_4 AAU_RSV_10 | Reserved AAU types. |
| 11..127 | AAU_UNSPEC_11 AAU_UNSPEC_127 | Unspecified AAU types. |

The aau_payload() is defined as shown in Table 4.

Table 4 – Syntax of aau_payload().

| Syntax | No. of bits | Mnemonic |
| --- | --- | --- |
| aau_payload() |  |  |
| { |  |  |
| aau_timestamp; | 32 | uimsbf |
| if (aau_unit_type == AAU_CONFIG) |  |  |
| aau_config_unit() |  |  |
| else if (aau_unit_type == AAU_ANIMATION) |  |  |
| aau_animation_unit() |  |  |
| else if (aau_unit_type == AAU_JOINT) |  |  |
| aau_joint_unit() |  |  |
| else if (aau_unit_type == AAU_LANDMARK) |  |  |
| aau_landmark_unit() |  |  |
| } |  |  |

The aau_payload() syntax construct contains the following syntax elements:

- aau_timestamp is the timestamp of the AAU in ticks. The timestamp in seconds can be calculated as timestamp/timescale, where timescale is signalled in the configuration AAU.

- Discussion

Based on the status of the CD of the specification, the carriage of avatar animation data is unclear and is not sufficiently defined. For example, there is no sample entry called DataSampleEntry defined in ISO/IEC 14496-12. And the sample entry 4CC requirement ('urim') doesn’t match the definition of AvatarAnimationSampleEntry. That requirement is not needed. The 4CC of the sample entry is sufficient for identifying the type of metadata and there is no need for the URI.

- Proposal

  - Avatar Animation Tracks

Avatar animation streams can be carried (stored) in ISOBMFF media containers, potentially along with other types of media such as audio and video, as avatar animation tracks. Avatar animation tracks are timed-metadata tracks whose samples carry avatar animation data. An avatar animation has a sample entry of type AvatarAnimationSampleEntry as defined below.

aligned(8) class AvatarAnimationConfigurationRecord {

unsigned int(3) unit_size_precision_bytes_minus1;

unsigned int(3) weight_precision;

bit(2) reserved = 0;

unsigned int((unit_size_precision_bytes_minus1 + 1)*8) config_unit_length;

bit(config_unit_length * 8) config_unit;

}

aligned(8) class AvatarAnimationConfigurationBox extends FullBox('avaC', version=0, flags=0) {

AvatarAnimationConfigurationRecord() ava_animation_config;

}

aligned(8) class AvatarAnimationSampleEntry() extends MetadataSampleEntry('ava1') {

AvatarAnimationConfigurationBox config;

}

The semantics for the fields defined in AvatarAnimationConfigurationBox are as follows:

unit_size_precision_minus1 indicates the length in bytes of the AAUnitLength field in an animation sample of the associated stream minus one. For example, a size of one byte is indicated with a value of 0. The value of this field shall be one of 0, 1, or 3 corresponding to a length encoded with 1, 2, or 4 bytes, respectively.

weight_precision is the length in bytes of the weight values within each sample. The value of precision shall be greater than 0 and smaller or equal to 4.

config_unit_length indicates the size of the configuration AAU carried in this AvatarAnimationConfigurationBox.

config_unit is an AAU of type AAU_CONFIG (i.e., a configuration avatar animation unit).

  - Avatar Animation Track Sample Format

The samples of an avatar animation track include avatar animation data. Each sample carries avatar animation data associated with a particular timestamp in the media (presentation) timeline. For example, a sample may contain one or more AAUs which belong to the same presentation time. The format of each avatar animation sample of the track is defined as follow.

aligned(8) class AvatarAnimationSample {

// sample_size size of sample from SampleSizeBox

for (int i=0; i < sample_size; ) {

unsigned int((AvatarAnimationConfigurationRecord.unit_size_precision_bytes_minus1 + 1)*8) AAUnitLength;

bit(AAUnitLength * 8) AAUnit;

i += (AvatarAnimationConfigurationRecord.unit_size_precision_bytes_minus1 + 1) + AAUnitLength;

}

}

The semantics of the fields defined in AvatarAnimationSample are as follows:

AAUnitLength is the size of the AAU measured in bytes. The length field includes the size of both the AAU header and the AAU payload but does not include the length field itself.

AAUnit contains a single AAU as defined in [5], where the payload is based on the sample formats defined in clause 8.

An avatar animation sample may be designated as a sync sample if all the AAUs in the sample are keyframe AAUs. An avatar animation sync sample shall satisfy all the following conditions:

- It shall be possible to independently process the sample.

None of the samples that come after the sync sample have any processing dependency on any sample prior to the sync sample.

All samples that come after the sync sample can be successfully processed.

  - Association of Avatar Models with Animation Tracks

As described in [2], an ARF document is stored as an item in ISOBMFF containers. A MetaBox in an ISOBMFF media container may signal one or more ARF document items. To associate each avatar with the corresponding avatar animation track(s), an avatar’s ARF item in the container shall be grouped with the animation tracks using an entity group signaled by an AvatarAnimationGroupBox which extends EntityToGroupBox, as defined in ISO/IEC 14496-12, and defined as follows.

aligned(8) class AvatarAnimationGroupBox()
    extends EntityToGroupBox('avag', version=0, flags) {

unsigned int(8) avatar_id;
}

The semantics of the fields of AvatarAnimationGroupBox are as follows:

avatar_id is the unique identifier for the avatar associated with this entity group.

An AvatarAnimationGroupBox is an EntityToGroupBox with grouping_type set to the 4CC 'avag', which identifies the type of grouping in the entity group as that for grouping an avatar with its animation tracks. The EntityToGroupBox shall contain at most one item identifier (item_ID), corresponding to the item carrying the ARF document, and one or more track identifiers (track_IDs) for the avatar animation tracks associated with the avatar represented by the ARF document.

There shall be a corresponding AvatarAnimationGroupBox in the GroupsListBox for each avatar that has associated avatar animation tracks in the file.

- Recommendations

It is proposed to adopt the proposal in section 4 of this contribution for the carriage of avatar animation streams in ISOBMFF-based ARF containers and to update future draft versions of ISO/IEC 23090-39 accordingly.

- References

- WG03N01316, “Procedures and Test Formats for Avatar Representation Formats as Part of MPEG-I”, MPEG#147, Sapporo, Japan, July 2024.

- WG03N01453, “Text of ISO/IEC CD 23090-39 Avatar Representation Formats”, MPEG#149, Geneva, Switzerland, January 2025.

- m70326, “[SD][Avatar] MPEG Avatar Animation Stream”, MPEG#148, Kemer, Türkiye, November 2024.

- m71290, “[SD][Avatar][ARF] Animation Bitstream and Compression with Landmarks”, MPEG#149, Geneva, Switzerland, January 2025.

- WG03N01468, “EE Description for Avatar Representation Formats”, MPEG#149, Geneva, Switzerland, January 2025.

## 1.9 Extracted from TuC

## 1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case  can be with live content while another in the on-demand content.

## 1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria #1 | Description | The technical solution should minimize/optimize … |

## 1.12 Timeline

## MPEG #150: Document solutions and evaluation results and recommend changes to the CD text.1.13 References

[1] WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] WG03 N0XXXX, “WD of ISO/IEC 23090-38 Avatar representation formats”, Kemer, Turkey, November 2024.

[4] WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.

EE4: Content Discovery and Partial Access (ongoing)

1.1	Introduction

This Exploration Experiment (EE) aims to evaluate solutions to address the issue of content discovery and partial access.

1.2 Problem statement

MPEG is currently specifying an Avatar Representation Format (ARF) for the storage and animation of 3D avatars. The ARF specifies an ARF document that provides a detailed description of all the content of the ARF container. The ARF document itself is stored inside the ARF container.

When retrieving or handling ARF containers, e.g. to animate the avatar of another participant in an AR call, the user needs to retrieve only a subset of that ARF container.

Partial access is required for the following reasons:

- ARF container contains more assets than the ones needed and authorized for access during an Avatar session

- ARF container stores the assets in multiple levels of detail, which makes the file size relatively large. A user only needs a selected set of assets at a specific level of detail and wouldn’t need to download all levels of detail.

However, because the ARF document contains a complete description of the assets and levels of detail and it is stored inside the container, there is no way to know and select what subset is needed prior to downloading the full container.

1.3 Use cases relevant for the EE

The work of the EE is based on the requirements defined in N00359 and use cases identified in m64008.

1.4 Related (WG2) and Extracted (new) Requirements

The following requirements are relevant for this EE:

- Total or partial delivery and presentation of the avatar attributes, depending on the client network and device for instance.

- Store and access the different components and assets of the base model independently in the file,

- enable the protection of parts of the base Avatar against tampering,

- enable the carriage and access to authentication features as part of the supported media streams.

1.5 Relation to other activities (EE, requirements, etc…)

1.6 Mandates

The mandate of this EE is to develop and evaluate a solution for the discovery of the content of an ARF container and for partial access to it.

1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | L |
| Interdigital | Joao, Quentin | Joao.Regateiro@InterDigital.com Quentin.Avril@InterDigital.com | P |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | P |

(P = proponent, L = leader)

1.8 Information about proposed technologies

The following contributions on Content Discovery and Partial Access have been submitted:

Meeting #149

m71327 ARF Manifest

Meeting #150

m72262 [Avatar][EE4] Enabling Partial Access for MPEG ARF

This proposal has been accepted for inclusion in the EE document to be further studied as part of the EE discussion. The following comments were recommented to be added to the EE document for reference:

Discussion:

- @ibouazizi similarity to our discussion, but where to stop? Proposes at the some level of details we stop

  - @joao.regateiro mostly agree, but need to check how we subselect.

  - @ibouazizi why would have multiple meshes in the same LOD?

The current version of the proposal is described below:

1.  Introduction

This contribution presents one solution to enable partial access to the MPEG Avatar Representation (ARF) which is the main concern of EE#4 [1]. The ability to access parts of the ARF document and container permits applications to easily extract information from the document without the need to load the complete ARF container that can occupy several megabytes or gigabytes of storage.

2.  Avatar Manifest

As detailed in Table 1, the proposed avatar manifest file format first contains high-level information of the ARF document. The manifest file format can be represented in human readable format, such as JSON-based.

Table 1: Proposed metadata avatar format

| Name | Type | Default | Description | Required |
| --- | --- | --- | --- | --- |
| version | string | N/A | File version | Yes |
| name | string | N/A | Name of the avatar. | Yes |
| description | string | N/A | Description of the manifest file related to the ARF document it refers. | Yes |
| id | integer | N/A | Identifier of the ARF document to which the manifest is apply on. | Yes |
| representation | AvatarRepresentation | N/A | A lower-level representation of an avatar. The representation references an ARF document with a specific set of features. | Yes |

The version describes what version of ARF encoding the manifest is using.

The name provides the name of the avatar of the ARF document reference by the identifier.

The description gives a semantic description of the manifest file in relation to its reference to the ARF document e.g., “avatar with only head representation and facial expression animation streams”.

The id corresponds to the identifier of the ARF document. This identifier is used to find and retrieve the ARF file that the manifest applies to. The identifier of and ARF file shall be unique among other formats, thus the id of the manifest shall match one single ARF file.

The AvatarRepresentation contains information related to different components of the ARF file that permits to discriminate the components pointed to by the manifest. The representation points to an “asset” and contains additional “content” information about that specific asset.

Table 2 details the semantic of an avatar representation. Each representation defines an asset and the content of the asset. The content of the asset is represented with “ContentDescription”, which constructs the reduced ARF file by pointing to select components of the original ARF file.

Table 2: Semantic of AvatarRepresentation

| Name | Type | Default | Description | Required |
| --- | --- | --- | --- | --- |
| asset | integer | N/A | Identifier of the avatar asset in the ARF document | Yes |
| content | ContentDescriptor | N/A | Description of part of the avatar contained in the ARF document that is to be included in the ‘reduced content’ ARF document. | Yes |

The asset references an item in the list of assets of the ARF container it refers to.

The content contains the description of the component of the reduced ARF file.

Table 3 details the semantic of a content descriptor. The content descriptor defines the lowest level of data representation, including the geometry, animation or skeletal structured.

Table 3: Semantic of a ContentDescriptor

| Name | Type | Default | Description | Required |
| --- | --- | --- | --- | --- |
| lod | integer | N/A | lod of the avatar representation. | Yes |
| skin | integer | N/A | skin of the avatar representation. | No |
| mesh | integer | N/A | mesh of the avatar representation. | No |
| skeleton | integer | N/A | skeleton of the avatar representation. | No |
| blendshapeSet | integer | N/A | BlendshapeSet of the avatar representation. | No |
| landmarkSet | integer | N/A | LandmarkSet of the avatar representation. | No |

The lod references an item in the list of lods of the ARF container the manifest refers to.

The skin references an item in the list of skins of the ARF container the manifest refers to.

The mesh references an item in the list of meshes of the ARF container the manifest refers to.

The skeleton references an item in the list of skeletons of the ARF container the manifest refers to.

The blendshapeSet references an item in the list of blendshapeSets of the ARF container the manifest refers to.

The landmarkSet references an item in the list of landmarkSets of the ARF container the manifest refers to.

  - Manifest for ARF Decoding

This section illustrates the creation of ARF files and how the manifest file will be introduced to permit partial access.

Figure 1: Avatar processing pipeline using ARF encoding and manifest file conditioning the decoder to output a subset of the original encoded ARF document. This reduced-content ARF document becomes the output of the codec.

The manifest file may specify individual content items and resources from the source ARF file that should be included in the ‘reduced content’ ARF file that would be delivered to the client.

Proposal

It is proposed to consider the proposed ARF manifest format as a candidate solution for the EE#4 to permit partial access of ARF containers.

References

[1] WG03N01379, “EE Description for Avatar representation formats”, MPEG#147, Kemer, Turkey, November 2024.

m72366 Progressive download of ARF container

This proposal has been accepted for inclusion in the EE document to be further studied as part of the EE discussion. The following comments were recommented to be added to the EE document for reference:

Discussion:

- @ahmedhamza is it a single request, or a client driven access to the file?

  - @ibouazizi agree, we need to study the use case a bit more. For example I have an index of all assets and the client does selection. Or there is one avatar and you download once the quality gets better. Other use cases, where the user does not event wanna know.

  - @ahmedhamza may also need different solutions for different use cases

  - @ibouazizi good study topics

- @joao.regateiro when we refer to textures, this is not defined.

  - @ibouazizi may need more details.

- @joao.regateiro other than ISO BMMF possible?

  - @ibouazizi zip does not work, it is arbitrary.

The current version of the proposal is described below:

1 	Introduction

The ISO/IEC 23090-39 Avatar Representation Format (ARF) is a developing standard for representing realistic 3D human avatars in the MPEG-I framework​. ARF uses a JSON-based description of an avatar’s structure (geometry, textures, animations, etc.) along with associated binary asset data (mesh files, image textures, etc.). High-fidelity avatars can be quite large in size, which poses challenges for timely transmission and rendering in real-time applications.

In particular, requiring a client to download an entire ARF container of the base avatar before rendering leads to high startup latency and poor user experience. To address this, the concept of progressive download allows an avatar to be transmitted and reconstructed with incremental quality, so that a basic version can be displayed quickly and then refined as more data arrives.

This contribution proposes a solution to realize progressive download for ARF avatars by leveraging the ISO Base Media File Format (ISOBMFF) container structure and HTTP byte-range requests.

2 	Progressive Download using ISOBMFF

To solve the above problem, we propose using the ISO Base Media File Format (ISOBMFF) as a container for ARF that is organized to support progressive, byte-range accessible content.

The solution has several key components:

- ISOBMFF Container Structure for ARF with LoDs: We define an organization of the avatar data within an ISOBMFF file such that the ARF document comes first, followed by binary asset data grouped by increasing Level of Detail. All LoD asset data are stored sequentially in the file. This ordering ensures that a prefix of the file contains a complete low-detail avatar, and additional bytes correspond to higher details.

- External Index (Manifest) File: We introduce a lightweight external index that enumerates the byte ranges within the ISOBMFF file for each significant component: the ARF document, and for each asset, each LoD’s data. This index allows a client to issue HTTP GET range requests for specific parts of the file without needing to download the whole file or parse it fully upfront.

- Inter-dependency coding of LoDs: higher-detail assets may be encoded as dependent enhancements of lower-detail representations. Rather than storing completely separate meshes or textures for each LoD, the higher LoD data represents difference information (deltas or residuals) that refine the lower LoD. Thus, LoD0 provides a base layer that can stand on its own, LoD1 provides data that upgrades LoD0 to a finer quality, LoD2 upgrades further, and so on. This inter-dependent layering applies to both mesh geometry and texture images.

- Progressive parsing and reconstruction: The container is structured to remain parsable even if not fully downloaded. The ARF document (at the start of the file) can be parsed independently and contains references or placeholders for the assets. As each additional chunk is downloaded, the client can incrementally integrate it to progressively enhance the avatar. Even with a partial file, the client will have a coherent, if lower-detail, avatar that can be displayed.

3.1 	ISOBMFF File Structure for Progressive ARF

The ISO Base Media File Format is a widely used container format, known for its flexibility in organizing media data and metadata. We leverage ISOBMFF to package the ARF base avatar such that progressive download is enabled.

In the proposed file format layout, the content is arranged as follows:

- File Header and Basic Boxes: An ftyp box identifies the file type, and a meta box at the beginning contains high-level metadata. This metadata includes either a manifest of item locations or track information that tells the structure of the file. Crucially, all metadata needed to find assets is at the front of the file, so that it can be read without reading the entire file.

- ARF document: Immediately after the header/metadata, the ARF document is placed. The ARF document contains the avatar’s descriptive information. By placing it at the start, we ensure a client can retrieve and parse the avatar’s structure with minimal download.

- For the key assets, such as head and body of the avatar:

  - Level of Detail 0 Assets (LoD0): Following the ARF document, the binary assets for the lowest level of detail are stored. LoD0 represents the base avatar, e.g. a simplified mesh (with relatively few vertices/triangles) and low-resolution texture images. These assets are encoded such that they can be used independently to render a recognizable avatar, albeit with reduced detail.

  - Level of Detail 1 Assets (LoD1): After LoD0, the next chunk of the file contains the LoD1 of the key avatar assets. LoD1 provides refinements that, when combined with LoD0, yields a higher-detail avatar. The data for LoD1 could include additional geometry information (e.g., vertices to increase mesh density, indices for additional triangles, or delta corrections to LoD0 vertex positions) and enhanced texture detail (e.g., higher resolution texture data or high-frequency details). LoD1’s data is placed immediately after LoD0’s bytes in the file.

  - The same goes for higher LoD levels.

- Other assets such as garments may be interleaved or follow the LoDs of the main assets of the base avatar.

The following figure depicts how the file is laid out in increasing order of LoD levels.

Figure 1: ISOBMFF container layout for progressive ARF.

The ARF document is stored first (yellow segment), followed by assets grouped by Level of Detail: LoD0 (blue), LoD1 (pink), LoD2 (green). Each subsequent LoD’s data is appended after the previous. Byte offsets increase from left to right, so a client can stop downloading at any LoD boundary and still have a complete avatar at that level.

3.2 External Index File for Byte-Range Access

While the container is self-describing to some extent, we propose an external index file (or manifest) in JSON format to make it easy for clients to access the ARF container progressively. This external index can be retrieved ahead of the ARF container download. Its purpose is to facilitate HTTP range requests by providing a map of component to byte range.

A possible JSON index format could be as follows:

{ "file": "avatar123.progressive.arf", "components": [ { "name": "ARF_JSON", "offset": 0, "length": 5000 }, { "name": "LoD0_Geometry", "offset": 5000, "length": 6000 }, { "name": "LoD0_Textures", "offset": 11000, "length": 4000 }, { "name": "LoD1_GeometryDelta", "offset": 15000, "length": 12000 }, { "name": "LoD1_TextureResidual", "offset": 27000, "length": 8000 }, { "name": "LoD2_GeometryDelta", "offset": 35000, "length": 20000 }, { "name": "LoD2_TextureResidual", "offset": 55000, "length": 20000 } ] }

Hypothetical index for an ARF container.

4 	Conclusion and Proposal

This contribution outlined a solution to support progressive download and rendering of MPEG-I ARF avatars using the ISOBMFF-based container and LoD-based layering of content. The solution addresses the issue of large base avatar downloading times by allowing partial, incremental data access and decoding.

We propose that this solution is documented as part of the EE on discovery and partial access and to perform further work to support partial progressive access to the ARF container.

5 	References

[1] 	ISO/IEC JTC1/SC29/WG03 N01453, “Text of CD ISO/IEC 23090-39 Avatar Representation Formats”, MPEG Meeting #149, Geneva, CH, February

m72371 Discovery and selection of assets from an ARF container

This proposal has been accepted for inclusion in the EE document.

1	Introduction

During the previous MPEG meeting, we formulated the problem of discovery and selection of assets within an ARF container. The problem arise in the following 2 scenarios:

- Avatar receiver wants to select from assets and LoDs available in an ARF container and then only download the selected assets,

- Avatar sender wants to grant access rights to a subset of assets and LoDs to a particular receiver through a third-party server.

In both scenarios, means to discover the content of an ARF container and partially access them would be required.

We propose the definition of an ARF Manifest, which is stored outside the ARF container, to address the problem of discoverability and partial access.

2	ARF Manifest

The ARF Manifest is a much more light-weight version of the ARF document, which indexes the content of an ARF container.

The ARF Manifest should contain the following information:

- User information: name, age, gender

- Identifier of the main ARF container, e.g. a URI/URL to the ARF container

- List of asset identifiers, which must all be described in the main ARF container

- Subset of the level of details for these assets

- List of compressors needed to decode the partial ARF container

- DRM protection information to gain access to the components of the partial ARF container

The partial ARF container is a logical container that is created through partial access to the main ARF container.

Since the ARF Manifest is eventually an entitlement document, its integrity has to be protected through the usage of proper integrity protection mechanisms, such as the storage of an encrypted hash.

3	Proposal

We propose to define an ARF Manifest based on the content of section 2 to enable discovery and partial access to ARF containers.

m72383 [SD][AVATAR] Supporting Asset Selection in ISOBMFF ARF Containers

This proposal has been accepted for inclusion into the revised CD with the following comment:

- Accepted but make the box in the discussion optional for now. Add a note to the text to study further whether this should be made mandatory here or it is something that applications can deal with.

- We adopt the proposal in section 3 of this contribution for supporting selective access to avatar assets and associated animation streams in ISOBMFF-based ARF containers and to update future draft versions of ISO/IEC 23090-39 accordingly

- The author (@ahmedhamza) works with the editor (@ibouazizi) to integrate the text into the standard doc.

The current version of the proposal is documented below.

1. Introduction

In the MPEG#137 meeting, the MPEG Systems workgroup (WG03) has agreed on creating a new specification (ISO/IEC 23090-39) for defining an MPEG Avatar Representation Format and the committee draft (CD) of the specification was published in [1]. A number of exploration experiments on different aspects related to the avatar representation format are also ongoing and documented in [2], including EE4 on content discovery and partial access.

This contribution focuses on ISOBMFF-based ARF containers and proposes a solution to enable a player to select and access individual assets of a base avatar as well as any animation data associated with these assets.

2. Background

In [2], the user’s base avatar model in ISOBMFF-based containers is stored as metadata items with the MetaBox being declared at the file level. A PrimaryItemBox shall be present and shall contain the item identifier of the item that contains the ARF document.

The following requirements are also defined:

- The HandlerBox shall have a handler_type set to 'AVRF'

- The primary item shall declare content_type of "model/ARF+json"

- It may contain an item protection box that defines the encryption for the components of the base avatar model that are protected.

- each component of the base avatar model, including the different LoD variants, shall be stored as an independent item.

- Proposal

  - Selecting Avatar Assets

To select an asset from an ISOBMFF ARF container, a player first needs to retrieve the ARF document from the primary item in the ISOBMFF container and identify which asset and level-of-detail it needs. The player then identifies the set of items that correspond to the components of that asset and LoD combination.

Each component item is associated with an AvatarComponentInfoProperty that describes which avatar, asset, and level-of-detail the component is associated with. A corresponding AvatarComponenatInfoProperty instance shall be present in the ItemPropertyContainerBox of the ItemPropertiesBox, defined in ISO/IEC 14496-12, for each component item.

The AvatarComponentInfoProperty is defined as follows.

Aligned(8) class AvatarComponentInfoProperty

extends ItemProperty(‘avcp’) {

unsigned int(1) static_association_flag;

bit(7) reserved = 0;

if (static_association_flag) {

unsigned int(8) avatar_id;

unsigned int(8) asset_id;

}

unsigned int(4) component_type;

unsigned int(4) level_of_detail;

}

The semantics of the fields of AvatarComponentInfoProperty are as follows:

static_association_flag is a flag indicating if the component is associated with a single avatar. Value 0 indicates that the component may be associated with more than one avatar. Value 1 indicates that the component is associated with a single avatar whose identifier is given by the avatar_id field.

avatar_id is the unique identifier for the avatar that this component is associated with. This field is only present if static_association_flag is set to 1.

asset_id is the unique identifier for the avatar asset that this component is associated with. This field is only present if static_association_flag is set to 1.

component_type is an integer indicating the type of the component. Values 0 to 5 designate the component types: skeleton, skin, mesh, node, blend shape set, and landmark set, respectively. Other values are reserved for future use.

level_of_detail indicates the level of detail of the asset to which the component is associated.

The association between each component item and its AvatarComponentInfoProperty is done using the ItemPropertyAssociationBox, defined in ISO/IEC 14496-12. The essential bit flag shall be set to 1 for each property entry in the ItemPropertyAssociationBox referring to an AvatarComponentInfoProperty, signalling that it is an essential property of the item.

To identify all the components that relate to a particular avatar model in the container, a SingleItemTypeReferenceBox with reference type ‘avcr’ shall be present in the ItemReferenceBox, where the from_item_ID field is set to the item_ID of the avatar item and list of to_item_IDs corresponding to each component item.

  - Association of Assets with Avatar Animation Tracks

The set of avatar animation tracks associated with an asset shall be grouped with the asset using an AvatarAssetAnimationGroupBox defined as follows.

aligned(8) class AvatarAssetAnimationGroupBox()
    extends EntityToGroupBox('avag', version=0, flags) {

unsigned int(8) avatar_id;

unsigned int(8) asset_id;

unsigned int(4) level_of_detail;
}

The semantics of the fields of AvatarAssetAnimationGroupBox are as follows:

avatar_id is the unique identifier for the avatar associated with this entity group.

asset_id is the unique identifier for the avatar asset that this entity group is associated with.

level_of_detail indicates the level of detail of the asset to which the component is associated.

Note that in [3] we proposed an AvatarAnimationGroupBox to group the avatar animation tracks associated with an avatar model. That entity group included only one field for the avatar_id. It is possible to extend that entity group to include the asset_id and level_of_detail fields instead of defining two separate entity groups one for the association with the avatar and one for association with an avatar asset.

- Recommendations

It is proposed to adopt the proposal in section 3 of this contribution for supporting selective access to avatar assets and associated animation streams in ISOBMFF-based ARF containers and to update future draft versions of ISO/IEC 23090-39 accordingly.

- References

- WG03N01453, “Text of ISO/IEC CD 23090-39 Avatar Representation Formats”, MPEG#149, Geneva, Switzerland, January 2025.

- WG03N01468, “EE Description for Avatar Representation Formats”, MPEG#149, Geneva, Switzerland, January 2025.

- m72382, “[SD][AVATAR] Carriage of Avatar Animation Streams in ISOBMFF ARF Containers”, MPEG#150, Online, April 2025.

1.9 Extracted from TuC

1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case can be with live content while another in the on-demand content.

1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria #1 | Discovery | The technical solution should facilitate the discovery of the content of an ARF container. |
| Criteria #2 | Partial Access | The technical solution should enable the partial access to the ARF container. |
| Criteria #3 | Authorization | The technical solution should enable the ARF owner to grant partial access to ARF container. |

1.12 Timeline

MPEG #150: Document solutions and evaluation results and recommend changes to the CD text.

1.13 References

[1] 	WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] 	WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] 	WG03 N01398, “WD of ISO/IEC 23090-39 Avatar representation formats”, Kemer, Turkey, December 2024.

[4] 	WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.

EE5: Animation Controllers (ongoing)

1.1 Introduction

This Exploration Experiment (EE) aims to evaluate and develop avatar animations based on controllers. This animation technique is an extension of blendshapes (or morph targets) which allows the deformation of the avatar mesh given a set of weights.

Controllers are widely used in the content creation industry, since they can allow complex animations given few parameters. This parameter reduction compared to other techniques like blendshapes also reduces the memory footprint or bandwidth of animations parameters.

Controllers can be encoded in the MPEG Avatar Representation Format (ARF) [1] using widely used and proven approaches based on channels encoding.

## Deformation function depending on a weight

Controllers are features widely used in industry to animate rigs, including humanoid rigs. They are, to some extent, enhanced morph targets (or blendshapes). Like morph targets, a weight is attached to each controller: a zero weight has usually no effect, a positive weight starts the modification of the shape and a negative weight can have an inverse modification. There is also a weight range for each controller: the amount of modification is controlled and cannot go beyond predefined values.

Mathematically speaking, a controller can be expressed as a function which, given a mesh M and a weight w, returns a new mesh M’:

fM,w→M'

However, contrary to morph targets, modifications are not limited to a weighted addition of vertex displacements (or a linear combination of meshes in the case of blendshapes). In fact, a controller can perform any modification on a mesh given a weight. These modifications are not limited to the vertex coordinates and can also modify the texture for instance. Furthermore, the effect of the weight can be non-linear, contrary to morph targets, which only supports linear weight combinations.

In a way, a controller looks like an animation, except that time is replaced by weight, and a weight can be negative. An animation can be seen as a (possibly non-linear) function of time which maps a modification of the mesh. An animation can, depending on the encoding format, make many different modifications, like changing the morph target weights or changing the transform of a node. However, animations are made to do modifications from time 0 to the end of its time range. They are not supposed to be run with random time values. Furthermore, animations are not made to be combined, and especially with different random time positive values. On the other hand, controllers, like morph targets, are made to be combined with different random weight values (positive or negative).

Figure 3 illustrates the combination of two controllers:

- The first controller updates two morph targets and two joint transforms:

  - The first morph target moves eyelids to the left

  - The second morph target moves eyelids to the right

  - The first joint transform rotates the left eye around axis Y

  - The second joint transform rotates the right eye around axis Y.

- The second controller also updates two morph targets and two joint transforms:

  - The first morph target moves eyelids to the up

  - The second morph target moves eyelids to the bottom

  - The first joint transform rotates the left eye around axis X

  - The second joint transform rotates the right eye around axis X.

Note that each controller performs four updates (two morph targets and two rotations) given a single weight value. Without a controller, 10 values are required: one for each morph target and four for each rotation.

Figure 3 - Combining two controllers to animate the avatar gaze.

The first controller animates the eyes from left to right using weight w1.
The center row of Figure 3 shows the avatar state when weight w1=-1 (left), when weight w1=0  (center) and when weight w1=1 (right).

The second controller animates the eyes from bottom to top using weight w2.
The center column of Figure 3 shows the avatar state when weight w2=-1 (bottom), when weight w2=0 (center) and when weight w2=1 (top).

These two controllers can be used together to create numerous gazes. For example, the top-right avatar in the figure shows a state when both controllers have the same weight, e.g., w1=1 and w2=1. The top-left avatar in the figure is for first controller weight w1=-1 and second controller weight w2=1.

## Use in Asset Creation

Controllers are widely used and more than commonplace in the industry of visual effects, animation and gaming. They provide additional control levels to artists. Their use has now become mandatory for a better, simpler and quicker animation process.

Figure 4: Professional facial rig used in VFX industry. Facial controllers panel is on the right and quickly enables to create realistic facial expressions as depicted in the bottom pictures.

When making a picture film, a game or any virtual experience, characters’ assets go through a long and complex asset creation pipeline. Excluding character designers and storytelling artists, modelers (modeling artists) are, first, sculpting blendshapes, considered as the lowest level of control for animation. An industrial consensus has converged for a few decades to the use of the FACS (Facial Action Coding System) for the creation of facial blendshapes because movements of individual facial muscles are encoded by the FACS. Manipulating FACS blendshapes to create facial expressions is time-consuming, not intuitive and not ergonomic. This is the reason why riggers (rigging artists) are, in a second step, setting up the asset rig on top of the mesh and blendshapes created previously. The rig contains all the elements that will be used to deform the assets’ mesh (skeleton + skinning, controllers etc.). This is where the controllers play a key role in aggregating blendshapes, defining new interpolations, linking skeleton joint and blendshapes etc.

## Benefits of Controllers

Compared to blendshapes and joint transforms, controllers have the following advantages:

- Reduced bandwidth. Transmitting the parameters of an animation using controllers requires less memory than blendshapes and joint transforms, since a single controller can update several blendshapes and joint transforms. In the previous example, each controller (1 parameter) updates two blendshapes (2 parameters) and 2 rotations (8 parameters if using quaternions, 32 if using matrix transforms).

- Genericity. Controllers can be tuned to control high-level animations and their behavior be updated without the need for changing the input parameters. For example, controllers can be defined to rotate the elbow. In a usual case, they only update a joint transform. But if, for some reason, additional modifications are required, there is no need to change the user input and signal to transmit. 
For example, controllers can also use blendshapes to show the activity of muscles if the avatar lifts something heavy. Without controllers, the user must send updates for the muscles blendshapes: the joint transform is not generic enough to know how to tune such blendshapes without additional knowledge. With controllers, the user only has to send the parameters for the controllers, and the avatar can show additional update.

- Semantics. Controllers can be designed for high level changes of an avatar which requires the combination of several updates. For example, controllers can be defined to change the physical properties of an avatar (weight, size, etc.). The meaning of such controllers is understandable by any user which can use them to update their avatar. For example, a user who wishes to reflect the fact that (s)he has gained or lost weight could use a dedicated controller.

1.2 Problem statement

The following statements for animation controllers are identified:

- Providing precision and flexibility needed to create realistic and engaging avatar movements

- Providing high-level control of avatar’s animations

- Providing high-level description of human characteristics (emotions, motions, appearance, morphology etc.)

- Providing use cases and needs for real-time animation

1.3 Use cases relevant for the EE

The work of the EE is based on the requirements defined in N00359 [2] and use cases identified in m64008 [4].

1.4 Related (WG2) and Extracted (new) Requirements

- The following requirements, based on the document N0347 [5] are relevant for this EE:

- Facial and body animation, including facial expressions due to communication or social interactions and avatar motion in scenes.

- High and low-level control mechanisms to efficiently animate the avatar and efficiently transmit animation parameters over the network (between server and client).

1.5 Relation to other activities (EE, requirements, etc…)

1.6 Mandates

1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Interdigital | Quentin Avril | quentin.avril@interdigital.com | L |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | P |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | P |

(P = proponent, L = leader)

1.8 Information about proposed technologies

The following contributions have been submitted:

- Meeting #147

  - m68986 [42.2] [SD] – Avatar JSON Interchangeable Format

- Meeting #148

  - m69421 Avatar JSON Interchangeable Format

  - m69474 Avatar JSON Interchangeable Format high level overview

  - m69936 Draft Procedures and Test Scenarios for MPEG Avatar Representation Format as part of MPEG-I

- Meeting #149

  - m71239 Animation Controllers in ARF (Avatar Representation Format)

Meeting #150

m72265 [Avatar][EE5] Animation Controllers in Avatar Representation Format

  - This proposal has been accepted as part of the EE description, with the following comment: To move this to DIS in the first stage, we would like to get clearer indication on support for this (NBs, other companies) to determine whether it is in the priority 1 list. We will decide at MPEG#151 whether this will be part of phase 1. The content is described below.

1 Introduction

This contribution is a follow up on avatar animations based on controllers. The previous contributions were:

- (MPEG#149) m71239 - [SD][Avatar][ARF] Animation Controllers in ARF (Avatar Representation Format) [1]

- (MPEG #148) : m69421 - Avatar JSON Interchangeable Format [2]

The principal advantage of using controllers is the reduction of the memory footprint or bandwidth of animations parameters (compared to other techniques like blendshapes) and the high-level control of animation components that they provide.

Controllers can be encoded in the MPEG Avatar Representation Format (ARF) [3] using widely used and proven approaches based on channels encoding. They enable to control:

- Blendshape weights

- Joint transforms

- Controller weights

- Parametric texture weights

- Texture UV transforms

- Uniform shader variables

2 Summary

Controllers are features widely used in industry to animate rigs, including humanoid rigs. They are, to some extent, enhanced morph targets (or blendshapes). By combining several blendshapes and joint transforms into a single object, they add higher-level semantics to the representation of avatar animation. Like morph targets, the action of each controller is driven by a scalar weight. However, controller weights may be set to a negative value to represent the inverse transformation to the one associated to the opposite positive weight.

Advantages of Controllers

Compared to blendshapes and joint transforms, controllers have the following advantages:

- Reduced bandwidth – Far fewer controllers are needed to drive the animation of an avatar than individual blendshapes and joint transforms. As a result, far fewer animation parameters need to be transmitted in the animation stream.

- Genericity – A controller represents a high-level animation control of an avatar using the combination of several lower-level controls such as blendshapes or joint transforms. The behavior of the controller can be fine-tuned for each avatar by changing the properties of these lower-level controls and their relative weights. However, the control parameters that drive the controllers and are exposed in the animation stream remain unchanged regardless of these internal modifications. Hence, the same set of controllers can be used to animate avatars with a variety of morphologies and behaviors. For example, a controller can be defined to rotate the arm around the elbow. Typically, this will involve a single joint transform. However, for more realism, the controller representation may need to be enriched with corrective blendshapes to model the deformation of the upper arm caused by the biceps bulge. The addition of these corrective blendshapes can be performed without changing the controller weight that is exposed in the animation. The weights driving the influence of the corrective blendshapes depend on the rotation angle of the arm around the elbow. This dependency can be encoded once and for all inside the controller by non-linearly adapting the corrective blendshape weights to the elbow rotation angle. Without controllers, the corrective blendshape weights must be dynamically adapted to the elbow joint rotation in each animation stream involving a rotation of an avatar arm.

- Semantics - The meaning of the controllers is easily understandable by any user. For example, a user who wishes to reflect the fact that (s)he has gained or lost weight could use a dedicated controller with this semantic. Hence, controllers facilitate the interoperability of avatar representations across platforms.

3 Gaze Example

As described in the previous contribution m71239, let’s take the example of the avatar gaze on Morgan [3]. Morgan has:

- 4 blendshapes for the eyelids (eyeUp, eyeDown, eyeLeft and eyeRight)

- 2 joints for the eye globes (one for the left one and one for the right one).

The blendshapes are controlled by a weight encoded as a float value from 0 to 1. The joints are controlled by an orientation encoded as a tuple composed of 3 floating values corresponding to the orientations around the 3 axis X, Y and Z.

- 4 blendshape weights: weyeUp, weyeDown, weyeLeft and weyeRight

- 2 joint tuples: (x, y, z) and (x’, y’, z’)

- In that scenario, animating Morgan’s gaze requires 4 + 2x3 = 10 floating values to be streamed.

Now, instead of this animation setup, let’s use 2 controllers. One controller in charge of controlling the horizontal gaze and one controller for the vertical gaze:

- The 1st controller, in charge of the horizontal gaze, controls 2 blendshapes (eyeLeft and eyeRight) and 2 joint transforms.

- The 2nd controller, in charge of the vertical gaze, controls 2 blendshapes (eyeUp, eyeDown) and 2 joint transforms (rotation of each eye around the X axis).

Each controller is controlled by a weight encoded as one single floating value wvertical and whorizontal. With these two controllers, all the gaze positions can be performed.

- In that scenario, animating Morgan’s gaze requires 2 floating values to be streamed.

4 Control of texture, UV and shaders

Texture

The use of controllers can be extended to the control of visual elements such as textures, UV coordinates and/or shaders. This allows a powerful and high-level semantic control of the avatar’s elements. For example, based on the same gaze example, one can decide that both controllers (vertical and horizontal) also control the texture colors using parameters. This allows to add more realism in creating skin color variations (blood flow, skin stretch, skin squeeze etc.).

UV

Animating UVs can be useful to create spectacular effects in rendering with few transformations. For instance, to animation a fluid (like water or lava) a good UV setup and a shift allow one coordinate (U or V) can lead to very good if not photo realistic renderings.

Shaders

Animating shader could enable to change the various rendering properties of the avatar. It could be like the surface color or texture but also the vertex positions of the mesh. And of course, you can combine all of that into more complete and complex animations.

5 Encoding in ARF

Controllers are represented in ARF as a collection of channels, each of which represents a blendshape, a joint transform or another controller. Internally to the controller, each channel is controlled by a weight. This set of per-channel internal weights defines the behavior of the controller. The action of the controller as a whole is driven by an external weight that is exposed in the animation streams.

Controllers can be encoded in MPEG ARF through the extension of the components root property:

Table 1 - Properties of the components root property

| Name | Type | Use | Description |
| --- | --- | --- | --- |
| skeletons | array(Skeleton) | O | A list of skeletons used to describe the avatar skeletal asset. |
| skins | array(Skin) | O | A list of assets that are stored in this ARF container. |
| meshes | array(Mesh) | M | A list of geometries used to describe the avatar asset. |
| nodes | array(Node) | O | A list of nodes used to organize, merge and describe and transform the avatar components. |
| blendshapeSets | array(Blendshapes) | M | A list of blendshape sets used to describe the avatar animations. |
| landmarkSets | array(number) | O | A list of references to landmark sets in the ARF container. |
| controllerSets | array(ControllerSet) | M | A list of controller sets used to describe the avatar animations. |

All the detailed technical specifications are described in the m71239 document of meeting MPEG#149

Conclusion

We recommend adding support for controller-based animation in the MPEG Avatar Representation Format (ARF) [3], for the reasons presented in this contribution:

- Reducing memory footprint

- Reducing animation bandwidth

- Providing much more control levels and possibilities.

It is proposed to extract from the EE#5 the technical content relative to the support of controllers and merge it with an improved CD.

1.9 Extracted from TuC

1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case can be with live content while another in the on-demand content.

1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria#1 | Description | The technical solution should provide a high-level description of human characteristics (motion, emotion etc.) |
| Criteria#2 | Control | The technical solution should provide a high-level control of human animation |

1.12 Timeline

2022-03-30: MPEG #150: Evaluation documents and changes to the Committee Draft text.

1.13 References

[1] WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] WG03 N0XXXX, “WD of ISO/IEC 23090-38 Avatar representation formats”, Kemer, Turkey, November 2024.

[4] WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.

[5] ISO/IEC JTC 1/SC 29/WG02 N0347 “Requirements and state of the art for MPEG media avatar”, Online, January 2024

EE6: Seamless Asset Linking with Animation Capabilities (opened)

1.1	Introduction

This Exploration Experiment (EE) aims to evaluate solutions to address the issue of how to seamlessly link the geometry of multiple assets.

1.2 Problem statement

MPEG is currently specifying an Avatar Representation Format (ARF) for the storage and animation of 3D avatars. The ARF specifies an ARF document that provides a detailed description of all the content of the ARF container.

The ARF document itself is stored inside the ARF container which contains the ARF data represented as multiple independent assets (e.g., head and body), each with distinct meshes and skinning data connected via a common skeleton. However, ARF currently lacks explicit signaling or data structures that enable geometric fusion of these assets into a seamless, continuous avatar surface. As a result, assets often exhibit visible seams when dynamically combined, or require pre-processing, limiting modularity and interoperability.

1.3 Use cases relevant for the EE

1.4 Related (WG2) and Extracted (new) Requirements

The following requirements are relevant for this EE:

- Total or partial delivery and presentation of the avatar attributes, depending on the client network and device for instance.

- Support connectivity between distinct assets for seamless rendering and animation

- Support for assets attachments or anchoring on the geometry of the base avatar

1.5 Relation to other activities (EE, requirements, etc…)

1.6 Mandates

The mandate of this EE is to investigate the problem statement and create assets to demonstrate the use case and validate the issue, develop and evaluate a solution for liking geometry assets of an ARF container while preserving traditional animation techniques (blendshapes, skeletal animations). Provide storage, access and signaling of different assets to achieve seamless renderings when animating the based avatar with fused geometries.

1.7 Participants

| Participant | Contact | Email | Type |
| --- | --- | --- | --- |
| Interdigital | Joao Regateiro Quentin Avril | Joao.Regateiro@InterDigital.com quentin.avril@interdigital.com | L |
| Qualcomm | Imed Bouazizi | bouazizi@qti.qualcomm.com | P |
| Xidian Univ. | Anthony Trioux | anthony_trioux@xidian.edu.cn | P |

(P = proponent, L = leader)

1.8 Information about proposed technologies

The following contributions on asset linking have been submitted:

Meeting #150

m72369 ARF Asset Linking

1.9 Extracted from TuC

1.10 Test cases

The EE may define test cases for which the evaluation criteria will be analyzed. For instance, a first test case can be with live content while another in the on-demand content.

1.11 Evaluation criteria

List of criteria that will allow to compare the different technical solutions and converge to a unique solution. Criteria can be objective like memory efficiency, bitrate or subjective flexibility, compatibility with legacy solution, etc.

| Criteria | Description | Evaluation |
| --- | --- | --- |
| Criteria #1 | Fusion | The technical solution should facilitate the fusion of geometry between assets in the ARF container. |
| Criteria #2 | Animation | The technical solution should enable animation of the newly fused geometries seamlessly without artifacts that compromise the realism of the geometry, appearance and animation of the base avatar. |
| Criteria #3 | Swapping | The technical solution should enable selecting and swapping different geometries, such as, body geometries, accessories or other assets part of the base avatar. |

1.12 Timeline

MPEG #151: Document solutions and evaluation results.

1.13 References

[1] 	WG03 N01316, “Procedures and Test Scenario for MPEG Avatar Representation”, MPEG#147, Sapporo, Japan, July 2024.

[2] 	WG02 N00359, “Draft of MPEG-I Phase 3 requirements”, MPEG#146, Rennes, France, April 2024.

[3] 	WG03 N01453, “CD of ISO/IEC 23090-39 Avatar representation formats”, Geneva, Switzerland, January 2025.

[4] 	WG02 m64008 “Use Cases for MPEG Media Avatar”, Hanover, Germany, October 2023.